3GPP TSG RAN WG1#124                                 R1-2601034

Gothenburg, SE, February 9th ~ 13th, 2026

Agenda Item:	9.1.1

Source:	Transsion Holdings

Title:	Discussion on CSI spatial/frequency compression

Document for:	Discussion and decision

- Introduction

In RAN#108[1], a new WID for Rel-20 AI/ML evolution was approved[1], with the following objectives to enhance inference related aspects of CSI spatial/frequency compression without temporal aspects:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we discuss enhancements of inference related aspects of CSI spatial/frequency compression without temporal aspects.

- 2 Discussion

- 2.1 Inference related aspects

- 2.1.1 Measurement and report configuration

Regarding inference report configuration, the following agreement was achieved in RAN1#122 meeting.

Agreement: For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Regarding the spec impacts of CSI report configuration of CSI compression, the signaling to trigger the AI/ML model CSI compression, pairing mechanism between CSI generation model at UE and CSI reconstruction model at Network, and how Network indicates the CSI feedback payload to UE should be specified. In addition, regarding the CSI report related aspects, including the CSI-RS resource configuration, CSI reporting configuration, and CSI mapping, should be specified as well.

Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered:

- Signaling to trigger the AI/ML model CSI compression

- Pairing between CSI generation model and CSI reconstruction model

- Indicate the CSI feedback payload

- CSI-RS resource configuration

- CSI reporting configuration

- 2.1.2 CQI determination

Regarding CQI determination, two options on CQI calculation were achieved in RAN1#112 meeting.

Agreement: In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Regarding Option 1a, if CQI is calculated based on target CSI with realistic channel measurement, the calculated CQI is overestimated since the CSI reconstruction error is not considered. Regarding Option 1b, if CQI is calculated based on target CSI with realistic channel measurement and potential adjustment, the CQI compensation can be achieved based on the adjustment information and the calculated CQI may be more accurate. Regarding Option 1c, if CQI is calculated based on legacy codebook, the calculated CQI may be underrated due to more channel information loss incurred by legacy codebook. Regarding Option 2a, if CQI is calculated based on CSI reconstruction output, the CQI is calculated based on the assumption that CSI reconstruction model is deployed at UE side. Hence, the model shall be transferred or delivered between two sides. Regarding Option 2b, if CQI is calculated using two stage approach, the calculated CQI may be outdated since two stage approach may result in large delay. Therefore, regarding CQI determination, we support Option 1b.

Proposal 2: Regarding CQI determination, support Option 1b:

- Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

- 2.1.3 Quantization methods and payload determination

Regarding quantization method and payload determination, the following agreements were achieved in RAN1#122b meeting.

Agreement: Support NW side optionally exchanges codebook to the UE side. - If NW side exchanges a codebook, the exchanged codebook is associated to the pairing ID. - FFS SQ / VQ / both is supported - FFS the details, e.g., whether exchange the NW-defined codebook or the ID of a standardized SQ codebook in case of multiple standardized codebooks. - If NW side does not exchange a codebook or an ID of a standardized SQ codebook in case of multiple standardized codebooks, a standardized SQ codebook is associated to the pairing ID. - Note: For the same quantization resolution, a common codebook is used for all the scalars/segment(s), all the layers of pre-coding matrix and all parameter pairs/triplets for the associated pairing ID. Agreement: If target CSI type is pre-coder matrix, at least support per-layer payload parameter pairs {32, 2}, {64, 2}, {128, 2}, {192, 2} - FFS: Whether support per-layer payload parameter pairs {32, 1}, {48, 2}, {96, 1}, {96, 2}, {192, 1}, {128,10/128} - Note: The above pair refers to the pair of latent dimension and quantization resolution, i.e., {d, Q} for SQ or {d, Q/L} for VQ. Agreement: If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 128 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength. Rank1; Rank2; Rank3; Rank4. {64, 2}; {64, 2}, {64, 2}; {32, 2}, {32, 2}, {64, 2}; {32, 2}, {32, 2}, {32, 2}, {32, 2}. Agreement: If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 256 bits, support the following payload combinations across layers and ranks. The layers are reported in the order of decreasing strength. Rank1; Rank2; Rank3; Rank4. {128, 2}; {128, 2}, {128, 2}; {64, 2}, {64, 2}, {128, 2}; {64, 2}, {64, 2}, {64, 2}, {64, 2}. Agreement: If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 64 bits, select one alternative from following alternatives in RAN1 #124. - The layers are reported in the order of decreasing strength. - Note: For the purpose of comparison, consider the following: - For comparing Alt.1 and Alt. 2, consider payload scalability. Rank1; Rank2; Rank3; Rank4. Alt1; {32, 2}; {32, 2}, {32, 2}; NA; NA. Alt2; {32, 2}; {32, 2}, {32, 2}; {32, 1}, {32, 1}, {32, 2}; {32, 1}, {32, 1}, {32, 1}, {32, 1}. Agreement: If target CSI type is pre-coder matrix, at least for SQ, for the PC with rank-1 payload of 384 bits, select one alternative from following alternatives in RAN1 #124. - The layers are reported in the order of decreasing strength. Rank1; Rank2; Rank3; Rank4. Alt1.1; {192, 2}; {192, 2}, {192, 2}; {96, 2}, {96, 2}, {192, 2}; {96, 2}, {96, 2}, {96, 2}, {96, 2}. Alt1.2; {192, 2}; {192, 2}, {192, 2}; {192, 1}, {192, 1}, {192, 2}; {192, 1}, {192, 1}, {192, 1}, {192, 1}. Alt2; {192, 2}; {192, 2}, {192, 2}; {128, 2}, {128, 2}, {128, 2}; {64, 2}, {64, 2}, {128, 2}, {128, 2}. Alt3.1; {192, 2}; {192, 2}, {192, 2}; {128, 2}, {128, 2}, {128, 2}; {96, 2}, {96, 2}, {96, 2}, {96, 2}. Alt3.2; {192, 2}; {192, 2}, {192, 2}; {128, 2}, {128, 2}, {128, 2}; {192, 1}, {192, 1}, {192, 1}, {192, 1}.

Quantization at CSI generation part refers to the process of projecting floating values to binary bits, which is usually the last step in CSI generation model; on the other hand, dequantization at CSI reconstruction part refers to the reverse procedure, which is usually the first step in CSI reconstruction model.

Regarding quantization methods, scalar quantization and vector quantization were studied in Rel-18 and Rel-19. For scalar quantization, each element in the latent vector will be mapped to a new discretized element and the elements of latent vector will be quantized one by one, where each element can be quantized with a few numbers of bits, i.e., the number of bits per latent dimension. Since each element in the latent vector is quantized with a few numbers of bits, the number of bits per latent dimension should be aligned between Network and UE so that the reported bits can be dequantized in Network.

Since the bounded floating value of each element in the latent vector is mapped to a certain level, the the minimum and maximum range of quantization should be aligned between Network and UE.

Proposal 3: Regarding configuration of scalar quantization, the following can be aligned between Network and UE:

- The number of bits per latent dimension, i.e., Ql,ν

- The minimum and/or maximum range of quantization

For vector quantization, the elements of whole latent vector or the elements of segment of latent vector will be mapped to a pre-defined codeword or vector at a time. If the length of the latent vector is large and vector quantization is based on the elements of whole latent vector, it is practically difficult to design codebook. Hence, vector quantization based on the elements of segment of latent vector is preferable. For vector quantization based on the elements of segment of latent vector, the whole latent vector can be divided into several segments by segment size and the elements of segment of latent vector can be quantized with a few numbers of bits, i.e., the number of bits per segment. Hence, the number of bits per segment and segment size should be aligned between CSI generation part at UE and CSI reconstruction part at Network to achieve a satisfying performance.

Proposal 4: Regarding configuration of vector quantization, the following can be aligned between Network and UE:

- The number of bits per segment, i.e., Ql,ν

- Segment size, i.e., L

- The size of codebook

For vector quantization, the elements of segment of latent vector will be mapped to a pre-defined codeword or vector at a time and hence it is necessary to design codebook to support vector quantization. Regarding the codebook design, following two alternatives can be considered. Regarding Alt-1, a unified codebook is designed for all the segments of latent vector. By this way, a unified codebook can be shared for different segments of latent vector, where the complexity of codebook design and the elements of segment of latent vector mapping to codeword can be reduced. Regarding Alt-2, a dedicated codebook is designed for each segment of latent vector. By this way, a dedicated codebook can be applied to a dedicated segment, where the codebook may provide more accurate quantization.

Proposal 5: Regarding codebook design of vector quantization, the following alternatives can be considered:

- Alt-1: A unified codebook is designed for all the segments of latent vector

- Alt-2: A dedicated codebook is designed for each segment of latent vector

Regarding payload parameter, several candidate options of payload parameter (e.g.,dl,ν, L,Ql,ν) were achieved in the RAN1#122b meeting. Since the payload parameters (e.g.,dl,ν, L,Ql,ν) should be aligned to determine payload size and generate CSI feedback payload, the payload parameter should be fixed payload parameter or determined from multiple candidate options. Therefore, payload parameter can be configured by Network in CSI report configuration or reported by UE in CSI part 1 to achieve the alignment of payload parameters (e.g.,dl,ν, L,Ql,ν).

Proposal 6: Regarding the alignment of payload parameters (e.g.,dl,ν, L,Ql,ν), payload parameter can be configured by Network in CSI report configuration or reported by UE in CSI part 1.

- 2.1.4 UCI mapping

In legacy, PMI indicators are mapped to group 0/1/2 for eType II CSI feedback. If PUSCH resource is insufficient, a portion of Part 2 CSI information can be omitted according to the group index and priority value, which is defined based on layer index, SD basis index and FD basis index.

For CSI spatial/frequency compression, since the target CSI is compressed per layer, it is straightforward that compressed target CSI can be omitted per layer. However, compressed target CSI of an entire layer is dropped, the omission granularity is coarse. Thus, in addition to CSI omission per layer, the compressed target CSI can be omitted per segment for vector quantization and per scalar of latent space for scalar quantization.

Proposal 7: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered:

- Alt-1: Layer-based priority rule

- Alt-2: Segment based priority rule for vector quantization

- Alt-3: Scalar of latent space based priority rule for scalar quantization.

In legacy, regarding UCI mapping of Part 2 CSI, group 0/1/2 is defined for eType II CSI feedback, where wideband CSI is included in group 0 and even/odd subband CSI is included in group 1/2. However, for CSI spatial/frequency compression, since the target precoding matrices corresponding to all subbands are jointly compressed into a latent space, wideband and subband information can not derived from the compressed target CSI. Thus, the compressed target CSI can not be grouped based on the wideband and subband information. RAN1 should study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

Proposal 8: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

- 2.2 Performance monitoring

Regarding NW side monitoring , the following conclusion was achieved in RAN1#120b meeting.

Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side.

To correctly calculate SGCS, NW should be aware of the paired reconstructed CSI and Target CSI. Since multiple logged target CSI may be reported via high layer signaling, if the association between CSI feedback and target CSI does not exist, NW can not evaluate the performance. Hence, the association between CSI feedback and target CSI may be necessary. Regarding the association between CSI feedback and target CSI, the following two options can be considered. Regarding Option1, paired Target CSI and CSI feedback can be reported. However, since the associated CSI feedback has already been reported to the NW before, repeated CSI report may lead to the waste of UL resource. Regarding Option2, Target CSI and time information of paired CSI feedback can be reported, which may not lead to the waste of UL resource. By the way, association between CSI feedback and target CSI can be build with the time information of paired CSI feedback.

Proposal 9: Regarding the content of CSI report for NW side performance monitoring, the following options can be considered:

- Option1: Paired Target CSI and CSI feedback

- Option2: Target CSI and time information of paired CSI feedback

- 3 Conclusion

In this contribution, we discuss potential enhancements of inference related aspects of CSI spatial/frequency compression without temporal aspects. Based on the discussion above, we provide the following proposals:

Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered:

- Signaling to trigger the AI/ML model CSI compression

- Pairing between CSI generation model and CSI reconstruction model

- Indicate the CSI feedback payload

- CSI-RS resource configuration

- CSI reporting configuration

Proposal 2: Regarding CQI determination, support Option 1b:

- Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

Proposal 3: Regarding configuration of scalar quantization, the following can be aligned between Network and UE:

- The number of bits per latent dimension, i.e., Ql,ν

- The minimum and/or maximum range of quantization

Proposal 4: Regarding configuration of vector quantization, the following can be aligned between Network and UE:

- The number of bits per segment, i.e., Ql,ν

- Segment size, i.e., L

- The size of codebook

Proposal 5: Regarding codebook design of vector quantization, the following alternatives can be considered:

- Alt-1: A unified codebook is designed for all the segments of latent vector

- Alt-2: A dedicated codebook is designed for each segment of latent vector

Proposal 6: Regarding the alignment of payload parameters (e.g.,dl,ν, L,Ql,ν), payload parameter can be configured by Network in CSI report configuration or reported by UE in CSI part 1.

Proposal 7: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered:

- Alt-1: Layer-based priority rule

- Alt-2: Segment based priority rule for vector quantization

- Alt-3: Scalar of latent space based priority rule for scalar quantization.

Proposal 8: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

Proposal 9: Regarding the content of CSI report for NW side performance monitoring, the following options can be considered:

- Option1: Paired Target CSI and CSI feedback

- Option2: Target CSI and time information of paired CSI feedback

- 4 Reference

[1]RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm