3GPP TSG RAN WG1 #124			R1-2601167

Gothenburg, SE, Feb 9th – 13th, 2026

Source:	NTT DOCOMO, INC.

Title:	Study on R2D design and procedures for Ambient IoT outdoor for active device

Agenda Item:	9.3.2.1

Document for: Discussion and decision

# Introduction

At the RAN#109 meeting, a revised SID “enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” was approved [1]. The objectives are as follows.

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

In this contribution, necessary and feasible changes to the Rel-19 air interface to support A-IoT in the outdoor scenarios are discussed.

- Discussion

  - General aspects on physical layer

    - Channel coding and repetition

At the previous meetings, it was agreed to study FEC and repetition for R2D and extensively studied. The remaining discussion point is the necessity on FEC and repetition.

In our assessment of the necessity for FEC and repetition in R2D scenarios, two key considerations should be addressed: the overall need for coverage enhancement, and, where applicable, the selection of appropriate coverage enhancement techniques.

Since D2R is typically identified as the coverage bottleneck, we generally consider coverage enhancement for R2D to be of lower priority. However, link budget analyses indicate that R2D may not always meet the target distance requirements of Release 20 (e.g., 500m), particularly when a penetration margin of 20dB is applied. In such cases, implementing coverage enhancement techniques for R2D may also become necessary if the scenario/use case should be addressed.

If coverage enhancement is required, the selection of appropriate techniques warrants further discussion. Besides FEC and repetition, the adoption of M=1 may also be considered. The effectiveness of M=1 has been discussed in previous meetings, and it has been noted that its impact on coverage is not consistently superior to M=2. Specifically, M=1 provides advantages over M=2 primarily when power pooling across symbols and power boosting during the ON duration are allowed, assuming that the entire OFDM symbol constitutes an OFF duration for M=1. Otherwise, M=1 does not offer performance improvements relative to M=2. Consequently, the introduction of M=1 is not regarded as a promising approach for enhancing coverage.

In comparing repetition and Forward Error Correction (FEC), FEC offers additional coding gain and generally outperforms repetition at equivalent data rates, making it the preferred method. Additionally, although repetition may be less complex than FEC, studies in the previous meetings indicate that FEC is feasible for both device 2b/C, with computational complexity and power consumption dependent on the selected coding scheme (such as TBCC or RM).

Proposal 1:

For R2D, coverage enhancement can be deprioritized.

- Introduction of M=1 should be deprioritized.

- If necessary, FEC should be prioritized compared to repetition.

  - R2D signal design

    - R2D signal for start indication

It was agreed that it is feasible to use binary sequence-based SIP (i.e., m-sequence or Golay sequence) for timing indication of the start of R2D transmission and for timing synchronization/tracking. Observations on sequence type, number of sequences and sequence length have been captured in the agreement.

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, - At least, it is feasible to use binary sequence-based SIP (i.e., m-sequence or Golay sequence), and the binary sequence-based SIP can be used for timing synchronization/tracking.

One remaining issue we would like to note that is the distinguishability of SIP from other R2D. In Rel-19, SIP pattern is distinguishable from PRDCH so that devices will not misunderstand the chips in PRDCH data payload as an SIP. The same issue needs to be considered in Rel-20 SIP design. When the sequence length of SIP is long, the probability that a sequence of 0 and 1s same as SIP exists in PRDCH data payload can be negligible. While at least when the sequence of SIP is short, distinguishability from other R2D should be considered.

Proposal 2:

The sequence of SIP should be distinguishable from other R2D in case the sequence length is short.

    - R2D signal for chip duration determination for L1 control

At the RAN1#122bis meeting, it was agreed to study the following options for the functionality of chip duration determination of the L1 R2D control information and data payload in the PRDCH. Observations have been made on the agreed options at the RAN1#123 meeting.

Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration

For chip duration determination of L1 R2D control, we further make the following observations. Options 3 and 4 have an advantage as flexible chip duration of L1 R2D control. Meanwhile, L1 R2D control can be used to schedule broadcast/multi-cast PRDCH, it schedules unicast PRDCH at least. Considering such unicast L1 R2D control, it is beneficial to support flexibility on data rate, e.g., high or low data rate can be used for L1 R2D control depending on channel condition of a device.

For Alt.4-1 (using binary sequences for SIP), device needs to blind detect multiple sequences for SIP detection, which leads to higher device complexity.

Observation 1:

For chip duration determination of L1 R2D control,

- options 3 and 4 have an advantage as flexible chip duration of L1 R2D control.

- alt.4-1 has a disadvantage that device needs to blind detect multiple sequences for SIP detection, which leads to higher device complexity.

    - R2D signal for frequency acquisition/synchronization and timing synchronization/tracking

At the RAN1#123 meeting, it was agreed that the R2D periodic synchronization signal is necessary at least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization. It was also agreed that the R2D sync signal is a binary sequence, and the number of sequences is less than or equal to 4, where if more than 1 sequence is supported, for potential reader identification/differentiation. In addition, it was agreed at least periodic synchronization signal is necessary and whether synchronization signal is strictly periodic at reader can be further studied.

Agreement R2D periodic synchronization signal is a binary sequence in time domain - The binary sequence reuses Rel-19 R2D waveform - The binary sequence has a pre-defined chip length - Device assumes that the binary sequence has a predefined length - The number of binary sequence(s) is less than or equal to 4 - If more than 1 sequence is supported, it is for potential reader identification/differentiation - Study whether Manchester coding is applied to the binary sequence - Study necessity of other aspects, including overlaid sequence Agreement At least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization, the periodic synchronization signal transmission is necessary. - Default transmission periodicity is predefined - FFS (in SI or WI): whether synchronization signal transmission is strictly periodic at reader

Regarding whether synchronization signal transmission is strictly periodic at reader, as also discussed in our companion paper [2], for initial access, R2D synchronization signal for frequency acquisition (FAS) should be received by devices at first to identify the frequency of R2D Rx among multiple candidate frequencies as shown in figure 1.  As discussed in section 2.3, given the limited device availability duration for device 2b/C with high peak power consumption, it is not feasible to keep monitoring long duration until device can receive broadcast information. Periodic transmission of synchronization signal and the corresponding broadcast information reliably ensures the transmission of at least one broadcast message within the specified period and no significant drawbacks have been identified. More detailed discussion can be found in section 2.3. Thus, for frequency acquisition, periodic transmission of synchronization signal should be considered as baseline.

Another aspect needs to be studied is whether additional periodic synchronization without corresponding broadcast information is needed. For initial frequency acquisition, the periodic synchronization signal should be transmitted with corresponding broadcast information. In addition to frequency acquisition, periodic synchronization signal can be also useful for time synchronization/tracking. It can be considered that D2R resources are periodically allocated by single R2D, such as first D2R transmission of DO-A, etc. For such periodically  allocated D2R transmission, accumulated time error due to SFO should be considered especially for device 2b with post-sync SFO up to 1000ppm. The accumulated time error over time T can be SFO*T, where T is the time gap between the last synchronization signal and D2R Tx. As discussed in section 2.3, the periodicity of broadcast information can be up to few hundreds of ms. If periodicity of synchronization signal is same as broadcast information, the accumulated time error can be up to few hundreds of us assuming 1000ppm SFO. Thus, for timing synchronization/time tracking for periodically allocated D2R transmission, additional synchronization signal without corresponding broadcast information, e.g., as TAS in figure 1, can be beneficial to reduce accumulative time error due to SFO for D2R transmission while avoiding additional overhead of broadcast information.

Figure 1: Periodic R2D synchronization signal for frequency acquisition

Proposal 3:

For R2D synchronization signal for frequency acquisition/synchronization and timing synchronization/tracking, periodic transmission should be considered as baseline.

- For frequency acquisition/synchronization, the periodic synchronization signal is transmitted with corresponding broadcast information.

Observation 2:

For time synchronization/tracking, additional synchronization signal without corresponding broadcast information can be beneficial for reducing accumulated time error due to SFO while avoiding additional overhead of broadcast information.

For sequence type of R2D synchronization signal for frequency synchronization, sequences with good auto-correlation property, e.g., m-sequency or Golay sequence can be considered.

For the number of sequences, device complexity on correlation-based detection should be considered given that devices need to detect the sequence among multiple candidate frequencies. If single sequence is used, information for reader identification/differentiation can be carried in broadcast information.

Proposal 4:

For sequence type of R2D synchronization signal for frequency synchronization, m sequence or Golay sequence with good auto-correlation property should be considered.

Observation 3:

For the number of sequences of R2D synchronization signal for frequency synchronization, device complexity on correlation-based detection should be considered assuming that device performs correlation-based detection to detect the signal on multiple candidate frequencies at least for initial frequency acquisition.

    - R2D signal for CFO/LO calibration

At the RAN1#122bis meeting, it was agreed to study the following options for the time location(s) of CFO calibration signal. At the RAN1#123 meeting, observations on whether the CFO calibration signal is transmitted periodically or aperiodically were made.

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

For time location of CFO calibration signal, since different options can be applied for CFO calibration of different R2D/D2R Tx, more detailed assumption of “in relation to” for each option should be further clarified.  For example, for option 1, whether the CFO calibration signal is assumed to be before or after L1 R2D control information should be clarified. From our perspective:

- In option 1, CFO calibration signal is before L1 R2D control information. Option l can be applied for CFO calibration to receive L1 R2D control (if needed), to receive PRDCH (if needed), to transmit aperiodic D2R triggered by the R2D, and to transmit first X occasions of periodic D2R triggered by the R2D where X depending on periodicity and CFO drift.

- In option 2, CFO calibration signal is after data payload of PRDCH. Option 2 can be applied for CFO calibration to transmit aperiodic D2R Tx triggered by the R2D, and to transmit first X occasions of periodic D2R triggered by the R2D where X depending on periodicity and CFO drift.

- In option 3, CFO calibration signal is before D2R Tx. Option 3 can be applied for CFO calibration to transmit D2R .

- In option 4, CFO calibration signal is after synchronization signal. Option 4 can be applied for CFO calibration to receive broadcast information (if needed), to transmit D2R.

- In option 5, there can be two sub-alternatives:

  - Option 5-1: CFO calibration signal is before broadcast information. Option 5-1 can be applied for CFO calibration to receive broadcast information (if needed), to transmit D2R.

  - Option 5-2: CFO calibration signal is after broadcast information. Option 5-2 can be applied for CFO calibration to transmit D2R.

Regarding whether the CFO calibration signal is periodic or aperiodic, we further made following observation; assuming that if CFO calibration signal is transmitted periodically with hundreds of ms periodicity, aperiodic CFO calibration signal can further minimize CFO due to CFO drift. However, considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, CFO calibration via periodic CFO calibration signal can be sufficient. Thus, for the above options, option 1/2/3 can minimize the CFO due to CFO drift, while option 4/5 can be sufficient considering low CFO drift and hundreds of ms periodicity.

Proposal 5:

For time location of CFO calibration signal, clarify the options agreed in RAN1#122bis meeting as following:

- Option 1, CFO calibration signal is before L1 R2D control information.

- Option 2, CFO calibration signal is after data payload of PRDCH.

- Option 3, CFO calibration signal is before D2R Tx.

- Option 4, CFO calibration signal is after synchronization signal.

- Option 5, there can be two sub-alternatives.

  - Option 5-1: CFO calibration signal is before broadcast information.

  - Option 5-2: CFO calibration signal is after broadcast information.

Observation 4:

For time location of CFO calibration signal,

- Option l can be applied for CFO calibration for L1 R2D control (if needed), PRDCH (if needed), aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

- Option 2 can be applied for CFO calibration for aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

- Option 3 can be applied for CFO calibration for aperiodic D2R Tx and periodic D2R Tx.

- Option 4 can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

- Option 5-1 can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

- Option 5-2 can be applied for aperiodic D2R Tx and periodic D2R Tx.

Observation 5:

For time location of CFO calibration signal,

- Aperiodic CFO calibration signal can minimize CFO due to CFO drift, while considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient.

- Option 1/2/3 can minimize CFO due to CFO drift. Option 4/5 can be sufficient considering low CFO drift and hundreds of ms periodicity

  - Transmission scheme for broadcast information

At the previous meetings, potential introduction on broadcast information has been discussed. While 9.3.2.3 focuses on the procedure to acquire broadcast information (e.g., single-step or two-step) and corresponding contents, the transmission scheme of broadcast information would be discussed 9.3.2.1.

RAN1#122bis Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal. RAN1#123 Agreement - The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception - On how A-IoT devices acquire the broadcast information, study the following two cases: - Case B1: Single-step acquisition of the broadcast information - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information) - On whether to use PRDCH to carry the broadcast information, - For Case B1, PRDCH is used to carry the broadcast information. - For Case B2 - PRDCH is used to carry the SIB1-like broadcast information. - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information Agreement Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C. - FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader - FFS (in SI or WI): periodicity of the transmission of broadcast information

Periodic or aperiodic

Given the limited device availability duration for device 2b/C with high peak power consumption, it is not feasible to monitor long duration until device can receive broadcast information. It is therefore necessary to ensure that at least one broadcast information and its corresponding synchronization signal are received within a specified duration to facilitate efficient initial access procedures. Accordingly, the necessity of periodic broadcast transmission has been acknowledged. On top of this, it can be further studied whether this transmission should be strictly periodic from the reader's perspective, as well as to determining the appropriate periodicity.

Regarding whether the transmission of broadcast information should be strictly periodic at the reader, both periodic and aperiodic approaches can work assuming that at least one broadcast message is transmitted within the specific duration from the device perspective. The following advantages and disadvantages have been identified:

- Periodic Transmission: This approach reliably ensures the transmission of at least one broadcast message within the specified period. No significant drawbacks have been identified.

- Aperiodic Transmission: This approach offers operational flexibility for the reader, e.g., to avoid potential collisions with other R2D transmission occasions. However, assuming that flexible timing offsets are introduced in Rel-20, readers can readily address R2D collisions, such flexibility would be less critical. Additionally, this approach may introduce additional workload, e.g., to discuss time tolerances for broadcast information transmissions.

In summary, periodic transmission should be considered as the baseline assumption.

Observation 6:

For broadcast information acquisition, both periodic and aperiodic transmission at reader can work. Aperiodic transmission can provide flexibility for reader while it may lead to additional workload, e.g., on how much time tolerance can be allowed for broadcast information transmission.

Proposal 6:

For broadcast information, periodic transmission should be considered as baseline.

For the periodicity of broadcast information, device should be able to sustain at least one periodicity to search broadcast information and corresponding synchronization signal, and the subsequent procedure such as PRDCH decoding. Considering device sustainable operation time for device 2b/C, the feasible periodicity can be up to tens of ms or a few hundreds of ms, depending on the assumption on energy storage. Therefore, the periodicity cannot be too long as beyond feasible device sustainable operation time. On the other hand, excessively short periodicity is not preferable from NW overhead and NW energy consumption perspective. Balancing these considerations, an appropriate broadcast periodicity typically falls within several hundred milliseconds.

Proposal 7:

For the study of broadcast information,

- periodic transmission should be considered.

- NW overhead/energy saving and feasible device sustainable operation time should be considered for the periodicity.

Observation 7:

For the periodicity of broadcast information, up to a few hundreds of ms can be feasible depending on energy storage assumption.

Whether to use PRDCH/Tx parameters of PRDCH

For the physical channel which carries broadcast information, PRDCH should be the baseline while how much flexibility is required can be further discussed in our view. More specifically, L1 R2D control information has been studied to indicate parameters for PRDCH such as M value, TBS, etc., and it can be further studied whether such L1 R2D control is necessary even for PRDCH which carries broadcast information or the parameter(s) can be fixed in spec. For example, L1 R2D control preceding PRDCH which carries broadcast information may not be necessary to indicate TBS if payload size of broadcast information is fixed. Otherwise, if payload size of broadcast information is variable, L1 R2D control still can be necessary. In addition, even if L1 R2D control is necessary, it can be further discussed whether full flexibility of indication is necessary or not. For example, TBS can be up to 1000bits for PRDCH in general but such large payload size may not be applicable for broadcast information. Assuming such case, applicable value set for PRDCH which carries broadcast information can be restricted to reduce required bit fields in L1 R2D control. In our view, it is too restrictive to fix all the parameters of PRDCH for broadcast information and similar approach as other PRDCH, i.e., indication by L1 R2D control can be considered with limited flexibility.

Proposal 8:

PRDCH should be the baseline to carry the broadcast information.

For each parameter of the PRDCH such as M value, TBS, etc., how much flexibility should be ensured can be further studied. Opt.1) fixed value is applied, Opt.2) indicated by R2D L1 control with/without restriction on applicable values.

Time and frequency resources for transmission of the broadcast information

Assuming the initial access procedure discussed in 9.3.2.3, the frequency resource is determined based on the R2D signal which is used for frequency acquisition. More specifically, frequency resource of PRDCH which carries broadcast information is the same as the R2D signal for frequency acquisition and CFO calibration signal if any. For time resource, the start of broadcast information is determined in relation to R2D signal for frequency acquisition, broadcast information can be immediately after the R2D signal or with an L1 R2D control in between.

Proposal 9:

For the study of broadcast information, time and frequency resources for transmission of the broadcast information is determined in relation to R2D synchronization signal for frequency acquisition.

- Frequency resource of the broadcast information is the same as the R2D signal.

- Time resource of the broadcast information can be immediately after the R2D signal or with an L1 R2D control in between.

Figure 2: Time location of broadcast information in relation to R2D signal.

- Conclusion

In this contribution, we discussed necessary and feasible changes to the Rel-19 air interface to support A-IoT in the outdoor scenarios. Based on the discission, following observations and proposals are made.

General aspects on physical layer

Proposal 1:

For R2D, coverage enhancement can be deprioritized.

- Introduction of M=1 should be deprioritized.

- If necessary, FEC should be prioritized compared to repetition.

R2D signal design

Proposal 2:

The sequence of SIP should be distinguishable from other R2D in case the sequence length is short.

Observation 1:

For chip duration determination of L1 R2D control,

- options 3 and 4 have an advantage as flexible chip duration of L1 R2D control.

- alt.4-1 has a disadvantage that device needs to blind detect multiple sequences for SIP detection, which leads to higher device complexity.

Proposal 3:

For R2D synchronization signal for frequency acquisition/synchronization and timing synchronization/tracking, periodic transmission should be considered as baseline.

- For frequency acquisition/synchronization, the periodic synchronization signal is transmitted with corresponding broadcast information.

Observation 2:

For time synchronization/tracking, additional synchronization signal without corresponding broadcast information can be beneficial for reducing accumulated time error due to SFO while avoiding additional overhead of broadcast information.

Proposal 4:

For sequence type of R2D synchronization signal for frequency synchronization, m sequence or Golay sequence with good auto-correlation property should be considered.

Observation 3:

For the number of sequences of R2D synchronization signal for frequency synchronization, device complexity on correlation-based detection should be considered assuming that device performs correlation-based detection to detect the signal on multiple candidate frequencies at least for initial frequency acquisition.

Proposal 5:

For time location of CFO calibration signal, clarify the options agreed in RAN1#122bis meeting as following:

- Option 1, CFO calibration signal is before L1 R2D control information.

- Option 2, CFO calibration signal is after data payload of PRDCH.

- Option 3, CFO calibration signal is before D2R Tx.

- Option 4, CFO calibration signal is after synchronization signal.

- Option 5, there can be two sub-alternatives.

  - Option 5-1: CFO calibration signal is before broadcast information.

  - Option 5-2: CFO calibration signal is after broadcast information.

Observation 4:

For time location of CFO calibration signal,

- Option l can be applied for CFO calibration for L1 R2D control (if needed), PRDCH (if needed), aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

- Option 2 can be applied for CFO calibration for aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

- Option 3 can be applied for CFO calibration for aperiodic D2R Tx and periodic D2R Tx.

- Option 4 can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

- Option 5-1 can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

- Option 5-2 can be applied for aperiodic D2R Tx and periodic D2R Tx.

Observation 5:

For time location of CFO calibration signal,

- Aperiodic CFO calibration signal can minimize CFO due to CFO drift, while considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient.

- Option 1/2/3 can minimize CFO due to CFO drift. Option 4/5 can be sufficient considering low CFO drift and hundreds of ms periodicity

Transmission scheme of broadcast information

Observation 6:

For broadcast information acquisition, both periodic and aperiodic transmission at reader can work. Aperiodic transmission can provide flexibility for reader while it may lead to additional workload, e.g., on how much time tolerance can be allowed for broadcast information transmission.

Proposal 6:

For broadcast information, periodic transmission should be considered as baseline.

Proposal 7:

For the study of broadcast information,

- periodic transmission should be considered.

- NW overhead/energy saving and feasible device sustainable operation time should be considered for the periodicity.

Observation 7:

For the periodicity of broadcast information, up to a few hundreds of ms can be feasible depending on energy storage assumption.

Proposal 8:

PRDCH should be the baseline to carry the broadcast information.

For each parameter of the PRDCH such as M value, TBS, etc., how much flexibility should be ensured can be further studied. Opt.1) fixed value is applied, Opt.2) indicated by R2D L1 control with/without restriction on applicable values.

Proposal 9:

For the study of broadcast information, time and frequency resources for transmission of the broadcast information is determined in relation to R2D synchronization signal for frequency acquisition.

- Frequency resource of the broadcast information is the same as the R2D signal.

- Time resource of the broadcast information can be immediately after the R2D signal or with an L1 R2D control in between.

# Reference

[1] RP-252964, “Revised SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, Jun. 2025.

[2] R1-2601169, “Study on other procedures for Ambient IoT outdoor for active device”, NTT DOCOMO, INC., Feb. 2026.