3GPP TSG RAN WG1 #124	R1-2601331

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	10.5.2.3

Source:	KDDI

Title:	Discussion on uplink transmission scheme(s) for uplink channels

Document for:	Discussion and decision

Introduction

At the RAN Working Group 1 #123 meeting, discussions on AI/ML-based DMRS enhancement for the 6G air interface were held under agenda item 11.6, and the following agreements were reached [1].

Agreement From RAN 1 perspective, the following use cases can be matched to the identified primary agendas of RAN1 Use cases; Primary agendas. Low overhead DMRS with AI/ML receiver; UL & DL DMRS associated with PUSCH/PDSCH Note: Sub-Case C: DMRS-free may be related to modulation.

In this contribution, we provide our views on the design of the 6G air interface, specifically focusing on DMRS design for 6G PUSCH, focusing on port extension requirements and overhead reduction methods to support massive MIMO systems.

Discussions

DMRS Design for 6G PUSCH

Increase in the number of DMRS ports for 6G Massive MIMO

In 5G NR, DMRS supports up to 24 orthogonal ports with Enhanced Type 2 configuration and double symbol length, as specified in TS 38.211. The DMRS configuration types include Type 1 with comb-2 structure supporting up to 8 ports, and Type 2 with FD-CDM2 structure supporting up to 12 ports. Enhanced Types introduced in Rel-18 extend these limits to 16 and 24 ports, respectively.

For 6G massive MIMO systems, the number of antenna elements is expected to scale up to 128, 256 or more, significantly exceeding current 5G deployments. To fully utilize the spatial multiplexing capability of such large-scale antenna arrays and support increased MU-MIMO layers, the number of orthogonal DMRS ports needs to be increased beyond the current maximum of 24.

Proposal 1: Study increasing the number of orthogonal DMRS ports beyond 24 for 6G massive MIMO.

When considering methods to increase the number of orthogonal DMRS ports, there is an inherent trade-off between the number of ports and overhead. Conventional approaches to increase the number of ports include adding CDM groups in the frequency domain or increasing the number of symbols used in the time domain. Both approaches consume additional resources that could otherwise be used for data transmission.

In particular, increasing the number of symbols used in DMRS in the time domain from 2 to 3 or 4 directly reduces the number of available data symbols per slot. For example, with 14 symbols per slot, double symbol DMRS results in approximately 14.3% overhead (2/14), while triple symbol would increase this to 21.4% (3/14). This increase in overhead could significantly offset the capacity gains achieved through increased spatial multiplexing via additional DMRS ports.

Therefore, simply increasing DMRS resources in the time domain may not be an efficient approach, as the loss in data capacity can limit the overall channel capacity improvement.

Proposal 2: Study overhead-efficient approaches when increasing the number of DMRS ports.

AI-based DMRS Overhead Reduction

In RAN1, AI/ML-based approaches are being actively discussed to enable increased numbers of ports while addressing overhead challenges. According to the recent discussions in R1-2508811[2], three main sub-cases have been considered for AI-based DMRS overhead reduction:

Observation 2.2 For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver. - [25 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver. - [12 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver. - [9 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver. - Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C. Note: whether/how to capture the observation in the TR is a separate discussion

Sub-case A is sparse orthogonal DMRS. This approach uses an AI receiver to interpolate and estimate the channel from sparse reference signals while reducing DMRS density in the time domain and/or frequency domain. Sub-case A offers advantages in terms of impact on specifications, primarily requiring the definition of new sparse DMRS patterns while maintaining orthogonality with existing DMRS structures. This facilitates coexistence with legacy UEs, particularly in the MRSS band, and minimizes impact on MU-MIMO pairing.

Sub-case B is superimposed pilot. This approach overlays DMRS and data on the same resources, potentially eliminating dedicated DMRS overhead. However, it introduces challenges in signal separation and has more significant specification impact.

Sub-case C is DMRS-free transmission. This approach fundamentally changes demodulation procedures, which may require significant specification changes.

Given the favorable characteristics of Sub-case A in terms of specification impact and legacy coexistence, prioritizing sparse DMRS study is a reasonable approach to enable overhead reduction while supporting port extension for 6G massive MIMO. To increase the priority of other sub-cases in the study, simulations must demonstrate gains significantly exceeding those of Sub-case A.

Proposal 3: Prioritize sparse DMRS study for AI-based overhead reduction.

Conclusion

In this contribution, we discussed DMRS design considerations for 6G PUSCH, focusing on port extension requirements and overhead reduction methods.

Proposal 1: Study increasing the number of orthogonal DMRS ports beyond 24 for 6G massive MIMO.

Proposal 2: Study overhead-efficient approaches when increasing the number of DMRS ports.

Proposal 3: Prioritize sparse DMRS study for AI-based overhead reduction.

- References

- 3GPP TSG RAN WG1 #123 chairman’s notes.

- R1-2508811, Moderator summary #4 on AI/ML for 6GR, Samsung (Moderator).