3GPP TSG RAN WG1 #124							    R1-2601541

Gothenburg, Sweden, February 09 – 13, 2026

Source:	Moderator (OPPO)

Title:	FL summary #2 for AI 10.5.4.3: 6GR HARQ related aspects

Agenda item:	10.5.4.3

Document for:	Discussion and Decision

Introduction

During the RAN#110 meeting, the latest version of SID for RAN WG level study on 6G radio was revised/updated in RP-253876 [1]. According to the SID objective section, parts that are directly / indirectly relevant to this agenda item are provided in the following for convenience.

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. … From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. … The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - … - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - … - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - …

This contribution provides a summary of submitted contributions, discussion topics and outcomes that are related to the HARQ operation in 6GR (blue text part of objective 2) during this RAN1 meeting. Note that, all past outcomes including agreements, conclusions and working assumptions reached during this SI are captured in Section 7 (Appendix) of this document.

Collection of agreements / outcomes in RAN1#124

## Tuesday (10/Feb)

To be collected during the meeting.

Proposals for online handling

## Tuesday (10/Feb)

Proposal 1-1 (II): In 6GR, DL and UL HARQ operation designs should consider the following aspects:

  - latency

  - reliability

  - coverage

  - power saving (NW and UE)

  - NW and UE complexity (e.g., processing, scheduling, multiplexing, memory)

  - diverse services/applications/traffics

  - system/HARQ retransmission efficiency (including link adaptation)

  - feedback efficiency/overhead

  - FFS: device type

Note: the design of DL and UL HARQ does not necessarily be the same

Proposal 1-2 (II): For DL HARQ in 6GR, study both following reporting mechanisms until RAN1#124bis.

- Mechanism 1: HARQ-ACK information bits are transmitted via L1 signalling (e.g., in PUCCH/PUSCH)

- Mechanism 2: HARQ-ACK information bits are transmitted via higher layer signalling (e.g., MAC CE)

Proposal 1-3 (II): For DL and UL data transmission, study how to support asynchronous and adaptive HARQ operation.

- Option 1: HARQ feedback timing/payload is linked to the PDCCH/PDSCH reception timeline

- Option 2: HARQ feedback timing/payload is not linked to the PDCCH/PDSCH reception timeline

Proposal 1-5 (I): Study HARQ-ACK payload size range in 6GR, considering the following aspects:

- multi-carrier operation

- number of slots (e.g., based on DCI triggering)

- frame structure

- number of HARQ processes

- CBG or TB based HARQ-ACK feedback

## Wednesday (11/Feb)

Proposal [for conclusion] 1-1 (II): In 6GR, DL and UL HARQ operation designs should consider at least the following aspects:

  - latency

  - reliability

  - coverage

  - power saving (NW and UE)

  - NW complexity

  - UE complexity

  - diverse services/applications/traffics

  - system/HARQ retransmission efficiency/system throughput (including link adaptation)

  - feedback efficiency/overhead (including impact on DL control resources)

  - Interaction/joint-optimization with other adaptation mechanisms (e.g., higher layer repetition, ARQ)

  - FFS: device type

Note: the design of DL and UL HARQ does not necessarily be the same

Proposal 1-2 (II): For DL HARQ in 6GR, study both following reporting mechanisms until RAN1#124bis.

- Mechanism 1: HARQ-ACK information bits are transmitted on PUCCH/PUSCH via L1 signalling

  - HARQ-ACK information bits on PUCCH/PUSCH via L1

  - HARQ-ACK information bits on PUSCH via L1

- Mechanism 2: HARQ-ACK information bits are transmitted on PUSCH via higher layer signalling (e.g., MAC CE)

Proposal 1-3 (II): For DL and UL in 6GR, support asynchronous and adaptive HARQ operation

- Asynchronous HARQ refers to retransmission can only be occurred in non-deterministic occasion once the corresponding initial transmission is scheduled.

- For adaptive HARQ, it refers that the transmission parameters and resources for the retransmission can be adaptively adjusted.

Proposal 4-0 (I): In 6GR, at least support TB level granularity for HARQ-ACK feedback

- FFS: HARQ-ACK feedback granularity that is other than the TB level granularity

Proposal 4-1 (II): Study whether/how to support at least the following for HARQ-ACK feedback / HARQ operation

- HARQ-ACK codebook/payload details

- HARQ-ACK disabling/enabling

- NACK-only / ACK-only feedback / trigger

- ACK/NACK for PDCCH (e.g., for DTX, DCI without scheduling PDSCH)

- Cross-layer HARQ and ARQ integration/interaction (including DL and UL aspects)

- Cross-cell/CC HARQ retransmission

- Assistance information (e.g., soft value, buffer headroom)

Proposals for offline discussion

## Tuesday (10/Feb)

Proposal [for conclusion] 1-1 (II): In 6GR, DL and UL HARQ operation designs should consider at least the following aspects:

  - latency

  - reliability

  - coverage

  - power saving (NW and UE)

  - NW complexity(e.g., inter-carrier interaction, processing, scheduling, multiplexing)

  - UE complexity (e.g., inter-carrier interaction, processing, scheduling, multiplexing, memory)

  - diverse services/applications/traffics

  - system/HARQ retransmission efficiency/system throughput (including link adaptation)

  - feedback efficiency/overhead (including impact on DL control resources)

  - Interaction/joint-optimization with other adaptation mechanisms (e.g., higher layer repetition, ARQ)

  - FFS: device type

Note: the design of DL and UL HARQ does not necessarily be the same

## Wednesday (10/Feb)

Proposal 1-2 (II): For DL HARQ in 6GR, study both following reporting mechanisms until RAN1#124bis.

- Mechanism 1: HARQ-ACK information bits are transmitted on PUCCH/PUSCH via L1 signalling

  - HARQ-ACK information bits on PUCCH/PUSCH via L1

  - HARQ-ACK information bits on PUSCH via L1

- Mechanism 2: HARQ-ACK information bits are transmitted on PUSCH via higher layer signalling (e.g., MAC CE)

Proposal 1-3 (II): For DL and UL in 6GR, support asynchronous and adaptive HARQ operation

- Asynchronous HARQ refers to retransmission can only be occurred in non-deterministic occasion once the corresponding initial transmission is scheduled.

- For adaptive HARQ, it refers that the transmission parameters and resources for the retransmission can be adaptively adjusted.

Proposal 4-0 (I): In 6GR, at least support TB level granularity for HARQ-ACK feedback

- FFS: HARQ-ACK feedback granularity that is other than the TB level granularity

Proposal 4-1 (II): Study whether/how to support at least the following for HARQ-ACK feedback / HARQ operation

- HARQ-ACK codebook/payload details

- HARQ-ACK disabling/enabling

- NACK-only / ACK-only feedback / trigger

- ACK/NACK for PDCCH (e.g., for DTX, DCI without scheduling PDSCH)

- Cross-layer HARQ and ARQ integration/interaction (including DL and UL aspects)

- Cross-cell/CC HARQ retransmission

- Assistance information (e.g., soft value, buffer headroom)

Proposal 1-5 (I): Study HARQ-ACK payload size range in 6GR (i.e., min and max values), considering the following aspects:

Proposal 2-2 (II): Study a single UE processing time [capability] for PDSCH and PUSCH ,or multiple UE processing times for PDSCH and PUSCH, and impacting factors.

- Note, processing time for PDSCH including PDSCH processing and HARQ-ACK feedback preparation time

Topics for discussion

## [ACTIVE] Topic #1: High-level aspects of HARQ operation

Background: Traditionally, Hybrid Automatic Repeat Request (HARQ) is an effective error-control mechanism that combines Forward Error Correction (FEC) with Automatic Repeat Request (ARQ) to achieve high-speed and reliable data transmission. Based on early L1 indication of erroneous data TB after CRC checksum from the receiver in LTE and NR, the transmitter performs a retransmission of the same data TB with the same or different RV (or portion(s) of the data TB) which allows the receiver to “correct” the erroneous data TB via the FEC coding. If uncorrectable, the receiver requests a retransmission again. Therefore, the essence of HARQ operation consists of fast indication of HARQ information and retransmission(s) of data information with the same or different RV allowing the receiver to perform combining of soft-bits (Chase combining or incremental redundancy combining) with the previously received erroneous bits to recover the original data TB.

HARQ operation in LTE and NR downlink, DL HARQ-ACK information is generated as L1 UCI and transmitted in PUCCH/PUSCH by a UE for single or multiple scheduled/received TB(s) in unicast/multi-cast PDSCH. The scheduled/received TB(s) could be in a single DL subframe/slot or over multiple DL slots, for a single serving cell only or across multiple serving cells in the case of carrier aggregation. In terms of DL HARQ feedback and retransmission timings, a synchronous operation was adopted in LTE, where there was a strict / fixed timing relationship of 4ms (4 subframes) for the DL HARQ feedback from the UE (after a PDSCH reception) and for a TB retransmission from the eNB (after a DL HARQ feedback). In NR, on the other hand, an asynchronous DL HARQ and adaptive retransmission operation are adopted, where the HARQ feedback timing and PUCCH resources for the feedback are dynamically indicated to the UE via DCI, and the gNB adaptively chooses the timing for TB retransmission by utilizing the “HARQ process number” field in DCI.

HARQ operation in LTE and NR uplink, a dedicated PHY channel (PHICH) was specified and used by the BS (eNB) to explicitly transmit UL HARQ-ACK information corresponding to a PUSCH transmission from the UE. In NR, a dedicated PHY channel was not specified and used for this purpose any longer. Instead, the BS (gNB) uses the NDI and HARQ process number fields in DCI formats for UL grant to implicitly signal UL HARQ information to the UE by explicit indicating whether an erroneous received UL TB associated to a HARQ process needs to be retransmitted.

HARQ-less operation in NR NTN: Due to extremely long propagation delays caused by satellite distances from the UE, HARQ feedback/retransmission can be disabled per HARQ process in NR NTN.

### Contribution summary

- Lessons learnt / shortcomings of NR HARQ operation and scheduling timeline: [8] [11] [19] [22] [24] [25] [29]

  - Overly complex timing relationships and inconsistent processing deadlines, and non-deterministic control interactions between the UE and the gNB

  - Unduly complex and multidimensional prioritization/multiplexing rules based on the payload (SR, HARQ-ACK, CSI, etc.) and PHY channels.

  - UCI dropping during UCI multiplexing

  - Diverse HARQ-ACK codebook types, and only limited type(s) is used in practice

  - Tight coordination across scheduled cells is required for DAI updating

  - No differentiation between NACK and DTX for PDSCH transmission

  - Reliability issues in HARQ operation observed in the field.

  - Common issues caused by DCI misdetection

  - Limitations of CBG-based retransmissions

  - lack of coordination between HARQ and higher‑layer ARQ mechanisms

  - High UE implementation complexity, including both large soft‑buffer memory requirements and the associated high‑bandwidth memory‑access demands.

- Key HARQ design principles/requirements for 6GR [2] [3] [4] [5] [11] [12] [16] [22] [23] [24] [25] [26] [29]

  - Efficient and reliable DL HARQ operation for low error probabilities, feedback latency and signaling overhead

  - Maintain a simple structure with more complex configurations

  - HARQ feedback mechanisms to facilitate parallel processing of different CA serving cells without requiring low-latency inter-carrier dependencies of schedulers and DCI.

  - Trade-off between flexibility and complexity of asynchronous, adaptive HARQ retransmission

  - Trade-off between number of HARQ processes and the cost and power consumption of UE

  - Trade-off between signaling overhead and fine-grained retransmission

  - More efficient HARQ entity management to support more flexible and efficient transmission

  - Cross-layer integration design for optimizing the efficiency and latency of retransmission;

  - Designs accommodating different traffic and service requirements, e.g., TN, NTN and AI

  - Simple UCI multiplexing rule and timeline.

  - Simplifying gNB implementation for inter-cell coordination.

  - Relax and minimize the need for scheduler interaction across cells

  - Preserve proven operational properties of HARQ‑ACK transmission with low‑latency feedback, minimal HARQ RTT, low‑overhead payloads, and avoidance of uplink‑control coverage limitations

  - Maximize parallelism among uplink carriers.

- Support of asynchronous HARQ operation with flexible (re)transmission scheduling/indication: [2] [4] [5] [23] [24] [26] [27]

  - UL-SCH / PUSCH operation can rely on asynchronous adaptive HARQ operation. Each of the PUSCH re-transmissions is scheduled by an UL grant. The timing of a PUSCH re-transmission is determined by the timing of the DCI scheduling the asynchronous PUSCH re-transmission and the indicated time offset.

  - Asynchronous DL HARQ operation allows for more flexible TDD/UL DL configurations supported, different minimum UE PDSCH processing times as well as increased BS implementation & scheduling flexibility.

  - The flexibility of re-transmissions being explicitly scheduled and not tied to a fixed timing is beneficial as it allows for re-transmissions to be sent as soon as possible, thus reducing the delay of the data transmission, and for a more flexible scheduling of the re-transmissions which improves the overall efficiency of the data transmission.

  - Reduces the constraints for scheduler and allows for more efficient HARQ operation.

- UL HARQ aspects: [3] [16] [27] [28]

  - Explicit UL HARQ-ACK feedback or UL HARQ process status feedback can be used by L2, so that fast ARQ retransmissions can be achieved.

  - It has been discussed on RAN2 that HARQ status report for PUSCH is beneficial to allow faster RLC retransmission. Study how to support an indication of UL HARQ status, e.g., the following options

    - Option 1. Indication in UL grant

      - E.g., by combination of NDI field and a dedicated field

      - E.g., by a field to indicate HARQ reTX or RLC reTX

    - Option 2. Indication in DCI other than UL grant

      - E.g., DL assignment

      - E.g., a dedicated format

    - Option 3: Dedicated channel

- Enabling/disabling HARQ-ACK feedback: [3] [6] [7] [12] [14] [16] [21] [25] [27] [28]

  - Study HARQ disable in 6GR for NTN and TN according to new requirements e.g. error tolerant HARQ and dynamic QoS scheduling.

  - Aiming at reducing HARQ-ACK feedback overhead and saving energy for both gNB and UE.

  - It is beneficial if the scheduler could indicate on a per-transmission basis or semi-static whether HARQ-ACK is to be included in a codebook or not for a PDSCH.

  - Large RTT use cases, such as NTN, need to be addressed by the HARQ mechanism for 6GR.

  - Consider HARQ-less transmissions in downlink and uplink.

  - When HARQ disabling is configured, UE may report assistance information e.g., statistics of CSI information, CQI considering UE side ACK/NACK, UE expected MCS/PRB allocation/RV/etc. in a semi-persistent or event-triggered manner.

- DL HARQ-ACK information is considered as L1 uplink control signaling (i.e., transmit via PUCCH): [3] [4] [5] [6] [7] [8] [9] [12] [14] [16] [17] [20] [21] [11]/[22] (small UCI payload) [28]

  - Low/minimal latency, and timing control

    - L2 signalling suffers from high latency. Based on the current MCE-CE processing delay, the latency exceeds 3ms.

    - L2 signalling based HARQ-ACK feedback would release the precise timing control over PDSCH and its HARQ-ACK feedback.

    - L1 HARQ-ACK enables a feedback timing close-loop that is an order of magnitude faster than higher layer procedures. Moreover, using higher-layer procedures introduce non-deterministic processing delays and scheduling jitters.

    - Minimal or no cross-layer interaction between the PHY and MAC layers.

  - Higher reliability and better coverage

    - Overall performance efficiency; higher-layer procedures cannot provide comparable gains in reliability and robustness.

    - Dedicated channels with different encoding rates can be allocated for L1 signalling and data to meet different reliability requirements.

    - Ease of controlling of L1 signaling reliability, due to less or no other procedure involved.

    - HARQ-ACK transmission at L2 results in link-level performance degradation.

    - PHY-based UCI has better performance since polar coding gets closer to the theoretical limit at low rates;

    - UCI does not have HARQ procedure.

    - Maximizes link performance and coverage at least for low HARQ-ACK payloads

  - System / resource / power efficiency

    - L1 signaling provides more efficiency, i.e., no additional header/padding overhead. Even a small amount of redundant information means a significant waste of resources and power consumption.

    - PUCCH facilitates efficient orthogonal multiplexing for multiple UEs.

    - L2-based HARQ-ACK feedback requires additional UL DCIs for scheduling PUSCH for HARQ-ACK transmission, and reduced coverage.

  - Better flexibility, usage and capacity

    - During random access, 1-bit ACK/NACK for Msg4 is fed back on PUCCH.

    - PUCCH formats provide flexibility for handling many scenarios including different HARQ-ACK information payload size, and different UL control channel durations.

    - A large number of UEs can be multiplexed on the same time resource by CDM (e.g., cyclic shift, TD-OCC).

- HARQ and CSI transmission using PUSCH (e.g., L2 MAC sub-PDUs or L1 PUSCH): [16] [11]/[22] (large UCI payload) [23] [26] (at least CSI) [29]

  - PUCCH is inherently vulnerable to transmission errors, resulting either in erroneous assumption of a correctly received packet or superfluous retransmissions for an already successfully received transport block.

  - To avoid/minimize scheduling restrictions, complex timelines and multiplexing rules

  - Most HARQ-ACK (including multi-bit) can be carried on PUSCH, possibly with a UCI-only PUSCH option.

  - The main advantage of L2 reporting is that there is no need for pre-configured PHY resources for the UCI, and the report can be transmitted on any PUSCH.

  - If joint encoding between UCI and data is used, UCI can be considered as L2 (MAC) data. Considering reliability difference between data and control, at least CSI can be handled as MAC.

### FL Proposals for Round 1 discussion

[High Priority] Proposal 1-1 (I): In 6GR, DL and UL HARQ operation designs should consider the following aspects:

  - latency

  - reliability

  - coverage

  - power saving (NW and UE)

  - complexity (e.g., processing, scheduling, multiplexing)

  - diverse services/applications

  - system/HARQ retransmission efficiency

  - feedback efficiency

  - device type

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | For complexity, we would like to add memory (or area-efficiency) as well. Also, we think instead of high-level agreements / proposals (like Proposal 1-1), it is more helpful if we list the issues we are trying to solve first (lessons learned from NR). |
| InterDigital |  | Not sure if we need “device type”. It would not be preferred to have different designs for different device types. We would be fine with listing issues to solve as suggested by QC. Suggest clarifying what are reliability aspects. At least “HARQ-ACK misinterpretation” should be listed. |
| Spreadtrum |  | Generally fine to limit it to DL HARQ. But for UL HARQ, some bullets are not needed. |
| FUTUREWEI |  | “device type” should not be included as not to be discussed in WG until further update from plenary (sometime after Sept 2026) |
| OPPO |  | For complexity, it would be better to also clarify it is for both NW and UE, similar with the power saving aspect. |
| Samsung | No | Requires further discussion. Some aspects, such as latency, reliability and complexity, are clear. Other aspects, such as NW/UE power savings, coverage, diverse services, device types, … are not. |
| TCL |  | For complexity, it needs to be managed for both UE and the base station side, UE needs lightweight processing due to limited power and hardware, while base station face heavy computation and scheduling demands. Balancing both sides ensures scalability, low latency, and reliable operation. |
| CATT |  | We are ok with taking latency, reliability, coverage, power and complexity into account for HARQ operations. However, further clarifications are required regarding the requirements listed in the proposal, including those for diverse services/applications, system/HARQ retransmission efficiency, feedback efficiency. |
| ZTE |  | Generally fine with the guidance/proposal. For each bullet, are there some details on the requirement, value, threshold or the design target? Furthermore, how to understand each bullet? For the latency, it could be the scheduling offset, feedback offset, RTT, etc.. For the reliability, it could be the reliability for DL/UL traffic transmission based on the HARQ operation, the reliability for the feedback channel to provide HARQ-ACK, or something else. For the coordination or combination of each bullet, it may be ambiguous for some bullets. For example, coverage can be merged with reliability or can be deleted since coverage is also related to the reliability. Efficiency shall be a single bullet (e.g. system efficiency, feedback efficiency) like other bullets, or could be merged with complexity as one potential example/component since efficiency could also have impacts on complexity. Device type can be merged with devise services/device with applications deleted because it is not clear about the meaning of the applications. Lastly, it is preferred not to discuss this proposal for a long time if no details of each bullet. |
| CMCC |  | We are fine to consider latency. Since first stage of 6GR design aims at services with tolerant latency requirement unlike URLLC, 6GR HARQ design should consider tolerant latency instead of strict latency. We are fine to consider reliability, coverage, power saving (NW and UE), complexity, system/HARQ retransmission efficiency, diverse services/applications. The consideration of feedback efficiency is not clear. We prefer to have union HARQ design for different device types. |
| DCM |  | Although it may be OK to discuss this kind of proposals, lessons learned from 5G, and more concrete design targets for 6GR may be more helpful. Besides, the aspects of “diverse services” and “device type” can be considered for HARQ design with low priority. |
| China Telecom |  | For complexity, we propose to explicitly distinguish between the aspects impacting the UE and gNB. On the UE side, key complexity considerations should include processing, multiplexing/demultiplexing and related capabilities. On the gNB side, the complexity factors would revolve around processing (e.g., precoding computation, multi-TRP coordination) and scheduling flexibility/dynamics. |
| vivo |  | We are not sure if some aspects are applicable for UL HARQ operation, maybe “at least DL HARQ operation” is more proper for now. “device type” is a bit unclear since it has not been determined by plenary. So we suggest to remove it or keep it as FFS. Agree with other companies that “complexity” should consider both network side and UE side. |
| Lenovo |  | We don’t understand how to take “device type” into consideration. |
| xiaomi |  | We share similar view with CATT. In addition, we suggest to consider the proposal separately for DL and UL HARQ. |
| Huawei, HiSilicon | Yes, with modifications | In our view, the topic of “Improved link adaption” should be also be considered in the list. Additionally, the bullets “latency” and” reliability” are not clear on what to study since they are too general. We suggest to revise the wording of both bullets to “feedback latency” and “feedback reliability” Add to the bullet points: - traffic characteristics to services subbullet - “/overhead” to feedback efficiency Remove/ffs device types. |
| Nokia |  | Some aspects are general and should be considered in 6G, but preferably after adding more details. Other aspects need to be clarified, e.g., diverse services, device type. Overall, we think we should have a unified solution and avoid ad-hoc designs. |
| Panasonic |  | We support the suggestion from Qualcomm to add memory (or storage) aspect. We also not sure device type is needed or not. |
| MediaTek |  | We are generally fine with most of the points listed, however; - We need to clarify the targeted performance and the relevance of some of the points (e.g., latency, reliability, and coverage) to avoid optimizing for corner cases. For example, RAN1 shouldn’t introduce specific channels/mechanisms to optimize the coverage of 1-bit HARQ-ACK transmission, as this would not be the bottleneck channel. Also, for efficiency, the gNB usually accumulates multiple HARQ-ACK bits in a codebook. - The design should consider practical operational scenarios rather than over-optimizing for corner cases. |
| NEC |  | Fine with the direction. However, we agree with some companies point that more detail is needed to consider whether to support it, for example, it is not clear how “device type” impact HARQ operation designs. |
| ETRI |  | In principle OK, however, we suggest adding one more bullet for a unified adaptation framework - Interaction/joint-optimization with other adaptation mechanisms (repetition, ARQ) To FL) On feedback efficiency bullet, Could you clarify what specific aspects are intended? (e.g., feedback overhead reduction, feedback resource efficiency, load balance between PUCCH/PUSCH)? |
| LGE |  | Various services and traffic types are introduced in 6G. In particular, in AI-token traffic, each token may have different error-tolerance characteristics, which leads to different reliability requirements. URLLC, eMBB, and NTN traffic also have different latency requirements. In addition, 6G is expected to support wideband signals (e.g., 400 MHz), which exhibit higher frequency selectivity compared with conventional 5G signals. As a result, the error rate may vary across frequencies, which could affect the efficiency of HARQ feedback and retransmissions. On top of the above, in order for ensuring flexible scheduling/HARQ operation to well cooperation with various 6GR operation scenarios. Therefore, in our perspective, HARQ operation designs in 6G should consider the below aspects at least. - latency - reliability - diverse services/applications - system/HARQ retransmission efficiency - feedback efficiency - flexibility of scheduling/HARQ-ACK timing |
| Fujitsu |  | We are generally fine with the proposal. However, the proposal seems to have both use case (device type, service/application) and technical aspects. We prefer to mainly focus on the technical aspects. |
| CEWiT |  | - Our understanding is that UL HARQ is based on RAN2 requirement or HARQ-ARQ tighter interworking and need not follow the same design aspects as DL HARQ. So we propose to separate DL and UL HARQ designs. - 6G requires a simpler design and not a design customized for each aspect as listed. So, we prefer at least to remove ‘diverse services/applications’ and ‘device type’ |
| Ericsson1 |  | We are supportive of general direction to list key considerations and problems to solve for 6GR HARQ.. However, more discussion is needed to get common understanding of what is in the list. For ‘latency’, HARQ design should consider realistic UE timelines including end-to-end system impact. For example, if UL TCP ACKs for DL data are slow, then optimizing DL HARQ turnaround time does not help after a certain point. This should be included in the study. Similarly, for ‘coverage’/ ‘reliability, the payload sizes at which HARQ feedback actually becomes the bottleneck should be studied. For ‘complexity’, both UE and network aspects should be considered. 6GR HARQ design should not impose tight coordination between different cells in CA and this also should be included in the study. |
| SONY |  | Generally fine and we have similar views as Qualcomm to consider the lesson learned from 5G. For complexity, we need to consider the timeline, particularly on the (time) flexibility for the feedback. |
| FL |  | Thank you for all very constructive inputs. It is understood some aspects listed in the original Proposal 1-1 (I) were not very clear to everyone. Let me try to clarify them here. Firstly, it should be noted that during the new design of HARQ operations in 6GR, some aspects may be considered for certain design of HARQ-ACK feedback while some others may not. Hopefully, the clarifications below can help better understand the intention. - Latency: This can be related to how quickly HARQ-ACK information can be feedback, latency of delivering a TB, and etc. - Reliability: This can be related to ACK-to-NACK probability, NACK-to-ACK probability, robustness to DCI missing, DTX/NACK differentiation and etc.) - Coverage: This could be used for scheme comparison between L1 and L2 based UCI signalling, also meeting enhanced overall coverage including UL, and targeting comparable coverage to 5G mid-band captured in the SID objective. - power saving (NW and UE): This could be used for comparison between different design schemes. - complexity (e.g., processing, scheduling, multiplexing): This could be used for comparison between different design schemes and UE processing capability. - diverse services/applications: This aspect relates to having a unified or even non-unified designs for e.g., AI, immersive services, NTN and etc. - system/HARQ retransmission efficiency: This is can be related to comparison between design schemes, resolving HARQ operating issues / deficiency seen in 5G(e.g., resolving issues / known problems in 5G) - feedback efficiency: E.g., number of A/N bits per TB, total HARQ-ACK payload and etc. - device type: This could be related to e.g., number of HARQ processes (8 for IoT, 16 for eMBB, 32 for FWA), memory capacity, support of certain HARQ features (disabling of HARQ, NACK-only), and etc. It is also worth noting that the design of UL HARQ operation in 6GR does not need to be the same as that in 5G, or be the same as DL HARQ operation in 6GR. Considering all the comments and suggested modifications made in the first round. The FL updates the proposal as follow with changes in red colour font. In 6GR, DL and UL HARQ operation designs should consider the following aspects: - latency - reliability - coverage - power saving (NW and UE) - NW and UE complexity (e.g., processing, scheduling, multiplexing, memory) - diverse services/applications/traffics - system/HARQ retransmission efficiency (including link adaptation) - feedback efficiency/overhead - FFS: device type Note: the design of DL and UL HARQ does not necessarily be the same |

[High Priority] Question 1-2 (I): For DL HARQ in 6GR, which one or whether both of the following reporting mechanisms should be studied?

- Mechanism 1: HARQ-ACK information bits are transmitted via L1 signalling (e.g., in PUCCH/PUSCH)

- Mechanism 2: HARQ-ACK information bits are transmitted via higher layer signalling (e.g., MAC CE)

| Company | 1 / 2 / Both | Comments |
| --- | --- | --- |
| QC | 1 | Given that mechanism 2 results in added latency, spectrum inefficiency, UL coverage issues, PDCCH overhead, and additional burden on UE (need to support larger # of HARQ IDs due to longer HARQ RTT), we believe mechanism 1 should be assumed in 6GR. We do not see much benefit in mechanism 2. |
| InterDigital | 1 | We prefer to focus on mechanism 1, given issues with higher layer signalling approach (coverage, coupling latency with higher-layer data, etc.). |
| Spreadtrum | 1 | Same view as QC and InterDigital. |
| FUTUREWEI | 1 |  |
| OPPO | 1 | Similar view with other companies. |
| Samsung | 1 | There is a number of reasons for keeping L1, including reliability, latency, efficiency, overhead, … Reasons given against L1 are either N/A or can have relatively simple fixes to be considered in 6GR. |
| TCL | 1 | From a latency and reliability perspective, Mechanism 1 is generally preferable for 6G DL HARQ. Sending HARQ-ACK via L1 signaling on PUCCH or PUSCH provides fast delivery with minimal processing delay and offers more robust. In contrast, MAC-CE based reporting introduces higher processing and scheduling delay and may be less reliable. |
| CATT | 1 | We prefer to study Mechanism 2, as Mechanism 1 has notable issues including low UCI transmission efficiency, poor reliability and high latency. |
| ZTE | 1 | Based on the nice FL summary, Mechanism 1 should be studied due to benefits on latency, reliability/coverage, efficiency and capacity. BTW, it is preferred to discuss this issue with highest priority. Because it has impacts on the discussion in other agenda (e.g., PUCCH transmission scheme). |
| CMCC | 1 | We prefer Mechanism 1 with benefits in reliability. |
| DCM | 1 | Performance target of UCI and higher layer data is basically different; L1 signaling is better for UCI, for efficient/easy performance management, i.e., due to clear drawbacks of mechanism 2 on latency and reliability. |
| China Telecom | 1 | Considering latency will be an important requirement for the 6GR design, the design should be an improvement of NR instead of the fallback of NR. We don’t see the motivation of mechanism 2. |
| vivo | 1 | We have similar views with other companies, and support only study mechanism 1 in 6GR. |
| xiaomi | 1 | We share similar view with ZTE. This issue affects the other topics, such as HARQ-ACK codebook type, PUCCH formats design, etc. Thus, we may discuss it with highest priority. |
| Huawei, HiSilicon | 1 only | Based on the aspects described in our contribution with respect to using L1 signalling versus any other higher layer reporting mechanism, we only support to study Mechanism 1. |
| Nokia | 1 | Mechanism 2 introduces more issues (latency, reliability) and the benefit is not clear. It is preferred to focus on mechanism 1. |
| Panasonic | 1 | We think UCI transmission via higher layer signaling such as MAC-CE can be beneficial to large UCI payload size (e.g., the order of 100 bits or more). As far as HARQ-ACK with such a large payload size is not likely, we think Mechanism 1 can work. |
| MediaTek | 2 | We prefer to streamline most of the UCI feedback on L2. This will avoid the need to adopt multiple channels and reporting mechanisms, as well as coupling between PHY resource allocation and payload type/size. With such a unified approach, the UE and gNB implementation complexity for UCI reporting can be significantly reduced. RAN1 should study both approaches. |
| NEC | 1 | Mechanism 1 is beneficial. The motivations are very clear as mentioned above. |
| ETRI | Both | Mechanism 1 can be considered as a baseline. Mechanism 2 can also be considered for specific scenarios (e.g., NTN with inherently long RTT) |
| LGE | 1 | As commented by other companies and in our contribution, considering UCI reliability/latency requirement as well as UL resource/header overhead, Mechanism 1 is preferred as UCI TX format for 6GR. |
| Fujitsu | 1 | Same view with other companies. |
| CEWiT | Both | Though we agree with the majority view on the HARQ-ACK information as L1-signaling, the current NR specification imposes tight timing and size constraints over HARQ feedback, making it in-efficient in terms of PUCCH resource utilization. Considering MAC-CE can lead to a very efficient HARQ-ACK design and as mentioned by MediaTek will reduce UCI complexity. |
| Ericsson1 |  | The NR feedback scheme is very complex with rigid timelines, UL Tx switching procedures differing between PUSCH and PUCCH, complications when aggregating carriers across numerologies and/or duplex schemes, complex multiplexing/prioritization rules between PUSCH/PUCCH etc. These problems should be discussed and understood before deciding on the transmission scheme to use. At this point in time we think both options should be kept open (although we think that option 2 avoids many of the aforementioned problems). |
| SONY | 1 | At least 1 as the baseline. Further study on the feasibility of 2, including capacity impacts of transmitting MAC CE (via PUSCH) rather than using an efficient PUCCH. |
| FL |  | Given that there are multiple companies think that both options or Option 2 (higher layer signalling) only should be further studied for carrying HARQ-ACK information for DL HARQ in 6GR, the FL proposes to continue the study of both options until the next RAN1 meeting. For DL HARQ in 6GR, which one or whether both of the following reporting mechanisms should be studied? study both following reporting mechanisms until RAN1#124bis. - Mechanism 1: HARQ-ACK information bits are transmitted via L1 signalling (e.g., in PUCCH/PUSCH) - Mechanism 2: HARQ-ACK information bits are transmitted via higher layer signalling (e.g., MAC CE) |

[High Priority] Proposal 1-3 (I): For DL and UL data transmission, study how to support asynchronous and adaptive HARQ operation.

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | Proposal is unclear. The terms “asynchronous” and “adaptive” may be interpreted differently by different companies. For example, is it correct understanding that HARQ operation in NR is “asynchronous” and “adaptive”? |
| InterDigital |  | Agree that QC that “asynchronous” is a bit ambiguous. Many companies identify the benefit of decoupling HARQ-ACK indication from corresponding PDSCH indication (for DL) for simplifying UL collision handling. Perhaps we can agree on “study how to support independent indication of PDSCH and corresponding HARQ-ACK”? |
| Spreadtrum | No | Same view as QC. |
| OPPO | Yes | We think the “asynchronous” and “adaptive” is ok since it was used in NR TR 38.802 as following: Asynchronous and adaptive DL HARQ is supported at least for eMBB and URLLC. From UE perspective, HARQ ACK/NACK feedback for multiple DL transmissions in time can be transmitted in one UL data/control region. Asynchronous and adaptive UL HARQ is supported at least for eMBB. To our understanding, synchronous HARQ means that the retransmission can only be occurred in deterministic occasion once the corresponding initial transmission is scheduled, otherwise it is asynchronous HARQ. For adaptive HARQ, it refers that the transmission parameter for the retransmission can be adaptively adjusted. |
| Samsung | Yes |  |
| TCL |  | Agree with QC. |
| CATT | Yes |  |
| ZTE | Yes | Based on the comparison of LTE and NR, asynchronous and adaptive HARQ operation shall be supported in 6GR. And all the other proposals can be regarded as how to support asynchronous and adaptive HARQ operation. Maybe the proposal can be updated as below: For DL and UL data transmission in 6GR, support asynchronous and adaptive HARQ operation. |
| CMCC | Yes |  |
| DCM | Yes | Maybe it is better to mention TR38.802 as a kind of note, to clarify them. |
| China Telecom |  | Similar view with QC, the current proposal lacks clarity, specifically regarding the definitions of "asynchronous" and "adaptive" in this context. We appreciate OPPO's subsequent clarification which suggests that the intent is to discuss asynchronous operations and mechanisms for improved retransmission handling, but the fundamental question remains unaddressed: What exactly constitutes an "asynchronous and adaptive HARQ operation" in this proposed design? The description is currently too abstract, concrete examples would be necessary to illustrate the intended operation. |
| vivo | Yes | We tend to agree with OPPO, and support the proposal. |
| xiaomi | Yes | We share the similar view with ZTE. In addition, based on the explanation of OPPO, maybe we can add a note for the Asynchronous and adaptive. Furthermore, the DL and UL can be separately proposed in two sub-bullets. |
| Huawei, HiSilicon | Yes | In our view, such an operation is needed to provide HARQ in 6G with the required flexibility. It is already supported in 5G, therefore we believe it worth support directly for 6G. Suggest remove “study how to”. We can certainly add enhancements on top. |
| Nokia |  | We agree with the statement. NR HARQ is already async and adaptive, and such properties should be kept in 6G. |
| Panasonic |  | We agree to have common understanding on “asynchronous” and “adaptive”. In our view, NR HARQ operation is dynamically scheduled synchronous and adaptive HARQ retransmission operations. |
| MediaTek |  | We should include a description of these terms to avoid any misunderstanding. Also, as part of the “asynchronous” aspect (or in a separate proposal, if preferred), we should discuss the feature to decouple the HARQ-ACK indication (e.g., codebook index or codebook construction) from the PHY resource allocation. |
| NEC | Yes |  |
| ETRI | Yes (conditionally) | we are ok to study, provided that our understanding is aligned with the intention of the proposal. In our understanding: Asynchronous HARQ: the retransmission occasion is not fixed with respect to the initial transmission. Adaptive HARQ: retransmission parameters (e.g., MCS, resource allocation, repetition level) can be adjusted for each retransmission. |
| LGE |  | We share the same view as Qualcomm regarding the need to clarify the definitions of ‘asynchronous’ and ‘adaptive’ HARQ operations. We believe that retransmissions and HARQ-ACK reporting should not be bound by a fixed timeline; instead, they should maintain the flexibility to be explicitly scheduled by the gNB, consistent with the principles established in 5G NR. |
| Fujitsu |  | We share same view with QC that we need to clarify the ambiguous terms of “asynchronous” and “adaptive”. |
| CEWiT | Yes |  |
| Ericsson1 |  | Agree with general direction but as other companies commented, more clarifications are needed. In NR, rigid timelines e.g., k1, were significant source of complexity. For 6GR HARQ mechanism should be such that UL HARQ feedback payload is not tightly coupled to PDCCH/PDSCH slot indices. |
| SONY |  | RAN1 needs to have a common understanding on “asynchronous” and “adaptive”. |
| FL |  | I think OPPO has tried to explain the meaning of “asynchronous and adaptive HARQ operation” in the above response input as it has been used and captured in NR technical report TR 38.802. To summarize, - Asynchronous HARQ refers to retransmission(s) occurs in a none pre-determined occasion once the corresponding initial transmission is scheduled. - Adaptive HARQ refers that the transmission parameter for the retransmission can be adaptively adjusted. As discussed with company offline, there is a request to decouple of UL/DL scheduling, timing and payload in 6GR, and this has been mentioned in some contributions to this meeting. Therefore, the proposal has now been modified as follow: For DL and UL data transmission, study how to support asynchronous and adaptive HARQ operation. - Option 1: HARQ feedback timing/payload is linked to the PDCCH/PDSCH reception timeline - Option 2: HARQ feedback timing/payload is not linked to the PDCCH/PDSCH reception timeline |

[Medium Priority] Proposal 1-4 (I): In 6GR, study whether/how to support disabling/enabling HARQ operation (e.g., HARQ-ACK feedback or HARQ retransmission)

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | No | We first need to discuss and agree how to define HARQ operation in 6GR (before studying whether/how to disable it). Also, what is the use case or the issue we are trying to address? NTN is being discussed separately, and MBS is not being considered in the first release of 6GR. |
| InterDigital |  | Suggest deleting “or HARQ retransmission” which seems to restrict network behaviour without clear reason? |
| Spreadtrum | Y | HARQ disable is benefit for XR or AI traffic. |
| OPPO | Y | To InterDigital: for NTN, HARQ-less operation is supported for UL, which means HARQ-retransmission for UL is disabled. |
| Samsung | No | Need/benefit for enabling/disabling HARQ needs to first be identified. |
| TCL | Y | Supporting the ability to enable or disable HARQ operation is valuable because different services have diverse latency and reliability needs. For ultra-low-latency or deterministic services, disabling HARQ retransmission may be beneficial, while enhanced reliability services may require full HARQ operation. Flexible HARQ control also helps optimize power consumption and reduce unnecessary signaling for lightweight or massive IoT devices. |
| ZTE | Yes | We are fine with the proposal due to the benefits on resource efficiency for long RTT service(e.g., NTN), service without requirement on high reliability(e.g, broadcast, multicast) or SPS transmission with sparse traffic. In addition, besides disabling/enabling HARQ operation, ACK/NACK only feedback should also be included here to study whether/how to support. Because it is also beneficial for resource efficiency. |
| CMCC | Y | At least in NTN scenario, HARQ enable/disable is supported to tolerate the larger RTT |
| DCM | Y | OK to discuss, including whether the feature is necessary or not. Although disabling HARQ retransmission, especially for NTN, could be beneficial on e.g., reducing HARQ stalling possibility under limited resource constraints or bad channel conditions, reducing UE side soft buffer burden, at the same time we are not sure whether that way is the best. |
| China Telecom |  | We know the intention of this proposal, but we are not sure whether this proposal is necessary. In our opinion, considering the disabling/enabling HARQ operation is more related to NTN agenda, we may need to wait for the NTN agenda’s conclusion first, and then reusing their conclusion. |
| vivo |  | We prefer to postpone this discussion until RAN1 have a basic design on HARQ operation. |
| xiaomi | Yes | We are OK with the proposal at least for NTN scenario. |
| Huawei, HiSilicon | Yes but comments | This is related to the discussion of number of HARQ processes as alternative solutions discussed in 5G. Suggest to have a general approach that HARQ process number is configurable, and regarding the maximum number of HARQ processes, we can study different solutions including disabling/enabling HARQ, increased HARQ processed number etc. |
| Nokia | No | Whether to disable the 6G HARQ operation may be discussed at a later stage, after it is designed. |
| Panasonic |  | Although HARQ feedback disabling is one option for long RTT scenario, it can cause frequent RLC retransmissions, which causes significant overhead and delay. On the other hand, having the soft buffer for all slots in the RTT up to e.g., 540 slots would pose undesired UE complexity. Therefore, HARQ feedback with and without soft combining (without storing the received data in the soft buffer) to allow MAC retransmission should be studied. |
| MediaTek |  | We should discuss the design objective, like “why HARQ operation need to be disabled”. |
| NEC | Yes | We believe disabling/enabling HARQ operation is beneficial for some of the services in 6GR. For example, some AI services. |
| ETRI | Yes | This feature could be beneficial for certain scenarios (e.g., NTN, AI token communications). |

| LGE |  | We agree to study the enabling/disabling of HARQ operations in 6G. Furthermore, in addition to studying the basic HARQ enabling/disabling mechanism, we believe this study should be extended different HARQ operation and flexible feedback content to accommodate diverse service requirements, not limited to NTN. |
| --- | --- | --- |
| CEWiT |  | We don’t see the reasoning to generalize the proposal. Beyond NTN with large RTT, use cases or scenarios for disabling HARQ needs to be discussed first. |
| SONY | Yes | Disabling HARQ may be needed in XR and NTN use-cases. |

## [ACTIVE] Topic #2: HARQ processing and management

Background:

UE processing time requirement for PDSCH (Tproc,1): In Clause 5.3 of TS 38.214, T_(proc,1) is defined as a UE processing capability (procedure time required by the UE) after the end of the last symbol of the PDSCH carrying the TB being acknowledged and the first symbol of the PUCCH resource to be used to carry the acknowledging HARQ-ACK information. Another words, it defines the minimum time required by the UE to process a received PDSCH and prepare the corresponding HARQ-ACK feedback for transmission. There are two PDSCH processing capabilities; processing capability 1 for normal UEs (mandatory) and processing capability 2 for low-latency UE.

UE processing time requirement for PUSCH (Tproc,2): In Clause 6.4 of TS 38.214, Tproc,2 is defined as a UE processing capability (procedure time required by the UE) after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling a PUSCH and the first uplink symbol of the scheduled PUSCH allocation. Another words, it defines the processing delay required by the UE to decode the PDCCH (grant), prepare the uplink data, and encode it before the first symbol of the corresponding PUSCH transmission. Similarly, there are two PUSCH processing capabilities; processing capability 1 for normal UEs (mandatory) and processing capability 2 for low-latency UE.

Number of HARQ processes in DL and UL based on UE reported capability: In 5G NR, the maximum number of HARQ processes is generally 16 for both DL and UL, a significant increase from 8 in LTE to accommodate shorter time slots and higher speeds. This number is configurable via RRC, with a default of 8 if the configuration is not provided. For specific scenarios like NTN and FR2-2, this number can be extended to 32.

### Contribution summary

- UE processing time requirement

  - PDSCH processing time (Tproc,1) and PUSCH preparation time (Tproc,2) should be dependent on one or more of the following [3] [4] [5] [17] [20] [23] [24] [29]

    - Power states / energy efficiency (e.g., power saving mode, full/fast processing mode scaled according to DL BW)

    - Device type

    - Single PDSCH processing time capability and single PUSCH preparation time capability

    - Realistic UE processing timelines

    - LLR reading access bandwidth restriction in terms of maximum number / sum TBS / sum data rate of DL TBs with non-zero RV in a slot across multiple CCs.

    - Number of spatial MIMO layers and BW

- Number of HARQ processes per cell: [4] [5] [6] [7] [17] [21] [28] [24]

  - Configurable

  - Max. number of HARQ processes = 32

  - Less than NR for SCS ≤ 30KHz

  - Unified design between TN and NTN

- HARQ process management and UE soft-buffer: [4] [24] [28]

  - In 6G, it is needed to study the mapping between TBs/CWs and HARQ process considering both CW-to-layer mapping and HARQ processing capability wastage. Moreover, 6G introduces a larger bandwidth, i.e., up to 200/400MHz whilst a HARQ process manages at most 100MHz bandwidth in 5G. Therefore, it is necessary to study whether and how to enhance the HARQ process management including e.g., flexible mapping to/across different frequency/layers in terms of multiple HARQ processes enabled at the same time due to the increase bandwidth.

  - A simple UE report indicating soft buffer 'headroom' could offer a practical and meaningful input for gNB scheduler decisions. This report indicates: (a) whether the UE has additional soft buffer memory available, and (b) if so (positive headroom), how much; and if not (negative headroom), the severity of the buffer shortage.

    - Consider event-based reporting in case of low / near-zero headroom or negative headroom.

    - Consider transmitting this report as a UCI or MAC-CE.

- Cross carrier HARQ retransmission: [3] [5] [13] [17] [25] [29]

  - Currently, HARQ processes were managed independently per cell/carrier in 5G.

  - Study scheduling aspects related within MCSC, including

    - Framework of UL transmission and DL reception across carriers within a cell

    - Cross carrier HARQ retransmission

  - N-carrier Single Cell (NCSC), i.e., a serving cell configured with N-carrier (N > 1), can be a candidate for 6GR CA operation. Study cross-carrier PDXCH retransmissions within a same cell.

  - Study cross-carrier HARQ process sharing, where one HARQ process can be used for multiple carriers.

  - Allowing cross-carrier HARQ process multiplexing and re-sequencing, i.e., reassigning HARQ process IDs and feedback timing to avoid such overlaps, could improve resource utilization, and better support dynamic scheduling cell selection in multi-carrier operation. This can also help to simplify HARQ feedback report timeline.

  - Carrier-specific HARQ utility prevents cross-carrier repetition and retransmissions, limiting the system performance.

Proposal: Study mechanisms for enabling cross-carrier repetition and retransmissions.

  - RAN1 to study cross-carrier HARQ, i.e., HARQ process management across multiple CCs to enable HARQ retransmissions at a CC that is different from CC used for the initial transmission of the same TB

    - For HARQ process indexing and indication,

      - If HARQ processes across multiple CCs are indexed separately (per CC as in NR), the DCI indication scheme that can distinguish which TB correspond which HARQ process needs to be studied.

      - If HARQ processes across multiple CCs are indexed jointly, the maximum number of HARQ processes for a carrier group needs to be discussed.

Send LS to RAN2 to inform RAN1 understanding on the benefit of supporting HARQ process management across multiple CCs and request discussion on how to support it from RAN2 perspective.

### FL Proposals for Round 1 discussion

[High Priority] Proposal 2-1 (I): For DL and UL HARQ in 6GR, from specification perspective, the maximum number of HARQ processes supported at the UE is 32 per cell. Further study whether UE supported number of HARQ processes can be lower than 32 based on one or more of the following

  - Device type (e.g., IoT, eMBB, FWA)

  - Device power state (e.g., energy saving, full)

  - Vertical application (e.g., NTN)

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC | No | 6G should go to the other way, i.e., decrease # of HARQ IDs and not increase. In first release of NR, the 16 HARQ IDs was an over-design, and in practice, these many HARQ IDs are not needed. In 6GR, there are more motivation to decrease the number of HARQ IDs due to 1) larger data rate 2) higher memory cost. Of course, there are some specific cases where 16 (or even larger) number of HARQ IDs may be needed like for FR2 in FR1-FR2 CA. These should be addressed based on the motivation and it should be case by case. For NTN, these aspects are under discussions in 10.7. |
| InterDigital |  | It is unclear why 32 is the starting point. May reformulate in more neutral way, i.e. study the maximum number of HARQ processes. |
| Spreadtrum | No | It is too early to go with an exact maximum HARQ process number. This is the first meeting to study HARQ aspects. |
| FUTUREWEI |  | “device type” should not be included as not to be discussed in WG until further update from plenary (sometime after Sept 2026) |
| OPPO |  | Share similar view with other companies that it is premature to conclude 32 as the maximum number of HARQ processes, even from specification perspective, maybe the first step can be as following: For DL and UL HARQ in 6GR, study the maximum number of HARQ processes supported at a UE from specification perspective based on diverse services/applications/device types. |
| Samsung |  | OK with 32 HARQ processes. OK to discuss cases where the number can be smaller but justification for each one should first be provided. We suggest having a “TBD” for the cases where the number of HARQ processes is less than 32. For less than 32 HARQ processes, the motivation of considering Device power state is unclear. We suggest to remove. |
| TCL | No | The need for 32 HARQ processes in 6G is not yet clear, further study will be needed. |
| CATT |  | OK with the 32 HARQ processes. We suggest taking the following aspects into account for the maximum number of HARQ process: - Deployment scenarios, such as FR1 TDD PCell and FR2 TDD SCell in TN. - DL peak rate |
| ZTE | Yes | Thanks for the FL summary. Besides NTN and FR2-2, it should be emphasized that HARQ processes can be extended to 32 for FR1 and FR2-1, due to typical FR1-FR2 CA scenarios. Since the frequency ranges for 6GR are similar/adjacent to the frequency ranges for NR, 32 HARQ processes per cell should be supported as baseline if the definition of cell is the same as NR, e.g. one carrier per cell. Note, for NR SUL cell with NUL carrier and SUL carrier, 32 UL HARQ processes can be configured for each carrier with the restriction of no simultaneous transmission on both carriers. If one cell with multiple carriers is supported with simultaneous transmission, larger HARQ processes(e.g. 64) may be required per cell. For lower than 32, it is not clear why eMBB, NTN, FWA are listed together with IoT. It is preferred to delete eMBB, NTN, FWA. |
| CMCC |  | OK with the 32 HARQ processes. For both TN and NTN in 5G, the maximum number of HARQ processes supported at the UE is 32, which one of the usage of 32 HARQ process in NTN is to match the longer satellite round trip delay to avoid stop-and-wait in HARQ procedure. We are fine to ffurther study whether supported number of HARQ processes can be lower than 32 considering device type and/or device power state. |
| DCM |  | Generally fine. Besides, for more efficient design, HARQ process can be defined not per CC but across CCs. This aspect should be added. |
| China Telecom |  | We can accept 32 as a potential maximum number, but we don’t think 32 as the fixed starting point for the baseline is the most efficient path forward. We propose to use X as the initial value, allowing companies to continue their internal evaluations and enables the discussion to focus first on the fundamental framework and procedures. The exact value for 'X' can be determined later. |
| vivo |  | We are OK to take maximum of 32 HARQ processes per cell as the starting point, but also fine with the revision of only “study the maximum number of HARQ processes” for current stage. |
| Lenovo |  | OK with 32 as the maximum number of HARQ process. And different UEs could report their capability, and gNB could configure the set of candidates HARQ processes for them. |
| xiaomi | No | It is a bit early to determine the maximum HARQ process as 32, and the motivation for reduce the HARQ process number for the listed cases is not clear. |
| Huawei, HiSilicon | No | We are not supportive of the proposal. In our view, the number of HARQ processes should be configurable. The actual maximum number of HARQ processes can be later defined once the high-level aspects of HARQ are agreed for 6G, so we propose to keep it FFS Other impacting factors including TBS, number of TBs/CWs that can be handled at the same time, should also be considered. |
| Nokia |  | This can be discussed later. |
| Panasonic |  | We agree to study maximum HARQ process number more. |
| MediaTek | No | We should discuss the HARQ operation design and the number of HARQ process will depend on the agreed framework. |
| NEC |  | It should firstly identify the motivation on the support of extended HARQ process number, and then discuss whether to extend it. |
| ETRI | No | we cannot see the reason why NTN UE supports the number of HARQ process lower than 32. In our understanding, NTN scenarios may need more processes due to long RTT (current 32 HARQ processes cannot cover all the NTN scenarios (e.g. GEO, MEO, … ) (32 HARQ processes can cover LEO case only) To FL) Could you clarify whether the intention of the proposal is to allow a flexible maximum HARQ process number per cell depending on device type, power state, or application requirements? |
| LGE |  | Same view as spreadtrum. In this meeting, it is more important to review the overall HARQ-related issues rather than discussing the maximum number of HARQ processes. |
| CEWiT | No | - First , the justification for increasing the HARQ process is not clear. - Second, the definition of HARQ process per cell is not very clear as we are yet to understand if cell refers to a carrier or a group of carriers. So we propose to not discuss this now. |
| Ericsson1 |  | This is a detailed aspect that can be considered later. The important aspect is the amount of buffer memory in the UE, not the number of HARQ processes as such. |
| SONY |  | It is too early to discuss this proposal at this stage. We don’t see the need to support more HARQ processes than for 5G NR. |

[High Priority] Question 2-2 (I): Which one of the following options should be studied for defining UE processing time/capability for processing PDSCH and PUSCH

  - Option 1：Single PDSCH processing and single PUSCH processing capability

  - Option 2：Multiple UE processing times/capabilities for processing PDSCH and PUSCH based on one or more of the following

    - Power states / energy efficiency

    - Device type

    - Number of spatial MIMO layers

    - Restriction in terms of BW or TBS

FFS: The value of UE processing time(s)

| Company | Option 1 / 2 | Comments |
| --- | --- | --- |
| QC |  | At least for the power state / energy efficiency, we see the need for a longer processing timeline, but it is not a capability (it is a power saving state controlled by NW). For device type, we are open to further study. For other 2 factors, the motivation is unclear and having a separate processing capability result in various complications such as out-of-order scheduling. Hence, we suggest: - Option 2：Multiple UE processing times/capabilities for processing PDSCH and PUSCH based on one or more of the following - Power states / energy efficiency - FFS: Device type - Number of spatial MIMO layers - Restriction in terms of BW or TBS To clarify, in the FL summary, our proposal on LLR reading access is unrelated to processing time (it is a separate UE capability on number of simultaneous ReTx across multiple CCs). |
| InterDigital |  | We can remove “capabilities” in both options to simplify the discussion. |
| Spreadtrum |  | Option 1 can modified to one processing capability per a device type. For Option 2: - Power states / energy efficiency in Option 2 is a type of relaxation on top of Option 1, not a capability. and the last two bullets are also included, due to the following agreement during last meeting Companies to report the assumed technique to achieve the reduction in the maximum schedulable PDSCH throughput. Current techniques include at least: reduced scheduled maximum bandwidth, reduced scheduled maximum rank, and reduced scheduled duty cycle.. - Remove device type in Option 2, it should be in Option 1. |
| FUTUREWEI | 1 | Option 2 is against the agreed principles of 6G study for system simplification “device type” should not be included as not to be discussed in WG until further update from plenary (sometime after Sept 2026) |
| Samsung | Option 1 | Additional UE processing times to save few symbols do not make any material difference in overall latency and complicate specifications while there have been no such deployments in NR. |
| ZTE |  | It seems no need to select one of the two options at this stage. It is not clear whether the items listed in Option 2 could be merged together as one or multiple processing times/capabilities. Instead, the items with impacts on the processing times/capabilities should be identified first. After that, whether single or multiple processing times/capabilities can be studied. Therefore, the proposal can be changed as below: Study/identify the items with impacts on UE processing capabilities for PDSCH processing procedure and PUSCH processing procedure. |
| CMCC | 1 | In 5G design, two UE capabilities are designed for eMBB UEs and URLLC UEs. First stage of 6GR design to simplify gNB scheduling, single UE processing capability is supported. |
| DCM |  | We are not sure the exact intention; this is for non-CA case? or, Option 2 is intended for CA case and each CC includes a single channel? |
| China Telecom | 1 | We are open to both options, but from our view, option 1 can be a baseline for the discussion, hence we prefer option 1 a little. |
| vivo | Option 1 | We have similar views as Samsung. Impractical and complicated timeline should be avoided in 6GR. |
| xiaomi |  | We share the same view with ZTE. In addition, the impacts beyond NR should be discussed with higher priority. |
| Huawei, HiSilicon | FFS | Study should focus on the sub-bullets to identify impacting factors. |
| Nokia |  | Option 1 should be the baseline. Any addition can be further discussed. |
| Panasonic |  | For Option 2, we support the update by Qualcomm. |
| MediaTek |  | RAN1 should aim to limit the number of processing timelines. Optimizing for one or few symbols is not used in real deployments and has not practical gain. |
| NEC | Option 1 | It seems to Option 2 is not related to UE capability fully, but related the configuration/UE setting/other aspects partly. We should not complicate such issue in the initiation state of 6G. |
| ETRI | FFS | Option 1 can provide simple framework Option 2 can provide better absorption across diverse deployment scenarios. |
| OPPO |  | - We share similar view with QC that the wording “capability” should be removed; - Regarding MIMO layers and BW or TBS, we think it should be kept since it highly relates to UE processing time, e.g., UE may be able to complete processing within a certain processing time only for data spanning limited bandwidth, for more wideband processing, longer time is needed. |
| LGE | 1 | We also have similar view as Futurewei and Samsung that 6GR should pursue simplified/unified design especially for this PDSCH/PUSCH processing time in order to simplify UL PUCCH/PUSCH multiplexing procedure. So, Option 1 is preferred at this stage, and Option 2 can be considered later if some aspects would be found necessary to be reflected in the processing time. |
| Ericsson1 |  | Option2 is perhaps more realistic assumption considering IoT, NTN etc. |
| SONY |  | We want to ensure that the UE processing timeline is not too aggressive as processing timeline significantly impacts device complexity. If the whole of 6GR has a non-aggressive processing timeline, we are OK with option 1, otherwise 6GR should allow for differentiated processing timelines (option 2) in order to support diverse device types. |

[Low Priority] Proposal 2-3 (I): Study whether and how to support cross-cell/CC HARQ operation (e.g., HARQ retransmission in a cell/CC that is different from the cell/CC used for the initial transmission of the same TB)?

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | May need more discussion about motivation. What is the use case? Which problem it is trying to solve? What is the additional complexity with this? Under which conditions this is discussed / studies (e.g., does it include moving a TB originally scheduled on a 400MHz CC in FR2 to a CC in FDD with 20MHz BW?) |
| InterDigital | Yes |  |
| Spreadtrum | Yes | We support to study. And according to the use case, at least MCSC where one cell is associated with multiple carriers, cross-carrier retransmission is benefit. An initial transmission on one carrier can be retransmitted on another carrier within the same cell. This is achievable if the entire HARQ process sharing across multiple carriers. Cross-carrier retransmission can significantly improve overall transmission efficiency by breaking the limitations. For other questions from QC, they can be studied further. |
| Samsung | No | This is a recurring proposal that has never been agreed as implementations have a separate MAC entity per cell. The case of a single cell having multiple (disjoint) carriers can be separately discussed later for whether any special handling is needed after concluding on “multiple-carriers single cell”. |
| TCL | Yes | We support to study. For CA and SCMC, supporting cross-carrier re-transmission can be beneficial from a latency perspective. |
| CATT |  | Further discussions are needed on cross-cell HARQ operations, including the underlying motivation, applicable scenarios, and associated MAC layer complexity. |
| ZTE |  | We are open for the study. However, it may be impacted based on the different understandings of the framework for multi-carrier. Whether only single CC or multiple CCs can be configured in one cell. Even multiple CCs can be configured in one cell, the multiple CCs configured in one cell can be regarded as one virtualized CC/BWP or still based on multiple physical CCs/BWPs. Whether the HARQ entity defined in RAN2 for 6GR is still the same as that in NR. Therefore, we tend to agree the view of FL to study the issue with low priority. |
| CMCC |  | At least in SCMC (single cell multiple carrier), it is natural to support cross-carrier HARQ, since it has a single MAC entity. For CA case, it is also benefit to support the cross-carrier HARQ to improve the HARQ feedback and retransmission efficiency. |
| DCM | Y | This should be studied for more efficient communication. |
| China Telecom |  | We understand the intention, and agree to study. BTW, the OPPO’s command is strange, could it be a writing error or mix-up? [OPPO]: Thanks for the kind reminder, I moved the response to Question 2-2 |
| vivo |  | We share similar views as Samsung. For a normal cell as in NR, cross-cell HARQ operation has not been justified so far. For a SCMC cell, it can be discussed separately whether HARQ re-transmission can be performed in a carrier different from the carrier where initial transmission is performed. It should be noted that in this case HARQ re-transmission remains to be performed within the same cell, not cross-cell HARQ re-transmission. |
| xiaomi | No | The cross-cell HARQ operation has large impact on HARQ entity and MAC layer operation. For SCMC case, we are open to study cross-carrier HARQ retransmission. |
| Huawei, HiSilicon | Yes | We propose to reword the example from “HARQ retransmission” to “data retransmission” since we are defining the complete framework for cross-cell HARQ operation. Additionally, cross-cell/CC HARQ operation can be studied e.g., in a virtual cell which consists of multiple carriers. |
| Nokia |  | This is not something that should influence the 6G HARQ baseline design. Then whether cross-cell/CC HARQ operation is supported or not could be considered after 6G CA operation is defined. |
| Panasonic | Yes | Related to how to enable UE to support 400 MHz bandwidth, we think Option 2 and Option 3 should be further studied to support 400MHz bandwidth. One separate chain of RF and (i)FFT can be defined as BWP. These BWPs can be contiguous in the frequency or non-contiguous among bands or overlapped as multiple TRP operation. By having multiple active BWPs over multiple of carriers, HARQ combining among different carriers are possible. This allows more quicker adaptation of the channel condition and better diversity among different carriers. |
| MediaTek | Yes | We support the study of cross-cell/CC HARQ operation. This is practically useful for UL with Tx switching, where the UE switches from one band to another, and the current HARQ operation prevents repetition and retransmission across carriers/bands, limiting the applicability of UL Tx switching in practical operation. |
| ETRI |  | Ok to study cross-cell/CC HARQ operation as lower priority item. The associated increase in soft-buffer complexity, especially for DL HARQ, should also be considered |
| LGE |  | It is considered necessary to discuss this issue within the scope of the 6GR study. Furthermore, depending on the progress of the ongoing discussions on spectrum utilization from a general aspect agenda (10.5.0)—particularly those related to multi‑carrier single‑cell operation—the direction of this issue may be significantly impacted. In case of multi-carrier single-cell (MCSC) scenario, the feasibility of cross-carrier HARQ operations may be investigated, as these operations would occur within the same logical cell. However, for CA scenarios, we should be more cautious as it involves inter-layer complexity and requires coordination with RAN2. Therefore, any study on cross-carrier HARQ for CA should be conducted with careful consideration of its systemic impact. |
| CEWiT | Yes |  |

## [ACTIVE] Topic #3: HARQ-ACK feedback timeline

Background:

DL HARQ feedback timeline (K1): In NR, gNB dynamic indicates PDSCH-to-HARQ feedback timing K1 in DCI (i.e. slot offset between PDSCH and UL slot). The candidate K1 values are configured by RRC {0…15} or pre-defined {1...8}. For Rel-17 NTN, the set of K1 values was extended to {0…31} to support large time offset between DCI and UL resource for HARQ reporting.

Dynamic HARQ-ACK feedback indication: In NR, dynamic HARQ-ACK feedback timing and resources can be dynamically indicated in DCI scheduling PDSCH transmissions using a “PDSCH-to-HARQ_feedback timing indicator” (K1) and a PRI fields. In addition, a counter DAI can be also provided to indicate the current {serving cell, PDCCH monitoring occasion} pair, while the total DAI indicates the total number of {serving cell, PDCCH monitoring occasion} pairs scheduled up to the current occasion, helping the UE determine the Type-2 HARQ-ACK codebook size.

Complex UCI contents and reference timing determination: In NR, UCI multiplexing procedure is often characterized by its high complexity for a UE and also gNB, involving various timeline checks and iterative resource determination. Specifically, whenever multiple PUCCHs or a combination of PUCCH and PUSCH overlap in time domain in a UL slot, the UE shall verify that the UL channels to be multiplexed should satisfy multiple processing time requirements simultaneously. This typically involves checking the UE processing time for PDSCH (Tproc,1), UE preparation time for PUSCH (Tproc,2) and multiplexing time requirements (Tproc,1mux and Tproc,2mux). In addition, the reference timing for UCI contents multiplexing checking floats dynamically based on involved PHY channel resources.

### Contribution summary

- Lessons learnt / pain points of NR scheduling timeline, HARQ UCI multiplexing: [3] [9] [21] [23] [24] [28]

  - Selection of K1 values and resulting scheduling restrictions

  - PUCCH resource overriding issue

  - Complicated UCI multiplexing/prioritization rule without meaningful usage

  - Last DCI issue and large dependency of inter-cell coordination for total DAI

  - CSI reports blocking uplink scheduling / OoO scheduling restriction for A-CSI reporting

  - No ACKs on PUSCH after uplink grant reception

  - Aggregation cross numerologies restricting downlink scheduling

  - DAI coordination across downlink carriers

- DL HARQ report timing

  - Reuse NR-like PDSCH-to-HARQ feedback timing (K1 and K_offset from NTN) with similar or larger values: [3] [6] [7] [9] [10] [17] [20] [28]

    - Different services and scenarios

    - NNK1 for flexible HARQ-ACK timing

    - Dynamic indication by DCI

- DL HARQ reference timing:

  - Single deadline timing for UCI determination and multiplexing [9] [17] [18] [21] [26]

    - A check point in time for one-shot UCI determination, multiplexing (including HARQ-ACK, CSI, SR) and for overlapping channels

    - Based on a time offset from the starting boundary of the feedback UL slot (e.g., take into account of UE processing capability / time)

    - UE expects all channel associated with the HARQ-ACK information transmitted in the slot to be received before the check point time

    - Nothing can be reverted/overridden by NW after the check point time

    - Different UCI types and/or PUSCH data can still be multiplexed into a single channel but through a single indication. The indication must be received before the check point time

- Time window based HARQ-ACK feedback timing [5] [15] [17] [28]

  - For a PUCCH feedback occasion, UE determines HARQ-ACK information for PDSCH(s) received in a time window, where the time window take into account of UE processing time. UE generates a HARQ-ACK codebook in a streamlined manner without relying on DCI indication to firstly determine a PUCCH slot, then determine the occasions of PDSCH/PDCCH receptions corresponding to the slot, and further determine the valid PDSCH/PDCCH receptions with in the occasions.

  - A time domain window can be defined as the reference feedback interval, within which HARQ‑ACK for all channels is collected and reported in a dedicated feedback slot. A single HARQ‑ACK feedback occasion for the entire transmission should be supported within a reference time‑domain window.

  - The HARQ-ACK multiplexing into the PUCCH resource can be determined based on a deadline, Tdeadline, associated with an occasion of the PUCCH resource. For example, HARQ-ACK information of a PDSCH received before Tdeadline, is multiplexed on the corresponding occasion of the PUCCH resource. HARQ-ACK information of a PDSCH received after Tdeadline is multiplexed on a next transmission occasion of the PUCCH resource.

  - Time window-based multiplexing without consideration of symbol-level overlap.

- Triggered-based HARQ-ACK feedback timing (e.g., DCI trigger may be separated from DL grant): [5] [6] [8] [10] [12] [16] [18]

  - NW sends a dedicated DCI to trigger UE to transmit HARQ-ACK information corresponding to multiple PDSCHs, based on multiple factors such as latency requirement, the amount of data stored in the buffer and channel conditions.

  - A non-numerical value can be indicated in the downlink DCI to indicate that HARQ-ACK would be transmitted on PUSCH.

  - For the (e)Type-3 semi-static HARQ-ACK codebook, the feedback triggering mechanism can also work properly and retain compatibility with NR, i.e., triggering via the DCI.

  - Consider decoupling of PDSCH scheduling and the corresponding HARQ-ACK PUCCH resource allocation which are performed by using separate DCI, based on HARQ-ACK hold indication and PDSCH group indexing (as for enhanced Type-2 codebook in Rel-16 NR-U).

  - At the end of a data burst, an additional DCI (DCI7) may be transmitted to indicate the resource for HARQ-ACK of the last PDSCH, unless PUSCH anyway would need to be scheduled.

  - A DCI scheduling PDSCH transmission includes a trigger to multiplex the HARQ-feedback in a TB, e.g. by triggering a MAC CE containing the HARQ-ACK feedback. PUSCH resources for transmitting HARQ feedback in a MAC CE can be CG-based or dynamically scheduled.

  - Not each DCI indication is associated with a container for UCI report. In 6GR (unlike NR where each bit in Type-2 codebook is associated with a PUCCH resource) such resource/container indication is relaxed and once the container is indicated, UE will report the accumulated HARQ-ACK on the indicated resource.

### FL Proposals for Round 1 discussion

[High Priority] Proposal 3-1 (I): Study how to determine the feedback timing between control/data reception and HARQ-ACK feedback

- Option 1: Dynamic indication (e.g., in DCI)

- Option 2: Semi-static configuration (e.g., a time window, feedback resource periodicity)

- Option 3: Pre-defined rule(s) (e.g., based on PDSCH processing time)

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | Option 2 and 3 need to be clarified (Option 1 is clear as it is current NR). For example, in Option 3, how the PUCCH resource / slot is determined? If it is based on a periodic resource, how it is different than Option 2? |
| InterDigital |  | Option 1 is ok but quite broad. We should distinguish between “Dynamic indication from DCI that schedules corresponding PDSCH” and “Dynamic indication from any DCI”. The first approach resulted in complexity for UL collision handling in NR. For Option 3, suggest deleting the “e.g. based on PDSCH processing time” and reword as “Other trigger(s)”. |
| Spreadtrum | N | One clarification and one comment: - It is for control/DL dynamic scheduled data - Option 2 and Option 3 cannot work in TDD, e.g. the PUCCH resource is dropped due to DL symbol. Some even more complex rule should be applied on top of them. They are even worse than Option 1. |
| OPPO |  | To clarify, the proposal does not preclude the combination among multiple options, right? For example, as QC pointed, the HARQ-ACK resources are configured periodiccally while the determination of the target resource is based on pre-defined rules, e.g., UE PDSCH processing time., to our understanding, this is the combination of option 2 and option 3. |
| Samsung |  | OK to discuss |
| CATT |  | For Option 1, we consider it is to cover at least two cases: feedback timing explicitly indicated by K1 in the DCI that schedules the corresponding PDSCH, and feedback timing implicitly indicated by the time resource allocation in the UL grant that schedules HARQ-ACK transmission. Option 3 requires more details to differentiate from the Option2. |
| ZTE |  | It is more important to discuss how to simplify the UCI multiplexing/overriding rules/timeline which is the main pain-point of NR UL feedback. Therefore, proposal 3-2 should be discussed with high priority which is beneficial for complexity reduction of UCI multiplexing/overriding. For example, based any one of above Option 1/2/3 without proposal 3-2, there is no complexity reduction of UCI multiplexing/overriding. In other words, without one shot determination of UCI (e.g., define a check point time for UCI determination including multiplexing and overriding), resolving how to determine the timing between PDCCH/PDSCH and PUCCH is not helpful for the main pain-point resolution. Therefore, it is preferred to study the timing between control/data reception and HARQ-ACK feedback as low priority. |
| CMCC |  | Option 1 needs more details. We think it includes “DCI that schedules PDSCH” and “one shot DCI indication of HARQ reporting resource”. Option 3 may be not work independently, which can work together with option 1 or 2. As discussed in our companion Tdoc R1-2600400, since only 10% of HARQ feedback reports NACK, UE initiated NACK-only based HARQ feedback mode benefits in reducing reporting overhead and UE power consumption. Under UE initiated feedback mechanism, the determination of HARQ feedback timing needs to be associated with the when the NACK-only feedback event triggers. Thus, we suggest to modify Option 3 as “Option 3: Pre-defined rule(s) (e.g., based on PDSCH processing time, event trigger)”. |
| DCM | Y | Fine to study these options, and it seems better to clarify any combination of multiple options are possible. |
| China Telecom | Y | We think this proposal is clear, we support this proposal. |
| vivo |  | Share similar views with other companies that more clarification is needed for these options before discussing whether this proposal is agreeable. |
| Lenovo |  | We support option1. But we think it is used for determining the slot/subslot to transmit the HARQ-Ack but may not be used for determining the PUCCH resource in the slot/subslot. For the resource determination, more discussion is needed. |
| xiaomi |  | We are open to discuss it. However, option 3 is not very clear to us. |
| Huawei, HiSilicon | Yes | Option 1 should at least be supported. |
| Nokia |  | Option 1, based on NR, needs to be further discussed as it is quite general. Benefits of Options 2 and 3 need to be understood. |
| Panasonic |  | On Option 1, we agree to InterDIgital that dynamic indication in DCI can be distinguished between via DCI that schedules the corresponding PDSCH and via any DCI. |
| MediaTek |  | We think we should discuss if there is need for coupling between the DL data scheduling and the PHY resource scheduling for HARQ-ACK. If these are decoupled, the HARQ-ACK feedback should be ready based on the UE processing timeline. When the HARQ-ACK is transmitted, should be based on the network scheduling. |
| ETRI | Yes | It is reasonable to study all options for different scenarios: - Option 1 (Dynamic): For tight feedback scenarios with frequent channel variation - Option 2 (Semi-static): For predictable scenarios or energy saving - Option 3 (Pre-defined): For very simple devices or specific cases |
| LGE |  | Open to study other options, but Option 1 should be baseline for 6GR as in 5G NR, in order for ensuring flexible scheduling/HARQ operation to well cooperation with various 6GR operation scenarios. |
| Fujitsu |  | We are ok to study, but it is needed to clarify more. For example, Option1 can interpret two ways as (1) PDSCH to HARQ feedback timing in scheduling DCI or (2) oneshot triggering by DCI. |
| CEWIT | Yes | We agree to study the options with at least option1 supported |
| Ericsson1 |  | The meaning of different options needs more discussion. Rigid timelines e.g., k1, like in NR should be avoided as they were significant source of complexity. 6GR HARQ mechanism should be such that UL HARQ feedback payload is not tightly coupled to PDCCH/PDSCH slot indices. UE sending feedback in response to NW indication e.g., in UL grants avoids the complex timing relationships. |
| SONY | Yes | OK to study these options. |

[Medium Priority] Proposal 3-2 (I): Study whether and how to define a check point in time for UCI determination and/or multiplexing (including HARQ-ACK, CSI, SR)

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | Ok in principle, but this study may need to wait until RAN1 can progress on basic HARQ operation first. |
| InterDigital |  | Agree with QC that this is too early. |
| Spreadtrum | Y(partly) | Two comments: 1. The contents in bracket can remove first, to avoid the arguments of UCI types. 2. it includes UCI on PUCCH and PUSCH Study whether and how to define a check point in time for UCI determination and/or multiplexing on PUCCH and/or PUSCH (including HARQ-ACK, CSI, SR). |
| OPPO |  | We suggest to replace the term “check point” with “reference point”. To our understanding, “check point” is something very related to UE implementation. What is worth to be specified is the “reference point” and “UE processing time”, which restricts the deadline for network scheduling. As for which time UE start to do processing, it totally depends on UE implementation. |
| Samsung | Yes | We agree with the intention. But considering the agenda is for HARQ and that CSI/SR and other potential UCI are handled in other AIs, we suggest to start with HARQ first. |
| TCL | Y | We are agree with the proposal. |
| CATT |  | We are ok to study the check point in time for UCI and/or multiplexing with the HARQ. |
| ZTE | Yes | Support to study the mechanism like one shot determination of UCI (e.g., define a check point time for UCI determination including multiplexing and overriding). The main pain-point of NR UL feedback is the UCI multiplexing/overriding rules/timelines. Because the processing of DL data associated with UL feedback may be based on the time order with minimum processing time defined only from the end of control/data reception, that is, without check point time for UCI determination. Therefore, it is preferred to study an efficient mechanism on UCI determination(e.g., multiplexing, overriding) with high priority. The proposal can be updated as below. Study whether and how to define a check point in time for UCI determination and/or multiplexing (including HARQ-ACK, CSI, SR) - E.g. the check point is T_offset before the slot with UCI transmission |
| CMCC | Y |  |
| DCM | Y |  |
| China Telecom |  | Similar with OPPO, a checkpoint seems UE has some capability to determine something adaptively. Considering UE behavior is based on spec., the reference point is appropriate. |
| vivo |  | We support it in principle. Agree with QC that this can be discussed later until RAN1 agree on basic HARQ operation. We are also fine with OPPO’s suggestion. |
| xiaomi | yes | we suggest to add “at least” in the bracket to avoid limit the potential new UCI types in 6G. |
| Huawei, HiSilicon |  | The “check point” is not clear. We suggest to have clear definition before the study. |
| Nokia |  | This is something to consider after HARQ design progresses. |
| Panasonic |  | Support the update by Spreadtrum. |
| MediaTek |  | We support introducing a reference timeline where after it the UE doesn’t expect a change in the multiplexing rules and the PHY resources. |
| NEC |  | We are fine with the direction of this proposal. However, as some companies mentioned, the other type of UCI except HARQ-ACK are included in this proposal. We should avoid the duplicated discussion(for example UCI multiplexing) in other agenda. Besides, some new type of UCI may be introduced in 6GR(for example in ISAC). |
| ETRI |  | Not at this stage. This topic could be discussed later after Question 2-2 is resolved |
| LGE |  | We are also OK to study whether and how to define a check point in time for UCI multiplexing and/or PUCCH determination. |
| Fujitsu |  | We think the discussion of this issue is too early. We can discuss this issue later after decision of timing related issue about HARQ feedback. |
| CEWiT |  | We agree to the proposal with considering only HARQ to start with as mentioned by Samsung |

## [ACTIVE] Topic #4: HARQ-ACK feedback operation

Background: The following HARQ reporting contents are supported in 5G NR.

Single ACK/NACK feedback bit per TB: This is the basic / simplest form of HARQ-ACK feedback supported in LTE and NR. Only one bit for ACK or NACK is used to convey PDSCH decoding / CRC checking result for a received TB.

Multiple ACK/NACK feedback bits per TB (CBG based): For CBG-based HARQ-ACK in NR, when large TB sizes are scheduled, which can be segmented into several CBs, then the CBs can be divided into at least one CBG, and a HARQ-ACK bit is reported for each CBG. CBG-based HARQ re-transmission can achieve higher spectrum efficiency without re-transmitting CBGs which have been received by the UE correctly, especially for the case when different CBGs may experience different channel conditions or have different transmission performance. However, multiple ACK/NACK bits (up to 8 bits) are required for one TB to aid for CBG-based HARQ-ACK re-transmission, thus, larger HARQ-ACK feedback overhead compared to TB-based HARQ-ACK.

Single ACK/NACK bit for multiple TBs (A/N bundling): For TB-based HARQ-ACK, bundling operation can be performed to reduce HARQ-ACK feedback overhead, with coarse granularity for HARQ re-transmission, and the following bundling operation are supported in 5G NR:

  - Spatial bundling, wherein binary AND operation is performed between HARQ-ACK information bit for the first TB and that for the second TB conveyed in a PDSCH, to generate only one HARQ-ACK bit for the PDSCH.

  - Time domain bundling, wherein TDMed PDSCH(s) scheduled by a single DCI can be divided into at least one PDSCH bundling group. When spatial bundling is not configured, for each enabled TB for a PDSCH bundling group, binary AND operation is performed among HARQ-ACK information bits for the enabled TB and for all PDSCHs contained in the PDSCH bundling group, to generate only one HARQ-ACK bit for the enabled TB and for the PDSCH bundling group. When spatial bundling is configured, binary AND operation is performed between HARQ-ACK information bit for the first TB and that for the second TB conveyed in each PDSCH contained in the PDSCH bundling group, to generate only one HARQ-ACK bit for each PDSCH, then binary AND operation is performed among HARQ-ACK bits for all PDSCHs contained in the PDSCH bundling group, to generate only one HARQ-ACK bit for the PDSCH bundling group.

HARQ-ACK codebook

  - NR Type-1 HARQ-ACK codebook is generated based on PDSCH candidate occasions of DL slots determined by PDSCH-to-HARQ feedback timing set (i.e. K1 list) configured by RRC parameter dl-DataToUL-ACK. The payload size scales with carrier number, entry number in K1 list, CBG number for a TB. There is redundant payload in Type-1 HARQ-ACK codebook and reporting overhead is large especially in CA scenario or with CBG transmission.

  - NR Type-2 HARQ-ACK codebook has comparatively small and variable payload size depending on DAI counter in DCI. DAI is counted firstly in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index. If last DCI is missed, gNB and UE may have misalignment of payload size and gNB cannot decode HARQ-ACK codebook correctly.

  - NR Type-3 HARQ-ACK codebook was introduced in NR-U scenario where PUCCH scheduled by DCI may not be transmitted by a NR-U UE due to LBT failure. To acquire these accumulated HARQ bits, gNB triggers one shot Type-3 HARQ-ACK codebook by one-shot HARQ-ACK request field in DCI. Type-3 codebook can be triggered by a DL-related DCI Format (e.g., DCI Format 1_1 or DCI Format 1_2 or DCI Format 1_3) with or without a DL assignment. HARQ bits of Type-3 HARQ-ACK codebook are ordered by CBG index, TB index, HARQ process ID and cell ID. The payload size of Type-3 HARQ-ACK codebook is larger than Type-2 HARQ-ACK codebook. There is redundant payload in Type-3 HARQ-ACK codebook, for example UE needs to feedback HARQ bits even if no new TB of one HARQ process ID is scheduled. Compared with Type-2 HARQ-ACK codebook, Type-1 and Type-3 HARQ-ACK codebook can ensure alignment of payload size between gNB and UE when last DCI is missed.

NACK-only feedback in sidelink and MBS

  - In both SL and MBS, NACK-only based HARQ feedback was introduced for groupcast/multicast scenario where the transmitter re-transmits a TB when one UE in the group feeds back a NACK on a common HARQ resource. This is particular useful to save PSFCH / PUCCH feedback resources and reporting overhead where receiver UEs do not need to a large number of ACKs for a correctly decoded TB.

### Contribution summary

- HARQ-ACK feedback for single TB of one cell should be supported: [3] [5] [19] [20]

- Study NR Type-1 HARQ-ACK codebooks as a starting point for further enhancement: [4] [5] [6] [21] [24]

  - No ambiguity issue caused by DCI missing; The NACK-bit set for the slot without PDSCH scheduling as prior information does not affect the actual coding rate; The size of the codebook does not change significantly, so the number of PUCCH resources pre-configured to a UE can be reduced or other more optimized ways of determining PUCCH resource can also be considered

  - Enhancement on payload size reduction

  - In 6GR, it is preferred to optimize Type-1 HARQ-ACK codebook for CA with less overhead. Study the construction of type 1 HARQ-ACK codebook with only activated cells.

  - Type1 HARQ-ACK codebook, if supported, should be simplified and its large payload size (excessive NACK padding) issue should be addressed properly.

- Study NR Type-2 HARQ-ACK codebooks as a starting point / baseline: [2] [4] [5] [6] [7] [8] [9] [10] [11] [12] [15] [16] [17] [21] [24] [26] [28] [29]

  - Target for a 6G HARQ-ACK feedback design that allows decoupling of DL scheduling information between cells or cell groups.

  - Study mechanisms to improve the reliability by addressing or mitigating the impact due to miss detection of DCI at the UE or UCI at the BS.

  - Study how to achieve efficient PUCCH allocation.

  - For non-co-located CA, study how to resolve DAI counting between carriers.

  - For 6GR Type-2 HARQ-ACK codebook, cells with the same SCS are grouped together, and C-DAI and T-DAI update are confined within the cell group.

  - In practice, only Type-2 HARQ-ACK codebook is widely used in NR commercial networks.

  - Potential enhancements: 1) incorporate the one-shot feedback capability from the Type III codebook; 2) streamline the type I codebook structure and maintain it as a reliable fallback mechanism for the type II codebook.

    - Support both sub-codebook based procedures and the cell-based feedback scheme to enhance the type-II codebook for the first stage of 6GR codebook design.

    - Reusing the NR codebook retransmission mechanism for the potential 6GR codebook design.

  - Study the independent construction of Type-2 HARQ-ACK codebook for each serving cell.

  - Study payload size reduction compared to that in 5G NR, for multi-carrier scheduling

    - E.g., generate a sub-codebook for each scheduling-cell group that shares the same maximum number of scheduled cells

    - E.g., generate a codebook based on C-DAI accumulated according to the number of scheduled PDSCHs, where the C-DAI field size is larger

  - Study enhanced Type-2 HARQ-ACK codebook to improve the efficiency by reducing “filler” bits. To achieve such codebook design, the DL-DCI can have a LENGTH field of W bits indicating the modulo-2W codebook length. UE updates codebook length based on LENGTH field indicated in the DCI. The last Ni positions are assigned to the Ni PDSCHs scheduled by that DCI. On each DCI, codebook grows by the number of PDSCH scheduled. At any point in time, the last Nk,Tx bits (ending at position Lk,Tx) of the codebook correspond to the last transmitted DCI. The design concept for the enhanced Type-2 codebook is illustrated in below figure. With the use of LENGTH field to indicate the codebook length, there is no need for any of the NR DAI fields (counter DAI or total DAI).

- Study NR Type-3 HARQ-ACK codebooks as a starting point or optional: [2] [5] [6] [7] [8] [9] [10] [12] [16] [17] [24]

  - Study Type-3 HARQ-ACK codebooks and enhancements necessary to support HARQ-ACK reporting with decoupled scheduling.

  - The generation of semi-static HARQ-ACK codebook is relatively simple and DCI overhead is low; No ambiguity issue caused by DCI missing; The NACK-bit set for the slot without PDSCH scheduling as prior information does not affect the actual coding rate; The size of the codebook does not change significantly, so the number of PUCCH resources pre-configured to a UE can be reduced or other more optimized ways of determining PUCCH resource can also be considered

  - The enhanced Type-3 HARQ-ACK codebook enables decoupling between downlink scheduling and uplink feedback, which relaxes the gNB scheduling limitation, and this codebook type has stability while also offering a certain degree of flexibility

  - CMCC [7]: Enhancement on payload size reduction

  - Study potential enhancements for enhanced Type-3 HARQ-ACK codebook in 6GR so that it can include HARQ-ACK for non-scheduling DCIs (i.e., including DCIs indicating SPS release, or TCI state update, without scheduling any PDSCH reception).

  - Additional triggering mechanisms over NR can be studied, such as triggering by an UL related DCI format;

- Study new type of HARQ-ACK codebook: [6]

  - UE feedback HARQ-ACK without multiplexing HARQ-ACK corresponding to different DCIs, this can be ensured by gNB implementation. gNB should avoid indicating same resources for PDSCHs corresponding to different DCIs. Since one DCI scheduling multiple PDSCHs in different slots or different carriers is supported in 5G NR later releases, this can also be supported in 6GR, not only DCI overhead can be reduced, but also HARQ-ACK feedback can be simplified.

- For 6GR, further study CB or CBG-based HARQ-ACK feedback for a TB: [4] [5] [8] [10] [12] [13] [17] [21] [24] [27] [30]

  - Considering the further demanded spectral efficiency in 6G and potential larger TBS, finer transmission granularity enabled by CBG should also be inherited, with potential enhancements for new traffic requirements or characteristics.

  - In practice, TB decoding failure often results from a small number of CBs are in error. Even for NR CBG, many CBs are still redundantly retransmitted. Enhanced CB-level HARQ-ACK feedback to minimize the probability of correctly decoded CBs being expressed as NACK can be studied in 6GR.

Proposal scheme: One or more codebooks (e.g., based on AI/ML mechanism) are coordinated between NW and UE, where each codebook contains K highly probable states from 2L CB-level HARQ-ACK feedback states, the value of K is limited by UCI/DCI payload, and L is the number of CBs. UE transmits the index of one row ensuring no False Negative (NACK->ACK) and minimal False Positive (ACK->NACK).

Figure: One example of enhanced CB-level HARQ-ACK feedback

  - Proposed scheme: The overhead of the CBG-based HARQ-ACK codebook can be further reduced through a two-stage HARQ-ACK feedback mechanism. Specifically, the HARQ-ACK feedback is separated into Transport Block (TB)-level feedback and CBG-level feedback, which are encoded separately. The size of the CBG-level HARQ-ACK feedback is dependent on the decoding result of the TB-level HARQ-ACK feedback: if the TB-level feedback for a TB is ACK, then the CBG-level feedback for the TB may be omitted or simplified; if the TB-level feedback for the TB is NACK, then the CBG-level feedback is activated to indicate CBG-level ACK/NACK bits for the CBGs of the TB. This two-stage design achieves more refined overhead control while retaining the CBG-based retransmission advantage.

  - A single TB may carry multiple types of traffics. Each traffic may require different levels of feedback granularity. Also, in AI token traffic, each token can have distinct characteristics such as error tolerance and differentiated importance and priority, which may further necessitate differentiated feedback resolution. Additionally, each CB may experience different channel environments depending on the impact of CLI, signal bandwidth, or etc.

Proposal: Study the adaptive CBG size for HARQ-ACK feedback in 6GR.

Proposal: Study enhancements for selecting CBs for CBG formation in 6GR, considering channel quality or traffic characteristics.

  - Usually, the performance for different CBGs could be different. The CBGs closed to the DMRS should have a better performance than the CBGs far from the DMRS. Thus, it is necessary to study the overhead reduction for HARQ feedback for CBG based transmission, e.g., non-uniform CB-to-CBG mapping.

Proposal: Study the overhead reduction schemes for HARQ feedback for CBG based transmission, e.g., non-uniform CB-to-CBG mapping.

  - Typically, only one or a small number of CBGs would fail to decode and the size of the CBG-based HARQ-ACK information could then be reduced in order to capture its benefit without introducing unnecessary overhead.

Proposal: Study support of CBG-based HARQ-ACK retransmissions in 6GR.

- Study mechanisms to reduce the size of HARQ-ACK information when CBG-based HARQ-ACK feedback is used.

  - Given that the probability of ACK is much higher than that of NACK, the number of feedback bits needs to be reduced for TBs with ACK, while for TBs with NACK, more precise CBG-level feedback is required to improve the retransmission efficiency.

Proposal: Study feedback overhead reduction on CBG-based HARQ-ACK codebook, e.g. mixed TB/CBG-based HARQ-ACK codebook, or two-stage HARQ-ACK feedback.

  - When a TB fails, there are relatively small number of CBs in error statistically.

    - HARQ retransmission of the whole TB is not resource efficient.

    - NR’s CBG-based feedback / retransmission does not offer a good {retransmission efficiency, feedback overhead} trade-off, which may be partly the reason that this feature is not widely deployed.

| Entry 1 (1 CB set) | {1,2,3,4,5,6,7,8} |
| --- | --- |
| Entry 2 (2 CB sets) | {1,3,5,7}; {2,4,6,8} |
| Entry 3 (3 CB sets) | {1,4,7}; {2,5,8}; {3,6} |
| Entry 4 (4 CB sets) | {1,5}; {2,6}; {3,7}; {4,8} |
| Entry 5 (5 CB sets) | {1,6}; {2,7}; {3,8}; {4}; {5} |
| Entry 6 (6 CB sets) | {1,7}; {2,8}; {3}; {4}; {5}; {6} |
| Entry 7 (7 CB sets) | {1,8}; {2}; {3}; {4}; {5}; {6}; {7} |

Proposal: Study robust codebook construction methods including the following:

- For r=1,2,…,nCB-1, define an entry r with r CB sets, such that i’th (i=1,2,…,r) CB set is formed by CBs [i:r:nCB] (Matlab notation).

- For a given number of feedback bits, K, select 2K-2 feedback states from the entries above to include in the codebook, and also include “all Acks” and “all Nacks” as two additional feedback states.

  - The benefit of this is to decode early one or more complete MAC subPDUs which are not dependent on the remaining Code Blocks. Hence, HARQ feedbacks can be sent immediately to the transmitter which relate to multiple MAC subPDUs. This will also facilitate a parallel processing of the same transport block.

Proposal: Study mapping integer number of MAC subPDUs to one Code Block Group (CBG) and the corresponding HARQ feedback mechanism.

- CB / CBG-based HARQ-ACK feedback should not be supported or FFS: [20] [28]

  - Increased DCI overhead. System-level PDCCH overhead is more critical problem in real fields.

  - Spec complexity. Large amount of 3GPP discussion time is consumed for CBG-based approach.

  - Impracticality. No/little NW uses this feature so far.

  - 6G can evaluate multi-bit HARQ-ACK per TB for large TB scenario and high-rate scenarios, including the benefit/cost trade-off in UCI overhead, implementation complexity, etc.

- NACK-only or ACK-only HARQ feedback: [7] [12] [14] [21]

  - UE initiated HARQ feedback mechanism as UE initiated beam reporting supported in NR can be applied to NACK-only based HARQ feedback mode.

    - In the first stage, the UE transmits an Event-Trigger UCI (ET-UCI) indicator that indicates gNB the second stage transmission.

    - In the second stage, the UE transmits NACK-only based HARQ feedback which contains the specific HARQ bits.

  - UE initiated HARQ-ACK reporting: When the scheduler operates at high reliability point, e.g. targeting 99% ACK for the first transmission, providing HARQ feedback for every PDSCH is wasteful. In this scenario it would be preferable that the UE reports HARQ feedback only when NACK occurs to reduce UCI overhead and improve coverage. Conditions for triggering HARQ-ACK feedback can be discussed as part of the study. The exact mechanism for UE-initiated HARQ-ACK reporting should also be further studied.

  - For SPS PDSCH, study ACK only or NACK only for HARQ-ACK overhead reduction.

- HARQ-ACK for DCIs not scheduling PDSCH: [20]

  - In 5G NR, HARQ-ACK information can be generated for some specific DCI formats not scheduling any PDSCH reception, e.g. a DCI format indicating SPS PDSCH release (Rel-15) or SCell dormancy (Rel-16) or TCI state update (Rel-17). This provides a reliable feedback mechanism for the gNB to confirm UE’s successful reception of critical downlink control signaling.

### FL Proposals for Round 1 discussion

[High Priority] Proposal 4-1 (I): Study whether and how to support the following HARQ-ACK reporting contents

- Option 1: Single feedback bit for a single TB/codeword (e.g., non-codebook based)

- Option 2:  Multiple feedback bits for a single TB/codeword (e.g., CB/CBG)

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | Eventually, both options may be needed, but for Option 2, the shortcomings of CBG-based feedback should be addressed given no / minimal deployment despite the availability of feature in the first release of NR. BTW, “non-codebook based” in Option 1 is unclear to us. What does it mean? It can be a special case of e.g., TB-based Type2 codebook. |
| InterDigital |  | Why “(e.g. non-codebook based)” in Option 1? We can have codebook without CBG-level feedback. Also, “single feedback bit” for single TB seems to preclude soft-HARQ feedback (discussed in AI 10.5.5). |
| Spreadtrum | Y (partly) | “(e.g. non-codebook based)” can be removed. |
| OPPO |  | Share similar view with Spreadtrum to remove the “e.g. non-codebook based”. To our understanding, this proposal is only for study of TB based or CBG based or CB based feedback, nothing to do with the HARQ codebook. |
| Samsung | Yes | We understand that option 1 and option 2 are not exclusive. |
| TCL |  | The part inside the parentheses is not needed in Option 1. |
| CATT |  | To our understanding, this proposal aims to study both TB-based and CBG-based HARQ-ACK feedback. However, like other companies, we are unclear on the intent of the examples of non-codebook based, and CB/CBG. |
| ZTE |  | Both options are useful for HARQ-ACK reporting. Option 1 should be supported as the fallback mechanism. Option 2 is more efficient compared with TB-based HARQ-ACK codebook. For 6GR with larger payload transmission in 200/400 MHz bandwidth, CBG-based HARQ-ACK feedback has higher resource efficiency for retransmission compared with TB-based feedback. For further improvement, the CBG-based HARQ-ACK overhead can be reduced due to the probability of ACK is much higher than of NACK. The codebook design for Option 2 should be based on a trade-off between efficiency and complexity. On the other hand, Considering TB-based HARQ-ACK codebook was deployed widely in commercial network, it is preferred to support 1-bit for each TB in a HARQ-ACK codebook as the fallback of Option 2. The two options can be updated as below. - Option 1: Single feedback bit for a single TB/codeword (e.g., non-codebook based) - Option 2: One or Multiple feedback bits for a single TB/codeword (e.g., codebook based) |
| CMCC |  | Generally ok and we are confused about the non-codebook example. |
| DCM | Y | We are OK with both, including Option 2. Meanwhile, NR CBG is completely not OK, due to many disadvantages such as increased overhead and complexity. For Option 2, NR CBG should not be the baseline and therefore we prefer to remove “(e.g., CB/CBG)”, which may imply NR CBG could be the baseline. |
| China Telecom |  | Thanks for the CATT’s clarification, we understand this proposal is based on TB-based feedback or CBG-based feedback. However, non-codebook based single feedback bit for a single TB/codeword is still confusing. |
| vivo |  | Support the proposal in principle. Agree with other companies that “non-codebook based” in Option 1 can be removed to avoid misleading, and the two options are not exclusive. |
| Lenovo |  | non-codebook is unclear to us. |
| xiaomi |  | We principally support the proposal with removing the “non-codebook based”. |
| Huawei, HiSilicon | Yes | Both options should be studied as both are supported in NR. However, we prefer lower HARQ-ACK feedback overhead. |
| Nokia |  | We support to study both options as non-exclusive. In option 1, “(e.g. non-codebook based)” is confusing and we suggest to remove it. |
| Panasonic |  | We share the views with above companies that inclusion of “non-codebook-based” is unclear. |
| NEC | Yes | Both two Options are fine. Besides, Single ACK/NACK bit for multiple TBs should also be kept as Option 3 for efficient feedback. |
| ETRI | Y | OK to study these aspects while minimizing the number of codebook type options, compared to NR, and maintaining necessary flexibility. |
| LGE |  | Same view as QC. Both options (TB‑based and CBG‑based operations) need to be studied. We also think that the terms “single feedback bit” and “(e.g., non‑codebook based)” in Option 1 may not be necessary. We can consider a TB‑based codebook. In addition, when operating HARQ‑ACK reporting, it may be necessary to transmit informations other than the A/N in order to support the diverse services proposed in 6G. Thus, it seems too restrictive to support only single feedback bit for a single TB at this stage. For a common understanding, the options can be rephrased as follows: Option 1: TB‑based HARQ‑ACK feedback Option 2: CB/CBG‑based HARQ‑ACK feedback |
| Fujitsu |  | We share same view with other companies that we need to both options for HARQ reporting (Option 1 and Option 2 are not exclusive). |
| CEWiT | Yes | We support the proposal |
| Ericsson1 |  | OK to study above options but they should perhaps be categorized something like Option1: “HAR-ACK per TB” and Option 2: “HARQ-ACK per CBs/CBGs in a TB”. |
| SONY |  | “non-codebook based” in Option 1 should be clarified. We support studying both the options in the proposal. |

[High Priority] Proposal 4-2 (I): Study whether and how to support the following HARQ-ACK reporting contents

- Option 1: Dynamic HARQ-ACK codebook for multiple TBs/codewords/PDSCHs/HARQ processes/carriers with or without ACK/NACK bundling

- Option 2: Semi-static HARQ-ACK codebook for multiple TBs/codewords/PDSCHs/HARQ processes/carriers with or without ACK/NACK bundling

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | What is the meaning of “dynamic” and “semi-static”? NR’s type 2 and Type 1? Is Type 3 considered as semi-static? How about enhanced Type 3? It is good to be clear about each option to avoid confusion down the road. |
| InterDigital |  | Agree with QC, this classification is a bit unclear. |
| Spreadtrum |  | Agree with QC, our understanding is Option 1 is for Type2, Option 2 is for Type1 and Type3. |
| OPPO |  | Our understanding is that option 1 is for type-2 while option 2 is for type-1 and type-3. As company pointed out, it should be clarified which option does enhanced type-3 belong to. |
| Samsung | Yes | We understand that option 1 and option 2 are not exclusive. The terms dynamic and semi-static should be defined to avoid any misunderstanding. For example, Type-3 HARQ ACK CB can be considered as semi-static, when the HARQ processes and carriers in CB are configured by RRC. However, enhanced Type-3 HARQ ACK CB with dynamic indication of a code point can be considered as dynamic. This needs further discussion and alignment among companies. |
| TCL |  | Ok with the proposal. |
| CATT |  | Prior to discussing this proposal, further clarification is needed on the definitions of the dynamic HARQ-ACK codebook and the semi-static HARQ-ACK codebook. For instance, is the enhanced Type 3 HARQ-ACK codebook in NR classified as a semi-static HARQ-ACK codebook? Additionally, is the new HARQ-ACK codebook we proposed—HARQ-ACK feedback without multiplexing HARQ-ACK corresponding to different DCIs- considered as a dynamic HARQ-ACK codebook? |
| ZTE | Yes | It is preferred to simplify the proposal with Option 1: Dynamic HARQ-ACK codebook and Option 2 semi-static HARQ-ACK codebook. As the multiple TBs already comprise multiple codewords, PDSCHs, HARQ process and carriers, the original version seems too redundant. |
| CMCC |  | Besides two options, NACK-only based HARQ codebook can also be studied, where HARQ bits (NACK bits) of partial TBs will be feedbacked. |
| DCM |  | Same view with QC. Option 2 should not mean NR type 1 as Nr type 1 is too complicated, not practical. We prefer to adopt NR Type-2 CB as baseline due to its dynamic size adaptation and efficient feedback mechanism, and NR type-3 kind of mechanism may also be possibility. |
| China Telecom |  | Maybe we need some example for the dynamic or static codebook, otherwise the proposal is still unclear. |
| vivo |  | Share similar views with other companies that the definition or meaning of dynamic/semi-static HARQ-ACK codebook should be clarified at first. |
| xiaomi | No | We suggest to list the options with NR legacy HARQ-ACK codebook types as baseline to avoid the ambiguous understandings. |
| Huawei, HiSilicon | Yes |  |
| Nokia |  | Share similar view with some companies above: the meaning of “dynamic” and “semi-static” should be clarified, especially in regard of Type 3 / enhanced Type 3 codebook. |
| Panasonic |  | Agree with Qualcomm. Our understanding is dynamic HARQ-ACK codebook is Type 2 in NR and semi-static codebook is Type 1 and Type 3 in NR. |
| NEC |  | Agree with QC to add the classification in this proposal. |
| ETRI | Y | OK to study these aspects while minimizing the number of codebook type options, compared to NR, and maintaining necessary flexibility. |
| LGE |  | Agree with QC and OPPO. Our understanding is also that option 1 is for type-2 while option 2 is for type-1 and type-3, and clarification seems necessary for e-Type-3 whether it belongs to option 1 or option 2. |
| Fujitsu |  | The proposal seems to introduce new HARQ-ACK codebook in 6GR. But we prefer to down-select the HARQ-ACK codebooks in 5G, and enhance it in 6GR. In this sense, it is preferable to explicitly describe such as “dynamic HARQ-ACK codebook defined in Type-2 in 5G” or “semi-static HARQ-ACK codebook defined in Type-1/Type-2 in 5G”. |
| CEWiT |  | We prefer the wordings to clarify QC’s concern on Type 1/2/3 before agreeing to the proposal. |
| Ericsson1 |  | In our view, it is not urgent to discuss such detailed aspects at this point of time. Prefer to converge a bit more on general principles in other proposals. |

[Medium Priority] Proposal 4-3 (I): Study whether and how to support the following

- NACK-only / ACK-only feedback (e.g., for multicast)

- HARQ-ACK for DCI not scheduling PDSCH (e.g., for SPS release)

- ACK/NACK for PDCCH (e.g., for DTX)

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC |  | We should start with problem statement. What are the problems we are trying to solve? For example, why multicast is mentioned above? Do we have multicast in the first release of 6GR? |
| InterDigital | Yes | We could add “XR” as example for the first one. |
| Spreadtrum |  | Same view as QC for multicast. |
| OPPO |  | It seems that the second sub-bullet is a sub-set of the third one, so we suggest to combine these two: Study whether and how to support the following - NACK-only / ACK-only feedback (e.g., for multicast) - HARQ-ACK for DCI not scheduling PDSCH (e.g., for SPS release) - ACK/NACK for PDCCH (e.g., for DTX, for DCI without scheduling PDSCH) |
| Samsung |  | Multicast is not part of the SI, so the first item should be removed. Support the second and third items. Regarding the “ACK/NACK for PDCCH”, we suggest to capture just “DTX for PDCCH” as, if any support, the mechanism is FFS. |
| CATT |  | We propose studying NACK-only triggered HARQ-ACK feedback, where the UE transmits HARQ-ACK feedback only when it needs to report at least a NACK. |
| ZTE | Yes | Support the study on NACK/ACK-only feedback and HARQ-ACK for PDCCH. Regarding the HARQ-ACK for PDCCH, it may comprise the DCI for release, dormancy indication, TCI update or BWP switching. While all above can be covered by HARQ-ACK for PDCCH(e.g., for DTX), DCI for BWP etc. For NACK/ACK only feedback, besides multicast, it is also beneficial for unicast services with SPS configuration. If an SPS configuration is configured for sparse DL transmissions, it is sufficient to provide only ACK for each transmission. If an SPS configuration is configured for dense DL transmissions, providing only NACK is sufficient. As a result, it can reduce the number/resource of HARQ-ACK feedback. The proposal can be updated as below: Study whether and how to support the following - NACK-only / ACK-only feedback - ACK/NACK for PDCCH |
| CMCC |  | Regarding the NACK-only feedback, it’s not related to MBS PDSCH, but the unicast PDSCH. As discussed in our companion Tdoc R1-2600400, since only 10% of HARQ feedback reports NACK, UE initiated NACK-only based HARQ feedback mode benefits in reducing reporting overhead and UE power consumption. Regarding the second and third sub-bullet, we also think they can be merged. We are fine with ZTE’s version |
| DCM |  | Same view with QC. For the last two, it is better to wait for discussion progress of the feature itself, e.g., what kind of DCI is supported, which may need feedback. |
| vivo |  | We are OK with the intention of the proposal. Share similar view with other companies that “multicast” in the first bullet is doubtable. |
| xiaomi | No | For the first bullet, in our view, the multicast is deprioritized in 6G first release, the scenario needs more clarification. For the second and third bullet, does the HARQ-ACK for DCI not scheduling PDSCH means unicast DCI, and the ACK/NACK for PDCCH means group common DCI? If it is the cases, we suggest to update it as: ACK/NACK for PDCCH(including unicast DCI and group common DCI) |
| Huawei, HiSilicon | Yes | We propose to include in the first sub-bullet that ACK-only feedback should also be considered for unicast. [Medium Priority] Proposal 4-3 (I): Study whether and how to support the following - NACK-only / ACK-only feedback (e.g., for unicast and multicast) - HARQ-ACK for DCI not scheduling PDSCH (e.g., for SPS release) - ACK/NACK for PDCCH (e.g., for DTX) We can also rewrite the second bullet as: HARQ-ACK for PDSCH without scheduling DCI |
| Nokia |  | The use case for the first item is not clear, so it should be removed. We are fine to study second and third items, but the mechanism should be FFS. Both second and third items could be combined as “feedback for PDCCH (e.g., for DTX and for DCI not scheduling PDSCH)”. |
| Panasonic |  | Agree to Qualcomm. |
| NEC |  | The contents in all brackets should be removed. |
| ETRI | Y | OK to study these aspects while minimizing the number of codebook type options, compared to NR, and maintaining necessary flexibility. |
| LGE |  | We share the view with QC. We should discuss and clarify what to solve first. |
| Fujitsu |  | We are ok to study, however, it might be better to remove the example use cases in the options to avoid unnecessary intentions. |
| SONY |  | For PDCCH, why are only limiting to ACK/NACK feedback? We can also still study NACK/ACK-only feedback (similar view as ZTE). |

## [ACTIVE] Topic #5: Other HARQ operating features

### Contribution summary

- Downlink assignment mis-detection issues: [2] [4] [7] [8] [11] [17] [21] [24]

PDCCH mis-detection

  - Study feedback techniques with reasonable overhead for PDCCH detection as part of the 6GR PDSCH HARQ feedback framework to limit the impact of missed DL assignments on the adaptive DL HARQ operation and to improve the DL control reliability, including:

    - NACK with multiple values to differentiate between PDSCH decoding failure and not detecting DL assignment

    - Indication of the number of detected PDCCH(s) or failed PDSCH decoding(s) for the reported HARQ-ACK codebook

  - In 6GR, study whether/how to indicate DCI missed detection for NACK information bits in a HARQ-ACK codebook.

HARQ-ACK codebook size mismatch

  - 6GR should study to address the HARQ-ACK CB size mismatch issue when one or more last DCIs are missing, especially in cases where this issue leads to severe consequences like Nack-to-Ack errors (as in the case of 1-to-2 error with PF0 and PF1 in NR).

No distinction between NACK and DTX

  - HARQ feedback mode for PDCCH aiming at differentiating NACK/DTX of PDSCH can be studied in 6GR HARQ design.

  - The gNB cannot distinguish between NACK and DTX. It is proposed that HARQ-ACK codebook to include dual feedback parts: a first part comprising ACK/DTX bits for DCIs and a second part comprising ACK/NACK bits for PDSCHs, as illustrated in below figure. This design enables the gNB to clearly differentiate between ACK and DTX, eliminating the ambiguity in ReTx scheduling decision-making.

  - For 6GR HARQ-ACK codebook generation, study UE reporting of DTX with multiple HARQ-ACK information bits.

- AI services or related traffic: [3] [4] [5] [10] [16] [25] [30]

  - AI-based traffic represents another typical service type. Due to the fact that different AI tasks are associated with dynamically varying traffic characteristics, along with divergent latency and data rate requirements, this introduces new challenges in resource management and control signaling. A notable AI services is the transmission of tokenized data over mobile networks, which is characterized by error tolerant data traffic and highly variable QoS requirement.

  - The traffic characteristics of AI/ML service are analyzed, including error tolerance, priority differentiation, as below

    - Error tolerance: the data for AI/ML model processing are resilient to a range of error, e.g., 10~20% loss, due to the generative or predictive capability of AI/ML models.

    - Priority differentiation: Different data may have different error tolerance capabilities. Data have dependency, e.g., the loss of preceding data for an image would cause higher reconstruction error than the subsequent data.

| UE number per cell |  | 5 | 15 | 25 | 35 | 45 | 55 | 65 | 75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ratio of satisfied UE number/overall UE number (%) | Benchmark HARQ solution | 98.8 | 95.2 | 84.8 | 77.7 | 68 | 60.5 | 51.6 | 49.4 |
|  | Enhanced HARQ solution | 100 | 98.8 | 93.6 | 88.6 | 82 | 75.4 | 67.6 | 66 |
| Gain (%) |  | 1.2 | 3.8 | 10.4 | 14 | 20.6 | 24.6 | 31 | 33.6 |

  - The new traffic characteristics of AI and immersive services, including error tolerance and priority differentiation, provide new opportunities for improved user experience with enhancement of HARQ design. Study HARQ design to accommodate the error tolerance and data traffic with different characteristics, e.g., different priorities.

  - A single TB can consist of one or more traffics, and each traffic can have different characteristics. For example, a certain traffic (such as XR traffic) within a single TB may have more stringent reliability or latency requirements than other traffic (such as eMBB traffic). Also, in AI token traffic, each token may have different characteristics (e.g., error tolerance, reliability, etc).

Proposal: Study enhancements for selecting CBs for CBG formation in 6GR, considering channel quality or traffic characteristics.

Proposal: Study selective HARQ feedback/retransmission considering traffic-awareness in 6GR.

- Cross-layer HARQ and ARQ operations: [3] [5] [10] [17] [22] [24]

  - Observations: In NR, RLC ARQ will only perform retransmissions after receiving the corresponding status report. The minimum delay in obtaining it, after the last HARQ transmission, is at least the time required to send a PDU via HARQ. However, if the UE or gNB can determine that the HARQ process for a given TB has failed, such as after exhausting all HARQ transmission attempts, it can immediately trigger an ARQ retransmission for that TB. This eliminates the need to wait for the status report, thereby reducing overall latency. Especially it was agreed to study mechanisms for faster ARQ retransmissions at the transmitter side based on HARQ process status in RAN2.

  - On the one hand, the separation between HARQ and higher-layer ARQ creates redundancy overhead, thus more efficient retransmission mechanism through tighter inter-layer coordination should be studied to achieve energy saving and high efficiency; On the other hand, ARQ in NR is based on event-triggered or timer expire-driven status report, resulting in long retransmission delays and significantly reduced user experience. Faster ARQ retransmission based on inter-layer coordination should be considered in 6GR.

  - In 5G NR, in case when a NACK-to-ACK error occurs for a certain PDSCH transmission, recovery of the error at the PHY layer is not possible. As a result, the retransmission is handled at the RLC layer based on ARQ operation, which can incur significantly increased end to end latency as well as larger buffer size requirements at the UE and the gNB.

  - NACK-to-ACK error in DL cannot be currently recovered by lower layers in NR. While it may not be practical to make N2A errors small enough, it is possible to detect and report this error event via PHY / MAC layers by tighter interactions between HARQ and ARQ operations instead of solely relying on higher-layer SN-based hole detection mechanisms as in RLC-AM in NR.

  - Proposals: Study faster ARQ retransmission based on PHY assistance to reduce data transmission delay and improve user experience.

  - Study PHY layer UL feedback of NACK-to-ACK events in 6GR for improving DL traffic latency and UE buffer implementation.

  - 6GR to support UE detecting and reporting unsuccessful HARQ termination events at TB level for DL for fast recovery from NACK-to-ACK errors.

- Unknown TBS with reserved MCS usage issue: [10] [11] [24]

  - 6GR should address the unknown TBS issue when reserved MCS is used for HARQ retransmission while DCI scheduling initial transmission is missed.

    - Study low-overhead HARQ-ACK feedback enhancements to solve this issue so that UE can indicate whether TBS is unknown when reserved MCS is used.

- Two-stage UCI transmission: [6] [21] [30]

  - Proposal: In NR Rel-19, UEIRI is supported for UE-triggered beam report without gNB scheduling, which helps reduce signaling overhead and improves UE energy saving. Similarly, a 2-step UCI mechanism can be considered in 6GR. This mechanism extends the scope of UE-triggered reporting to more UCI types, such as BSR, NACK triggered HARQ-ACK feedback, RI feedback, CLI report and others. As shown in below figure, by step-1 UCI, UE reports small number of UCI bits to indicate whether there is step-2 UCI and also indicates the type of step-2 UCI. When UE wrongly decodes the PDSCH and needs to report NACK, the UE first reports step-1 UCI to indicate the following reporting of HARQ-ACK codebook. This improves resource utilization and UE power saving through on-demand UCI transmission.

  - Proposal:  The first stage UCI transmission has a smaller UCI size to reduce resources. The second stage UCI transmission is an optional transmission based on the first stage UCI value. From a statistical perspective, the second level UCI does not need to be transmitted in most cases. In this way, after receiving the first stage UCI transmission, the network can recycle and reuse the resources of the second stage UCI transmission to save resources. For example, the first-stage HARQ-ACK is 1 bit, which is obtained through an "AND" operation of the TB based HARQ-ACK codebook. The second-stage HARQ-ACK is a TB based HARQ-ACK codebook, and it is only transmitted when the first-stage HARQ-ACK is a NACK. Therefore, the resources for the second-stage HARQ-ACK can be dynamically recycled after the first-stage HARQ-ACK is received as an ACK.

  - Proposal: To reduce the effective number of HARQ feedback bits, one portion of the HARQ feedback indicates the number of ACK in the bitmap, and another portion of the HARQ feedback indicates the sequence out of the possible bitmap sequence containing the given number of ACKs. A two - stage signaling of HARQ feedback as UCI could be used for this purpose.

- PUCCH overriding issue: [9] [24]

  - When the last one or more DCIs are missing, PUCCH overriding mechanism in NR will lead to ambiguity on

    - which PUCCH resource to use for UCI multiplexing, and

    - which PUSCH resource to use for HARQ-ACK multiplexing when the last one or more DCIs are missing.

  - Proposal: 6GR aims to avoid repeatedly determining a PUCCH resource for reporting HARQ-ACK corresponding to multiple DCIs received continuously

    - Option 1: The UE always determines a HARQ-ACK codebook and corresponding PUCCH resource in a UL slot at the last moment, e.g. determined by applying a predefined time offset from the starting boundary of the UL slot.

    - Option 2: The gNB indicates a last DCI for a UL slot, which triggers the UE to determines a HARQ-ACK codebook and corresponding PUCCH resource in the UL slot.

  - Proposal: Study management of HARQ-ACK feedback resources for 6GR to achieve enough flexibility and avoid excessive complexity, e.g. consider solutions to avoid determining a HARQ-ACK feedback resource implicitly based on HARQ-ACK payload.

  - Proposal: Study mechanisms on how to avoid PUCCH overriding to resolve the ambiguity issue on which PUCCH resource to use for HARQ-ACK transmission or UCI multiplexing.

- Fast HARQ-ACK feedback: [4] [10]

  - Reducing HARQ-ACK feedback latency can significantly improve the system performance, and reduce the OLLA convergence time, enabling more time domain opportunities for BS/UE energy saving.

  - Consider using SBFD, simplified dynamic TDD, and dynamic PUCCH carrier switching among all available UL carriers, in order to achieve fast HARQ-ACK feedback.

| Figure: Performance gain from fast HARQ in non-full buffer scenario. | Figure: Performance gain from fast HARQ in full buffer scenario. |
| --- | --- |

  - In 6GR, URLLC requirements are expected to be comparable to or more stringent than those in 5G NR from Day-1, and thus to meet such requirements, it is essential to ensure high DL resource efficiency as well as low-latency and highly-reliable HARQ-ACK feedback. Therefore, HARQ-ACK codebook retransmission and dynamic PUCCH cell switching should be considered as essential features in 6GR.

- HARQ feedback for MIMO operation: [13] [16]

  - In NR, the mTRP operation is based on one HARQ entity. However, for non-ideal backhaul scenario, e.g., multi-DCI based mTRP, the initial transmission and retransmission of a TB is usually from the same TRP. From the network side, network can guarantee the initial transmission and retransmission for a TB is always from the same TRP based on its own implementation. But the UE may need to implement the cross-TRP retransmission, since from the specification point of view, the HARQ processes are share among the TRPs. Such HARQ process sharing also requires unnecessary DCI overhead for the HARQ process number indication. Thus, for mTRP operation, it is necessary to study the TRP-specific HARQ processes.

Proposal: Study the TRP-specific HARQ processes pool for the multi-DCI based multi-TRP operation.

  - HARQ-ACK corresponding to different PDSCH transmissions scheduled by different DCIs from different TRPs can be jointly or separately fed back. In 6GR, both multiple DCI and single DCI based scheduling schemes can be supported with more than two TRPs for the distributed MIMO or cell-free MIMO. Moreover, there may be different coordination capability for backhaul between TRPs given the flexible network deployment. One potential use case is multi-DCI based multi-TRP operation with more than two TRPs and each of the DCI may schedule one or more PDSCH with more than one CWs. How to support the HARQ-ACK feedback with different coordination capability should be considered. For HARQ-ACK codebook, the design can be made with considering the tradeoff between DL throughput/scheduling efficiency and UCI overhead, where separated and/or joint HARQ-ACK codebook may be used. And, the bundling or multiplexing schemes can be further considered for feedback bits associated with different TRPs.

Proposal: Study the HARQ feedback mechanism for multi-TRP deployment.

- NDI toggling issue: [24]

  - HARQ retransmission and initial transmission can be mixed up when all DCIs scheduling initial transmission and retransmissions of a given TB are missed. Such issues have been observed in the field especially in the presence of DRX operation.

Proposal: 6GR should study to address the reliability issue with 1-bit NDI toggling.

    - A potential solution is to increase the NDI bit-width to 2 bits.

- Link adaptation to mitigate improper MCS assignment: [4]

  - Different portions of encoded bits from multiple transmissions could be transmitted sequentially to improve link adaptation, as discussed in Section 3.2.4 of R1-2600154, by which the effective code rate of this transmitted block can automatically adapt to instantaneous channel conditions, which could help harvest the channel capacity that would otherwise be wasted due to conservative or aggressive MCS assignment in conventional link adaptation. It can be observed in the following simulation curves that the potential improvement for a HARQ scheme for better link adaptation is significant, with around 40% gain for optimal MCS assignment scheme over traditional HARQ in some of the SNR range (e.g., at SNR=0-5dB).

- HARQ retransmission based on TX repetition: [22]

  - NR provides mechanisms to increase repetition in response to decoding failures, but does not provide explicit feedback-driven mechanisms to support repetition reduction for throughput optimization. In the worst case, once repetition is increased, there is no direct indication to trigger repetition reduction based on decoding efficiency. As a result, excessive repetitions may remain undetected, leading to unnecessary throughput loss. This motivates further study on enhanced repetition control mechanisms that can detect and mitigate excessive repetitions under 6GR assumptions.

Proposal: Study enhanced repetition control mechanisms under 6GR assumptions with following considerations.

    - Study mechanisms enabling accurate and efficient repetition adaptation for avoiding both over-repetition and under-repetition.

    - Study whether additional feedback information beyond ACK/NACK could enhance repetition control efficiency (including whether it could detect and mitigate excessive repetitions).

- HARQ feedback without soft combining: [26]

  - Due to long RTT in NTN, increased number of HARQ processes and configurable HARQ feedback disabling are supported. Although HARQ feedback disabling is one option, it can cause frequent RLC retransmissions, which causes significant overhead and delay. On the other hand, having the soft buffer for all slots in the RTT up to e.g., 540 slots would pose undesired UE complexity. Therefore, HARQ feedback with and without soft-combining (without storing the received data in the soft buffer) to allow MAC retransmission should be studied.

Proposal: HARQ feedback with and without soft-combining (without storing the received data in the soft buffer) to allow MAC retransmission should be studied.

### FL Proposals for Round 1 discussion

[Medium Priority] Proposal 5-1 (I): Study whether and how to support cross layer HARQ and ARQ integration

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC | Yes | Also, suggest listing aspects specific to RAN1 such as whether/how to address N2A error at lower layers. To FL: The discussion in our paper for NDI toggling issue is focusing on HARQ operation (not ARQ necessarily, even though there can be secondary aspects for ARQ). In addition, we discussed the issue of reserved MCS for HARQ ReTx, which is also not related to ARQ. It would be good if these issues (which are actually observed in the field) can be captured separate from the ARQ discussions. |
| InterDigital |  | Cross-layer HARQ and ARQ is discussed by RAN2. At this stage we should focus on studying how to increase reliability of HARQ feedback and mitigating HARQ feedback misinterpretation. |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Samsung |  | Wait for RAN2 progress |
| TCL | Yes |  |
| CATT |  | Further discussions are needed on the rationale for supporting cross-layer HARQ and ARQ integration. |
| ZTE |  | Maybe the intention is whether/how to support efficient ARQ operation due to ‘NACK to ACK’ error in L1 DL/UL transmission. It is not preferred to study cross layer HARQ and ARQ integration due to the complexity operation/interaction across L1 and higher layer. While, we are open to study how to resolve the RAN1 issue related to the issue of ARQ operation identified in RAN2. Note, the study should be based on the LS from RAN2 if any. |
| CMCC | Yes | Wait for RAN2 |
| DCM | Y | Both DL and UL, right? |
| China Telecom |  | Similar to InterDigital, Samsung and CMCC. |
| vivo |  | We prefer to focus on RAN1 issues firstly, and wait for RAN2 progress. |
| Lenovo |  | Wait for RAN2. |
| xiaomi |  | Wait for RAN2 progress and identify the RAN1 impact. |
| Huawei, HiSilicon | Yes | It should be noted though that the main benefit of introducing a receiver-side HARQ process status indication is to facilitate fast triggering of a L2 ARQ retransmission, possibly after failing to receive a number of HARQ retransmissions. Therefore, on the question of How, if such HARQ Process status is provided by L1 to the transmitter side, then it should be indicated by a DCI for the UL transmission case, and by UCI for the DL transmission case. In line with the RAN2 agreement, the UL transmission case should be addressed with priority given that the existing HARQ-ACK feedback is only implicit via a DCI scheduling the same HARQ process ID, which may lead to ambiguity as to which TB/MAC PDU is intended under DCI misdetection. Therefore, as starting point, RAN1 should study the pros and cons of indicating HARQ Process Status for one or multiple HARQ processes to facilitate fast ARQ retransmissions. |
| Nokia |  | It is not clear what to study in RAN1 at this stage. |
| Panasonic | Yes |  |
| MediaTek | Yes |  |
| ETRI | Yes | This could be beneficial for enabling a unified adaptation framework across diverse scenarios. |
| LGE | Yes | In our view, it is fine to study how the physical layer can help with ARQ retransmissions, especially for N2A errors. Physical layer support to handle NACK-to-ACK error may reduce ARQ retransmission latency and ensure faster data recovery. |
| CEWiT | Yes |  |
| SONY |  | Wait RAN2 progress. |

Moderator assessment: For the following proposal, it is observed in contributions that proposals for AI and XR related services and traffics are provided, but lacking of details. Hence, companies are encouraged to provide further technical details and information.

[Medium Priority] Proposal 5-2 (I): Study whether and how to support HARQ operation in 6GR for other services / traffic, e.g., AI related, immersive service

| Company | Yes / No | Comments |
| --- | --- | --- |
| QC | No | AI traffic is under discussion in SA and RAN2. It seems no need for RAN1 to start discussing traffic types now. This discussion can happen later after RAN2 characterizes the properties and identifies the need for specific handling. |
| InterDigital |  | These use cases should be taken into account in the general design. |
| Spreadtrum |  | Same view as InterDigital. |
| Samsung | No | A need/applicability for separate solutions than for typical eMBB should be first identified/agreed. |
| CATT | No |  |
| ZTE |  | Prefer to study as low priority. If there are significant requirements of other services/traffic compared with current services/traffic (e.g., eMBB, URLLC, MBS, NTN, XR, IoT, etc.), it can be studied whether/how to support HARQ operation in 6GR. Considering the requirements of new/other services/traffics are not clear as this stage, it is preferred to wait the studies from other RAN sub-groups. |
| DCM | N | What is the expected RAN1 work is unclear. |
| China Telecom | N | We think we should wait for the corresponding’s conclusion and then discuss this proposal, not now. |
| vivo |  | Share similar views as other companies that new services, as well as corresponding characteristics & requirements, should be discussed in other agendas at first. |
| xiaomi | No | We share same view with QC. |
| Huawei, HiSilicon | Yes | It is important to take into consideration the specific nature of new traffic in 6GR, e.g., AI related traffic, due to its different characteristics. Study whether and how to enhance HARQ operation in 6GR for other services / traffic, e.g., AI related, immersive service Such services are already supported, so anyway HARQ operation should be supported and the discussion point is about the additional enhancements dedicated to such services. |
| Nokia | No | Perhaps later, after a need is identified. |
| Panasonic |  | Same view as InterDigital. |
| NEC | Yes | We should not close the door in the initiation stage for adopting HARQ operation for some new services in 6G, as the benefit is existing indeed. |
| ETRI |  | Rather than treating AI/XR as isolated features, ETRI suggests integrating them into a unified adaptation framework, including joint consideration of repetition control and HARQ retransmissions |
| OPPO | Yes | First, according to the above Proposal 1-1, diverse service/applications should be taken into account for HARQ design, so the AI traffic as well as immersive service should not be an exception. In addition, SA4 and RAN2 has already started to study these kind of traffic types and eventually it may have some impacts on HARQ design. So it would be good for this agenda to also take these traffic characteristics into consideration as early as possible. |
| LGE | Yes | A variety of services and traffic types are expected to be introduced in 6G. In addition, each service or traffic type has different characteristics —for example, XR traffic (real‑time operation, massive data transmission), AI token traffic (different characteristic according to diverse token), and eMBB traffic. These characteristics lead to different requirements in terms of error tolerance, reliability, priority, and so on. For instance, real-time or broadcast services place higher importance on achieving reliability in the initial transmission, and therefore HARQ retransmission may be less critical for them compared with other services. Therefore, the design of HARQ operation should be carefully considered to accommodate such diverse services and traffic types. |
| CEWiT | No | We believe this can be taken care later after progress in 1-1 |
| SONY |  | Other means other than eMBB? It is unclear. |

## [CLOSED] Topic #6: Others

### Contribution summary

- Handling of Scheduling Requests (SRs): [11] [21] [23] [28]

  - The sequential SR+BSR+PUSCH transmission procedure introduces unavoidable delay, making it unsuitable for 6G latency-sensitive services. A more viable and forward-looking approach is to enhance the SR so that it carries additional information and functions as a unified mechanism. In this design, an enhanced SR would combine the traditional ‘resource request’ role of SR with the ‘buffer status reporting’ capability of BSR, allowing the base station to perform immediate and informed UL scheduling based on a single control transmission. In our view, the most straightforward realization of this concept is to introduce a multi-bits SR scheme, with such an approach, the UE can deliver a coarse-granularity buffer indication early, enabling the base station to trigger timely data scheduling, followed by a second, more accurate BSR to refine the resource allocation.

  - In 6GR, study fast reporting of SR/BSR. Either SR enhancement on PUCCH or BSR enhancement on contention based PUSCH can be considered. In 6GR, study methods for improving SR resource utilization, such as cluster SR resource configuration, or SR resource configuration adaptation.

  - Means to reduce the delay from data appearing in the UE buffer to the actual transmission of data with low overhead is therefore of interest to study in 6G. Using contention-based resources to send muti-bit SRs or BSRs could be a fast and low overhead approach to speed up the SR->grant1->BSR->grant2->UL data timeline to a faster SR/BSR->grant->UL data approach.

Figure: Illustration of the SR-grant-BSR-grant cycle in 5G.

  - PUCCH seems to be straightforward, in terms of capacity, reliability, and the motivation to transmit SR. In other words, when PUSCH is available, BSR can be transmitted instead of SR and SR transmission on PUSCH is meaningless.

- Handling of channel state information reporting (CSI): [28]

  - DCM [28]: For CSI: There is less motivation to use PUCCH as payload size is typically larger than HARQ-ACK/SR and CDM is not so beneficial as abovementioned. It is not well justified to prioritize PUCCH but deprioritize PUSCH.

- UCI on PUSCH and simultaneous PUCCH/PUSCH transmission: [21]

  - In 6GR, UCI on PUSCH transmission should be extended to support UCI of unknown size based on potential different generation modes. The transmission resources of the UCI are determined by network, and the UE can decide to transmit the UCI based on a certain generation mode according to its actual requirement, such as some information used to restore/enhance transmission performance. In this way, the time-frequency resources of the cell can still be scheduled by the network, and the UE could generate flexible size of UCI to improve system efficiency. For example, UCI of unknown size may comprise the mixed TB/CBG based HARQ-ACK codebook. Different generation modes may include AI based and non-AI based UCI generation. Therefore, RAN1 should study how to transmit UCI of unknown size or generation mode based on UCI on PUSCH in PHY. The size/generation mode of UCI transmitted by UE can be notified to the network with a small amount of indication information, which can be independently encoded and transmitted. That is to say, if the UE decides to report a UCI of unknown size/generation mode for the network, the indication information and the UCI need to be independently encoded and transmitted in the PHY based on the UCI on PUSCH.

  - In 6GR, besides inter-band CA, simultaneous transmission between PUCCH and PUSCH for a UE can be studied, including specific UCI type(s) transmitted on PUCCH or PUSCH, requirements on DMRS design, low-PAPR techniques and power control between PUCCH and PUSCH.

- Unused transmission occasion: [21]

  - In NR, the reception of unused transmission occasion UTO-UCI requires the network to be awakened, as UTO-UCI needs to be decoded. In 6GR, some enhancements can be studied for network energy saving, such as support of signals including UTO-UCI that can be aware by the network in micro sleep mode.

- UL scheduling timing: [3]

  - NR UL scheduling timing indication can be baseline for 6GR, i.e. UL scheduling timing is determined by the slot offset K2 in TDRA, between PDCCH and the first PUSCH reception.

  - For PUSCH preparation time in 6GR, it has:

    - One basic PUSCH processing time for a device type

    - Differentiated PUSCH processing time for diverse device Types

    - Further study additional relaxation considering large bandwidth

  - Study CB-PUSCH transmission for BSR/DSR, from RAN1 perspective.

### FL Proposals for Round 1 discussion

Moderator assessment: It is currently unclear whether to handle in this agenda topics related to fast SR/BSR reporting, CSI reporting over PUSCH, UCI on PUSCH, simultaneous PUCCH/PUSCH transmissions, unused transmission occasion, and UL scheduling timing.

[Low-Priority] Question 6-1 (I): Whether to study in this agenda the support of

- fast SR/BSR reporting

- CSI reporting over PUSCH

- UCI on PUSCH

- simultaneous PUCCH/PUSCH transmissions

- unused transmission occasion

- UL scheduling timing in this agenda

| Company | Comments |
| --- | --- |
| QC | If UCI includes HARQ-ACK, ok to discuss under this agenda. Otherwise (SR/BSR/CSI specifics) do not belong to this agenda. |
| Spreadtrum | Our understanding is - SR/BSR is in 10.5.5 - CSI is in 10.5.3 - Yes for UCI on PUSCH - Simultaneous PUCCH/PUSCH has a separate agenda. - PUSCH preparation time, such as one capability or multiple, can be in this agenda. Due to it is similar has PDSCH processing capability. - UL scheduling timing can be in other agenda, not sure in 10.5.4.1 or 10.5.2.3. |
| OPPO | Our understanding is: UCI on PUSCH is yes and others is no. One more clarification question: does PUCCH format and PUCCH resource allocation related issue should be discussed in this agenda item? |
| Samsung | Some, like UCI on PUSCH, touch on “HARQ related aspects” but most do not relate to this agenda. |
| CATT | We need to study the HARQ-ACK on the PUSCH in this agenda. |
| ZTE | We are open to study the issues if they are related to HARQ operation or HARQ-ACK feedback. For SR/BSR/CSI reporting, simultaneous PUCCH/PUSCH transmissions and UL scheduling timing, it depends on the arrangement of chairman/FL whether these issues can be discussed in this agenda. For UCI on PUSCH, especially for HARQ-ACK multiplexing on PUSCH, it can be discussed in this agenda. For the unused transmission occasion, it can also be discussed in this agenda since the UL feedback may be required for the indication of unused transmission occasions. |
| DCM | Any agenda is OK, as long as it is clarified and companies can have common understanding. |
| vivo | UCI on PUSCH can be discussed in this agenda. |
| Lenovo | We think simultaneous PUCCH/PUSCH transmissions should not be discussed here, only how to generate HARQ-ack codebook and how to transmit HARQ-ack should be discussed here. |
| xiaomi | We think HARQ-ACK on PUSCH is included in the agenda. |
| Huawei, HiSilicon | We can include the study of fast SR/BSR reporting. The other options might be related to other agenda item’s progress and we think we can postpone the discussion to a later point in time if needed. |
| Nokia | It depends where HARQ-ACK is transmitted, but UCI on PUSCH and UL scheduling may be affected. |
| Panasonic | Agree to Qualcomm. |
| ETRI | We are open to study items related to adaptation (e.g., UCI on PUSCH, CSI reporting over PUSCH). 6GR’s new feedback related to adaptation (repetition, HARQ, ARQ) could be also considered at this agenda |

| LGE | We also think that UCI including HARQ-ACK on PUSCH can be studied either in this agenda or in 10.5.2.3, but not for other topics. (for example, the fast SR/BSR reporting is more appropriate to be discussed in 10.5.5.) |
| --- | --- |

- References

- RP-253876	Revised SID: Study on 6G Radio	NTT DOCOMO, CMCC, AT&T, Vodafone

- R1-2600043	Views on HARQ related aspects in 6GR	Nokia

- R1-2600123	Discussion on 6GR HARQ related aspects	Spreadtrum, UNISOC

- R1-2600155	HARQ related aspects for 6GR	Huawei, HiSilicon

- R1-2600205	Discussion on HARQ for 6GR	OPPO

- R1-2600310	Discussion on HARQ related aspects for 6GR	CATT

- R1-2600400	Discussion on 6GR HARQ design	CMCC

- R1-2600440	Discussion on 6G HARQ related aspects	Xiaomi

- R1-2600515	Discussion on 6GR HARQ related aspects	vivo

- R1-2600555	Discussion on HARQ related aspects for 6GR	LG Electronics

- R1-2600576	Discussion on HARQ related aspects	TCL

- R1-2600616	HARQ related aspects	InterDigital, Inc.

- R1-2600635	HARQ Related Aspects	Google

- R1-2600653	Discussion on HARQ related operation	NEC

- R1-2600703	Discussion on HARQ related aspects	China Telecom

- R1-2600714	HARQ for 6GR	Lenovo

- R1-2600767	On 6GR HARQ Related Aspects	Samsung

- R1-2600839	On HARQ related aspects	Apple

- R1-2600856	Discussion on HARQ related aspects	Fujitsu

- R1-2600926	Discussion on HARQ related Aspects for 6GR air interface	Sharp

- R1-2600977	Discussion on HARQ related aspects for 6GR	ZTE Corporation, Sanechips

- R1-2601013	Discussion on HARQ related Aspects	ETRI

- R1-2601048	Scheduling and HARQ operation for 6GR	Ericsson

- R1-2601099	HARQ related aspects	Qualcomm Incorporated

- R1-2601105	HARQ related aspects	Ofinno

- R1-2601116	Discussion on HARQ related aspects for 6GR	Panasonic

- R1-2601139	Discussion on 6G HARQ Related Aspects	Sony

- R1-2601193	Discussion on HARQ-related aspects for 6GR	NTT DOCOMO, INC.

- R1-2601359	Discussion on HARQ related aspects in 6GR	MediaTek Inc.

- R1-2601401	Discussion on HARQ related Aspects	CEWiT

Contact information

The contact information below is collected during RAN1#124 meeting. Companies are requested to update this information, if required.

| Company | Delegate name(s) | Email address(es) |
| --- | --- | --- |
| OPPO | Yi Zhang Kevin Lin | zhangy@oppo.com kevin.lin@oppo.com |
| Spreadtrum | Huan Zhou | Huan.zhou@unisoc.com |
| FUTUREWEI | Brian Classon | brian@classonconsulting.com |
| CMCC | Tuo Yang | yangtuo@chinamobile.com |
| NTT DOCOMO | Shohei Yoshioka You Luhua Huiting Cheng | syouhei.yoshioka.py@nttdocomo.com youlh@docomolabs-beijing.com.cn huiting.cheng.pc@nttdocomo.com |
| China Telecom | Yi Gu | guy6@chinatelecom.cn |
| Lenovo | Ruixiang | Marx6@lenovo.com |
| MediaTek | Mohammed Al-Imari | Mohammed.al-imari@mediatek.com |
| NEC | Chen Wei | chen_wei@nec.cn |
| ETRI | Dukhyun You Cheulsoon Kim | dhyou@etri.re.kr cs.kim@etri.re.kr |
| LG Electronics | Sukchel Yang Inho Ha Hyangsun You | suckchel.yang@lge.com Inho.ha@lge.com sssun.you@lge.com |
| SONY | Basuki Priyanto | basuki.priyanto@sony.com |

Appendix (outcomes of past meetings)

## RAN1#124 (09 – 13 February 2026)

To be collected at the end of meeting.