3GPP TSG-RAN1 Meeting #124	R1-2601730

# Gothenburg, SE, Feb. 9th – 13st, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.769 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Inclusion of outcomes of Study on enhancements for solutions for Ambient IoT in NR outdoor for active devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Huawei, Ericsson, Qualcomm, Apple, Vivo, Xiaomi, CATT, CMCC, Sony, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Ambient_IoT_Outdoor_Active |  |  |  |  |  | Date: |  |  | 2026-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The TR needs to include the outcomes of the Rel-20 SI on enhancements for solutions for Ambient IoT in NR outdoor for active devices |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreements made to-date are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SI outcomes are unreported in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6A.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Endorsed by RAN4 in R4-2602939; Revision of R4-2602927 (first draft: R4-2602114) |  |  |  |  |  |  |  |  |

==============Start of change==============

<Unchanged parts omitted>

## 6A.z	Coexistence of ambient IoT and NR/LTE

### 6A.z.1	Co-existence scenarios and cases

The coexistence evaluation is conducted considering the different scenarios listed in Table 6A.z.1-1

Table 6A.z.1-1: Co-existence scenarios

| Operation mode | Device type | Evaluation case | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | NR DL | device | R2D |
| In-band | Device 2b | 8 | NR UL | reader | D2R |
|  |  | 9 | NR DL | device | R2D |
|  | Device C | 10 | NR UL | reader | D2R |
|  |  | 11 | NR DL | device | R2D |

### 6A.z.2	Co-existence evaluation assumptions

#### 6A.z.2.1	Deployment

Simulation assumptions related to network layout is captured for D4T1 in Table 6A.z.2.1-1.

Table 6A.z.2.1-1. Deployment parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900MHz (Band 8) |
| BW for NR | 10MHz with 15KHz SCS |
| R2D BW | 180kHz, for 15kHz SCS |
| D2R BW | 15kHz |
| Waveform | R2D: OFDM-based OOK D2R: BPSK |
| A-IoT DL power control | No |
| A-IoT UL power control | Option 1: Use fixed Tx power without power control - Device 2b :-10dBm - Device C: 5dBm Option 2: Use power control as following equation - Company report CLx-ile, PMax and Rmin - Target SNR:15dB(starting point) or other values to be reported |
| Traffic model | Ful buffer |
| NR BS/A-IoT reader deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS/A-IoT reader distance: [X]m - X= 750m for device 2b/C NR BS height: 25m the distance between macro NR BS/A-IoT reader and device 2b/C as 50-500 m |
| Frequency reuse | Frequency reuse - 3(Baseline) - other options are not precluded |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area - 100% outdoor 0 % in cargo trucks (baseline) - Not preclude to provide simulation result in X% in cargo trucks - For example, X=80 Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 35m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| MCL for NR BS to NR UE | MCL of 70 dB |
| Pathloss model | Use UMa pathloss model defined in TR38.901 |
| penetration loss | 20dB |
| Shadow fading margin | 6dB for UMa NLOS |

#### 6A.z.2.2	NR BS/ A-IoT reader RF characteristics

The NR BS and A-IoT reader are defined for two different antenna configurations as illustrated in Table 6A.z.2.2-1 and Table 6A.z.2.2-2.

Table 6A.z.2.2-1: NR BS RF parameters

| NR BS Parameter | Value |
| --- | --- |
| Macro-BS Tx power (dBm) | 46dBm |
| BS antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |
| BS antenna gain (dBi) | 15dBi including feeder loss |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | Outdoor |
| ACLR (in-band) | 30 dBc |
| ACLR (standalone) | Same with legacy NR BS |
| ACS (in-band) | No interference caused by ACS impact |
| ACS(standalone) | Same with legacy NR BS |

Table 6A.z.2.2-2: A-IoT reader RF parameters

| A-IoT reader parameters |  | Value |
| --- | --- | --- |
| A-IoT reader total Tx power | Standalone | 43dBm |
|  | In-band | 38dBm |
| A-IoT reader receiver Noise Figure (dB) |  | 5dB |
| A-IoT reader antenna gain (dBi) |  | 15dBi including feeder loss |
| Antenna configuration |  | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |

#### 6A.z.2.3	NR UE/ A-IoT device RF characteristics

Assumptions relevant for modelling the NR UE and A-IoT device RF characteristics are captured in Table 6A.z.2.3-1 and Table 6A.z.2.3-2.

Table 6A.z.2.3-1: NR UE RF parameters

| NR UE Parameter | Value |
| --- | --- |
| UE TX power in dBm | -40 to 23 dBm |
| NR UE Antenna gain (dBi) | 0dBi |
| Height of UE antenna (m) | 1.5m |
| NR UE Tx emission（dB） (standalone) | Same with NR UE ACLR |
| NR UE Tx emission (dB) (inband) | Same with NR UE IBE |
| NR UE ACS（dB） (Standalone) | 33dB |
| NR UE ACS（dB） (In-band) | No interference caused by ACS impact |
| NR UE Noise Figure（dB） | 9dB |
| Antenna configuration | Omni direction antenna |

Table 6A.z.2.3-2: A-IoT device RF parameters

| A-IoT device parameters | Value |  |
| --- | --- | --- |
|  | Device 2b | Device C |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device max output power（dBm） | -10 | 5 |
| A-IoT device noise figure (dB) | 15 | 13 |
| Guard band | 0 kHz | 0 kHz |
| Antenna configuration | Omni direction antenna | Omni direction antenna |

### 6A.z.3	Co-existence simulation methodology

#### 6A.z.3.1a Coexistence evaluation methodology 1

The coexistence evaluation methodology can be summarized as:

1) Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader based on coupling loss.

a)	The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other.

b)	The device can access the reader with best receiving power among all readers.

3)	Once association is done, round robin scheduling is used.

4)	For inter-system interference (between A-IoT and NR):

-	When AIoT as victim, SINR with ACI define as following:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

The noise and intra/inter-system interference are within the wanted signal bandwidth.

The SINR correspoinding to the 5 percentile and 50 percentile point of the SINR CDF curve with ACI are obtained. If the SINR is above the decoding threshold, it can be considered that inter-system interference is negligible.

-	When NR as victim, if throughput loss is smaller than and equal to 5%, it can be considered that inter-system interference is negligible.

5) For A-IoT as victim, collect the data of SINR with and without ACI.

6) Repeat Step 1-5 for different ACIR values.

#### 6A.z.3.1b	Coexistence evaluation methodology 2

The simulation procedure when using the outage as performance metric:

- The baseline performance is achieved by generating the CDF curve of users versus SINR without enabling the aggressor system.

- Choosing one SINR decoding threshold to be used for baseline performance corresponding to a certain outage ratio: If a certain outage ratio (e.g. 5%) is decided to be used for baseline performance, the UE percentile point corresponding to the SNR decoding threshold in SINR CDF curve should be equal or lower than this outage ratio. the outage ratio means that the percentage of the UE in the CDF curve below the SINR decoding threshold representing the percentage of UE experiencing outage as these UE are below the SINR decoding threshold.

- Turning on the aggressor system and generating the new CDF curve of users versus SINR, multiple CDF curves can be generated with different ACIR.

- By comparing the CDF curve of baseline generated in step 3 and CDF curve with a certain ACIR generated in step 4, an outage degradation data point can be obtained by observing the percentage difference of UEs corresponding to the SINR decoding threshold in these two curves, i.e. the data point is chosen to compare is the percentage in baseline CDF curve and CDF curve corresponding to the SINR decoding threshold when aggressor system is enabled for the same SINR decoding threshold in step 3.

- Generating an outage degradation curve versus ACIR.

#### 6A.z.3.2	Coupling loss

The Coupling Loss (CL) is defined as the loss in signal between NR BS-to- NR UE, A-IoT reader -to- A-IoT device. CL is defined as the loss including propagation loss and antenna gains.

### 6A.z.4	Co-existence evaluation results

The simulation results submitted for the ambient IoT and NR co-existence study from all contributing companies in RAN4 were summarized in [R4-2602113].

### 6A.z.5	Summary of co-existence evaluation

In the Rel-20 SI, RAN4 evaluated standalone deployment mode involves the non-colocation of A-IoT BS and NR BS, while inband deployment mode involves the colocation of A-IoT BS and NR BS. This chapter presents the co-existence simulation results for ambient IoT based on baseline assumptions in clause 6A.z.2, categorized by operation mode and device type. The conclusions below are based on the coexistence results. ACLRe (Efficient ACLR) and ACSe (Efficient ACS) were used for evaluation parameter. The details of ACLRe and ACSe deviation can refer to [R4-2523061].

#### 6A.z.5.1	Standalone with device 2b

For standalone operation mode of device 2b, the observations of each case are listed in Table 6A.6.5.1-1.

Table 6A.z.5.1-1: Standalone device 2b co-existence observations

| Operation mode | Device type | Case No. | Aggressor | Victim | Co-existence observation |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | Negligible NR UL throughput degradation is observed if the device ACLRe is ≥ 20dB (flat model) when the Tx power is fixed at -10dBm. |
|  |  | 2 | NR UL | reader | The average SINR (with ACI) @5%-ile based on the collected results is -4.8dB with the standard deviation of 2.3dB. The average SINR (with ACI) @50%-ile based on the collected results is 12.9dB with the standard deviation of 1.5dB. The applied ACSe for the reader ranges from 18dB to 20dB. |
|  |  | 3 | Reader | NR DL | Across the reported results, contributions indicate that the throughput loss@50%-ile is ≤ 5% (negligible degradation), whereas at 5% -ile most contributions show more than 10% throughput loss. Several contributions further report that applying reader‑side mitigation schemes (including reader Tx power reduction, adjusting reader antenna tilt angle, reducing ISD) result in throughput loss ≤ 5% at both 5%-ile and 50%-ile. |
|  |  | 4 | NR DL | device | The average SINR (with ACI) @5%-ile based on the collected results is 3.5dB with the standard deviation of 2.5dB. The average SINR (with ACI) @50%-ile based on the collected results is 17.4dB with the standard deviation of 0.6dB. The applied ACSe for the device ranges from 40dB to 53dB. |

#### 6A.z.5.2	Standalone with device C

For standalone operation mode of device C, the observations of each case are listed in Table 6A.z.5.2-1.

Table 6A.z.5.2-1: Standalone device C co-existence observations

| Operation mode | Device type | Case No. | Aggressor | Victim | Co-existence observation |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device C | 5 | Device | NR UL | Negligible NR UL throughput degradation can be achieved if the device ACLRe(flat model) is ≥34dB when the Tx power is fixed at 5dBm, or the device ACLRe(flat model) is ≥20dB when Tx power control is applied. One company also observed that increaseing the frequency reuse factor also reduced the NR UL throughput degradation. |
|  |  | 6 | NR UL | reader | The average SINR (with ACI) @5%-ile based on the collected results is -0.2dB with the standard deviation of 2.2dB. The average SINR (with ACI) @50%-ile based on the collected results is 16.2dB with the standard deviation of 1.1dB The applied ACSe for the reader ranges from 45dB to 58dB. One company observed 4% outage without ACI based on frequency reuse 9 and SNR decoding threshold 5dB and observed 5% additional outage degradation with ACI. The company also observed 17% outage without ACI based on frequency reuse 3 and SNR decoding threshold 5dB. |
|  |  | 7 | NR DL | device | The average SINR (with ACI) @5%-ile based on the collected results is 1.5dB with the standard deviation of 3.7dB. The average SINR (with ACI) @50%-ile based on the collected results is 16.8dB with the standard deviation of 1.5dB. The applied ACSe for the device ranges from 33dB to 50dB. One company observed 4% outage without ACI based on frequency reuse 9 and SNR decoding threshold 5dB and observe with 5% additional outage with ACI. The company also observed 17% outage without ACI based on frequency reuse 3 and SNR decoding threshold 5dB |

#### 6A.z.5.3	Inband with device 2b

For inband operation mode of device 2b, the observations of each case are listed in Table 6A.z.5.3-1.

Table 6A.6.5.3-1: Inband device 2b observations

| Operation mode | Device type | Case No. | Aggressor | Victim | Co-existence observation |
| --- | --- | --- | --- | --- | --- |
| Inband | Device 2b | 8 | NR UL | reader | The average SINR (with ACI) @5%-ile based on the collected results is -6.1dB with the standard deviation of 2.6dB. The average SINR (with ACI) @50%-ile based on the collected results is 10.7dB with the standard deviation of 1.9dB The applied ACSe for the reader ranges from 45dB to 46dB. |
|  |  | 9 | NR DL | device | The average SINR (with ACI) @5%-ile based on the collected results is 3.0dB with the standard deviation of 2.6dB. The average SINR (with ACI) @50%-ile based on the collected results is 16.0dB with the standard deviation of 1.1dB The applied ACSe for the device ranges from 33dB to 50dB. |

#### 6A.z.5.4	Inband with device C

For inband operation mode of device C, the observations of each case are listed in Table 6A.z.5.4-1.

Table 6A.z.5.4-1: Inband device C co-existence Observtions

| Operation mode | Device type | Case No. | Aggressor | Victim | Co-existence observation |
| --- | --- | --- | --- | --- | --- |
| Inband | Device C | 10 | NR UL | reader | The average SINR (with ACI) @5%-ile based on the collected results is -0.1dB with the standard deviation of 1.1dB. The average SINR (with ACI) @50%-ile based on the collected results is 15.9dB with the standard deviation of 1.0dB The applied ACSe for the reader ranges from 31dB to 51dB. One company observed 4% outage without ACI based on frequency reuse 9 and SNR decoding threshold 5dB and 17% outage without ACI based on frequency reuse 3 and SNR decoding threshold 5dB. For outage with ACI, it is observed with 5% additional outage. |
|  |  | 11 | NR DL | device | The average SINR (with ACI) @5%-ile based on the collected results is 3.2dB with the standard deviation of 2.8dB. The average SINR (with ACI) @50%-ile based on the collected results is 16.2dB with the standard deviation of 2.0dB The applied ACSe for the device ranges from 31dB to 48dB. One company observed 4% outage without ACI based on frequency reuse 9 and SNR decoding threshold 5dB and 17% outage without ACI based on frequency reuse 3 and SNR decoding threshold 5dB. For outage with ACI, it is observed with 5% additional outage. |

#### 6A.z.5.5	General remarks on coexistence findings

Across the assessed cases, the results consistently show that R2D and D2R demodulation thresholds are critical to coexistence performance and therefore represent a key design driver for the A‑IoT system in the outdoor scenario.

When NR DL is the victim, some mitigation schemes are helpful to reduce the A-IoT system inteference, e.g., increasing the frequency reuse factor, lowering the A-IoT reader power, reducing the network grid shift and adjusting the AIoT reader antenna tilt, otherwise, the thoughtput degradation of NR is observed.

When NR UL is the victim, applying Tx power control for device C is beneficial. Otherwise, stringent ACLR requirement of the device might be required.

For both device 2b and C, when A-IoT is the victim system, the performance degradation on the A-IoT D2R and R2D links is observed. Whether this degradation will have impact on A-IoT system depends on the SINR decoding thresholds.

Lower SINR threshold for R2D/D2R decoding could increase the resilience of the A-IoT system to the interference from NR as well as relax the RF design parameters such as ACLR and ACS.

The ISD of 750 meters is assumed in the coexistence simulations. The results show that lower SINR(@5%-ile and @50-ile) is obtained at the reader side for device 2b compared with device C, due to the difference in max Tx power. Whether SINR difference will have impact on A-IoT system also depends on the SINR decoding thresholds and the choice of ISD. For device C, its higher output power compared to device 2b results in higher ACLR requirement assuming fixed maximum power is applied.

When taking the above analysis into account, the A-IoT system can coexist with the NR system. In addition, the evaluation results are dependent of the assumed baseline parameters and device/reader characteristics (e.g., RF parameters ACLR/ACS). Frequency reuse 3 is used in simulation, and higher reuse factor is also considered in RAN4 coexistence study to improve the A‑IoT intra‑system performance.

==============End of change==============