3GPP TSG-RAN WG1 #61bis	R1-104253

June 28th – July 2nd, 2010

Dresden, Germany

Agenda item:	6.8

Source: 		        Qualcomm Incorporated

Title: 	eICIC Solutions Details

Document for:	Discussion/Decision

# 1		Introduction

In this contribution, we provide details of the baseline and other solutions for eICIC listed in RAN 1 LS for eICIC [1].

# Baseline solution

## Time domain

Time domain approach can take one of the following forms:

- Almost blank subframes

- Almost blank subframes with MBSFN configuration

- Almost blank subframes with OFDM symbol shift

- Blank subframes for TDD only

### Almost blank subframes

#### Description

This approach relies on backhaul signaling for the subframe partitioning, to enable coordinated scheduling across nodes. Almost blank subframes are fully backward compatible and characterized by lack of unicast transmission on any physical channel:

- PSS/SSS transmission occurs in subframe 0 and 5 (FDD) and 0, 1, 4 and 5 (TDD for 5 ms periodicity), and 0 and 1 (TDD for 10 ms periodicity)

- PBCH transmission occurs in subframe 0

- CRS transmission occurs in all subframes

  - CRS transmission in the data region is avoided if a subframe is configured as MBSFN subframe

- PDCCH transmission masked with P-RNTI may occur only in subframes configured for paging (subframes 0, 4, 5 and 9)

- PDCCH transmission masked with SI-RNTI occur only in subframe 5 in even SFNs

- PCFICH transmission occurs only in subframes where PDCCH transmission occurs

#### Backward compatibility

Almost blank subframes are fully backward compatible

#### Impact on specifications

There is an impact on the network side and the UE side.

Network side impact:

- Determine how to handle backhaul signaling for subframe coordination

  - Inter-cell coordination of almost blank subframes utilization

UE side impact:

- Determine how to handle

  - Radio link monitoring (RLM) procedure

  - RRM measurements

  - Channel feedback

  - Detection of PSS/SSS

  - Detection of PBCH

  - Interference from CRS on PCFICH, PHICH, PDCCH and PDSCH

  - Interference from PDCCH masked with P-RNTI and SI-RNTI (for SIB-1 only) and associated PCFICH on PCFICH, PHICH and PDCCH

### Almost blank subframes with MBSFN configuration

#### Description

Unlike pure almost blank subframe approach, this method does not require backhaul signaling for subframe partitioning. Instead, it is only necessary to convey MBSFN configurations between coordinating nodes. Remaining aspects are the same as for almost blank subframes.

#### Backward compatibility

Almost blank subframes with MBSFN configuration are fully backward compatible

#### Impact on specifications

There is an impact on the network side and the UE side.

Network side impact:

- Determine how to handle backhaul based signaling to include MBFSN configuration

UE side impact:

- Determine how to handle

  - Radio link monitoring (RLM) procedure

  - RRM measurements

  - Channel feedback

  - Detection of PSS/SSS

  - Detection of PBCH

  - Interference from PCFICH/PHICH/PDCCH on PCFICH/PHICH/PDCCH

.

### Almost blank subframes with OFDM symbol shift

#### Description

In this approach, in addition to almost blank subframes, OFDM symbol shift is configured to avoid CRS interference to PDCCH region. Data RE muting is utilized to prevent RLF of Rel 8/9 UEs. The remaining aspects are the same as for almost blank subframes.

#### Backward compatibility

Impact on Rel 8/9 UE performance due to RE muting utilized to prevent RLF for Rel 8/9 UEs needs to be evaluated.

#### Impact on specifications

There is an impact on the network side and the UE side.

Network side impact:

- Same as almost blank subframes

UE side impact:

- Same as almost blank subframes

### 2.1.4 Blank subframes for TDD only

#### Description

This approach utilizes different DL/UL partitioning at different nodes to create blank DL subframes. Requires backhaul signaling for DL/UL partitioning coordination.

#### Backward compatibility

Blank subframes are fully backward compatible

#### Impact on specifications

There is an impact on the network side.

Network side impact:

- Determine how to handle backhaul signaling for DL/UL partitioning coordination.

## Power control

Power control can take one of the following forms:

- Backhaul signaling of power reduction

- Power setting based on HeNB measurement

#### TDD commonality

### 2.2.1 Backhaul signaling of power reduction

#### Description

There is backhaul signaling of power reduction recommendation from eNB A to eNB B. Power reduction applies to CRS and control transmission of recipient node.

#### Backward compatibility

Backhaul signaling of power reduction is fully backward compatible.

#### Impact on specifications

There is an impact on the network side.

Network side impact:

- Determine how to handle backhaul signaling power reduction recommendation.

### 2.2.2 Power setting based on HeNB measurement

#### Description

Power setting is based on HeNB measurement of the strongest macro eNB interferer, as described in Section 7.3.2.1 in TR36.921.

#### Backward compatibility

Power setting based on HeNB measurement is fully backward compatible.

#### Impact on specifications

There is an impact on the network side.

Network side impact:

- Determine how to handle power setting based on HeNB measurements.

# Other solutions

### 3.1 Load coordination on PDCCH

#### Description

Relies on backhaul signaling for load coordination on PDCCH.

#### Backward compatibility

Load coordination on PDCCH methid is fully backward compatible.

#### Impact on specifications

There is an impact on the network side.

Network side impact:

- Determine how to handle load coordination on PDCCH.

# References

- R1-1-xxxx RAN 1 LS for eICIC.