3GPP TSG-RAN WG1 #68	R1-120632

Dresden, Germany, 6 – 10 February 2012

Source:	MediaTek Inc.

Title:	Analysis of single receive RF chain

Agenda Item: 7.7.2

Document for: 	Discussion

# Introduction

Motivated by the goal of avoiding, in the future, the need to maintain a separate GSM/GPRS network just for MTC devices, the study item of provisioning of low-cost MTC UEs based on LTE was proposed and approved [1]. The most important factor for the business success of LTE based MTC is obviously cost if satisfactory coverage & power consumption can be ensured as well.

In the last RAN1 #67 meeting, it was agreed that the reference for the cost comparison for the low cost MTC device will be a single band, single RAT, Cat-1 UE, and operating on a 20 MHz carrier. Also, the following techniques have been identified for further analysis:

- Reduction of maximum bandwidth

- Single receive RF chain

- Reduction of peak rate

- Reduction of transmit power

- Half duplex operation

A tabulated analysis of most of the proposed techniques can be found in [4]. In this contribution, we present more detailed analysis on single receiver RF chain. We follow the structure of the TR section 6 so that most of the text in all the sections can be considered as text proposal.

---------------------------Text Proposal Start (other than the text in brackets) ------------------------------------

# Description

Single receive RF chain is a technique to achieve cost saving by not requiring the MTC devices to have two receive antennas and RF chains.

It is expected that this technique will have most of the impact on receiver performance which should assessed together with the expected cost saving.

# Analysis/evaluation of performance against requirements

An analysis of the technique against system requirements is provided in this section. The analysis is to be considered in combination with the cost analysis:

Table 1. Impact of single RF chain

| Metric | Impact (Yes/No) |
| --- | --- |
| Coverage same as GSM/EGPRS [and legacy LTE] | Yes |
| Minimum Data rate | No |
| Power consumption | Yes |
| Impact to non-MTC UE | No |
| eNB Hardware impact | No |
| Impact on specification | No |
| Cell spectral efficiency | Yes |
| ….. |  |
| …… |  |

[Editor’s Note: Whilst the Low cost MTC UE based on LTE is required to meet all the requirements, a particular requirement may not be applicable to an identified technique. Evaluation/analysis of impact (positive/negative) to be provided below for only for the requirement’s that has an impact (indicated by “Yes” above in the table). Below shown are example placeholders for some analysis/evaluation of some of the requirements]

# Coverage Analysis

Note the coverage requirement in [1],

- Ensure that service coverage is not worse than GSM/GPRS, at least comparable and preferably improved beyond what is possible for providing MTC services over GPRS/GSM today (assuming deployment in the same spectrum bands). The same defined LTE cell coverage footprint as engineered for “normal LTE UEs” should apply for low-cost MTC UEs.

the coverage goal not only is expected to be at least comparable to that of GPRS/GSM, but also should be at least the same as legacy LTE.

It is expected that the coverage footprint will be reduced for all the DL physical channels, compared to cases for normal LTE UEs with 2 Rx antennas. Among the DL physical channels, PDCCH/PBCH/PCFICH/PHIGH are of particularly concern with respect to coverage. When MTC UEs also has a reduced maximum bandwidth, the lack of frequency diversity in physical control channel is expected to further degrade the coverage, especially for option-1 described in [5].

Before a link budget based analysis is performed, simulation parameters need to be agreed. Antenna gain imbalance (frequency dependent) will affect normal LTE UE’s control channel performance as well, i.e., it will reduce the gap seen for single Rx as compared to dual-Rx normal LTE UEs.

If the objective of same coverage as normal LTE is critical to the business case (which might be the case), solutions that can enhance control channel performance under single Rx should be explored in the related ePDCCH work item.

[Our Proposal:

- Evaluate coverage under both legacy PDCCH and ePDCCH. Solutions that can enhance control channel performance under single Rx should be explored in the related ePDCCH work item.

]

# Power Consumption

Requirement for power consumption in [1] states:

- Ensure that overall power consumption is no worse than existing GSM/GPRS based MTC devices.

Power consumption during the reception period in the connected mode is expected to be reduced with single RF. A quantitative estimation of power consumption based on receiver complexity is FFS.

# Analysis/evaluation of cost reduction

The main objective/requirement of the study item is still cost reduction, i.e. [1],

To understand the feasibility of creating a type of terminal that would permit the cost of terminals tailored for the low-end of the MTC market to be competitive with that of GSM/GPRS terminals targeting the same low-end MTC market

Quantitative cost analysis for different proposed technique is conducted based on percentage cost of affected cost drivers (e.g., [3]). In particular, the cost drivers are listed according to functional blocks, each with a percentage cost value with respect to RF or baseband portion. Functional blocks whose cost will likely scale with bandwidth (i.e., sampling rate) or data rate are listed separately, thus the cost reduction due to change in bandwidth support and/or data rate can be accounted for more easily in overall cost saving analysis. In order to understand the overall contribution to cost reduction from a technique, especially in combination with other cost reduction techniques, it should be noted that the relative percentage savings of each different technique are multiplicative.

A list of potentially affected cost areas is captured in the table below:

Table 2. Cost areas & analysis for single RF chain

|  | Cost Analysis |
| --- | --- |
| RF | Number of RF transceiver (21) |
| BB |  |
| A/D, D/A | Reduced by half |
| FFT/IFFT size | Reduced by half |
| Data buffering | Reduced by half |
| Receiver processing & demod (CE & LLR computation) | Reduced due to no rank-2 MIMO |

[Our observation:

- Total cost saving: 25%

]

---------------------------Text Proposal Ends ------------------------------------

# Conclusion

In this contribution, we present more detailed analysis on single receiver RF chain.

We propose:

- Consider texts in all the sections as text proposal  (other than the text in brackets)

- Evaluate coverage under both legacy PDCCH and ePDCCH. Solutions that can enhance control channel performance under single Rx should be explored in the related ePDCCH work item.

- Note the total cost saving of 25%

# References

- R1-111112, Study Item “Provision of low-cost MTC UEs based on LTE”

- R1-113616, “TR36.888 - Study on provision of low-cost MTC UEs based on LTE”

- R1-120636, “Cost Drivers of LTE modem and Text Proposal”, MediaTek, RAN1#68

- R1-113867, “LTE MTC Device Cost Reduction Measures with Cost Analysis”, MediaTek, RAN1#67

- R1-120636, “Analysis of reduced maximum bandwidth”, MediaTek, RAN1#68