3GPP TSG RAN WG1 #70	        	            R1-123965

Qingdao, China, August 13 – 17, 2012

Agenda item:	7.5.1.3

Source: 	Ad-hoc Chairman (Samsung)

Title: 		        Summary of COMP Feedback session

Document for:	Discussion and Decision

      - 7.5.1.3 CSI feedback modes for CoMP

        - 7.5.1.3.1 Periodic feedback

R1-123295	Email discussion [69-10]: Details of collision handling and compression/multiplexing in case of 2 or more CSIs being configured in the same reporting instance for CoMP CSI feedback	Fujitsu

Note:

  - One Rel-11 “CSI process” is the association of one channel part (one NZP CSI-RS resource from the CoMP measurement set) and one interference part.

  - An  “CSI process” can also be the CSI report of a cell with CSI-RS or CRS, but without IMR(Rel-10 or earlier)

  - Note 1: This does not preclude the possibility of reporting CSI for multiple “CSI processes” in the same PUCCH.

Agreement:

- All the Rel 10 CSI reporting modes are supported for CoMP in Rel 11.

- All the Rel 10 CSI reporting types are supported for CoMP in Rel 11.

- The Rel 10 rules for collisions between different CSI reports in the non-CA case also apply for non-CA CoMP for the case of collision between CSI reports within one “CSI process”.

R1-123969	Way Forward on CoMP Feedback Modes

Samsung, DoCoMo, Intel, InterDigital, LGE, NEC, Ericsson

Also supported by ST-Ericsson

Working assumption

- Rel-11 supports the feedback configuration and reporting for simultaneous CA and CoMP.

  - Strive for reduction of UE complexity in CSI report design, e.g. limiting number of CSI processes, etc

- Indexing scheme for CSI processes:

- Alt1: Indexing is defined within a given CC

- Alt 2: Indexing is defined across all configured CCs

Working assumption:   Alt 1.

- Dropping rule is supported based on reporting type and CSI process/CC index

  - Alt 1: Reporting type  CSI process index  CC index

  - Alt 2: Reporting type  CC index  CSI process index

Agreement:

Alt 1 of dropping rule is agreed.

Examples:

Alt 1: P1, S0 -->S0

Alt 2: P1, S0 -->P1

Alt 1/2: P0, S0 -> P0

CSI multiplexing

- Alt1: FFS, taking into account the outcome of CA CSI multiplexing discussion

- Alt2:  At least for single carrier case, support  Multiplex of CSIs on PUSCH

- Alt3:  At least for single carrier case, support  Multiplex of CSIs on PUCCH format 3

- Compression schemes suitable for CoMP can be considered on PUCCH Format 2.

R1-123976	WF on rank and subband constraints between CSI Processes

Ericsson, Alcatel Lucent, Alcatel-Lucent Shanghai Bell, CATT, ST-Ericsson, Huawei, Hi Silicon, LG Electronics, New Postcom, Samsung

Take the following two bullets as working assumption.

- A RI-reference-process can be configured for a CSI process

  - PMI/CQI of the process is calculated conditioned on the RI of its RI-reference-process ,if configured, that is reported in the same or the most recent preceding subframe

- A subband-reference-process can be configured for a CSI process

  - Subband CQI of the process reflecting transmission over the same subband as indicated for the subband-reference-process of the same bandwidth part, that is reported in the same or the most recent preceding subframe

Oppose: Nokia, NSN,

Continue discussion. If possible, come back on Friday

- FFS: A PMI-reference-process can be configured for a CSI process

  - CQI of the process is calculated conditioned on the reported PMI of its PMI-reference-process if configured

    - RI associated with the reported PMI is used

  - The PMI-reference-process is only applicable in case the CSI process is configured in the same feedback mode as the PMI-reference-process

- For periodic reporting

  - A CSI Process with a reference CSI Process shall not be reported until the reference property of the reference CSI Process has been reported at least once

  - The CSI process must be configured in the same periodic feedback mode as a reference CSI process

  - The reference RI/subband of the most recently periodically reported reference-CSI-process shall be used

    - No mixing between aperiodic and periodic reports

  - There is no rank/subband signalling compression

    - FFS: If compression can be achieved with minimum implementation/specification effort

  - The collission handeling will be unaffected by this proposal

Collision Handling

R1-123104	Periodic CSI feedback collision handling for CoMP	Huawei, HiSilicon

R1-123138	Periodic feedback modes for CoMP	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

R1-123190	Periodic CSI feedback for DL CoMP	Texas Instruments

R1-123364	Periodic CSI Feedback for Rel-11 CoMP	NTT DoCoMo

R1-123334	Multiple CSI in peridoic feedback for CoMP	MediaTek

R1-123468	CSI Collision Handling for CoMP	Samsung

R1-123685	Periodic CSI feedback reporting in support of CoMP	Qualcomm Inc.

R1-123278	Periodic CSI feedback configuration and behaviour	Panasonic

R1-123347	Discussion on periodic CSI feedback for CoMP transmission	Fujitsu

R1-123433	Periodic feedback for CoMP	New Postcom

R1-123581	On periodic feedback	Renesas Mobile Europe Ltd

R1-123641	Periodic CSI feedback modes for DL CoMP	Nokia Siemens Networks, Nokia

CSI Multiplexing (on PUCCH Format 3/PUSCH)

R1-123171	Discussion on periodic feedback modes for CoMP	Intel Corporation

R1-123216	Further discussion on periodic feedback	CATT

R1-123380	Periodic CSI feedback modes for CoMP	ZTE

R1-123469	Feedback for Multiple Interference Assumptions	Samsung

R1-123516	Discussion on periodic feedback for CoMP	LG Electronics

R1-123831	On Periodic Reporting of CSI Processes	Ericsson, ST-Ericsson

Configuration

R1-123830	Configuration of CSI Processes for Periodic and Aperiodic Reporting	Ericsson, ST-Ericsson

R1-123467	CSI Feedback Configuration for CoMP	Samsung

R1-123260	CSI Collision Procedures	Sharp

R1-123323	Periodic CSI feedback for DL CoMP	Pantech

R1-123402	Periodic CSI feedback for CoMP	InterDigital Communications, LLC

R1-123420	Discussion on periodic CSI feedback for CoMP	Hitachi Ltd.

R1-123465	CoMP CSI Feedback Including Preferred-TP Indicator	Samsung

R1-123542	Discussion on Periodic CSI Reporting for CoMP Operations	ITRI, Acer

R1-123593	Collision handling of CSI reports for DL CoMP	HTC

R1-123594	Multiplexing of CSI reports for DL CoMP	HTC, ITRI

R1-123811	Discussion on periodic CSI feedback mode for CoMP 	China Unicom

        - 7.5.1.3.2 Aperiodic feedback

Agreement (Last Meeting):

- In the case of a single CC configuration where multiple CSIs are configured for COMP,  2-bit CSI request field will be used in DCI format 0 (if in UE SS) and DCI format 4 for triggering of aperiodic CoMP feedback

  - The candidate CSI reports are configured by RRC

  - 1-bit CSI request remains in format 0 in the case of CSS

    - FFS the payload content of the report

  - Multiple CSI feedbacks could be multiplexed within one report instance

    - FFS how to configure these multiple feedbacks into one report instance

  - FFS the semi-static and dynamic signaling details

  - FFS if CSI request field is extended to other than 2 bits, by adding new bits or using existing codepoints

  - FFS simultaneous usage of CSI request field for CoMP and Carrier Aggregation

Email discussion until June 30 on FFS items (Tim Moulsley, Fujitsu)

R1-123296	Email discussion [69-11]: FFS aspects of aperiodic CSI feedback for CoMP	Fujitsu

Proposal from R1-123969

- For 2-bit CSI request,

  - ’00’ state indicates no trigger

  - Selection between two or more RRC configured sets of CA+CoMP processes

- For 1-bit CSI request

  - ’0’ state indicates no trigger

  - ‘1’ state - One of the states in the 2-bit case may be reused (e.g., 01)

R1-123832	On Aperiodic Reporting of CSI Processes	Ericsson, ST-Ericsson

R1-123105	Aperiodic CSI feedback triggering for CoMP	Huawei, HiSilicon

R1-123139	Aperiodic feedback modes for CoMP	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

R1-123365	Aperiodic CSI Feedback for Rel-11 CoMP	NTT DoCoMo

R1-123470	Aperiodic CSI Triggering for CoMP	Samsung

R1-123517	Discussion on aperiodic feedback for CoMP	LG Electronics

R1-123686	Aperiodic CSI feedback reporting in support of CoMP	Qualcomm Inc.

R1-123642	Aperiodic CSI feedback for DL CoMP	Nokia Siemens Networks, Nokia

R1-123217	Further discussion on aperiodic feedback	CATT

R1-123279	Aperiodic CSI feedback configuration, triggering and behaviour	Panasonic

R1-123434	Aperiodic feedback for CoMP	New Postcom

R1-123381	Aperiodic CSI feedback modes for CoMP	ZTE

Feedback Modes; Common RI/PMI/Selected-Subband

R1-123172	Discussion on aperiodic feedback modes for CoMP	Intel Corporation

R1-123232	UCI Multiplexing on PUSCH for DL CoMP	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

R1-123466	Configuration of UE selected reporting modes 	Samsung

R1-123403	Aperiodic CSI feedback for CoMP	InterDigital Communications, LLC

R1-123595	Aperiodic feedback to support DL CoMP	HTC

Other

R1-123247	Aperiodic CSI feedback for CoMP	NEC group

R1-123421	Discussion on aperiodic CSI feedback for CoMP	Hitachi Ltd.