3GPP TSG-RAN Meeting #70 	R1-123990

# Qingdao, China, 13th - 17th August, 2012

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.214 | CR | CR0018 | rev | 4 | Current version: | 10.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL Relative Time of Arrival |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE-NBPS |  |  |  |  |  |  | Date: |  | 03/17/2012 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) |  |
| Reason for change: |  | Define physical-layer measurement in E-UTRAN to support network-based positioning method with reference time defined in TS 36.459 and reference performance specification in TS 36.111 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new physical-layer measurement “UL relative time of arrival” in TS 36.214, Section 5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network based positioning feature is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 36.201: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Layer – General Description “.

[3]	3GPP TS 36.211: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”.

[4]	3GPP TS 36.212: “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding “.

[5]	3GPP TS 36.213: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures “.

[6]	3GPP TS 36.321: “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification“.

[7]	3GPP TS 36.331: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification “.

[8]	3GPP2 CS.0005-D v1.0 “Upper Layer (Layer 3) Signaling Standard for CDMA2000 Spread Spectrum Systems Release D”.

[9]	3GPP2 CS.0024-A v3.0 “cdma2000 High Rate Packet Data Air Interface Specification”

[10]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception “.

[11]	3GPP TS 36.355: “Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)”

[12]	3GPP TS 36.455: “Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)”

[13]		3GPP TS 36.459: “Evolved Universal Terrestrial Radio Access (E-UTRA);  SLm Application Protocol (SLmAP)”

[14]	3GPP TS 36.111: “Evolved Universal Terrestrial Radio Access (E-UTRA); Location Measurement Unit (LMU) performance specification; Network Based Positioning Systems in E-UTRAN”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Ec/No	Received energy per chip divided by the power density in the band

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1x RTT	CDMA2000 1x Radio Transmission Technology

CPICH	Common Pilot Channel

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FDD	Frequency Division Duplex

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile communication

HRPD	CDMA2000 High Rate Packet Data

LMU				Location Measurement Unit

P-CCPCH	Primary Common Control Physical Channel

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SRS	Sounding Reference Signal

TDD	Time Division Duplex

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

## 5.2	E-UTRAN measurement abilities

The structure of the table defining a E-UTRAN measurement quantity is shown below.

| Column field | Comment |
| --- | --- |
| Definition | Contains the definition of the measurement. |

The term "antenna connector" used in this sub-clause to define the reference point for the E-UTRAN measurements refers to the "BS antenna connector" test port A and test port B as described in [10]. The term "antenna connector" refers to Rx or Tx antenna connector as described in the respective measurement definitions.

### 5.2.1	DL RS TX power

| Definition | Downlink reference signal transmit power is determined for a considered cell as the linear average over the power contributions (in [W]) of the resource elements that carry cell-specific reference signals which are transmitted by the eNode B within its operating system bandwidth. For DL RS TX power determination the cell-specific reference signals R0 and if available R1 according TS 36.211 [3] can be used. The reference point for the DL RS TX power measurement shall be the TX antenna connector. |
| --- | --- |

### 5.2.2	Received Interference Power

| Definition | The uplink received interference power, including thermal noise, within one physical resource block’s bandwidth of resource elements as defined in TS 36.211 [3]. The reported value shall contain a set of Received Interference Powers of physical resource blocks as defined in TS 36.211 [3]. The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches. |
| --- | --- |

### 5.2.3	Thermal noise power

| Definition | The uplink thermal noise power within the UL system bandwidth consisting of resource blocks as defined in [3]. It is defined as (No x W), where No denotes the white noise power spectral density on the uplink carrier frequency and denotes the UL system bandwidth. The measurement is optionally reported together with the Received Interference Power measurement, it shall be determined over the same time period as the Received Interference Power measurement, The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches. |
| --- | --- |

### 5.2.4	Timing advance (TADV)

| Definition | Type1: Timing advance (TADV) type 1 is defined as the time difference TADV = (eNB Rx – Tx time difference) + (UE Rx – Tx time difference), where the eNB Rx – Tx time difference corresponds to the same UE that reports the UE Rx – Tx time difference. Type2: Timing advance (TADV) type 2 is defined as the time difference TADV = (eNB Rx – Tx time difference), where the eNB Rx – Tx time difference corresponds to a received uplink radio frame containing PRACH from the respective UE. |
| --- | --- |

### 5.2.5	eNB Rx – Tx time difference

| Definition | The eNB Rx – Tx time difference is defined as T eNB-RX – TeNB-TX Where: T eNB-RX is the eNB received timing of uplink radio frame #i, defined by the first detected path in time. The reference point for TeNB-RX shall be the Rx antenna connector. T eNB-TX is the eNB transmit timing of downlink radio frame #i. The reference point for TeNB-TX shall be the Tx antenna connector. |
| --- | --- |

### 5.2.6	E-UTRAN GNSS Timing of Cell Frames for UE positioning

| Definition | TE-UTRAN-GNSS is defined as the time of the occurrence of a specified LTE event according to a GNSS-specific reference time for a given GNSS (e.g., GPS/Galileo/Glonass system time). The specified LTE event is the beginning of the transmission of a particular frame (identified through its SFN) in the cell. The reference point for TE-UTRAN-GNSS shall be the Tx antenna connector. |
| --- | --- |

### 5.2.7	Angle of Arrival (AoA)

| Definition | AoA defines the estimated angle of a user with respect to a reference direction. The reference direction for this measurement shall be the geographical North, positive in a counter-clockwise direction. The AoA is determined at the eNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

### 5.2.8	UL Relative Time of Arrival (TUL-RTOA)

| Definition | The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in LMU j, relative to the configurable reference time [13], [14]. The reference point [14] for the UL relative time of arrival shall be the RX antenna connector of the LMU node when LMU has a separate RX antenna or shares RX antenna with eNB and the eNB antenna connector when LMU is integrated in eNB. |
| --- | --- |