3GPP TSG-RAN WG1 Meeting #72bis	R1-131648

Chicago, USA, 15– 19 Apr 2013

Source:	China Unicom

Title:	Link-level simulation results for S-UMTS downlink

Agenda item:		6.5.2

Document for:	Discussion

# 1	Introduction

The channel bandwidth of 5MHz has been designed for regular UMTS FDD by using 3.84Mcps. To solve the problem on the unavailability of enough frequency spectrum resource, the scalable UMTS has been proposed. In last meeting two typical scenarios are determined for S-UMTS [1], including

- Scenario 1: Stand-alone deployment

Stand-alone S-UMTS carrier with reduced bandwidth (1/2 of the legacy carrier bandwidth).

- Scenario 2: Dual-carrier deployment

Dual-carrier operation with legacy carrier as the primary carrier and one S-UMTS carrier with reduced bandwidth (½ or ¼ of the legacy carrier bandwidth) as the secondary carrier, and with only one uplink (on primary carrier).

Candidate physical layer solutions to consider [1]

- Reducing the chip rate to ½ or ¼ from the existing 3.84 Mcps operation

  - Solutions without changing chip rate may be presented

- Possible options for frame/subframe/slot timing

  - Extend the slot duration to maintain the same number of chips/slot

  - Keep the 0.667 ms slot duration and reduce the number of chips/slot

  - Other solutions may be presented

Besides, the simulation assumption has been processed in email discussion, and this contribution analyze the S-UMTS link based on current simulation assumption.

# 3	S-UMTS Downlink Analysis

In the analysis presented in this document, the simulation assumption is given in Table 1 as follows:

Table 1: General Simulation Assumptions for standalone S-UMTS Downlink

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900 MHz |
| Scaling factor | 1 [5MHz], 2[2.5MHz] |
| eNB antenna configuration | 1 Tx |
| UE antenna configuration | 2 Rx |
| Channel model | PA3, VA30, VA120 |
| SNR | -2:1:2 |
| Power Control | ON |
| Power of HS-PDSCH | 0.8628 |
| HARQ | ON（based on IR） |
| AMC | ON（based on CQI） |
| Channel coding | 1/3 Turbo coding |
| Modulation | QPSK、16QAM（according to CQI） |
| UE Receiver Type | RAKE and receive antenna diversity |
| RAKE | Ideal |
| Diversity Receiver type | LMMSE |
| Channel Estimation | Ideal |
| Spreading factor of HS-PDSCH | 16 |

Using the assumptions described in Table 1, we evaluate the throughput for N=1, 2. Table 2 displays the throughput of S-UMTS the DL link level simulation results for HSPA, comparing with UMTS and S-UMTS implementation with scaling factor N =2, for the PA3, VA30 and VA120 channels.

Table2: Throughput under different channel models

| SNR | Throughput Gain |  |  |
| --- | --- | --- | --- |
|  | PA3 | VA30 | VA120 |
| -2 | -0.92% | -2.12% | -1.55% |
| -1 | -1.90% | -1.75% | -1.47% |
| 0 | -1.40% | -0.51% | -0.92% |
| 1 | -1.10% | -0.71% | -0.71% |
| 2 | -0.90% | -0.60% | -0.30% |

6	Conclusions

Based on the simulation results given in Table 2, we have following proposal

Proposal: S-UMTS has approximately the same throughput in the downlink as regular UMTS when scaling factor=2.

7	References

R1-130805, “LS on Scalable-bandwidth UMTS”, RAN WG1.