3GPP TSG RAN WG1 Meeting #73	R1-132416

Fukuoka, Japan, 20th May – 24th May, 2013

Agenda Item:	6.2.3.2

Source:	CATR

Title:	Views on signalling mechanisms for TDD UL-DL reconfiguration

Document for:	Discussion

# Introduction

In 3GPP RAN#58 meeting, WI of TDD eIMTA was agreed. One of the scopes in WI is to make time scale and corresponding signalling mechanism enhancement. In 3GPP RAN1#72bis, the following agreement was achieved[1]:

- A signaling mechanism which explicitly or implicitly indicates TDD UL-DL reconfiguration by either

    - PHY signaling (not including PBCH/MIB signaling), or

    - MAC signaling

- PBCH/MIB signaling issue could be revisited if reliability issue of the above method becomes severe

In this contribution, we will give some views on signaling mechanisms for TDD UL-DL reconfiguration.

# Discussion

From the agreement on #72bis, the main methods for TDD UL-DL reconfiguration will be focused on PHY signaling or MAC signaling. We compare the advantage and disadvantage of these two methods mainly on the following aspects:

- Throughput performance

- Signaling reliability

- Standardization effort

The adaptation time scale of MAC is on the order of a few tens of milliseconds while PHY signaling methods can make this adaptation time in several milliseconds. We show the performance of different reconfiguration times. FTP1 model is used with 0.5Mbype file size. TDD configuration 1 is set as reference. Static CCIM is adopted. The detail of simulation assumptions are given in Appendix. From Figure 1, we can see the performance of 10ms and 40ms reconfiguration are quite near. The loss caused by 40ms reconfiguration is even below 5%.

| UL/DL Cell average packet throughput |  |
| --- | --- |

Figure 1 Packet throughput in multi-cell (Pico) scenario with CCIM

If MAC CE is used for UL-DL reconfiguration, ACK/NACK could be used to ensure reliability. However, the ambiguity issue exists. In [2][3], the activation time is proposed to solve this problem. If PHY is used for UL-DL reconfiguration, the reliability of this method will not be as good as MAC when ACK/NACK is absent.

For MAC signaling, new MAC header will be introduced to indicate the new UL-DL reconfiguration. For PHY signaling, two ways should be used. One is implicit indication and the other is explicit indication. Although implicit indication could minimize standardization work, some additional standardization works like DCI indication are also propose to enhance the performance of dynamic UL-DL reconfiguration[3][4][5].

From the point of views of throughput, reliability and standardization, no obvious advantage or disadvantage are observed between MAC and PHY signaling. However, if we use MAC to indicate UL-DL reconfiguration, some works will have to be done to solve the ambiguity problem. This will bring extra uncertainty. Thus, we propose

Proposal 1: use PHY signaling to indicate UL-DL reconfiguration. The details of PHY signaling should be FFS.

# Conclusion

In this contribution, we show some observations on signaling mechanisms for TDD UL-DL reconfiguration and the following proposal is given:

Proposal 1: use PHY signaling to indicate UL-DL reconfiguration. The details of PHY signaling should be FFS.

Reference

- [1] Minutes of 3GPP RAN1#72BIS

[2] R1-131066, “Signaling to reconfigure TDD UL/DL allocation”, ZTE,RAN#72BIS

- [3] R1-130052, “Signalling mechanisms for TDD UL-DL reconfigurations”, CATT, RAN#72.

[4] R1-130292, “False alarm handling in UL-DL reconfiguration”, Samsung, RAN#72.

[5] R1-130883, “Signaling mechanisms for TDD eIMTA”, Huawei, HiSilicon, RAN#72BIS.

Appendix

Table 1 simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| Scenario | Co-channel and multiple pico cells |
| System bandwidth | 10 MHz |
| Carrier frequency | 2 GHz |
| Inter-site distance | 500 m |
| Macro deployment | The typical 19-cell and 3-sectored hexagon system layout Note that macro cells are deployed but not activated |
| Pico deployment | 40m radius, random deployment |
| Number of pico cells per sector | 4 |
| Minimum distance between pico cells | 40 m |
| Minimum distance between UE and pico | 10 m |
| Pico antenna pattern | 2D, Omni-directional |
| Pico antenna gain | 5 dBi |
| UE antenna gain | 0 dBi |
| Pico noise figure | 13 dB |
| UE noise figure | 9 dB |
| Maximum pico TX power | 30 dBm |
| UE power class | 23 dBm (200 mW) |
| Number of Ues per pico cell | 10 Ues uniformly dropped around each of the Pico cells within a radius of 40m |
| Shadowing standard deviation between outdoor Pico cells | 6 dB |
| Shadowing correlation between Ues | 0 |
| Shadowing correlation between picos | 0.5 |
| Pico-to-pico pathloss | LOS: if R<2/3 km, PL(R)=98.4+20log10(R) else, PL(R)=101.9+40log10(R), R in km NLOS: PL= 40log10(R)+169.36, R in km Case1: ProbI=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) |
| Pico-to-UE pathloss | PLLOS(R)=103.8+20.9log10(R) PLNLOS(R)=145.4+37.5log10(R) For 2GHz, R in km Case1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) |
| UE-to-UE pathloss | If R<=50m, PL=98.45+20*log10(R), R in km If R>50m, PL=55.78 +40*log10(R), R in m (Xia model) |
| Traffic model | - FTP model 1 in TR36.814 - Independent traffic modeling for DL and UL per UE - Fixed size of 0.5Mbytes as in TR36.814 - Poisson distributed with arrival rate λ. The arriving rate for UL file is derived by the ratio of DL and UL arriving rate - Independent traffic generation per cell - Same arriving rate for all the cells |
| UE antenna configuration | 1 Tx, 2 Rx |
| Pico eNB antenna configuration | 2 Tx, 2 Rx |
| Link adaptation | MCS selection with 10% BLER, assuming ideal CSI If the highest MCS is selected, the BLER may be less than 10%, which shall be modelled |
| Set of TDD UL-DL configurations | The seven TDD UL-DL configurations defined in Rel-8 can be used for reconfigurations |
| Cyclic prefix length | Normal CP in both downlink and uplink |
| Special subframe configuration | Configuration #8 |
| Packet drop time | The packet drop time is modelled according to 36.814 (i.e. 8s for 0.5MB) |
| Downlink/uplink receiver type | MMSE for both downlink and uplink |
| UL modulation order | {QPSK, 16QAM, 64QAM} |
| Shadowing standard deviation between Pico and UE | 3dB for LOS and 4dB for NLOS |
| Small scale fading | Not modelled |
| UE UL Power control | Po: -76dBm, alpha: 0.8 |
| Interference mitigation scheme | Static CCIM |
| Threshold for CCIM | 80dB |