3GPP TSG-RAN WG1 #73	R1-132785

# Fukuoka, Japan, 20th – 24th May 2013

# Source: 	NTT DOCOMO (Email rapporteur), Huawei, HiSilicon, ETRI, ZTE, MediaTek, Panasonic, CMCC, LG Electronics

Title:	Assumptions for evaluation related to small cell discovery

Agenda Item:	6.2.5.2.2

Document for:	Discussion and decision

- The further evaluation campaign for the performance of legacy mechanism should be done based on following assumptions in addition to the agreement at RAN1#72bis

  - Target false alarm probability:

    - 0.001 is used for initial evaluation purpose and it should not impact the RAN4 requirement design

    - False alarm probability is defined as a detection probability of cells in the case of noise only input

    - In the legacy mechanism, the conclusive false alarm probability in the PSS/SSS detection should satisfy the above target

  - Target set of detectable cells:

    - Actual target set of detectable cells per carrier frequency should be determined based on gain achievable with, such as, interference coordination and load balancing.

    - Proposals for target set definition:

      - Alt.1: Small cells within RSRP gap = Y, Y=15 dB is baseline at this stage.

      - Alt.2: Top N small cells of a UE with RSRP >= X, N >= 3 and X=-127 dBm are baseline at this stage.

  - Evaluation metrics:

    - Detection probabilities and RSRP measurement accuracies for at least top 3 small cells are evaluated

    - Number of measurement samples, i.e., PSS/SSS/CRS subframes, used for the detection/measurement should be shown (at least 1 subframe case should be evaluated)

  - Other baseline assumptions:

    - UEs only within cluster region are considered for the evaluation

    - It is assumed that UE does not have any PCI information of surrounding small cells in the evaluation of legacy mechanism

    - Synchronized transmission of PSS/SSS/CRS with timing/frequency offsets is assumed

    - Full buffer traffic load is assumed for the discovery performance evaluation

    - +/- 3 μs timing offset and +/- 0.1 ppm frequency offset among small cells are used

    - EPA multipath fading channel (3 km/h) is used

    - Actual detection/measurement algorithms should be implemented