3GPP TSG RAN WG1 Meeting #74	R1-133326

Barcelona, Spain, 19th -23rd August 2013

Agenda Item:	7.2.7.3

Source:	CATR

Title:	Initial calibration results of 3D channel model

Document for:	Discussion and decision

Introduction

At RAN1#73 meeting, three evaluation cases for 3D channel modeling calibration are agreed as working assumption. The first phase is to calibrate geometry and coupling loss, elevation related parameters (without modeling of fast fading), with K=1 and K=10.

In this contribution, we give our initial calibration results for the first phase, and also give our views of the values of θetilt and α.

Evaluation results

Downtilt distribution of UMA and UMI scenarios

Figure 1 Distribution of dowtilts of UMA and UMI scenarios

From the figure1, we can see that in UMA scenario, the elevation angle between UE and eNB is larger than 90 degree, and 85% UEs are distributed between 90 and 100 degree. And in UMI scenario, the elevation angle between UE and eNodeB, is between 90 degreen, and 80% UEs is distributed between 85 and 100 degree.

Performance of different α values

Figure 2 coupling loss and geometry distribution of UMA scenario (different α values)

Figure 3 coupling loss and geometry distribution of UMI scenario (different α values)

From the figure 2 and figure 3 above, in both UMA and UMI scenarios, we could see that the larger the value of α is, the smaller the coupling loss is, but the geometry distribution is not affected by α too much, because it cannot change the beam width of half power, and also cannot change the coverage of eNodeB.

Performance of different downtilt values of  θetilt

Figure 4 coupling loss and geometry distribution of UMA scenario (different θetiltvalues)

Figure 5 coupling loss and geometry distribution of UMI scenario (different θetiltvalues)

From figure 4 and figure 5, we could see that when the downtilt value equals to 102 degrees, the geometry performance is best.

Performance of different vertical antenna element space

Figure 6 antenna gain of different dv         Figure 7 geometry distribution of different dv(UMA)

Figure 8 coupling loss and distribution of different dv(UMI)

There is 10dB gain when K=10, compared with K=1, which is the antenna array gain because of the increment of antenna numbers. And the vertical antenna distance of 0.5 lamada is better than 0.8 lamada, because the beam width of half power is larger from the figure 6 shown above.

Antenna modeling

Port based antenna pattern is described as in table A.2.1.1-2 of TR 36.814, the antenna pattern (vertical) for 3 sector cell sites with fixed antenna patterns,

= 10,  SLAv = 20 dB

Element based antenna pattern is described as

，3dB=65o, SLAv=30 dB，

Figure 9 port based and element based antenna pattern

Figure 10 geometry distribution of different antenna modeling method

It can be seen that the main lobes of the two modeling methods match quite well with the major difference being on the side lobes. The difference in the side lobes can have non-negligible impact on the total coupling loss and geometry distribution calculation.

Conclusion

In this contribution we presented our initial calibration results under the agreed assumptions [1], and have the following observations:

- The larger the value of α is ,the smaller the coupling loss is, but the geometry distribution is not affected by α value too much, because it cannot change the beam width of half power, and also cannot change the coverage of eNodeB.

- When the downtilt value θetilt equals to 102 degree, the geometry performance is best.

- There is 10dB gain when K=10 compared with K=1, which is the antenna array gain because of the increment of antenna numbers. And the vertical antenna distance of 0.5 lamada is better than 0.8 lamada, because the beam width of half power is larger.

- For the antenna modeling, we compare the two candidate modeling methodologies, the main lobes of the two modeling methods match quite well with the major difference being on the side lobes. The difference in the side lobes can have non-negligible impact on the total coupling loss and geometry distribution calculation.

- References

“Report of 3GPP TSG RAN WG1 #72bis”, 3GPP TSG RAN #72bis, Chicago, USA, 15th - 19th April 2013.

“Report of 3GPP TSG RAN WG1#72”, 3GPP TSG RAN #73, Fukuoka, Japan, 20th-24th May 2013.

R1-131858	Initial geometry results for calibration	Huawei, HiSilicon.

- Appendix

Table 1: Simulation assumptions

|  |  | Urban Micro cell with high UE density (3D-UMi) | Urban Macro cell with high UE density (3D-UMa) |
| --- | --- | --- | --- |
| SECTION-1 |  |  |  |
| Layout |  | Hexagonal grid, 19 micro sites,3 sectors per site | Hexagonal grid, 19 macro sites,3 sectors per site |
| UE mobility (movement in horizontal plane) |  | 3kmph | 3kmph |
| BS antenna height |  | 10m | 25m |
| Total BS Tx Power |  | 41/44 dBm for 10/20MHz | 46/49 dBm for 10/20MHz |
| Carrier frequency |  | 2 GHz | 2 GHz |
| Min. UE-eNB 2D distance |  | 10m [other values FFS] | 35m |
| UE height (hUT) in meters | general equation | hUT=3(nfl – 1) + 1.5 | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 | 1 |
|  | nfl for indoor UEs | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Indoor UE fraction |  | 80% | 80% |
| SECTION-21) |  |  |  |
| UE distribution (in x-yplane) | Outdoor UEs | uniform in cell | uniform in cell |
|  | Indoor UEs | uniform in cell | uniform in cell |
| ISD |  | 200m | 500m (FFS: 200m) |

1)Assumptions in SECTION-2 are for calibration purposes only in this SI. Assumptions in SECTION-2 are to be revisited for evaluating relative performance of proposed solutions in future SIs