3GPP TSG RAN WG1 Meeting #75		R1-135265

San Francisco, USA, 11th – 15th November 2013

Agenda item:	6.2.7.3

Source:	NEC

Title: 		   Phase-2 calibration results for fast fading parameters

Document for:	Discussion and Decision

Introduction

In the RAN1#74bis meeting, the agreements for the Phase-2 calibration were the following [1]:

- Phase-2 calibration details

  - BS antenna configuration:

    - Config 1: K=1, M=2, N=2, ULA, 0.5λ H/V  spacing

    - Config 2: K=M=10, N=2, X-pol, 0.5λ H/V spacing with the antenna weights in the working assumption with θtilt = 12 degrees

  - MS antenna configuration: 2 antennas with the same pol as BS

  - System bandwidth: 10 MHz

  - The following metrics for the serving cell are calibrated for each antenna configuration (collected over multiple runs)

    - CDFs of ESD and ESA

    - CDF of average wideband SINR before receiver (i.e., geometry)

    - CDF of largest (1st) singular value in PRBs at t=0

    - CDF of smallest (2nd) singular value in PRBs at t=0

    - CDF of the ratio between the largest singular value and the smallest singular value in PRBs at t=0

- Additional details

  - Dimension of the channel matrix:

    - 2 x (number of BS antenna ports)

  - Singular value calculation

    - Derived with channel matrices where antenna gain is applied but PL and shadowing are not modeled,

    - Singular values are calculated on a per PRB basis by

      - eig(∑HHH)/N , where the summation is across the PRB and N is number of subcarriers in the PRB

In this contribution, we provide the initial simulation results for the Phase-2 calibration according to the simulation assumptions agreed in the previous meetings. We also provide the other details of our simulation assumptions in the appendix.

Discussion

It is clear that different parameters like as ESD, Mean of EoD will impact the distribution of EoD. In addition, the cell selection will be impacted when RSRP calculations base on UE attachment with all rays. Therefore, it is more preferable to provide the coupling loss CDF curves, as we have done in the following part, to calibrate the fast fading channel model both for antenna configuration 1(K=1,M=2,N=2,ULA,0.5λ H/V spacing) and configuration2(K=M=10, N=2, X-pol, 0.5λ H/V spacing with the antenna weights in the working assumption with θtilt = 12 degrees).

Basically, the distribution of ESD and ESA are not depended on antenna configuration, i.e. the CDFs of ESD and ESA should be overlapped completely for antenna config.1 and antenna config.2 since the ESD is a log normal distribution and mean of ESD is depended on UE height, 2D distance between UE and eNB. However, the cell selection for UEs has marginal difference for the two configurations since different antenna configuration may lead to different RSRP calculation when UE attachment method based on all rays is used. That explains why the marginal gap of ESD distribution between the two antenna configuration as shown in Figure 3 and Figure 8.

Since co-polarization is used both for antennas of eNB and UE in config.1, but config.2 is assumed with cross polarized antennas, the different configuration lead that the relation of antennas is smaller in config.1 than one of config.2. Therefore, rank 1 is selected more frequently in config.1. In contrast, there are more rank 2 selection in config.2 from statistical scheduling results. As shown in the Figure 4 and Figure 9 for scenario UMa and UMi respectively, the difference between the first singular value and the second singular value for antenna config.1 is larger than that of config.2. The larger singular value represent more power in the corresponding eigen-vector. By eigen-decomposition of the channel matrix, the larger difference of the two eigen-values represents larger antenna correlation. Therefore, the phenomenon in the Figure is identical with our analysis.

UMa Scenario

Figure 1 CDF of coupling loss distribution for UMa

Figure 2 CDF of geometry distribution for UMa

Figure 3 CDF of ESD and ESA for UMa

Figure 4 CDF of sv(singular value) in PRBs at t=0 for UMa

Figure 5 CDF of ratio between the largest singular value and the smallest singular value in PRBs at t=0 for UMa

UMi Scenario

Figure 6 CDF of coupling loss distribution for UMi

Figure 7 CDF of geometry distribution for UMi

Figure 8 CDF of ESD and ESA for UMi

Figure 9 CDF of sv(singular value) in PRBs at t=0 for UMi

Figure 10 CDF of ratio between the largest singular value and the smallest singular value in PRBs at t=0 for UMi

Conclusion

In this contribution, we provided the initial simulation results for the Phase-2 calibration based on some assumptions listed in the appendix.

References

[1] Chairman’s Notes 3GPP TSG RAN1#74bis, Guangzhou, China, 07th – 11th August 2013.

[2]R1-134813, "Remaining details of fast fading modeling for 3D-UMa and 3D-UMi", NSN, Nokia 3GPP TSG RAN1#74bis, Guangzhou, China, 07th – 11th August 2013.

[3] R1-134582, "Modeling of EoD offset ", CMCC 3GPP TSG RAN1#74bis, Guangzhou, China, 07th – 11th August 2013.

[4] R1-132897, " Channel coefficients generation and remaining antenna modeling issues ", Huawei, HiSilicon 3GPP TSG RAN1#74, Barcelona, Spain, 19th – 23rd August 2013.

Appendix

Shadow fading[1]:

| Shadow fading std[dB] | LOS | NLOS | O-to-I |
| --- | --- | --- | --- |
| 3D UMa | 4 | 6 | 7 |
| 3D UMi | 3 | 4 | 7 |

Cell selection:

- For RSRP calculations needed for UE attachment (including coupling loss calculations), all rays of all clusters shall be used for a given link between a UE and a transmission point

,

ESD[2]:  log-normal parameters µ and σ for 3D-UMa and 3D-UMi.

UMa

NLOS2I case can be given by µ =max[-1,-1.6(d2D/1000)+0.57+0.008(hUT)]

LOS2I case can be given by µ = max[-1,-1.35(d2D /1000)+0.43+0.008(hUT)]

as σ=0.71 for NLOS2I and σ=0.38 for LOS2I cases respectively.

UMi

NLOS2I case can be given by µ = max[-1,-2.3(d2D/1000)+0.78+0.02(hUT)],

LOS2I case can be given by µ =max[-1,-1.6(d2D /1000)+0.64+0.02(hUT)]

σ=1.4 for NLOS2I and σ=0.38 for LOS2I cases respectively.

median EOD or EOD offset [3]:

UMa

LoS:

NLoS: where

UMi

LoS:

NLoS:  where

Other Working assumption:

- for EOD

- PAS step:

- Generation of EOD

  - Xn~ uniform distribution to the discrete set of {1,–1}

  - Yn ~ N(0,σEoD/7)

  - W ~ N(0, σEoD-offset)

- Generation of rays within a cluster

  - Reuse Table B.1.2.2.1-2 (cEoD TBD)

  - Number of clusters and number of rays per cluster as in 36.814

  - FFS how to restrict the value within a valid range

- EOA

- PAS step:

- Generation of EOA:

  - Xn~ uniform distribution to the discrete set of {1,–1}

  - Yn ~ N(0,σEoA/7)

  - = 900 zenith for indoor users, LOS EOA for outdoor UEs

- Generation of rays within a cluster:

  - Reuse Table B.1.2.2.1-2 (cEoA TBD)

  - Number of clusters and number of rays per cluster as in 36.814

  - FFS how to restrict the value within a valid range

- Set equal random initial phase for both VV and HH polarizations in the LOS case, i.e., .

- Decomposition of cross polarization follows A.2.1.6.1 of 36.814

- Other parameters based on parameters of winner+.