3GPP TSG RAN WG1 #75							R1-136001

San Francisco, USA, 11th – 15th November 2013

Title:	Way forward on PDCCH, PDSCH, PUCCH and PUSCH for MTC enhanced coverage

Agenda Item:	6.2.2.2.4

Source:	Vodafone Group, Ericsson, Huawei, HiSilicon, LG Electronics, Samsung, Alcatel-Lucent, ETRI, NSN, Panasonic, Fujitsu, Sony, ZTE, Mediatek, CATT, NEC, Sierra Wireless, u-blox,

Document for:	Decision

# Introduction

This document proposes a way forward on PDCCH/PUCCH for MTC enhanced coverage.

# PDCCH for enhanced coverage

For UE-specific search space, it is proposed to agree that:

- (e)PDCCH to schedule PDSCH is supported.

- Repetition of (e)PDCCH with multiple levels is supported.

- From the UE perspective, the possible starting sub-frames of (e)PDCCH repetitions are limited to a subset of sub-frames.

The following is FFS:

- Whether to use the same PDCCH candidate m (including same aggregation level) in each repetiton, with similar principle for ePDCCH.

# PUCCH for enhanced coverage

It is proposed to agree that:

- No support of periodic CSI over PUCCH

It is proposed to agree as a “Working Assumption” that:

- ACK/NACK on PUCCH is supported. FFS on the configurability of ACK/NACK.

- Dedicated SR is supported but no further optimization beyond PUCCH repetition for SR (e.g. no new formats).

# PDSCH for enhanced coverage

It is proposed to agree that:

- Repetition of PDSCH across multiple sub-frames is supported.

- Multiple repetition levels in time domain are specified.

# PUSCH for enhanced coverage

It is proposed to agree that:

- Repetition of PUSCH across multiple sub-frames is supported.

- Multiple repetition levels in time domain are specified.