Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #75 v1.0.0

(San Francisco, USA, 11th – 15th November 2013)

Document for:	Approval

Table of contents

1	Opening of the meeting	5

1.1	Call for IPR	5

1.2	Network usage conditions	5

2	Approval of Agenda	5

3	Approval of Minutes from previous meeting	5

4	Incoming Liaison Statements	6

4.1	Incoming LS received during the week	7

5	UTRA	7

5.1	Maintenance of UTRA Releases 4 – 11	8

5.1.1.	FDD	8

5.1.2.	TDD	8

5.2.	DCH Enhancements	8

5.2.1.	DL transport channel processing and multiplexing	8

5.2.2.	DL control channel design	9

5.2.2.1.	DL DPCCH slot format optimization	9

5.2.2.2.	DL ACK indication for UL frame early termination	9

5.2.3.	UL transport channel processing and multiplexing	9

5.2.4.	UL control channel design	9

5.2.4.1.	UL DPCCH slot format optimization	9

5.2.4.2.	UL ACK indication for DL frame early termination	9

5.2.5.	Other	9

5.3.	Study on UMTS Heterogeneous Networks	10

5.3.1.	E-DCH decoupling	10

5.3.2.	Network assisted interference cancellation	10

5.3.3.	Impacts of combined cells	11

5.3.4.	Other	11

5.4.	Study on Scalable UMTS	12

5.4.1.	System level evaluation using bursty traffic	12

5.4.2.	Evaluation of user plane latency and implications to user experience	13

5.4.3.	Evaluation of the impacts of time-dilated UMTS to UE performance	13

5.4.4.	Evaluation of the impacts of time-dilated UMTS on network performance and optimization	13

5.4.5.	Coverage analysis	13

5.4.6.	Scalable BW UMTS by filtering	13

5.4.7.	Other	14

5.5.	Study on Further EUL Enhancements	15

5.5.1.	Enabling high user bitrates in a mixed-traffic scenario	15

5.5.2.	Rate Adaptation to support improved power and rate control for high rates	16

5.5.3.	Reduced UL control channel overhead for HSPA	16

5.5.4.	Low-complexity uplink load balancing solutions	17

5.6.	Study on Enhanced Broadcast of System Information	17

5.7.	Other	17

6	E-UTRA	18

6.1	Maintenance of E-UTRA Releases 8 – 11	18

6.2	LTE Release 12	20

6.2.1	Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation	20

6.2.1.1	Remaining details of interference mitigation schemes	21

6.2.1.1.1	Remaining details of UL power control	21

6.2.1.1.2	Remaining details of CSI feedback	22

6.2.1.1.3	Other	23

6.2.1.2	Remaining details of signalling for TDD UL-DL reconfiguration	23

6.2.1.3	Remaining HARQ details	25

6.2.1.4	Other	26

6.2.2	Low Cost & Enhanced Coverage MTC UE for LTE	27

6.2.2.1	Remaining details of new UE category/type	27

6.2.2.2	Techniques for coverage improvement	28

6.2.2.2.1	PBCH	28

6.2.2.2.2	PRACH	29

6.2.2.2.3	(E)PDCCH/PUCCH	31

6.2.2.2.4	PDSCH/PUSCH	31

6.2.2.2.5	SS	33

6.2.2.2.6	Other	33

6.2.3	LTE TDD-FDD Joint Operation including Carrier Aggregation	33

6.2.3.1	TDD – FDD carrier aggregation	33

6.2.3.2	Other	39

6.2.4	LTE Coverage Enhancements	39

6.2.4.1	TTI bundling enhancements for UL VoIP	39

6.2.4.2	Coverage enhancement for TDD	40

6.2.4.3	Other	41

6.2.5	Further MBMS Operations Support	41

6.2.6	Study on Small Cell Enhancements – Physical-layer Aspects	43

6.2.6.1	Higher order modulation	43

6.2.6.2	Small cell on/off and discovery	46

6.2.6.3	Further assessment of solutions for radio-interface based synchronization	52

6.2.6.4	Other	55

6.2.7	Study on 3D-channel model for Elevation Beamforming and FD-MIMO	56

6.2.7.1	Remaining Details of fast fading modelling for 3D-UMa and 3D-UMi	56

6.2.7.2	Remaining simulation assumptions for channel model calibration / baseline	58

6.2.7.3	Initial calibration results	59

6.2.7.4	Details of 3D UMa with one high rise per sector with 300m ISD	60

6.2.7.5	Other	61

6.2.8	Study on LTE Device to Device Proximity Services	61

6.2.8.1	D2D communication	61

6.2.8.1.1	Synchronization design	61

6.2.8.1.2	D2D data communication physical channel design details	63

6.2.8.1.3	Resource allocation method	64

6.2.8.1.4	Other	68

6.2.8.2	D2D discovery	68

6.2.8.2.1	Discovery signal and message design details	68

6.2.8.2.2	Resource allocation method	69

6.2.8.2.3	Other	70

6.2.8.3	D2D transmission timing	70

6.2.8.4	Other	72

6.2.9	Study on CoMP for LTE with Non-Ideal Backhaul	73

6.2.9.1	Further evaluation of coordinated scheduling and coordinated beamforming	73

6.2.9.2	Signalling for inter-eNB operation	76

6.2.9.3	Other	77

6.2.10	Study on Network-Assisted Interference Cancellation and Suppression for LTE	77

6.2.10.1	Remaining details of system level modelling methodology	77

6.2.10.2	System level evaluation results	79

6.2.10.3	Network coordination/signalling and possible spec impact	80

6.2.10.4	Other	83

6.2.11	Other	83

7	Closing of the meeting	83

Annex A:	List of Tdocs at RAN1 #75	84

Annex B:	List of CRs agreed at RAN1 #75	85

Annex C-1:	List of Outgoing LSs from RAN1 #75	87

Annex C-2:	List of Incoming LSs from RAN1 #75	88

Annex D:	List of Approved updated WIDs	89

Annex E:	List of draft TSs/TRs agreed at RAN1 #75	90

Annex F:	List of actions	91

Annex G:	List of participants at RAN1 #75	99

Annex H:	TSG RAN WG1 meetings in 2014 - 2015	100

Main facts summary

3GPP TSG WG RAN1 #75 meeting, hosted by the North American Friends of 3GPP took place at the Hilton San Francisco Union Square Hotel - USA.

The meeting started at 9:02 on Monday 11th November and finished at 17:10 on Friday 15th November 2013.

The number of attending delegates, having signed the participants’ paper list, was 304.

The week was scheduled as follows:

- Monday: Common session on Agenda items 1, 2, 3, 4, 5, 7.1 (E-UTRA maintenance Releases up to 11) followed by Small cell on/off and discovery (AI 7.2.6.2).

- Tuesday (LTE): Parallel sessions on LTE TDD-FDD Joint Operation (AI 7.2.3), Study on NAICS (AI 7.2.10), Study on Provision of Low-cost MTC UEs based on LTE (AI 7.2.2) and Small Cell Enhancements and discovery (AI 7.2.6.2) and solutions for radio-interface based synchronization (AI 7.2.6.3) chaired by Satoshi Nagata on one hand. Study on LTE D2D Proximity Services ( AI 7.2.8.1 on Discovery signal design), Study on CoMP for LTE with Non-Ideal Backhaul (AI 7.2.9) chaired by Matthew Baker followed by Study on 3D-channel model for Elevation Beamforming and FD-MIMO (AI 7.2.7) chaired by Wanshi Chen on the other hand.

- Tuesday (HSPA): Maintenance of UTRA Releases 4 – 11 (AI 6.1), followed by Study on DCH Enhancements for UMTS (AI 6.2) and Study on Scalable UMTS (AI 6.4) chaired by Karri Ranta-aho from NSN.

- Wednesday (LTE): Parallel sessions on Study on NAICS (AI 7.2.10) chaired by Satoshi Nagata followed by LTE Coverage Enhancements (AI 7.2.4) and Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) chaired by Wanshi Chen on one hand. Further MBMS Operations Support (AI 7.2.5) followed by Study on CoMP for LTE with Non-Ideal Backhaul (AI 7.2.9) and Study on LTE Device to Device Proximity Services (AI 7.2.8) chaired by Matthew Baker on the other hand.

- Wednesday (HSPA): Continued Study on Scalable UMTS (AI 6.4) chaired by Karri Ranta-aho from NSN, followed by Study on UMTS Heterogeneous Networks (AI 6.3) chaired by Carmela Cozzo from Huawei.

- Thursday (LTE): Parallel sessions dedicated to Small Cell Enhancements (AI 7.2.6), Study on Provision of Low-cost MTC UEs based on LTE (AI 7.2.2) and LTE TDD-FDD Joint Operation (AI 7.2.3) chaired by Satoshi Nagata on one hand. Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) followed by remaining details on AI 7.2.7 (3D-channel model) chaired by Wanshi Chen and continued Study on LTE Device to Device Proximity Services ( AI 7.2.8) chaired by Matthew Baker on the other hand.

- Thursday (HSPA): Study on Further EUL Enhancements (AI 6.5) chaired by Karri Ranta-aho from NSN.

- Friday morning: Common LTE session to continue on Small Cell Enhancement and LTE TDD-FDD Joint Operation including Carrier Aggregation until morning coffee break. Then revisions.

- Friday afternoon: Revisions

The list of action points that required RAN1 close follow-up is listed in Annex F (end of document).

The number of contribution documents for this meeting was 1043, and those documents were categorized as followed.

| Agenda Item | Input Document | Discussed Document |
| --- | --- | --- |
| From AI 4 to 6.9.3 | 1041 | 425 |

Note: The amount of documents includes those discussed during the email discussion session post meeting.

# Opening of the meeting

Mr Satoshi Nagata (RAN1 Chairman) welcomed the participants of the 75th RAN WG1 meeting and opened the meeting at 09:02.

Mr Sharat Chander from AT&T welcomed the delegates on behalf of the host, the “North American Friends of 3GPP” (Alcatel Lucent, AT&T, Ericsson, Intel, InterDigital Communications, NII Holdings, QUALCOMM, Rogers Wireless, Sprint, T-Mobile USA, TruePosition and Verizon Wireless) and detailed the domestic arrangements for the full week.

## Call for IPR

The Chairman drew the attention to Members’ obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

The attention of the members of this Technical Specification Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The members take note that they are hereby invited: - to investigate in their company whether their company does own IPRs which are, or are likely to become Essential in respect of the work of the Technical Specification Group. - to notify the Director-General, or the Chairman of their respective Organizational Partners, of all potential IPRs that their company may own, by means of the IPR Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

# Approval of Agenda

| R1-135000 | Draft Agenda for RAN1#75 meeting | RAN1 Chairman |  |
| --- | --- | --- | --- |

Satoshi Nagata (Chairman) proposed the agenda for the meeting, as well the schedule of the week.

Discussion: Note that SID on 3D-channel model for Elevation Beamforming and FD-MIMO should be the updated version in RP-130811 (and not RP-122034).

Decision: The agenda is approved.

# Approval of Minutes from previous meeting

| R1-135001 | Final Report of RAN1#74bis meeting | MCC |  |
| --- | --- | --- | --- |

The document was presented by Patrick Mérias (ETSI Mobile Competence Center) and provides the outcomes of last meeting in Guangzhou.

Discussion: MCC warned the group on the fact that a number of SI needs completion by Dec timeframe and requires a number of TRs to be prepared for approval at the next plenary.

Decision: The agenda is approved.

# Incoming Liaison Statements

| R1-135002 | LS on Rel-12 L1 parameters | RAN2, NSN | = R2-133719 |
| --- | --- | --- | --- |

The document was presented by Timo Lunttila from NSN and asks RAN1 to provide Rel-12 L1 parameters in the format similar to the attached excel sheet with the necessary details - should be made available before RAN2#85bis (March 2014).

Decision: The document is noted. Consider the attached excel sheet for preparing the Rel-12 L1 parameters.

| R1-135004 | LS on text proposal for TR 36.843 on D2D including RAN2 agreements | RAN2, Qualcomm | = R2-133726 |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and asks RAN1 to incorporate content of R2-133699 in to TR 36.843.

Decision: The document is noted and it is agreed including RAN2 agreements into TR36.843 – rapporteur (Qualcomm)

Public Safety UE-Network Relays

| R1-135003 | LS on Public Safety UE-Network Relays | RAN2, Vodafone | = R2-133720 |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and informs that RAN2 will focus on communication in- and out of coverage before being able to study relay solutions further.

Discussion (Question / Comment): No action to RAN1.

Decision: The document is noted.

| R1-135006 | Reply LS on Public Safety UE-to-Network Relays | RAN3, Vodafone | = R3-131971 |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and highlights that until the details of the PC5/PC9 interface are known, it is difficult for RAN3 to comment on how the bearer mapping between the PC5/PC9 to the Uu interface would work and identifies aspects that would require further investigation.

Discussion (Question / Comment): No action to RAN1.

Decision: The document is noted.

GCSE with eMBMS

| R1-135005 | Response LS on GCSE with eMBMS | RAN2, Qualcomm | = R2-133728 |
| --- | --- | --- | --- |

The document was presented by Peter Gaal from Qualcomm and provides SA2 with preliminary feedback on group communication for LTE, even though many aspects are yet to be discussed in RAN2.

Discussion (Question / Comment): No action to RAN1.

Decision: The document is noted.

| R1-135007 | Reply LS on GCSE with eMBMS | RAN3, NSN | = R3-131972 |
| --- | --- | --- | --- |

The document was presented by Karri Ranta-aho from NSN and provides SA2 with RAN3 views and answers.

Discussion (Question / Comment): No action to RAN1.

Decision: The document is noted.

| R1-135008 | Reply LS on maximum UL timing difference between TAGs | RAN4, Huawei | = R4-135610 |
| --- | --- | --- | --- |

The document was presented by Zhang Xingwei from HiSilicon and is a reply to RAN1 LS in R1-132819 stating that the worst case is the initial transmission case and the value for the maximum transmission timing difference between TAGs considering relevant tolerance is 32.47s for inter-band carrier aggregation with multiple TAGs. More details in R4-135611.

Discussion (Question / Comment): Mr Chair indicated that 3 contributions will be presented and discussed in RAN2.

Decision: The document is noted. RAN2 will take care of the response LS.

| R1-135009 | 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | ITU-R Ad-Hoc, Telecom Italia | = RT-130054 |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia and requests RAN WGs to review attachments to the LS. The baseline for the material to be submitted to ITU-R will be the 3GPP specifications approved in Rel. 11 in March 2013.

Decision: The document is noted. Check the contents until Friday and feedback corrections/comments, if any to Telecom Italia.

Friday 15th

| R1-136034 | [Draft] 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | Telecom Italia |  |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia. Telecom Italia thanked Ericsson for providing extensive feedback and help drafting the reply LS.

CATT:  further checking might be needed.

Decision: The document is noted and the draft LS is for email approval until 20th November.

| R1-135832 | LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | SA4, Samsung | = S4-131406 |
| --- | --- | --- | --- |

The document was presented by Thomas Novlan from Samsung and asks RAN1

- Q1: about the UE capabilities regarding the support for the reception of multiple MBMS bearers simultaneously. If the support is limited to certain UE categories, SA4 is interested for the appropriate pointers to the UE capability definitions.

- Q2: about the implications of enabling simultaneous MBMS bearer reception on the network side.

Decision: The document is noted.

| R1-135872 | Draft Response LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Peter Gaal from Qualcomm and provides the following answers.

- Answer to Q1:  For the physical layer of the LTE air-interface, there is no requirement or capability defined regarding the number of parallel MBMS user services or the number of bearers that the UE may be able to process. For the MBMS capable UE’s physical layer processing, requirements are defined in terms of the number of bits of a MCH transport block received within a TTI, which the UE has to be able to process. The maximum MCH transport block size, as defined in TS36.306, is UE category dependent.  The maximum MCH transport block size is defined per carrier. It is optional for an MBMS capable LTE UE to receive MBMS service simultaneously on more than one carrier. The maximum MCH transport block size, while it limits the total number of bits that can be conveyed for all MBMS services in a subframe, is defined independent of the number of MBMS bearers.

- Answer to Q2:  There is no implication for the LTE physical layer, since the maximum number of bits of a MCH transport block is independent of the number of bearers or services. There may be implication on both the network and UE complexity for processing beyond the physical layer but this aspect falls outside of RAN1’s expertise.

Discussion (Question / Comment): Panasonic: Clarify "the number of bearers that the UE may be able to process"

NEC: Clarify " It is optional for an MBMS capable LTE UE to receive MBMS service simultaneously on more than one carrier"

Decision: The document is noted. Continue offline discussion until Friday to update reply LS – (Qualcomm)

Friday 15th :

| R1-135974 | Draft Response LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | Qualcomm Inc. | (R1-135872) |
| --- | --- | --- | --- |

Decision: The document is noted and final LS is agreed in R1-136072.

## Incoming LS received during the week

| R1-136004 | 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | RAN3, Telecom Italia | = R3-132375 |
| --- | --- | --- | --- |

Answers related to RAN3 - nothing special relevant to RAN1.

Decision: The document is noted.

# UTRA

| R1-135991 | Chairpersons’ summary of the UMTS sessions | UMTS Session Chairpersons (Karri Ranta-aho, NSN, Carmela Cozzo, Huawei, Matthew Baker, Alcatel-Lucent) |  |
| --- | --- | --- | --- |

The document was presented by the relevant chairpersons and provides the outcomes of their respective HSPA sessions.

Decision: The document is endorsed and content is incorporated as below.

## Maintenance of UTRA Releases 4 – 11

### FDD

| R1-135618 | 25.214 CR DRAFT (Rel-11, F) Allow (De)activate DRX / DTX with HS-SCCH Order in Multiflow Mode with Intra-NodeB Operation | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: RAN1 agrees that the proposed change may lead to additional issues, and finds it easier for RAN2 to remove the possibility of using HS-SCCH orders for CPC DTX/DRX activation and deactivation when the UE is in Multiflow mode. LS to RAN2 to be sent in R1-135986.

Email review until 22nd of November, Junqiang (ZTE)

### TDD

None.

## DCH Enhancements

WID in RP-131357. The enhancements are intended to target AMR voice over DCH, and/or SRB over DCH (where applicable.)

### DL transport channel processing and multiplexing

| R1-135775 | On the need for Joint Coding of Class-A,B,C vocoder bits | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

- Proposal 1: Agree that class-A, class-B, and (if present) class-C bits from AMR vocoder shall be concatenated into a single transport block and protected by a common CRC and jointly encoded for transmission on enhanced DCH

Decision: The document is noted.

Conclusion: For DL, when FET is operational, either concatenate AMR Class A, B and C bits, or concatenate AMR class A, B, C and DCCH bits. UL FFS.

| R1-135438 | Discussion on DCH Enhancement Solutions | MediaTek Inc. |  |
| --- | --- | --- | --- |

- Proposal 1 : Agree the proposed encoding chain “interleave and {repeat or truncate}” shall be standardized.

- Proposal 2 : Agree DCCH shall be concatenated with speech DTCH.

Decision: The document is noted.

| R1-135440 | A Hybrid Design for DCH Enhancements | MediaTek Inc. |  |
| --- | --- | --- | --- |

- Proposal 1: TPC rate shall be dynamic and it can be reduced to around 750 Hz

- Proposal 2: Adopt the new downlink rate matching/interleaving encoding chain to be standardized for DCH enhancements.

Decision: The document is noted.

| R1-135776 | Analysis of Rate-Matching and Interleaving Enhancements | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

- Proposal 1: For voice over downlink DCH Enhancements, the R99 rate-matching shall be used except that the RM attributes of the transport channel carrying DCCH shall be treated as zero while running this algorithm whenever that transport channel does not deliver a DCCH block (i.e., pseudo-flexible rate-matching).

- Proposal 2: For voice over uplink DCH Enhancements, the DTCH shall be mapped to 10ms TTI and every DTCH packet shall be repeated prior to start of physical-layer procedures (CRC attachment, encoding, etc.) (i.e., agree on compress-repeat scheme.)

Decision: The document is noted.

For downlink:

- Option 1: Concatenate DTCH and DCCH and process both as a single block, rate match by truncating or repeating bits

- Option 2: Process DTCH and DCCH separately, when DCCH is absent consider its rate matching attribute to be zero and rate match DTCH to fill all the bits on DPCH

Final decision to be taken in RAN1#76.

| R1-135439 | System Level Performance for FET Design | MediaTek Inc. |  |
| --- | --- | --- | --- |

- ...it can be observed that Mechanism B (“Downlink Frame Early Termination (FET) Option 2” + “Removal of dedicated pilots” + “DL ACK Indication for UL Frame Early Termination Option 3”) has the best power saving capability, which achieves the highest cell throughput

Decision: The document is noted.

| R1-135777 | Design of DL DCH Enhancements | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

- Proposal 1: Voice packets shall use 20ms TTI on downlink for DCH Enhancements.

- Proposal 2: The UE can be configured to meet a specified BLER target at a specified decoding attempt.

Decision: The document is noted.

Conclusion: Voice packets shall use 20ms TTI on downlink for DCH Enhancements.

### DL control channel design

#### DL DPCCH slot format optimization

| R1-135778 | DL DPCH slot formats for DCH Enhancements | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

- Proposal 1:  Agree that only slot formats 19, 20, 22, where TPC fields are moved to the end of the slot, are further considered as candidates for DL DPCH slot format.

Decision: The document is noted.

| R1-135680 | Considerations on DL DPCH Slot Format for DCH enhancement | Huawei,HiSilicon |  |
| --- | --- | --- | --- |

- Proposal: DL DPCH slot format #19 and #20(#22) are adopted as enhanced DL DPCH slot formats.

Decision: The document is noted.

Conclusion: If the ACK-channel design does not use TPC symbols on DL DPCH, then the slot formats 19, 20 and 22 are adopted as enhanced DL DPCH slot formats. If the ACK-channel design uses TPC symbols on DL DPCH then either slot formats {17, 18, 21} or slot formats {19, 20, 22} will be chosen.

#### DL ACK indication for UL frame early termination

| R1-135779 | DL ACK Performance in DCH Enhancements | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

- We have presented an initial analysis of 1-slot E-HICH-like on-off-keyed Ack/Nack and 1-symbol BPSK and on-off-keyed Ack/Nack. The 1-symbol approach should use BPSK, since on-off keying requires prohibitively high power offsets.

Decision: The document is noted.

### UL transport channel processing and multiplexing

None.

### UL control channel design

#### UL DPCCH slot format optimization

None.

#### UL ACK indication for DL frame early termination

| R1-135619 | Uplink ACK Channel Design for DL FET | ZTE |  |
| --- | --- | --- | --- |

- Proposal 1:	Further evaluate the design option 1 [new channel] and option 2 [TDM of ACK and TFCI in DPCCH] as potential candidates for FET ACK indication design for DL FET.

Decision: The document is noted.

### Other

| R1-135441 | Remaining Issues of DCH Enhancement | MediaTek Inc. |  |
| --- | --- | --- | --- |

- Proposal 1: TFCI performance has to be improved in order to guarantee the BLER convergence and consider the uplink transmission with FET.

- Proposal 2: The effect of abnormal power control should be considered into simulation for practical performance evaluation. Alternatively, solutions to prevent abnormal power control should be investigated.

- Proposal 3: It has to be studied further that how to determine the target BLER in modified OLPC so that TTI BLER is smaller than performance requirement.

Decision: The document is noted.

## Study on UMTS Heterogeneous Networks

SID in RP-121436, Open issues in RP-131347

### E-DCH decoupling

| R1-135622 | Simulation Result for E-DCH Decoupling combined with CIO in UMTS HetNet | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135681 | DL system performance with E-DCH decoupling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135682 | UL system performance with E-DCH decoupling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135683 | Further considerations on E-DCH decoupling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135867 | Comparison of E-DCH decoupling and early Serving Cell Change towards LPN | NSN | (R1-135866) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135712 | System Level Simulation and SI Reliability Results for E-DCH decoupling | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Control channels

| R1-135621 | E-DCH Decoupling for Legacy UE | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135711 | HSUPA DL control channels reliability in E-DCH decoupling | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

TP - Summarize the results from R1-135712, R1-135681,R1-135682 and R1-135683. For control channel reliability related to range of operation of EDCH decoupling, refer to existing control channel evaluation in the TR.

Add a section on conclusion for E-DCH decoupling operation.

Email discussion on summary results for EDCH decoupling until Wed Nov 20 – (R1-135989) NSN (Ryszard)

### Network assisted interference cancellation

| R1-135557 | Summary of email discussion on summary of link results for different IC receivers [74bis-15] | BlackBerry UK Limited |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135146 | System level simulation for NAIC in HetNet | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted. Revise in R1-135868, capturing the TP part from R1-135146, including some rewording of the second bullet.

Friday 15th

| R1-135868 | System level simulation for NAIC in HetNet | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Shin Horng Wong from ALU and proposes:

- Observation 1: UEs with NAIC receivers can operate with a larger CIO range compared to UEs using non-NAIC receivers.

- Observation 2: The benefit of NAIC can be further improved using optimised biasing compared to using only fixed biasing.

- Observation 3: There is no significant benefit in using NAIC under low load.

Discussion (Question / Comment): Qualcomm  these observations should be captured in the TR.

Decision: The document is noted and content is agreed for inclusion in TR.

| R1-135685 | Pre-decoding IC results with fixed MCS interference | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Coordinated scheduling

| R1-135782 | Link Performance of RRS In Hetnet | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135783 | System Performance of RRS in Hetnet | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

Email discussion until Wed Nov 20. Check results in R1-135782 and R1-135783.  Qualcomm (Arjun)

| R1-135684 | TP on simulation results of coordinated scheduling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted. Revise in R1-135869, adding a sentence about the impact on macro capacity.

| R1-135869 | TP on simulation results of coordinated scheduling | Huawei, HiSilicon | (R1-135684) |
| --- | --- | --- | --- |

| R1-135730 | Text Proposal on Benefit of Network Assisted Interference Cancellation for UEs with Single Receive Antenna | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted. Revise in R1-135870, adding a sentence about the impact on macro capacity.

| R1-135988 | Text Proposal on Benefit of Network Assisted Interference Cancellation for UEs with Single Receive Antenna | Ericsson | (R1-135870) |
| --- | --- | --- | --- |

| R1-135784 | TP on System Performance of Post Decoding NAIC | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135686 | Performance of MIMO with NAIC | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135687 | TP on MIMO with NAIC | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135713 | Network Assistance for Interference Suppressing Receivers | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

### Impacts of combined cells

| R1-135731 | Legacy UE Performance in Combined Cell Accounting for Differential Propagation Delays | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135732 | Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135733 | Link Level Analysis of Spatial Reuse Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135734 | System Level Analysis of Spatial Reuse Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135735 | Text Proposal on Legacy UE Performance in Combined Cell Accounting for Differential Propagation Delays | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135736 | Text Proposal on Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135737 | Text Proposal on Link Level Analysis of Spatial Reuse Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135738 | Text Proposal on System Level Analysis of Spatial Reuse Performance for Legacy UE in Combined Cell | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135785 | Impact of Combined Cells with F-PICH on Legacy Users | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135786 | Performance of Spatial Reuse with F-PICH | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Email discussion: TP summarizing the results in R1-135785 and R1-135786 until Wed Nov 20. Qualcomm (Arjun).

### Other

| R1-135688 | On the reliability of F-DPCH from the non-serving LPN | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is agreed.

| R1-135690 | 25.800 CR0001 (Rel-12, B) Update of Technical Report on UMTS Heterogeneous Networks | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135787 | Performance of Co-located Combined Cells | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Prepare the conclusion in R1-135990

| R1-135990 | TP on Conclusions for UMTS HetNet | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document is for email discussion until Wed Nov 20. Huawei (Carmela).

## Study on Scalable UMTS

SID in RP-130221. Open issues in RP-131346.

### System level evaluation using bursty traffic

| R1-135741 | Standalone System Simulation Results for Band I | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135742 | Downlink System Simulation Results for Time-Dilated UMTS Considering Inter-Carrier Leakage - Equal PSD | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135743 | Downlink System Simulation Results for Time-Dilated UMTS Considering Inter-Carrier Leakage - Equal Carrier Power | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135788 | Downlink System Simulation Results for Multicarrier TD-UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135739 | On Uplink System Simulations | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135740 | On the System Simulation Scenarios for Evaluating Time-Dilated UMTS | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135691 | TP on system level evaluation results for Scalable UMTS - bursty traffic | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Add the number of users supported for each scaling factor.

Need to discuss where to capture these results in the TR.

| R1-135744 | Text Proposal on Downlink System Simulation Results for Time-Dilated UMTS Considering Inter-Carrier Leakage - Equal PSD | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135745 | Text Proposal on Downlink System Simulation Results for Time-Dilated UMTS Considering Inter-Carrier Leakage - Equal Carrier Power | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135789 | TP on System Simulation Results for Multicarrier TD-UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement: Add a sentence that all the users in these evaluations are assumed to be capable of carrier aggregation, and the performance has not been studied with lower proportions of carrier aggregation capable UEs.

Updated TP is agreed in R1-135914.

| R1-135790 | TP on System level evaluation results for Stand-alone Time-Dilated UMTS - bursty traffic | Qualcomm Incorporated, Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted. Ericsson is grateful but does not co-source it.

Draft a new TP for the beginning of section 7 capturing general comments about the interpretation of the results / deployment philosophy to capture the deleted text from 5790 (Ericsson).

Ericsson to update the TP with corrections, plus including tables to replace figures z4 and y6; also include corresponding text for Band I.

| R1-135917 | TP on System level evaluation results for Stand-alone Time-Dilated UMTS - bursty traffic | Ericsson, Qualcomm | (R1-135790) |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135791 | TP on System level evaluation results for Stand-alone Time-Dilated UMTS - full buffer | Qualcomm Incorporated, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted. Text seems agreeable; discuss later where in TR to put it.

Status of system level evaluations:

The “System level evaluation using bursty traffic” open issue is complete apart from the following aspects which are remaining:

- different levels of UE penetration for non-full buffer multicarrier scenario have not been studied

- uplink evaluations have only been carried out for one operating point

  - discussion needed on how to evaluate carriers with different numbers of users, e.g. uplink system level evaluations using TDM scheduling have not been carried out.

Capture in SR.

| R1-135925 | Way Forward on System level evaluations for Time-Dilated UMTS | Qualcomm | R1-135926 |
| --- | --- | --- | --- |

Decision: The document is noted. Change title of section in Proposal 2 to “Less Relevant Simulation Results”

Proposal: Include the following sentence in the conclusions

- Full buffer simulation results comparing time dilated UMTS and UMTS spectral efficiencies are in Section xxx.

Agreed with the above modifications. Corresponding TP to be prepared in R1-135926.

| R1-135926 | TP on System level evaluation results | Qualcomm Incorporated | (R1-135925) |
| --- | --- | --- | --- |

Decision: The document is for email checking/approval by Tuesday 19th November.

### Evaluation of user plane latency and implications to user experience

Completed.

### Evaluation of the impacts of time-dilated UMTS to UE performance

| R1-135792 | TP on Impact of Time Dilated UMTS on CELL_FACH | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted and is agreed for inclusion in the TR with the addition of the following sentence in section 7.1.x.1.2:

“The resources needed for control signalling for time dilated UMTS is the same as for UMTS despite supporting half the number of users.”

Completed.

### Evaluation of the impacts of time-dilated UMTS on network performance and optimization

| R1-135692 | TP on scalable UMTS impacts on network performance | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is withdrawn – superseded by TP agreed by email.

Completed.

### Coverage analysis

Completed.

### Scalable BW UMTS by filtering

| R1-135751 | Comparison Between Time-Dilated and Filtered UMTS | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

If further work were to be done on scalable UMTS, the feasibility of filtered UMTS and its ability to overcome some of the main drawbacks of time-dilation and narrow band UMTS in general should be investigated.

| R1-135693 | Initial link level simulation results for scalable BW UMTS by filtering | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135714 | Scalable Bandwidth UMTS by Filtering | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135753 | Filtered UMTS - Initial Link Level Simulation Results | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135795 | Link analysis of Scalable UMTS with Filtering for HSPA | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135796 | Link Performance of AMR12.2k in Scalable UMTS with Filtering | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135715 | TP on uplink performance of Scalable Bandwidth UMTS by Filtering | NSN | R1-136039 |
| --- | --- | --- | --- |

Decision: The document is noted. Add details of simulation setup including receiver and power control.

TP is updated and agreed in R1-136039 for inclusion in the TR.

| R1-135716 | TP on Scalable Bandwidth UMTS by Filtering | NSN | R1-136040 |
| --- | --- | --- | --- |

Decision: The document is noted. Remove baseband aspects for both NB and UE.

TP is updated and agreed in R1-136040 for inclusion in the TR.

| R1-135797 | TP on Performance of Scalable UMTS with Filtering | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted. Omit system model. Otherwise agreed for inclusion in TR.

Not treated.

| R1-135725 | RAN2 impacts for Scalable Bandwidth UMTS by Filtering | NSN |  |
| --- | --- | --- | --- |
| R1-135726 | RAN4 impacts for Scalable Bandwidth UMTS by Filtering | NSN |  |
| R1-135746 | Impact of Filtered UMTS on UE Core Requirements | Ericsson |  |
| R1-135747 | Impact of Filtered UMTS on UE Performance Requirements | Ericsson |  |
| R1-135748 | Impact of Filtered UMTS on RRM Requirements | Ericsson |  |
| R1-135749 | Impact of Filtered UMTS on BS Core Requirements | Ericsson |  |
| R1-135750 | Impact of Filtered UMTS on BS Performance Requirements | Ericsson |  |
| R1-135752 | Comparison Between Time-Dilated and Filtered UMTS - RAN2 Aspects | Ericsson |  |

### Other

| R1-135626 | Draft TR 25.701 of Scalable UMTS | China Unicom |  |
| --- | --- | --- | --- |

Decision: The document is noted. To be updated in R1-135969 – as v1.0.2.

| R1-135969 | TR 25.701 v1.0.2 of Scalable UMTS | China Unicom | (R1-135626) |
| --- | --- | --- | --- |

Check for consistency of terminology “scalable UMTS” vs. specific solutions for scalable UMTS – then update the TR as v.1.0.3 in R1-135916..

| R1-135916 | TR 25.701 v1.0.3 of Scalable UMTS | China Unicom | (R1-135969) R1-136041 |
| --- | --- | --- | --- |

Decision: The document is agreed as v1.1.0 in R1-136041

| R1-135627 | TP on overall conclusions for Scalable UMTS | China Unicom |  |
| --- | --- | --- | --- |

Not treated as incorporated into 5798.

| R1-135815 | Conclusions on Scalable UMTS | Ericsson, NSN, Broadcom Corporation, MediaTek Inc., Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Intel Corporation | (R1-135754) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135798 | TP on Conclusions for Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted. Also supported by China Unicom, Huawei, HiSilicon.

Work offline until Friday morning to see how 5815 can be modified to reach consensus - R1-135927. Capture both throughput and spectral efficiency in relation to link level conclusions. Try to present a more balanced version of the last sentence.

Text as modified in R1-136042 is agreed up to “…with intermediate penetration levels were not conducted” at the top of page 4.

Email discussion/approval until Friday 22nd November on only the text from this point onwards.

After completion of email approvals, email approval of next update of TR until Wednesday 27th November.

## Study on Further EUL Enhancements

SID in RP-130347. See LS in R2-130758.

| R1-135987 | LS to RAN2 on RAN1 input on Further EUL enhancements TR25.700 | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and LS is agreed.

### Enabling high user bitrates in a mixed-traffic scenario

| R1-135757 | On Improved TDM Scheduling Schemes | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Create a TP of table 4 of R1-133738 to motivate for enhanced TDM, generalize the ‘grant detection’ to ‘improved TDM scheduling schemes’

| R1-135856 | TP on the benefit of improved TDM Scheduling Schemes | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135695 | Link level simulation results for lean carrier | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135755 | Further Link Level Evaluations of Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135696 | TP on link level simulation results for lean carrier | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135756 | Text Proposal on Link Level Evaluations of Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: The proponents to compile the results in a single TP

| R1-135852 | Text Proposal on Link Level Evaluations of Lean Carrier | Ericsson, Huawei |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135698 | TP on specifications and implementation impacts for lean carrier operation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Huawei and Ericsson to discuss the content of the text based on R1-135698 to be liaised to RAN2.

| R1-135853 | TP on specifications and implementation impacts for lean carrier operation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135694 | TP on system level simulation assumptions for lean carrier | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135761 | Text Proposal on System Level Simulation Assumptions for Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: The assumptions in 5694 are agreed with the additional sentence above the first table: “The system simulation assumptions for Lean carrier are shown in Table 1 and Table 2. The traffic model in Table 2 is to be seen as a guideline and other traffic models may be used as well.”, update in 5985

| R1-135985 | TP on system level simulation assumptions for lean carrier | Huawei, HiSilicon, Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135758 | System Level Evaluations of Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135759 | Text Proposal on System Level Evaluations of Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Add more explanation on the variation of the cell throughput and RoT, and if possible, also characterization of how much inter-cell interference is seen on average.

| R1-135854 | TP on System Level Results for Lean Carrier | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135697 | Open issues of lean carrier operation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135760 | Text Proposal on Lean Carrier Conclusions | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Ericsson to collect detailed feedback and produce an update in 5855. Check how to capture the issues raised in 5697, if needed.

| R1-135855 | Text Proposal on Lean Carrier Conclusions | Ericsson, NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

### Rate Adaptation to support improved power and rate control for high rates

| R1-135623 | Further Link Evaluation for HSUPA Rate Adaptation | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135851 | System simulation results for uplink rate adaptation | Huawei, HiSilicon | (R1-135699) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135718 | Further clarification regarding simulation results for Novel Rate Adaptation schemes | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135721 | TP on LLS results for Novel Rate Adaptation | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Collect detailed comments and provide update of 5721 in 5858, consider including also results from 5623, if the raised questions can be explained.

| R1-135858 | Link Level Simulation Results for Improved Rate Adaptation | NSN, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135722 | TP on SLS results for Novel Rate Adaptation | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Collect detailed comments to 5722 and provide update in 5859, include results from 5851 and 5799. In addition to results in 5722, provide explanation of why RoT overshoot is larger with modified 2-loop and add a statement that if the modified 2-loop is operated differently is may be possible that the overshoot is mitigated.

| R1-135859 | TP on System Level Simulation Results for Improved Rate Adaptation | NSN, Huawei, HiSilicon, Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135799 | Updated results for Rate Adaptation in HSUPA | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135719 | TP on SHO operation description in HSUPA with Novel Rate Adaptation | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: The TP is agreed. When including to the TR, consider replacing the figures with references to the figures already existing in the TR. Update to 5984

| R1-135984 | TP on SHO Operation in HSUPA with Rate Adaptation | NSN, Ericsson |  |
| --- | --- | --- | --- |

| R1-135720 | TP on SHO operation simulation results for Novel Rate Adaptation | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135717 | TP on conclusions on Novel Rate Adaptation scheme motivation and benefits | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Add ranges of gains in the conclusions, provide update in 5861.

| R1-135861 | TP on Conclusions on Rate Adaptation | NSN, Ericsson, Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

### Reduced UL control channel overhead for HSPA

| R1-135723 | LLS results for DPCCH SIR target setting for overhead reduction study | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135700 | TP on updated simulation assumptions for uplink overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

DPCCH overhead reduction

| R1-135624 | Uplink DPCCH Overhead Reduction | ZTE |  |
| --- | --- | --- | --- |

- Proposal 1:	The method that the uplink DPCCH is not transmitted in INACTIVE HARQ processes is adopted as a solution for control channel overhead reduction.

- Proposal 2:	NodeB controls the method switched ON or OFF via HS-SCCH Order.

Decision: The document is noted.

| R1-135625 | TP on Uplink DPCCH Overhead Reduction | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Due to the late introduction of the concept and lack of performance analysis, the TP is not included in the study item TR. Should the study item be extended, or a work item with a scope fitting for this concept be opened, more analysis can be provided to justify the concept.

HS-DPCCH overhead reduction

| R1-135701 | Updated system simulation results for HS-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135702 | TP on system simulation results for HS-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Update the results with UE throughput figures. Use the wording: “The UL performance gains are mainly due to the uplink power saving if the UE switches to a different CQI reporting cycle in case of no downlink transmission.” In the paragraph above Figure 5.X.3.2-1. Also add unit to the caption text of the figure.

| R1-135862 | TP on system simulation results for HS-DPCCH overhead reduction | Huawei, HiSilicon | (R1-135702) |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

E-DPCCH overhead reduction

| R1-135703 | Updated system simulation results for E-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135704 | TP on system simulation results for E-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Updated results in 5865

| R1-135865 | TP on system simulation results for E-DPCCH overhead reduction | Huawei, HiSilicon | (R1-135704) |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

| R1-135705 | TP on conclusions for reduced UL control channel overhead | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: consider input provided and in addition include the following:

- HS-DPCCH overhead reduction was considered in context of multi-RAB, and the evaluation was done for Multi-RAB.

- E-DPCCH overhead reduction results need to be considered (as discussed with 5704)

| R1-135863 | TP on conclusions for reduced UL control channel overhead | Huawei, HiSilicon, NSN | (R1-135857) |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed.

### Low-complexity uplink load balancing solutions

No inputs.

## Study on Enhanced Broadcast of System Information

SID in RP-131386.

Mr Chair's recommendation: The study should focus on identifying the need for additional broadcast capacity, and should not work on the technical solutions for enhancing the broadcast capacity.

No inputs

## Other

| R1-135050 | 25.222 CR0213r1 (Rel-12, B) Introduction of HSPA signalling enhancements for more efficient resource usage for LCR TDD | CATT, TD Tech, Potevio, New Postcom, CATR |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed.

| R1-135051 | 25.224 CR0273r1 (Rel-12, B) Introduction of HSPA signalling enhancements for more efficient resource usage for LCR TDD | CATT, TD Tech, Potevio, New Postcom, CATR |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed.

| R1-135850 | Multiflow and CLTD with the assisting cell feedback | NSN | (R1-135724) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135762 | Multiflow and Closed Loop TX Diversity | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion: Draft an LS to RAN2 indicating that in scenarios where there is a large imbalance between the serving and assisting cells in inter-site Multiflow, the ability for the network to choose the assisting cell as the CLTD feedback-providing cell may allow for the network to configure a lower HS-DPCCH gain factor in those scenarios.

| R1-135864 | LS on Multiflow and CLTD with assisting cell feedback | RAN1, NSN |  |
| --- | --- | --- | --- |

Decision: The document is for email approval until November 22nd.

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 11

36.211-related

non-MBSFN region

| R1-135611 | Clarification on non-MBSFN Region | Intel, Panasonic, Fujitsu, Huawei, HiSilicon, NEC, Texas Instruments, Qualcomm Inc., CATT, Interdigital, Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Seunghee Han from Intel and provides a summary of the offline email discussion on the non-MBSFN region and suggests TPs to 36.211 and 36.213 Rel-11 specs.

Discussion (Question / Comment): Also supported by BlackBerry, Samsung.

Decision: The document is noted.

| R1-135818 | Discussion on specifications for non-MBSFN region | NEC | (R1-135272) |
| --- | --- | --- | --- |

The document was presented by … from … and deals with the issue that current specifications may not allow UE to use higher layer parameter non-MBSFNregionLength in deriving non-MBSFN region of MBSFN subframes with PMCH. And also whether the PDCCH region length (derived from CFI / PCFICH) is same as the higher layer parameter.

It is proposed to agree on TP in [R1-135611] for Rel-12 with magic words to clarify agreement in Rel-9 and remove potential ambiguity which might cause unnecessary discussion again.

Decision: The document is noted.

Conclusion: Proposal 1 in R1-135611 is agreed.

RAN1 will clarify UE behaviour and continue offline discussion until Friday aiming at clarifying proposals 2 and 3 – (Intel)

Friday 15th

| R1-136074 | Proposal on PMCH mapping in an MBSFN subframe | Intel |  |
| --- | --- | --- | --- |

The document was presented by Seunghee Han from Intel and provides proposals based on the offline discussions..

Discussion (Question / Comment): Alcatel Lucent cannot agree on issue 2: "Allow a UE to utilize non-MBSFNregionLength to determine the length of the non-MBSFN region" - no change on UE behaviour

Fujitsu: issue 3 could be agreed in principle but may require the wording to be refined

Decision: The document is noted.

| R1-136075 | 36.211 CR0186 (Rel-11, F) Correction on the derivation of the non-MBSFN region by PCFICH | Intel, Panasonic, Fujitsu, Huawei, HiSilicon, NEC, Texas Instruments, Qualcomm Inc., CATT, Interdigital, Ericsson, Samsung, BlackBerry |  |
| --- | --- | --- | --- |

The 36.211 CR is related to issue 1 (proposal 1 in R1-135611 agreed earlier in the week.

Decision: The document is noted and CR is agreed.

| R1-136076 | 36.213 CR0446 (Rel-11, F) Correction on deriving the length of the non-MBSFN region | Intel, Panasonic, Fujitsu, Huawei, HiSilicon, NEC, Texas Instruments, Qualcomm Inc., CATT, Interdigital, Ericsson, Samsung, BlackBerry |  |
| --- | --- | --- | --- |

Alcatel Lucent: The CR should be Cat. B as this is a new feature

LGE, Broadcom, NSN, Nokia, Cisco: also support the proposal

Alcatel Lucent withdrew their objection for the sake of progress

Decision: The document is noted and CR is agreed.

Finally it is noted that Proposal 3 in R1-136074 is agreed in principle -  wording to be revisited. Check it in RAN1#76.

36.212-related

| R1-135183 | 36.212 CR0154R3 (Rel-12, B) Introduction of Rel 12 feature for Downlink MIMO Enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Min Zhang from ALU and introduces the support downlink MIMO enhancement with enhanced CSI feedback.

Decision: The document is noted and CR is agreed.

36.213-related

| R1-135097 | Clarification on Downlink Assignment using EPDCCH | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Seunghee Han from Intel and proposes to agree on one of the following two options:

- Option 1: Specify that the UE behaviour is unspecified when a DL assignment using EPDCCH fully overlaps with the PRBs carrying the EPDCCH.

- Option 2: Specify that the UE assumes this is an inconsistent case when a DL assignment using EPDCCH fully overlaps with the PRBs carrying the EPDCCH.

Discussion (Question / Comment): ZTE: how can we specify an unspecified behaviour?  not pushing any UE behaviour

Ericsson, Huawei: no need to change current specification

Fujitsu: Network won't assume any UE behaviour anyway - RAN1 understanding is that UE behaviour is not specified sounds a good conclusion.

Decision: The document is noted. It is concluded that RAN1 understanding is that UE behaviour is not specified.

| R1-135147 | 36.213 CR0444 (Rel-10,F) Correction on CSI reporting type and parameters | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Min Zhang from ALU and proposes to fix the parameter defined for the first and the second codebook indices of 8Tx as they not aligned with corresponding codebook tables, e.g. the first codebook index is re-presented by i1.or ic1 or n1  in 36.213 specification. It is proposed that:

- ic1 and n1 are replaced with i1

- ic2 and n2 are replaced with i2

Discussion (Question / Comment): CATT WI code is missing in the cover sheet.

Decision: The document is noted and CR0444 is agreed as rev 1 in R1-135888 assuming WI code is added.

| R1-135148 | 36.213 CR0445 (Rel-11,A) Correction on CSI reporting type and parameters | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR0445 is agreed as rev 1 in R1-135889 assuming WI code is added.

Friday 15th : It is noted that WI code is still missing in rev 1 of the CR - agreed as rev 2 in R1-136073.

| R1-135184 | 36.213 CR0431R4 (Rel-12, B) Introduction of Rel 12 feature for Downlink MIMO Enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Min Zhang from ALU and introduces the support downlink MIMO enhancement with enhanced CSI feedback.

Decision: The document is noted. Continue offline discussion until Friday to update it including followings – (ALU)

- Page 13,

  - When PTI is not transmitted (due to not being configured) or the most recently transmitted PTI is equal to 1 for a UE configured in transmission mode 8, or for a UE configured in transmission mode 9 or for a UE configured in transmission mode 10 without a ‘RI-reference CSI process’ for a CSI process, or the transmitted PTI is equal to 1 reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with a ‘RI-reference CSI process’ for the CSI process, or the transmitted PTI is equal to 1 for a ‘RI-reference CSI process’ reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with the ‘RI-reference CSI process’ for the CSI process, and the most recent type 6 report for the CSI process is dropped:

    - The wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 8, 9, and 10) report has period, and is reported on the subframes satisfying. The integer  is defined as, where  is the number of bandwidth parts.

    - Between every two consecutive wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 8, 9, and 10) reports, the remaining  reporting instances are used in sequence for subband CQI reports on  full cycles of bandwidth parts except when the gap between two consecutive wideband CQI/PMI reports contains less than  reporting instances due to a system frame number transition to 0, in which case the UE shall not transmit the remainder of the subband CQI reports which have not been transmitted before the second of the two wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 9 and 10) reports. Each full cycle of bandwidth parts shall be in increasing order starting from bandwidth part 0 to bandwidth part . The parameter is configured by higher-layer signalling.

- Page 34,

For 2 antenna ports, each PMI value corresponds to a codebook index given in Table 6.3.4.2.3-1 [26.211] as follows:

Friday 15th

| R1-135894 | 36.213 CR0431R5 (Rel-12, B) Introduction of Rel 12 feature for Downlink MIMO Enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | (R1-135184) |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed.

eMBMS on Scell

| R1-135285 | Necessary clarifications for eMBMS reception on SCell | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Peter Gaal from Qualcomm and deals with the following topics needing clarification regarding MBMS reception:

- SI change notification on a DL-only cell

- Time synchronization of MBMS cell

- Maximum UE processing capability

- Whether it is mandatory to receive MBMS on SCell

These clarifications should be adopted in the Rel-12 specification. If agreed, it should be further discussed which, if any, of these clarifications might be applicable to earlier releases.

Decision: The document is noted.

| R1-135773 | eMBMS on Scell | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Mattias Frenne from Ericsson and suggests agreeing on the following:

- CR0443 in R1-134984 is applicable to both Rel-10 UEs and Rel-11 UEs

- UEs can expect that paging related to MBMS specific SI change is transmitted on the same cell where it receives PMCH

And to discuss further how to capture the following observation:

- When receiving MBMS on a non-serving cell simultaneously to a connection to a serving cell, the UE cannot assume that the time synchronization requirements of CA hold between these cells

Discussion (Question / Comment): Mr Chair requested to highlight the differences with R1-135285

Ericsson: Paging is aligned with Qualcomm's contribution - Time alignment is where Ericsson has different proposal.

Decision: The document is noted.. Continue offline discussion until Friday – (Qualcomm)

- What kind of assumptions can be considered for TBS processing capability and/or timing synchronization?

  - It is noted to distinguish above assumptions between unicast and broadcast

- For timing synchronization, what kind of benefit can we have for UE implementation?

- Which Release should be considered?

Friday 15th : No progress during the week - continue discussion until RAN1#76.

## LTE Release 12

### Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation

WID in RP-121772 - LTE_TDD_eIMTA-Core

| R1-136028 | Summary of Ad-hoc session on Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and provides the outcomes of eIMTA session.

Discussion (Question / Comment): CATT commented the conclusion of R1-135960 in section 6.2.1.1.3 - as stated in the summary, one could understand that RAN3 may decide on DL HARQ reference configuration - this is not the case

Further discussion in RAN3 is not precluded - there is no conclusion in RAN1 that such signalling is needed but signalling could be used in RAN3.

Decision: The document is endorsed and incorporated below (with change in section 6.2.1.1.3.

#### Remaining details of interference mitigation schemes

##### Remaining details of UL power control

Open Loop Power Control

| R1-135010 | Further study on open loop power control for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135452 | Remaining Details of Uplink Power Control for TDD eIMTA | LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document (Section 2.1 and Section 2.2 only) is noted.

| R1-135558 | Remaining details for enhanced UL power control for TDD eIMTA | Nokia, NSN |  |
| --- | --- | --- | --- |

Decision: The document (Section 2 and Section 3 only) is noted.

Closed loop Power Control, including PHR

| R1-135053 | PHR in TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135816 | Remaining details of uplink power control in TDD eIMTA | CATT | (R1-135052) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135185 | UL power control for eIMTA | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135287 | Configurable UL TPC accumulation | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135966 | Way Forward on subframe set dependent PHR for TDD eIMTA | Huawei, HiSilicon, Sharp, Potevio, Interdigital, ZTE, Alcatel Lucent, Alcatel Lucent Shanghai Bell, CATR, TI, MediaTek, LGE, Panasonic |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

- P0 and alpha configuration for the two subframe sets is via RRC signalling

- For power control command step size, no change relative to Rel-11

- FFS PHR related issues till RAN1#76, especially regarding whether current PHR mechanism can have PHR reports for the two subframe sets

- FFS till RAN1#76, including at least the following issues:

  - Application of power control commands

    - Alt 1: separate power control commands only

    - Alt 2: configurable between separate and joint power control commands

  - TPC timing issues, if any, for configuration #0

  - SRS power control related issues

Working assumption:

- The association of (P0, alpha) with a UL subframe is separately configured via RRC

- For PUCCH PC, no enhancements (including both over-the-air and backhaul enhancements) relative to Rel-11

| R1-135942 | Way Forward on uplink power control in TDD eIMTA | ZTE, Potevio, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted. FFS till RAN1#76.

Not treated.

| R1-135011 | Signalling on closed loop power control for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135098 | Discussion on remaining details of UL PC for eIMTA support | Intel Corporation |  |
| R1-135244 | On UL Power Control Enhancements for EIMTA | Texas Instruments |  |
| R1-135286 | UL power control for eIMTA | Qualcomm Inc. |  |
| R1-135334 | Necessity of enhanced PC for PUCCH | Sharp |  |
| R1-135335 | Signalling mechanism for enhanced UL PC for PUSCH and SRS | Sharp |  |
| R1-135336 | PHR reporting and calculation for eIMTA | Sharp |  |
| R1-135364 | Some Remaining Issues about Uplink Power Control | ZTE |  |
| R1-135387 | UL Power control enhancement in eIMTA | Panasonic | (R1-134369) |
| R1-135410 | UL power control enhancement with dynamic subframe sets for TDD eIMTA | ITRI |  |
| R1-135419 | Power headroom reporting in TDD eIMTA systems | MediaTek Inc. |  |
| R1-135499 | Remaining Details of UL Power Control in TDD eIMTA | HTC |  |
| R1-135559 | Power headroom report for enhanced UL power control in TDD eIMTA | NSN, Nokia | (R1-134508) |
| R1-135598 | On remaining details for UL power control with eIMTA | InterDigital | (R1-134556) |
| R1-135616 | UL Power Control for e-IMTA | ASUSTeK |  |
| R1-135638 | Remaining details for enhanced uplink power control | Ericsson |  |
| R1-135814 | Remaining details of UL power control | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-135149) |

##### Remaining details of CSI feedback

| R1-135054 | Summary of email discussion [74bis-02] CSI measurement and report for eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- When a UE decodes explicit L1 signalling of reconfiguration correctly and detects a valid UL-DL configuration, the UE shall measure CSI only within the subframes indicated as DL subframe or special subframe by the explicit L1 signalling of reconfiguration

- If UE does not detect L1 signalling conveying a valid UL-DL configuration for a radio frame, the UE shall measure CSI only within the subframes indicated as DL subframe or special subframe by SIB configuration

- Aperiodic CSI feedback shall be supported for all the configured measurement subframe set(s)

Proposal:

- For TM10, a UE can be configured with up to two CSI processes, each associated with a CSI-IM resource configuration

  - If two CSI processes are configured, the two CSI-RS processes are associated with a same NZP CSI-RS configuration

  - If two CSI processes are configured, a 2-bit A-CSI field is present in USS DCI

    - Higher layer signalling configures the CSI measurement subframe set to be reported for each state using the 2-bit CSI request field

  - For CSS DCI, aperiodic CSI is also supported, where the CSI measurement set(s) to be reported is based on the state “01” in the table specified for the 2-bit CSI request field for the UEs capable of 2 or more CSI-RS processes

    - If the UE is only capable of one CSI-RS process, only the first CSI measurement set is reported

Thursday 14th

| R1-135954 | Way Forward on Interference Measurement for eIMTA | ZTE, Huawei, HiSilicon, LG Electronics, Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted. Continue discussion until RAN1#76.

Not treated.

| R1-135012 | CSI enhancements for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135055 | CSI feedback in TDD eIMTA | CATT |  |
| R1-135099 | Discussion on CSI enhancements for eIMTA support | Intel Corporation |  |
| R1-135150 | Remaining details of CSI feedback | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-135186 | Discussion on CSI feedback for eIMTA | Samsung |  |
| R1-135245 | Views on CSI measurement for LTE TDD eIMTA | Texas Instruments |  |
| R1-135288 | CSI measurement and reporting in eIMTA | Qualcomm Inc. |  |
| R1-135337 | Enhancement of CSI reporting for eIMTA | Sharp |  |
| R1-135338 | CSI subframe measurement set configuration for eIMTA | Sharp |  |
| R1-135367 | CSI feedback enhancement for TDD eIMTA | ZTE |  |
| R1-135444 | Discussion on CSI feedback for eIMTA | Potevio |  |
| R1-135453 | Remaining Details of CSI Measurement and Report for TDD eIMTA | LG Electronics |  |
| R1-135500 | On aperiodic CSI reporting in TDD eIMTA systems | HTC |  |
| R1-135535 | CSI measurement and reporting in TDD eIMTA | Broadcom Corporation | (R1-134476) |
| R1-135639 | Remaining details of CSI enhancement for eIMTA | Ericsson |  |

##### Other

| R1-135960 | Way Forward on backhaul signalling for eIMTA | LG Electronics, Nokia, NSN, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Potevio |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

- No consensus to introduce DL HARQ reference configuration via X2 interface in RAN1

  - It is up to RAN3 to discuss on this issue

Not treated.

| R1-135056 | Further discussion on backhaul signalling details in TDD eIMTA | CATT |  |
| --- | --- | --- | --- |
| R1-135100 | Discussion on the remaining details of backhaul signalling for eIMTA Support | Intel Corporation |  |
| R1-135187 | Remaining details of backhaul signalling for eIMTA | Samsung |  |
| R1-135289 | Remaining details of backhaul signalling for eIMTA | Qualcomm Inc. |  |
| R1-135506 | Further Discussion on Backhaul Signalling for eIMTA | NTT DOCOMO |  |
| R1-135560 | On the remaining issues of necessary backhaul signalling for TDD eIMTA | NSN, Nokia |  |

#### Remaining details of signalling for TDD UL-DL reconfiguration

| R1-135013 | Physical layer signalling design for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135151 | Remaining details of signalling for TDD UL-DL reconfiguration | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135188 | Signalling for adapting TDD UL-DL configurations | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- The explicit reconfiguration DCI:

  - Alt 1: only contains the reconfiguration indication

  - Alt 2: contains the reconfiguration indication and at least one other type of information (e.g., power control)

- The number of eIMTA-RNTI configured for the UE is always 1 regardless of non-CA or CA operations

Agreement:

- The group common DCI is only in Pcell CSS for a UE

| R1-135965 | Offline discussion summary on signalling for TDD UL/DL reconfiguration | Sharp, CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135964 | Fallback operation without valid UL-DL configuration via reconfiguration DCI | Sharp, Huawei, HiSilicon, LG Electronics, Nokia, NSN, Samsung, Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- If UE detects L1 signalling conveying a valid UL-DL configuration for a radio frame,

  - UE shall monitor the non-DRX DL subframes or special subframes indicated by explicit L1 signalling

Working Assumption:

- If UE does not detect L1 signalling conveying a valid UL-DL configuration for a radio frame,

  - UE shall monitor the non-DRX DL subframes or special subframes for PDCCH or EPDCCH as indicated by SIB-1 configuration

| R1-135963 | Modification windows for periodic subframe in eIMTA | Sharp, Huawei, HiSilicon, Nokia, NSN, Potevio, Samsung, Texas Instruments |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working assumption:

- The periodicity of the reconfiguration signals includes at least the following:

  - 10, 20, 40, 80ms

- A UE shall monitor the reconfiguration signals only in downlink subframes or special subframes according to SIB1

  - The UE may be configured to monitor the reconfiguration signals in more than one subframe between two adjacent periodic instances

    - FFS which subframes

- The detected configuration is valid for  a window where the window has a duration equal to the periodicity of the reconfiguration signals

  - May subject to an application delay

- A UE shall assume the same configuration for two or more reconfiguration signals, if the two or more reconfiguration signals are detected in subframes within a window of a duration equal to the periodicity of the reconfiguration signals

  - The UE cannot assume the same configuration for reconfiguration signals detected in subframes across different windows

- FFS detailed configuration table for the reconfiguration signals – Zukang (CATT)

Thursday 14th

- FFS detailed configuration table for the reconfiguration signals – Zukang (CATT), initial draft in R1-136025

| R1-136025 | Configuration of subframes for explicit UL-DL reconfiguration DCI | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

- Companies are encouraged to check the draft and provide comments to Zukang

- Email discussion R1-136025 aiming to reach consensus till 11/29 – Zukang (CATT).

Not treated.

| R1-135057 | Remaining details of L1 signalling for UL-DL reconfiguration | CATT |  |
| --- | --- | --- | --- |
| R1-135101 | On remaining details of signalling design and HARQ feedback for eIMTA | Intel Corporation |  |
| R1-135128 | Signalling options for TDD UL-DL reconfiguration | Fujitsu |  |
| R1-135246 | Outstanding signalling details for TDD UL/DL reconfiguration | Texas Instruments |  |
| R1-135255 | Further considerations on reconfiguration signalling for eIMTA system | NEC |  |
| R1-135290 | Discussion on signalling details for TDD UL-DL reconfiguration | Qualcomm Inc. |  |
| R1-135365 | Remaining Details of Signalling for TDD UL-DL Reconfiguration | ZTE |  |
| R1-135389 | Analysis and proposal for explicit+implicit reconfiguration signalling | Panasonic | (R1-134370) |
| R1-135390 | Reconfiguration message remaining details | Panasonic |  |
| R1-135420 | Fallback solution on missing TDD UL-DL configuration | MediaTek Inc. |  |
| R1-135454 | Remaining Details of signalling for TDD eIMTA | LG Electronics |  |
| R1-135507 | Some Viewpoints about Reconfiguration Signalling Design | NTT DOCOMO |  |
| R1-135561 | On details of UL-DL reconfiguration signalling | NSN, Nokia |  |
| R1-135640 | Efficient transmission of explicit reconfiguration signalling | Ericsson |  |
| R1-135727 | Remaining issues for explicit L1 reconfiguration signalling | Sharp |  |
| R1-135728 | On UE behaviour in Fallback mode for eIMTA | Sharp |  |

Thursday 14th

| R1-135961 | Way Forward on the changeability of UL-DL switching period | LG Electronics, CATT, Ericsson, ST-Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working assumption:

- Reconfiguration between 5ms and 10ms switching point periodicity shall be supported

- FFS issues related to subframe #6, particularly,

  - How to handle the case when eIMTA is enabled, whether or not/how to support broadcast (e.g., SI/P/RA-RNTI) related operations in subframe #6 if the subframe is a special subframe indicated by SIB1 but a regular DL subframe indicated by the dynamic subframe configuration

#### Remaining HARQ details

HARQ Timing

| R1-135729 | Restrictions on UL/DL reconfiguration in eIMTA | Sharp |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

- FFS whether or not to specify a set of valid combinations between L1 signalled configurations and DL/UL HARQ reference configurations

The following working assumption is confirmed:

- Working assumption:

  - For UE configured with TDD eIMTA, uplink scheduling timing and HARQ timing follow UL-DL configuration signalled in SIB1

PUCCH formats

| R1-135247 | HARQ-ACK feedback for EIMTA | Texas Instruments |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- For UEs configured with eIMTA and CA, up to 2 CCs with DL HARQ reference configuration #5 can be supported

Conclusion:

- FFS at least the following issues:

  - Supported PUCCH format(s) for eIMTA enabled UEs and for the supported PUCCH format(s), the corresponding design details. Potential PUCCH formats include:

    - ACK/NAK bundling

    - PUCCH format 1b with channel selection

    - PUCCH format 3

  - Whether or not to standardize solutions resolving PUCCH resource collision issues, and if so, detailed solutions

  - HARQ-ACK on PUSCH related issues (e.g., DAI handling, ACK/NAK payload size, etc.)

  - For UEs configured with eIMTA and CA, how to handle the case when two or more CCs have different HARQ reference configurations for both DL and UL

| R1-135967 | Way Forward on PUCCH resource allocation for eIMTA | NEC, Sharp, Nokia, NSN, LGE |  |
| --- | --- | --- | --- |

Continue discussion until RAN1#76.

Not treated.

| R1-135014 | Discussion on HARQ operation related issues for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135015 | HARQ-ACK PUCCH resource allocation | Huawei, HiSilicon |  |
| R1-135016 | HARQ-ACK feedback on PUSCH | Huawei, HiSilicon |  |
| R1-135058 | Remaining issues on HARQ in TDD eIMTA | CATT |  |
| R1-135059 | PUCCH resource allocation for TDD eIMTA | CATT |  |
| R1-135189 | UCI payload aspects for eIMTA | Samsung |  |
| R1-135190 | Remaining issues on HARQ timing for eIMTA | Samsung |  |
| R1-135191 | HARQ-ACK feedback procedures for eIMTA | Samsung |  |
| R1-135256 | PUCCH and PHICH resource reservation for TDD eIMTA system | NEC |  |
| R1-135257 | UL&DL timing coupling issue for TDD eIMTA system | NEC |  |
| R1-135291 | eIMTA PUCCH Resource Allocation | Qualcomm Inc. |  |
| R1-135292 | eIMTA HARQ-ACK transmission | Qualcomm Inc. |  |
| R1-135339 | PUCCH resource of HARQ transmission for eIMTA | Sharp |  |
| R1-135366 | Remaining Details of HARQ | ZTE |  |
| R1-135391 | Remaining Details for HARQ Timing | Panasonic |  |
| R1-135411 | HARQ resource collision issue in TDD eIMTA system | ITRI |  |
| R1-135421 | Discussion on HARQ-ACK PUCCH resource for TDD eIMTA | MediaTek Inc. |  |
| R1-135445 | On remaining details of HARQ for eIMTA | Potevio |  |
| R1-135455 | Remaining HARQ Details for TDD eIMTA | LG Electronics |  |
| R1-135501 | HARQ details in TDD eIMTA systems | HTC |  |
| R1-135508 | Discussion on Remaining Issues for eIMTA HARQ | NTT DOCOMO |  |
| R1-135536 | PUCCH resource mapping for TDD eIMTA | Broadcom Corporation |  |
| R1-135562 | On HARQ-ACK PUCCH Formats and Resource allocation for TDD eIMTA | NSN, Nokia |  |
| R1-135563 | Remaining details on UL and DL reference configurations | NSN, Nokia |  |
| R1-135599 | On HARQ feedback and PUCCH resource allocation for eIMTA | InterDigital |  |
| R1-135641 | Remaining details for HARQ feedback mapping | Ericsson |  |

#### Other

CRS and DL transmission modes

| R1-135060 | CRS in reconfigurable subframes | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- UE can be configured by higher layers via dedicated signalling to assume MBSFN subframes in at least some SIB1-UL subframes when the SIB1-UL subframes are determined by the UE to DL subframes

- All existing DL transmission schemes are supported in eIMTA, regardless of fixed or flexible subframes

| R1-135642 | Further details for eIMTA | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document (Section 2.1 only) is noted.

Not treated.

| R1-135061 | On reconfiguration between special subframe and downlink subframe | CATT |  |
| --- | --- | --- | --- |
| R1-135192 | On other issues for eIMTA | Samsung |  |
| R1-135193 | Discussion on transmission modes to support eIMTA | Samsung |  |
| R1-135293 | DRX operation for eIMTA | Qualcomm Inc. |  |
| R1-135294 | Soft buffer management for IMTA | Qualcomm Inc. |  |
| R1-135340 | Presence of CRS in reconfigured DL subframes | Sharp |  |
| R1-135413 | Maximum number of DL HARQ processes for TDD eIMTA | ITRI |  |
| R1-135456 | Other Design Aspects for TDD eIMTA | LG Electronics |  |
| R1-135537 | Backwards compatibility in TDD eIMTA | Broadcom Corporation | (R1-134479) |
| R1-135564 | On Transmission Modes for TDD eIMTA | NSN, Nokia |  |

### Low Cost & Enhanced Coverage MTC UE for LTE

WID in RP-130848 - LC_MTC_LTE-Core

| R1-135802 | Guidance on delay tolerance level of MTC applications | Vodafone Group |  |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and suggests below guidance to the ongoing radio interface design work on “enhanced coverage for MTC applications”.

- When considering the coverage enhancement in comparison to defined LTE cell coverage footprint engineered for "normal LTE UEs", the following is provided for guidance on latency:

  - Mobile originated data latency (5 seconds): From event trigger to reception of reported application message by eNode B (relevant for the “exception report” scenario in TR36.888).

  - Mobile terminated data round-trip latency (10 seconds): From downlink application level “command message” arrival at eNode B until reception of application level “response message” by eNode B (relevant for “triggered report” scenario in TR36.888).

Discussion (Question / Comment): Ericsson: to what extend latency evaluation needs to be considered?

Panasonic: minimum requirement is unclear. Main concern is how often system information is provided for MTC applications.

Ericsson: Vodafone should provide more detailed information, otherwise maybe a confusion how to use the proposed figures for MTC application investigation.

Qualcomm: is 5 years battery life for MTC needed?

Huawei: it is a useful paper. TR36.888 can be considered as a reference.

Sierra: agree with Huawei’s view.

ZTE: how can the number of TR36.888 be reused for MTC applications?

Intel: more background information from Vodafone may be helpful

LGE: can Vodafone provide additional information of max. delay tolerance level?

Decision: The document is noted.

Tuesday's session conclusion:

- Consider R1-135802 only as information when considering what is needed for “enhanced coverage” scenario for MTC applications

- Continue to study trade-off among latency, power consumption, efficiency, modem complexity and standardization effort

Possible observation:

- Latency of smart meter is already provided in TR36.888 as a guidance for MTC application

Continue offline discussion until Thursday – (Vodafone)

Friday 15th : email discussion

#### Remaining details of new UE category/type

Note: The frequency location of reduced bandwidth will be further discussed after progress on PDSCH and PDCCH timing/HARQ in coverage enhancement.

Not treated.

| R1-135018 | Discussion on remaining issues for new UE category | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135062 | Discussion on bandwidth reduction for new category UE | CATT |  |
| R1-135102 | Further Discussions on new UE category/type for low cost MTC UE | Intel Corporation |  |
| R1-135152 | Coverage enhancement requirements for MTC UE | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135153 | Considerations on Low Cost MTC UE | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135194 | PDSCH frequency allocation for MTC UEs | Samsung |  |
| R1-135254 | Low Cost UE TBS Discussion | Sierra Wireless |  |
| R1-135266 | PDSCH Frequency location of reduced bandwidth for low cost MTC | NEC |  |
| R1-135341 | Further consideration on the reduced PDSCH frequency allocation for low-cost MTC UE | Sharp |  |
| R1-135356 | Remaining details of new UE category for low cost MTC | ZTE |  |
| R1-135457 | Support of low cost features and enhanced coverage for MTC UE | LG Electronics |  |
| R1-135570 | Number of DL HARQ Processes for Low-Cost MTC UE | NSN, Nokia |  |
| R1-135600 | Additional considerations for the new Low Cost UE category | InterDigital | (R1-134558) |
| R1-135643 | Remaining details of low cost MTC UE | Ericsson |  |
| R1-135763 | PDSCH baseband bandwidth reduction for low complexity MTC UE | Sony |  |
| R1-135392 | TBS size and simultaneous reception capability on low category UE with enhanced coverage mode | Panasonic |  |

#### Techniques for coverage improvement

Not treated.

| R1-135422 | On the need of PDCCH for SIB and other Common Channels | MediaTek Inc. |  |
| --- | --- | --- | --- |
| R1-135423 | System information transmission in coverage enhancement mode | MediaTek Inc. |  |

##### PBCH

| R1-135943 | Way forward on P-BCH for MTC enhanced coverage | Vodafone Group |  |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone.

- Agree that we only select ONE of the following options that define the repetition burst within the 40ms PBCH cycle:

  - Option 1: 1 repetition in SF#0, so 2x the instances of existing PBCH.

  - Option 2: 1 repetition in SF#0 + repetition in SF#5 in odd frames (so 3x the instances of existing PBCH).

  - Option 3: 1 repetition in SF#0 + repetition in 1 other sub-frame in all frames (so 4x the instances of existing PBCH).

  - Option 4: 1 repetition in SF#0 + repetition in 3 other sub-frames in all frames (so 8x the instances of existing PBCH).

- Agree that “user data and MIB repetition are assumed not to be sent in the same PRBs.”

- Agree that we shall only select ONE of the options below for configuration of transmission across 40ms cycles:

  - Option A: Always send repetition in every 40ms cycle.

  - Option B: Dynamic on/off of repetitions on a per 40ms cycle basis.

  - Option C: Specify a pattern across a given number of cycles.

Discussion (Question / Comment): CATT the number of repetition is not necessary an integer value

Decision: The document is noted and modified as follows.

Agreements:

- Agree that we only select ONE of the following options that define the repetition burst within the 40ms PBCH cycle:

  - Option 1: Repetition in SF#0

  - Option 2: Repetition in SF#0 + repetition in SF#5 in odd frames.

  - Option 3: Repetition in SF#0 + repetition in 1 other sub-frame in all frames

  - Option 4: Repetition in SF#0 + repetition in 3 other sub-frames in all frames

  - FFS until the next meeting which REs should be excluded for PBCH repetition

- Agree that “user data and MIB repetition are assumed not to be sent in the same PRBs.”

- Agree that we shall only select ONE of the options below for configuration of transmission across 40ms cycles:

- Option A: Always send repetition in every 40ms cycle.

- Option B: Dynamic on/off of repetitions on a per 40x ms cycle basis.

- Option C: Repetition based on pattern(s) across a given number of cycles.

| R1-135019 | Further evaluations on PBCH coverage enhancement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135103 | Coverage enhancement of PBCH for low cost MTC | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Not treated.

| R1-135063 | PBCH coverage improvement for MTC UEs | CATT |  |
| --- | --- | --- | --- |
| R1-135064 | SIB coverage improvement for MTC UEs | CATT |  |
| R1-135129 | PBCH support for Deep Hole MTC devices | Fujitsu |  |
| R1-135154 | PBCH Coverage Enhancement for MTC UE | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135195 | PBCH coverage enhancements for MTC UEs | Samsung |  |
| R1-135252 | MIB and SIB Acquisition Time Guidance Discussion | Sierra Wireless |  |
| R1-135253 | Comparison between Repeating PBCH 3X (a little) or 20X (a lot) | Sierra Wireless |  |
| R1-135295 | PBCH channel design for MTC coverage enhancements | Qualcomm Inc. |  |
| R1-135357 | Evaluation and Analysis on Potential PBCH Coverage Enhancement Solutions | ZTE |  |
| R1-135358 | SIB Coverage Improvement | ZTE |  |
| R1-135363 | Summary of email discussion on SIB coverage improvement for MTC UE | ZTE |  |
| R1-135393 | Summary of PBCH transmission for MTC | Panasonic | (R1-134373) |
| R1-135414 | PBCH coverage improvement technique for MTC devices | ITRI |  |
| R1-135458 | PBCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-135459 | Evaluation on PBCH repetition for MTC coverage enhancement | LG Electronics |  |
| R1-135502 | PBCH Coverage Improvement for MTC Devices | HTC |  |
| R1-135571 | Performance Evaluation of PBCH With Repetition | NSN, Nokia |  |
| R1-135572 | SIB Coverage Enhancement | NSN, Nokia |  |
| R1-135601 | On PBCH Coverage Enhancement for LC-MTC | InterDigital | (R1-134559) |
| R1-135644 | System information for enhanced coverage MTC UE | Ericsson |  |
| R1-135823 | MTC Band Allocation Clarifications | Fujitsu | (R1-135133) |

##### PRACH

| R1-135944 | Way forward on P-RACH for MTC enhanced coverage | Vodafone Group |  |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone.

- WA on usage of existing PRACH formats from RAN1#74bis is confirmed.

- Enhanced coverage UEs and legacy UE may share the same time/frequency resource. In this case, enhanced coverage UEs will use CDM to multiplex with legacy UEs.

- In addition define time-frequency resource region separate for “enhanced coverage” UEs.

  - Within new region, CDM is allowed.

  - Specification of FDM in this region is FFS.

  - Specification of Frequency Hopping within this region is FFS.

- NOTE: RACH resource mapping for the “low complexity UE not requiring enhanced coverage” is FFS.

Decision: The document is noted, modified and agreed as:

Agreements:

- WA on usage of existing PRACH formats from RAN1#74bis is confirmed.

- Enhanced coverage UEs and legacy UE may share the same time/frequency resource. In this case, enhanced coverage UEs will use CDM to multiplex with legacy UEs.

  - FFS for multiplexing repetition level(s) within shared time/freq. resources

- In addition define additional time/freq. resource region(s) separate for “enhanced coverage” UEs.

  - Within new region, at least CDM is allowed.

- FFS for Frequency Hopping

- NOTE: RACH resource mapping for the “low complexity UE not requiring enhanced coverage” is FFS.

| R1-135104 | Coverage enhancement of PRACH for low cost MTC | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Thursday 14th session

Continued the remaining part of WF in R1-135944:

- Specified maximum numbers of levels: Working assumption of 3 (this does not include “zero coverage extension”). More evidence needed if we were to extend this.

- eNB-configurable number of levels (1, 2, 3) up to specified max level.

- Number of repetitions per level:

  - Working assumption: configurable value.

  - FFS ranges of this value per level – come back later in week.

- 1 attempt = configured number of repetitions.

- If UE does not receive a RAR after 1 attempt, it moves to next highest level (e.g. 5 to 10, and 10 to 15).

- At highest level, FFS on how many attempts are allowed, and the overall procedure (e.g. Backoff etc.).

- Starting level:

  - Start at the lowest level as baseline in initial IDLE mode access case. We could allow optimisations so UE can instead decide the starting level (e.g. based on defined measurements or other means), but FFS on the way of detecting and specifying that (more analysis invited for next meeting).

  - In RRC connected mode could be configured by eNB (dedicated RRC signalling).

- NOTE: Pmissed-detection relaxation only impacts the max number of possible repetitions overall we need to specify.

Agreements:

- Specified maximum numbers of levels: Working assumption of 3 (this does not include “zero coverage extension”). More evidence needed if we were to extend this.

- eNB-configurable number of levels (1, 2, 3) up to specified max level.

- Number of repetitions per level:

  - FFS for configurable value.

  - FFS ranges of this value per level – come back later in week.

- 1 attempt = configured number of repetitions.

- FFS: Power ramping is supported

- If UE does not receive a RAR after 1 attempt, it moves to next highest level (e.g. 5 to 10, and 10 to 15).

- At highest level, FFS on how many attempts are allowed, and the overall procedure (e.g. Backoff etc.).

Possible agreements:

- Starting level for contention based random access procedure:

  - Case 1: FFS between starting at the lowest level or defining it based on measurement or based on other ways

  - Case 2: working assumption: In RRC connected mode could be configured by eNB (dedicated RRC signalling).

Continue offline discussion until Friday – (Panasonic) mainly focusing on starting level

Friday 15th

| R1-136057 | Offline report on the handling on PRACH for enhanced coverage mode | Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Hidetoshi Suzuki from Panasonic.

- Methods for PRACH starting level

  - Method 1: FFS between Start from lowest level or based on defined measurements

  - Method 2: Configured by eNB (FFS: dedicated RRC, MAC,  and/or (e)PDCCH)

  - Method 3: UE remembers the last eNB and the last successful level

- Relation with legacy PRACH procedure and enhanced coverage mode PRACH procedure is FFS.

- It should be checked following state/procedures cover all PRACH usages.

| State | Procedure | Possibility |
| --- | --- | --- |
| Attach | - | Method 1 or 3 |
| Idle to connected | - | Method 1 , 2, 3 |
| connected | contention PRACH for SR | Method 1, 2, 3 |
| connected | PDCCH order contention (?) | Method 1, 2, 3 |
|  | PDCCH order non-contention | Method 2 |
| connected | Non-contention PRACH for handover | FFS as the procedure is supported or not |

Decision: The document is noted.

Email discussion until 6th December about starting level by exploiting R1-136057.

| R1-136060 | HARQ support for enhanced coverage mode | Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Hidetoshi Suzuki from Panasonic.

- HARQ in UL and DL is supported in coverage enhanced mode

  - FFS on the details of HARQ realization for PUSCH

  - FFS on the number of HARQ processes

- No need for UE to decode PCFICH in coverage enhanced mode.  Not to specify PCFICH repetition.

  - FFS on how UE derives CFI

Decision: The document is noted and agreed as above.

Not treated.

| R1-135020 | Considerations on the random access of MTC UEs with coverage improvement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135065 | Random access design for UEs in enhanced coverage mode | CATT |  |
| R1-135131 | Coverage Hole Depth Signalling supporting Low Cost MTC devices | Fujitsu |  |
| R1-135155 | PRACH Coverage Enhancement for MTC UE | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135196 | PRACH coverage enhancements for MTC UEs | Samsung |  |
| R1-135296 | RACH channel design for MTC coverage enhancements | Qualcomm Inc. |  |
| R1-135342 | Discussion on PRACH coverage enhancement for MTC UE | Sharp |  |
| R1-135359 | PRACH coverage improvement for MTC UEs | ZTE |  |
| R1-135382 | Determining the repetition level during initial random access and non-initial random access | Huawei, HiSilicon |  |
| R1-135424 | Discussion on PRACH and RACH procedure in coverage enhancement mode | MediaTek Inc. |  |
| R1-135460 | RACH procedure for MTC coverage enhancement | LG Electronics |  |
| R1-135509 | Discussion on multi-level PRACH coverage enhancement | NTT DOCOMO |  |
| R1-135573 | PRACH Coverage Enhancement | NSN, Nokia |  |
| R1-135645 | Random access for enhanced coverage MTC UE | Ericsson |  |
| R1-135822 | PRACH Support for Coverage Deficit Devices | Fujitsu | (R1-135130) |

##### (E)PDCCH/PUCCH

Not treated.

| R1-135021 | Discussion on (E)PDCCH and PUCCH coverage improvement for MTC UEs | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135066 | (E)PDCCH coverage improvement for MTC UEs | CATT |  |
| R1-135067 | PUCCH coverage improvement for MTC UEs | CATT |  |
| R1-135105 | Coverage enhancement of DL/UL control channels for low cost MTC | Intel Corporation |  |
| R1-135132 | (e)PDCCH Support for Coverage Deficit MTC Devices | Fujitsu |  |
| R1-135156 | Considerations for (E)PDCCH in coverage enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135157 | Considerations for PUCCH in coverage enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135198 | PDCCH coverage enhancements for MTC UEs | Samsung |  |
| R1-135199 | PUCCH coverage enhancements for MTC UEs | Samsung |  |
| R1-135297 | Coverage enhancements for (E)PDCCH/PUCCH | Qualcomm Inc. |  |
| R1-135360 | Discussion on Control Channel Coverage Improvement | ZTE |  |
| R1-135361 | Simplification of PHICH and PCFICH functionalities for coverage improvement MTC UEs | ZTE |  |
| R1-135394 | (E)PDCCH coverage enhancement for MTC | Panasonic |  |
| R1-135425 | Discussion on (E)PDCCH coverage enhancement | MediaTek Inc. |  |
| R1-135426 | Functionality and coverage analysis of PUCCH | MediaTek Inc. | (R1-134445) |
| R1-135461 | PDCCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-135510 | RNTI handling in (E)PDCCH coverage enhancement | NTT DOCOMO |  |
| R1-135574 | PUCCH UCI in Coverage Enhancement Mode | NSN, Nokia | (R1-134519) |
| R1-135575 | PDCCH Coverage Enhancement | NSN, Nokia | (R1-134520) |
| R1-135646 | Physical downlink control channel for enhanced coverage MTC UE | Ericsson |  |
| R1-135647 | Physical uplink control channel for enhanced coverage MTC UE | Ericsson |  |
| R1-135674 | MTC Coverage improvement for Control Channels | Motorola Mobility |  |
| R1-135946 | Way Forward for (E)PDCCH coverage enhancement | MediaTek, Panasonic, CATR, CATT, ZTE, Sony, LGE, Alcatel-Lucent, Alcatel Shanghai Bell, Fujitsu, Sierra Wireless, Samsung |  |

##### PDSCH/PUSCH

Not treated.

| R1-135022 | Timing relationship between PDCCH and PDSCH for coverage improvement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135068 | Timing relationship between (E)PDCCH and PDSCH for UEs in enhanced coverage mode | CATT |  |
| R1-135197 | PDSCH/PUSCH coverage enhancements for MTC UEs | Samsung |  |
| R1-135267 | PDSCH and PUSCH coverage enhancements for low cost MTC | NEC |  |
| R1-135298 | Coverage enhancements for PDSCH/PUSCH | Qualcomm Inc. |  |
| R1-135362 | Discussion on Traffic Channel Coverage Improvement | ZTE |  |
| R1-135427 | Discussion on timing relationship for (E)PDCCH and PDSCH | MediaTek Inc. |  |
| R1-135428 | On UL HARQ functionality and PHICH enhancement | MediaTek Inc. | (R1-134443) |
| R1-135462 | PDSCH/PUSCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-135511 | Views on PDSCH coverage enhancement | NTT DOCOMO |  |
| R1-135576 | Data Channel Coverage Enhancement | NSN, Nokia | (R1-134518) |
| R1-135609 | Code spreading for PUSCH and PDSCH coverage improvement in FDD | III |  |
| R1-135648 | Data transmission for enhanced coverage MTC UE | Ericsson |  |
| R1-135675 | MTC Coverage improvement for Data Channels | Motorola Mobility |  |

Thursday 14th session

Mr Chair decided looking first at PDCCH/PDSCH timing issue.

| R1-135945 | Way forward on PDCCH/PDSCH timing for MTC enhanced coverage | Vodafone Group |  |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone.

- If/when PDSCH is indicated via (E)PDCCH:

  - The relation of PDSCH timing to (E)PDCCH timing shall be known to UE and shall not be configurable by higher layer parameter dedicated only for this purpose and not configurable by L1. FFS on how to derive it.

  - Assigned PDSCH is transmitted not before end of (E)PDCCH.

Decision: The document is noted, modified and agreed as below:

Agreements:

- For UEs in enhanced coverage mode for MTC, if/when PDSCH is indicated via (E)PDCCH:

  - The relation of PDSCH timing to (E)PDCCH timing shall be known to UE and shall not be configurable by higher layer parameter dedicated only for this purpose and shall not be indicated by (E)PDCCH. FFS on how to derive it or fixed by spec.

  - Assigned PDSCH is transmitted not before end of (E)PDCCH, i.e., if subframe n is the last (E)PDCCH repetition then PDSCH start n + k (k > 0)

| R1-136001 | Way forward on PDCCH, PDSCH, PUCCH and PUSCH for MTC enhanced coverage | Vodafone Group, Ericsson, Huawei, HiSilicon, LG Electronics, Samsung, Alcatel-Lucent, ETRI, NSN, Panasonic, Fujitsu, Sony, ZTE, Mediatek, CATT, NEC, Sierra Wireless, u-blox |  |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and provides the outcomes of Wednesday offline discussion.

- PDCCH for enhanced coverage

  - For UE-specific search space, it is proposed to agree that (e)PDCCH to schedule PDSCH is supported.

    - Repetition of (e)PDCCH with multiple levels is supported.

    - From the UE perspective, the possible starting sub-frames of (e)PDCCH repetitions are limited to a subset of sub-frames.

  - The following is FFS:

    - Whether to use the same PDCCH candidate m (including same aggregation level) in each repetition, with similar principle for ePDCCH.

- PUCCH for enhanced coverage

  - It is proposed to agree that no support of periodic CSI over PUCCH

  - It is proposed to agree as a “Working Assumption” that:

    - ACK/NACK on PUCCH is supported. FFS on the configurability of ACK/NACK.

    - Dedicated SR is supported but no further optimization beyond PUCCH repetition for SR (e.g. no new formats).

- PDSCH for enhanced coverage

  - It is proposed to agree that:

    - Repetition of PDSCH across multiple sub-frames is supported.

    - Multiple repetition levels in time domain are specified.

- PUSCH for enhanced coverage

  - It is proposed to agree that:

  - Repetition of PUSCH across multiple sub-frames is supported.

  - Multiple repetition levels in time domain are specified.

Decision: The document is noted.

Agreements:

- For UEs in enhanced coverage mode for MTC

  - For UE-specific search space,

    - (E)PDCCH to schedule PDSCH is supported.

    - Repetition of (E)PDCCH with multiple levels is supported.

    - From the UE perspective, the possible starting sub-frames of (E)PDCCH repetitions are limited to a subset of sub-frames.

Following is email discussion until RAN1 #76 meeting

- Whether to use the same PDCCH candidate m (including same aggregation level) in each repetition, with similar principle for (E)PDCCH

Working assumption:

- For UEs in enhanced coverage mode for MTC,

  - No support of repetition of periodic CSI over PUCCH

    - FFS: Periodic CSI over PUCCH without repetition

  - ACK/NACK on PUCCH is supported. FFS on the configurability of ACK/NACK.

  - Dedicated SR is supported but no further optimization beyond PUCCH repetition for SR (e.g. no new formats).

Agreements:

- For UEs in enhanced coverage mode for MTC,

  - Repetition of PDSCH across multiple sub-frames is supported.

  - Multiple repetition levels in time domain are specified.

Agreements:

- For UEs in enhanced coverage mode for MTC,

  - Repetition of PUSCH across multiple sub-frames is supported.

  - Multiple repetition levels in time domain are specified.

##### SS

Not treated.

| R1-135158 | Considerations for PSS/SSS in coverage enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

##### Other

Not treated.

| R1-135069 | PCFICH and PHICH coverage improvement for MTC UEs | CATT |  |
| --- | --- | --- | --- |
| R1-135134 | System Information Support for Deep Hole Low Cost MTC Devices | Fujitsu |  |
| R1-135135 | PCFICH related issues of Low Cost MTC UEs based on LTE | Fujitsu |  |
| R1-135159 | Simplification of MTC UE operation in coverage enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135200 | PHICH and PCFICH for low cost & enhanced coverage MTC UE | Samsung |  |
| R1-135299 | Power Consumption Considerations for Coverage Enhancements for MTC | Qualcomm Inc. |  |
| R1-135300 | Draft LS on power saving techniques for MTC | Qualcomm Inc. |  |
| R1-135446 | Simplified functionalities of PCFICH for low-cost MTC UEs | ITL Inc. |  |

### LTE TDD-FDD Joint Operation including Carrier Aggregation

WID in RP-131399 - LTE_CA_TDD_FDD-Core

#### TDD – FDD carrier aggregation

| R1-135275 | Downlink HARQ timing in TDD-FDD carrier aggregation | ETRI |  |
| --- | --- | --- | --- |

The document was presented by … from ETRI and proposes:

- Cross-carrier scheduling should be supported for TDD-FDD CA.

- The HARQ timing for the PCell follows the HARQ timing of its own cell.

- If the UE is configured with FDD PCell and TDD SCell, the HARQ-ACK transmission in response to PDSCH reception on TDD SCell follows the FDD HARQ timing, i.e. , the UE transmits HARQ-ACK in subframe n +4 in response to PDSCH reception in subframe n.

- For the UE configured with TDD PCell and FDD SCell, and cross-carrier scheduling, cross-carrier scheduling across subframes (e.g. multi-subframe or cross-subframe scheduling) should be considered.

- For the UE configured with TDD PCell and FDD SCell, the DL HARQ timing for the FDD SCell considers the following two options.

  - If multi-subframe scheduling is not introduced, TDD PCell timing for some DL subframes + additional timing for the remaining DL subframes

  - If multi-subframe scheduling is introduced, potentially new HARQ timing for all the DL subframes

Decision: The document is noted.

| R1-135355 | Issues for TDD-FDD CA | ZTE |  |
| --- | --- | --- | --- |

The document was presented by Ms Huiying Fang from ZTE and proposes:

- PUCCH transmission is restricted to only PCell.

- Cross-carrier scheduling is supported for TDD-FDD CA.

- When FDD cell is PCell and TDD cell is SCell, the HARQ-ACK timing of SCell PDSCH could follow PCell timing regardless of self-scheduling or cross-scheduling.

- When TDD cell is PCell and FDD cell is SCell, the HARQ timing of SCell PDSCH could follow a TDD reference configuration HARQ-ACK timing or new HARQ-ACK timing regardless of self-scheduling or cross-scheduling.

- When FDD cell is scheduling cell and TDD cell is scheduled cell, the scheduled cell PUSCH timing should follow modified PUSCH timing.

- When TDD cell is scheduling cell and FDD cell is scheduled cell, the scheduled cell PUSCH timing should follow a reference configuration PUSCH timing or modified FDD PUSCH timing.

Decision: The document is noted.

| R1-135577 | Support of cross-carrier scheduling in TDD-FDD CA | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Tommi Koivisto from Nokia and notes that the use cases and scenarios where cross-carrier scheduling is required and which are not already supported by other existing functionalities are very unclear. In addition, it highlights the complexity involved in the specification of TDD-FDD cross-carrier scheduling. It is proposed:

- Do not support cross-carrier scheduling between FDD and TDD cells

  - Focus the effort in the TDD-FDD CA work item on the self-scheduling case

Discussion (Question / Comment): Samsung, Qualcomm do not agree with the conclusion

Decision: The document is noted.

| R1-135513 | On the need of PUCCH on multiple UL serving cells for TDD-FDD CA | NTT DOCOMO |  |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo and draws the following observations:

- Non-co-located CA deployment is a promising way to achieve local area/UE throughput enhancement and mobility robustness simultaneously.

  - Macro cell offers mobility robustness by acting as the PCell.

  - Small cells within the macro cell offer local area/UE throughput enhancement by acting as SCells.

  - Strict cell planning/optimization of small cells may not be necessary since important signals, e.g., RRC signalling, are transmitted to/from macro cell and mobility is managed by the macro cell

  - Macro cell and small cell can be aggregated in the DL similar to co-located CA if the amount of DL traffic is not high.

- However, in a non-co-located CA deployment, it is quite challenging to rely on the current PUCCH principle in which UEs transmit the PUCCH on only the PCell.

  - This is based on the PUCCH overhead for the macro cell.

It is proposed:

- RAN1 should agree on the following considerations

  - The current PUCCH mechanisms for CA do not work well in non-co-located CA scenarios.

  - It is desirable to support PUCCH offloading mechanisms to a SCell, while not degrading mobility performance.

- Alt. 3: Utilizing the PUCCH on multiple UL serving cells should be supported in Rel. 12.

  - This should be applicable at least to UL TDD-FDD CA capable UEs.

- RAN1 should decide how to resolve the PUCCH issue between options 1 and 2.

  - Option 1: Specify the PUCCH on multiple UL serving cells for TDD-FDD CA.

  - Option 2: Support PUCCH on multiple UL serving cells in SCE higher-layer SI/WI in addition to the PUCCH on multiple eNBs for inter-eNB CA

- If option 1 is agreed upon, then separate PUCCHs on TDD and FDD cells should be discussed after concluding TDD-FDD CA specification work for DL-CA capable and UL-CA non-capable UEs

- If option 2 is agreed upon, then RAN1 should send a LS to RAN and RAN2 to inform them that RAN1 identified the PUCCH issue regarding non-co-located CA during TDD-FDD CA WI, and request that the RAN and RAN2 take this information into account in future discussions of SCE higher-layer SI/WI.

Decision: The document is noted.

| R1-135248 | General Aspects for TDD-FDD Carrier Aggregation | Texas Instruments |  |
| --- | --- | --- | --- |

The document was presented by Anthony Ekpenyong from TI and deals with SCell PUCCH and support of cross-carrier scheduling.

- At least when there are UL resources available in a subframe for PCell and SCell, PUCCH should be transmitted on the PCell.

- RAN1 should agree on what conditions PUCCH may be permitted on a SCell.

- For scheduling of DL and UL transmission on a SCell, first priority should be given to the self-scheduled case.

- Further study is required to determine if any enhancement is needed for periodic CSI reporting.

Decision: The document is noted.

At this stage, Mr Chair decided looking at the proposed WFs

| R1-135898 | Way Forward on HARq timing for TDD FDD CA | LG Electronics, Huawei, HiSilicon, Qualcomm, ZTE, ETRI, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, NSN, Sharp, Intel, Samsung, Ericsson, ST-Ericsson, ITL Inc., KT Corp. |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE.

- For TDD-FDD carrier aggregation, PUCCH on PCell-only is supported as in Rel-10/11 carrier aggregation, regardless of whether or not UE is configured with UL-CA

  - Discuss further whether or not to additionally support PUCCH on SCell

- For the case of PUCCH transmission on PCell only:

  - For PDSCH/PUSCH transmitted on PCell, scheduling/HARQ timing follows existing PCell timing regardless whether PCell is TDD or FDD carrier

  - For PUSCH transmitted on SCell with self-scheduling, scheduling/HARQ timing follows existing Scell timing regardless whether SCell is TDD or FDD

  - When Pcell is FDD carrier and SCell is TDD carrier, for PDSCH transmitted on Scell with self-scheduling, HARQ timing follows Pcell timing

  - When PCell is TDD carrier and SCell is FDD carrier, for PDSCH transmitted on Scell with self-scheduling, HARQ timing is FFS

Discussion (Question / Comment): CATT supports the WF, willing to further clarify the FFS

MediaTek: why PCell-only in first main bullet? Don’t see the benefit - switching between PCell and SCell PUCCH shall be considered

Decision: The document is noted, modified and agreed as follows:

- For TDD-FDD carrier aggregation, at least PUCCH on PCell-only is supported as in Rel-10/11 carrier aggregation, regardless of whether or not UE is configured with UL-CA

  - Discuss further whether or not to additionally support PUCCH on SCell

    - FFS: TDM switching between Pcell and Scell PUCCH

- For the case of PUCCH transmission on PCell only:

  - For PDSCH/PUSCH transmitted on PCell, scheduling/HARQ timing follows existing PCell timing regardless whether PCell is TDD or FDD carrier

  - For PUSCH transmitted on SCell with self-scheduling, scheduling/HARQ timing follows existing Scell timing regardless whether SCell is TDD or FDD

  - When Pcell is FDD carrier and SCell is TDD carrier, for PDSCH transmitted on Scell with self-scheduling, HARQ timing follows Pcell timing

Possible agreements:

  - When PCell is TDD carrier and SCell is FDD carrier, for PDSCH transmitted on Scell with self-scheduling, HARQ timing is FFS

    - Continue offline discussion until Thursday – (LGE)

************

Thursday 14th

| R1-136009 | Offline discussion on self-scheduling for TDD-FDD CA | LGE, Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE and suggests two options:

- Option 1) FDD SCell PDSCH timing follows TDD PCell timing + additional new timing for remaining subframes of FDD SCell

  - Supported by: ZTE, Huawei, HiSilicon, LG, Samsung, Sharp, ETRI, ITL Inc., NEC, HTC, NTT DOCOMO, KT

- Option 2) The PDSCH HARQ timing of FDD SCell follows the DL reference TDD U/D configuration, where the reference TDD U/D configuration is one of the existing 7 U/D configurations

  - 2-c) The DL reference TDD U/D configuration is configured by higher layers

  - Supported by: Potevio, Nokia, NSN, CMCC, CATT, Intel, Ericsson, CATR, IAESI

Discussion (Question / Comment): ALU in favour of option 2-c

Decision: The document is noted; further modified as:

Option 1) FDD SCell PDSCH timing depends on TDD PCell timing + additional new timing for remaining subframes of FDD SCell

- If UL/DL configuration 5 is used, the number of HARQ processes is less than 17

  - Supported by 14 companies: ZTE, Huawei, HiSilicon , Samsung, Sharp, ETRI, ITL Inc., NEC, HTC, NTT DOCOMO, KT, Ericsson, TI, LG

Option 2-c) The PDSCH HARQ timing of FDD SCell follows the DL reference TDD U/D configuration, where the reference TDD U/D configuration is one of the existing 7 U/D configurations

- 2-c) The DL reference TDD U/D configuration is configured by higher layers

  - Supported by 14 companies: Potevio, Nokia, NSN, CMCC, CATT, Intel, CATR, IAESI (under the consensus above), AL, Qualcomm, BlackBerry, ASB, SEQUANS, Mediatek

Conclusion:

Email discussion until RAN1 #76 meeting about the detail of Options 1) and 2-c) of HARQ timing of PDSCH transmission on SCell with self-scheduling when PCell is TDD carrier and SCell is FDD carrier.

| R1-136016 | WF on TDD/FDD CA when the Pcell is TDD | Huawei, HiSilicon, ETRI, HTC, ITL Inc., KT Corporation, LG Electronics, Samsung, Sharp, ZTE |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei.

- When PCell is TDD and SCell is FDD, for PDSCH transmitted on SCell with self-scheduling, HARQ timing allows scheduling all the DL subframes on the FDD Scell for one UE.

Discussion (Question / Comment): Also supported by NEC (assuming it is not restrictive to the 3 examples listed in)

Ericsson introduced their alternative in 6017 aiming as more explicit solution at this meeting.

Decision: The document is noted.

| R1-136017 | Alternative HARQ timing proposal for TDD PCell and FDD Scell | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson.

- When PCell is TDD carrier and SCell is FDD carrier, for PDSCH transmitted on Scell with self-scheduling the following HARQ timings applies

  - FDD SCell PDSCH timing follows a DL reference TDD U/D configuration + additional new timing for remaining subframes of FDD Scell

    - When Pcell SIB1 UL/DL configuration is 0, 1, 2 and 6 the FDD SCell PDSCH timing follows DL reference TDD U/D configuration 2 + additional new timing for remaining subframes of FDD SCell

    - When PCell SIB1 UL/DL configuration is 3 and 4 the FDD SCell PDSCH timing follows DL reference TDD U/D configuration 4 + additional new timing for remaining subframes of FDD SCell

    - When PCell SIB1 UL/DL configuration is 5 the FDD SCell PDSCH timing follows DL reference TDD U/D configuration 5

Decision: The document is noted.

************

| R1-135899 | Way Forward on cross carrier scheduling for TDD FDD CA | LG Electronics, Qualcomm, ETRI, NEC, CATT, ZTE, Sharp, Intel, CMCC |  |
| --- | --- | --- | --- |

The document was presented by Daesung Hwang from LGE.

- For the case of PUCCH transmission on PCell only:

  - Cross carrier scheduling of PDSCH/PUSCH on TDD carrier by FDD carrier is supported

    - In this case, scheduling/HARQ timing for PDSCH/PUSCH on TDD carrier is FFS

  - Cross carrier scheduling of PDSCH/PUSCH at least in subset of subframes on FDD carrier by TDD carrier is supported

    - In this case, scheduling/HARQ timing for PDSCH/PUSCH on FDD carrier is FFS

Discussion (Question / Comment): Huawei  support effort needs to be considered - worth looking at 5933

Decision: The document is noted.

| R1-135933 | WF on cross-carrier scheduling in TDD/FDD CA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei.

- Take the simple solutions that reuse Rel-11 principles in slides 2 and 3 as baseline for supporting cross-carrier scheduling in TDD/FDD CA in Rel-12

- Further discuss the detailed solution for UL cross-carrier scheduling if the scheduling serving cell is TDD and the scheduled serving cell is FDD

  - a modified FDD PUSCH timing may be considered instead of a virtual TDD UL/DL configuration for the FDD cell

Discussion (Question / Comment):

Decision: The document is noted. Off line discussion (Huawei)

| R1-135906 | Draft LS on the need of PUCCH on multiple UL serving cells for non co-located CA | NTT DoCoMo |  |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo and informs RAN/RAN2 on non-co-located CA deployment scenarios issues identified so far by RAN1.

Discussion (Question / Comment): Also supported by NEC

Qualcomm suggested to add "it should be designed to be also applicable for future CA scenario within an eNB"

Qualcomm: Concerns that CA scenario” is not the existing CA. Prefer a unified solution if we support PUCCH on Scell

CATT: sending this LS means RAN1 has identified PUCCH overhead issue - that is not the case, no common consensus of PUCCH overhead issues in RAN1 - should be further discussed in RAN

Fujitsu: Similar views with Qualcomm.

ALU: Additional effort may be needed in RAN2 to use a solution of dual connectivity to support TDD/FDD CA

Panasonic: Additional effort may not be needed in RAN2, but limitation on the scenario may need discussion somewhere until plenary

MediaTek: Not clear the definition of separate PUCCH.

NTT DoCoMo: It is clear the definition of separate PUCCH. We only copied TR terminology to LS.

NEC: Recommend to change “separate PUCCH” to “simultaneous PUCCH”

LGE: Not sure what is “Simultaneous PUCCH”

Decision: The document is noted. Off line discussion during morning coffee break.

| R1-135941 | Draft LS on the need of PUCCH on multiple UL serving cells for non co-located CA | CATT proposal | (R1-135906) |
| --- | --- | --- | --- |

The document was presented by Zukang Shen from CATT.

Discussion (Question / Comment): Intel: Similar view with CATT. No problem of PUCCH overhead.

Decision: The document is noted. Continue offline discussion until Thursday – (NTT DOCOMO)

Thursday 14th

| R1-136014 | Draft LS on the need of PUCCH on multiple UL serving cells for non-co-located CA | NTT DOCOMO, Ericsson, Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo.

Discussion (Question / Comment): Qualcomm: with no or minimum additional effort in RAN2.

RAN1 schedule to be updated.

Decision: The document is noted. Final LS is agreed in R1-136024, assuming the following change:

It is RAN1 understanding that enhanced PUCCH mechanism (referred as separate PUCCH in [3]) is required to support inter-node radio resource aggregation for dual connectivity. RAN1 considers that it is potentially beneficial when such a mechanism is directly used for CA in scenario 4, irrespective of the duplex mode (TDD or FDD), assuming this can be done with no or minimal additional effort.”

Not treated.

| R1-135023 | PUCCH transmission for TDD-FDD CA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135024 | Details for TDD-FDD CA with PUCCH on FDD Pcell | Huawei, HiSilicon |  |
| R1-135025 | Details for TDD-FDD CA with PUCCH on TDD Pcell | Huawei, HiSilicon |  |
| R1-135026 | Restriction on combinations of FDD/TDD serving cells for TDD-FDD CA | Huawei, HiSilicon |  |
| R1-135070 | Half duplex UE operation for FDD-TDD CA | CATT |  |
| R1-135071 | HARQ/scheduling timing with FDD as PCell for FDD-TDD CA | CATT |  |
| R1-135072 | PDSCH timing with TDD as PCell for FDD-TDD CA | CATT |  |
| R1-135073 | PUSCH timing with TDD as PCell for FDD-TDD CA | CATT |  |
| R1-135074 | UCI transmission for FDD-TDD carrier aggregation | CATT |  |
| R1-135075 | PUCCH on Scell for FDD-TDD CA | CATT |  |
| R1-135106 | On support of TDD-FDD Carrier Aggregation | Intel Corporation |  |
| R1-135160 | Scheduling and HARQ timing for TDD-FDD carrier aggregation | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-135201 | HARQ operation for TDD-FDD CA with TDD Pcell | Samsung |  |
| R1-135202 | Multiple TA for TDD-FDD CA | Samsung |  |
| R1-135203 | Views on open questions for TDD-FDD CA | Samsung |  |
| R1-135204 | Specification support for FDD-TDD CA | Samsung |  |
| R1-135258 | HARQ timing for TDD-FDD CA | NEC |  |
| R1-135274 | Uplink scheduling/HARQ timing in TDD-FDD carrier aggregation | ETRI |  |
| R1-135301 | Solution for TDD-FDD CA | Qualcomm Inc. |  |
| R1-135343 | DL scheduling and HARQ-ACK feedback for TDD-FDD CA | Sharp |  |
| R1-135344 | UL scheduling and HARQ-ACK feedback for TDD-FDD CA | Sharp |  |
| R1-135345 | Support of non simultaneous Rx/Tx capable UEs for TDD-FDD carrier aggregation | Sharp |  |
| R1-135383 | Standard impact of TDD-FDD CA for UEs not supporting simultaneous transmission and reception | Huawei, HiSilicon |  |
| R1-135429 | PUCCH transmission on Scell for TDD-FDD CA | MediaTek Inc. |  |
| R1-135430 | TDM UL transmission for TDD-FDD DL CA capable UE with single transmitter | MediaTek Inc. |  |
| R1-135442 | Discussion on PDSCH timing in TDD-FDD CA | Potevio |  |
| R1-135443 | Discussion on PUSCH timing in TDD-FDD CA | Potevio |  |
| R1-135447 | HARQ and scheduling timing for TDD-FDD CA | ITL Inc. |  |
| R1-135463 | Basic assumptions and requirements for TDD-FDD CA | LG Electronics |  |
| R1-135464 | Considerations on HARQ-ACK and DCI for TDD-FDD CA | LG Electronics |  |
| R1-135465 | DL HARQ timing for TDD-FDD carrier aggregation | LG Electronics |  |
| R1-135466 | UL HARQ timing for TDD-FDD carrier aggregation | LG Electronics |  |
| R1-135512 | Scheduling/HARQ design for TDD-FDD CA other than PUCCH for UL-CA | NTT DOCOMO |  |
| R1-135556 | Solutions for TDD-FDD joint carrier aggregation | BlackBerry UK Limited |  |
| R1-135578 | HARQ-ACK timing in TDD-FDD CA | Nokia, NSN |  |
| R1-135615 | HARQ considerations for TDD-FDD CA | Texas Instruments |  |
| R1-135628 | Discussion on PUCCH transmission for TDD-FDD CA | CMCC |  |
| R1-135629 | Discussion on full/non-full duplex UE for TDD-FDD CA | CMCC |  |
| R1-135630 | Discussion on the HARQ timing design of TDD-FDD carrier aggregation for full duplex UE | CMCC |  |
| R1-135649 | HARQ feedback and PUCCH design for TDD-FDD CA | Ericsson |  |
| R1-135650 | Cross-carrier scheduling support for TDD-FDD CA | Ericsson |  |
| R1-135651 | Further discussion on PDSCH timing with TDD as PCell for FDD-TDD | Ericsson |  |
| R1-135652 | CSI reporting for TDD-FDD CA | Ericsson |  |
| R1-135774 | Discussion on DL HARQ operation in TDD-FDD Carrier Aggregation | KT Corp. |  |
| R1-135813 | Solution based on traffic asymmetry for avoiding requirements of fast Tx UE switching | IAESI | (R1-135243) |
| R1-135835 | On support of DL and UL cross-carrier scheduling in TDD-FDD CA | NEC | (R1-135259) |

Thursday 14th

| R1-136003 | Summary of offline discussions on cross-carrier scheduling for TDD-FDD CA | Huawei |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei.

Discussion (Question / Comment): ALU did not join the offline discussion and requested time for further checking.

Decision: The document is noted. After lunch, no need updating the summary - Alcatel Lucent is fine with the alternatives on the table but Alt UL-C2 may need to be reviewed.

Working assumption:

- For DL, no cross-/multi- subframe scheduling in Rel-12 for TDD-FDD CA

Conclusion:

Email discussion until 24th January for cross-carrier scheduling to try to achieve one solution for DL, UL, respectively, if no consensus, no cross-carrier scheduling in Rel-12 for TDD-FDD CA.

- For DL cross-carrier scheduling,

- Alt. DL-A:

  - For DL cross-carrier scheduling, the DL HARQ timing of the scheduled serving cell follows the PCell’s timing

- Alt. DL-B:

  - For DL cross-carrier scheduling, the DL HARQ timing of the scheduled serving cell follows the PCell’s timing

    - except for a cross-carrier scheduled FDD serving cell when the PCell is TDD

      - reuse the HARQ timing for self-carrier scheduling on FDD SCell when the TDD is PCell (option 1 or option 2-c in R1-136009])

- For UL cross-carrier scheduling,

- If the scheduling serving cell is FDD and the scheduled serving cell is TDD, for UL cross-carrier scheduling where PHICH is transmitted on the scheduling serving cell, the scheduling/HARQ timing of TDD scheduled serving cell follows:

  - Alt. UL-A1: the TDD scheduled serving cell’s UL/DL configuration

  - Alt. UL-B1: 10ms RTT: 4ms between UL grant/PHICH and PUSCH, 6ms between PUSCH and PHICH

- If the scheduling serving cell is TDD and the scheduled serving cell is FDD, for UL cross-carrier scheduling where PHICH is transmitted on the scheduling serving cell, the scheduling/HARQ timing of FDD scheduled serving cell follows:

  - Alt. UL-A2: a UL-reference UL/DL configuration with no new timing

    - Candidate UL reference UL/DL configuration (for down selection):

      - TDD scheduling serving cell’s UL/DL configuration

      - Fixed reference UL/DL configuration 0

      - Configurable reference UL/DL configuration amongst a set of candidates

  - Alt. UL-B2: 10ms RTT: 4ms between UL grant/PHICH and PUSCH, 6ms between PUSCH and PHICH

  - Alt. UL-C2: Timing follows reference config. which can be one of 7 UL-DL config. or FDD timing.

- Alt. D: No additional spec. change to support cross-carrier scheduling for TDD-FDD CA

- Alt. E: For the case when cross-carrier scheduling cell is FDD, no spec. change compared to Rel-10 CA both for DL and UL for TDD-FDD CA.

#### Other

None

### LTE Coverage Enhancements

WID in RP-130833 - Cov_Enh_LTE-Core

| R1-136029 | Summary of Ad-hoc session on LTE Coverage Enhancements | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and provides the outcomes of the Coverage Enhancements sessions.

Decision: The document is endorsed and incorporated below.

| R1-135281 | Email discussion on remaining issues of LTE coverage enhancements | China Telecom |  |
| --- | --- | --- | --- |

Decision: The document is noted.

#### TTI bundling enhancements for UL VoIP

From 5281:

Agreement:

- Reduction of RTT to 12ms is supported in enhanced TTI bundling for both UL VoIP and medium data rate for FDD in Rel-12.

  - In this case, the number of UL HARQ processes is 3

Not treated.

| R1-135027 | Further analysis on TTI bundling enhancements for UL VoIP for FDD | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135107 | On TTI Bundling Enhancements for UL VoIP | Intel Corporation |  |
| R1-135136 | Alt.6 for TTI bundling enhancements for UL VoIP | Fujitsu |  |
| R1-135161 | TTI Bundling Enhancement for UL VoIP | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135205 | Discussion on LTE coverage enhancement methods for VoIP | Samsung |  |
| R1-135282 | Further views on TTI bundling enhancements for UL VoIP | China Telecom |  |
| R1-135302 | TTI bundling enhancements for UL VoIP | Qualcomm Inc. |  |
| R1-135347 | Proposals on TTI bundling enhancements for UL VoIP | ZTE |  |
| R1-135467 | Discussion on TTI bundling enhancements for UL VoIP | LG Electronics |  |
| R1-135579 | LTE coverage enhancements for UL VoIP users in FDD mode | NSN, Nokia |  |
| R1-135602 | Recommendations on TTI bundling enhancements for VoIP | InterDigital | (R1-134561) |
| R1-135653 | Remaining details of TTI bundling enhancements for UL VoIP | Ericsson |  |

#### Coverage enhancement for TDD

| R1-135028 | Discussion on TTI bundling extension for TDD | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135162 | TTI Bundling Enhancement for TDD | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135260 | On the need to extend coverage enhancement for Configurations #0, #1, #6 | NEC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135672 | Considerations on UL VoIP enhancement for TDD | Coolpad |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135976 | Clarification on the baseline for VoIP transmission delay in TDD UL-DL configuration #0, #3, #6 | Huawei, HiSilicon, CMCC, CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135980 | Way forward on coverage enhancement for TDD | CATT, CATR, CMCC, Coolpad, Huawei, HiSilicon, Intel |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposals:

- Alt 1: No TTI bundling enhancements for TDD

  - ALU, ASB, Intel, LGE, Samsung, NEC, Ericsson, ZTE

  - Objected by: CATT, Coolpad, Qualcomm, Huawei, HiSi, CMCC

- Alt 2: TTI bundling enhancement is only supported for UL-DL configurations #0, and #6

  - Huawei, HiSi, Intel, CATT, Ericsson, CMCC

  - Objected by: Samsung, NEC

    - Samsung: due to different understanding of the delay budget regarding whether scheduling is taken into account or not, no additional gains are expected for TTI bundling

    - NEC: more typical use cases are #1 and #2. Simulations showed that changing RTT brings marginal gains

- Alt 3: TTI bundling enhancement is only supported for UL-DL configurations #0, #3, and #6

  - CATT, CATR, CMCC, Coolpad, Huawei, HiSilicon, Intel

  - Objected by: Ericsson, NEC, Samsung

- Alt 4: TTI bundling enhancement is supported for all UL-DL configurations

  - ZTE, Qualcomm, NEC, Fujitsu

  - Objected by: Intel, Ericsson, Huawei, HiSi, ALU, ASB, LGE

- Alt 5: TTI bundling enhancement is only supported for UL-DL configurations #1 and #2

  - NEC

- Alt 6: TTI bundling enhancement is only supported for UL-DL configurations #2, #3 and #4

  - Samsung, NEC

Conclusion:

- No consensus in any of the above alternatives

- Question:

  - Does the packet delay budget of 50ms account for any scheduling delay?

Not treated.

| R1-135076 | Coverage enhancement for TDD | CATT |  |
| --- | --- | --- | --- |
| R1-135108 | Discussion on Coverage Enhancement for TDD | Intel Corporation |  |
| R1-135206 | Consideration on coverage enhancement for TDD | Samsung |  |
| R1-135261 | On the need to extend coverage enhancement for Configurations #2, #3, #4, #5 | NEC |  |
| R1-135303 | TTI bundling for TDD | Qualcomm Inc. |  |
| R1-135468 | Discussion on TTI Bundling Extension for TDD configurations | LG Electronics |  |
| R1-135503 | Discussion on TTI bundling enhancement for TDD | HTC |  |
| R1-135580 | Coverage enhancements in TDD mode | NSN, Nokia |  |
| R1-135632 | Discussion on LTE coverage enhancement for TDD | CMCC |  |
| R1-135654 | On necessity of coverage enhancement for TDD | Ericsson |  |

#### Other

Not treated.

| R1-135029 | Remaining issues on enhanced TTI bundling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135163 | Signalling for TTI Bundling Enhancement for VoIP and PUSCH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135283 | Enabling and disabling of enhanced TTI bundling in Rel-12 | China Telecom |  |
| R1-135346 | Parameters related to enhanced TTI bundling | ZTE |  |
| R1-135469 | Remaining issues on TTI bundling enhancements for VoIP and medium rate PUSCH | LG Electronics |  |
| R1-135581 | Medium data rate PUSCH enhancements in FDD mode | NSN, Nokia |  |

| R1-135983 | Way Forward on configuration of TTI bundling enhancement | ZTE, CTC, Coolpad , Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | (R1-135979) |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

- For Rel-12 UEs, higher layer signalling is introduced to configure one of the following UL TTI bundling operations:

  - TTI bundling with Rel-12 bundling pattern without resource allocation restriction (subject to UL grant), or

  - TTI bundling with Rel-8 bundling pattern without resource allocation restriction (subject to UL grant)

Prepare an LS to RAN2 capturing the above agreements (excluding the conclusion for TDD), visit on Thursday – Zhisong

Thursday 14th

| R1-136022 | Draft LS on introduction of LTE coverage enhancement | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

The LS (along with the attached spreadsheet) is agreed with the following change:

- For Rel-12 UEs, higher layer signalling is introduced to configure one of the following UL TTI bundling operations for FDD:

- A new parameter is expected to be introduced in the higher layer,

- Also mentioned FDD in the spreadsheet

Agreed in R1-136055.

Friday 15th : The agreed LS should also be sent to RAN4 - post meeting decision for email approval until Nov 22nd.

### Further MBMS Operations Support

WID in RP-131369 - MBMS_LTE_OS-Core

| R1-136030 | Summary of Ad-hoc session on Further MBMS Operations Support | Ad-Hoc Chairman (Alcatel Lucent) |  |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and provides the outcomes of MBMS sessions.

Discussion (Question / Comment): The WI is considered completed from RAN1 perspective.

Decision: The document is endorsed and incorporated below.

| R1-135207 | Discussion on measurements for further MBMS operations support | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135667 | On radio measurements for further MBMS support | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135668 | MBSFN radio measurement definitions | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135839 | MBMS measurements for further enhancement of MBMS operation | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, CHTTL | (R1-135164) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135304 | Discussion of MBSFN measurements | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135305 | Initial MBSFN measurement evaluation | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135273 | Details of MBMS measurements | NEC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

- Working assumption confirmed to adopt MBSFN RSRP per MBSFN area

- Working assumption confirmed to adopt MBSFN RSRQ per MBSFN area

- MBSFN RSSI averaging is over only OFDM symbols carrying MBSFN RS

- Adopt an MCH BLER measurement per MCS per MBSFN area

  - FFS whether the measurement is per MCH or can be averaged across MCHs for a given MCS in the same MBSFN area.

- The measurements are made only in subframes and on carriers where the UE is decoding PMCH

- RAN4 to define the range and quantisation for the measurements

Background on MCH BLER measurement:

  - yes: NEC, E///, ALU, ASB, CATT, VzW, HW, HiSi,

  - no: Samsung, Nokia, NSN, LGE

Samsung has concerns about the complexity/testing for the UE.

Prepare draft CR for 36.214 based on the above agreements - Ericsson in R1-135911

Prepare draft LS to RAN2 and RAN4 to inform about the agreed measurements and send the draft CR – Qualcomm in R1-135912

Review on Thursday afternoon.

Thursday 14th

| R1-135911 | Draft CR on MBSFN radio measurements | Ericsson, Alcatel-Lucent, Qualcomm, Broadcom, Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Modifications:

- Delete sentence beginning “higher layers…”

- Add for all measurements inter-frequency measurements in []

- Add for all measurements: Note 3: The measurement is made only in subframes and on carriers where the UE is decoding PMCH.

- Delete “[FFS The MCH BLER estimation may consider MCH transport blocks of different MCHs of the same MBSFN area using the same MCS.]”

- Cover page: change to “MCH BLER” in summary of change.

CR is endorsed in R1-135918 36.214CR020. (Awaiting feedback from RAN2 to enable deleting square brackets - the CR will be approved as part of a package with the corresponding RAN2 CRs)

| R1-135912 | Draft LS on further MBMS operations support for E-UTRAN | Qualcomm Incorporated | R1-135919 |
| --- | --- | --- | --- |

Decision: The document is noted. Modify as follows:

RAN1 believes it is up to RAN2 to decide the applicability of the measurements to RRC_IDLE and RRC_CONNECTED.

…

RAN1 respectfully asks RAN2 to inform RAN1 about the applicability of the measurements for RRC_IDLE and RRC_CONNECTED and to take the above agreements into account in their future work on defining the MDT functionality for further MBMS operations support for E-UTRAN.

Delete use cases and scenarios

Update attachment filename x2.

Delete “[FFS whether the measurement is per MCH or can be averaged across MCHs for a given MCS in the same MBSFN area.]”

Add “the attached endorsed CR”.

Final LS is agreed in R1-135919.

Also discuss offline until Thurs. afternoon to see if consensus can be reached on MBMS CQI and/or RSTD.

Thursday 14th: no consensus.

Not treated.

| R1-135030 | Measurements for PMCH MCS adaptation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135031 | Details of MBMS measurements | Huawei, HiSilicon |  |
| R1-135077 | Measurement for MBMS | CATT |  |
| R1-135470 | Considerations on MBSFN UE measurement | LG Electronics |  |
| R1-135582 | Measurements for further MBMS operations support | Nokia, NSN |  |
| R1-135801 | Further MBMS operations support | Intel Corporation |  |

### Study on Small Cell Enhancements – Physical-layer Aspects

SID in RP-122032 - FS_LTE_SC_enh_L1 - Open issues in RP-131373

#### Higher order modulation

Mr Chair's recommendation: Focus on the standard impacts of introduction of 256QAM in downlink transmission, as well as evaluations considering the impact of power back-off.

Performance evaluation

| R1-135078 | Further evaluations of higher order modulation | CATT |  |
| --- | --- | --- | --- |

The document was presented by Zukang Shen from CATT and presents further evaluation results for 256QAM. It is observed that 256QAM can improve the spectral efficiency for four spatial layers transmission, and CRS based transmission also benefits from the introduction of 256QAM. Hence, it is proposed that at least four spatial layers transmission is supported for 256QAM, and both DMRS and CRS based transmission schemes are supported for 256QAM.

Decision: The document is noted.

| R1-135583 | Effect of eNB power-backoff on 256QAM system performance | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Klaus Hugl from Nokia and shows 256QAM DL modulation performance results in Scenario 2a:

- Observation 1: The needed lower power eNBs (<24dBm) to support 256QAM in Scenario 2 will result in lower system throughputs and increased macro cell load levels/decreased macro offloading capabilities compared to the agreed 30dBm low power nodes.

- Observation 2: The required power back-off needed for 256QAM operation assuming as reference 24dBm eNB hardware should be less than 2dBs to see any increase in average user throughput from 256QAM PDSCH operation in Scenario 2a.

- Observation 3: Any kind of required power back-off needed for 256QAM operation assuming some 24dBm eNB hardware results in significant coverage/5% throughput loss in Scenario 2a.

- Observation 4: When taking any required power back-off into account for lower power small cell nodes (≤24dBm), 256QAM operation does not provide overall system gains in small cell Scenario 2a. This becomes even more evident when comparing with slightly higher power nodes (30dBm) in this scenario.

- Observation 5: The required power back-off enabling 256QAM operation in Scenario 2b does not create coverage issues and it is not reducing the macro offloading capabilities. Mean system throughput gains of 12-16% are visible in Scenario 2b also taking the power back-off into account.

- Observation 6:  256QAM DL PDSCH operation requiring additional power back-off in lower power small cell eNBs is only reasonable in case of strong path-loss decoupling of macro and small cell nodes as in small cell scenario 2b due to the wall penetration loss in case of indoor small cells. For outdoor small cells, as in Scenario 2a, 256QAM PDSCH operation does not provide tangible performance benefits taking into account the lower output power needed to support 256QAM.

Decision: The document is noted.

| R1-135657 | Text Proposal for 256QAM in TR 36.872 | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Ms Yu Yang from Ericsson and proposes:

- Sort simulation results in TR 36.872 according to evaluation scenarios and TX/RX EVM to make it more readable.

- The observation and conclusion on the performance of 256QAM in the TR 36.872 should be only based on the results with simulation assumptions which take full consideration of RAN4 LS feedback.

- RAN1 should define new observations and conclusions for 256QAM at RAN1#75 meeting.

Decision: The document is noted.

| R1-135032 | Further evaluation on 256QAM | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei and concludes the following observations:

- With 2dB transmit power back-off, the geometry has negligible effect in the SCE scenarios.

- With 2dB power backoff, no more than 1% UEs change the serving cell from small cell to macro

  - hence power back-off has trivial impact on small cell coverage

  - Besides, a bias could be used for small cell range extension to keep the ratio of small cell UE (i.e. in small cell coverage) the same as without power backoff, with very limited impact on system performance

- With 2dB power backoff, the mean UPT performance has less than 2% change

  - hence power back-off has trivial impact on system performance

It can be concluded that low Tx EVM (3~4%) could be achieved by power backoff with little impact on coverage and system performance

- Low Tx EVM could also be achieved by eNB implementation

- The gain of 256QAM is feasible with practical EVM values.

Discussion (Question / Comment): Load balancing not applied for case 1 (was questioned by Intel)

Decision: The document is noted.

From there, Mr Chair drew the following observations:

- In scenario 2a sparse, the cell average UPT gain with 256QAM is x%

  - The gain decreases due to shrinkage of small cell coverage when 2 dB power back-off is considered

  - The coverage impact due to power back-off can be reduced by adjusting cell association bias

    - Intel: Not sure about this bullet because we can use cell association bias for baseline

    - ALU, Ericsson: also agreed with Intel’s view

    - ZTE: We can increase cell association bias when 2 dB power back-off is considered

    - Huawei: Cell association bias can decrease the impact of power back-off

    - CATT: Agree with Huawei’s view

- In scenario 2b sparse, the cell average UPT gain with 256QAM is x%

  - The gain is still visible even when 2 dB power back-off is considered

- In scenario 3 sparse, the cell average UPT gain with 256QAM is x%

  - The gain is still visible even when 2 dB power back-off is considered

- Impact of power back-off on the system performance is limited, at least for scenarios 2b and 3 with no more than 3 dB power back-off

  - Intel: Not sure about above bullet because we do not have RAN4 feedback, and not sure 3 dB power back-off is valuable one

- Continue offline discussion and prepare CR until Thursday – (Huawei)

Tuesday 12th

| R1-135958 | Draft CR on higher order modulation for Small cell enhancements | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei.

Decision: The document is noted and is modified as below:

From the evaluations in this section, it is observed that:

- In scenario 2a sparse, the cell average UPT gain with 256QAM without applying transmit power back-off is x~y% with CRS interference model

  - If 256QAM is operated without an additional implementation specific solution, e.g., adjusting cell association bias, the gain for eNB with transmission power of 30dBm decreases and even show loss due to shrinkage of small cell coverage with transmit power back-off from 30dBm to 23,22,21dBm

  - Cell mean UPT reduction is less than 2% with transmit power back-off from 30dBm to 28dBm

  - Cell mean UPT reduction is 7% with transmit power back-off from 24dBm to 21dBm

  - Note that RAN4 did not provide EVM values for eNB transmission power of 30 dBm

- In scenario 2b sparse, the cell average UPT gain with 256QAM is x~y% with CRS interference modelling

- In scenario 3 sparse, the cell average UPT gain with 256QAM is x~y% with CRS interference modelling

- At least for scenarios 2b and 3 with no more than 3 dB power back-off, the impact of power back-off  on the system performance is limited

Note: Including results of other simulated scenarios, all above results of x – y% should only consider EVM values recommended by RAN4

Note that the different evaluations between the sources for each scenario result from different assumptions, such as Tx/Rx EVM, CRS interference modelling and RU value.

Continue offline discussion and prepare CR until Thursday including x – y% values – Xia Yuan (Huawei)

Thursday 14th

| R1-136038 | Draft CR on higher order modulation for Small cell enhancements | Huawei, HiSilicon | (R1-136013) |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei.

Decision: The document is noted. CR is agreed in R1-136051 as CR0002 of TR36.872.

Specification impacts

| R1-135208 | Discussion on specification impacts of 256QAM | Samsung |  |
| --- | --- | --- | --- |

The document was presented by … from Samsung and proposes the possible alternatives for new design of CQI/MCS/TBS tables:

- Alt.1: Maintain the same number of entries in the existing tables and reformulate the entries

- Alt.2: Introduce more entries for 256QAM support on top of existing ones

Decision: The document is noted.

| R1-135395 | Specification Impact of Introducing 256QAM | Panasonic | (R1-132138) |
| --- | --- | --- | --- |

The document was presented by Michael Einhaus from Panasonic and proposes the following:

- The MCS field in the DCI should not be extended in order to minimize the impact on specification and robustness of downlink control signalling.

- The MCS field interpretation should be configured semi-statically by means of higher-layer signalling.

- A certain number of QPSK MCS levels should be replaced by 256QAM MCS levels in the MCS field interpretation that supports 256QAM, but at least one QPSK MCS level should be kept for robust transmissions.

- The TBS table should be extended in order to support larger transport block sizes for PDSCH transmissions with 256QAM

- The CQI field should not be extended in order to minimize the impact on specification and robustness of uplink control signalling.

- The CQI field interpretation should be configured semi-statically by means of higher-layer signalling.

- A certain number of QPSK CQI levels should be replaced by 256QAM CQI levels in the CQI field interpretation that supports 256QAM, but at least one QPSK QCI level should be kept for low SINR indication.

- MCS and CQI field interpretation should be linked to each other in the sense that higher layer signalling is used for semi-static configuration of a certain combination of MCS and CQI field interpretation.

- The impact on the differential CQI offset should be discussed.

Decision: The document is noted.

| R1-135450 | Specification Impacts for Higher Order Modulation in DL | KDDI |  |
| --- | --- | --- | --- |

The document was presented by Takeo Ohseki from KDDI and deals with the potential specification impacts on CQI/MCS/TBS tables for 256QAM support. It is stated as a preferred choice to increase:

- The number of bits for new MCS table for supporting 256QAM.

- The number of indices for new TBS table for supporting 256QAM.

- The number of bits for new CQI table in order to feedback the accurate channel state information.

Decision: The document is noted.

Alcatel Lucent: If 256QAM is introduced, then there is specification impact.

LGE; size of feedback field = 4 or 5 bits - size of feedback field is FFS according to AT&T - Fujitsu is in favour of LGE's proposal - agreed by AT&T

Conclusion:

- Introduce new CQI/MCS entries if 256QAM is introduced

  - The size of CQI/MCS table is FFS

  - Higher layer signalling is used to configure CQI/MCS table and/or 256QAM

- Introduce new TBS entries if 256QAM is introduced

- Size of feedback field is 4 or 5 bits if 256QAM is introduced

Thursday 14th

| R1-136037 | Way forward on 256QAM | Huawei, HiSilicon, ACER, AT&T, CATR, CATT, CHTTL, CMCC, Coolpad, China Unicom, Deutsche Telekom, DISH, ETRI, Hitachi, HTC, III, ITRI, Interdigital, KDDI, Kyocera, LGE, New Postcom, Panasonic, Potevio, Qualcomm, Samsung, SK Telecom, Telecom Italia, T-mobile, Verizon, ZTE | (R1-136012) |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei.

- Support higher order modulation, i.e. 256QAM, in Rel-12

  - To enhance the spectrum efficiency in downlink transmission

Discussion (Question / Comment): Intel  simulation results do not show significant gain to justify adopting such new feature.

Decision: The document is noted. To be revisited Friday morning.

Friday 15th: Ericsson suggested discussing the conclusion rather than spending too much time on this WF - conclusion may include the pros/cons using high modulation.

Mr Chair commented that there is clear majority in favour of supporting 256QAM, final decision may be left to plenary.

Intel re-emphasized the need for justification of the benefits from the supporting companies.

Fujitsu happy to include the recommendation to support it in the TR.

Nokia suggested that Rel-12 shouldn't not be mentioned - also agreed by Alcatel Lucent that there is no current commitment for Rel-12

AT&T suggested adding "for indoor cell scenarios with low mobility" - makes sense to Alcatel Lucent

AT&T then if the above add-on is agreed, REl-12 may be mentioned - Alcatel Lucent: do not mention the release is better

Intel did not object and agreed with the recommendation for the sake of making progress

Conclusion: CR to be drafted in R1-136058 (Huawei) to reflect that the followings are captured in the TR in higher order modulation section:

Agreements:

- Followings are captured in TR in higher order modulation section

- RAN1 finds DL higher order modulation, i.e. 256QAM beneficial in evaluated indoor sparse small cell scenarios with low mobility

- RAN1 recommends to support DL higher order modulation, i.e. 256QAM

| R1-136058 | Draft CR on higher order modulation conclusion | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed in R1-136079 as CR0007 of TR36.872

Not treated.

| R1-135033 | Standard impacts of 256QAM | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135079 | Analysis on specification impact of higher order modulation | CATT |  |
| R1-135165 | System analysis of supporting 256 QAM | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-135249 | Standard and other impacts of introducing 256 QAM | Texas Instruments |  |
| R1-135306 | Higher order modulation | Qualcomm Inc. |  |
| R1-135348 | Evaluation and standard impact on EVM and receiver impairment for small cell 256QAM | ZTE |  |
| R1-135471 | Specification impact on CQI for 256QAM support | LG Electronics |  |
| R1-135472 | Specification impact on DL signalling for 256QAM support | LG Electronics |  |
| R1-135584 | On Standardization Impact of DL Higher Order Modulation | Nokia, NSN |  |
| R1-135610 | Continuous wave jamming impact on high order modulation small cell transmission | III |  |
| R1-135655 | On standard impacts to support 256QAM in downlink | Ericsson |  |
| R1-135873 | Views on Higher Order Modulation in Rel. 12 | Hitachi Ltd. | (R1-135403) |
| R1-135879 | On 256QAM based on RAN4 feedback | Ericsson | (R1-135656) |
| R1-135880 | Evaluation results for 256QAM | Intel Corporation | (R1-135612) |

#### Small cell on/off and discovery

Mr Chair's recommendation:Focus on the enhancements for transition time reduction, including support of discovery and measurement enhancements.

Performance evaluation for small cell on/off and discovery

| R1-135081 | Further evaluations of small cell ON/OFF with 2Mbytes packet size | CATT |  |
| --- | --- | --- | --- |

The document was presented by Zukang Shen from CATT and provide evaluation results on small cell ON/OFF with realistic transition time scales and with 2M bytes packet size. Comparing the results with 0.5M bytes and 2M bytes, it is observed that the effect of OFF-to-ON transition time on packet throughput is reduced with the increase of the packet size. In other words, gains of small cell ON/OFF can still be observed with 6 MBSFN subframes configured.

It is also clarified that the UE-cell association ratio is different for the cases with and without small cell ON/OFF.

Decision: The document is noted.

| R1-135349 | Further evaluations on small cell ON/OFF | ZTE |  |
| --- | --- | --- | --- |

The document was presented by Huaming Wu from ZTE and concludes:

- Small cell on/off shows significant gains in both average and edge UE throughput when off-to-on transition delay is less than or equal to 200 ms.

- The gain of small cell on/off decreases as the transition time delay increases. There is a significant performance loss with large transition delay at 1000 ms, which is in the order of the delay time with existing RAN3 on/off mechanism.

- The on/off transition delay time impacts more in the case of 10 small cells in a cluster than in the case of 4 small cells in a cluster.

- Semi-static small cell on/off does maintain performance gain even when realistic hand-over procedure and minimum on time interval considered.

Decision: The document is noted.

| R1-135431 | Further evaluation for small cell on/off with different transition time scales | MediaTek Inc. |  |
| --- | --- | --- | --- |

The document was presented by Peikai Liao from MediaTek and concludes:

- Observation #1: Compared to the scheme of always on, there is no User Packet Throughput (UPT) gain to support dormant mode only in small cells.

- Observation #2: When the traffic load is low to medium (e.g. RU of macrocells is lower than 60%), UPT gain (up to 40% UPT gain) can be obtained for both macro and SC UEs from the support of small cell on/off operation if the transition time is less than 50 ms.

- Observation #3: When the traffic load is sufficiently high (e.g., RU of macrocells is higher than 60%), no UPT gain can be obtained from the support of small cells on/off operation no matter how small the transition time is.

- The target transition time for small cell on/off operation should be less than 50 ms.

- With the target transition time, enhancements to support small cell on/off operation should include both enhanced detection/measurement and enhanced procedures.

Decision: The document is noted.

| R1-135514 | Details of Small Cell On/Off with Small Cell Discovery | NTT DOCOMO |  |
| --- | --- | --- | --- |

The document was presented by Kazuaki Takeda from NTT DoCoMo and deals with small cell on/off mechanisms with a discovery signal in the operation of CA. The key point to reducing the transition time to data transmission and minimizing the CRS interference is to enable the UE to perform the measurement for both the on-state and off-state small cells. In order to achieve such a measurement, a small cell discovery signal is an essential feature. It is also found that the small cell on/off is effective in densifying the small cells to enhance the small cell layer capacity.

- Strive for specification of enhanced measurement and a discovery signal to facilitate efficient small on/off operation in Rel-12.

Decision: The document is noted.

| R1-135876 | Evaluation results for different small cell on/off operational modes | Ericsson | (R1-135659) |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson and concludes:

- The extent of small cell densification should be motivated by enhanced capacity.

- The small cell on/off feature is justified only if the obtained additional gains cannot be achieved by less densified networks.

- Small cell on/off operation based on HO results in performance losses in all cases even with optimistic evaluation assumptions

- Small cell on/off as an SCell activation/deactivation results in performance losses in most cases and only with marginal gains in the specific scenario of random CRS shift and dense clusters with optimistic evaluation assumptions.

- Small cell on/off on the serving cell shows gains in all cases irrespective of the load, cluster densification or effective CRS shift with larger gains obtained in cases of random CRS shift and denser clusters.

  - With random CRS shift, gains in order of 15-20% and 29-37% for 4 and 10 small cells per cluster, respectively, are achieved. These gains are somewhat reduced to 6-12% with sector aligned CRS.

- The only time scale for small cell on/off that shows sufficient performance increase to motivate it to be specified is the off-to-on and on-to-off transition times of 0 ms, i.e. small cell on/off by the serving cell.

Discussion (Question / Comment): Fujitsu questioned why gains increase (SC on/off with (t1,t2)=(0,0)ms and 10 SCs/cluster) with the resources utilization

Decision: The document is noted.

| R1-135893 | Draft_CR on small cell on/off for small cell enhancements | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei and proposes to update TR with the Small cell on/off evaluation results and corresponding observations related to evaluation results.

Discussion (Question / Comment): Huawei requested companies to check whether the results are correctly reported and give feedback if not.

Ericsson: Change the structure of performance evaluations part per scenario, and change the structure of performance evaluations part per scheme

TI: Change the structure of performance evaluations part per scheme

Huawei: Prefer to keep the current structure

ALU: Prefer to change conclusion

Decision: The document is noted. A possible conclusion is agreeing R1-135893 to include additional results and to adjust corresponding observations in TR

Continue offline discussion until Tuesday – (Huawei)

Tuesday 12th

| R1-135959 | Draft_CR on small cell on/off for small cell enhancements | Huawei, HiSilicon | (R1-135893) |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei and includes:

- Some updated values based on comments from the companies provided results.

- Added notes “Note that, in this set of results, relative long on period is assumed and hence transition time is not modelled” for evaluation results for a) Semi-static on/off scheme based on traffic load and b) Semi-static on/off scheme based on UE association.

- Added observations to differentiate the evaluation results for transition time of 3 ranges below 100ms, i.e., [80ms, 100ms], [20ms, 50ms], and [10ms, 15ms], which in a sense roughly align with what may be achieved by handover, activation/deactivation, and some faster procedure enhancements.

Discussion (Question / Comment): Update still required to reflect below conclusion on discovery signal.

Decision: The document is noted. Continue offline discussion until Thursday based on R1-135959 mainly focusing on conclusion parts – (Huawei)

Note that backward compatibility in Sec. 7.1.1.3.1 can be discussed in offline.

Thursday 14th

| R1-136020 | Draft_CR on small cell on/off for small cell enhancements | Huawei, HiSilicon | (R1-135959) |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

Discussion (Question / Comment): TI complains changing a text already in the TR without full consensus from the group.

CATT’s results in R1-135081 are not properly captured - should be in table 7.1.1.2-3 instead of table 7.1.1.2-1.

Decision: The document is noted. CR is agreed in R1-136052 as CR0003 of TR36.872 by adding following change:

7.1.1.6	Conclusions

The following are concluded:

- Dormant mode based on the current RAN3 mechanism is the starting point for possible enhancement related to small cell semi-static on/off.

- Reduced transition time of small cell on/off can increase the performance

  - RAN1 finds it beneficial to introduce the small cell on/off transition time reduction depending on the detailed scheme

  - The gain increases with decreasing transition time.

  - The small cell on/off with reduced transition time can use existing handover, CA activation/deactivation, dual connectivity (if supported) procedures

  - New L1 procedure for activated Scell operating on/off can further reduce transition time depending on the detailed solutions

- To support an enhanced transition time reduction, discovery procedure/signal(s) is needed:

  - Cells operating a cell on/off may transmit discovery signal(s) supporting at least for cell identification, coarse time/frequency synchronization, intra-/inter- frequency RRM measurement of cells and QCL

    - Note that QCL is not always necessary or possible depending on the procedure

    - Note that all QCL features might not needed

    - Need of RLM is FFS

  - This may include support of discovery and measurement enhancement(s) in DL and its usage in related procedures

7.1.1.3.1 	Feasible time scales based on legacy procedures

- The small cell on/off schemes that can be supported by legacy procedures include semi-static on/off based on traffic load, UE association, and/or based on data burst with transition time of a few hundred milliseconds (when all UEs connected to the cell are CA capable) to a few seconds.

- Small cell on/off with a short transition time might need new procedures.

- Cells with short time scale of small cell on/off are not expected to be able to serve legacy UEs without performance loss.

Discovery signal

| R1-135892 | Operator's views on efficient inter-frequency small cell discovery | NTT DOCOMO, TeliaSonera |  |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo and concludes:

- Observation #1: There is a demand from operators that the maximum number of carriers a UE can monitor for cell reselection or handover should be increased.

- Observation #2: Issues on the maximum number of inter-frequency measurement carriers would be worse in future small cell deployments.

- RAN1 should seek drastic solutions for inter-frequency small cell discovery enhancements in SCE SI/ WI, taking into account the issues discussed in RAN2/ RAN4.

Decision: The document is noted.

| R1-135166 | Small Cell Discovery Performance based on UL Signals | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, CHTTL |  |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and proposes uplink SRS-based blind handover and a small cell pseudo-ON scheme, showing the following benefits:

- Backward compatible to all UEs from Release 8 onwards.

- Minimizing the transition time of small cell ON/OFF to a few tens of milliseconds in the range of the delay of non-ideal backhaul.

- Minimizing the interference to the neighbouring cells by not transmitting downlink signals until sufficiently strong uplink signals are detected.

Decision: The document is noted.

| R1-135381 | Design target for discovery enhancement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei and proposes:

- DRS should enable UE to reliably detect at least the three strongest small cells.

- Defining new RRM measurement procedure or requirement for discovery of small cells in OFF mode is needed.

- If a DRS is introduced, it should be applied for small cells in OFF state in the first place. Whether a unified solution is needed for small cells in ON state and small cells in OFF state needs further study.

- Minimize the impacts from the transmission of DRS to the small cells in ON state.

- DRS design should also target scalability and future-proof.

Discussion (Question / Comment): CATT wondered why "the minimum number of small cells UE should reliably detect could be set to three"  the number 3 has been assumed by a number of companies.

Decision: The document is noted.

| R1-135833 | On RSRP gap requirement for small cell discovery | MediaTek Inc. | (R1-135432) |
| --- | --- | --- | --- |

The document was presented by Peikai Liao from MediaTek and lists the following observations:

- More small cells can be detected within a RSRP gap. The larger gap, the more detectable small cells.

- The portion of UEs associated with the strongest small cell decreases and the portion of UEs associated with 3rd and 4th strongest small cell increases when the RSRP gap grows larger.

- The RSRP measurement error and the target detection rate requirement have large impact on UE association ratio to small cells with different received signal strength.

- DRS-based cell detection, compared to CRS-based cell detection, provides better UPT when the same RSRP gap is applied.

- RSRP gap = 10 dB provides the best UPT.

It is proposed that RSRP gap requirement for small cell discovery should be 10 dB.

Decision: The document is noted.

At this stage, Mr Chair drew the following:

- To support small cell on/off operation, a discovery signal should be introduced. Other features are NW implementation.

Samsung: other features are NW implementation - does this applies to measurement enhancements?

Fujitsu: no need to discuss what's on the NW side - better remove the sentence

Panasonic: discovering signal AND discovery procedure is needed

Conclusion:

- To support an enhanced transition time reduction, discovery procedure/signal(s) is needed

- Cells operating a cell on/off may transmit discovery signal(s) supporting at least for cell identification, coarse time/frequency synchronization, intra-/inter- frequency RRM measurement of cells and QCL

  - Note that QCL is not always necessary or possible depending on the procedure

  - Note that all QCL features might not needed

  - Need of RLM is FFS

| R1-135516 | Discussion on issues for inter-frequency measurement | NTT DOCOMO |  |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo and deals with possible inter-frequency measurement enhancement based on the outcome of Rel. 12 SCE efficient discovery studies. A text proposal to TR 36.872 is attached to it.

Decision: The document is noted.

Possible conclusion and observation:

- If new discovery signal(s)/procedure(s) are defined, they may be used in cells not operating a cell on/off

- Discovery and measurement enhancement(s) in Rel. 12 should enable UE to reliably detect and measure at least the three strongest small cells subject to meeting the side conditions (e.g., for RSRP)

- Continue offline discussion including TP in R1-135516 and a possibility to include additional performance evaluation results to TR until Tuesday – Hiroki (NTT DOCOMO)

Tuesday 12th

| R1-135957 | Way Forward on Discovery and Measurement Enhancement(s) | NTT DOCOMO, ZTE, ETRI, Panasonic, Samsung |  |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo and concludes:

- If new discovery signal(s)/procedure(s) are defined for small cell on/off, they may be considered in cells not operating a cell on/off

  - The new discovery signal(s)/procedure(s) do not replace existing Rel-8/11 discovery signal(s)/procedure(s).

Observation:

- Discovery and measurement enhancement(s) in Rel. 12 should enable UE to reliably detect and measure at least the three strongest cells subject to meeting the side conditions (e.g., for RSRP)

Decision: The document is noted and is modified as below:

Possible conclusion:

- If new discovery signal(s)/procedure(s) are defined for small cell on/off, they may be considered in cells not operating a cell on/off at least for inter-frequency measurement

  - From UE perspective, it may no need to distinguish cells supporting on/off and cells not supporting on/off

  - UE supporting new discovery signal(s)/procedure(s) should also support Rel-8/11 discovery signal(s)/procedure(s)

- Definition of new discovery signal(s) includes a possibility of existing signal(s) itself with new time and/or freq. resource configuration without excluding Rel-8 resource configuration

Possible observation:

- Discovery and measurement enhancement(s) in Rel. 12 should enable UE to reliably detect and measure at least the three strongest cells subject to meeting the side conditions (e.g., for RSRP)

Continue offline discussion including TP in R1-135516, a possibility to include additional performance evaluation results to TR, and cell perspective until Thursday – (NTT DOCOMO)

Thursday 14th

| R1-136005 | Draft_CR on small cell discovery for small cell enhancements | NTT DoCoMo |  |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo.

Decision: The document is noted. Continue offline discussion until Friday – (NTT DOCOMO)

Possible agreement:

- It is identified that energy-efficient inter-frequency small cell discovery is beneficial, especially in the case that the number of inter-frequency measurement carriers is increased to more than three. Multiple factors may influence the energy-efficiency of inter-frequency small cell discovery, for example:

  - the number of processing samples required to achieve accurate detection,

  - the number of processing samples required to achieve accurate measurements,

  - the energy required per sample, e.g. due to discovery and measurement signal processing complexity, and the energy cost of carrier switching,

  - the degree of network synchronization and assistance, e.g. timing information, and

  - Whether or not a UE can discover the presence of inter-frequency cells without performing inter-frequency measurements, e.g. by detecting a small cell discovery signal on a coverage carrier.

Continue offline discussion including Section 7.2.1.6 in R1-136005 until Friday – (NTT DOCOMO)

Friday 15th: Revision in R1-136056

| R1-136056 | Draft_CR on small cell discovery for small cell enhancements | NTT DoCoMo | (R1-136005) |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo.

Discussion (Question / Comment): CATT: section 7.2.1.5 (UL signal) likely misplaced - where to place it needs to be figured out.

Discovery performance part was debated.

Decision: The document is noted and is agreed in R1-136059 as CR0006 of TR36.872 by adding following update

6.2.6.3	Evaluation on opportunistic activation and deactivation of small cells based on UL signals

In evaluations, the existing UL signals are utilized as the discovery signal by the dormant cell to detect proximity of active UEs.  UL transmissions can be utilized to trigger a cell to transition from a dormant state to start transmission of DL-SS/RS.

Based on one source evaluation, if SRS SINR threshold is set to be 10dB in scenario 1 defined in this TR, there are 80% UEs detected by the 1st strongest neighbouring cell, 60% UEs detected by the 2nd strongest neighbouring cell and 40% UEs detected by the 3rd strongest neighbouring cell.

Not treated.

| R1-135034 | Discovery signals for energy-efficient small cell discovery | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135035 | Small cell on/off transition time reduction | Huawei, HiSilicon |  |
| R1-135036 | Discovery and measurement enhancements | Huawei, HiSilicon |  |
| R1-135080 | Discussion on mechanisms for small cell ON/OFF | CATT |  |
| R1-135082 | (Non)-Backward compatibility of small cell ON/OFF | CATT |  |
| R1-135083 | RRM measurement for small cells | CATT |  |
| R1-135109 | Evaluation on small cell discovery signal | Intel Corporation |  |
| R1-135138 | Specification support for UL based cell discovery | Fujitsu |  |
| R1-135209 | Discussion on transition time and adaptation enhancements for small cells | Samsung |  |
| R1-135210 | Further discussion on small cell discovery | Samsung |  |
| R1-135211 | Further discussion on small cell load shifting and cell association | Samsung |  |
| R1-135250 | On enhancements for transition time reduction for small cell on/off and discovery | Texas Instruments |  |
| R1-135262 | Small cell on/off mechanism and discovery signal | NEC |  |
| R1-135276 | Small cell discovery and cell state transition | ETRI |  |
| R1-135307 | Small cell on/off | Qualcomm Inc. |  |
| R1-135308 | Small cell discovery | Qualcomm Inc. |  |
| R1-135350 | Further considerations on enhanced transition procedures of small cell on/off scheme | ZTE |  |
| R1-135351 | On Measurements based on Companion Discovery Signal (CDS) | ZTE |  |
| R1-135352 | Backwards compatibility of semi-static small cell on/off | ZTE |  |
| R1-135353 | On the benefits of introducing a discovery signal | ZTE |  |
| R1-135396 | Definition of backward compatibility | Panasonic |  |
| R1-135397 | Discussion on small cell on/off transition time reduction procedure | Panasonic | (R1-134374) |
| R1-135405 | Discussion of transition time in dormant mode on-off switching | Hitachi Ltd. |  |
| R1-135434 | New small cell discovery and measurement scheme based on CRS | MediaTek Inc. |  |
| R1-135448 | Discussion on Enhancement for Semi-static Small Cell on/off | ITL Inc. |  |
| R1-135473 | Further discussions on small cell on/off | LG Electronics |  |
| R1-135515 | Views on target design for small cell discovery enhancement | NTT DOCOMO |  |
| R1-135539 | Small cell semi-static ON/OFF with a discovery signal | Broadcom Corporation |  |
| R1-135585 | On small cell discovery and on/off enhancements | NSN, Nokia |  |
| R1-135608 | Enhancements for transition time reduction | InterDigital |  |
| R1-135634 | Enhancement of Small Cell operation efficiency | CMCC |  |
| R1-135658 | Small cell on/off operational modes | Ericsson |  |
| R1-135660 | IDLE mode support for semi-static small cell on/off operation | Ericsson |  |
| R1-135661 | On network signalling support for small cell on/off | Ericsson |  |
| R1-135662 | Use cases for discovery signals | Ericsson |  |
| R1-135663 | On the design of discovery signals | Ericsson |  |
| R1-135664 | Conclusion for small cell on/off and discovery signal | Ericsson |  |
| R1-135673 | Evaluation of small cell on/off with realistic time scale | Coolpad |  |
| R1-135676 | Further Considerations on Small Cell On/Off | Motorola Mobility |  |
| R1-135677 | Evaluation of Small Cell On/Off | Motorola Mobility |  |
| R1-135824 | Backwards compatibility of small cell on/off | Fujitsu | (R1-135137) |
| R1-135834 | On detection rate requirement for small cell discovery | MediaTek Inc. | (R1-135433) |

#### Further assessment of solutions for radio-interface based synchronization

Mr Chair's recommendation: Focus on evaluations of the achievable accuracy of network listening, and on the benefits of standardized solutions.

Performance evaluation

| R1-135037 | Evaluation results for network listening | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei and lists the following observations:

- For the case where the non-air-interface based frequency provider is available, if the SINR for network listening is worse than a threshold, e.g., -13dB, the target cell will be out of synchronization.

- According to frequency performance by network listening, in order to meet the frequency synchronization target, the maximum hop number supported within each cluster should be restricted.

- When only one small cell within each cluster directly acquires the reference timing from the macro cell or GPS ,a large ratio of small cells are out of synchronization if frequency synchronization is performed by network listening. Accordingly, the limiting factor of the achievable accuracy is frequency synchronization.

- When all small cells within each cluster directly acquire the reference timing from the macro cell which can provide the best SINR, the ratio of small cell clusters which will select more than one macro cell to be the source cells is as larger as 90%.

- When all small cells within each cluster directly acquire the reference timing from the same macro cell, the ratio of small cells will be out of synchronization is larger than 15%.

- When all small cells within each cluster directly acquire the reference timing from the same macro cell, how to ensure all small cells within each cluster select the same reference timing should be FFS.

In order to meet the synchronization performance requirement, it is proposed that RIBS enhancements (detailed in R1-135038) are supported to improve synchronization accuracy.

Decision: The document is noted.

| R1-135586 | Performance Evaluation of Radio-Interface based Synchronization | NSN, Nokia |  |
| --- | --- | --- | --- |

The document was presented by Timo Lunttila from NSN and concludes that, for the source cell selection Case 2, with no limitation on the number of small cells that directly acquire the reference timing from Macro cell layer, all the indoor small cells can synchronize with the overlaid Macro cell.

- The existing radio-interface based synchronization mechanism works decently also in the dens small cell scenario. There is no need for any further enhancement to the radio-interface based synchronization.

Decision: The document is noted.

| R1-135633 | Further evaluations on radio-interface based small cell synchronization | CMCC |  |
| --- | --- | --- | --- |

The document was presented by Xiaodong Shen from CMCC and provides the following observations:

- As the SINR improved, the timing error of single-hop synchronization is also reduced, and better SINR threshold is needed to satisfy the multiple-hop timing error requirement.

- In the case of multiple-hop synchronization, a part of small cells cannot match the relative SINR threshold and will lose the synchronization reference timing by tracking the legacy CRS.

- Performance benefit of improving the timing accuracy can be achieved by introducing some enhancements to the radio-interface based network listening for small cells.

It is proposed that:

- Network listening by tracking the legacy CRS periodically with one measurement sample as the traditional way cannot guarantee all small cells in the cluster to catch the synchronization source cell, and enhancement is needed for further study in the WI phase;

- Solutions for radio-interface based small cell synchronization should take the timing error requirement for supporting multiple-hop synchronization into account.

Decision: The document is noted.

Possible observations: (based on the assumptions agreed in RAN1 #74bis meeting, R1-134966)

- Based on contributions in this meeting, by link-level evaluations, for each hop, all companies assumed 1 msec measurement interval with the period of 10 sec inter-measurement period,

  - For the time sync. performance with no propagation delay, and no implementation margin

    - About 90% time sync. error falls into [-1 - -0.1 s, +0.1 – 1 s] when SINR is about -10 dB

  - For the freq. sync. performance with no freq. error at source cell and assuming source cell is GPS

    - About 90% freq. sync. error falls into [-150 Hz - -100 Hz, + 100Hz - 150 Hz] when SINR is -10 dB

  - When case 1 for source cell selection is assumed

    - 4 – 6 hops are observed; freq. sync. accuracy and/or time sync. accuracy may not be assured for all the small cells

    - This observation does not assume muting and CRS IC

    - Note that the achievable accuracy may depend on definition of sync. error, source cell selection mechanism

    - Freq. sync. accuracy and time sync. accuracy can be assured for all small cells when coordinated muting is assumed as described in TR36.922

  - When case 2 for source cell selection is assumed, assuming each small cell listens to macro cell with the strongest received signal

    - Almost all the small cells are at an SINR higher than -5 dB

    - A substantial number of small cell clusters listen to more than one macro cells

    - Time sync. of small cells is not assured when macro cell layer is not synchronized

      - Time sync. of small cells can be assured when macro cell layer is synchronized assuming additional signalling

      - Schemes to ensure time and freq. sync. when macro cell is not synchronized have been proposed

- Need to prepare CR considering all evaluation results in this meeting, all detailed assumptions, and formatting

- Note that if we assume different values for measurement interval and inter-measurement period, different observation might be derived

- Continue offline discussion until Tuesday – (Huawei)

Possible observations proposed by Ericsson:

- Radio-IF based sync. is feasible based on the existing signals and depending on scenarios

- Signalling enhancement may be considered for some scenarios

- Muting and CRS IC can be supported by exploiting existing solutions

- Continue offline discussion until Tuesday – (Huawei)

Comments:

- ALU: Not sure how any sync. can work below SINR -10 dB. NW listening should work for any situations

- Ericsson: Observation highly depends on the scenario.

Tuesday 12th

| R1-135956 | Draft CR on Radio-interface based synchronization for Small cell enhancements | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei.

Discussion (Question / Comment): Structure of tables is proposed to further collect results. Ericsson requested more time to share their view on potential different structure.

Decision: The document is noted.

Thursday 14th

| R1-136027 | Corrections on Radio-interface based synchronization for small cell enhancements | Huawei, HiSilicon, CATR | (R1-135956) |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei.

Discussion (Question / Comment): LGE requested adding the following in section 7.3.1.3:

- synchronization status of overlaid macro layer

- source cell selection mechanism

Huawei: could be captured in another CR

Huawei rephrased the first bullet as "synchronization status of the original synchronization sources"

Decision: The document is noted. CR is agreed in R1-136053 as CR0004 of TR36.872 by adding in Section 7.3.1.3:

- Synchronization status of the original synchronization sources

- Source cell selection mechanism

Not treated.

| R1-135084 | Air interface based synchronization for small cells | CATT |  |
| --- | --- | --- | --- |
| R1-135168 | System analysis and performance evaluation of radio-interface based synchronization | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, CHTTL |  |
| R1-135212 | Further discussion on radio-interface based synchronization solutions | Samsung |  |
| R1-135354 | Further evaluations on Radio-interface Based Synchronization | ZTE |  |
| R1-135474 | Considerations on small cell network synchronization based on network listening | LG Electronics |  |
| R1-135551 | On solutions for radio-based synchronization | BlackBerry UK Limited |  |
| R1-135829 | Simulation results on radio-interface based synchronization | CATR | (R1-135679) |
| R1-135874 | Discussion and Conclusions for Radio-Interface Based Small Cell Synchronization | Ericsson | (R1-135665) |
| R1-135875 | Evaluation of Radio-Interface Based Small Cell Synchronization | Ericsson | (R1-135666) |

Thursday 14th

Standards impact

| R1-135038 | Enhanced mechanisms for network listening | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei and concludes:

- Specification work is needed to support the indication of stratum level and network listening RS resource.

  - It is preferable to indicate the stratum level by explicit or implicit physical layer signalling

  - It is preferable to specify one predefined relationship between synchronization stratum level and the occurrence of network listening RS.

- Introduce flexible GP configuration for TDD system to increase the supported hop number.

- Define the maximum hop number of network listening.

  - The maximum hop number should be larger than two. The exact value is FFS

- Confirm by evaluation the solutions to improve synchronization detection accuracy. Candidate solutions include:

  - PDSCH-RE-muting to the network listening RS

  - SFN transmission of the network listening RS

  - Transmit network listening RS in consecutive subframes

  - Joint use of the above solutions can also be considered.

- The network listening solution needs to:

  - be applied and compatible with eIMTA and small cell on/off

  - facilitate inter-operator synchronization

Decision: The document is noted.

| R1-135309 | Network synchronization by network listening | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Peter Gaal from Qualcomm and proposes to capture in the small cell study item conclusions the following:

- Adopt the network listening techniques specified in TR36.922 for small cell synchronization.

- It is important to enable backhaul messaging in support of eNB network listening in all interfaces applicable to open small cell operation, including X2.

- Interaction with other Rel-12 features should be considered such as

  - eIMTA

  - Small cell On/Off

The target requirements for eNB synchronization, both for FDD and TDD should be those specified in [3] for TDD HeNB

Decision: The document is noted.

| R1-135406 | Impact of small cell on-off switching on radio interface-based network synchronization | Hitachi Ltd. |  |
| --- | --- | --- | --- |

The document was presented by … from Hitachi and proposes that:

- Whether received signal power, distance, SINR, stratum level, or some combination thereof should be the criterion for synchronization source cell selection (with and without on-off switching) needs further evaluation for different hops.

- Interference mitigation techniques such as on-off switching should be evaluated to enhance SINR and therefore synchronization accuracy, with priority for small cell scenarios 2b and 1.

- The accuracy of synchronization based on current and enhanced dormant mode discovery signals should be evaluated.

- The impact of source cells being switched OFF for time scales corresponding to the baseline dormant mode mechanism should be evaluated.

- Coordinated on-off switching should be adopted to minimize adverse impact on existing and new network listening mechanisms.

Decision: The document is noted.

| R1-136035 | Corrections on the summary of RIBS for small cell enhancements | Huawei, HiSilicon, Ericsson, CATR |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei.

Discussion (Question / Comment): LGE can the changes be explained?

LGE: simulation assumptions do not fully support propagation delay, frequency error, implementation margin, feasible solution not identified

Ericsson would agree adding statement on implementation margin, not on the other concerns

Decision: The document is noted. CR is agreed in R1-136054 as CR0005 of TR36.872 by reflecting the followings:

It is beneficial to support radio-interface based synchronization for cases when other methods such as GNSS or synchronization over backhaul are not available. Network listening solutions have been considered and evaluated.

It is observed that radio-interface based synchronization (network listening) is feasible based on the existing signals for the scenarios defined in the simulation assumptions according to the evaluation results as described in Sec. 7.3.1.10.

- Note that not all following aspects are taken into account in evaluations

  - Errors due to propagation delay

  - Frequency error

  - Implementation margin

Applicability/compatibility of synchronization approaches with other ongoing studies should be considered.

To support network listening in small cell deployments, the following potential standards impacts may need to be considered:

- Depending on the scenarios, improvement on the achievable synchronization accuracy by improving the hearability of received network listening RS at the target cells

- The indication of the synchronization stratum level of up to approximate 4-6 hops

For the deployment among the cells of different TDD operators deployed in the same band and same region, mechanisms to facilitate inter-operator synchronization should be considered.

#### Other

Not treated.

| R1-135110 | EPDCCH ICIC in Small Cell Environment | Intel Corporation | (R1-134136) |
| --- | --- | --- | --- |
| R1-135169 | Draft text proposal on physical layer support of dual connectivity | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, CHTTL |  |
| R1-135263 | Views on interference coordination for semi-static small cell on/off | NEC |  |
| R1-135264 | Physical layer aspects for dual layer connectivity | NEC |  |

Thursday 14th

| R1-136002 | Summary of offline session on Small Cell Enhancements | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

- Higher order modulation  R1-136012/R1-136013

- Small cell on/off  R1-136020

- Discovery  R1-136005

- Solutions for radio interface based synchronization  R1-136027

Discussion (Question / Comment): NTT DoCoMo commented that consensus was reached during the offline discussion:

- If new discovery mechanism is defined for small cell on/off, it may be used for other purposes without further optimization.

Decision: The document is noted.

Agreement

- If new discovery mechanism is defined for small cell on/off, it may be used for other purposes without further optimization.

The following point needs further discussion:

- If new discovery mechanism, new discovery mechanism in Rel. 12 should enable UE to detect and measure at least the three strongest cells within RSRP gap of X dB

  - Sensitivity threshold is needed as well

Objected by Alcatel Lucent

Agreements:

- If new discovery mechanism is defined, new discovery mechanism should consider the design criteria that UE is able to detect and measure at least A strongest cell(s) including the strongest cell within RSRP gap of B dB

  - Sensitivity threshold, C is needed as well

  - A >= 1

  - B >= 0

  - Note that A, B, and C need to be discussed

  - RSRP gap is the gap from the strongest RSRP cell

  - Reliability requirement of detection probability, D by discovery signal up to E second needs to be discussed

  - Accuracy requirement of measurement, F by discovery signal up to G second needs to be discussed

  - Note that achievability of those values depend on deployment scenarios

### Study on 3D-channel model for Elevation Beamforming and FD-MIMO

SID in RP-122034 revised in RP-130811 - FS_LTE_3D_channel

| R1-136031 | Summary of Ad-hoc session on 3D-channel model for Elevation Beamforming and FD-MIMO | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and provides the outcomes of 3D sessions.

Discussion (Question / Comment): all email discussions deadline be shifted one week earlier with the exception for High Rise Scenario.

Decision: The document is endorsed and incorporated below.

| R1-135587 | TP for polarization and mechanical tilt modelling | NSN, Nokia | R1-136021 |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135312 | Field pattern of cross-polarized antenna | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135708 | Text Proposal on Mechanical and Electrical Antenna Tilting | Fraunhofer IIS | (R1-134948) |
| --- | --- | --- | --- |

Decision: The document is noted.

Continue offline discussion and revisit on Thursday potential TP incorporating the proposals in the above 3 document – Bishwarup (NSN)

Thursday 14th

| R1-136021 | TP for mechanical tilt | NSN, Nokia | (R1-135587) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-136036 | WF on Modelling Field Pattern of Polarized Antenna | Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- R1-136021 is agreed in principle except the yellow part

- For the yellow part, email discussion to conclude the yellow part taking into account R1-136036 till 11/29/2013 11/22/2013 – Bishwarup (NSN)

Not treated.

| R1-135709 | An Approach for a 3D Channel Model with Time Evolution (QuaDRiGa) | Fraunhofer IIS | (R1-134794) |
| --- | --- | --- | --- |
| R1-135971 | Text Proposal on Mechanical and Electrical Antenna Tilting | Fraunhofer IIS | (R1-135708) |

#### Remaining Details of fast fading modelling for 3D-UMa and 3D-UMi

| R1-135819 | Remaining Details of Fast Fading Channel for 3D UMa | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-135170) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135820 | Remaining Details of Fast Fading Channel for 3D UMi | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-135171) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135900 | Remaining details of fast fading modelling for 3D channel | Samsung | (R1-135213) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135895 | Remaining issues of EoD/EoA modelling | Qualcomm Inc. | (R1-135310) |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- In Step 4 for large-scale parameters, the randomly generated r.m.s. ESD/ESA values are limited to 52°, i.e., σθ ← min(σθ, 52°).

| R1-135821 | Remaining details on Fast Fading Model for 3D-channel | NTT DOCOMO | (R1-135518) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135528 | Views on fast fading modelling for 3D UMA and 3D UMI | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135840 | Remaining details of fast fading modelling | Huawei, HiSilicon | (R1-135531) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135948 | Remaining details of fast-fading for 3D-UMa and 3D-UMa | NSN, Nokia | (R1-135588) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135902 | Remaining issues on fast fading modelling of 3D UMa/UMi | CMCC | (R1-135635) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135765 | Properties of EoD for 3D-Channel Model | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135947 | Way Forward on EOD parameters | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted. Discuss further offline using R1-135947 as a starting point, and check on Thursday

Thursday 14th

Working assumption:

- The following tables are as a working assumption.

  - The proposed average ESD is smaller than that of Winner+

Table 1: Elevation parameters for 3D UMa

| Scenarios |  | 3D-UMa |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O-to-I |  |
|  |  |  |  | LOS O-to-I | NLOS O-to I |
| ZoD spread (σZSD) log10([]) | ZSD | max[-0.5, -2.1(d2D/1000)+ +0.75] | max[-0.5, -2.1(d2D/1000) +0.9] | max[-0.5, -2.1(d2D/1000)-0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |
|  | ZSD | 0.40 | 0.49 | 0.40 | 0.49 |

Table 2: Elevation parameters for 3D-UMi

| Scenarios |  | 3D-UMi |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O-to-I |  |
|  |  |  |  | LOS O-to-I | NLOS O-to I |
| ZoD spread (σZSD) log10([]) | ZSD | max[-0.5, -2.1(d2D/1000)+0.01*¦1.5- 10¦+0.75] | max[-0.5, -2.1(d2D/1000) +0.9] | max[-0.5, -2.1(d2D/1000)+0.01*¦hUT - hBS¦+0.75] | max[-0.5, -2.1(d2D/1000)+0.01*max(hUT - hBS,0)+0.9] |
|  | ZSD | 0.4 | 0.6 | 0.4 | 0.6 |

| R1-135951 | Way Forward on UE orientation and UE antenna pattern in 3D-channel | LG Electronics, Fraunhofer, Elektrobit, ETRI, Anite, KDDI |  |
| --- | --- | --- | --- |

Decision: The document is for email discussion till November 22nd 11/29 – Jonghyun (LGE).

| R1-136011 | Way Forward on ZOD offset and ZOD generation | NSN, Nokia, Samsung, CMCC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

The proposal in R1-136011 is agreed as a working assumption except the following:

- offset,ZOD=0 for W for all cases

- Equation (3) and Tables 2/3

  - Equation (3)/Tables 2/3 as a baseline for calibration/baseline simulations only (there are concerns about them). Email discussion to conclude issues related to equation (3) and Tables 2/3 till November 22nd 11/29 – Bishwarup (NSN) - (alternative proposals to be provided 11/22 in order to conclude the issues. If no alternative proposals, Equation (3) and Tables 2/3 will be agreed as working assumption).

| R1-136026 | Some Remaining Details for 3D channel modelling, | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Samsung, LG Electronics, Huawei, HiSilicon, NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- Proposals in slides 2, 3, and 4 R1-136026 are agreed

- Proposals in slide 5 are as a working assumption with the following change:

  - Assuming that generated ZoD and ZoA in Step 7 have been wrapped within [0, 360]

- Note: the proposals in R1-136026 are generally for simplicity. Vertical autocorrelation distance of 3m discussed in slide 4 is for simplicity due to lack of measurement results, and with this, there is no correlation between different floors, although it may not represent the reality.

Not treated.

| R1-135214 | Discussion on UE-side modelling for 3D channel | Samsung |  |
| --- | --- | --- | --- |
| R1-135311 | TP for 3D channel model | Qualcomm Inc. |  |
| R1-135475 | UE orientation and antenna pattern in 3D channel model | LG Electronics |  |
| R1-135540 | Computation of Antenna Fields for 3D System Simulation | Broadcom Corporation |  |
| R1-135999 | Remaining details of fast fading modelling for 3D channel | Samsung | (R1-135900) |

#### Remaining simulation assumptions for channel model calibration / baseline

Calibration/Baseline Simulation Assumptions

| R1-136018 | Way Forward on simulation assumptions of baseline performance simulation | Huawei, HiSilicon, CATR, CATT, CMCC, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, InterDigital, Intel, Samsung, LGE, KDDI |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

For both UMa and UMi:

- Antenna configuration

  - 4 horizontal antenna ports at BS, cross-polarization, antenna spacing  0.5λ

  - 2 receive antennas at UE, cross-polarization

- Transmission scheme

  - TM10

    - Where the ideal interference is from PDSCH, which can be measured by IMR. A single CSI process is assumed.

  - SU-MIMO with rank adaption

- CRS port 0 is used for RSRP computation for UE attachment

- Output of baseline performance simulation

  - Performance of the transmission scheme under 3D channel model

    - UE distribution follows TR36.873

    - K=M=10,N=2 , the complex weight factor for vertical element  is defined in TR36.873

  - Evaluation metrics:  cell average spectrum efficiency, 5th percentile cell edge spectrum efficiency

- Note: This does not imply that this antenna configuration (N=2,M=10) is prioritized over others in future SIs.

UE Attachment

| R1-135952 | Way Forward on RSRP calculation formula | LG Electronics, CATR, Huawei, HiSilicon, Ericsson, Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Thursday 14th

| R1-136000 | Way Forward on RSRP calculation formula | LG Electronics, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Broadcom, CATR, CATT, Fraunhofer IIS, Hitachi, Huawei, HiSilicon, InterDigital, Ericsson, NEC, Qualcomm | (R1-135952) |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- R1-136000 is agreed with the clarification that TXpower is CRS transmitted power per RE

Not treated.

| R1-135039 | Simulation assumptions for baseline performance evaluation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135085 | Discussion on simulation assumptions for calibration | CATT |  |
| R1-135111 | On baseline simulation assumption for 3D beamforming | Intel Corporation |  |
| R1-135172 | UE Attachment Modelling for Phase 2 Calibration | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135215 | Assumptions for baseline simulations | Samsung, CHTTL |  |
| R1-135284 | Discussion on large scale parameters generation | China Telecom |  |
| R1-135313 | Discussion on RSRP modelling | Qualcomm Inc. |  |
| R1-135409 | Remaining Simulation Assumptions for 3D Channel Model Calibration | Hitachi Ltd. |  |
| R1-135476 | Phase 2 simulation assumptions for evaluating the baseline performance | LG Electronics |  |
| R1-135589 | Remaining simulation assumptions for channel model calibration/baseline | NSN, Nokia |  |
| R1-135766 | On RSRP determination for UE to node association | Ericsson |  |

#### Initial calibration results

Phase-1 Calibration

| R1-135949 | Summary of Phase-1 calibration results according to [74bis-12] email discussion | NSN, Nokia | (R1-135590) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135767 | Initial calibration results for 3D channel model | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document (Section 4 only) is noted.

Email discussion till November 22nd 11/29 to conclude one of the following two alternatives – George (Ericsson):

- Alt 1: geographical distance based wrapping

- Alt 2: radio distance base wrapping

Conclusion:

- Phase-1 and phase-2 calibration results to be updated by taking into the account decisions made in this meeting (including additional agreements via email discussion) – PoC Bishwarup (NSN)

Not treated.

| R1-135040 | Initial calibration results of 3D channel modelling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135086 | Phase I calibration results for 3D channel modelling | CATT |  |
| R1-135087 | Initial calibration results for 3D fast fading channel modelling | CATT |  |
| R1-135112 | Phase one calibration results for 3D channel model | Intel Corporation |  |
| R1-135139 | Initial calibration results for 3D channel modelling | Fujitsu |  |
| R1-135216 | Phase-1 calibration results for 3D channel model | Samsung |  |
| R1-135265 | Phase-2 calibration results for fast fading parameters | NEC |  |
| R1-135314 | Initial calibration of channel models for elevation beamforming and FD-MIMO | Qualcomm Inc. |  |
| R1-135408 | Initial Calibration Results for 3D Channel Modelling | Hitachi Ltd. |  |
| R1-135477 | Initial results for phase-1 calibration | LG Electronics |  |
| R1-135519 | Phase 1 Calibration Results for 3D-Channel Model | NTT DOCOMO |  |
| R1-135520 | Initial Phase 2 Calibration Results for 3D-Channel Model | NTT DOCOMO |  |
| R1-135526 | Initial phase 2 calibration results of 3D channel model | ZTE |  |
| R1-135549 | Initial results for 3D Channel Model Phase 2 and different UE attachment options | Orange | Late |
| R1-135604 | Phase 2 Calibration Results for 3D Channel Model | InterDigital |  |
| R1-135636 | Initial fast fading calibration results | CMCC |  |
| R1-135811 | Phase 1 Calibration Results for 3D Channel Model | CHTTL |  |
| R1-135812 | Phase 2 Calibration Results for 3D Channel Model | CHTTL |  |

#### Details of 3D UMa with one high rise per sector with 300m ISD

| R1-135637 | Details of high rise channel modelling impacts | CMCC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-136015 | Way Forward on High-Rise Scenario | CMCC, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Broadcom, CATR, CATT, CHTTL, Huawei, HiSilicon, KDDI, Qualcomm, Samsung, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- Observation:

  - Scenario description

    - One high-rise per sector with 300m ISD

  - It is important to model high rise UEs within buildings to ensure consistency with the proposed scenario

- Proposal: High-Rise buildings are modelled in system level evaluation:

  - UEs in high-rises are dropped within 25m radius of the position of their respective high rise, elevation as already agreed

  - Note: clustered UE dropping is already performed in heterogeneous deployment scenario Config 4b as well as Small cell scenarios 1, 2a, 2b

- Slide 4 in R1-136015 provides examples of alternatives for the respective issues. Companies are encouraged to further study the alternatives on slide 4. Other alternatives are not precluded.

  - Email discussion to study alternatives for each respective issue until 1/17/2014, targeting convergence of views – Hui (CMCC)

Not treated.

| R1-135174 | Proposals for 3D UMa scenario with one high rise building per sector | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |
| R1-135217 | Details of 3D UMa with one high rise per sector with 300m ISD | Samsung, CHTTL |  |
| R1-135315 | Views on 3D UMa high-rise macro cell scenario | Qualcomm Inc. |  |
| R1-135407 | Details of 3D UMa with one high rise per sector | Hitachi Ltd. |  |
| R1-135449 | Views on Fast Fading Modelling in the 3D-UMa-with-one-high-rise-per-sector-with-300m-ISD Scenario | KDDI |  |
| R1-135478 | Discussion on 3D UMa with one high rise per sector with 300m ISD | LG Electronics |  |
| R1-135527 | Discussion on channel model for high rise scenario | ZTE |  |
| R1-135541 | Observations on Large Scale Parameter Models for 3D System Simulation | Broadcom Corporation |  |
| R1-135706 | O2I channel measurement results on elevation related parameters | CMCC | (R1-134577) |
| R1-135768 | Considerations on the 3D UMa with one high rise per sector with 300m ISD Scenario | Ericsson |  |
| R1-135841 | Modelling of large scale parameters in high rise scenario | Huawei, HiSilicon | (R1-135384) |

#### Other

| R1-135218 | Discussion on elevation beamforming and FD-MIMO | Samsung, CHTTL |  |
| --- | --- | --- | --- |

Decision: The document is noted. Email discussion until 11/26th for TR36.873 – Bishwarup (NSN)

Not treated.

| R1-135550 | Shadow fading modelling for microcell scenario based on a measurement campaign | Orange | Late |
| --- | --- | --- | --- |

### Study on LTE Device to Device Proximity Services

SID in RP-122009 - FS_LTE_D2D_Prox

| R1-136032 | Summary of Ad-hoc session on Study on LTE Device to Device Proximity Services | Ad-Hoc Chairman (Alcatel Lucent) |  |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and provides the outcomes of D2D sessions.

Discussion (Question / Comment): To be updated with revision of scenarios and requirements for 36.843.

Decision: The document is endorsed and incorporated below.

#### D2D communication

Mr Chair's recommendation: Focus on broadcast D2D communication for the public safety use case, on the understanding that basic groupcast and relay functionality (for network-UE relay case) are supported by broadcast D2D communication.

If possible, consider optimisations to enhance efficiency of the relay operation.

Note that impact to existing operator services and resources is included in the evaluations.

##### Synchronization design

Procedure

| R1-135114 | Preliminary performance analysis of D2D synchronization | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135398 | Clarification on D2D synchronization procedure | Panasonic |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135402 | Public safety usage and impacts on synchronization | EADS |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135219 | Time synchronization for D2D broadcast communication in out of network coverage | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135591 | D2D synchronization and transmission timing | Nokia, NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135268 | Synchronisation of D2D UEs outside of EUTRAN network coverage | NEC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135316 | Multi-hop D2D synchronization performance | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135091 | Discussion on synchronization for D2D operation | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Signal and channel design

| R1-135903 | Synchronization Signals and Channel Design for D2D Discovery and Communication | Ericsson | (R1-135804) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135929 | Way Forward on the D2D Synchronization Procedure | Ericsson, Qualcomm, Intel, Huawei, HiSilicon, Fujitsu, LGE, GDB |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working Assumption:

- Before starting to transmit D2DSS, a UE scans for synchronization sources

  - If a synchronization source is detected, the UE may synchronize its receiver to it before it may transmit D2DSS

    - FFS whether/when a UE transmits D2DSS

    - If a UE transmits D2DSS, the rules for determining which source the UE uses as the timing reference for its transmissions of D2DSS are as follows:

      - Synchronization sources which are eNBs have a higher priority than synchronization sources which are UEs

      - FFS whether synchronization sources which are UEs within network coverage have a higher priority than synchronization sources which are UEs outside network coverage

        - Note that this would imply that there is a means to distinguish between a D2DSS transmitted by a UE in coverage and a D2DSS transmitted by a UE out of coverage

      - Other details are FFS (including timing advance details)

  - If no synchronization source is detected, a UE may nevertheless transmit D2DSS

- A UE may reselect the synchronization source it uses as the timing reference for its transmissions of D2DSS if the UE detects a change in the synchronization source(s)

  - Detailed rules FFS

Wednesday 13th session

| R1-135932 | Way Forward on D2D synchronization | LG Electronics, Qualcomm, Huawei, HiSilicon, ETRI, CATT |  |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE.

- At a given time, the reference point in time for the D2D signal transmission time is derived from at most one D2DSS (which may or may not be comprised of SFN transmissions from multiple synchronization sources).

  - The D2D signal transmission time of a UE at different time instances can be derived from different D2DSSs.

  - Methodology for D2D synchronization signal selection is FFS

    - Example: Deriving D2D signal transmission time based on a selected D2DSS out of multiple received D2DSSs

Discussion (Question / Comment): Possible clarification of the proposal: the UE shall select the D2DSS from which its signal transmission time is derived according to the resource the UE uses to transmit in.

Decision: The document is noted. Discuss further offline.

Working assumption:

- At a given time, the reference point in time for the D2D signal transmission time is derived from at most one D2DSS

  - Note that “one D2DSS” might in practice in some situations be comprised of SFN transmissions from multiple synchronization sources (although of course this is not visible to the receiver).

Proposal:

- Definition: A synchronization head is a synchronization source other than eNBs that does not derive transmit synchronization from any other D2D synchronization sources

  - Note: this WF does not necessarily imply additional functionalities for the synchronization cluster head as compared to other D2D-enabled UEs

- UEs may act as synchronization source and relay the synchronization received from D2DSS/PD2DSCH received from a D2D synchronization source

- The synchronization associated from a certain eNB or synchronization head can be relayed at most by n hops

  - FFS values for n are 2,3,4.

Wednesday 13th session

| R1-135975 | D2D AdHoc Session Outcome | Qualcomm Incorporated (Rapporteur) |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- Proposed Working Assumption:

  - UEs may transmit D2DSS/PD2DSCH derived from D2DSS/PD2DSCH received from a D2D synchronization source.

  - Definition: An independent synchronization source is a synchronization source other than an eNodeB that does not derive transmit synchronization from any other D2D synchronization sources

- Proposal:

  - UEs may transmit D2DSS/PD2DSCH derived from D2DSS/PD2DSCH received from a certain eNB or independent synchronization source by at most by n hops

    - FFS values for n are 2,3,4.

Discussion (Question / Comment): Independent synchronization source is unclear to Telecom Italia.

ALU: definition and proposal have not been agreed in the offline session.

Orange: valid for in coverage or out of coverage cases? Qualcomm: both

Decision: The document is noted. Discuss further offline.

Not treated.

| R1-135041 | System considerations for D2D synchronization | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135113 | Discussion on D2D synchronization | Intel Corporation |  |
| R1-135140 | Analysis of synchronization mechanism for D2D communication | Fujitsu |  |
| R1-135175 | Synchronization design for D2D communication | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135277 | On the D2DSS and PD2DSCH | ETRI |  |
| R1-135317 | Signal Design and Resource Allocation for D2D Synchronization | Qualcomm Inc. |  |
| R1-135368 | Considerations on synchronization of D2D broadcast communication | ZTE |  |
| R1-135479 | Discussion on design of D2DSS and PD2DSCH | LG Electronics |  |
| R1-135532 | Design considerations for D2DSS | Huawei, HiSilicon |  |
| R1-135542 | Synchronization Design for D2D communication | Broadcom Corporation |  |
| R1-135554 | Configuration and Reception of D2D Synchronization and Control Channels | BlackBerry UK Limited |  |
| R1-135803 | Synchronization Procedures for D2D Discovery and Communication | Ericsson |  |

##### D2D data communication physical channel design details

Not treated.

| R1-135092 | Discussion on physical channel types for D2D communication | CATT |  |
| --- | --- | --- | --- |
| R1-135141 | Father analysis on control Information for D2D communication | Fujitsu |  |
| R1-135176 | Design and performance evaluation for D2D broadcast | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-135220 | Physical channel design for D2D communication | Samsung |  |
| R1-135270 | Considerations of subframe design for D2D communication | NEC |  |
| R1-135278 | Physical channel design for D2D communications | ETRI |  |
| R1-135318 | D2D broadcast link level simulation results for VOIP | Qualcomm Inc. |  |
| R1-135319 | Control for D2D broadcast communication | Qualcomm Inc. |  |
| R1-135321 | Signal design for D2D broadcast communication | Qualcomm Inc. |  |
| R1-135369 | Control Channel Design for D2D link | ZTE |  |
| R1-135385 | On the need of a feedback loop for D2D communication | Huawei, HiSilicon |  |
| R1-135386 | Physical channel design for D2D communication | Huawei, HiSilicon |  |
| R1-135399 | Physical channel design in D2D communication | Panasonic |  |
| R1-135480 | D2D Communication Physical Channel Design | LG Electronics |  |
| R1-135494 | ProSe device-to-device voice communications and impact upon channel design | General Dynamics Broadband UK |  |
| R1-135495 | ProSe device-to-device communication control channel design | General Dynamics Broadband UK |  |
| R1-135606 | L1 channel design for D2D communication | InterDigital |  |
| R1-135613 | Discussion on D2D data physical channel | Intel Corporation |  |
| R1-135805 | On Scrambling of D2D Physical Channels | Ericsson |  |
| R1-135806 | Physical Channels Design for D2D | Ericsson |  |
| R1-135934 | Way Forward on D2D PUSCH Transmission | Qualcomm Inc., General Dynamics Broadband, ITRI, Samsung |  |

##### Resource allocation method

| R1-135116 | System level analysis of the D2D broadcast communication in out of network coverage scenarios | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexey Khoryaev from Intel and provides system level analysis of VoIP peer-to-peer communication in different public safety scenarios. It is noticed that TDM coordination has superior performance comparing to other studied resource selection schemes and concludes that assistance in resource selection provides improved performance.

Decision: The document is noted.

| R1-135042 | Resource assignment for D2D communication | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Philippe Sartori from Huawei and proposes:

- For in-network coverage, the resource allocation is managed by the network (eNB).

- For out-of-network communication, one UE acts as a group master for scheduling purpose. This solution enables a common design for in-network and out-of-network communication.

- Each member of a D2D group is allocated resources by the eNB. The resources are allocated in a semi-persistent manner.

- Compare orthogonal and non orthogonal resource splitting between cellular and D2D communication.

Decision: The document is noted.

| R1-135607 | Evaluation results for D2D PUSCH resource allocation methods | InterDigital | (R1-134567) |
| --- | --- | --- | --- |

The document was presented by Marian Rudolf from InterDigital and recommends to base channel selection for broadcast D2D communication on the principle that D2D UE’s select their D2D transmission resources from a pool of available D2D resources combined with a listen-before-talk principle prior to starting their transmissions in order to allow for spatial reuse.

Decision: The document is noted.

| R1-135094 | Resource allocation for D2D communication | CATT |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Peng from CATT and proposes:

- The resource allocation mechanism for D2D communication shall incorporate both contention based and scheduling based manners.

- The resource of D2D link shall include two parts which can be preconfigured or configured by eNB/CH:

  - In the contention part, D2D UEs can occupy the resource in contention based manner.

  - In contention free part, D2D UEs can occupy the resource in scheduling based manner which are controlled by eNB/CH.

- The information of contention and contention free parts shall be included in D2D transmission, which can benefit the resource and interference coordination for D2D communication.

Decision: The document is noted.

| R1-135222 | Evaluation results of resource allocation scheme for D2D communication | Samsung |  |
| --- | --- | --- | --- |

The document was presented by Hyunseok Ryu from Samsung and provides performance comparison between centralized and distributed resource allocation mechanisms for D2D broadcast communication.

- When the frequency resource granularity is smaller, distributed resource allocation with an implicit coordination can achieve better performance than a centralized resource allocation mechanism.

- Distributed resource allocation should be the baseline approach for D2D broadcast communication in Rel-12.

  - Frequency-domain resource allocation for D2D broadcast communication should consider RB- or RBG-level granularity.

Decision: The document is noted.

| R1-135320 | Distributed vs. centralized broadcast communication control | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Saurabha Talvidar from Qualcomm and concludes:

- CH based resource allocation yields lower performance than the distributed resource allocation.

- CH based resource allocation can lead to excessive resource orthogonalization.

- The modified CH resource allocation scheme yields lower performance than the distributed resource allocation scheme, given the same amount of resource usage.

- CH based scheme leads higher fraction of links being in outage for VoIP.

It is proposed that distributed resource allocation should be the baseline for D2D broadcast communication.

Decision: The document is noted.

| R1-135481 | Discussion on resource allocation in D2D communications | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE and proposes that a clustering-based resource allocation schemes that enable the following properties should be studied for D2D communications outside network coverage.

- One cluster head UE transmits D2D synchronization reference for each cluster, and each cluster is associated with a set of time resource.

- A transmitter joins one or more clusters if the signal of the synchronization signal is higher than a threshold. It participates in the contention for the cluster head election otherwise.

- Each transmitter uses the resource subset of the time resource set associated with the cluster which that transmitter belongs to.

- Resource allocation should enable interference coordination by avoiding two closely located transmitters use the same time/frequency resource.

Decision: The document is noted.

| R1-135483 | Discussion on D2D Communication Scheduling Method | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE and proposes:

- The three-step scheduling method, comprising resource request, resource grant, and communication channel transmission, is further studied for as the general procedure of D2D communication scheduling.

- Inside network coverage, eNB is the scheduling device for the three-step method. RACH procedure can be the basis of the scheduling for RRC_Idle transmitters and receivers. Scheduling for RRC_Connected UEs can be based on UL SPS.

- Outside network coverage, a UE which is elected as the synchronization reference can be the scheduling device. Further study is needed on the collision resolution capability of the resource request and the efficiency of having the scheduling device.

Decision: The document is noted.

| R1-135808 | On scheduling for D2D communication | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Ms Iana Siomina from Ericsson and suggests adopting the scheduling procedure as shown in Figure 1.

- The use of scheduling assignments carried by the physical direct data channel is advantageous from a flexibility/performance trade-off

Decision: The document is noted.

| R1-135400 | Consideration on contention based and scheduling based resource allocation schemes in D2D | Panasonic | (R1-134815) |
| --- | --- | --- | --- |

The document was presented by Lilei Wang from Panasonic and proposes a procedure (combination of contention based and scheduling based resource allocation schemes for broadcasting transmission in case of out-of-network coverage).

- Coarse resource allocation should be considered for broadcasting transmission

- Resource allocation is under master-slave structure

Decision: The document is noted.

WFs

| R1-135936 | Way Forward on resource scheduling for D2D broadcast communication | ZTE, Qualcomm Inc., Panasonic, Samsung, III |  |
| --- | --- | --- | --- |

The document was presented by Yifei Yuan from ZTE.

- When transmitting UEs are out of network coverage, the resources used for D2D broadcast traffic are selected from a resource pool by each transmitting UE

  - The resource pool can be preconfigured, or semi-statically configured

  - The details are FFS on how the resource allocation is decided

Discussion (Question / Comment): Also supported by Kyocera, Nokia, ETRI, NSN

LGE: concern on resources collision  mechanisms are needed

Out of coverage definition was debated.

Decision: The document is noted.

| R1-135809 | On Out of Coverage Definitions for D2D | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Stefano Sorrentino from Ericsson and deals with the need for a metric used for the device to determine whether the device is in network coverage or out of network coverage.

- Even D2D capable devices out of network coverage as well as devices in RRC_idle monitor the metric in order to determine whether the device is inside or outside network coverage in order to assume the correct behaviour.

- A suitable metric for out of network detection for D2D communication is based on PSS/SSS or CRS signal level or quality monitoring and a mapping function to corresponding MIB and relevant SIB decoding performance.

- FFS whether to use PSS/SSS, the filter constant, sampling interval as well as the thresholds and actual mapping function.

Decision: The document is noted.

Proposal:

The selection from the pool is done by:

- Alt 1: as a baseline, each transmitting UE (a hybrid scheme where the selection may be done by a central device is not currently precluded – FFS): Samsung, Broadcom, Harris, ALU, ASB, QC, ZTE, ETRI, Kyocera, ITRI, III, E///, CATT, Fujitsu, GDB, HTC, USDoC

- Alt 2: a central device: HW, HiSi, LGE, Orange, Panasonic, EADS

Huawei objected to making Alt 1 a working assumption due to performance issue - would like to see more result on contention cases.

| R1-135322 | D2D broadcast resource allocation and interference management algorithms | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document (section 3.2) was presented by Saurabha Talvidar from Qualcomm and lists the following observations:

- 80% of associated TX-RX UE pairs see < 2% packet loss with 4 transmissions/packet

- 86% of associated TX-RX UE pairs see < 5% packet loss with 4 transmissions/packet

- IBE models do not significantly impact VOIP performance, and this suggests that the VOIP performance is limited at a link level.

- No significant variation in performance with number of transmitters per cell, and this suggests that the VOIP performance is limited at a link level.

- The packet delay is within VOIP latency target by design,

- VOIP latency target can be met even with D2D sharing resources with WAN as long as one in 8 sub-frames is allocated for D2D.

- With time and frequency diversity techniques, VOIP performance can be improved to less than 5% of links being in outage. Time-diversity and frequency-diversity techniques should be studied further.

Discussion (Question / Comment): how collisions are modelled in these simulations? There is no contention according to Huawei.

Decision: The document is noted.

Huawei sustained their objection

USDoC – typical use case is when the number of users at an incident is within a pre-planned range.

Working Assumption:

- When transmitting UEs are out of network coverage, the resources used for D2D broadcast traffic are selected from a resource pool

  - The resource pool can be preconfigured, or semi-statically configured

  - The details are FFS on how the resource is selected from the pool

  - If the resource pool is semi-statically configured, the method of semi-statically configuring the resource pool is FFS

- Note that the criterion for “out of coverage” for the purpose of this UE behaviour would need to be defined.

Agreed Way Forward:

- Evaluate further until RAN1#76 whether the selection is done by each transmitting UE and/or by a central node, including modelling of contention and time delay between sensing and transmission.

- Email discussion until Friday 29th November to consider revisiting the RSRP threshold.

| R1-135886 | Way Forward on Resource Allocation Procedure for D2D Broadcast Communication | Panasonic, Huawei, HiSilicon, LGE, CATT |  |
| --- | --- | --- | --- |

The document was presented by Lilei Wang from Panasonic.

- Scheduling request, resource grant and D2D data broadcast communication are basic three steps for resource allocation procedure in network coverage case

  - FFS on whether and how these steps could be applied in out-of-network coverage case

Decision: The document is noted.

| R1-135931 | Way Forward on resource allocation and interference coordination for D2D communication and discovery | LG Electronics, Intel, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE.

- The following observation is captured in TR 36.843

  - Time-domain resource coordination is beneficial in mitigating the in-band emission impact in D2D communication and discovery.

    - Time-domain coordination by allocating frequency resources of a subframe to a group of closely-located transmitter UEs

- Time-domain resource coordination should be considered in studying resource allocations for D2D communication and discovery

Decision: The document is noted.

| R1-135935 | Way Forward on resource allocation for D2D broadcast communication | ZTE, Qualcomm Inc., Fujitsu, ETRI, III |  |
| --- | --- | --- | --- |

Mr Chair decided to leave it to next meeting. Just noted.

| R1-135093 | Multiplexing between cellular link and D2D link | CATT |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Peng from CATT and concludes:

- FDM shall not be used for multiplexing D2D signal and cellular signal from individual UE perspective.

  - Observation 1: In case that cellular link is operated in frame structure type 1, the collision between cellular link and D2D link from an individual UE aspect could be avoided by eNB scheduler.

  - Observation 2: In frame structure type 1, the collision between cellular link and D2D link from an individual UE could be avoided through dropping D2D transmission by UE self.

- In case that cellular link is operated in frame structure type 2, D2D UE shall transmit the ACK/NACK feedback according a reference TDD UL/DL configuration which is indicated by eNB, where the reference TDD UL/DL configuration is derived from D2D subframe configuration and TDD UL/DL configuration of cellular link.

Discussion (Question / Comment): Qualcomm (alternative proposal to second main bullet)  UE does not transmit one or other of cellular feedback and D2D signal.

Decision: The document is noted. Discuss further offline the second main bullet.

Agreement:

- FDM shall not be used for multiplexing D2D signal and cellular signal from individual UE perspective.

CATT suggested looking at:

| R1-135939 | Way Forward on multiplexing between cellular signal and D2D signal | CATT, ETRI, Fujitsu, Huawei, HiSilicon, Intel, ZTE |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Peng from CATT.

From individual UE perspective, on a given carrier:

- Multiplexing transmissions of D2D signal and cellular signal is only in TDM

- D2D signal reception and cellular UL transmission do not use full duplex

- Mechanisms for handling/avoiding collisions between cellular signal and D2D signal are FFS. Options include but not limited to:

  - by eNB scheduler and/or

  - by  dropping D2D transmission by D2D Tx UE

  - Other collision avoidance mechanisms between cellular ACK/NACK feedback from an individual UE and D2D signal

    - e.g. cellular ACK/NACK feedback from an individual UE is transmitted according to a reference TDD UL/DL configuration which is indicated by eNB for frame structure type 2

Decision: The document is noted.

Agreement:

- From individual UE perspective, on a given carrier D2D signal reception and cellular UL transmission do not use full duplex

Discuss further the other aspects of the WF.

Not treated.

| R1-135115 | Discussion on D2D resource allocation method | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-135142 | RRM for D2D communication | Fujitsu |  |
| R1-135221 | Resource allocation for D2D broadcast communication | Samsung |  |
| R1-135370 | Study on D2D Resource Allocation | ZTE |  |
| R1-135482 | Multiplexing of Uu and D2D communication | LG Electronics |  |
| R1-135543 | D2D communication channel access | Broadcom Corporation |  |
| R1-135592 | Resource allocation, multiplexing and resource access | Nokia, NSN |  |
| R1-135807 | On resource allocation for D2D broadcast communication | Ericsson | Late |
| R1-135878 | Frequency Reuse for D2D Broadcast Communication | ITRI | (R1-135418) |
| R1-135930 | Way Forward on Broadcast Communication Scheduling Procedure | Ericsson, Qualcomm |  |

##### Other

Not treated.

| R1-135043 | System-level performance of D2D communication | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

#### D2D discovery

##### Discovery signal and message design details

| R1-135223 | Physical channel design for D2D discovery | Samsung |  |
| --- | --- | --- | --- |

The document was presented by … from Samsung and is summarized as:

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: Sequence is DM RS of the message. | - Simple structure - Low specification effort | - High computational burden to receiving UE - Not favourable in case of variable discovery information size |
| Option 2 Sequence is a signal other than DM RS of the message | - Simple detection scheme to receiving UE - Supporting flexibility of the size of the discovery message | - More resources required - Specification effort required - Degradation in sequence detection performance if synchronization among UEs is not guaranteed |

Decision: The document is noted.

| R1-135143 | On D2D discovery signal design options | Fujitsu |  |
| --- | --- | --- | --- |

The document was presented by Chenxi Zhu from Fujitsu and is summarized as:

- To confirm the working assumption that discovery uses sequence plus message.

- Two-step D2D discovery should be supported.

- Synchronous discovery and asynchronous discovery should be further investigated.

Decision: The document is noted.

Friday morning session

| R1-135504 | D2D discovery signal and message content | HTC |  |
| --- | --- | --- | --- |

The document was presented by … from HTC and proposes:

- Discovery signal consists of message and demodulation RS sequence for Type 1 discovery in case D2D UEs are within network coverage.

- Discovery signal consists of message and separate sequence for non-synchronized D2D discovery is FFS.

- Fixed T2=0 is used for UEs in idle mode or out of coverage.

- Fixed T2 is used for UEs with an active timing advanced value.

Decision: The document is noted.

| R1-135323 | D2D discovery signal design | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and proposes the followings:

- Use the same physical layer design for both Open ProSe and Restricted ProSe discovery. Discovery information for Restricted ProSe should be encoded appropriately.

- Message part of a discovery signal should be used for transmitting discovery information. Sequence part should be used for channel estimation. Sequence part of a discovery signal should be DMRS.

- Message and sequence transmissions for discovery should be co-located in frequency and time.

- Discovery sub-frame should be similar to an uplink WAN sub-frame used for PUSCH transmission. However the last symbol of the sub-frame could be used as a gap.

- Use normal cyclic prefix for discovery transmissions. For large discovery range extended cyclic can be used.

- DMRS signals transmitted as part of discovery signal should not depend on cell identity.

- QPSK modulation should be used for discovery.

- Turbo coding should be used for discovery.

- Same channel interleaver as PUSCH should be used for discovery.

- No inter-slot hopping should occur for discovery transmissions. The resource element mapping should be identical to PUSCH resource mapping with no inter-slot hopping.

- Discovery scrambling mechanism should be identical to the PUSCH scrambling mechanism with the exception that the scrambling sequence should not be dependent on UE or cell identity.

- A CRC length of 24 should be used for discovery message transmission.

Decision: The document is noted.

WFs

| R1-135977 | Way Forward on Restricted and Open Discovery | Qualcomm, Ericsson, ZTE, Samsung, Intel, General Dynamics Broadband, Fujistu |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- At RAN2#83bis it was agreed that "Open and restricted Prose Discovery should have similar RAN2 mechanism to avoid complexity."

- Proposal: Open and Restricted discovery should have the same physical layer design.

Discussion (Question / Comment): Huawei: wait RAN2 to respond to the LS - not clear what kind of mechanism it refers to - a bit premature to state "same physical layer design".

If same message size from RAN2 for open and restricted discovery, Huawei would like to see the same physical layer design.

Decision: The document is noted.

Possible working assumption

- Open and Restricted discovery should have the same physical layer design if opened and restricted message are of the same size.

Debate whether to state "same" or "similar"

Working assumption

- Open and restricted messages are undistinguishable in the physical layer if open and restricted messages have a same size

| R1-135978 | Way Forward on Message and Sequence for Discovery | Qualcomm, Ericsson, ZTE, Intel, Kyocera, General Dynamics Broadband |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- If a sequence transmitted by a UE is not on the same TTI as its discovery message transmission, the sequence transmitted should not depend on UE specific information and the discovery information transmitted

- If sequence transmitted by a UE for discovery is on the same TTI as its discovery message transmission, the sequence should be DMRS

Discussion (Question / Comment): LGE: DMRS sequence is not sufficient as important timing offset are expected in D2D.

Ericsson asked removing their name as co-source - they co-sourced 5977 but not that one.

Decision: The document is noted. Continue study until RAN1 #76 meeting.

Not treated.

| R1-135044 | D2D discovery message design | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135088 | Details of discovery sequence and message design | CATT |  |
| R1-135117 | On message-based D2D discovery signal design | Intel Corporation |  |
| R1-135177 | Discovery signal design details | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-135324 | Reference signal design for discovery | Qualcomm Inc. |  |
| R1-135371 | Performance Aspects of Discovery Signals | ZTE |  |
| R1-135416 | AGC Impact to Discovery Signal Design | ITRI |  |
| R1-135484 | D2D Discovery Signal Format | LG Electronics |  |
| R1-135496 | ProSe device-to-device discovery sequence design | General Dynamics Broadband UK |  |
| R1-135497 | ProSe device-to-device discovery link performance | General Dynamics Broadband UK |  |
| R1-135521 | Evaluation on PRB size and period of sending discovery signals | NTT DOCOMO |  |
| R1-135529 | Proposal to support longer D2D discovery message sizes | Motorola Solutions |  |
| R1-135593 | D2D discovery signal design | Nokia, NSN |  |
| R1-135671 | Channel coding considerations for D2D discovery signal | Kyocera |  |
| R1-135764 | Considerations for asynchronous discovery in D2D | Sony |  |
| R1-135810 | Discovery Channel Design for D2D | Ericsson |  |
| R1-135827 | D2D discovery resource size and mapping to physical resources | Intel Corporation | (R1-135118) |

##### Resource allocation method

Not treated.

| R1-135045 | Benefits of type-2 discovery | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135046 | Resource allocation for type-1 discovery | Huawei, HiSilicon |  |
| R1-135089 | Further discussion on resource allocation for D2D discovery | CATT |  |
| R1-135119 | On resource allocation for D2D discovery | Intel Corporation |  |
| R1-135224 | Resource allocation method for D2D discovery | Samsung |  |
| R1-135269 | D2D Discovery Resource Allocation under Network Coverage | NEC |  |
| R1-135279 | Discussion on resource allocation for D2D discovery | ETRI |  |
| R1-135325 | Resource allocation for D2D discovery | Qualcomm Inc. |  |
| R1-135372 | Resource Allocation for D2D Discovery Type 1 | ZTE |  |
| R1-135373 | Discussions on necessity of supporting Discovery Type 2 | ZTE |  |
| R1-135451 | Views on Type2 discovery resource allocation | KDDI |  |
| R1-135485 | Resource Allocation and UE Behaviour for D2D Discovery | LG Electronics |  |
| R1-135486 | Discussion on multiplexing of Uu and D2D discovery | LG Electronics |  |
| R1-135498 | ProSe device-to-device discovery system simulations | General Dynamics Broadband UK |  |
| R1-135505 | Resource allocation for D2D discovery | HTC |  |
| R1-135522 | Views on D2D discovery resource allocation | NTT DOCOMO |  |
| R1-135530 | Near-Far Mitigation for D2D Discovery | Huawei, HiSilicon |  |
| R1-135594 | Resource allocation and inter-cell discovery | Nokia, NSN |  |
| R1-135617 | Method of resource allocation for D2D discovery | ASUSTeK |  |
| R1-135828 | D2D discovery resource selection for Type 1 discovery | Intel Corporation | (R1-135120) |
| R1-135877 | RF Blocking effect evaluation under FDM-based radio resource allocation | ITRI | (R1-135417) |
| R1-135938 | Way Forward on resource allocation for type 1 D2D discovery | CATT, ETRI, Huawei, HiSilicon, Intel, ITRI, LGE, NTT DoCoMo |  |
| R1-135982 | Way Forward on Resource Allocation for Discovery | Huawei, HiSilicon, LGE, KDDI, NTT DoCoMo, ETRI, Intel, Panasonic |  |

##### Other

Not treated.

| R1-135225 | In-band emission impact of D2D discovery | Samsung |  |
| --- | --- | --- | --- |

#### D2D transmission timing

| R1-135178 | D2D transmission Timing | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and deals with the timing control methods for discovery UEs. The following observations and proposals are made.

- With T2=TA for active-TA UEs and T2=0 for UEs without active TA, the potential timing offset range of the received discovery signals at the discovery receiver is larger than (about double) the timing offset range with uniform timing control.

- With uniform timing control (i.e., T2=0 for all discovery UEs), all the received intra-cell discovery signals at the discovery receiver have positive time delay compared with the DL receiving timing reference.

- Proposal-1: All discovery UEs, whether they have active timing advance values or not, use T2=0.

- Proposal-2: A guard time is used at the end of the discovery subframe to avoid inter-subframe interference and at the same time enable TX/RX switching.

Decision: The document is noted.

| R1-135326 | D2D transmission timing | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and deals with different options for timing of discovery signal of RRC_CONNECTED UEs.

- Observation 1:  Not using downlink timing for transmission and reception of discovery by RRC_CONNECTED UEs leads to larger spread in the timing of discovery signal at a receiver.

- Observation 2:  The additional delay spread can impact reception of discovery signals from UEs that are far away.

- Observation 3: The additional increase in timing difference due to not using downlink timing for transmission and reception of discovery by RRC_CONNECTED UEs can be 1-2us.

Based on these observations it is proposed that RRC_CONNECTED UEs should use downlink timing for transmission of discovery signals.

Decision: The document is noted.

| R1-135226 | D2D discovery timing impact on PUCCH | Samsung |  |
| --- | --- | --- | --- |

The document was presented by Hyunseok Ryu from Samsung and discusses transmit timing for D2D discovery and analyses synchronization performance in terms of CCDF of the received SINR at D2D receiver and eNB receiver.

- If different transmit reference timing is used for different D2D transmitters, D2D receivers suffer from intra-cell interference due to ISI and ICI.

- If normal CP is assumed, D2D PUSCH transmission based on DL reference timing significantly affect to PUCCH performance.

- If extended CP is assumed, timing misalignment problem can be avoided at the expense of a higher overhead to legacy UE’ PUCCH from the CP.

It is proposed that D2D transmitters in RRC_Connected state should transmit discovery information based on DL reference timing. It should be further studied how to guarantee PUCCH performance while maintaining normal CP in discovery subframes.

Decision: The document is noted.

| R1-135955 | On D2D discovery transmission timing | Intel Corporation | (R1-135121) |
| --- | --- | --- | --- |

The document was presented by Debdeep Chatterjee from Intel and proposes:

- For FDD system deployments:

- Confirm the current working assumption that UEs without an active timing advance value transmit discovery signals using T2 = 0.

- UEs with an active timing advance value (TA) transmit discovery signals using T2 = 0 for transmissions on Type 1 Discovery resources and using T2 = TA/2 for transmissions on Type 2 Discovery resources.

- For TDD system deployments:

- Revise the current working assumption that UEs without an active timing advance value transmit discovery signals using T2 = NTAoffset (where NTAoffset =624Ts).

- UEs with an active timing advance value (TA) transmit discovery signals using T2 = NTAoffset (where NTAoffset =624Ts) for transmissions on Type 1 Discovery resources and using T2 = TA/2 + NTAoffset for transmissions on Type 2 Discovery resources.

- For synchronous LTE deployments, ProSe-enabled UEs use DL timing of the serving cell as their synchronization reference for transmission and reception, i.e. T1 = DL serving cell reference time.

- For asynchronous LTE deployments, ProSe-enabled UEs follow DL timing of the serving cell for intra-cell discovery.

- For inter-cell discovery, ProSe-enabled UEs derive synchronization either:

- by directly acquiring the PSS/SSS transmissions from the neighbouring cell,

- by using PRS subframes of neighbouring cells when available,

- by receiving UE-relayed synchronization signals carried as part of discovery announcement signals by certain UEs in the neighbouring cell.

- For asynchronous LTE deployments, the configuration of D2D Discovery Zones of neighbouring cells can either be:

- signalled by the serving cell;

- directly acquired by the ProSe-enabled UEs using assistance information from the serving cell about the relevant SIB transmitted by the neighbouring cell;

- carried as part of discovery announcement signal transmitted by certain UEs in the neighbouring cell.

- For asynchronous LTE deployments, while UEs can derive synchronization based on timing of neighbouring cells for reception of inter-cell discovery signals, they may only transmit discovery signals within the D2D DZs configured by the respective serving cell and using the serving cell DL reference time, i.e., T1 = DL serving cell reference time.

Decision: The document is noted.

| R1-135047 | Considerations on timing for D2D | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Philippe Sartori from Huawei and deals with the timing, the CP length design and the subframe design for D2D communication/discovery. It is proposed that:

- the ‘active-like’ UE’s Tx timing is adjusted by the timing advance to the timing reference from eNodeB or group master.

- for D2D communication and discovery, the ‘idle-like’ UE’s Rx timing is adjusted by cyclic prefix length to the downlink timing reference; the ‘active-like’ UE’s Rx timing is adjusted by the uplink timing advance to the downlink timing reference..

- the length of cyclic prefix for D2D discovery and communication should be larger than 6.22µs (2.22µs + 1µs+3µs).

Decision: The document is noted.

| R1-135937 | Way Forward on D2D discovery transmit timing | Samsung, Qualcomm, ETRI |  |
| --- | --- | --- | --- |

The document was presented by Hyunseok Ryu from Samsung.

- For type 1 discovery

  - In FDD: RRC_Connected D2D UEs inside network coverage transmit discovery signal based on DL reference timing (T2 = 0).

  - In TDD: T2 is FFS

Discussion (Question / Comment): Mr Chair suggested whether the above could be taken as Working Assumption.

CATT: same principle be applied to both FDD and TDD

Huawei: T2=0 on FDD creates interference to the eNB - not acceptable

Telecom Italia: same view as Huawei

Decision: The document is noted and modified as below proposed WA:

- For type 1 discovery

  - In FDD: RRC_Connected D2D UEs inside network coverage transmit discovery signal based on DL reference timing (T2 = 0).

  - In TDD: T2 is either 0 or 624Ts

In favour of the proposed WA

- Qualcomm, Samsung, LGE, Intel, USoC, CATT, GDBUK, ETRI, Alcatel Lucent, ASB, ZTE (assuming there is no multiplexing between PUSCH and discovery messages within a subframe)

Opposed to

- Huawei, HiSilicon, Orange (impact on cellular systems unclear), Telecom Italia

Agreed way forward:

- Impact on cellular system of RRC_Connected D2D UEs transmitting discovery signal based on DL reference timing is to be evaluated

- If the impact on cellular system is found to be insignificant, then the following working assumption will be made:

  - For type 1 discovery

    - In FDD

      - RRC_Connected D2D UEs transmit discovery signal based on DL reference timing (T2 = 0).

    - In TDD

      - RRC_Connected D2D UEs transmit discovery signal based on one fixed value of T2

        - If the value is not 0, the working assumption for RRC_idle TDD UEs will be revisited for consistency

Not treated.

| R1-135090 | The value of T2 for D2D transmission timing | CATT |  |
| --- | --- | --- | --- |
| R1-135144 | Potential issues of D2D discovery signal transmission timing | Fujitsu |  |
| R1-135487 | D2D Discovery Signal Transmission Timing | LG Electronics |  |
| R1-135488 | Discussion on Synchronization for D2D Communications | LG Electronics |  |
| R1-135544 | In-Coverage ProSe Synchronization and Timing | Broadcom Corporation |  |
| R1-135614 | Discussion on D2D transmission timing | Intel Corporation |  |
| R1-135817 | D2D transmission timing considerations | HTC | (R1-135670) |

#### Other

| R1-135836 | Report on email discussion on "R1-134931 Revision of TP to 36.843 after sessions" | Telecom Italia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

TP for 36.843 (R1-135908) – revisit on Wednesday

Wednesday 12th

| R1-135908 | Revision of Scenarios and requirements TP to 36.843 after email discussion | Telecom Italia, Orange, KDDI, Huawei, HiSilicon, ALU, ZTE, LGE, Nokia, ITRI |  |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia.

Discussion (Question / Comment): Qualcomm objected as part of TP is coming from relevant SA1 requirements (normative TS) - how can TR change requirements from a TS?

Decision: The document is noted. It is proposed to consider the following approach:

Requirements relating to:

- accommodation of potentially large numbers of concurrently participating ProSe-enabled UEs,

- impact on E-UTRA radio usage, network usage and battery consumption, …

according to [REF TS 22.278] shall be accommodated.

Friday 15th

| R1-135981 | Revision of Scenarios and requirements TP to 36.843 after email discussion | Telecom Italia | (R1-135908) |
| --- | --- | --- | --- |

Decision: The document is agreed. Prepare LS to SA1 on D2D proximity criteria for email approval until 13th December – (Qualcomm, Telecom Italia)

| R1-135327 | Text Proposal for TR 36.843 | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted and agreed for inclusion in next update of TR.

| R1-135897 | Draft TR 36.843 v0.3.0 on “Study of LTE Device to Device Proximity Services-Radio Aspects” | Qualcomm Inc. | (R1-135328) |
| --- | --- | --- | --- |

Decision: The document is noted.

Approved in R1-135909 as v0.3.0 with the inclusion of TP from 5327 and the following change: According to the RAN plenary prioritization, RAN1 and RAN2 will focus on a D2D ProSe discovery mechanism for in-coverage (scenarios 1C and 1D).

Update of TR as v0.3.1 to be prepared including all agreements reached during RAN1#75 in R1-135910.

Email approval until Friday 22nd November for v.1.0.0 for submission to RAN#62 for information.

Not treated.

| R1-135415 | Spectrum utilization model for D2D communications and discovery | ITRI |  |
| --- | --- | --- | --- |
| R1-135489 | Issues on D2D Signal Transmissions and Receptions across Carriers | LG Electronics |  |
| R1-135885 | ProSe WID status | Qualcomm Incorporated (Rapporteur) |  |

### Study on CoMP for LTE with Non-Ideal Backhaul

SID in RP-130847 - FS_COMP_LTE_nibackhaul

| R1-136033 | Summary of Ad-hoc session on Study on CoMP for LTE with Non-Ideal Backhaul | Ad-Hoc Chairman (Alcatel Lucent) |  |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and provides the outcomes on CoMP NIB sessions.

Discussion (Question / Comment): Conclusion on 5924 to be included.

Decision: The document is endorsed and incorporated below.

| R1-135907 | Draft TR 36.874 v1.1.0 on CoMP NIB | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Add HW’s results for scenario 2a

In section 5.2, remove the descriptions and names of schemes, and put e.g. “Source X [ref]” with a ref to relevant tdoc.

For any missing schemes, provide the appropriate row for the Table to Hyojin.

Samsung to provide an update in R1-135913 – revisit on Thursday.

Agreement: Keep all the tables in the TR but prioritise the results for the mandatory evaluation cases with larger numbers of sources when drawing conclusions

Thursday 14th

| R1-135913 | TR 36.874 V1.1.0 Coordinated multi-point operation for LTE with non-ideal backhaul | Samsung | (R1-135907) |
| --- | --- | --- | --- |

Decision: The document is noted and agreed as version 1.1.0 in R1-135920.

#### Further evaluation of coordinated scheduling and coordinated beamforming

Downlink

| R1-135887 | Further Evaluation Results for CoMP-NIB | NTT DOCOMO | (R1-135523) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135595 | Simulation Results for CoMP scenario.2 with non-ideal backhaul | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135596 | Simulation Results for small cell Scenarios with non-ideal backhaul | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135825 | Evaluation of coordinated scheduling for SCE scenario 1 with non-ideal backhaul | Samsung | (R1-135228) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135826 | Evaluation of coordinated scheduling for SCE scenario 2a with non-ideal backhaul | Samsung | (R1-135229) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135928 | Evaluation of coordinated scheduling for CoMP scenario 2 with non-ideal backhaul | Samsung | (R1-135230) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135241 | Collaborative distributed scheduling - updated information | IAESI, DAC-UPC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135240 | Assessment of differences between centralized and knowledge-based distributed scheduling | IAESI, DAC-UPC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135890 | Performance Evaluation of X2 signalling supporting DL eCoMP with NIB | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-135179) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135376 | Further evaluation of CS with non-ideal backhaul | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135769 | System Level Evaluations of Coordinated Scheduling Using Slow Backhaul | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135940 | Further performance evaluation of CoMP with non-ideal backhaul | Huawei, HiSilicon | (R1-135048) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135953 | Way Forward on CoMP-NIB performance | LG Electronics, China Unicom, CMCC, Deutsche Telekom, ETRI, Fujitsu, KDDI, KT Corp., LG Uplus, NTT DOCOMO, Samsung, SK Telecom, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted. Also supported by Telecom Italia

Proposal:

- Following observation for SCE scenarios 1 and 2a with NIB is captured in TR36.874

  - CoMP with NIB can offer performance benefits over FeICIC for SCE scenario 1 and intra-site CoMP for SCE scenario 2a with low backhaul delay and at least high load level.

| R1-135950 | Way Forward on CoMP-NIB | Huawei, HiSilicon, NSN, Nokia, Ericsson, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- Capture the followings into CoMP-NIB TR 36.874

  - Most companies show only small CoMP-NIB gains in the most favourable scenarios (with 5 ms backhaul latency) and all but one company show losses in less favourable scenarios (with 50 ms backhaul latency).

  - CoMP gain is not enough to justify any inter-eNB signalling enhancement requiring large standardization effort.

    - Enhancements within existing eNB to eNB signalling framework for ICIC may be considered.

Discuss offline until Thursday – aim for a consensus proposal for the conclusions for the TR.

Thursday 14th

| R1-136007 | Way Forward on CoMP NIB performance | Samsung, China Unicom, CMCC, Deutsche Telekom, SK Telecom, Telecom Italia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- Following observations are captured in TR36.874

  - It was observed that CoMP-NIB gain varies as a factor of deployment scenario, backhaul delay, coordination scheme, resource utilization factor, and coordination size

    - For CoMP scenario 2 with coordination size of 9, CoMP with NIB can provide performance gain (with median… %  for mean UPT and 6.8% for 5%-tile UPT) over intra-site CoMP in case of low backhaul delay and at least high load level

    - For SCE scenarios 1 and 2a, CoMP with NIB can offer performance gains (up to 11% for mean UPT and 27% for 5%-tile UPT) over FeICIC for SCE scenario 1 and intra-site CoMP for SCE scenario 2a in case of low backhaul delay and at least high load level

In addition one company submitted results for

CoMP scenario 2 with coordination size of 21, showing performance gain (up to 13.1% for mean UPT and 24.7% for 5%-tile UPT) over intra-site CoMP in case of low backhaul delay and at least high load level

| R1-136010 | Way Forward on eCoMP | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- Following observation should be captured in TR36.874

  - CoMP with NIB can offer some performance benefits over FeICIC for SCE scenario 1 and intra-site CoMP for SCE scenario 2a with 5ms backhaul delay at high load.

    - The cell edge gain number is around 10%

  - In CoMP scenario.2, inter-site CoMP doesn’t provide significant gain over intra-site CoMP

  - Signalling enhancements within existing eNB to eNB signalling framework for ICIC can be considered.

    - Note: The observed gain number is based on the median of reported results

Way forward: prepare a TP stating median value and showing a scatter plot for each of the 3 scenarios, for both 5% UPT and UPT average.

Try to include conclusion statements as well – revisit at 11am - R1-135921 – Hyojin.

| R1-135921 | TP on CoMP-NIB Performance | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

The following version is agreed in principle for inclusion in the TR – final version of the TP to be prepared by Hyojin in R1-135922.

To be updated as follows:

- add the scatter plots from 5921

- Delete section 6.4 and move the text to section 8.

- Modify as below for SCE scenario 1 with 10 cells

- Add “in case of 5ms backhaul delay” to all cases

- Revision in R1-135923 – approved in principle, subject to addition of similar set of bullets and scatter plots for 50ms backhaul latency.

Final version to be provided for approval in R1-135924 – revisit on Friday.

Following observations are captured in conclusions of TR36.874

- It was observed that CoMP-NIB gain varies as a factor of deployment scenario, backhaul delay, coordination scheme, resource utilization factor, and coordination size

  - For CoMP scenario 2 with coordination size of 9 in case of 5ms backhaul delay, it is observed that

    - Mean UPT gain has a median of -4%

    - 5% UPT gain has a median of -2.3%

  - For SCE scenario 1 in case of 4 small cells within one macro area, it is observed that

    - Mean UPT gain has a median of 9.4%

    - 5% UPT gain has a median of 11.1%

  - For SCE scenario 2a in case of 4 small cells within one macro area, it is observed that

    - Mean UPT gain has a median of 13.5%

    - 5% UPT gain has a median of 15.7%

  - For SCE scenario 2a in case of 10 small cells within one macro area, it is observed that

    - Mean UPT gain has a median of 26%

    - 5% UPT gain has a median of 20.8%

In addition:

- For CoMP scenario 2 with coordination size of 21 one company submitted results and observed in case of 5ms backhaul delay (with negative impact on data packet waiting time prior to transmission):

- mean UPT gain of 13.1%

- 5% UPT gain of 24.7%

- For SCE scenario 1 in case of 10 small cells within one macro area, one company submitted results and observed in case of 5ms backhaul delay:

- Mean UPT gain of 1.4%

- 5% UPT gain of 7.4%

Include scatter plots in the conclusion section of TR 36.874.

For the medians of even numbers of results, use the mean of the central two values.

Also add a similar set of bullets and scatter plots for 50ms backhaul latency.

Add:

- In summary in CoMP scenario 2 for 9 cell coordination, inter-site CoMP does not provide significant gain over intra-site CoMP with 5ms backhaul latency, with median of -4% and range of -7 to +7%.

Aim is to approve a version of the TR at this meeting with conclusions based on what is available.

Friday 15th

| R1-135924 | Draft TR 36.874 V1.2.0 Coordinated multi-point operation for LTE with non-ideal backhaul | Samsung | (R1-135923) |
| --- | --- | --- | --- |

Discussion (Question / Comment): Ericsson noted that some companies have provided multiple results, resulting in unfair weight when drawing the conclusion

Samsung: multiple results are due to different CoMP schemes being evaluated

Alcatel Lucent: if multiple evaluations have been performed, fair to choose the best scheme

Samsung; why such complain comes up at the last moment

Alcatel Lucent: for scatter plots part, all results should be captured

Mr Chair: all performance results should be into TR - for the conclusion part, derived from the best results

Decision: The document is noted and agreed in principle by deleting all median values in Section 8, update in R1-136077

Agreed in R1-136078 as TR36.874 v.1.2.0 by accepting all changing tracks

Email approval of the TR until Friday 22nd November to add additional results (which must be provided by Wednesday 20th November) and corresponding update of the TR conclusions. Each company should provide the result based on the best scheme for the calculation of median values.

Reference for each point will be added in the scatter plots.

Also discuss whether to add a note about the load level.

Not treated.

| R1-135227 | Draft TP for candidate CoMP techniques in CoMP-NIB TR 36.874 | Samsung |  |
| --- | --- | --- | --- |
| R1-135239 | Coordinated Sounding for CoMP BF (CoMP BF-CoS) including calibration information | DAC-UPC, IAESI |  |
| R1-135329 | Coordinated scheduling under Non Ideal backhaul | Qualcomm Inc. |  |
| R1-135490 | Further evaluation results for DL CoMP with non-ideal backhaul | LG Electronics |  |
| R1-135800 | Evaluation of uplink CoMP with non-ideal backhaul link | Intel Corporation |  |
| R1-135831 | Response document to R1-135227 on TP for candidate CoMP Techniques in CoMP-NIB TR 36.874 | IAESI, DAC-UPC |  |
| R1-135891 | Performance Evaluation of X2 signalling supporting UL eCoMP with NIB | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-135180) |
| R1-136023 | Simulation Results for CoMP scenario.2 with non-ideal backhaul | NSN, Nokia | (R1-135595) |

#### Signalling for inter-eNB operation

| R1-135251 | Discussion on signalling for CoMP with NIB | Texas Instruments |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135545 | Signalling aspects for CoMP with non-ideal backhaul | Broadcom Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135181 | Proposals for X2 signalling supporting eCoMP with NIB | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135049 | Inter-eNB signalling for supporting CoMP with non-ideal backhaul | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135231 | Discussion on signalling for inter-eNB operation | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135770 | Signalling for Inter-eNB Operation of CoMP | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-135492 | Discussion on inter-eNB signalling for UL CoMP with non-ideal backhaul | LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-136008 | Way Forward on CoMP NIB signalling | Samsung, China Unicom, DAC-UPT, Deutsche Telekom, IAESI, SK Telecom, Telecom Italia |  |
| --- | --- | --- | --- |

For email discussion/approval until Thursday 21st November.

Not treated.

| R1-135096 | Considerations on coordination signalling for inter-eNB CoMP | CATT |  |
| --- | --- | --- | --- |
| R1-135122 | Discussion on backhaul link signalling for downlink CoMP | Intel Corporation | (R1-134144) |
| R1-135145 | Discussion on inter-eNB signalling in support of CoMP with non-ideal backhaul | Fujitsu |  |
| R1-135242 | X2 support for centralized and distributed solutions | IAESI |  |
| R1-135377 | Signalling for inter-eNB CoMP with NIB | ZTE |  |
| R1-135404 | CoMP Schemes and Backhaul Signalling for NIB | Hitachi Ltd. |  |
| R1-135412 | View on CSI-RS Resource Coordination for CoMP | ITRI |  |
| R1-135491 | Considerations on inter-eNB signalling for DL CoMP with non-ideal backhaul | LG Electronics |  |
| R1-135524 | Details on Backhaul Signalling for CoMP-NIB | NTT DOCOMO |  |
| R1-135552 | Signalling between inter-eNB to support CoMP with NIB | BlackBerry UK Limited |  |
| R1-135597 | Signalling design considerations to support inter-eNB CoMP | NSN, Nokia |  |

#### Other

Not treated.

| R1-135271 | Scheduling and Signalling Issues in CoMP-NIB | NEC |  |
| --- | --- | --- | --- |

### Study on Network-Assisted Interference Cancellation and Suppression for LTE

SID in RP-130404 - FS_LTE_NAICS

#### Remaining details of system level modelling methodology

| R1-135123 | Discussion on evaluation methodology for SLIC receiver | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and discusses system-level evaluation methodology for SLIC receiver proposed in [R1-134634-Qualcomm]. The accuracy of the model may not be sufficient to reflect instantaneous interference suppression capability of SLIC receiver required by the system-level simulations. It was observed that average interference suppression parameter is also sensitive to the channel model (power delay profile and Tx/Rx antenna correlation). In case of the interfering parameter estimation error of blind SLIC receiver the average interference suppression exceeds 1, i.e. may amplify the interfering signal, which is different from the assumption proposed in [R1-134634]. The modulation detection error probability depends from modulation scheme used on interfering layer, which is also different from the assumption made in [R1-134634].

Discussion (Question / Comment): Samsung  Receiver algorithms may impact the results of evaluation - Intel suggested looking in 5125 - Qualcomm refrained from discussing the details of receiver algorithm

Decision: The document is noted.

| R1-135534 | System level modelling methodology for SLIC receiver | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ningbo Zhang from Huawei and provides a SNR-to-variance mapping table that could be taken into consideration when modelling SLIC receiver in the system level simulation.

Discussion (Question / Comment): Channel estimation error was questioned.

Decision: The document is noted.

| R1-135565 | Discussion on system level modelling methodology | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Mihai Enescu from NSN and proposes:

- The common functional blocks in the different candidate receivers should be modelled in the same manner across different L2S models. These include serving and interfering cell channel estimation, covariance estimation, and modelling of the turbo codec performance.

- Strive for a commonly agreed L2S methodology, which is benchmarked and validated across the interested companies, and in the end captured in the NAICS SI TR.

- The L2S should be validated by measuring the accuracy of BLER prediction per instantaneous channel realizations, and not only by comparing the predicted and observed average throughput.

Decision: The document is noted.

| R1-135904 | On System Level Evaluations for NAICS | Ericsson | (R1-135771) |
| --- | --- | --- | --- |

The document was presented by Lars Lindbom from Ericsson and provides a link-level abstraction model applicable to SLIC and E-LMMSE-IRC receivers, including model validation results for SLIC. The following observations were made:

- The link-to-system model for E-LMMSE-IRC follows directly by omitting the SLIC cancellation loop in Figure 1(CE and CEhat equals zero)

- For SLIC we consider a similar approach as proposed in [R1-134634] where residuals after cancellation of interferers were modelled as a scaled version of the signals being cancelled. A major difference though is that the link abstraction model in this contribution is based on instantaneous channel realizations and not averaged channels

- The link-level abstraction model and the link-level results match well for the SLIC receiver

Decision: The document is noted.

Working assumption:

- Link-abstraction method should apply to instantaneous channel - prepare TP under this condition

  - If there are critical issues by exploiting above methodology, updated methodology can be considered

- Prepare TP for link abstraction method with all receiver types until Wednesday – (Mediatek)

Wednesday 13th

| R1-135970 | [Draft] TP for link to system modelling for TR36.866 | MediaTek |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion (Question / Comment): Ericsson suggested to capture each company's proposals in a different TP structure - may be fixed off line.

Decision: The document is noted. Prepare a TP in R1-135972 for link abstraction method with all receiver types until Friday – (Mediatek)

Companies are invited to review it and provide feedback to MediaTek.

| R1-135973 | TP for System Level Modelling Methodology for NAIC | Qualcomm Incorporated | (R1-135331) |
| --- | --- | --- | --- |

The document was presented by Hao Xu from Qualcomm.

Discussion (Question / Comment):

Alternative 1: no need to capture performance results into TR in RAN1#75

- Ericsson (some results are based on assumptions that have not been agreed), LGE

Alternative 2: capture performance results

- Orange, ZTE, MediaTek, Intel, Samsung, Qualcomm (further updates in results are expected - to be noted in TR)

Decision: The document is noted.

Friday 15th

| R1-135972 | [Draft] TP for link to system modelling for TR36.866 | MediaTek | (R1-135970) |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion (Question / Comment): Huawei suggested editorial formatting to show Alternatives 1,2… as sub-sections

Ericsson: references [7] [8] may not be needed to be listed - will be referred in Tdoc

Decision: The document is noted. Updated and finally agreed in R1-136071.

Not treated.

| R1-135232 | Prediction accuracy of link abstraction method for SLML receiver | Samsung |  |
| --- | --- | --- | --- |
| R1-135233 | Draft TP on link abstraction method of SLML receiver for TR 36.866 | Samsung |  |
| R1-135330 | System level modelling methodology for NAIC | Qualcomm Inc. |  |
| R1-135331 | TP for System level modelling methodology for NAIC | Qualcomm Inc. |  |
| R1-135546 | TP for Link to System Modelling of WLMMSE-IRC | Broadcom Corporation |  |
| R1-135548 | Text Proposal for turbo-CWIC receivers System Level Modelling Methodology | Orange |  |
| R1-135566 | L2S mapping methodology for E-LMMSE-IRC receiver | Nokia, NSN |  |
| R1-135882 | Details for system level modelling of ML/R-ML receivers and text proposal | MediaTek Inc. | (R1-135435) |
| R1-135901 | Link abstraction method for iterative ML receiver | Samsung | (R1-135234) |
| R1-135905 | TP for link to system modelling for E-LMMSE-IRC receiver | Nokia, NSN |  |

#### System level evaluation results

| R1-135235 | System level evaluation of SLML receiver for NAICS scenario 1 | Samsung |  |
| --- | --- | --- | --- |

The document was presented by … from Samsung and provides system level simulation results for SL R-ML receiver under CoMP deployment for NAICS scenario 1.

- SL R-ML receiver provides the promising performance gain under CoMP deployment scenario, similar as under Rel-10 deployment scenario. The performance gain is up to +26.6% compared with MMSE-IRC receiver without any specific optimization on scheduling coordination under NAICS scenario 1. Further performance gain can be expected once all the link abstraction parameters are finalized.

Decision: The document is noted.

| R1-135881 | Symbol level SIC performance evaluation | ZTE | (R1-135379) |
| --- | --- | --- | --- |

The document was presented by … from ZTE and provides system level simulation results of symbol level SIC.

- SLIC can provide significant gains for the cell-edge and average user throughput with network assistance/coordination.

Decision: The document is noted.

| R1-135883 | System level results for R-ML receivers for scenario #1 | MediaTek Inc. | (R1-135436) |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek and provides system-level simulation results for system with R-ML receivers. It is observed 9~27% gain on 5%-tile UEs and 13%~19% gain on cell average for RU within the range between 40% and 80%.

Decision: The document is noted.

Companies are invited to check the excel sheet provided by MediaTek until Wednesday

Tuesday's session conclusion:

- Continue offline discussion until Wednesday aiming at preparing observations and evaluation summary for TR – (Mediatek)

- Comments:

  - Ericsson: so far, a clear methodology has not been achieved yet. It is too early to capture observations.

  - Samsung: does not agree with Ericsson as companies already provided clear methodology and asked to validate each of them in last meeting.

  - LGE: agree with Ericsson. Samsung and Mediatek did not consider a channel estimation error for their methodology in their results. Different companies assumed different assumptions in their methodology.

Wednesday 13th session

| R1-135968 | WF on NAICS | LGE, Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Youngtae Kim from LGE and states that it is premature to reach any conclusion on NAICS system-level performance evaluations

- Further study on system performance evaluations is needed, which should at least take the following into consideration:

  - What to signal and signalling overhead/reliability

  - Evaluations with blind detection impacts

  - Impact of scheduling restrictions due to coordination

  - Non-ideal backhaul latency

  - CRS/DMRS IC

  - Inter-cell as well as intra-cell interference cancellation/suppression

Discussion (Question / Comment): Mr Chair asked whether the intention is to extend SI deadline although this is up to plenary to discuss.

MediaTek

Qualcomm: semi static part should be the first priority to look at - dynamic impact should be for further study phase

Decision: The document is noted.

Conclusion:

- Capture in TR the performance results corresponding to the agreed evaluation assumption in RAN1#75

- Add a note in the title of performance results section that all results are “preliminary performance results”

- Add the  note “All results have not considered any assistance signalling overhead, backhaul latency more than 2 msec in inter-cell NAICS in scenarios 1 and 2a”

For possible further conclusion:

- Add a note that “Methodology may need further discussion”

Continue offline discussion until Friday – (Mediatek)

Companies are still invited to check the excel sheet provided by MediaTek until Friday

Friday 15th

| R1-136050 | [Draft] TP for TR36.866 (System level evaluation results) | MediaTek |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion (Question / Comment): Final version although title still shows [Draft]

Decision: The document is noted and TP is agreed.

Not treated.

| R1-135124 | System-level performance evaluation of R-ML/ML receivers with and without signalling assistance | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-135237 | System level evaluation of iterative ML receiver for NAICS scenario 1 | Samsung |  |
| R1-135378 | System level performance evaluation for E-MMSE-IRC | ZTE |  |
| R1-135533 | System level simulation results for NAICS in scenario 1 | Huawei, HiSilicon |  |
| R1-135837 | System level performance of E-LMMSE-IRC | Nokia, NSN | (R1-135567) |
| R1-135896 | System level evaluation of SLML receiver for NAICS scenario 2 | Samsung | (R1-135236) |

#### Network coordination/signalling and possible spec impact

| R1-135125 | Discussion on network signalling for NAICS receivers | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and deals with

In this contribution we have discussed two mechanisms for network signalling assistance of the NAICS receiver. The first mechanism relies on backhaul link signalling between eNB to provide information about semi-static and cell common parameters to the victim UEs. The second approach is based on the signalling of interfering parameters directly from the aggressor eNB by using broadcast transmission on PDCCH/EPDDCH/PBCH or the like physical channels. For each signalling mechanisms some examples of the parameters of the interfering signals were proposed. It has been also concluded that signalling of interfering signal parameters (both semi-static and dynamic) might be useful for performance improvement and UE complexity reduction. Therefore, further and careful study is required to understand the set of the dynamic parameters for signalling with consideration of the trade-offs between receiver complexity, performance gain and signalling overhead.

Decision: The document is noted.

| R1-135333 | Network coordination/signalling | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Hua Xu from Qualcomm and proposes:

- To have a reduced set of data to CRS ERPE ratios utilized by the network to be semi-statically signalled.

- To allow UE with advanced receivers to assume resource allocation utilized by the network to be the same across a PRB pair.

- No need to signal spatial precoding scheme.

- No need to signal modulation order.

- Further restriction/coordination for DMRS based transmissions is FFS.

Decision: The document is noted.

| R1-135547 | Signalling and Coordination in NAICS | Broadcom Corporation |  |
| --- | --- | --- | --- |

The document was presented by Ms Helka-liina Maattanen from Broadcom and proposes that channel estimation, detection and decoding related parameters are either signalled on physical layer or on higher layer. The physical layer signalling would preferably be carried on the PDCCH from the serving cell whereas eICIC framework could be extended for higher layer signalling purposes. The split between the higher layer and physical layer signalling could however be studied further.

Discussion (Question / Comment): Ericsson questioned on TM being considered

Decision: The document is noted.

| R1-135553 | NAICS Coordination Performance and Requirements for TM9/10 | BlackBerry UK Limited |  |
| --- | --- | --- | --- |

The document was presented by Mark Harrison from RIM and draws the following observations:

- There can be significant link level gain for E-LMMSE-IRC in multipath channels such as ETU and with two dominant interferers

- There may be a small or no gain for E-LMMSE-IRC in near frequency flat channels such as EPA and with two dominant interferers:

Finally it is proposed for E-LMMSE-IRC, SL-SIC, and R-ML receivers, to further study mechanisms to

  - allow the UE to find the DMRS used on interfering cells

  - assign mutually orthogonal DMRS ports among desired and interfering PDSCHs

  - reduce DMRS port interference by setting some DMRS ports to zero power

- For SL-SIC and R-ML receivers, further study mechanisms to find the modulation used on interferers

- Further study the performance of DMRS coordination mechanisms among desired and interfering cells at the system level.

Decision: The document is noted.

| R1-135884 | On network coordination/signalling for NAICS and possible specification impact | MediaTek Inc. | (R1-135437) |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek and is summarized in the following:

- Network synchronization assumption/detection and any frequency/timing error compensation may be up to UE implementation and thus have no spec impact.

- It will be beneficial if some network coordination can be assumed by the UE on CP and subframe/slot-level alignment. This kind of coordination should not pose much complexity or constraint in deployment.

- It is beneficial to avoid UE decoding of interference PCFICH for the purpose of checking PDSCH alignment.  Dynamic signalling of neighbouring cell PCFICH is only possible with ideal backhaul. But semi-statically signalling PDSCH region is feasible with network coordination which can post only slight constraints to network operation.

- The knowledge of MBSFN subframe may be assumed at the UE using, for example, the existing mechanism developed for FeICIC on semi-static signalling of neighbour cell MBSFN subframes.

- Interference presence on each PRB pair may be detected from DMRS, but a UE cannot make the detection based on CRS only for CRS-based interference PDSCH. Detection of interference presence and its TM/PMI/RI based on data RE has not been widely evaluated in RAN4 in terms of performance degradation and complexity. Network assistance can be studied in parallel.

- Dynamic network signalling of PDSCH allocation can help, but large overhead is a concern. Network coordination is also possible (e.g., aligned PRBs) but with undesirable scheduling constraint. A partial-signalling partial-detection based approach can be further studied.

- Similar to MBSFN subframe signalling, Cell ID and the number of CRS ports may be signalled using existing Rel-11 mechanism developed for FeICIC.

- PMI/RI in CRS-based TM or RI/nSCID in the DMRS-based TMs may be feasible for signalling in the case of aligned PRB allocation.  Otherwise, detection on a per-PRB level is required from DMRS or data RE in the case of CRS-based TMs.  Performance degradation and complexity have not been widely evaluated in RAN4 and network coordination/signalling can be further studied in parallel.

- Data RE to CRS EPRE ratio, used in TM10, and PB (on OFDM symbols containing CRS) are all RRC configured and it is possible for semi-static signalling.

- ML/R-ML requires modulation order information on a per-PDSCH basis. It may be blindly detected on a per-PRB level with performance degradation or with signalling, which all requires further study.

- The current CRS, CSI-RS, and CSI-IM may not allow UEs to estimate the precoded interference channel and its presence/absence which are all important for NAICS to estimate the achievable CQI/RI. CSI feedback enhancements for NAICS receivers require further study and potential specification impact.

Decision: The document is noted.

| R1-135962 | Way Forward on NAICS network signalling/coordination | MediaTek, Blackberry |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek and proposes to capture below observations in the “Specification Impact” section of TR36.866

- Compared to requiring NAICS receivers to detect all interference parameters,  some network signalling/coordination can be beneficial for reducing receiver complexity and/or improve performance with increased robustness

  - The subset of transmission parameters that can be considered for signalling and that for receiver detection are FFS

  - The transmission parameters are listed in the RAN4 agreed receiver assumption section of TR36.866,  and generally include:

    - Parameters that are higher-layer configured per the current specifications (e.g., TM, cell ID, MBSFN subframes, CRS antenna ports, PA, PB, data-to-CRS power ratio)

    - Parameters that are dynamically signalled per the current specifications (e.g., CFI, PMI, RI, MCS, resource allocation, DMRS ports, n^DMRS_ID used in TM10)

    - Other deployment related parameters (e.g., synchronization, CP, subframe/slot alignment)

  - Some higher-layer configured parameters may be signalled or detected

    - Such assistance signalling may use higher layers regardless of whether the associated transmission parameter is higher-layer configured or dynamic

  - Some dynamic parameters may be coordinated, but with scheduling constraint, or detected or signalled or a combination of the three

  - Other deployment related parameters may be coordinated or detected.

  - Semi-static coordination signalling or coordination is suited for non-ideal backhaul

  - Dynamic coordination may be feasible only under ideal backhaul

    - The need of dynamic signalling and its mechanism (e.g., broadcast vs. unicast, from serving or interference cell) still need further study, taking into account the trade-off between gain and signalling overhead.

  - Other potential PHY impact needs further study (e.g., CSI feedback)

Discussion (Question / Comment): Also supported by ETRI.

Decision: The document is noted, modified and agreed as follows (to be captured in the “Specification Impact” section of TR36.866:

Agreements:

- Some transmission parameters are listed in the RAN4 agreed receiver assumption section of TR36.866, and include:

  - Parameters that are higher-layer configured per the current specifications (e.g., TM, cell ID, MBSFN subframes, CRS antenna ports, PA, PB)

  - Parameters that are dynamically signalled per the current specifications (e.g., CFI, PMI, RI, MCS, resource allocation, DMRS ports, n^DMRS_ID used in TM10)

  - Other deployment related parameters (e.g., synchronization, CP, subframe/slot alignment)

- Compared to requiring NAICS receivers to detect all interference parameters,  some network signalling/coordination can be beneficial for reducing receiver complexity and/or improve performance with increased robustness under intra-cell and inter-cell interference scenario

  - The transmission parameters that can be considered for signalling and that for receiver detection are FFS

    - Note that assistance signalling can be different from transmission parameters

  - Some transmission parameters may be detected or corresponding signalling of those parameters may be introduced

    - Such assistance signalling may use higher layers regardless of whether the associated transmission parameter is higher-layer configured or dynamic

  - Some dynamic assistance signalling can be considered if sufficient system-level gain is shown, and some dynamic parameters may be coordinated, but with scheduling constraint, or detected or signalled or a combination of the three

  - Other deployment related parameters may be coordinated or detected.

  - Semi-static coordination signalling or coordination is suited for non-ideal backhaul

  - Dynamic coordination may be feasible only under ideal backhaul

  - Other potential PHY impact needs further study (e.g., CSI feedback)

Friday 15th

| R1-136049 | [Draft] TP for TR 36.866 (Spec impact) | MediaTek |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion (Question / Comment): Final version although title still shows [Draft]

Decision: The document is noted and TP is agreed.

Not treated.

| R1-135017 | Discussion on network coordination/signalling and specification impact for advanced receivers | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-135182 | Signalling of Network Assistance Information in NAICS | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-135238 | Discussion on possible specification impacts for NAICS | Samsung |  |
| R1-135280 | Signalling for NAICS | ETRI |  |
| R1-135380 | Consideration on Network Signalling and Coordination for NAICS | ZTE |  |
| R1-135401 | Signalling mechanisms for interference information | Panasonic | (R1-134380) |
| R1-135493 | Considerations on network coordination and signalling for NAICS | LG Electronics |  |
| R1-135568 | System performance based on non-linear receivers | NSN, Nokia |  |
| R1-135772 | Signalling and coordination aspects of NAICS | Ericsson |  |
| R1-135838 | On the NAICS system and specification impact | NSN, Nokia | (R1-135569) |

#### Other

Not treated.

| R1-135126 | On network assisted reception of dual-connectivity | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-135127 | NAICS receivers for downlink control channel | Intel Corporation | (R1-134127) |

### Other

Rel-12 CRs have been treated under AI 6.1.

# Closing of the meeting

RAN1 Chairman, Mr Satoshi Nagata expressed his appreciation to North American Friends of 3GPP for hosting the meeting and providing good facilities, thanked the different ad-hoc chairs for allowing efficient parallel sessions during the week and the delegates for their hard working effort. Safe trip back home to all.

The meeting was closed at 16:55.

# Annex A:	List of Tdocs at RAN1 #75

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #75

| TS | CR | Rev | Rel | Title | Cat | Vsn | TD# | Source to WG | Work Item | Clauses affected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.222 | 0213 | 1 | Rel-12 | Introduction of HSPA signalling enhancements for more efficient resource usage for LCR TDD | B | 11.0.0 | R1-135050 | CATT, TD Tech, Potevio, New Postcom, CATR | LCR_TDD_HSPA_sign_enh-Core | 4.6.1.1.1, 4.6.1.2.1, 4.6B.1.6, 4.6D.1.3, 4.6H.1.1, 4.6I.1.1, 4.10.1.2, 4.10A.1.2 |
| 25.224 | 0273 | 1 | Rel-12 | Introduction of HSPA signalling enhancements for more efficient resource usage for LCR TDD | B | 11.1.0 | R1-135051 | CATT, TD Tech, Potevio, New Postcom, CATR | LCR_TDD_HSPA_sign_enh-Core | 5.1.1.6 |
| 25.800 | 0001 | 1 | Rel-12 | Update of Technical Report on UMTS Heterogeneous Networks | B | 12.0.0 | R1-135849 | Huawei, HiSilicon | FS_UTRA_hetnet | 7.1.3.6.2; 7.1.3.6.3; 7.1.3.6.4; 7.1.8.2.1; 7.1.8.3; 7.1.8.3.1; 7.1.8.3.2; 7.1.8.3.3; 7.1.8.3.5; 7.2.1.1.6; 7.2.1.2; 7.2.1.3; 7.3.5.3; 7.3.5.4; 7.3.5.5; 7.3.5.6; 7.3.5.6.1; 7.3.5.6.2; 7.3.5.6.3; 9. |
| 36.211 | 0184 | 3 | Rel-12 | Introduction of Rel 12 feature for Downlink MIMO Enhancement | B | 11.4.0 | R1-135996 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | LTE_eDL_MIMO_eEnh | 6.3.4.2.3 |
| 36.211 | 0186 | - | Rel-11 | Correction on the derivation of the non-MBSFN region by PCFICH | F | 11.4.0 | R1-136075 | Intel, Panasonic, Fujitsu, Huawei, HiSilicon, NEC, Texas Instruments, Qualcomm Inc., CATT, Interdigital, Ericsson, Samsung, BlackBerry | LTE-Phy | 6.1 |
| 36.212 | 0154 | 3 | Rel-12 | Introduction of Rel 12 feature for Downlink MIMO Enhancement | B | 11.3.0 | R1-135183 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | LTE_eDL_MIMO_eEnh | 5.2.2.6.1, 5.2.2.6.2, 5.2.2.6.1, 5.2.3.3.1, 5.2.3.3.2 |
| 36.213 | 0431 | 5 | Rel-12 | Introduction of Rel 12 feature for Downlink MIMO Enhancement | B | 11.4.0 | R1-135894 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | LTE_eDL_MIMO_eEnh | 7.2 |
| 36.213 | 0444 | 1 | Rel-10 | Correction on CSI reporting type and parameters | F | 10.10.0 | R1-135888 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | LTE_eDL_MIMO_eEnh | 7.2, 7.2.4 |
| 36.213 | 0445 | 2 | Rel-11 | Correction on CSI reporting type and parameters | A | 11.4.0 | R1-136073 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell | LTE_eDL_MIMO_eEnh | 7.2, 7.2.4 |
| 36.213 | 0446 | - | Rel-11 | Correction on deriving the length of the non-MBSFN region | F | 11.4.0 | R1-136076 | Intel, Panasonic, Fujitsu, Huawei, HiSilicon, NEC, Texas Instruments, Qualcomm Inc., CATT, Interdigital, Ericsson, Samsung, BlackBerry | LTE-Phy | 9.1.3 |
| 36.872 | 0002 | - | Rel-12 | CR on higher order modulation for Small cell enhancements | F | 12.0.0 | R1-136051 | Huawei, HiSilicon | FS_LTE_SC_enh_L1 | 6.1 |
| 36.872 | 0003 | - | Rel-12 | CR on small cell on/off for Small cell enhancements | F | 12.0.0 | R1-136052 | Huawei, HiSilicon | FS_LTE_SC_enh_L1 | 7.1 |
| 36.872 | 0004 | - | Rel-12 | Corrections on Radio-interface based synchronization for small cell enhancements | F | 12.0.0 | R1-136053 | Huawei, HiSilicon, CATR | FS_LTE_SC_enh_L1 | 7.3.1, A.4 |
| 36.872 | 0005 | - | Rel-12 | Corrections on the summary of RIBS for small cell enhancements | F | 12.0.0 | R1-136054 | Huawei, HiSilicon, Ericsson, CATR | FS_LTE_SC_enh_L1 | 7.3.3 |
| 36.872 | 0006 | - | Rel-12 | CR on small cell discovery for Small cell enhancements | F | 12.0.0 | R1-136059 | NTT DOCOMO | FS_LTE_SC_enh_L1 | 7.2 |
| 36.872 | 0007 | - | Rel-12 | CR on higher order modulation conclusion | F | 12.0.0 | R1-136079 | Huawei, HiSilicon | FS_LTE_SC_enh_L1 | 6.1 |

# Annex C-1:	List of Outgoing LSs from RAN1 #75

| R1 | Response to (Ic LS) | To | Cc | Title | Contact | Ref'd /Attachd Tdoc | Release | WI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-135987 |  | R2 | RAN | LS to RAN2 on RAN1 input to Further EUL Enhancements TR | Ericsson |  | Rel-12 | FS_EDCH_enh |
| R1-136072 | S4-131406 (R1-135832) | S4 | R2, S2 | Response LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | Qualcomm |  | Rel-12 | MI-EMO |
| R1-136065 |  | R2 | R4 | LS on introduction of LTE coverage enhancement | ZTE | L1 parameters for LTE_cov_enh.xls | Rel-12 | Cov_Enh_LTE-Core |
| R1-135919 |  | R2, R4 |  | LS on further MBMS operations support for E-UTRAN | Qualcomm | R1-135918 | Rel-12 | MBMS_LTE_OS |
| R1-136024 |  | RAN | R2 | LS on the need of PUCCH on multiple UL serving cells for non-co-located CA | NTT DoCoMo |  | Rel-12 | LTE_CA_TDD_FDD-Core, FS_LTE_SC_enh_hilayer |
| R1-136045 | RT-130054 | RAN ITU-R Ad Hoc | R2, R3, R4, R5 | 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | Telecom Italia |  | - | - |
| R1-135986 |  | R2 |  | LS on HS-SCCH Orders for CPC DTX/DRX Activation / Deactivation in Multiflow Mode | ZTE |  | Rel-11 | HSDPA_MFTX-Core |
| R1-135864 |  | R2 |  | LS on Multiflow and CLTD with assisting cell feedback | NSN |  | Rel-11 | HSDPA_MFTX-Core, HSPA_UL_TxDiv-CL-Core |
| R1-136067 |  | S1 |  | LS on D2D proximity criteria | Qualcomm, Telecom Italia |  | Rel-12 | FS_LTE_D2D_Prox |

# Annex C-2:	List of Incoming LSs from RAN1 #75

| Source | Original Tdoc nbr | RAN1 Tdoc nbr | To | Cc | Title | Response to (Ic LS) | Release | WI | Contact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2 | R2-133719 | R1-135002 | R1 |  | LS on Rel-12 L1 parameters |  | Rel-12 | LTE_TDD_eIMTA-Core, Cov_Enh_LTE-Core, LC_MTC_LTE-Core, LTE_CA_TDD_FDD-Core, MBMS_LTE_OS-Core | NSN |
| R2 | R2-133720 | R1-135003 | S2, R1, R3 |  | LS on Public Safety UE-Network Relays |  | Rel-12 | ProSe | Vodafone |
| R2 | R2-133726 | R1-135004 | R1 |  | LS on text proposal for TR 36.843 on D2D including RAN2 agreements |  | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R2 | R2-133728 | R1-135005 | S2 | R1, R3, R4 | Response LS on GCSE with eMBMS | S2-133846 (R2-133066) | Rel-12 | FS_LTE_GC | Qualcomm |
| R3 | R3-131971 | R1-135006 | S2 | R1, R2 | Reply LS on Public Safety UE-to-Network Relays | S2-133808 | Rel-12 | ProSe | Vodafone |
| R3 | R3-131972 | R1-135007 | S2 | R1, R2 | Reply LS on GCSE with eMBMS | S2-133846 (R3-131883) | Rel-12 | FS_LTE_GC | NSN |
| R4 | R4-135610 | R1-135008 | R1 | R2 | Reply LS on maximum UL timing difference between TAGs | R1-132819 | Rel-11 | LTE_CA-Core | Huawei |
| ITU-R Ad Hoc | RT-130054 | R1-135009 | CT, SA, R1, R2, R3, R4, R5 |  | 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” |  | - | - | Telecom Italia |
| S4 | S4-131406 | R1-135832 | S2, R1, R2 |  | LS on the UE capabilities regarding support of simultaneous MBMS bearer reception |  | Rel-12 | MI-EMO | Samsung |
| R3 | R3-132375 | R1-136004 | RAN ITU-R Ad Hoc | R1, R2, R4, R5 | 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | RT-130054 | - | - | Telecom Italia |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #75

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-135998 | TR36.843 v1.0.0 on Study on LTE Device to Device Proximity Services-Radio | QUALCOMM Incorporated | Agreed |
| R1-136046 | TR25.701 v2.0.0 of Scalable UMTS | China Unicom | Endorsed |
| R1-136083 | TR 36.866 v1.0.0 Network-Assisted Interference Cancellation and Suppression for LTE | MediaTek | Endorsed |
| R1-136084 | TR 36.873 v1.2.0 on 3D channel model for LTE | NSN | Agreed |
| R1-136086 | TR 36.847 v2.0.0 | CMCC |  |
| R1-136088 | TR 36.874 V2.0.0 Coordinated multi-point operation for LTE with non-ideal backhaul | Samsung | Endorsed |

# Annex F:	List of actions

- Outgoing LS.

[75-01] Enrico Buracchini, Telecom Italia

| R1-136034 | [Draft] 3GPP internal LS on update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | Telecom Italia |  |
| --- | --- | --- | --- |

Email approval until 20th November, 5 P.M. CET

Done: The final LS is agreed in R1-136045 according to email decision made on Nov 21st.

[75-21] Junqiang Liu, ZTE

| R1-135986 | LS on HS-SCCH Orders for CPC DTX/DRX Activation / Deactivation in Multiflow Mode | ZTE |  |
| --- | --- | --- | --- |

Email review until 22nd November

Done: The LS is agreed according to email decision made on Nov 23rd.

[75-28] Ryszard Dokuczal, NSN

| R1-135864 | LS on Multiflow and CLTD with assisting cell feedback | RAN1, NSN |  |
| --- | --- | --- | --- |

Email approval until 22nd November

Done: The LS is agreed according to email decision made on Nov 23rd.

[75-17] Peter Gaal, Qualcomm and Enrico Buracchini, Telecom Italia
LS to SA1 on D2D proximity criteria

Email approval until 13th December

Done: The LS is agreed in R1-136067 according to email decision made on Jan 16th.

[75-32] Zhisong Zuo, ZTE

| R1-136065 | LS on introduction of LTE coverage enhancement | RAN1, ZTE | (R1-136055) |
| --- | --- | --- | --- |

Email approval until 22th November by including RAN4 to "cc" part

Done: The LS is agreed according to email decision made on Nov 24th.

- CR approval

[75-33] Carmela Cozzo, Huawei

| R1-135849 | 25.800 CR0001r1 (Rel-12, B) Update of Technical Report on UMTS Heterogeneous Networks | Huawei, HiSilicon | (R1-135690) |
| --- | --- | --- | --- |

Email approval until 25th November

Done: According to Mr Chair's decision on Nov 26th, the revision 1 of CR0001 is agreed.

[75-34] Min Zhang, Alcatel Lucent

| R1-135996 | 36.211 CR0184R3 (Rel-12, B) Introduction of Rel 12 feature for Downlink MIMO Enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Email approval until 27th November

Done: According to Mr Chair's decision on Nov 28th, the revision 3 of CR0184 is agreed.

- Text proposal for TS and TR

[75-22] Ryszard Dokuczal, NSN

| R1-135989 | TP on summary results for E-DCH decoupling | NSN, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Email discussion until 20th November

Done: The TP is agreed according to email decision made on Nov 23rd.

[75-23] Arjun Bharadwaj, Qualcomm
Link and system performance of RRS in HetNet
Email discussion until 20th November

Done: Huawei expressed concerns about Qualcomm's results (email dated on Nov 22nd) and cannot agree on these results and consequently on the TP. Mr Chair closed the discussion on Nov 23rd - No consensus.

[75-24] Arjun Bharadwaj, Qualcomm
TP summarizing results in R1-135785 and R1-135786
Email discussion until 20th November

Done: According to Mr Chair's decision on Nov 23rd, the TP is agreed as:

| R1-135997 | Text proposal on IPICH based solution for Combined Cells | QUALCOMM Incorporated |  |
| --- | --- | --- | --- |

[75-25] Carmela Cozzo, Huawei

| R1-135990 | TP on Conclusions for UMTS HetNet | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Email discussion until 20th November

Done: According to Mr Chair's decision on Nov 23rd, the TP is agreed.

[75-26] Arjun Bharadwaj, Qualcomm

| R1-135926 | TP on System level evaluation results | Qualcomm Incorporated | (R1-135925) |
| --- | --- | --- | --- |

WF on system level evaluations for Time-Dilated UMTS is for email checking/approval until 19th November

Done: According to Mr Chair's decision on Nov 23rd, the TP is agreed.

[75-27] Karri Ranta-aho, NSN

| R1-136043 | Conclusions on Scalable UMTS | Ericsson, NSN, Broadcom Corporation, MediaTek Inc., Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Qualcomm, China Unicom, ZTE | (R1-136042) |
| --- | --- | --- | --- |

Email discussion/approval until 22nd November.

Done: According to Mr Chair's decision on Nov 26th, the TP on conclusions is agreed.

[75-31] Han Xiao, China Unicom

| R1-136044 | TR25.701 v1.2.0 of Scalable UMTS | China Unicom | R1-136046 |
| --- | --- | --- | --- |

After completion of email approvals of [75-27], email approval of next update of TR (v.1.2.0) until 27th November.

Done: TR version 1.2.0 is agreed in R1-136044 and endorsed as v2.0.0 in R1-136046 for approval, according to Mr Chair's decision on Nov 29th.

[75-18] Saurabha Tavildar, Qualcomm

| R1-135910 | TR36.843 v0.3.1 on Study on LTE Device to Device Proximity Services-Radio | Qualcomm |  |
| --- | --- | --- | --- |

Email approval until 22nd November for v.1.0.0 for submission to RAN#62 for information

Done: .The TR is agreed as version 1.0.0 in R1-135998, according to email decision of Mr Chair on Nov 23rd.

[75-30] Jeff Zhuang, Mediatek

| R1-136080 | TR 36.866 v0.6.0 Network-Assisted Interference Cancellation and Suppression for LTE | MediaTek | R1-136082 |
| --- | --- | --- | --- |

Email approval of RAN1/4 TPs into TR36.866 v0.6.0 until 22nd Nov.

Done: TR revised and agreed as version 0.7.0 in R1-136082, and endorsed as v1.0.0 in R1-136083 for one step approval, according to Mr Chair's decision on Nov 27th.

- Miscellaneous

[75-02] Zukang Shen, CATT

| R1-136025 | Configuration of subframes for explicit UL-DL reconfiguration DCI | CATT |  |
| --- | --- | --- | --- |

Email approval until 29th November

Done: Although how to restrict the number of configurations with a bitmap approach may still require further discussion, the configurations were finally agreed in R1-136066, according to Mr Chair’s decision on Dec 24th.

[75-03] Tim Frost, Vodafone

| R1-135802 | Guidance on delay tolerance level of MTC applications | Vodafone Group |  |
| --- | --- | --- | --- |

Provide information until 6th December

Done: For information, it is confirmed that additional delays can be accepted, but it is difficult to put a number on what would be unacceptable and RAN1 should continue to consider the trade-offs with battery consumption, system complexity, and (especially here for common channel operations) spectrum efficiency.

[75-04] June Xia, HiSilicon

| R1-136006 | Way forward on the remaining details of new UE category/type | Huawei, HiSilicon, CATT, CATR, ZTE, Alcatel-Lucent, Alcatel Shanghai Bell, Ericsson, ST-Ericsson |  |
| --- | --- | --- | --- |

Email discussion until 6th December

Done: The following proposals are agreed in R1-136061 for the new UE category/type:

- Both contiguous and non-contiguous resource allocation for unicast are supported.

- From RAN1 specification point of view, eNB does not require knowledge of the single Rx antenna property of the UE.

[75-05] Tim Frost, Vodafone

| R1-135943 | Way forward on P-BCH for MTC enhanced coverage | Vodafone Group |  |
| --- | --- | --- | --- |

Email discussion until RAN1 #76 meeting except for agreements part

Done: Status to be checked at RAN1#76.

[75-06] Hidetoshi Suzuki, Panasonic

| R1-136057 | Offline report on the handling on PRACH for enhanced coverage mode | Panasonic |  |
| --- | --- | --- | --- |

Email discussion until 6th December about starting level.

Done: Status of discussion to be provided at RAN1#76.

[75-07] Tim Frost, Vodafone

Whether to use the same PDCCH candidate m in each repetition for coverage enh.

Email discussion until RAN1 #76 meeting

Done: Status to be checked at RAN1#76.

[75-08] Joon Kui Ahn, LG Electronics

| R1-136009 | Offline discussion on self-scheduling for TDD-FDD CA | LGE, Ericsson |  |
| --- | --- | --- | --- |

Email discussion until RAN1 #76 meeting

Done: Check in RAN1#76 the detail of Options 1) and 2-c) of  HARQ timing of PDSCH transmission on SCell with self-scheduling when PCell is TDD carrier and SCell is FDD carrier.

- Option 1) FDD SCell PDSCH timing depends on TDD PCell timing + additional new timing for remaining subframes of FDD SCell

  - If UL/DL configuration 5 is used, the number of HARQ processes is less than 17

- Option 2-c) The PDSCH HARQ timing of FDD SCell follows the DL reference TDD U/D configuration, where the reference TDD U/D configuration is one of the existing 7 U/D configurations

  - The DL reference TDD U/D configuration is configured by higher layers

[75-09] David Mazzarese, Huawei

Cross-carrier scheduling for TDD-FDD CA

Email discussion until 24th January

Done: Status to be checked at RAN1#76.

[75-10] Bishwarup Mondal, NSN

| R1-136021 | TP for mechanical tilt | NSN, Nokia | (R1-135587) |
| --- | --- | --- | --- |

Email discussion until 22nd November to conclude the yellow part taking into account R1-136036

Done: The following is agreed assuming Mr Chair’s email conclusion on Dec 3rd:

1)      Use Alt-2 for calibration, which is considered to calibrate also Alt-1

2)      Use Alt-1, and Alt-2 for baseline (only cell-avg. and cell-edge numbers for baseline so not too many combinations to deal with).

Agree in principle to capture in TR that “Typical antennas have polarization parallelity that is lower than the polarization parallelity of dipoles and, in addition, typical antennas have polarization parallelity close to zero in the main beam.

[75-11] Jonghyun Park, LG Electronics

| R1-135951 | Way Forward on UE orientation and UE antenna pattern in 3D-channel | LG Electronics, Fraunhofer, Elektrobit, ETRI, Anite, KDDI |  |
| --- | --- | --- | --- |

Email discussion until 22nd November

Done: According to Mr Chair’s decision on Dec 3rd, the following working assumptions are agreed:

- Working assumption:  For distributions on UE orientation angles, take Option 1 to be used for calibration and baseline performance evaluations, and continue discussion whether to include Option 2 in the TR.

  - Option 1:  ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree

  - Option 2:  ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = arccos(X) with X ~ U[-1,1], ΩUT,g uniformly distributed on [0,360] degree

- Working assumption:  For UE antenna gain pattern in local coordinate systems, take Option 1 to be used for calibration and baseline performance evaluations, and continue discussion whether to include Option 2 in the TR.

  - Option 1:  Isotropic antenna gain pattern A’(θ’,ф’) = 1

  - Option 2:  Dipole antenna gain pattern A’(θ’,ф’) = D(cos((π/2)cosθ’)/sinθ’)2 with D=1.63

[75-12] Bishwarup Mondal, NSN

| R1-136011 | Way Forward on ZOD offset and ZOD generation | NSN, Nokia, Samsung, CMCC |  |
| --- | --- | --- | --- |

Email discussion until 22nd November to conclude issues related to equation (3) and tables 2/3

Done: According to Mr Chair’s decision on Nov 27th, the following working assumptions are agreed:

- Equation (3) in R1-136011 : , here  is a realization of ZSD

- Table 2 in R1-136011 + modifications in chairman’s notes

[75-13] George Jongren, Ericsson

Phase 1 calibration for 3D-channel

Email discussion until 22nd November to conclude one of the following two alternatives

- • Alt 1: geographical distance based wrapping

- • Alt 2: radio distance base wrapping

Done: According to Mr Chair's decision on Nov 29th, the following is agreed:

- Geographical distance based wrapping is baseline for calibration and baseline performance evaluations of urban macro and micro scenarios

- Companies are strongly encouraged to bring calibration and baseline performance results also for radio distance based wrapping

- Calibration excel sheet will clearly distinguish between the two wrapping methods

- The choice of wrapping method and wrapping area size will be revisited in next meeting

[75-14] Hui Tong, CMCC
High-rise scenario for 3D-channel
Email discussion until 17th January to study alternatives for each respective issue, targeting convergence of views

Done: Status of discussion to be provided at RAN1#76.

[75-15] Bishwarup Mondal, NSN
TR36.873 for 3D-channel
Email discussion until 26th November

Done: According to Mr Chair’s email dated on Nov 29th the following is agreed:

| R1-136084 | TR 36.873 v1.2.0 on 3D channel model for LTE | NSN |  |
| --- | --- | --- | --- |

[75-16] Stefano Sorrentino, Ericsson
Resource allocation method for D2D communication
Email discussion until 29th November to consider revisiting the RSRP threshold.

Done: According to Mr Chair’s email dated on Dec 12th the following is considered as working assumption:

- Two RSRP thresholds are considered as the working assumption for simulation of D2D communication: -112dBm and ->107dBm. Companies are encouraged to provide results based on these values.

[75-19] Hyojin Lee, Samsung
TR36.874 for CoMP-NIB
Email approval of the TR until 22nd November to add additional results (which must be provided by Wednesday 20th November) and corresponding update of the TR conclusions.

Done: According to Mr Chair’s email dated on Nov 27th the following is agreed:

| R1-136085 | TR 36.874 V1.3.0 Coordinated multi-point operation for LTE with non-ideal backhaul | Samsung | (R1-136078) |
| --- | --- | --- | --- |

[75-20] Hyojin Lee, Samsung

| R1-136008 | Way Forward on CoMP NIB signalling | Samsung, China Unicom, DAC-UPT, Deutsche Telekom, IAESI, SK Telecom, Telecom Italia |  |
| --- | --- | --- | --- |

Email discussion/approval until 21st November

Done: According to Mr Chair’s email dated on Dec 3rd, TR36.874 v2.0.0 is endorsed for approval by the plenary.

| R1-136088 | TR 36.874 V2.0.0 Coordinated multi-point operation for LTE with non-ideal backhaul | Samsung | (R1-136087) |
| --- | --- | --- | --- |

[75-29] Bishwarup Mondal, NSN
Calibration for 3D channel model
Email discussion until 30th January considering the results of [75-13]

-> Bishwarup, please explain the detailed target/plan about it.

Done: Status of discussion to be provided at RAN1#76.

LTE email discussions following RAN #62

[75-35] Xiaoming She, China Telecom
Cov. enh. for TDD related issue
Email approval until 24th January with the restrictions of no discussion within December, and start discussion from 1/6
Discussion point: To confirm that scheduling more than 3 PRBs is supported for TDD TTI bundling for PUSCH

Done: According to Mr Chair’s decision (1st Feb), the following is agreed:

- Scheduling more than 3 PRBs is supported for TDD TTI bundling for PUSCH in Rel-12.

[75-36]: Zukang Shen, CATT
PHR for TDD eIMTA

- Discuss whether existing PHR mechanism is sufficient for TDD eIMTA

- Discuss whether any potential gains by PHR enhancements for TDD eIMTA

Email approval until 24th January with the restrictions of no discussion within December, and start discussion from 1/6

Done: Status of discussion to be provided at RAN1#76.

[75-37]: Zukang Shen, CATT

DL aperiodic CSI measurement and reporting

- For TM 1 – 9, discuss the aperiodic CSI measurement and reporting mechanism for TDD eIMTA with two subframe sets

- For TM10,

- discuss the minimum UE capability on the number of CSI processes for TDD eIMTA UE

- discuss the CSI-IM configurations for TDD eIMTA

- discuss the triggering of aperiodic CSI reporting for TDD eIMTA

Email approval until 24th January with the restrictions of no discussion within December, and start discussion from 1/6

Done: Status of discussion to be provided at RAN1#76.

[75-38]: Zukang Shen, CATT

PUCCH for TDD eIMTA

- Discuss the supported PUCCH formats for HARQ-ACK feedback for TDD eIMTA

- Discuss the PUCCH resource allocation for HARQ-ACK for TDD eIMTA

Email approval until 24th January with the restrictions of no discussion within December, and start discussion from 1/6

Done: Status of discussion to be provided at RAN1#76.

[75-39]: Li Bo, Huawei

HARQ-ACK feedback on PUSCH for eIMTA

- Discuss HARQ-ACK feedback on PUSCH, when UL scheduling grant is transmitted earlier than the latest DL subframe within the bundling window, since UL DAI cannot indicate the total number of the subframes with PDSCH transmissions and with PDCCH/EPDCCH indicating downlink SPS release within the bundling window.

- Discuss whether to use UL DAI or UL index in DCI format 0 and 4, when UL-DL configuration 0 is configured in as UL reference configuration.

Email approval until 24th January with the restrictions of no discussion within December, and start discussion from 1/6

Done: Status to be checked at RAN1#76.

# Annex G:	List of participants at RAN1 #75

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2014 - 2015

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#76 | WG | 10 – 14 Feb 2014 | Prague | CZ Rep. |
| 3GPPRAN1#76bis | WG | 31 Mar – 04 Apr 2014 | Shenzhen | China |
| 3GPPRAN1#77 | WG | 19 – 23 May 2014 | Seoul | KR |
| 3GPPRAN1#78 | WG | 18 – 22 Aug 2014 | Dresden | Germany |
| 3GPPRAN1#78bis | WG | 6 – 10 Oct 2014 | Ljubljana | Slovenia |
| 3GPPRAN1#79 | WG | 17 – 21 Nov 2014 | San Francisco (TBC) | US |
| 3GPPRAN1#80 | WG | 9 – 13 Feb 2015 | TBD | EU |
| 3GPPRAN1#80bis | WG | 20 – 24 Apr 2015 | TBD | EU |
| 3GPPRAN1#81 | WG | 25 – 29 May 2015 | TBD | TBD |
| 3GPPRAN1#82 | WG | 24 – 28 Aug 2015 | TBD | China |
| 3GPPRAN1#82bis | WG | 5 – 9 Oct 2015 | TBD | TBD |
| 3GPPRAN1#83 | WG | 16 – 20 Nov 2015 | TBD | TBD |

| MEETING TYPES |  |
| --- | --- |
| AH = Ad Hoc | CM = Chairmen's meeting |
| JM = Joint | OR = Ordinary |
| PM = Preparatory Meeting | RG = Rapporteurs Group |
| RM = Resolution Meeting | SG = Steering Group |
| ST = Startup Meeting | TG = Task Group |
| WG = Working Group | XO = Extraordinary |

End of document