3GPP TSG RAN WG1 Meeting #76,	R1-140755

Prague, Czech Republic, 10th – 14th February 2014

Source:	Ericsson

Title:	TBS table design for downlink 256QAM

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

# Introduction

In RAN plenary meeting #62, a new WI on small cell enhancements – physical layer aspects was approved and it was decided to introduce 256QAM in the downlink for small cells in Rel-12 according to below agreement quoted from [1]:

- Spectrum efficiency enhancement with introduction of higher order modulation, i.e., 256QAM, in the downlink transmission, while keeping existing size of CQI feedback field and MCS indication.

The standard impacts of introducing downlink 256QAM have been discussed and the physical layer aspects in RAN1 mainly include CQI/MCS/TBS table design.

In our companion contributions [2] and [3], CQI table and MCS table for 256QAM are proposed. The sizes of both tables are kept and a majority of entries are unchanged to minimize specification and UE implementation/cost impacts. For MCS table, six new entries containing six new TBS indices 27-32 are proposed to be added for 256QAM support as discussed in [3]. The new TBS indices correspond to six new TBS rows with new TBS values (across all possible bandwidths) that are required to attach to the current TBS table. In this contribution, the design of new TBS table for 256QAM support is discussed and proposed.

# Discussion

In Rel-11 TS 36.213, the 27x110 TBS table for one spatial layer is populated with 178 unique transport block sizes (TBS) as given in Table 7.1.7.2.1-1 in [4]. To support multiple spatial layers, 51 additional unique TBS are defined for one-layer to two-layer, one-layer to three-layer and one-layer to four-layer TBS translation given in Table 7.1.7.2.2-1, Table 7.1.7.2.4-1, Table 7.1.7.2.5-1 in [4]. In total, there are 229 unique TBS defined in Rel-11. The design allows the TBS tables to be very efficiently stored using 8-bit indices to the unique TBS list instead of 19-bit integers representing the TBS directly.

The reduction of potentially 27x110=2970 TBS to only 178 TBS in the one layer table is achieved by allowing the actual code rate to deviate from the target code rate by ±2%. This is for example shown in Figure 1 for the 64QAM MCS (17-27), in Figure 2 for the 16QAM MCS (10-16), and Figure 3 for the QPSK MCS (0-9), respectively.

Observation:

- By allowing the actual code rate to deviate from the target code rate by ±2%, there are in total 229 unique TBS defined in Rel-11 enabling efficient TBS table storage.

Figure 1 Actual code rate and code rate inaccuracy in percentage for Rel-11 64QAM

Figure 2 Actual code rate and code rate inaccuracy in percentage for Rel-11 16QAM

Figure 3 Actual code rate and code rate inaccuracy in percentage for Rel-11 QPSK

To support 256QAM in Rel-12, six new entries containing six new TBS indices 27-32 are proposed to be added in the MCS table as discussed in [3]. Consequently, six additional rows of TBS need to be added to the existing 27x110 TBS table for one spatial layer. By allowing up to ±2% deviation from the target code rate, the 6x110 TBS can be designed without introducing any new TBS for 256QAM one spatial layer support, as given in Table 1.

It worth noting that the peak data rate in Table 1 is set by the largest TBS for NPRB = 100 according to Rel-8 LTE design principles.

With the new TBS table and the new MCS table proposed in [3], actual code rate and code rate in accuracy in percentage for 256QAM is shown in Figure 4. It can be seen that the deviation is well within ±2% aligned with Rel-11 design.

Observation:

- By allowing up to ±2% deviation from the target code rate, the new TBS table can be designed without introducing any new transport block sizes for 256QAM one spatial layer support in Rel-12.

Table 1 Six new row in TBS table for 256QAM support

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27 | 648 | 1320 | 1992 | 2664 | 3368 | 4008 | 4584 | 5352 | 5992 | 6712 |
| 28 | 696 | 1416 | 2152 | 2856 | 3624 | 4264 | 4968 | 5736 | 6456 | 7224 |
| 29 | 744 | 1480 | 2280 | 2984 | 3752 | 4584 | 5352 | 5992 | 6712 | 7480 |
| 30 | 776 | 1608 | 2408 | 3240 | 4008 | 4776 | 5736 | 6456 | 7224 | 7992 |
| 31 | 840 | 1672 | 2536 | 3368 | 4264 | 5160 | 5992 | 6712 | 7736 | 8504 |
| 32 | 872 | 1736 | 2664 | 3496 | 4392 | 5352 | 6200 | 6968 | 7992 | 8760 |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 27 | 7224 | 7992 | 8504 | 9144 | 9912 | 10680 | 11448 | 11832 | 12576 | 12960 |
| 28 | 7736 | 8504 | 9144 | 9912 | 10680 | 11448 | 12216 | 12960 | 13536 | 14112 |
| 29 | 8248 | 9144 | 9912 | 10680 | 11448 | 12216 | 12960 | 13536 | 14688 | 15264 |
| 30 | 8760 | 9528 | 10296 | 11448 | 12216 | 12960 | 13536 | 14688 | 15264 | 15840 |
| 31 | 9144 | 10296 | 11064 | 11832 | 12576 | 13536 | 14688 | 15264 | 16416 | 16992 |
| 32 | 9912 | 10680 | 11448 | 12216 | 13536 | 14112 | 15264 | 15840 | 16992 | 17568 |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 27 | 14112 | 14688 | 15264 | 15840 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 |
| 28 | 14688 | 15840 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 21384 |
| 29 | 15840 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 21384 | 22152 | 22920 |
| 30 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 22152 | 22920 | 23688 | 24496 |
| 31 | 17568 | 18336 | 19848 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 25456 |
| 32 | 18336 | 19848 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 25456 | 26416 |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 27 | 20616 | 21384 | 22152 | 22920 | 22920 | 23688 | 24496 | 25456 | 25456 | 26416 |
| 28 | 22152 | 22920 | 23688 | 24496 | 24496 | 25456 | 26416 | 27376 | 27376 | 28336 |
| 29 | 23688 | 24496 | 25456 | 25456 | 26416 | 27376 | 28336 | 29296 | 29296 | 30576 |
| 30 | 25456 | 25456 | 26416 | 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 31704 |
| 31 | 26416 | 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 32856 | 32856 | 34008 |
| 32 | 27376 | 28336 | 29296 | 30576 | 30576 | 31704 | 32856 | 34008 | 34008 | 35160 |
|  | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 27 | 27376 | 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 31704 | 32856 | 32856 |
| 28 | 29296 | 30576 | 30576 | 31704 | 31704 | 32856 | 34008 | 34008 | 35160 | 35160 |
| 29 | 31704 | 31704 | 32856 | 34008 | 34008 | 35160 | 35160 | 36696 | 37888 | 37888 |
| 30 | 32856 | 34008 | 35160 | 35160 | 36696 | 36696 | 37888 | 39232 | 39232 | 40576 |
| 31 | 35160 | 35160 | 36696 | 37888 | 37888 | 39232 | 40576 | 40576 | 42368 | 42368 |
| 32 | 36696 | 36696 | 37888 | 39232 | 39232 | 40576 | 42368 | 42368 | 43816 | 43816 |
|  | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 |
| 27 | 34008 | 34008 | 35160 | 35160 | 36696 | 36696 | 37888 | 37888 | 39232 | 39232 |
| 28 | 36696 | 36696 | 37888 | 37888 | 39232 | 40576 | 40576 | 40576 | 42368 | 42368 |
| 29 | 39232 | 39232 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 45352 | 45352 |
| 30 | 40576 | 42368 | 42368 | 43816 | 43816 | 45352 | 45352 | 46888 | 46888 | 48936 |
| 31 | 43816 | 43816 | 45352 | 45352 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 |
| 32 | 45352 | 45352 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 | 52752 |
|  | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 |
| 27 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 | 46888 |
| 28 | 43816 | 43816 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 | 48936 |
| 29 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 |
| 30 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 |
| 31 | 51024 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 |
| 32 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 |
|  | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |
| 27 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 |
| 28 | 51024 | 51024 | 52752 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 |
| 29 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 |
| 30 | 57336 | 57336 | 59256 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 |
| 31 | 59256 | 61664 | 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 |
| 32 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 71112 | 71112 |
|  | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 |
| 27 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 |
| 28 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 |
| 29 | 61664 | 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 |
| 30 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 |
| 31 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 | 73712 | 75376 | 75376 | 76208 |
| 32 | 71112 | 73712 | 73712 | 73712 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 |
|  | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 |
| 27 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 | 63776 | 66592 | 66592 |
| 28 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 |
| 29 | 68808 | 71112 | 71112 | 71112 | 73712 | 73712 | 73712 | 75376 | 75376 | 76208 |
| 30 | 73712 | 73712 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 | 78704 | 81176 |
| 31 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 |
| 32 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 | 84760 | 87936 | 87936 | 87936 |
|  | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 |
| 27 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 71112 | 73712 |
| 28 | 71112 | 73712 | 73712 | 73712 | 75376 | 75376 | 76208 | 76208 | 78704 | 78704 |
| 29 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 81176 | 84760 | 84760 |
| 30 | 81176 | 81176 | 84760 | 84760 | 84760 | 84760 | 87936 | 87936 | 87936 | 87936 |
| 31 | 84760 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 |
| 32 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 | 87936 |

Figure 4 Actual code rate and code rate inaccuracy in percentage for Rel-12 256QAM

To support 256QAM transport blocks mapped to multiple spatial multiplexing layers, 3 new tables need to be designed for one-layer to two-layer, one-layer to three-layer and one-layer to four-layer translation. The proposed TBS translation tables are given in Table 2, Table 3 and Table 4, respectively. Five new mappings are needed for TBS = 76208, 78704, 81176, 84760 and 87936 for each of the one-to-two, one-to-three and one-to-four TBS translation tables.

Allowing mapping inaccuracy of ±0.4%, the additional one-layer to two-layer mappings can be supported without introducing new TBS, as given in Table 2.

Table 2 TBS mapping table for one-layer to two-layer

| TBS_L1 | TBS_L2 |
| --- | --- |
| 76208 | 152976 |
| 78704 | 157432 |
| 81176 | 161760 |
| 84760 | 169544 |
| 87936 | 175600 |

Allowing mapping inaccuracy of ±1.25%, the additional one-layer to three-layer mappings can be supported without introducing new TBS, as given in Table 3.

Table 3 TBS mapping table for one-layer to three-layer

| TBS_L1 | TBS_L3 |
| --- | --- |
| 76208 | 230104 |
| 78704 | 236160 |
| 81176 | 245648 |
| 84760 | 254328 |
| 87936 | 266440 |

The additional one-layer to four-layer mappings can be supported with five new TBS, as given in Table 4.

Table 4 TBS mapping table for one-layer to four-layer

| TBS_L1 | TBS_L4 |
| --- | --- |
| 76208 | 305976 |
| 78704 | 314888 |
| 81176 | 327000 |
| 84760 | 339112 |
| 87936 | 354936 |

With the TBS table design given in Table 1, Table 2, Table 3 and Table 4, there are 5 new TBS added to the existing 229 TBS, i.e., 234 TBS in total to support all modulations up to 256QAM. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Observation:

- The proposed TBS table design need 5 new TBS added to the existing 229 TBS. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Based on above discussion, the new TBS table has a minimum specification impacts, with a good support for 256QAM. In the new TBS table, all entries of Rel-11 TBS are kept and a few TBS rows are added containing TBS to support 256QAM. All the TBS in the new rows are the existing TBS from Rel-11 TBS table. As for the multi-layer TBS translation tables, similarly, the existing TBS are used for one-layer to two-layer translation, and one-layer to three-layer translation table without introducing new TBS. As for one-layer to four-layer TBS translation table, only five new TBS are introduced to support higher peak rates with 256QAM. With this design, the new TBS table can be stored with 8-bit indices, which reduce the storage requirement and have minimum impact on implementation/cost. Therefore, we propose to introduce the proposed TBS table for 256QAM support in Rel-12.

Proposal:

- Introduce the proposed TBS table for 256QAM support in Rel-12.

# Conclusions

In this contribution, we propose a TBS table for 256QAM support in Rel-12, and made the following observations and proposals:

Observations:

- By allowing the actual code rate to deviate from the target code rate by ±2%, there are in total 229 unique TBS defined in Rel-11.

- By allowing up to ±2% deviation from the target code rate, the new TBS table can be designed without introducing any new transport block sizes for 256QAM one spatial layer support in Rel-12.

- The proposed TBS table design need 5 new TBS added to the existing 229 TBS. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Proposals:

- Introduce the proposed TBS table for 256QAM support in Rel-12.

# References

RP-132073, “WID Small cell enhancements-PHY”, Huawei

R1-140753, “CQI table design for 256QAM”, Ericsson

R1-140754, “MCS table design for 256QAM”, Ericsson

3GPP TS 36.213