Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #76 v1.0.0

(Prague, Czech Rep., 10th – 14th February 2014)

Document for:	Approval

Table of contents

1	Opening of the meeting	5

1.1	Call for IPR	5

1.2	Network usage conditions	5

2	Approval of Agenda	5

3	Approval of Minutes from previous meeting	5

4	Highlights from RAN Plenary	6

5	Incoming Liaison Statements	6

5.1	Incoming LS received during the week	10

6	UTRA	10

6.1	Maintenance of UTRA Releases 4 – 11	11

6.1.1	FDD	11

6.1.2	TDD	11

6.2	DCH Enhancements	12

6.2.1	DL transport channel processing and multiplexing	12

6.2.2	DL control channel design	12

6.2.2.1	DL DPCCH slot format optimization	12

6.2.2.2	DL ACK indication for UL frame early termination	12

6.2.3	UL transport channel processing and multiplexing	13

6.2.4	UL control channel design	13

6.2.4.1	UL DPCCH slot format optimization	13

6.2.4.2	UL ACK indication for DL frame early termination	13

6.2.5	Other	14

6.3	UMTS Heterogeneous Networks Enhancements	14

6.3.1	Reliability of HS-DPCCH and other uplink channels	14

6.3.2	Mitigating interference from not SHO UEs	15

6.3.3	E-DCH decoupling	15

6.3.4	Network assisted interference cancellation	16

6.3.4.1	NAIC UE reference architecture	16

6.3.4.2	Signalling to enable NAIC and impact to cell capacity	16

6.3.5	Other	17

6.4	Further EUL Enhancements	17

6.4.1	Enabling high user bitrates in a mixed-traffic scenario	18

6.4.1.1	DTX/DRX enhancements	18

6.4.1.2	Improved granting	18

6.4.1.3	Improved power control	19

6.4.2	HS-DPCCH overhead reduction	19

6.5	Enhanced Broadcast of System Information	20

6.6	Study on Scalable Bandwidth UMTS by Filtering	20

6.6.1	Evaluation for standalone scenario	20

6.6.2	Evaluation for multi-carrier scenario	22

6.6.3	Other	24

6.7	Other	25

7	E-UTRA	25

7.1	Maintenance of E-UTRA Releases 8 – 11	26

7.2	LTE Release 12	31

7.2.1	Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation	31

7.2.1.1	Remaining details on UL power control	31

7.2.1.2	Remaining details on CSI feedback	34

7.2.1.3	Remaining details of signalling for TDD UL-DL reconfiguration	35

7.2.1.4	Remaining HARQ details	38

7.2.1.5	Other	40

7.2.2	Low Cost & Enhanced Coverage MTC UE for LTE	40

7.2.2.1	Remaining details of new UE category/type	40

7.2.2.2	Techniques for coverage improvement	42

7.2.2.2.1	PBCH	42

7.2.2.2.2	PRACH	42

7.2.2.2.3	(E)PDCCH/PUCCH	43

7.2.2.2.4	PDSCH/PUSCH	44

7.2.2.2.5	SS	45

7.2.2.2.6	Other	45

7.2.3	LTE TDD-FDD Joint Operation including Carrier Aggregation	45

7.2.3.1	TDD – FDD carrier aggregation	45

7.2.4	Small Cell Enhancements – Physical-layer Aspects	52

7.2.4.1	Higher order modulation	52

7.2.4.2	Small cell on/off and discovery	56

7.2.4.2.1	Procedures for small cell on/off transition time reduction	56

7.2.4.2.2	Discovery signal(s) and measurements	60

7.2.4.3	Radio-interface based synchronization	63

7.2.4.4	Other	64

7.2.5	Physical layer functionalities required for operation of Dual Connectivity	64

7.2.6	Inter-eNB CoMP for LTE	68

7.2.7	Study on 3D-channel model for Elevation Beamforming and FD-MIMO	71

7.2.7.1	Remaining details of fast fading modelling for 3D-UMa and 3D-UMi	71

7.2.7.2	Remaining simulation assumptions for channel model calibration / baseline	72

7.2.7.3	Phase 1 and Phase 2 calibration results	73

7.2.7.4	Initial baseline results	73

7.2.7.5	Details of 3D UMa with one high rise per sector with 300m ISD	74

7.2.7.6	Other	75

7.2.8	Study on LTE Device to Device Proximity Services	75

7.2.8.1	D2D communication	75

7.2.8.1.1	Synchronization design	75

7.2.8.1.2	D2D data communication physical channel design details	78

7.2.8.1.3	Resource allocation	79

7.2.8.1.4	Other	83

7.2.8.2	D2D discovery	83

7.2.8.2.1	Discovery signal and message design details	83

7.2.8.2.2	Resource allocation	87

7.2.8.2.3	Other	91

7.2.8.3	D2D transmission timing	91

7.2.8.4	Other	92

7.2.9	Study on Network-Assisted Interference Cancellation and Suppression for LTE	93

7.2.9.1	System level evaluation results	93

7.2.9.2	Network assistance signalling – feasibility and performance impact	94

7.2.9.3	Network coordination – feasibility and performance impact	95

7.2.9.4	Other	96

8	Closing of the meeting	98

Annex A:	List of Tdocs at RAN1 #76	99

Annex B:	List of CRs agreed at RAN1 #76	100

Annex C-1:	List of Outgoing LSs from RAN1 #76	101

Annex C-2:	List of Incoming LSs from RAN1 #76	102

Annex D:	List of Approved updated WIDs	104

Annex E:	List of draft TSs/TRs agreed at RAN1 #76	105

Annex F:	List of actions	106

Annex G:	List of participants at RAN1 #76	110

Annex H:	TSG RAN WG1 meetings in 2014 - 2015	111

Main facts summary

3GPP TSG WG RAN1 #76 meeting, hosted by the European Friends of 3GPP took place at the CLARION CONGRESS HOTEL in PRAGUE.

The meeting started at 9:00 on Monday 10th February and finished at 17:05 on Friday 14th February 2014.

The number of attending delegates, having signed the participants’ paper list, was 270.

The week was scheduled as follows:

- Monday: Common session on Agenda items 1, 2, 3, 4, 5, 7.1 (E-UTRA maintenance Releases up to 11) followed by LTE D2D Proximity Services (AI 7.2.8).

- Tuesday (LTE): Parallel sessions on

  - LTE TDD-FDD Joint Operation (AI 7.2.3), Physical layer functionalities required for operation of Dual Connectivity (AI 7.2.5), Small Cell Enhancements (AI 7.2.4) and LTE D2D Proximity Services (AI 7.2.8) chaired by Satoshi Nagata on one hand.

  - Low Cost & Enhanced Coverage MTC UE for LTE (AI 7.2.2) chaired by Wanshi Chen, followed by Inter-eNB CoMP for LTE (AI 7.2.6) chaired by Matthew Baker, followed by Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) chaired by Wanshi Chen on the other hand.

- Tuesday (HSPA): Maintenance of UTRA Releases 4 – 11 (AI 5.1), followed by Further EUL Enhancements (AI 6.4) chaired by Karri Ranta-aho from NSN and UMTS Heterogeneous Networks (AI 6.3) chaired by Carmela Cozzo from Huawei.

- Wednesday (LTE): Parallel sessions on

  - LTE D2D Proximity Services (AI 7.2.8) chaired by Matthew Baker followed by Small Cell Enhancements (AI 7.2.4) and chaired by Satoshi Nagata on one hand.

  - One hour session focused on new UE category folloed by Study on NAICS (AI 7.2.9) chaired by Satoshi Nagata followed by Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) chaired by Wanshi Chen on the other hand.

- Wednesday (HSPA): Continued on UMTS Heterogeneous Networks (AI 6.3) chaired by Carmela Cozzo from Huawei followed by Study on Scalable UMTS by Filtering (AI 6.6) chaired by Matthew Baker.

- Thursday (LTE): Parallel sessions on

  - Small Cell Enhancements (AI 7.2.4), LTE TDD-FDD Joint Operation (AI 7.2.3), Physical layer functionalities required for operation of Dual Connectivity (AI 7.2.5), and Small Cell Enhancements (AI 7.2.4) chaired by Satoshi Nagata on one hand.

  - Study on 3D-channel model for Elevation Beamforming and FD-MIMO (AI 7.2.7), Low Cost & Enhanced Coverage MTC UE for LTE (AI 7.2.2) chaired by Wanshi Chen, followed by LTE Device to Device Proximity Services ( AI 7.2.8), Inter-eNB CoMP for LTE (AI 7.2.6) chaired by Matthew Baker, followed by Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) chaired by Wanshi Chen on the other hand.

- Thursday (HSPA): DCH Enhancements (AI 6.2) chaired by Karri Ranta-aho from NSN, followed by Enhanced Broadcast of System Information (AI 6.5) chaired by Carmela Cozzo from Huawei and continued on Scalable UMTS by Filtering (AI 6.6) chaired by Matthew Baker.

- Friday morning: Parallel sessions on

  - LTE TDD-FDD Joint Operation (AI 7.2.3) and LTE D2D Proximity Services (AI 7.2.8) chaired by Satoshi Nagata on one hand.

  - Scalable UMTS by Filtering (AI 6.6) chaired by Matthew Baker, followed by Low Cost & Enhanced Coverage MTC UE for LTE (AI 7.2.2) and Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation (AI 7.2.1) chaired by Wanshi Chen on the other hand.

- Friday afternoon: Revisions

Before closing the meeting, 3GPP Excellence Awards granted to Robert Love from Motorola Mobility.

The list of action points that required RAN1 close follow-up is listed in Annex F (end of document).

The number of contribution documents for this meeting was 1020, and those documents were categorized as followed.

| Agenda Item | Input Document | Discussed Document |
| --- | --- | --- |
| AI 5 to 7.2.9 | 1017 | 409 |

Note: The amount of documents includes those discussed during the email discussion session post meeting.

# Opening of the meeting

Mr Satoshi Nagata (RAN1 Chairman) welcomed the participants of the 76th RAN WG1 meeting and opened the meeting at 09:00.

Mr Yuichi Morioka from Sony welcomed the delegates on behalf of the host, the European Friends of 3GPP, Alcatel-Lucent, BlackBerry, Deutsche Telekom, Ericsson, Huawei, Intel, Motorola, Nokia, NSN, Orange, Qualcomm, SIMAlliance, Sony, TeliaSonera, Telefonica, Telecom Italia, Telenor, and Vodafone and detailed the domestic arrangements for the full week.

## Call for IPR

The Chairman drew the attention to Members’ obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

The attention of the members of this Technical Specification Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The members take note that they are hereby invited: - to investigate in their company whether their company does own IPRs which are, or are likely to become Essential in respect of the work of the Technical Specification Group. - to notify the Director-General, or the Chairman of their respective Organizational Partners, of all potential IPRs that their company may own, by means of the IPR Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

# Approval of Agenda

| R1-140000 | Draft Agenda for RAN1#76 meeting | RAN1 Chairman |  |
| --- | --- | --- | --- |

Satoshi Nagata (Chairman) proposed the agenda for the meeting, as well the schedule of the week.

Decision: The agenda is approved.

# Approval of Minutes from previous meeting

| R1-140001 | Final Report of RAN1#75 meeting | MCC |  |
| --- | --- | --- | --- |

The document was presented by Patrick Mérias (ETSI Mobile Competence Center) and provides the outcomes of last meeting in San Francisco.

Discussion: MCC reported the recent common practice to provide a copy of meeting document area from the local meeting server to the public server, in folder http://www.3gpp.org/ftp/Meetings_3GPP_SYNC/

Decision: The agenda is approved.

# Highlights from RAN Plenary

| R1-140002 | Highlights from RAN#62 | RAN1 Chairman |  |
| --- | --- | --- | --- |

The document was presented by Satoshi Nagata (Chairman) and provides the outcomes of last plenary meeting relevant to RAN1.

Discussion: MCC commented that TR36.843 on D2D will go for one step approval at next plenary meeting, resulting from the situation that RAN1 missed to present v1.0.0 in December.

Decision: The document is noted.

# Incoming Liaison Statements

D2D related

| R1-140004 | LS on text proposal for TR 36.843 on D2D | RAN2, Qualcomm | = R2-134590 |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and asks RAN1 to incorporate content of R2-134589 into TR 36.843.

Decision: The document is noted. It is agreed to include RAN2 agreements into TR36.843 – rapporteur (Qualcomm)

| R1-140008 | LS response on D2D Typical UE in-band Emissions | RAN4, Ericsson | = R4-137102 |
| --- | --- | --- | --- |

The document was presented by Ms Iana Siomina from Ericsson and recommends, for single cluster transmissions, that RAN1 assess the sensitivity of the D2D performance to the inband emission levels over the following 2 sets of levels for the offset parameters W, X, Y and Z [dB]: {W, X, Y, Z} = {0, 0, 0, 0}, {3, 6, 3, 3}

With respect to multi-cluster inband emissions, RAN4 has not agreed on a typical model.

Discussion: LGE  have we align simulation assumptions according to this LS.

Decision: The document is noted. It shall be treated in D2D session.

| R1-140010 | Reply to LS on notifying for information of including Annex about Public Safety ProSe Communication in TR 36.843 | SA1, Telecom Italia | = S1-135308 |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia and reminds that SA1 use cases and related requirements, have been already defined and agreed in TS 22.278 and have to be properly taken into account into stage 2 and stage 3 development.

Decision: The document is noted.

| R1-140013 | LS on discovery message size | RAN2, Qualcomm | = R2-134591 |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and informs RAN1 that RAN2 seeks information from SA2 about what is the expected overall size of discovery message, i.e. the expected size of ProSe Identities and size of any other expected fields to be transferred from upper layer. In addition, RAN2 does not intend to discuss latency.

Discussion: No action to RAN1.

CATT: can latency be clarified as RAN2 did not answer the topic?

Decision: The document is noted.

| R1-140185 | LS reply on discovery message size | SA2, Alcatel Lucent, Intel | = S2-140568 |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and states that SA2 has concluded that for public safety use cases there could be a difference in size with the non-public safety open discovery use case, mainly due to the judgment that some companies voiced that public safety requires fewer classification criteria and reluctance to agree to higher bits allocation.

Additionally, restricted and open discovery may lead to different message sizes also, in Public safety use cases.

Discussion: No need to change message size in RAN1, according to ALU. LGE believes that assumptions need to be reviewed.

Decision: The document is noted.

| R1-140186 | Reply to LS on discovery message size | SA3, Qualcomm | = S3-140226 |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and states that SA3 has not yet concluded on the scope and size of the security parameter(s) needed.

Discussion: No action to RAN1.

Decision: The document is noted. The above three contributions shall be further treated in D2D session

Wednesday 12th: Huawei will prepare a draft LS to at least RAN2 for further discussion and to aid the decision on whether an LS might be sent - R1-140942.

Friday 14th: No consensus

| R1-140015 | Reply LS on ProSe Lawful Interception | RAN, Telecom Italia | = RP-132107 |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia and shows RAN’s reply asking SA3-LI, SA3 and SA2 to provide a description of the aspects with potential impact to RAN of the solutions being developed which enable lawful interception and data retention, in order to allow RAN to assess the impact over E-UTRAN.

Decision: The document is noted.

| R1-140182 | Reply LS on ProSe Lawful Interception | SA2, Qualcomm | = S2-140494 |
| --- | --- | --- | --- |

The document was presented by Saurabh Tavildar from Qualcomm and informs RAN that SA3-LI is responsible to decide the requirements for Lawful Interception on ProSe.

Discussion: No action to RAN1.

Decision: The document is noted.

| R1-140181 | Reply LS on D2D proximity criteria | SA1, Telecom Italia | = S1-140319 |
| --- | --- | --- | --- |

The document was presented by Enrico Buracchini from Telecom Italia and states that SA1 has identified that only the operator is capable of controlling proximity criteria.

Discussion: Telecom Italia suggested to update the TR with this assumption on proximity criteria.

Decision: The document is noted. Offline discussions to find the wording for inclusion to TR (Telecom Italia, Qualcomm)

Wednesday 12th:

| R1-140936 | TP on D2D proximity criteria after LS exchange with SA1 | Telecom Italia, [Qualcomm] |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed for inclusion in next update of TR36.843

| R1-140184 | LS on ProSe Out of Coverage discovery | SA2, Deutsche Telekom | = S2-140560 |
| --- | --- | --- | --- |

The document was presented by Georg Wannemacher from DT and requests the specification of a solution for ProSe Discovery out of network coverage to be part of Rel-12.

Decision: The document is noted.

| R1-140647 | Discussion on ProSe out of coverage discovery | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Khoryaev from Intel and proposes that Model B should be studied by RAN WGs. The expected impact on the work in RAN1 and RAN2 is as follows:

- RAN1: Need to define evaluation methodology for Model B discovery.

- RAN2: Definition of protocol functionality for Model-B support.

Given that SA2 has agreed to support both discovery models (Model A and Model B), it is a good approach to consider both models for out of coverage scenario.

Discussion: Huawei  asked clarifying the impact to RAN1.

ALU  specifying Model B is likely not needed – Intel suggested that Model B is not precluded.

Decision: The document is noted.

| R1-140728 | Draft LS reply on ProSe Out of Coverage discovery | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Ms Iana Siomina from Ericsson and states that, if out of coverage direct discovery is re-prioritized, only one of the two topics (direct discovery or broadcast direct communication) can be completed within the SI before RAN#63.

Discussion: Broadcom  it is up to RAN to prioritize our work.

Decision: The document is noted.

Continue offline/online discussions by exploiting D2D discovery session (Ericsson, Intel)

Wednesday 12th:

| R1-140935 | [Draft] Reply LS on ProSe Out of Coverage discovery | Ericsson |  |
| --- | --- | --- | --- |

“RAN1 asks RAN plenary to reply to SA2 on behalf of RAN1.”

Decision: The document is noted. Ericsson to prepare an LS accordingly in R1-140943.

Discuss offline what else would be required to support out-of-coverage discovery.

Thursday 13th

| R1-140943 | [Draft] Reply LS on ProSe Out of Coverage discovery | Ericsson | (R1-140935) |
| --- | --- | --- | --- |

Modify 943 as follows:

1. Overall Description:

RAN1 kindly asks RAN plenary to reply to SA2 on behalf of RAN1.

2. Actions:

To TSG RAN:

RAN1 kindly asks RAN to respond to SA2.

Decision: The document is noted and final LS is agreed in R1-140950.

| R1-140017 | LS on parameter synchronization | SA3, Qualcomm | = S3-131152 |
| --- | --- | --- | --- |

The document was presented by Saurabh Tavildar from Qualcomm and raises the following questions to RAN1:

- Can it be assumed that all UEs within discovery range of a given UE have alignment – within a known offset if necessary – of a parameter (e.g. time, counter, etc.) whose value changes for all such UEs?

- What are the parameters of this value: e.g. how many bits long it is, how often does it change?

- If the value rolls over, can it be assumed that all UEs can keep track of a second value with similar properties, with which the aforementioned value can be augmented so as to preserve the alignment amongst these UEs.

Decision: The document is noted.

| R1-140457 | Discussion on LS on parameter synchronization | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Saurabh Tavildar from Qualcomm and notes that such system counter, if defined, would provide a common value to all UEs that receive this broadcast message. A draft LS based on the analysis above is provided in [R1-140432].

Decision: The document is noted.

| R1-140432 | Draft response LS on parameter synchronization | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Discussion: ALU better to wait until D2D session.

Decision: The document is noted.

Continue offline/online discussions by exploiting D2D session (Qualcomm)

Wednesday 12th:

| R1-140940 | Draft response LS on parameter synchronization | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted and final LS is agreed in R1-140941 with the following changes:

yet to be agreed in RAN1 or RAN2

Alternatively,

| R1-140005 | Reply LS on mobility support for Low Complexity MTC UEs and MTC coverage enhancement | RAN2, Vodafone | = R2-134596 |
| --- | --- | --- | --- |

The document was presented by Eric Bouton from Vodafone and states that, since accepting a 1000 bit limit might put restrictions on the extensibility of these SIBs in the future, RAN1 is asked to consider keeping the current limit of 2216 bits for the BCCH TBS size within the work on Low complexity MTC UEs.

Discussion: Vodafone planned some off line discussion on this topic during the day.

Decision: The document is noted. Offline/online discussions to consider the current limit of 2216 bits for the BCCH TBS size within the work on Low cost MTC UEs – (Vodafone)

Friday 14th: See AI 7.2.2.1

Random Access in dual connectivity

| R1-140006 | LS on Random Access in dual connectivity | RAN2, NEC | = R2-134603 |
| --- | --- | --- | --- |

The document was presented by Kevin Lin from NEC and informs that:

- RAN2 intends to support Contention Based Random Access for the Secondary eNB

- RAN2 agreed that the UE receives Msg2 from the eNB to which the preamble was sent.

- With regarding to parallel Random Access procedures, one for Master eNB and the other for Secondary eNB, RAN2 agreed that it is supported if the two preamble transmissions are not overlapping. However RAN2 has not concluded for the other case, i.e. when the two preamble transmissions are overlapping. RAN2 would like to ask RAN1 whether it is feasible to support parallel preamble transmissions, one for Master eNB RACH and the other for Secondary eNB RACH.

- At least one cell in Secondary eNB has configured UL and one of them is configured with PUCCH resources. The cell in the Secondary eNB which is configured with PUCCH resources cannot be cross-carrier scheduled.

RAN1 to answer whether it is feasible to support parallel preamble transmissions, one for MeNB RACH and the other for SeNB RACH.

Decision: The document is noted.

| R1-140166 | Discussion on parallel PRACH for dual connectivity | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Ms Sigen Ye from ALU and deals with the specification impact of supporting parallel preamble transmission, and it is proposed not to parallel preamble transmissions for dual connectivity.

Discussion: Samsung  better to wait until the online session as there are a number of contributions to be taken into account.

Decision: The document is noted.

Continue offline/online discussions by exploiting Dual Connectivity session

****

Thursday 13th

| R1-140294 | Discussion on random access in dual connectivity | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Ms Yunjung Yi from LGE and deals with specification impact to support parallel preamble transmissions. In general, it is supportable to transmit parallel PRACH transmission in non-power limitation cases for dual connectivity. When power limitation case occurs, PRACH on PCell can be prioritized and other PRACH can be dropped. Not to incur performance degradation, some coordination among MeNB and SeNB to avoid successive PRACH collisions and handling of dropped PRACH would be desirable.

Decision: The document is noted.

| R1-140295 | Draft LS response on random access in dual connectivity | LG Electronics |  |
| --- | --- | --- | --- |

| R1-140479 | Discussion on parallel PRACH preamble transmissions for dual-connectivity | NEC |  |
| --- | --- | --- | --- |

The document was presented by Kevin Lin from NEC and draws the following observations:

- Observation 1: It is a very rare case for a UE has to send parallel preambles (one for each eNB) in dual connectivity. Both PRACH configurations will need to have overlapping system frame number, subframe number, and PDCCH orders are sent in the same subframe or UE selected random backoff timing after expiration of RA response window coincide to the same UL subframe(s).

- Observation 2: RAN1 specification effort seems significant in order to support the rare case of parallel preamble transmission. A few different approaches are foreseen and each could have different specification impacts. Further studies are needed if decide to support parallel preamble transmissions in dual connectivity.

- Observation 3: Depending on the option selected to avoid parallel preamble transmission, specification impact may involve different RAN WGs. Overall, in our opinion, the required changes are not significant.

Discussion: .

Decision: The document is noted.

| R1-140890 | Draft LS response on Random Access in dual connectivity | NEC |  |
| --- | --- | --- | --- |

Inform RAN2 of the following agreements.

- It is feasible to support parallel PRACH preamble transmissions in non-power-limited case.

- Only one PRACH preamble transmission is allowed at any point in time in power-limited case.

Decision: The document is noted and final LS is agreed in R1-141007 by adding following points and adding contact person

- FFS: Whether or not and how UE support parallel PRACH transmission in the power limited case

- RAN1 is discussing FFS part

****

| R1-140007 | 3GPP RAN WG4 input to update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | RAN4, Telecom Italia | = R4-137081 |
| --- | --- | --- | --- |

Decision: The document is just noted for information.

| R1-140009 | Reply LS on Synchronization aspects in LTE-TDD networks | RAN4, Huawei, NSN | = R4-137156 |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei and shows RAN4’s answers to ECC PT1 on the status of currently developing report on synchronization for TDD networks in a multi-operator context.

Discussion: No action to RAN1.

Decision: The document is noted.

On GCSE with eMBMS

| R1-140011 | Further questions on GCSE with eMBMS | SA2, Vodafone | = S2-134580 |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and asks RAN1 to provide views on a) the implementation of UEs for Public Safety users that (in conjunction with transmitting and receiving unicast data) can receive multiple MBMS services and/or multiple MBSFN areas at the same time, and b) the relative power consumption of the UE in different cases listed in the LS.

Decision: The document is noted.

| R1-140016 | Response to LS on Further questions on GCSE with eMBMS | RAN, Vodafone | = RP-132109 |
| --- | --- | --- | --- |

The document was presented by Tim Frost from Vodafone and provides RAN feedback to SA2 GCSE work.

Discussion: No action to RAN1.

Decision: The document is noted.

| R1-140014 | Response LS on GCSE with eMBMS | RAN2, Alcatel Lucent | = R2-134594 |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and provides RAN2 feedback to SA2 GCSE work.

Discussion: No action to RAN1.

Decision: The document is noted.

| R1-140003 | Reply LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | RAN2, Samsung | = R2-134486 |
| --- | --- | --- | --- |

The document was presented by Youngbum Kim from Samsung and informs SA4 that an MBMS capable UE is only required to support reception of a single MBMS bearer service at a time i.e. reception of more than one MBMS bearer service up to UE implementation.

Discussion: No action to RAN1.

Decision: The document is noted.

| R1-140183 | Reply LS on the UE capabilities regarding support of simultaneous MBMS bearers | SA2, Samsung | = S2-140511 |
| --- | --- | --- | --- |

The document was presented by Youngbum Kim from Samsung and confirms that SA2 does not see any architectural limitations for supporting multiple eMBMS bearers simultaneously, i.e. any restrictions would be due to RAN aspects.

Discussion: No action to RAN1.

Decision: The document is noted.

| R1-140012 | Reply LS on FDD-TDD switching enhancement in FDD-TDD joint operation | RAN2, CMCC | = R2-134582 |
| --- | --- | --- | --- |

The document was presented by Xiaodong Shen from CMCC and informs that in Small Cell Enhancement--higher layer study item, dual-connectivity has been considered as a potential solution to address the user throughput improvement, mobility robustness and signalling load challenges, assuming that the involved cells may operate either in the same duplex scheme (i.e. FDD only or TDD only) or in the different duplex schemes (FDD/TDD) with regard to the deployment scenarios.

If 1 UL, 1 DL (i.e., same as the current dual mode UEs) as mentioned in RAN1 LS is required, it might not be the case that such use case / scenario / UE requirement can be satisfied by the current dual-connectivity solution as being developed.

From specification point of view, it is possible for a Rel-8 UE with the support of TDD/FDD handover to perform a handover in tens milliseconds level irrespective of the FDD/TDD modes. The use case/scenario/UE requirement identified by RAN1 can be satisfied by existing handover mechanism.  RAN2 thinks that lack of IOT of mandatory Rel-8 functionality should not be used as justification for introducing a new Rel-12 feature.

Decision: The document is noted. Use case / scenario identified in R1-134967 was discussed, and no more RAN1 work is needed in Rel-12 TDD-FDD joint operation WI.

## Incoming LS received during the week

| R1-141050 | LS on Text Proposal for TR 36.843 | RAN3, Qualcomm | = R3-140490 |
| --- | --- | --- | --- |
| R1-141051 | LS on Text Proposal for TR 36.843 on D2D | RAN2, Qualcomm | = R2-141009 |
| R1-141053 | Reply LS on Multiple Access Scheme for D2D | RAN4, Qualcomm | = R4-140969 |
| R1-141054 | Reply LS on AGC and Frequency Error for D2D | RAN4, Qualcomm, Ericsson | = R4-141241 |

Above LSs were received after the closing of RAN1#76. As they may affect the content of TR36.843.on D2D, these LSs are included in email discussion [76-19] for approval by RAN1 and if agreed, further inclusion in the next version of the TR.

Decision: Both documents are noted.

# UTRA

| R1-140907 | Chairperson's summary of the UMTS sessions | UMTS Session Chairpersons (NSN, Huawei, Alcatel-Lucent) |  |
| --- | --- | --- | --- |

The document was presented by the different chairpersons and reports the outcomes of the relevant HSPA sessions.

Decision: The document is endorsed and incorporated below, including the following updates.

## Maintenance of UTRA Releases 4 – 11

### FDD

| R1-140835 | 25.212 CR0322 (Rel-7, F) Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | NSN |  |
| --- | --- | --- | --- |

| Reason for change: | The Second or third transmission bit is defined to be Xsec,3 in section 4.6A.2.2.1, but the polarity of the bit is undefined. However, section 4.6A.2.2.1.3 “Second or third transmission mapping” defines the polarity for Xsec,1. The definitions Xsec,1 and Xsec,3 are not used elsewhere and it is evident that section 4.6A.2.2.1 should use the Xsec,1 defined in 4.6A.2.2.1.3 |
| --- | --- |
| Summary of change: | Changing Xsec,3 to Xsec,1 in section 4.6A.2.2.1 |
| Consequences if not approved: | The special information mapping definition uses a bit of undefined polarity. |

Decision: The document is noted and CR is agreed together with the correspondent shadow CRs to Rel-8 through Rel-11.

| R1-140900 | 25.212 CR0323 (Rel-8, A) Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | NSN |  |
| --- | --- | --- | --- |
| R1-140901 | 25.212 CR0324 (Rel-9, A) Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | NSN |  |
| R1-140902 | 25.212 CR0325 (Rel-10, A) Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | NSN |  |
| R1-140903 | 25.212 CR0326 (Rel-11, A) Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | NSN |  |

| R1-140678 | 25.214 CR0711 (Rel-10, F) Clarification on (de-)activation status of secondary carriers when higher layer indicates a change of HS-DSCH cells | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

| Reason for change: | Upon RRC reconfiguration indicates a change of Secondary_Cell_Enabled from a value not equal to 0 to a value not equal to 0 and does not indicate a change of the serving HS-DSCH cell, the initial activation and deactivation status of a configured secondary serving HS-DSCH cell is not specified if it is not configured as a secondary serving HS-DSCH cell prior to RRC reconfiguration. |
| --- | --- |
| Summary of change: | The following behavior is specified: Upon RRC reconfiguration indicates a change of Secondary_Cell_Enabled from a value not equal to 0 to a value not equal to 0 and does not indicate a change of the serving HS-DSCH cell, a configured secondary serving HS-DSCH cell shall be activated if it is not configured as a secondary serving HS-DSCH cell prior to RRC reconfiguration. |
| Consequences if not approved: | The UE behavior and network behavior are not clear. There might be misunderstanding on the initial activation and deactivation status of a configured secondary serving HS-DSCH cell between UE and NodeB. |

Decision: The document is noted.

| R1-140679 | 25.214 CR0712 (Rel-11, A) Clarification on (de-)activation status of secondary carriers when higher layer indicates a change of HS-DSCH cells | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Conclusion: RAN1 agrees that the initial activation status of a secondary HS-DSCH cell is always active. RAN1 will attempt to complete the 25.214 text by email reviewing the CRs to 25.214 with the understanding that RAN2 is also discussing the topic, and that RAN2 conclusions may render RAN1 CRs obsolete.

Email review until Friday, 21st of February – (Huawei)

### TDD

˂ No Contributions ˃

## DCH Enhancements

WID in RP-131357. The enhancements are intended to target AMR voice over DCH, and/or SRB over DCH (where applicable.)

### DL transport channel processing and multiplexing

| R1-140250 | Design of DL Transport Channel Processing and Multiplexing | MediaTek Inc. |  |
| --- | --- | --- | --- |

- Proposal: Agree the proposed downlink encoding chain “interleave-repeat” method shall be adopted to be standardized for DCH enhancements.

Decision: The document is noted.

| R1-140251 | DL DCH Enhancement Solution Performance Comparison | MediaTek Inc. |  |
| --- | --- | --- | --- |

- Observation 1: It reduces DL link gain around 0.6dB in order to transmit DL AI for UL FET.

Decision: The document is noted.

| R1-140700 | Analysis of Downlink Rate-Matching and Interleaving Enhancements | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal 1: For voice over downlink DCH Enhancements, the R99 rate-matching shall be used, except that the RM attributes of the transport channel carrying DCCH shall be treated as zero whenever that transport channel does not deliver a DCCH block (i.e., pseudo-flexible rate-matching).

Decision: The document is noted.

From RAN1#75 chairman’s notes:

For downlink:

- Option 1: Concatenate DTCH and DCCH and process both as a single block, rate match by truncating or repeating bits

- Option 2: Process DTCH and DCCH separately, when DCCH is absent consider its rate matching attribute to be zero and rate match DTCH to fill all the bits on DPCH

Final decision to be taken in RAN1#76.

Option 1 refers to inter-leave and repeat as proposed in 0250.

Option 2 refers to pseudo rate-matching as proposed in 0700.

- As agreed in RAN1#74bis, Option 1 0.1 dB in link efficiency gain, and an average decoding time gain up to 1 slot over option 2.

- Option 2 preserves the Rel’99 TrCH processing below CRC attachment, whereas option 1 requires modification on rate matching.

Conclusion: The RAN1 HSPA ad-hoc session in RAN1#76 was not able to reach consensus on which of the two options to select.

Friday 14th

| R1-141040 | Way Forward on Aspects of DCH Enhancements | Qualcomm, MediaTek, Huawei, HiSilicon, ZTE, Mstar, ITRI |  |
| --- | --- | --- | --- |

The document was presented by Arjun Bharadwaj from Qualcomm.

  - DL DCH design

    - Option 2, referred to as “pseudo rate-matching” as proposed in R1-140700, is adopted for DCH Enhancements Work Item.

Decision: The document is noted and above DL DCH design is agreed.

### DL control channel design

#### DL DPCCH slot format optimization

˂ No Contributions ˃

#### DL ACK indication for UL frame early termination

| R1-140701 | Performance of DL ACK signalling Mechanisms for UL FET | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal: Study design of Ack channel sent on DL supporting UL FET to improve its performance in soft handover.

Decision: The document is noted.

### UL transport channel processing and multiplexing

| R1-140252 | Uplink Design for DCH Enhancements | MediaTek Inc. |  |
| --- | --- | --- | --- |

Proposal 1: UL speech frame compression method (UL FET-Less design) shall be adopted to be standardized for DCH enhancements.

Decision: The document is noted.

| R1-140254 | System Performance Comparison of DCH Enhancement Solutions | MediaTek Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140702 | Comparison of UL DCH channel without FET and with reduced FET opportunity | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

In the no FET scenario:

- UL decides on using 10ms TTI or 20ms TTI prior to transmission of new TTI based on a threshold on UPH (UE power headroom).

- UE signals choice of TTI to NodeB via TFCI information.

Decision: The document is noted.

Working assumption:

- Adopt the UL FET-less operation, where UE transmits the voice frame over one or two radio frames

  - The switch between 10 and 20 ms transmission could be e.g. based on UE power headroom

  - DL FET-ACK is not needed

  - The 20 ms transmission interleaves the TB over the full 20 ms TTI according to Rel’99 design

Consider further:

- In addition to the UL FET-less operation, consider a mode where the FET is used for the uplink

  - It may be possible to configure the UE to monitor FET-ACK in DL in limited number of DL slots in order to reduce the impact of false-ACK

  - DL FET-ACK channel design is FFS

| R1-140253 | Uplink Compressed Mode Considerations | MediaTek Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observation: Enhanced DCH can be designed to work with compressed mode. Detailed solutions invited for RAN1#76bis.

### UL control channel design

#### UL DPCCH slot format optimization

˂ No Contributions ˃

#### UL ACK indication for DL frame early termination

| R1-140252 | Uplink Design for DCH Enhancements | MediaTek Inc. |  |
| --- | --- | --- | --- |

Proposal 2: UL ACK indication design (M-DPCCH-S0) shall be adopted to be standardized for enhanced DCH

Decision: The document is noted.

| R1-140703 | UL ACK and TFCI signalling for DL FET | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal: TDM ACK and early TFCI carried over UL DPCCH are adopted for signalling ACK for DL FET.

Decision: The document is noted.

| R1-140783 | Further Consideration on Uplink ACK Channel Design for DL FET | ZTE |  |
| --- | --- | --- | --- |

- Option 1: A new UL control channel, FET-DPCCH, is proposed, which reuses the structure of HS-DPCCH channel. During the first two slots of DTCH TTI the TFCI information is transmitted. Subsequent slots after TFCI is sent are dedicated to transmission of the FET ACK signal.

- Option 2: The UL DPCCH is slightly modified. In the first 7 slots (as an example) within a 20ms TTI the TFCI information is transmitted. The FET ACK indication is transmitted in the remaining slots after TFCI is sent.

- Option 3: In case of 750Hz TPC rate, the spared TPC symbols are used to transmit the FET ACK indication (ETI). In this example, the ETI can be sent every two slots in UL DPCCH from slot 1 to slot 29.

- Proposal 1: Further evaluate the design option 1 and option 2 as potential candidates for FET ACK indication design for DL FET.

Decision: The document is noted.

Agreement: The UL DPCCH-based structure for uplink control design is to be adopted, where

- In the first slots the DPCCH consists of pilot, TPC and TFCI

- In the remaining slots the TFCI bits are replaced with BPSK-encoded ACK/NACK-symbols

Possible agreements

- Npilot = 5 or 6

- Ntfci/Nack/nack = 3 or 2

- Ntpc = 2

Open issues

- The split of symbols between pilots and TFCI/ACK/NACK

- Whether a power boost is allowed for ACK-symbols

- Whether to repeat the FET-ACK bits in several slots for soft combining

Friday 14th

| R1-141040 | Way Forward on Aspects of DCH Enhancements | Qualcomm, MediaTek, Huawei, HiSilicon, ZTE, Mstar, ITRI |  |
| --- | --- | --- | --- |

The document was presented by Arjun Bharadwaj from Qualcomm.

- The following design of UL DPCCH channel for UL ACK indication is adopted for DCH enhancements work item:

    - Slot format for UL DPCCH contains  as proposed in R1-140252:

      - Npilot=6,

      - Ntfci/NACK/ACK=2

      - Ntpc=2

    - TFCI is sent in the first 10 slots as proposed in R1-140252.

    - ACK/NACK is sent by boosting and/or repetition of ACK symbols

Decision: The document is noted and the above UL DPCCH design is agreed.

### Other

| R1-140255 | System Performance Degradation due to Abnormal TPC | MediaTek Inc. |  |
| --- | --- | --- | --- |

Proposal 1: For practical system evaluation, the performance degradation due to abnormal TPC should be taken into account.

Decision: The document is noted.

| R1-140705 | Evaluation of ILPC impact due to erroneous FET and ACK decoding errors in UL | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140704 | OLPC algorithms to guarantee multiple target BLERs | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Observation: The final BLER of 1% after 20ms can be ensured by using a less aggressive BLER target setting at 10ms point. If a more aggressive BLER target setting is desired, then the second OLPC loop is beneficial to guarantee that 1% BLER after 20ms is not exceeded.

Decision: The document is noted.

## UMTS Heterogeneous Networks Enhancements

WID in RP-132074,

### Reliability of HS-DPCCH and other uplink channels

| R1-140140 | Consideration of SHO operation in HetNet | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Proposal 1: Consider increasing the range of βhs values.

Proposal 2: Allow the macro cell to indicate to the UE whether ILPC of a LPN should be ignored.

Proposal 3: Consider implementing a two pilot for ILPC where:

- DPCCH is the primary pilot that is power controlled only by the macro where the power of the HS-DPCCH is based on

- E-DPCCH act as a virtual secondary pilot that is power controlled by the macro and LPN where the powers of the E-DPDCHs are based on.

Decision: The document is noted.

| R1-140680 | On HS-DPCCH reliability and solutions | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Consider SIR target manipulation as the baseline performance for solving the HS-DPCCH issue.

Proposal 2: Consider dynamic power adjustment of control and data channels to enhance the performance of SIR target manipulation.

Decision: The document is noted.

| R1-140681 | Dynamic power adjustments of control and data channels | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Discuss the dynamic power adjustment method which re-interprets the legacy TPC commands.

Decision: The document is noted.

| R1-140706 | Solutions for UL/DL Imbalance | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal 1: Only solutions that require changes to the specifications are discussed in RAN1.

Proposal 2: The schemes to be considered for HS-DPCCH reliability are:

- SIR Manipulation

- Disabling LPN Power Control

- Introduction of a Secondary Pilot

Decision: The document is noted.

Conclusion on HS-DPCCH:

Solutions considered so far:

- Virtual secondary pilot (R1-140140 proposal 3)

- Dynamic rate and power adjustment (R1-140681)

- Dynamic SIR manipulation (R1-140706, 1st solution)

- Secondary pilot (R1-140706)

- Dynamic LPN ILPC restriction (R1-140706, 2nd solution)

Framework for evaluation (link level) is provided in the TR25.800, Section7.1.4.1.

Metrics to evaluate:

- ACK/NACK misdetection (HS-DPCCH); Happy Bit reliability (E-DPCCH)

- Uplink interference at the LPN

- UE Tx power

- Macro F-DPCH power consumption

- Impact to specification

### Mitigating interference from not SHO UEs

| R1-140682 | Mitigating uplink interference from non-SHO UEs | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Use the extended active set operation to identify the non-SHO UE and victim cell without setting up a radio link between them.

Decision: The document is noted.

### E-DCH decoupling

| R1-140142 | E-DCH operation in HetNet | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Proposal 1: E-DCH decoupling should be based on uplink measurements at the LPN rather than downlink measurement such as Event 1A.

Proposal 2: Introduce HS-SCCH order(s) to activate/deactivate E-DCH decoupling.

Decision: The document is noted.

| R1-140683 | Discussion on E-DCH decoupling | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: To adopt the E-DCH decoupling solution to solve the SI reception problem in HetNet deployment.

Proposal 2: To discuss the methods of initiating the E-DCH decoupling operation.

Decision: The document is noted.

| R1-140786 | E-DCH decoupling in HetNet | NSN |  |
| --- | --- | --- | --- |

Proposal 1: Consider presented specification impact analysis for introducing E-DCH decoupling.

Decision: The document is noted.

| R1-140787 | Bursty Traffic SLS Results for E-DCH Decoupling | NSN |  |
| --- | --- | --- | --- |

Proposal: Consider E-DCH decoupling as a basic method for solving SI reception issues.

Decision: The document is noted.

Conclusion on EDCH decoupling:

To adopt the E-DCH decoupling solution

Triggering:

RAN1 method – HS-SCCH order

RAN2 methods (refer to TR25.800)

| R1-140905 | Decoupling of E-DCH and HS-DSCH serving cell in UMTS HetNet | RAN1, NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted and LS is agreed.

### Network assisted interference cancellation

| R1-140806 | Way-Forward on Network Assisted Interference Cancellation | Ericsson, Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal 1: A NAIC solution needs to demonstrate no degradation to the average throughput of Macro legacy UEs as well as improved average throughput of a beneficiary LPN UE.

Proposal 2: Discuss and agree on one UE receiver baseline for assessing NAIC performance gains.

Proposal 3: Merits and solutions for signalling assistance information need to be studied further.

Proposal 4: Study whether the CQI outer-loop, often used to improve the robustness of link adaptation, may be used to reflect NAIC benefits.

Decision: The document is noted.

Conclusion:

Aspects to be addressed for NAIC to assess any potential benefits

System simulations for pre- and post-decoding IC

Evaluate impact on Macro legacy users

Evaluate and possibly address impact of CQI mismatch

Evaluate impact of signalling of NAIC

Evaluate complexity impact of implementing NAIC at UE and Network

Impact on specifications across all the WGs

#### NAIC UE reference architecture

| R1-140708 | Methodology for NAIC System Simulations | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal: The methodology described in this contribution is adopted to conduct system level simulation to evaluate the gains of NAIC.

Decision: The document is noted.

Conclusion:

The methodology in R1-140708 is accepted for system evaluation of NAIC. As baseline, the tables are accepted for the evaluation of post-decoding NAIC.

| R1-140684 | CQI mismatch with NAIC in Hetnet | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: The CQI mismatch issue needs to be resolved to improve the IC performance of LPN cell edge UEs.

Proposal 2: Resolve or mitigate the CQI mismatch issue while introducing NAIC operation.

Proposal 3: Consider RRS pattern based coordinated scheduling as a solution to mitigate the CQI mismatch in HSPA networks.

Decision: The document is noted.

#### Signalling to enable NAIC and impact to cell capacity

| R1-140685 | RRS pattern based coordinated scheduling for legacy IC UEs | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Introduce Iub signalling to indicate the RRS pattern to Macro and LPNs.

Decision: The document is noted.

| R1-140686 | RRS pattern based coordinated scheduling for new IC UEs | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Consider enhancements for more accurate CQI estimation and flexible transport format requirement on RRS subframes for new IC UE.

Decision: The document is noted.

### Other

| R1-140141 | Draft LS on signalling impact on CRE operation in UMTS HetNet | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140143 | Considerations on Cell Range Expansion | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Observation 1: Decentralised CIO adaptation requires:

- Signalling between LPN/macro cell and the RNC to update the adaptation metrics

- Signalling between network and UEs to update CIO value

Observation 2: The range of CIO values that a UE can tolerate in the CRE region depends on the UE receiver capability.

Observation 3:  It is beneficial that the LPN is able to reject and/or recommend another CIO value to the RNC when a UE is considered for handover to the LPN.

Proposal 1: Send an LS to RAN2 and RAN2 to inform them on the 3 observations discussed in this contribution.

Decision: The document is noted.

| R1-140709 | Introduction of DF-DC as a Multiflow Configuration | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal: The configuration of a serving and assisting secondary serving HS-DSCH cell in multiflow is to be taken into account in the RAN1 specifications.

Decision: The document is noted.

Conclusion: There was disagreement if DF-DC should be specified. RAN is asked to clarify this aspect and WID will be revised accordingly. This will be captured in the Status Report.

| R1-140788 | NCL parameters for link imbalance calculation | NSN |  |
| --- | --- | --- | --- |

Proposal 1: Consider a separate measurement type/event for link imbalance.

Proposal 2: Consider, optionally, sending network specific parameters ULRCF and LPNdesense to the UE so as to synchronize the calculation at the UE.

Decision: The document is noted.

Conclusion: RAN2 is discussing this topic, and based on the outcome of RAN2 discussion, RAN1 would consider further discussion.

## Further EUL Enhancements

WID in RP-132078.

| R1-140710 | E-DPCCH orders for TTI Switching | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal 1: Consider E-DPCCH orders for UE based indication of TTI switching.

Proposal 2: E-DPCCH orders are transmitted 5 times in a repeated fashion to ensure reliability.

Decision: The document is noted.

Conclusion: RAN1 sees that using E-DPCCH to indicate the switch in TTI length to the non-serving cells during SHO appears possible to achieve a certain target reliability (as usual, L1 indications are never 100% reliable). When moving from 2ms TTI to 10ms TTI it is possible to use reserved E-TFCIs to convey the message. When moving from 10ms to 2ms TTI no such reserved E-TFCIs exist, and thus one may need to be borrowed from the 10ms TTI E-TFCI tables.

Draft LS to RAN2 on email review until 28th of February, Arjun

| R1-140904 | LS to RAN2 E-DPCCH orders to indicate TTI switching | Qualcomm |  |
| --- | --- | --- | --- |

### Enabling high user bitrates in a mixed-traffic scenario

#### DTX/DRX enhancements

| R1-140687 | Design for DTX/DRX enhancements | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: The DTX parameters of the secondary carrier are independent of the DTX parameters of the primary carrier in support of DTX/DRX enhancements.

Proposal 2: The DTX cycles of the secondary carrier should be set as an integer multiple of DTX cycles of the primary carrier in support of DTX/DRX enhancements.

Decision: The document is noted.

| R1-140790 | DTX/DRX enhancements for further improved EUL | NSN |  |
| --- | --- | --- | --- |

Proposal: If the HS-SCCH order based secondary carrier activation/deactivation is considered insufficient, consider extending the uplink DTX operation with DC-HSUPA so that stand-alone DPCCH bursts may be eliminated from the secondary carrier, e.g. when the secondary carrier is scheduled a zero grant over E-AGCH.

Decision: The document is noted.

| R1-140807 | On Decoupling the DRX/DTX Parameters for Secondary Carriers | Ericsson |  |
| --- | --- | --- | --- |

Proposal 1: Introduce the value of “Infinity” to the UE S-DTX cycle 2 of the secondary carriers.

Proposal 2: Introduce an UE S-DRX cycle 2 along with an Inactivity Threshold for UE S-DRX cycle2.

Decision: The document is noted.

| R1-140837 | DTX/DRX improvements for higher rates and battery savings | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal 1: Introduce longer values for DTX cycle 2 for single and multi-carrier EUL operation

Proposal 2: Allow independent DTX parameter values to be signalled for the primary and secondary uplink carrier in DC-HSUPA

Proposal 3: Discuss the merits of the scheme presented for timer based de-activation of secondary band carrier in Dual-Band HSDPA operation

Decision: The document is noted.

| R1-140808 | On Increasing the DRX Cycle for Single and Primary Carriers | Ericsson |  |
| --- | --- | --- | --- |

Proposal 1: Introduce an UE DRX cycle 2 along with an Inactivity Threshold for UE DRX cycle2.

Decision: The document is noted.

Conclusions on uplink DTX

- Allow for configuring different DTX cycle lengths for the two carriers

- Allow for configuring different inactivity threshold for DTX cycle 2 for the two carriers

- Consider further extending the possible DTX cycle 2 durations for the secondary carrier, possibly also including ‘infinity’.

  - The possibility to waive Synch procedure AA after a long gap associated with infinitely long DTX cycle should be addressed (Synch procedure AA is used with DC-HSUPA when the secondary carrier is activated with an order)

  - The possibility to transmit DPCCH only together with E-DPCCH/E-DPDCH on the secondary carrier could be considered further as an alternative to infinitely long DTX cycle 2.

Downlink DRX options to be considered further

- Timer-based deactivation of secondary band carrier(s) in Dual-Band HSDPA operation enabling the UE to turn off the secondary band receiver for battery saving

- A UE DRX cycle 2 along with an Inactivity Threshold for UE DRX cycle2 for single (as well as multi) carrier operation for better battery saving from longer DRX opportunity

  - The maximum DRX cycle length may be limited by the latency implied to SRB

  - The battery saving benefit from the longer DRX cycle is limited by the network inactivity timer for moving the UE out of Cell_DCH

#### Improved granting

| R1-140789 | On improved grant handling for TDM operation | NSN |  |
| --- | --- | --- | --- |

Proposal: Streamline the further work to the subset of proposed approaches: New E-AGCH Timing for Deactivation and Grant Detection

Decision: The document is noted.

| R1-140688 | Improved granting for TDM operation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal: Improved granting for TDM operation is supported by means of allowing UE to monitor more than one E-AGCHs and R12 UE applies the E-AGCH for deactivation one TTI earlier than legacy.

Decision: The document is noted.

| R1-140809 | On Improving the TDM Operation for Enabling High User Bitrates | Ericsson |  |
| --- | --- | --- | --- |

Proposal 1: To improve the TDM operation by using a Time Limited Grant scheme that reuses the L1 structure of the E-AGCH.

Decision: The document is noted.

The improved grant handling for TDM operation to be considered further with the aim to decide on the concept for specification in RAN1#76bis

| R1-140791 | Fixing Gain Factors for Improved Granting | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

#### Improved power control

| R1-140711 | Impact of Power Control on Longer DTX/DRX Cycles | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140810 | On Improving the Power Control after Long DTX on Secondary Uplink Carrier | Ericsson |  |
| --- | --- | --- | --- |

Proposal 1: To propose solutions to a power control issue that becomes more relevant when dealing with long data interruptions, but that also holds for the current standardized solution in CPC.

Decision: The document is noted.

Conclusion: Further investigation is encouraged on whether the worse stability of the first E-DCH TTI after a DPCCH transmission gap can be mitigated.

### HS-DPCCH overhead reduction

| R1-140689 | Adaptive CQI cycle solution for HS-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal1: CQI cycle adaptive solution is adopted for HS-DPCCH overhead reduction.

Proposal2: HS-SCCH order is used to switch UE from a large CQI reporting cycle to a small CQI reporting cycle.

Decision: The document is noted.

| R1-140792 | Analysis of Potential Gains for Optimization of HS-DPCCH CQI Reporting Period | NSN |  |
| --- | --- | --- | --- |

Proposal: Do not consider adaptive HS-DPCCH reporting period optimization as a means for HS-DPCCH overhead reduction.

Decision: The document is noted.

| R1-140690 | DTX HS-DPCCH for multi-RAB scenario | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Agree on HS-DPCCH power reduction in case of power limited multi-RAB scenario.

Proposal 2: CQI/PCI transmit power can be reduced to zero when HS-DPCCH power reduction is applied.

Proposal 3: ACK/NACK transmit power can be reduced to zero or a minimum gain factor configured by higher layers when HS-DPCCH power reduction is applied.

Decision: The document is noted.

| R1-140712 | HS-DPCCH Overhead Reduction | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Proposal: Consider selective scaling of HS-DPCCH for control channel overhead reduction

Decision: The document is noted.

| R1-140811 | On a CQI Report Reduction for Multi-RABs with Speech | Ericsson |  |
| --- | --- | --- | --- |

Proposal 1: Introduce a UPH threshold, and a CQI_INACTIVITY_TIMER along with a CQI report cycle 2

Decision: The document is noted.

Options to consider for reducing the HS-DPCCH overhead

- Helping the UL DPDCH coverage in multi-RAB

  - Scale down CQI gain factor (to zero) before applying equal scaling to DPDCH/DPCCH

    - Always when power limited

    - When power limited and there is no DL activity

  - Increase the CQI feedback cycle (potentially up to infinity) if the UE is power limited and there is no DL activity

  - Scale down ACK/NACK gain factor (to zero or to a minimum value)

    - When power limited

  - Indicate the Node B of the power limitation to allow for the Node B not to schedule HSDPA and thus not incur ACK/NACK overhead (may already be there with UPH, happy bit, etc…)

- Helping to reduce the HS-DPCCH capacity consumption

  - Increase the CQI cycle (possibly up to stopping CQI reports)

    - After DL data inactivity

    - If Node B tells the UE to do so with HS-SCCH order

Not treated.

| R1-140691 | RAN1 Specification impact for the introduction of HS-DPCCH overhead reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

## Enhanced Broadcast of System Information

WID in RP-132077.

| R1-140793 | Initial analysis on design options for a new system information channel | NSN |  |
| --- | --- | --- | --- |

Proposal: Consider P/S-CCPCH as a baseline for further discussion on the physical channel design for the secondary BCH channel.

Decision: The document is noted.

| R1-140812 | BCH Enhancements - RAN1 aspects | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

Agreement: The physical channel carrying BCH2 is S-CCPCH. The spreading factor is 256.

Working assumption: TTI length is 10/20ms.

LS to RAN2 (Ericsson): Indicate the above agreement and working assumption.

| R1-140906 | LS on Enhanced Broadcast of System Information | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and LS is agreed.

To discuss in next meeting: configurability of which code, TBS and number of TB sizes.

## Study on Scalable Bandwidth UMTS by Filtering

SID in RP-132122.

| R1-140816 | Scalable UMTS Scenarios | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

### Evaluation for standalone scenario

| R1-140693 | 25.701 CR0001 (Rel-12,B) Link level simulation assumptions for filtered S-UMTS | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Thomas to discuss offline and prepare a single set of simulation assumptions in R1-140908 for inclusion in the TR, including:

- Modify the scaling of the overhead channels (PCPICH, PCCPCH, PICH and SCH)

- Agreeable modifications uplink simulation assumptions

Conclusion:

- Agree R1-140908 for inclusion in the TR modified as follows:

  - List all the different assumptions that are used in any of the sets of results that are to be included in the TR from R1-140984

  - Add a note that more realistic evaluations would use assumptions that are yet to be identified.

  - Add text in the TR pointing out where the assumptions used for the results that are already in the TR differ from the assumptions in R1-140908 and that they cannot be directly compared with the results that use the assumptions from R1-140908.

  - Also explain that the results for filtered UMTS cannot be directly compared with the results for time dilated UMTS.

  - Call the existing results “preliminary results that motivated further study”.

Prepare a TP corresponding to the above conclusion in R1-140986 – Ericsson.

For email approval until Tuesday 25th February.

| R1-140695 | 25.701 CR0003 (Rel-12,B) DL link level simulation results for filtered standalone Scalable UMTS assuming equal overhead power ratio | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted. Curves for inclusion in the TR should be limited to 20dB.

| R1-140696 | 25.701 CR0004 (Rel-12,B) DL link level simulation results for filtered standalone Scalable UMTS assuming N times overhead power ratio | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140714 | Downlink Link Evaluation Results for Filtering in Standalone Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Possible ways forward:

- Align simulation assumptions (especially for overhead scaling) between filtered and time dilated scalable UMTS, or

- Include results with different overhead scaling assumptions and note in the TR that the results for different scalable UMTS techniques should not be compared with each other.

- Aim to agree a common set of overhead assumptions, e.g.:

  - PCPICH: not scaled

  - PCCPCH: scaled

  - SCH: scaled

  - PICH: not scaled

  - HS-SCCH: scaled

Send an LS to RAN4 to ask for advice on impact on RRM measurements of PCPICH scaling / lack of scaling for filtered UMTS – AT&T to prepare a draft in R1-140945.

Note that this will not delay the completion of the SI but may provide useful guidance for a future WI.

| R1-140945 | [DRAFT] LS on impact on RRM measurements of P-CPICH scaling for Filtered UMTS | AT&T |  |
| --- | --- | --- | --- |

Decision: The document is noted and modified as follows:

RAN1 would like to understand how much power (Ec/Ior) should be allocated to P-CPICH in order to meet RAN4 RRM requirements since it would  provide useful guidance for deciding between scalable UMTS specification options.

2. Actions:

To RAN4

ACTION: 	RAN1 asks RAN4 to provide recommendations on the P-CPICH overhead that is needed for the two schemes to meet RRM requirements.

In the Annex, add a description of pure filtered UMTS equivalent to the description of CZ-FUMTS.

Updated draft to be provided by Qualcomm in R1-140947 – approved in R1-140985.

Observation: The degradation in throughput performance due to scaling of overhead channels is in the range 10-15% for N=2 depending on the scaling factors applied; scaling prevents any useful throughput in the case of N=4.

| R1-140716 | Uplink Link Evaluation Results for Filtering in Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted. Comment that it would also be interesting to look at difference in Eb/No

| R1-140794 | Uplink performance of 1.25 MHz S-UMTS by Filtering | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted. Correction to section 6.8.2.2.2 is agreed.

| R1-140813 | Filtered UMTS with and without Chip Zeroing - Initial Link Level Simulation Results | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observations based on R1-140813:

- These results are consistent with those in the TR

- The performance of F-UMTS and CZ-UMTS is comparable at low data rates

- At 1Mbps the difference is around 0.5dB for 2.5MHz;

  - The performance difference may differ for different bandwidths and may also be reduced if receiver optimisations are considered.

- Above 1.2Mbps, the difference reduces.

| R1-140884 | Filtered UMTS with and without chip zeroing - Further simulation results | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140821 | Text Proposal on Link Level Simulation Results for Filtered UMTS with and without Chip Zeroing | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140815 | Filtered UMTS with and without Chip Zeroing and Multiple Users - Initial Link Level Simulation Results | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observations:

- Chip zeroing shows a marginal improvement in all the low-rate cases studied here compared to filtered UMTS

- Higher data rate cases have not been studied here.

| R1-140823 | Text Proposal on Link Level Simulation Results for Filtered UMTS with and without Chip Zeroing - Multiple Users | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140814 | Carrier Bandwidth Impact on Filtered UMTS Performance | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observation: If FUMTS or CZ-FUMTS is specified, some flexibility in bandwidth may be possible, e.g. the exact carrier bandwidth need not be exactly 2.5 MHz.

| R1-140715 | Link Evaluation Results for AMR 12.2kbps for Standalone Scalable UMTS with Filtering | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observation: In the uplink, the two schemes have comparable performance since the the uplink voice operated at low SNR region. However, the downlink performance of filtered UMTS is slightly worse than the Enhanced filtered UMTS.

### Evaluation for multi-carrier scenario

| R1-140698 | 25.701 CR0006 (Rel-12,B) DL link level simulation results for multi-carrier filtered Scalable UMTS | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140719 | Downlink Link Evaluation Results for Filtering in Multicarrier Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140717 | System Simulation Results for Scalable UMTS with Filtering - Full Buffer | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140718 | System Simulation Results for Scalable UMTS with Filtering - Bursty Traffic | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140697 | 25.701 CR0005 (Rel-12,B) Cross carrier scheduling for multi-carrier filtered S-UMTS | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140948 | Way Forward on the addition of Simulation Results on Scalable UMTS with Filtering | Qualcomm, Huawei, HiSilicon, NSN, China Unicom |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

Include in the TR:

- Downlink Link Simulation Results

  - R1-140696	DL link level simulation results assuming N times overhead power ratio	Huawei, HiSilicon

  - R1-140714	Downlink Link Evaluation Results for Filtering 		Qualcomm Incorporated

    - Describe these results as indicative of the relative comparison between CZ-FUMTS and FUMTS, and with UMTS with an 8% margin of uncertainty

    - Curves for inclusion in the TR should be limited to 20dB.

    - Only include results for N=2

    - Mention that results should not be compared with Time Dilated UMTS

    - List the different overhead assumptions used for each set of results

    - Mention that more realistic evaluations that are not shown here would use xxx assumptions

    - Mention that results for N=4 are not shown.

- Uplink Link Simulation Results

  - Put just the following text in the TR:

    - It is not yet possible to identify which filtering scheme for the uplink would be preferable from the results obtained, since the results and any conclusions that might be drawn from them have sensitivities to different assumptions, and these sensitivities need to be evaluated for different assumptions that are identified to be relevant.

- DCH

  - Include results from R1-140715	Link Evaluation Results for AMR 12.2kbps for Standalone Scalable UMTS with Filtering	Qualcomm Incorporated

    - Plus observation recorded above.

- Carrier Aggregation

  - Include the following results:

  - R1-140719	Downlink Link Evaluation Results for Filtering in Multicarrier Scalable UMTS	Qualcomm Incorporated

  - R1-140718	System Simulation Results for Scalable UMTS with Filtering - Bursty Traffic	Qualcomm Incorporated

  - Note that the user penetration is assumed to be 100% etc. (use same text as for time dilated UMTS)

- SCH Analysis

  - Include the following results:

  - R1-140722	Searcher Analysis for Scalable UMTS with Enhanced Filtering 	Qualcomm Incorporated

    - Performance of SCH with CZ

  - R1-140795	SCH for S-UMTS  	NSN

    - Cell acquisition description and SCH design

  - Note that SCH analysis without chip zeroing is not available.

Prepare a TP based on the above - R1-140984 – for email approval until Tuesday 25th February.  - Qualcomm

### Other

| R1-140145 | Structure of TR25.701 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted and agreed without the inclusion of sections 6.8.4 – 6.8.7.

Conclusion: Do not include sections 6.8.4 – 6.8.7 in the TR, but work on open issues list for SR.

Add to status report open issues list:

- Uplink

  - Uplink simulation assumptions and performance

  - Analysis of PAPR impacts of chip zeroing and filtering

  - Analysis of performance and feasibility of PRACH solutions

- Downlink

  - Downlink N=4 standalone performance

  - [Analysis of performance of filtered SCH without chip-zeroing]

- RAN2 and RAN4 have not been consulted on any aspects within their scope related to FUMTS. RAN1 does not have the expertise to assess these aspects even though some contributions were submitted on these topics.

Qualcomm and Ericsson to prepare a list of open issues using the above as a starting point for inclusion in the SR for agreement by email until Tuesday 25th February.

| R1-140692 | Considerations on filtered S-UMTS with chip zeroing | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140694 | 25.701 CR0002 (Rel-12,B) Control channel overhear for filtered S-UMTS | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Observations:

- Operation of SCH with chip zeroing is feasible, with boosting to compensate the loss in autocorrelation peak (around 3dB for N=2 and up to 10dB for N=4)

- Operation of PRACH with and without chip zeroing is possible.

  - Study of PRACH performance has not been carried out.

- Possible methods for detecting system bandwidth have been identified.

| R1-140722 | Searcher Analysis for Scalable UMTS with Enhanced Filtering | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140795 | SCH for S-UMTS | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140799 | On RRC filter for S-UMTS | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140713 | Enhanced Filtering for Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted. Ericsson considers PAPR to be an important aspect that is not covered in this paper.

| R1-140725 | Impact of Enhanced Filtering for Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Qualcomm to prepare some text describing the possible alternatives for different channels including SCH and PRACH as an update to R1-140727 in R1-140946 – for email approval until Tuesday 25th February (Qualcomm), also including text from R1-140713, and Ericsson’s comment on PAPR.

Also include text from R1-140697 on cross-carrier scheduling; discuss offline to see which parts to include.

Avoid use of “S-UMTS” acronym.

| R1-140726 | Specification Impact of Enhanced Filtering for Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted. The following points are not covered in this document:

- Impact of cross-carrier scheduling for multi-carrier scenarios

- Possible pilot pattern optimisations

- RAN2 impacts (could be an analysis of the delta compared to what has already been identified for time-dilated S-UMTS)

- RAN4 impacts, including possibly PAPR

| R1-140727 | TP on Impact of Enhanced Filtering for Scalable UMTS | Qualcomm Incorporated |  |
| --- | --- | --- | --- |

Decision: The document is noted.

When updating in R1-140945, change “3GPP specifications” to “RAN1 specifications”, and mention RAN2/RAN4 impact: “Differences in RAN2/RAN4 impact compared to time-dilated S-UMTS have not been studied.”

| R1-140796 | On configuration of S-UMTS cells | NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140949 | TP on conclusions | QUALCOMM Incorporated, Huawei, HiSilicon, NSN, China Unicom |  |
| --- | --- | --- | --- |

Decision: The document is noted. Replace uplink part with comment from R1-140984.

For email approval until Tuesday 25th February - R1-140988.

Friday 14th

For email approval until Tuesday 25th February of one package CRs to TR25.701 – (NSN)

Not treated.

| R1-140144 | Impacts of S-UMTS by filtering | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |
| R1-140720 | TP on Enhanced Filtering for Scalable UMTS | Qualcomm Incorporated |  |
| R1-140721 | TP on Link Evaluation Results of Scalable UMTS with Filtering | Qualcomm Incorporated |  |
| R1-140723 | TP on Searcher Analysis for Scalable UMTS with Enhanced Filtering | Qualcomm Incorporated |  |
| R1-140724 | TP on System Simulation Results for Scalable UMTS with Filtering | Qualcomm Incorporated |  |
| R1-140797 | Impact of S-UMTS by filtering to higher layers | NSN |  |
| R1-140798 | Impact of S-UMTS by Filtering on RAN4 requirements | NSN |  |
| R1-140817 | Impact of Filtered UMTS and CZ-UMTS on UE Core Requirements | Ericsson |  |
| R1-140818 | Impact of Filtered UMTS on UE and BS Performance Requirements | Ericsson |  |
| R1-140819 | Impact of Filtered UMTS and its Optimization on RRM Requirements | Ericsson |  |
| R1-140820 | Impact of Filtered UMTS and CZ-UMTS on BS Core Requirements | Ericsson |  |
| R1-140822 | Text Proposal on Carrier Bandwidth Impact on Filtered UMTS Performance | Ericsson |  |

## Other

| R1-140677 | Enhancement to uplink power control algorithm 2 to reduce power consumption of DL TPC commands | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Proposal 1: Agree to introduce enhancement to uplink power control algorithm 2 by means of allowing UE to do soft combining of TPC commands in a given number of slots.

Decision: The document is noted.

| R1-140699 | Considerations on further enhancements to Scalable UMTS | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 11

New UE category (to support 450 Mbps data rate in the downlink)

| R1-140106 | Discussion on new UE category | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and proposes:

- Adopt in TS 36.306 a physical layer parameters of a new UE categories as proposed in Table 2.

- For Rel-11 UE Cats the maximum number of bits of a MCH transport block received within a TTI (=75376) can be kept the same as for associated Rel-10 UE Cat.

- A new Rel-11 Cat 9/10 UE must signal associated Rel-10 Cat 6/7 and Rel-8 Cat 4 to support backwards compatibility with Rel-10/9/8 eNBs.

- For new Rel-11 UE Cats reuse the existing rate matching patterns as associated Rel-10 UE Cats on any component carrier. Extend the Rel-10 rule defined in TS 36.212 as proposed in Figure 4.

- Reuse the existing UE procedures for storing the soft channel bits. Extend the Rel-10 rule defined in TS 36.213 as proposed in Figure 5.

Decision: The document is noted.

| R1-140107 | 36.212 DraftCR (Rel-11. F) Correction to the rate matching to support new UE category | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-140108 | 36.213 DraftCR (Rel-11. F) Correction to the storing of soft channel bits to support new UE category | Intel Corporation |  |

The draft CRs are noted.

| R1-140212 | Discussion of introduction of New UE category in Rel-11 | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei and proposes:

- Parameters related to new defined category 6a/7a in Rel-11 are shown in Tables 1-4

- A UE indicating category 6a shall also indicate category 4 and category 6. A UE indicating category 7a shall also indicate category 4 and category 7.

- UE should follow LBRM pattern of the Rel-11 category, when the UE is configured with transmission mode 10

- UE should store soft channel bits according to Rel-11 category, if the UE indicates a Rel-11 category.

Decision: The document is noted.

| R1-140856 | Proposal on new UE category | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

Decision: The document is noted.

At this point Mr Chair asked whether slides 2 and 3 (w.r.t Transport Block Sizes) could be agreed and whether any of below companies have a different proposal.

| R1-140434 | Draft CR 36212 New UE Category | Qualcomm Inc. |  |
| --- | --- | --- | --- |
| R1-140435 | Draft LS on New 450Mbps UE Category | Qualcomm Inc. |  |
| R1-140646 | On new UE categories for LTE Release 11 | NVIDIA |  |
| R1-140729 | Discussion on UE category support for new 450 Mbps UE class | Ericsson |  |
| R1-140730 | [DRAFT] LS on introduction of new UE categories | Ericsson |  |
| R1-140731 | Draft CR on 36.212 Introduction of new UE categories | Ericsson |  |
| R1-140838 | Need for additional UE categories between UE category 7.1 and UE category 8 | Sprint |  |

Fujitsu : be careful with the wording – RAN agreed introducing a new UE category, while RAN1 agreed on the details doing so – will be discussed when drafting the reply LS.

| R1-140433 | New 450Mbps UE Category | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by … from Qualcomm and proposes to:

- Introduce UE category 9 and 10 with the following principles;

  - Category 9: DL 450Mbps class paired with the UL processing requirement as defined for UL Cat. 6

  - Category 10: DL 450Mbps class paired with the UL processing requirement as defined for UL Cat. 7

- Adopt the downlink physical layer parameter values for the new UE categories shown in Table 1

- Adopt the uplink physical layer parameter values for the new UE categories shown in Table 2

- Define the following values for the “Maximum number of bits of a MCH transport block received within a TTI” for the new UE categories

  - Category 9: 75376 bits

  - Category 10: 75376 bits

- Modify the PDSCH rate matching parameters in TS36.212 [4] by introducing KC  = 3 for up to two layer MIMO and KC  = 1.5 for more than two layer MIMO for the new UE categories.

- Proposal 6: [RAN2] A UE indicating category 9 shall also indicate category 6 and category 4.

- Proposal 7: [RAN2] A UE indicating category 10 shall also indicate category 7 and category 4.

- Proposal 8: [RAN2] Introduce explicit network signalling to indicate the UE category among the signalled up to three categories that the UE shall assume.

Decision: The document is noted.

| R1-140733 | Draft CR on 36.306 Introduction of new UE categories | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson and proposed Categories 9 and 10 be added to the specification. For backwards compatibility reasons, it is assumed that UEs indicating category 9 or 10, will also indicate cat. 6 or 7 respectively (and 4 to be Rel-8 compatible).

Discussion: Fujitsu  for signalling completeness, Cat 4 should be added.

Decision: The document is noted and agreed in principle.

| R1-140732 | Draft CR on 36.213 Introduction of new UE categories | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson.

Decision: The document is noted and agreed in principle.

Conclusion:

Offline discussion on the followings until Tuesday morning – (Huawei)

  - Necessity to introduce explicit network signalling to indicate the UE category

  - Necessity of Categories 9/10 signal to Category 4

  - Rate matching and soft buffer size

  - Possible LS to RAN2

| R1-140838 | Need for additional UE categories between UE category 7.1 and UE category 8 | Sprint |  |
| --- | --- | --- | --- |

The document was presented by John Humbert from Sprint and concludes to:

- Create two UE categories, one with a peak DL throughput of 516 and the other with a peak DL data rate of 689 Mbps. UL 64 QAM should be added as an optional feature

- Add a new UE category that supports DL 689 Mbps. UL 64 QAM should be added as an optional feature

Discussion: UL 64 QAM was already discussed in RAN (June 2013) – clarification needed to what the situation is.

Decision: The document is noted.

Wednesday morning:

| R1-140878 | Way Forward on New UE Category | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

- Include in the LS to RAN2 stating “There is no agreement in RAN1 on the necessity to introduce explicit network signaling to indicate UE category”.

- Include “and 4” in 36.306 CR

- No CR for 36.212 is needed for introduction of UE categories 9 and 10

Decision: The document is noted. Already included in the draft LS.

| R1-140879 | [Draft] LS on introduction of new UE categories | Huawei |  |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson.

Discussion: NVIDIA  Concern about the sentence “there is no agreement in RAN1 on the necessity to introduce explicit network signalling to indicate UE category” – give the impression that RAN1 is still working on explicit signalling – could be removed as only Qualcomm asked for this add-on.

Qualcomm proposal: add “RAN1 is further discussing the applicability of TM3/4 4x4 MIMO to categories 9 and 10” – if the answer is no then there won’t be explicit signalling, if the answer is yes then signalling will be TBD.

Decision: The document is noted. Final LS is agreed in R1-140898 assuming same content as R1-140879 and adding contact name Daniel (Ericsson).

Note: RAN1 can further discuss applicability of TM3/4 4x4 MIMO to categories 9 and 10

| R1-140868 | Draft CR on 36.306 Introduction of new UE categories | Ericsson | (R1-140733) |
| --- | --- | --- | --- |

Discussion: Qualcomm  ok with the content at this stage.

Decision: The document is noted and CR is agreed.

| R1-140869 | 36.213 CR0451 (Rel-11, F) Introduction of new UE categories | Ericsson | (R1-140732) |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed.

| R1-140870 | 36.213 CR0452 (Rel-12, A) Introduction of new UE categories | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed.

Qualcomm: still discuss the need for 36.212 CR

Friday 14th: No additional discussion about a potential 36.212 CR.

MBMS common search space

| R1-140800 | Clarification of search space monitoring for MBMS | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Mattias Frenne from Ericsson and deals with the support of eMBMS on Scell and particularly the issue of monitoring common search space on SCells. In the related RAN2 specifications, the UE is required to receive system information on the cell for which it is receiving the MBMS service, irrespectively of whether the cell is serving or not or whether it is a primary or secondary cell. To align 36.213 with RAN2 specifications, a CR0443 [R1-134984] containing this sentence was agreed in principle for Rel-12 UEs:

The UE shall monitor common search space on a cell to decode the PDCCHs necessary to decode PMCH on that cell if configured to decode PMCH by higher layers.

CR0443 clarifies CSS monitoring on any cell where it decodes PMCH. Hence, the sentence is agnostic to whether the cell is a serving cell or non-serving cell and whether it is configured as a PCell or SCell to the UE. Therefore, CR0443 applies generally to any UE that support MBMS, is generic and applicable to both Rel-10 UEs and Rel-11 UEs

It is proposed to agree on the CR0443 for at least Rel-11 to align with RAN2 Rel-11 specifications. The associated CR for Rel-11 and Rel-12 are in [R1-140801],[ R1-140802] respectively.

Decision: The document is noted.

| R1-140801 | 36.213 CR0449 (Rel-11, F) Common search space monitoring for MBMS | Ericsson, AT&T, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Decision: The document is noted and CR is agreed in principle.

| R1-140802 | 36.213 CR0450 (Rel-12, A) Common search space monitoring for MBMS | Ericsson, AT&T, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

Discussion: Qualcomm want to delay the agreement due to RAN2 discussions and come back later in the week.

Both Ericsson, ALU wondered why such delay?

AT&T: why not to simply wait and confirm whether there is no impact (iOT)

Decision: The document is noted and CR is agreed in principle.

Conclusion: Check the necessity of CR header change until Friday (Ericsson)

Friday 14th

| R1-141035 | 36.213 CR0449R1 (Rel-11, F) Common search space monitoring for MBMS | Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, AT&T | (R1-140801) |
| --- | --- | --- | --- |
| R1-141036 | 36.213 CR0450R1 (Rel-12, A) Common search space monitoring for MBMS | Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, AT&T | (R1-140802) |

Both revised CRs were presented by Mattias Frenne from Ericsson and are agreed.

non-MBSFNregionLength

| R1-140412 | 36.211 CR0187 (Rel-12. C) On PMCH starting symbol in an MBSFN subframe | Intel, Samsung, NEC, Panasonic, BlackBerry |  |
| --- | --- | --- | --- |

The document was presented by Seunghee Han from Intel and states that the higher layer parameter non-MBSFNregionLength are used to indicate the starting symbol  for PMCH transmission.

Decision: The document is noted and CR is agreed.

| R1-140414 | On use of higher layer parameter non-MBSFNregionLength | NEC |  |
| --- | --- | --- | --- |

The document was presented by Kevin Lin from NEC and concludes that there is a short period where MSI value of a MCH is unknown to a UE which is receiving the MCH, at the beginning of every MSP. It is asked whether the following is a common understanding in RAN1:

- a UE receiving a MCH may assume that higher layer parameter non-MBSFNregionLength applies in subframes where PMCH transmission may be expected without knowledge of MSI of the MCH, and

- it is not possible for a UE receiving a MCH and configured in TM9/10 to know if it will be scheduled with PMCH or possibly PDSCH until it finishes decoding MSI. Therefore, such UE may be expected to prepare for both.

Discussion: Panasonic  already captured in spec, but if needed, ok to discuss it off line.

Decision: The document is noted. No change for RAN1 specs. Continue offline discussions until Friday – (NEC)

Friday 14th

Takahiro Sasaki from NEC confirms that there is no need to clarify the UE behaviour in RAN1 when PMCH decoding carrying MSI is not completed.

On aperiodic SRS in two-symbol UpPTS

| R1-140436 | On aperiodic SRS in two-symbol UpPTS | Qualcomm Inc., CATT, Ericsson, Texas Instruments, Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and deals with one issue regarding A-SRS in a two-symbol UpPTS with ISRS=0, and the following is proposed:

- There is no consensus in how to interpret type 1 SRS under ISRS=0, i.e., whether to transmit SRS in one of the two UpPTS symbols or to transmit in both UpPTS symbols

  - No further specification clarification (Rel-10 and beyond) is necessary.

Decision: The document is noted.

| R1-140672 | SRS trigger type 1 and two-symbol UpPTS | Motorola Mobility |  |
| --- | --- | --- | --- |

The document was presented by Vijay Nangia from MoM and draws the following observations:

- Observation 1: There is no ambiguity whether the Rel-10 specification supports SRS transmission on both symbols in UpPTS. The specification text is valid for both SRS trigger type 0 and SRS trigger type 1.

- Observation 2: TDD SRS configuration index ISRS = 0 assigns both symbols of UpPTS to the UE for SRS transmission.

- Observation 3: The specification does not support the interpretation of the UE behaviour to transmit in one of the two UpPTS symbols for SRS trigger type 1 and SRS configuration index, ISRS = 0.

- Observation 4: There is no need for plural “s” in “transmission”. The specification text is unambiguous.

- Observation 5: According to the specification, UE behavior for SRS trigger type 1, two symbol UpPTS and SRS configuration index ISRS = 0 is to transmit SRS in both UpPTS symbols.

The Rel-10 specification text is unambiguous and UE behavior is defined for SRS trigger type 1, two symbol UpPTS and SRS configuration index ISRS = 0. According to the specification, UE behavior is to transmit SRS in both UpPTS symbols.

Discussion: Qualcomm don’t agree with the conclusion.

Panasonic suggested that it could be noted that one company claims that there is no ambiguity while others believe there is.

Decision: The document is noted.

Monday’s afternoon: MoM suggested to let off line discussion continue and revisit on Friday. Samsung tend to go in MoM’s view but also agree that there is no consensus within RAN1.

Possible conclusion: No further specification clarification (Rel-10 and beyond) is necessary.

Qualcomm/Ericsson: Why not, in order not to repeat the same discussion, to note that different implementations are allowed with current specifications.

SRS configuration #0 in Table 8.2-5 is reserved (CATT’s comment)

Conclusion:

- Configuration #0 in Table 8.2-5 is reserved in Rel-10 and beyond

- CR shall be prepared – Revisit on Friday (Qualcomm)

Friday 14th

| R1-141044 | 36.213 CR0453R1 (Rel-10, C) Modification to I_SRS = 0 for trigger type 1 SRS and TDD | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | (R1-141018) |
| --- | --- | --- | --- |
| R1-141045 | 36.213 CR0454R1 (Rel-11, A) Modification to I_SRS = 0 for trigger type 1 SRS and TDD | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | (R1-141019) |
| R1-141046 | 36.213 CR0455R1 (Rel-12, A) Modification to I_SRS = 0 for trigger type 1 SRS and TDD | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | (R1-141023) |

The document was presented by Wanshi Chen from Qualcomm.

Decision: The documents are noted and the 3 CRs are agreed.

Others

| R1-140256 | Draft CR for introduction of LTE Coverage Enhancement. | ZTE |  |
| --- | --- | --- | --- |

The document was presented by Zhisong Zuo from ZTE and deals with the Rel-12 feature of TTI bundling enhancement for FDD with 3 HARQ processes.

Discussion: Mr Chair asked clarifying whether this was a Rel-12 CR: wrong current version of the spec, strange revision number?

Decision: The document is noted. May be included by spec editor in a unique Rel-12 CR (details to be further announced in the course of the week)

| R1-140478 | 36.213 DraftCR (Rel-11. F) Correction to CSI processing in TM10 | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and suggests modifying the text describing CSI calculation in TM10 to avoid possible ambiguities on the expected UE behavior.

Discussion: Different view from Huawei.

Decision: The document is noted.

| R1-140517 | Discussion on aperiodic CSI processing complexity in Rel-11 CoMP | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei and provides the following clarifications for Rel-11 CoMP aperiodic CSI reporting:

- “CSI process” in the agreement and in the relevant specification text is equivalent to a “CSI”, where a “CSI” is the association of a CSI process index and a CSI request received in one UL grant

- The UE is expected to determine Nu in every subframe where a CSI request is received

- Nu excludes the CSIs requested in the current subframe

- Nu excludes CSIs reported in the subframe where Nu is determined

- Nu includes all unreported CSIs, including the unreported CSIs that were already determined not to be expected to be updated

These points are understood to be non-ambiguous in the current specifications and given a reasonable interpretation of UE and eNB implementation, therefore no specification change is proposed for these clarifications. The potential ambiguity related to the duplicated instance of the word “other” may be corrected as follows:

- If seen as ambiguous, delete the first instance (CR) of “other” in “other Nu unreported CSI processes associated with other aperiodic CSI requests for the serving cell”. If no ambiguity, then no CR needed.

Discussion: Intel  what about the other two proposals in R1-140478.

Decision: The document is noted.

Conclusion: Continue off line discussion until Friday (Intel)

Friday 14th

| R1-141031 | 36213 CRxxxx (Rel-11, F) Correction to CSI processing in TM10 | Intel Corporation, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel who suggests to get it agreed in principle and let interesting companies review it more the details – ALU concern with the wording – the CR is for email approval by Friday 21st

| R1-140495 | 36.213 CR0447 (Rel-12, F) Correction to CSI Reporting | Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Alex Golitschek from Panasonic and corrects Table 7.2-1 and accompanying text that is contradicting other parts of sections 7.2, 7.2.1, 7.2.2 are voided/removed

Discussion: MCC: spelling error in Tdoc number.

Decision: The document is noted and CR is agreed.

| R1-140782 | 36.213 CR0448 (Rel-12, F) Clarification on PUCCH Mode 1-1 for 4Tx Dual Codebook | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Rohde-Schwarz |  |
| --- | --- | --- | --- |

The document was presented by Min Zhang from ALU and proposes that the mode 1-1 is configured to be submode 1 or submode 2 by the higher layer signalling PUCCH_format1-1_CSI_reporting_mode when 4Tx dual codebook is configured for TM8, 9 and 10.

Decision: The document is noted and CR is agreed.

## LTE Release 12

The document was presented by … from … and

Discussion: .

Decision: The document is noted.

### Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation

WID in RP-121772.

| R1-141010 | Summary of Ad-hoc session on Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and shows the outcomes of the eIMTA different sessions.

Decision: The document is endorsed and incorporated below with some add-ons.

#### Remaining details on UL power control

| R1-140064 | Summary of email discussion [75-36]: PHR for TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140892 | WF on PHR mechanism for TDD eIMTA | Huawei, HiSilicon, Panasonic, Potevio, LGE, CATR, Sharp, Alcatel Lucent, Alcatel Lucent Shanghai Bell, Nokia, NSN, Interdigital |  |
| --- | --- | --- | --- |

- For PHR reporting and triggering

  - Observation: Current PHR mechanism is not sufficient for TDD eIMTA.

  - One of the alternatives can be selected

    - PHR for the two subframe sets are reported simultaneously

    - Representative PHR (e.g. the minimum PHR is reported)

    - Subframe dependent PHR triggering

Decision: The document is noted.

Revisit on Wed.:

Alt1: Current PHR mechanism is sufficient to support PHR for the two subframe sets and no enhancement is needed

Alt2: Current PHR mechanism can allow PHR for the two subframe sets but not sufficient and enhancement is needed

- The UE can be configured to report PHR for the two subframe sets simultaneously or report PHR follows the Rel-11 behavior

  - If a UE is configured with simultaneous PUCCH and PUSCH, the reported PHR for the two subframe sets include type 1 and type 2 PHR in PUCCH subframes and type 1 PHR otherwise

  - Otherwise, the reported PHR for the two subframe sets include type 1 PHR

- Further offline discussion on the details

Wednesday 12th

| R1-140965 | WF on PHR for eIMTA | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Sharp, Huawei, HiSilicon, Nokia, NSN, LG Electronics, Potevio, InterDigital, MediaTek | (R1-140956) |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- Current PHR mechanism is sufficient to support PHR for the two subframe sets and no enhancement is needed

The following working assumptions from last meeting are confirmed:

- The association of (P0, alpha) with a UL subframe is separately configured via RRC

- For PUCCH PC, no enhancements (including both over-the-air and backhaul enhancements) relative to Rel-11

PUCCH

| R1-140296 | Remaining Details of Power Control enhancements for TDD eIMTA | LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document (Section 2.2 only) is noted.

Agreements:

- Downlink association set index K for PUCCH TPC accumulation is determined according to the configured DL HARQ reference configuration.

TPC commands (3/3A, #0, configurable TPC or not)

| R1-140109 | Discussion on remaining details of UL power control for eIMTA support | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document (Section 5 only) is noted.

| R1-140344 | UL Power Control for eIMTA – Basic Operation | Samsung |  |
| --- | --- | --- | --- |

Decision: The document (Section 2.2 only) is noted.

| R1-140540 | Remaining details on enhanced UL power control for TDD eIMTA | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document (Section 2 only) is noted.

Agreement:

- Accumulation of power control commands is not configurable between separate and joint for the two configured PUSCH subframe sets

Agreement:

- For the TPC commands in DCI formats 0/4/3/3A, the TPC commands are only applied to a subframe or subframes is per Rel-11 power command timing

  - That is, the power commands are only applied to the corresponding PUSCH/PUCCH power control process(es) associated with the subframe(s)

Power control for initial PUSCH

| R1-140437 | Remaining details of UL power control for TDD eIMTA | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document (Section 6 only) is noted.

Revisit on Thursday:

- Check whether or not the two PUSCH subframe sets can only be configured for eIMTA UEs.

- For contention based initial PUSCH transmission, the first subframe set (P0, alpha) is used

- For non-contention based initial PUSCH transmission, subframe set independent (P0, alpha) is applied if the UE is configured with two (P0, alpha) sets

Thursday 13th

Agreement:

- For initial PUSCH transmissions corresponding to RAR grant, Rel-11 power control is used

SRS

| R1-140018 | Remaining issues on power control for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document (Section 2 only) is noted.

| R1-140260 | Some Remaining Issues about Uplink Power Control | ZTE |  |
| --- | --- | --- | --- |

Decision: The document (Section 3 only) is noted.

| R1-140871 | WF on SRS in eIMTA | LG Electronics, CATT, Samsung, NSN, Nokia, Sharp |  |
| --- | --- | --- | --- |

Decision: The document is noted. Also supported by Huawei, HiSilicon, ALU, ASB

Proposed agreement:

- For type 1 SRS, the determination of the subframe where the type 1 SRS is due for transmission when triggered is based on SIB1

- For both type 0 and type 1 SRS, SRS transmissions can be configured in a UL subframe or UpPTS based on SIB1

  - If a UE detects L1 signaling conveying a valid UL-DL configuration for radio frame(s), and if the UL subframe or UpPTS for SRS transmission is changed to DL subframe, the UE shall drop the SRS transmission.

  - If a UE does not detect L1 signaling conveying a valid UL-DL configuration for radio frame(s),

    - (Working assumption only, revisit on Thursday): the UE still transmits the type 1 SRS in uplink subframes indicated based on SIB1, but the UE shall drop the type 0 SRS transmission in a subframe not indicated as UL subframe or UpPTS by the DL-HARQ reference configuration if there is no PUSCH transmission in the same subframe.

      - Further check offline whether the “if” condition may happen or not, and/or can be improved

Thursday 13th

Agreement:

- Under fallback, if the UE receives a UL grant corresponding to at least one UL subframe per SIB1 not in the set of UL subframes per DL HARQ reference configuration, the UE still treats it as a valid grant

- Under fallback, if the UE receives a NAK in PHICH triggering PUSCH transmission in a UL subframe per SIB1 not in the set of UL subframes per DL HARQ reference configuration, the UE transmits PUSCH

Agreement:

- For type 1 SRS, the determination of the subframe where the type 1 SRS is due for transmission when triggered is based on SIB1

- For both type 0 and type 1 SRS, SRS transmissions can be configured in a UL subframe or UpPTS based on SIB1

  - If a UE detects L1 signaling conveying a valid UL-DL configuration for radio frame(s), and if the UL subframe or UpPTS for SRS transmission is changed to DL subframe, the UE shall drop the SRS transmission.

  - If a UE does not detect L1 signaling conveying a valid UL-DL configuration for radio frame(s),

    - The UE still transmits the type 1 SRS in uplink subframes and special subframes indicated based on SIB1, but the UE shall drop the type 0 SRS transmission in a subframe not indicated as UL subframe or UpPTS by the DL-HARQ reference configuration if there is no PUSCH transmission in the same subframe.

| R1-140983 | Proposal on SRS fallback in TDD eIMTA | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- For SRS power control in a normal uplink subframe, the parameters of the PUSCH subframe set associated with the normal uplink subframe are used

- The RRC configured uplink subframe sets shall include special subframes in SIB1

- For SRS power control in a special subframe, the parameters of the PUSCH subframe set associated with the special subframe are used

CRS EPRE

| R1-140872 | WF on the downlink power control for eIMTA | LG Electronics, Ericsson, Sharp |  |
| --- | --- | --- | --- |

Decision: The document is noted. Revisit on Thursday.

Not treated.

| R1-140062 | Remaining details of uplink power control in TDD eIMTA | CATT |  |
| --- | --- | --- | --- |
| R1-140063 | PHR in TDD eIMTA | CATT |  |
| R1-140234 | Discussion on PHR in TDD eIMTA | MediaTek Inc. |  |
| R1-140291 | Remaining UL power control issue for e-IMTA | ASUSTeK |  |
| R1-140345 | UL Power Control for eIMTA – Remaining Issues | Samsung |  |
| R1-140541 | Power headroom report for enhanced UL power control in TDD eIMTA | NSN, Nokia |  |
| R1-140632 | Evaluation results on UL subframe sets signalling for enhanced UL PC | Sharp |  |
| R1-140633 | Remaining issues on PHR for eIMTA | Sharp |  |
| R1-140734 | On need of PHR enhancement for eIMTA | Ericsson |  |
| R1-140854 | Remaining details of UL power control for TDD UL-DL reconfiguration | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-140146) |

#### Remaining details on CSI feedback

Aperiodic CSI

| R1-140066 | Summary of email discussion [75-37]: DL aperiodic CSI measurement and reporting for TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

For CSI measurements for transmission modes 1-9 for both non-CA and CA cases:

- The two agreements on CSI measurement “only within DL/S SFs indicated by the L1 signaling if a valid one is detected” and “only within DL/S SFs indicated by SIB1 if a valid one is not detected” will be captured in the specification

For CSI measurements for all transmission modes for both non-CA and CA cases:

- The subframe sets for CSI measurements can be configured per CC, and if so, they are semi-statically configured per CC

For aperiodic CSI reporting in transmission mode 1~9 for a single serving cell, only CSI for one subframe set is reported in any subframe

Revisit on Thursday:

For aperiodic CSI reporting in transmission mode 1~9 for a single serving cell

- Alt 1: The CSI is reported for the subframe set associated with the PDCCH subframe, where the association of the PDCCH subframe and the subframe set is RRC configured separately, where the RRC configuration covers at least 2 frames

- Alt 1’: The CSI is reported for the subframe set associated with the PDCCH subframe, where the association of the PDCCH subframe and the subframe set is predefined by spec, i.e.,

  - For configuration #5, even frame for set 1 and odd frames for set 2

  - For all other TDD configurations, even subframe is associated with set 1 and an odd subframe is associated with set 2

- Alt 2: The CSI is reported for the subframe set associated with the CSI reporting subframe, where the association of the CSI reporting subframe and the subframe set is RRC configured separately, where the RRC configuration covers at least 2 frames

- Alt 2’: The CSI is reported for the subframe set associated with the CSI reporting subframe, where the association of the PDCCH subframe and the subframe set is predefined by spec, i.e.,

  - For configuration #5, even frame for set 1 and odd frames for set 2

  - For all other TDD configurations, even subframe is associated with set 1 and an odd subframe is associated with set 2

- Alt 3: The CSI reported in subframe n is reported for the subframe set based on the subframe associated with the latest DL subframe between subframe n-4 and the PDCCH subframe

  - Further details to be provided how it works

- Alt 4: The CSI reported for a subframe set in subframe n is based on a dynamic indication

Thursday 13th

Email discussion till 21/2/2014 – (CATT) – on the above alternatives for aperiodic CSI reporting in transmission mode 1~9 for a single serving cell, also taking into account the proposals in R1-141020.

| R1-141020 | WF on CSI Feedback for TDD eIMTA in TM1-9 | Qualcomm, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

| R1-140883 | Way Forward on eIMTA CSI Feedback in TM10 | Huawei, HiSilicon, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Qualcomm | R1-141042 |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140953 | Way Forward on minimum number of CSI processes and CSI-IMR for TDD eIMTA | ZTE, Intel, LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Email discussion till 28/2/2014 for A-CSI in TM10 and eIMTA – (Huawei)

- Take into account the WFs in R1-140953 and R1-141042

- Supporting companies for each proposal should provide detailed specification impact

| R1-141042 | Way Forward on eIMTA CSI Feedback in TM10 | Huawei, HiSilicon, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Qualcomm | (R1-140883) |
| --- | --- | --- | --- |

Periodic CSI

| R1-140110 | Discussion on remaining details of CSI feedback for eIMTA support | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document (Section 5 only) is noted.

| R1-140438 | CSI measurement and reporting in eIMTA | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document (Section 2.3 only) is noted.

| R1-140530 | Remaining details of CSI feedback for TDD eIMTA | Texas Instruments |  |
| --- | --- | --- | --- |

Decision: The document (Section 3.3 only) is noted.

| R1-141041 | Way Forward on Periodic CSI Reporting for eIMTA | Intel, LGE, Qualcomm |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Email discussion until 21/2/2014 regarding the proposals in R1-141041 – (Intel)

Agreement:

- The configuration of periodic CSI is such that the set of subframes for periodic CSI reporting are only in uplink subframes based on the DL HARQ reference uplink-downlink configuration configured for the UE.

Email discussion until 28/2/2014 on how to report P-CSI for flexible DL subframes when UE does not detect L1 signalling conveying a valid UL-DL configuration for a radio frame – (Qualcomm)

Not treated.

| R1-140019 | CSI enhancements for TDD eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140065 | CSI feedback in TDD eIMTA | CATT |  |
| R1-140147 | Remaining details of CSI feedback for TDD UL-DL reconfiguration | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140221 | On aperiodic CSI reporting in TDD eIMTA systems | HTC |  |
| R1-140261 | Remaining details of CSI feedback for TDD eIMTA | ZTE |  |
| R1-140297 | Remaining Details of CSI Measurement and Report for TDD eIMTA | LG Electronics |  |
| R1-140346 | Periodic CSI and SRS for eIMTA | Samsung |  |
| R1-140347 | Remaining issues on CSI feedback for eIMTA | Samsung |  |
| R1-140430 | Discussion on aperiodic CSI feedback for eIMTA | Potevio |  |
| R1-140634 | CSI enhancements for eIMTA | Sharp |  |
| R1-140735 | Remaining issues on CSI feedback for eIMTA | Ericsson |  |

#### Remaining details of signalling for TDD UL-DL reconfiguration

| R1-140067 | Remaining details of L1 signalling for UL-DL reconfiguration | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140111 | Remaining aspects for TDD UL/DL reconfiguration signalling | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140187 | Signalling for PUCCH/PUSCH configuration in eIMTA | Fujitsu |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140348 | Signalling for TDD UL-DL Reconfiguration | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140439 | Discussion on signalling details for TDD UL-DL reconfiguration | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140636 | Valid UL/DL configuration and fallback conditions in eIMTA | Sharp |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140736 | Remaining details on signalling for TDD UL-DL reconfiguration | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

- PUCCH is only transmitted in uplink subframes based on the DL HARQ reference uplink-downlink configuration configured for the UE.

- The following assumptions will be captured in the specification.

  - Whether to capture in the text format or a table format (e.g., the one in R1-140636) is up to 36.213 editor

  - For valid UL & DL reference configurations:

    - The set of UL subframes of the DL HARQ reference configuration should be a subset of the UL subframes of the UL HARQ reference UL/DL configuration

  - For validity of UL/DL configuration in a reconfiguration DCI under any valid UL & DL HARQ reference configurations:

    - The UE should not expect any subframe configured as UL subframe or special subframe in DL HARQ reference configuration is dynamically used as a DL subframe.

    - The UE should not expect any subframe configured as DL subframe or special subframe in UL HARQ reference configuration is dynamically used as a UL subframe.

The following working assumptions are confirmed:

- The size of DCI that carries reconfiguration bits is aligned to DCI format 1C.

- If UE does not detect L1 signaling conveying a valid UL-DL configuration for a radio frame,

  - UE shall monitor the non-DRX DL subframes or special subframes for PDCCH or EPDCCH as indicated by SIB-1 configuration

- 80ms periodicity for the reconfiguration signaling is supported.

Agreement:

- The number of eIMTA-RNTI configured for a UE is 1.

  - The eIMTA-RNTI is UE-specifically configured via RRC

  - Different UEs may be configured different eIMTA-RNTIs

- The set of  subframes that UE monitors for the explicit reconfiguration DCI is UE-specifically configured via RRC

- The mapping between a reconfiguration index and a cell is UE-specifically configured via RRC

Working assumption:

- The explicit reconfiguration DCI only carries information for explicit reconfiguration

- To be confirmed on or before Thursday if no other consensus can be reached

Thursday 13th: It is agreed that:

- The explicit reconfiguration DCI only carries information for explicit reconfiguration

Agreement:

- The set of subframes that a UE is possibly configured to monitor the reconfiguration signalling are based on:

  - For 10ms periodicity, the set of subframes are the DL subframes per SIB1

  - For 20ms periodicity, the set of subframes  are the DL subframes per SIB1 in the 2nd radio frame in the window

  - For 40ms periodicity, the set of subframes are the DL subframes per SIB1 in the 4th radio frame in the window

  - For 80ms periodicity, the set of subframes are the DL subframes per SIB1 in the 8th radio frame in the window

Agreement:

- The following is to be captured in the specification:

  - If a UE is configured to monitor in multiple subframes reconfiguration DCIs for radio frames , the UE may assume the same UL-DL configuration indicated by the reconfiguration DCIs for radio frames .

    - The same UL-DL configuration here refers to any cell configured for the UE with eIMTA-enabled

Revisit on Wed.:

- There are at least two possible UE behaviours:

  - UE is required to monitor all subframes to carry reconfiguration DCI(s) for radio frames  until one valid UL-DL configuration for radio frames  is detected and skips monitoring afterwards for radio frames

  - UE is required to monitor all subframes to carry reconfiguration DCI(s) for radio frames  until one valid UL-DL configuration is detected for radio frames  and continues monitoring afterwards for radio frames

- Whether or not/how to capture is FFS

Thursday 13th

Agreement:

- UE is required to monitor all subframes to carry reconfiguration DCI(s) for radio frames  at least until one valid UL-DL configuration for radio frames  is detected.

- How to capture the above is up to the spec editor

| R1-140873 | WF on UE assumptions in subframe #6 for eIMTA | LG Electronics, CATT, Sharp |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- The following working assumption is confirmed:

  - Reconfiguration between 5ms and 10ms switching point periodicity shall be supported.

Revisit on Wed.

Proposed agreement:

- If the subframe #6 is a special subframe indicated by SIB1 but a regular DL subframe indicated by the dynamic subframe configuration, the UE shall assume subframe #6 is a regular DL subframe

  - Whether or not to explicitly capture this in the specification is up to spec editor

Thursday 13th

Agreement:

- If the subframe #6 is a special subframe indicated by SIB1 but a regular DL subframe indicated by the dynamic subframe configuration, the UE shall assume subframe #6 is a regular DL subframe for unicast traffic

- If the subframe #6 is a special subframe indicated by SIB1 but a regular DL subframe indicated by the dynamic subframe configuration, the UE shall assume subframe #6 is a regular DL subframe for broadcast traffic

- From RAN1 perspective, the above bullet regarding unicast and broadcast traffic will not cause significant issues

- Draft LS to RAN2 regarding the above bullets, and ask RAN2 to provide feedback whether there are significant issues from RAN2 perspective or not and if so, RAN1 will re-visit the above agreement – Zukang (CATT) R1-141024, agreed in R1-141025 with the following updates:

- The sub-sub-bullet starting with “from RAN1 perspective” should be a sub-bullet

- “RAN1 respectfully asks”

Not treated.

| R1-140020 | Remaining details of signalling for TDD UL-DL reconfiguration | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140148 | Remaining details of signalling for TDD UL-DL reconfiguration | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140262 | Remaining Details of Signalling for TDD UL-DL Reconfiguration | ZTE |  |
| R1-140298 | Remaining Details of UL/DL Reconfiguration Signalling in TDD eIMTA | LG Electronics |  |
| R1-140480 | Remaining details on reconfiguration signalling for TDD eIMTA system | NEC |  |
| R1-140531 | On L1 signalling for dynamic TDD UL/DL reconfiguration | Texas Instruments |  |
| R1-140542 | Remaining details of signalling for TDD UL-DL reconfiguration | NSN, Nokia |  |
| R1-140635 | Remaining issues on changeability of switching period for eIMTA | Sharp |  |
| R1-140881 | WF on reconfiguration DCI for eIMTA | Huawei, HiSilicon, CATT, Intel |  |

#### Remaining HARQ details

| R1-140070 | Summary of email discussion [75-38]: PUCCH for TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working assumption:

- HARQ-ACK bundling is supported for HARQ-ACK feedback for TDD eIMTA, without further optimization w.r.t. bundling

Thursday 13th: It is agreed that:

- HARQ-ACK bundling is NOT supported for HARQ-ACK feedback for TDD eIMTA

Agreement:

- PUCCH format 1b with channel selection is supported for HARQ-ACK feedback for TDD eIMTA, along with the mapping table introduced in Rel-10 if eIMTA is enabled.

- PUCCH format 3 is supported for HARQ-ACK feedback for TDD eIMTA

Working assumption:

- Reuse the existing implicit PUCCH resource for subframes with the same DL HARQ timing between eIMTA and non-eIMTA UEs, and allocate new resources for other subframes in the bundling window, where the new resources are based on a separately RRC configured starting PUCCH offset (where the block interleaving for ACK/NAK resource indexing is still supported)

Thursday 13th: It is agreed that:

- For the case of PDCCH scheduling, reuse the existing implicit PUCCH resource for subframes with the same DL HARQ timing between eIMTA and non-eIMTA UEs, and allocate new resources for other subframes in the bundling window, where the new resources are based on a separately RRC configured starting PUCCH offset (where the block interleaving for ACK/NAK resource indexing is still supported)

Email discussion until 28/2/2014 - (NSN) on the case of EPDCCH scheduling regarding whether or not there are similar issues, and if so, whether or not the same solution for PDCCH can be adopted for EPDCCH. Also discuss for PDCCH the detailed equation for block interleaving and for PDCCH/EPDCCH the detailed subframe indexing.

Agreement:

- If TDD eIMTA is not enabled on the PCell but is enabled on at least one SCell, the UE behavior for HARQ-ACK feedback when PUCCH format 3 is configured follows that as specified in Rel-10/11.

- Otherwise, FFS (companies are encouraged to check R1-140069)

| R1-140069 | PUCCH for TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Thursday 13th

Agreement:

- If TDD eIMTA is not enabled on the PCell but is enabled on at least one SCell, the UE behavior for HARQ-ACK feedback when PUCCH format 3 is configured follows that as specified in Rel-10/11.

- Otherwise:

  - Email discussion until 21/2/2014 – Zukang (CATT) - on whether to following Rel-10/11 approach or the one proposed in R1-141026 or any other possible approaches

| R1-141026 | Way forward on HARQ-ACK feedback with PUCCH format 3 configured for eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140848 | Summary of email discussion on HARQ-ACK feedback on PUSCH for eIMTA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Email discussion 21/2/2014 (Aris, Samsung) on the following alternatives and conclude for each case:

Case 1: Usage of UL DAI and UL index in DCI format 0/4

- Alternative 1: The UL index is used in DCI format 0/4, when UL-DL configuration 0 is configured as UL reference configuration.

- Alternative 2-1: The UL DAI can be used in DCI format 0/4, using UL DAI in all subframes for all the dynamically indicated configurations other than configuration #0 and assuming predefined subframe-specific UL index values.

- Alternative 2-2: The UL DAI can be used in DCI format 0/4, using UL DAI in all subframes for some of the dynamically indicated UL-DL configurations

  - For configurations #0 and #6: UL index

  - For other configurations:  UL DAI

- Alternative 2-3: The UL DAI can be used in DCI format 0/4, where the usage of UL index or UL DAI in DCI format 0/4 depends on the index of DL/S subframe , i.e., using UL DAI in subframe #1 and #6 (with UL index value always set to 01).

Case 2: HARQ-ACK transmission on PUSCH when UL DAI is not transmitted in the latest subframe of bundling window

- Alternative 1: The number of HARQ-ACK bits is determined by UL DAI and no further specification is required.

- Alternative 2-1: The number of HARQ-ACK bits is determined by the size of the bundling window based on the DL HARQ timing reference configuration.

- Alternative 2-2: The number of HARQ-ACK bits is determined by the size of the bundling window of the dynamically indicated TDD UL-DL configuration.

- Alternative 2-3: The number of HARQ-ACK bits is determined by the sum of UL DAI plus X, where X is determined from the size of the bundling window for the DL HARQ timing reference configuration and the number of DL subframes located after the DL subframe used to transmit UL grant.

Not treated.

| R1-140021 | HARQ-ACK PUCCH resource allocation | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140022 | HARQ-ACK feedback bits on PUSCH | Huawei, HiSilicon |  |
| R1-140068 | Other remaining issues on HARQ in TDD eIMTA | CATT |  |
| R1-140149 | Remaining HARQ details for TDD UL-DL reconfiguration | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140222 | HARQ details in TDD eIMTA systems | HTC |  |
| R1-140235 | Remaining HARQ details for TDD eIMTA | MediaTek Inc. |  |
| R1-140263 | Remaining HARQ details | ZTE |  |
| R1-140299 | Remaining Details of HARQ Feedback for TDD eIMTA | LG Electronics |  |
| R1-140349 | UCI Multiplexing in PUSCH | Samsung |  |
| R1-140350 | Remaining issues on HARQ timings for eIMTA | Samsung |  |
| R1-140351 | HARQ-ACK feedback procedures for eIMTA | Samsung |  |
| R1-140431 | On remaining details of HARQ for eIMTA | Potevio |  |
| R1-140440 | HARQ-ACK feedback on PUCCH for TDD eIMTA | Qualcomm Inc. |  |
| R1-140441 | HARQ-ACK feedback on PUSCH for TDD eIMTA | Qualcomm Inc. |  |
| R1-140442 | DRX operation for eIMTA | Qualcomm Inc. |  |
| R1-140481 | PUCCH and PHICH resource reservation for TDD eIMTA system | NEC |  |
| R1-140482 | HARQ-ACK on PUSCH for TDD eIMTA system | NEC |  |
| R1-140496 | UL Index and DAI bits in DCI format 0/4 | Panasonic |  |
| R1-140532 | HARQ-ACK feedback on PUCCH for TDD EIMTA | Texas Instruments |  |
| R1-140533 | HARQ-ACK feedback on PUSCH for TDD EIMTA | Texas Instruments |  |
| R1-140543 | PUCCH formats for HARQ-ACK feedback | NSN, Nokia |  |
| R1-140544 | PUCCH resource allocation for HARQ-ACK feedback | NSN, Nokia |  |
| R1-140545 | UCI on PUSCH for TDD eIMTA | NSN, Nokia |  |
| R1-140615 | Remaining details of PUCCH design for eIMTA HARQ | NTT DOCOMO |  |
| R1-140637 | PUCCH resource allocation for dynamic HARQ-ACK in eIMTA | Sharp |  |
| R1-140638 | HARQ-ACK reporting on PUCCH and PUSCH for eIMTA | Sharp |  |
| R1-140737 | Remaining details on HARQ-ACK feedback for eIMTA | Ericsson |  |

#### Other

| R1-140071 | Other remaining issues for TDD eIMTA | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140444 | Handling of MBMS and PRS with eIMTA | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- For PRS and MBMS, Rel-11 configuration and behavior are unchanged.

| R1-141039 | WF on Soft Buffer Management for TDD eIMTA9 | Qualcomm, CATT, Samsung, LG Electronics, Sharp, ZTE, Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

For soft buffer management for TDD eIMTA

- For a UE configured with TDD eIMTA, the value of MDL_HARQ shall be determined by RRC configured DL HARQ reference configuration for soft buffer management

Friday 14th

Email approval until 14th March about RRC parameter lists – Zukang (CATT)

Email discussion/approval until 19th February w.r.t remaining open issues (focus on RAN2 possible impact) that have not been treated during the online session – Zukang (CATT)

Not treated.

| R1-140023 | Soft buffer partitioning for eIMTA DL transmission | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140300 | Other Design Aspects for TDD eIMTA | LG Electronics |  |
| R1-140352 | On other issues for eIMTA | Samsung |  |
| R1-140402 | Maximum number of DL HARQ processes for TDD eIMTA | ITRI |  |
| R1-140403 | Further discussion on backhaul signalling details in TDD eIMTA | ITRI |  |
| R1-140443 | Soft buffer management for IMTA | Qualcomm Inc. |  |
| R1-140579 | Dynamic TDD UL:DL configuration hopping for eIMTA | CEWiT |  |
| R1-140607 | Discussion on the remaining open issues for eIMTA | Broadcom Corporation |  |
| R1-140738 | Further details for eIMTA | Ericsson |  |

### Low Cost & Enhanced Coverage MTC UE for LTE

WID in RP-130848

| R1-141011 | Summary of Ad-hoc session on Low Cost & Enhanced Coverage MTC UE for LTE | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanshi Chen from Qualcomm and shows the outcomes of the MTC different sessions.

Decision: The document is endorsed and incorporated below with some add-ons.

#### Remaining details of new UE category/type

| R1-140024 | Discussion on TBS and simultaneous reception between common messages and unicast data for new category UEs | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140353 | Remaining details of new UE category/type | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140497 | Views on FFS on new UE category/type | Panasonic |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140583 | Transport Block Size support in low-complexity MTC UEs | Sony |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140865 | Way forward on some issues for low complexity MTC UE | Vodafone |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreements:

For PDSCH of the low complexity MTC UEs at least not in coverage enhancement:

- The maximum TBS shall be 1000 bits for unicast transmission on PDSCH.

- The maximum TBS shall be 2216 bits for data types referenced by SI-RNTI, P-RNTI, and RA-RNTI.

Prepare a reply LS to R1-140005 (Vodafone) – check on Thursday

Thursday 13th

| R1-140980 | DRAFT Reply to LS on mobility support for LC MTC UEs and MTC coverage enhancement | Vodafone Group |  |
| --- | --- | --- | --- |

Decision: The document is noted and agreed in principle with the following updates:

- Remove the head regarding RAN2 related tdoc number

- CC RAN

- The following has been agreed in RAN1 for the low complexity MTC UE not in coverage enhancement.

- In actions to RAN2, further asking RAN2 whether there are issues to have 2216 bits for data types referenced by P- and RA-RNTI.

Update the LS by incorporating the above changes (Vodafone) Revisit on Friday.  R1-141028

Final LS is agreed in R1-141029.

Proposed working assumption

- For the low complexity MTC UEs, these UEs support 15-RB PDSCH assignment(?)/bandwidth(?)

- Revisit on Thursday

Proposed agreement:

- Scheduling and HARQ timing for PDSCH and PUSCH for the low complexity MTC UEs not in coverage enhancement are the same as other UE categories

- Further discussion offline, revisit on Thursday

  - Related issues: DL system bandwidth, PDSCH scheduling (dynamic, semi-static, predetermined), EPDCCH vs. PDCCH

Thursday 13th

| R1-140981 | Summary of further discussions on MTC low complexity UE | Vodafone Group |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Send an LS to RAN2/RAN3 asking RAN2/RAN3 whether it is possible for an eNB to know whether a UE is TBS/bandwidth limited or not for paging and random access response – (ALU), check on Friday R1-141009

Friday 14th

| R1-141009 | Draft LS on Identification of TBS/bandwidth limited UE by eNB | Alcatel-Lucent | R1-141033 |
| --- | --- | --- | --- |

Decision: The document is noted and final LS is agreed in R1-141033.

Agreement:

- # HARQ processes for MTC UEs not in coverage enhancements:

- Keep the same number of processes unchanged from Rel-11

  - For HD-FDD, FFS

Email discussion  until March 14, 2014 on scheduling options for UEs not in coverage enhancements for broadcast and unicast channels, to further clarify tradeoffs of each option (refer to R1-140981) – Gustav, Sierra Wireless

Not treated.

| R1-140072 | Remaining details of new UE category/type | CATT |  |
| --- | --- | --- | --- |
| R1-140073 | Downlink bandwidth reduction for new category/type UEs | CATT |  |
| R1-140113 | Remaining issues of new UE category/type for low cost & enhanced coverage MTC UE | Intel Corporation |  |
| R1-140150 | Details of low cost MTC UE category | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140237 | Max TBS and simultaneous reception of common messages and unicast data | MediaTek Inc. |  |
| R1-140279 | Remaining details of new UE category for low cost MTC | ZTE |  |
| R1-140301 | Further discussion on details of new UE category/type | LG Electronics |  |
| R1-140445 | Remaining details of UE category type | Qualcomm Inc. |  |
| R1-140518 | Low Cost UE TBS and PRB limits discussion | Sierra Wireless |  |
| R1-140547 | Low-Cost MTC UE Category | NSN, Nokia |  |
| R1-140582 | Remaining details of bandwidth reduction for low-complexity MTC UEs | Sony |  |
| R1-140639 | Remaining issues on low-cost MTC Ues | Sharp |  |
| R1-140739 | Maximum BCCH TBS for low cost MTC UE | Ericsson |  |
| R1-140740 | Simultaneous reception in low cost MTC UE | Ericsson |  |
| R1-140741 | Resource allocation for low cost MTC UE | Ericsson |  |

#### Techniques for coverage improvement

Not treated.

| R1-140238 | System information transmission in coverage enhancement mode | MediaTek Inc. |  |
| --- | --- | --- | --- |
| R1-140498 | Proposal of common coverage enhanced subframe length | Panasonic |  |
| R1-140499 | Attempt of the summary of the email discussion on the handling on PRACH for enhanced coverage mode | Panasonic |  |

##### PBCH

Not treated.

| R1-140025 | Details on PBCH repetitions for coverage enhancement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140074 | Further discussion on PBCH coverage improvement for MTC UEs | CATT |  |
| R1-140151 | Coverage enhancement for PBCH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140188 | PBCH and System Information Support for Coverage Enhanced (CE) Devices | Fujitsu |  |
| R1-140223 | PBCH Coverage Improvement for MTC Devices | HTC |  |
| R1-140280 | PBCH Coverage Enhancement for MTC UEs | ZTE |  |
| R1-140281 | Discussion on SIB Coverage Improvement for MTC UEs | ZTE |  |
| R1-140302 | PBCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-140303 | Evaluation on PBCH coverage enhancement for MTC | LG Electronics |  |
| R1-140354 | PBCH Coverage Enhancements for MTC UEs | Samsung |  |
| R1-140446 | PBCH channel design for MTC coverage enhancements | Qualcomm Inc. |  |
| R1-140500 | PBCH coverage enhancement for low cost MTC | Panasonic |  |
| R1-140519 | MIB and SIB Acquisition Time Discussion and Guidance | Sierra Wireless |  |
| R1-140520 | 2X PBCH Repetition (option 1) Results and Evaluation | Sierra Wireless |  |
| R1-140548 | PBCH Coverage Enhancement | NSN, Nokia |  |
| R1-140648 | PBCH Coverage Enhancement for MTC UE | InterDigital |  |
| R1-140742 | System information for enhanced coverage MTC UE | Ericsson |  |
| R1-140827 | Coverage improvement for PBCH | Intel Corporation | (R1-140114) |

##### PRACH

Not treated.

| R1-140026 | Considerations on PRACH repetition levels and power adjustment of PRACH transmission | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140027 | PRACH resource multiplexing to support low cost MTC and coverage enhancement | Huawei, HiSilicon |  |
| R1-140075 | Remaining details on PRACH coverage improvement for MTC UEs | CATT |  |
| R1-140115 | Coverage improvement for PRACH | Intel Corporation |  |
| R1-140152 | Details of PRACH preamble coverage enhancement | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140153 | Coverage enhancement for RACH messages | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140189 | PRACH Support for Coverage Enhanced (CE) Devices | Fujitsu |  |
| R1-140190 | Coverage Depth Signalling supporting Low Cost MTC devices | Fujitsu |  |
| R1-140278 | PRACH Coverage Enhancement for MTC UEs | ZTE |  |
| R1-140304 | PRACH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-140305 | RACH procedure for MTC coverage enhancement | LG Electronics |  |
| R1-140355 | PRACH Coverage Enhancements for MTC UEs | Samsung |  |
| R1-140447 | RACH channel design for MTC coverage enhancements | Qualcomm Inc. |  |
| R1-140549 | PRACH Coverage Enhancement | NSN, Nokia |  |
| R1-140616 | Discussion on multi-level PRACH coverage enhancement | NTT DOCOMO |  |
| R1-140640 | On PRACH coverage enhancement for MTC UE | Sharp |  |
| R1-140649 | PRACH Coverage Enhancement for MTC UE | InterDigital |  |
| R1-140743 | Random access for enhanced coverage MTC UE | Ericsson |  |

##### (E)PDCCH/PUCCH

| R1-140028 | Discussion on issues for PDCCH coverage improvement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140240 | Analysis on (E)PDCCH search space design in coverage enhancement mode | MediaTek Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

FFS at least the following issues:

- The set of aggregation level(s)

- The set of repetition level(s)

- The number of decoding candidate(s)

- The starting subframe for a decoding candidate

- Linkage of resources across subframes for a same decoding candidate

  - Whether or not a same or different aggregation levels can be used in different subframes of a same decoding candidate

- Considering impact on other MTC UEs and normal UEs

| R1-141030 | PDCCH structure for MTC enhanced coverage | Vodafone Group |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

At least for unicast traffic:

- UE shall monitor more than one (E)PDCCH decoding candidate.

- FFS whether or not UE shall monitor multiple total aggregated resources of (E)PDCCH decoding candidates

  - This is for illustration only:

    - A Total Aggregated Resource (TAR) can be defined for a decoding candidate such that the TAR is given by Sum (ai) where i = 0…T-1, where T is repetition number and ai is the amount of resources in subframe i.  It is FFS whether ai is the same or different in different subframes.

    - Whether or not multiple TARs are monitored by a UE is FFS.

- The UE shall use the same (E)PDCCH decoding candidate m with an aggregation level defined for each subframe within the repetition window.

- FFS whether or not the option of UE monitoring only a single aggregation level with a single repetition is excluded.

- FFS whether to use the current hashing function, or a new method, to determine the starting location of unicast search space at each subframe within the repetition window. This depends on collision probability between UEs in coverage enhancement mode and normal UEs in a cell.

- Study tradeoff and performance of the following options:

  - Option 1: single (E)CCE aggregation level and multiple repetition levels

  - Option 2: multiple (E)CCE aggregation levels and single repetition level

  - Option 3: multiple (E)CCE aggregation levels and multiple repetition levels

  - Examples of the options can be found in R1-141030

Not treated.

| R1-140029 | Discussion on issues for PUCCH coverage improvement | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140076 | Necessity of (E)PDCCH scheduling PDSCH for SIB/paging/RAR | CATT |  |
| R1-140077 | Details on (E)PDCCH coverage improvement for MTC UEs | CATT |  |
| R1-140078 | PUCCH coverage improvement details for MTC UEs | CATT |  |
| R1-140116 | Coverage improvement for (E)PDCCH and PUCCH | Intel Corporation |  |
| R1-140154 | Coverage enhancement for (E)PDCCH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140239 | On the need of (E)PDCCH for SIB. RAR and Paging | MediaTek Inc. |  |
| R1-140275 | Discussion on PUCCH Coverage Improvement | ZTE |  |
| R1-140277 | Discussion on (E)PDCCH Coverage Improvement | ZTE |  |
| R1-140306 | (E)PDCCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-140307 | Evaluation on PDCCH coverage enhancement for MTC | LG Electronics |  |
| R1-140308 | UL channel transmission for MTC coverage enhancement | LG Electronics |  |
| R1-140356 | PDCCH Coverage Enhancements for MTC UEs | Samsung |  |
| R1-140357 | PUCCH Coverage Enhancements for MTC UEs | Samsung |  |
| R1-140415 | (E-)PDCCH for enhanced coverage of low cost MTC | NEC |  |
| R1-140448 | Coverage enhancements for (E)PDCCH/PUCCH | Qualcomm Inc. |  |
| R1-140483 | Power consumption issue in monitoring PDCCH of enhanced coverage MTC UE | NEC |  |
| R1-140484 | Method for saving battery power in monitoring PDCCH of enhanced coverage MTC UE | NEC |  |
| R1-140501 | (E)PDCCH coverage enhancement for MTC | Panasonic |  |
| R1-140550 | PDCCH Coverage Enhancement | NSN, Nokia |  |
| R1-140551 | PUCCH Coverage Enhancement | NSN, Nokia |  |
| R1-140584 | Necessity of PDCCH for common messages for coverage extended MTC UEs | Sony |  |
| R1-140585 | (E)PDCCH candidates during MTC coverage extension repetition | Sony |  |
| R1-140586 | Common search space RNTI handling in PDCCH repetition | Sony |  |
| R1-140617 | RNTI handling in (E)PDCCH coverage enhancement | NTT DOCOMO |  |
| R1-140641 | Discussion on PUCCH for MTC UEs in coverage enhanced mode | Sharp |  |
| R1-140673 | MTC Coverage improvement for Control Channels | Motorola Mobility |  |
| R1-140744 | On PDCCH/EPDCCH mapping for enhanced coverage MTC UE | Ericsson |  |
| R1-140745 | On enhanced DCI coverage for MTC UE | Ericsson |  |
| R1-140746 | On common control channels for enhanced coverage MTC UE | Ericsson |  |
| R1-140747 | HARQ timing for enhanced coverage MTC UE | Ericsson |  |

##### PDSCH/PUSCH

Not treated

| R1-140030 | Coverage enhancements on PDSCH and PUSCH | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140079 | Paging coverage improvement details for MTC UEs | CATT |  |
| R1-140080 | PDSCH/PUSCH coverage improvement for MTC UEs | CATT |  |
| R1-140156 | Coverage enhancement for PDSCH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140157 | Coverage enhancement for PUSCH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140241 | Discussion on CSI report in coverage enhancement | MediaTek Inc. |  |
| R1-140242 | Discussion on HARQ realization for PUSCH in coverage enhancement | MediaTek Inc. |  |
| R1-140276 | Discussion on PDSCH/PUSCH Coverage Improvement | ZTE |  |
| R1-140309 | PDSCH transmission for MTC coverage enhancement | LG Electronics |  |
| R1-140358 | PDSCH/PUSCH Coverage Enhancement for MTC UEs | Samsung |  |
| R1-140416 | Frequency diversity transmission for enhanced coverage MTC | NEC |  |
| R1-140417 | Uplink Reference Signal Enhancement for Low Cost MTC | NEC |  |
| R1-140449 | Coverage enhancements for PDSCH/PUSCH | Qualcomm Inc. |  |
| R1-140552 | PUSCH Coverage Enhancement | NSN, Nokia |  |
| R1-140674 | MTC Coverage improvement for Data Channels | Motorola Mobility |  |
| R1-140748 | Data transmission for enhanced coverage MTC UE | Ericsson |  |

##### SS

Not treated.

| R1-140413 | Consideration on acquisition of SS and system information for coverage improvement | Intel Corporation |  |
| --- | --- | --- | --- |

##### Other

Not treated.

| R1-140081 | Alternative mechanism to PCFICH and PHICH functionality | CATT |  |
| --- | --- | --- | --- |
| R1-140117 | Coverage improvement for PCFICH and PHICH | Intel Corporation |  |
| R1-140158 | Consideration on coverage enhancement for PCFICH and PHICH | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140205 | Discussion on SIB coverage enhancement | Huawei, HiSilicon |  |
| R1-140667 | Considerations on simplified functionalities of PCFICH for low-cost MTC UEs | ITL Inc. |  |
| R1-140749 | Mode selection for enhanced coverage MTC UE | Ericsson |  |

Friday 14th

Telecom Italia commented that unfortunately they could not attend MTC due to being involved in D2D. They raised the question whether assessment of the cost/complexity that is going to be moved on network side due to MTC low cost UE has been evaluated. Vodafone expressed their surprise why such questioning comes so late at the stage of the work.

Telecom Italia simply requested their comment to be minuted.

### LTE TDD-FDD Joint Operation including Carrier Aggregation

WID in RP-131399.

#### TDD – FDD carrier aggregation

| R1-140315 | Summary of email discussion on DL HARQ timing for TDD-FDD carrier aggregation with self-carrier scheduling | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE and recommends introducing HARQ timing option 1-1. For further details of option 1-1, the following options are recommended.

- CHsel option 2: Bundling ACK/NACK bits for DAI  4

  - Further discussion is needed on solutions for handling missing DAI > 4

- Proposal PF3) For TDD PCell and FDD SCell with HARQ timing option 1, when configured with PF3, maximum number of aggregated cells can be limited for the M>4 cases so that number of ACK/NACK bits on PF3 does not exceed 21.

Discussion: Ericsson  ok with option 1-1 assuming PUCCH format 3 only with M>=4.

Fujitsu also support option 1-1

Huawei  if we follow Ericsson’s view then option 1-2 should be used.

Decision: The document is noted.

| R1-140864 | WF on HARQ timing for TDD-FDD CA | LG Electronics, ETRI, DOCOMO, Nokia, NSN, Samsung, ITL, Orange, NEC |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE.

- When configured with ACK/NACK channel selection, for M>4, UE bundles (logical-AND operation) ACK/NACK bits for DAI  4 into single HARQ ACK/NACK bit to reuse the Rel-11 channel selection table

  - UE behaviour for handling of missing DAI>4 shall be decided between following two alternatives:

    - Alt. 1: When the total number of correctly received PDSCHs exceeds 4, the UE shall use the HARQ-ACK resource value transmitted in the TPC field on the last received PDCCH/EPDCCH for PDSCH scheduling

    - Alt. 2: For M>4, UE can understand if there exists more PDCCH/EPDCCH to be transmitted for PDSCH scheduling in the next DL subframe(s) within the HARQ-ACK bundling window by the value of the TPC field in the DCI transmitted with DAI>=4 for the SCell

- When configured with PF3, maximum number of aggregated cells is limited for the M>4 cases so that number of ACK/NACK bits on PF3 does not exceed 21.

Decision: The document is noted.

Option 1-1) LG, NEC, ZTE, Samsung, Orange, NTT DOCOMO, Nokia*, NSN*, Sharp, ETRI, KDDI, KT Corp., ITL, Ericsson, Fujitsu, Hitachi

Option 1-2) LG, Samsung, Orange, NTT DOCOMO, Huawei, HiSilicon, KDDI, KT Corp., ITL, Hitachi

Option 2c-B) Nokia, NSN, ALU, ASB, CMCC, CATT, CATR, Potevio, Coolpad

Option 2c-B1) Nokia, NSN, ALU, ASB, CMCC, CATT, CATR, Potevio, Coolpad

Option 2c-B2) Huawei, HiSilicon, CMCC

* NNSN is fine with option 1-1 with the following addition (CHsel option 2-2)

CATT – CMCC objected agreeing on option 1-1. Use cases and benefits of option 1-1 are unclear and specification impacts due to higher complexity. CATT asked operators clarifying the use cases of TDD PCell.

Tuesday morning session conclusion

- Majority companies support Option 1-1

- Continue offline discussion until Thursday

For TDD PCell and FDD SCell with HARQ timing option 1, when configured with PF1b with channel selection, HARQ-ACK bits for DAI>=4 for each cell are bundled by logical-AND operation. For PDSCH transmission on the FDD SCell indicated by the detection of PDCCH in subframes k_m, m=4 or m=5, if the total number of detected PDCCHs exceeds 4, the UE shall use the TPC field in the last received PDCCH to determine the HARQ-ACK resource values.

Thursday 13th

| R1-140958 | Alternative WF on HARQ timing for TDD-FDD CA | LG Electronics, NTT, NTT DOCOMO, Ericsson, ETRI, Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE.

- HARQ-ACK timing  for DL self-scheduling of FDD SCell in case of TDD PCell is defined as in following table

| HARQ timing | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0A | - | - | 6, [5] | [5], [4] | 4 | - | - | 6, [5] | [5], [4] | 4 |
| 1* |  |  | 7, 6 | [6], [5], 4 |  |  |  | 7, 6 | [6], [5], 4 |  |
| 2 | - | - | 8, 7, 6, [5], 4 | - | - | - | - | 8, 7, 6, [5], 4 | - | - |
| 3 | - | - | 11, [10], [9], [8], 7, 6 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 11, [10], [9], 8, 7 | 7, 6, 5, 4 |  |  |  |  |  |  |
| 5 | - | - | 13, 12, 11, [10], 9, 8, 7, 6, 5, 4 | - | - | - | - | - | - | - |
| 6 | - | - | [8], 7 | 7, [6] | [6], 5 | - | - | 7 | 7, [6], [5] | - |

- ACK/NACK channel selection can be configured only with the TDD PCell UL/DL configurations which don’t require DL HARQ timing with M>4 on the FDD SCell.

- When configured with PF3, maximum number of aggregated cells is limited for the M>4 cases so that number of ACK/NACK bits on PF3 does not exceed 21.

Discussion: Also supported by KDDI, KT Corp., Sharp, NEC, Nokia, NSN, Hitachi, Sony

Samsung is OK with the addition of the following sub-bullet under second bullet:

  - “HARQ ACK feedback with PUCCH format 1B with channel selection is supported only for configurations #0, #1, and #6

Decision: The document is noted.

| R1-140977 | Way forward on FDD Scell PDSCH HARQ timing with TDD as Pcell | CATT, ALU, ASB, CMCC, CATR, HTC, III, Potevio, Coolpad, Huawei, HiSilicon, Blackberry |  |
| --- | --- | --- | --- |

The document was presented by Zukang Shen from CATT.

- For the case of TDD as Pcell and FDD as Scell and self-carrier scheduling, the PDSCH HARQ timing on the FDD Scell follows a DL reference TDD UL-DL configuration

  - The DL reference TDD UL-DL configuration for the FDD Scell is determined by the following table

| TDD Pcell UL-DL configuration | Allowed DL reference TDD UL-DL configuration for FDD Scell (2 aggregated cells) | Allowed DL reference TDD UL-DL configuration for FDD Scell (more than 2 aggregated cells) |
| --- | --- | --- |
| 0 | 5 | 2 |
| 1 | 5 | 2 |
| 2 | 5 | 2 |
| 3 | 5 | 4 |
| 4 | 5 | 4 |
| 5 | 5 | N/A |
| 6 | 5 | 2 |

Discussion: Also supported by ITRI.

Decision: The document is noted.

Conclusion:

No consensus about TDD Pcell DL HARQ timing for TDD-FDD carrier aggregation with self-carrier scheduling, neither R1-140958 or R1-140977 – some suggested to report and let plenary decide

Fujitsu: Plenary won’t be able to make decision on such details – confirmed by Mr Chair

CATT: let’s plenary decide whether to extend or change the scope of WID

Nokia (rapporteur): requested the above conclusion to be clearly noted into the minutes

Cross-carrier scheduling

| R1-140209 | Summary of email discussion for cross-carrier scheduling for TDD-FDD CA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is just noted.

| R1-140210 | WF on TDD-FDD CA for cross-carrier scheduling | Huawei, HiSilicon, ETRI, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, LG Electronics, Intel, ITL, Samsung, NEC, Sharp, Ericsson, Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei.

- For DL cross-carrier scheduling: agree on Alt DL-A

  - For DL cross-carrier scheduling with PUCCH on PCell-only, the DL HARQ timing of the scheduled serving cell follows the PCell’s timing.

  - If the PCell is TDD, the PCell’s timing is defined as:

    - DL HARQ timing determined according to the PCell’s SIB1 UL/DL configuration, or DL-reference HARQ timing of the PCell for eIMTA

- Proposed agreement for UL cross carrier scheduling if the scheduling serving cell is FDD and the scheduled serving cell is TDD:

  - For UL cross-carrier scheduling where PHICH is transmitted on the scheduling serving cell, the scheduling/HARQ timing of TDD scheduled serving cell follows the TDD scheduled serving cell’s UL/DL configuration.

    - For eIMTA the TDD scheduled serving cell’s UL/DL configuration is the SIB1 UL/DL configuration signaled by tdd-Config-r10

    - PHICH-less operation is carried out for the UL subframes corresponding to IPHICH=1 on the scheduled TDD SCell if the UL/DL configuration of the scheduled TDD SCell is configuration 0.

- Proposed agreement for UL cross carrier scheduling if the scheduling serving cell is TDD and the scheduled serving cell is FDD:

  - For UL cross-carrier scheduling where PHICH is transmitted on the scheduling serving cell, the scheduling/HARQ timing of FDD scheduled serving cell follows:

    - 10ms RTT

      - 4ms between UL grant/PHICH and PUSCH

      - 6ms between PUSCH and PHICH

Decision: The document is noted, modified as follows:

Agreements:

- For FDD Pcell case,.

  - For DL cross-carrier scheduling: agree on Alt DL-A

    - For DL cross-carrier scheduling with PUCCH on PCell-only, the DL HARQ timing of the scheduled serving cell follows the PCell’s timing

  - For UL cross carrier scheduling if the scheduling serving cell is FDD and the scheduled serving cell is TDD:

    - For UL cross-carrier scheduling, the scheduling/HARQ timing of TDD scheduled serving cell follows the TDD scheduled serving cell’s UL/DL configuration.

      - For eIMTA the TDD scheduled serving cell’s UL/DL configuration is the SIB1 UL/DL configuration signaled by tdd-Config-r10

      - PHICH-less operation is carried out for the UL subframes corresponding to  IPHICH=1  on the scheduled TDD SCell if the UL/DL configuration of the scheduled TDD SCell is configuration 0

  - For UL cross carrier scheduling if the scheduling serving cell is TDD and the scheduled serving cell is FDD:

    - For UL cross-carrier scheduling, the scheduling/HARQ timing of FDD scheduled serving cell follows:

      - 10ms RTT

        - 4ms between UL grant/PHICH and PUSCH

        - 6ms between PUSCH and PHICH

Agreements:

- If TDD Pcell self-scheduling is supported, for TDD Pcell case,

  - For DL cross-carrier scheduling: agree on Alt DL-A

    - For DL cross-carrier scheduling with PUCCH on PCell-only, the DL HARQ timing of the scheduled serving cell follows the PCell’s timing

    - PCell’s timing is defined as:

      - DL HARQ timing determined according to the PCell’s SIB1 UL/DL configuration, or DL-reference HARQ timing of the PCell for eIMTA

  - For UL cross carrier scheduling if the scheduling serving cell is FDD and the scheduled serving cell is TDD:

    - For UL cross-carrier scheduling, the scheduling/HARQ timing of TDD scheduled serving cell follows the TDD scheduled serving cell’s UL/DL configuration.

      - For eIMTA the TDD scheduled serving cell’s UL/DL configuration is the SIB1 UL/DL configuration signaled by tdd-Config-r10

      - PHICH-less operation is carried out for the UL subframes corresponding to  IPHICH=1  on the scheduled TDD SCell if the UL/DL configuration of the scheduled TDD SCell is configuration 0

  - For UL cross carrier scheduling if the scheduling serving cell is TDD and the scheduled serving cell is FDD:

    - For UL cross-carrier scheduling, the scheduling/HARQ timing of FDD scheduled serving cell follows:

      - 10ms RTT

        - 4ms between UL grant/PHICH and PUSCH

        - 6ms between PUSCH and PHICH

Friday 14th

| R1-140974 | WF on HARQ feedback for TDD FDD CA | Huawei, HiSilicon, CATT, Samsung, LGE, ZTE |  |
| --- | --- | --- | --- |

The document was presented by Fredrik Berggren from Huawei.

Discussion: Also supported by ITL.

Decision: The document is noted, modified and agreed with the following addition:

Agreements:

- For the FDD Pcell case, and for the TDD Pcell case if TDD Pcell is supported and if PUCCH format 1b with channel selection is applicable,

  - PUCCH format 1b with channel selection and PUCCH format 3 are supported for TDD-FDD carrier aggregation

  - For PUCCH format 1b with channel selection

    - When PCell is using FDD and SCell is using TDD

      - FDD ACK/NACK tables are used

    - When PCell is using TDD and SCell is using FDD

      - TDD ACK/NACK tables are used

  - For PUCCH format 1b with channel selection

    - When PCell is using FDD and SCell is using TDD

      - If there is an associated UL subframe on the SCell, PUCCH format 1a/1b is used as for a single FDD serving cell

      - Otherwise, PUCCH format 1b with channel selection is used

  - For PUCCH format 1b with channel selection and HARQ-ACK+SR transmission in the same subframe

    - When PCell is using FDD and SCell is using TDD

      - Joint HARQ-ACK and positive SR transmission is by PUCCH format 1a/1b

        - If there is an associated UL subframe on the SCell, transmission is performed as for a single FDD serving cell, i.e., spatial bundling is not performed on the SR resource.

        - Otherwise, transmission is performed as for two FDD serving cells, i.e., spatial bundling is performed on the SR resource.

    - When PCell is using TDD and SCell is using FDD

      - Joint HARQ-ACK and SR transmission is applied as for TDD carrier aggregation in Rel-10/11.

  - For HARQ-ACK transmission on PUCCH format 3 or on PUSCH

    - When PCell is using FDD and SCell is using TDD

      - The number of HARQ-ACK bits transmitted in an UL subframe is determined based on the number of serving cells that have an associated DL subframe and the downlink transmission modes configured for each serving cell.

| R1-140960 | Proposal on multiple TA for TDD FDD CA | LGE |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE.

- Multiple TAGs can be configured between TDD cell and FDD cell

  - No additional handling is necessary  to the current specifications to keep the maximum TX timing difference of 32.47us between the TDD cell and FDD cell

- Regarding support of single TAG across TDD cell and FDD cell, the following should be discussed

  - When TDD cell and FDD cell are in a PTAG

    - UE’s TX subframe boundary between TDD cell and FDD cell can be aligned with no additional handling to the current specifications

  - When TDD cell and FDD cell are in a STAG

    - It should be clarified whether additional handling to the current specifications is necessary or not, to align the UE’s TX subframe boundary between TDD cell and FDD cell

Decision: The document is noted, modified and the following is agreed

Agreements

- Multiple TAGs can be configured between TDD cell and FDD cell

  - No additional handling is necessary  to the current specifications to keep the maximum TX timing difference of 32.47us between the TDD cell and FDD cell

- Regarding support of single TAG across TDD cell and FDD cell,

  - When TDD cell and FDD cell are in a PTAG

    - UE’s TX subframe boundary between TDD cell and FDD cell can be aligned with no additional handling to the current specifications

  - When TDD cell and FDD cell are in a STAG

    - UE shall use NTA offset equals 624 Ts for the STAG

DCI format for TDD FDD CA

| R1-140959 | WF on DCI format for TDD FDD CA | LG Electronics, Nokia, NSN, ETRI, NTT DOCOMO |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-141027 | Proposal on DCI format for TDD-FDD CA | LGE |  |
| --- | --- | --- | --- |

The document was presented by Joon Kui Ahn from LGE and is an extension of R1-140959 for DCI format definition to cover full set of TDD-FDD CA combinations. Not co-sourced from other companies than LGE due to lack of time for check.

- DL/UL DCI format for PCell follows existing specification

- In DL/UL DCI for TDD/FDD SCell with FDD PCell and PUCCH on PCell-only

  - DL/UL DAI field does not exist

  - 3 bits HARQ process number field is used

- In DL/UL DCI for TDD/FDD Scell with TDD PCell and PUCCH on PCell-only

  - DL/UL DAI field is used for all TDD PCell UL/DL configurations

  - 4 bits HARQ process number field is used

- In UL DCI for TDD Scell

  - UL index field exists only for TDD Scell UL/DL configuration 0 (replacing UL DAI field if exists)

- In UL DCI for FDD Scell

  - No UL index field exists

- 1 bit SRS request field in DCI format 2B/2C/2D (if aperiodic SRS is configured) exists only when TDD SCell is a scheduled cell regardless whether the scheduling cell is TDD or FDD

Decision: The document is noted.

| R1-141032 | Way Forward on DCI Formats for TDD/FDD CA | Huawei, HiSilicon, ZTE | (R1-140978) |
| --- | --- | --- | --- |

The document was presented by David Mazzarese from Huawei.

- When TDD/FDD CA is configured for a UE, DL/UL DCI formats for SCell include all TDD fields

  - Unused TDD fields are set to predefined value (e.g., 0) or reserved

  - The used TDD fields are according to the Table:

| Pcell | scheduling cell | scheduled cell | UL DCI |  | DL DCI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | UL index | UL DAI | DL DAI | SRS request | HARQ process number |
| Self-carrier scheduling |  |  |  |  |  |  |  |
| FDD | self | TDD | UL index |  |  | 1 bit | 3 bits |
| FDD | self | FDD |  |  |  |  |  |
| TDD | self | TDD |  |  |  |  |  |
| TDD | self | FDD |  | UL DAI | DL DAI |  | 4 bits |
| Cross-carrier scheduling |  |  |  |  |  |  |  |
| FDD |  | TDD | UL index |  |  | 1 bit | 3 bits |
| FDD |  | FDD |  |  |  |  | 3 bits |
| TDD |  | TDD | UL index | UL DAI | DL DAI | 1 bit | 4 bits |
| TDD |  | FDD |  | UL DAI | DL DAI |  | 4 bits |

Decision: The document is noted.

Agreement

- DL/UL DCI format for PCell follows existing specification

Remaining details are for email discussion/approval until Wed 19th February – (LGE), about the fields on DCI format to be needed for TDD-FDD CA.

After that email discussion/approval until 21th February about the DCI format design for TDD-FDD CA.

Email approval until 14th March about RRC parameter lists – (Nokia)

Not treated.

| R1-140031 | Remaining details for TDD-FDD CA | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140032 | Standard impact of TDD-FDD CA for UEs not supporting simultaneous transmission and reception | Huawei, HiSilicon |  |
| R1-140033 | Restriction on combinations of FDD/TDD serving cells for TDD-FDD CA | Huawei, HiSilicon |  |
| R1-140082 | PDSCH HARQ timing of FDD SCell with TDD as PCell | CATT |  |
| R1-140083 | UCI transmission for FDD-TDD carrier aggregation | CATT |  |
| R1-140105 | Half duplex UE operation for FDD-TDD CA | CATT |  |
| R1-140159 | Remaining issues on HARQ-ACK timing for TDD-FDD carrier aggregation | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140208 | Remaining details for self-carrier scheduling when PCell is TDD | Huawei, HiSilicon |  |
| R1-140215 | Remaining issues on HARQ timing for TDD-FDD CA | ETRI |  |
| R1-140229 | On DCI/UCI transmission for TDD-FDD carrier aggregation | China Telecom |  |
| R1-140230 | On specification impact of HARQ design in TDD-FDD CA | China Telecom |  |
| R1-140236 | TDM UL transmission for TDD-FDD DL CA-capable UE with single transmitter | MediaTek Inc. |  |
| R1-140310 | DL HARQ timing for TDD-FDD carrier aggregation with self-carrier scheduling | LG Electronics |  |
| R1-140311 | HARQ timing for TDD-FDD carrier aggregation with cross-carrier scheduling | LG Electronics |  |
| R1-140312 | DCI for TDD-FDD carrier aggregation | LG Electronics |  |
| R1-140313 | UCI for TDD-FDD carrier aggregation | LG Electronics |  |
| R1-140314 | Timing advance for TDD-FDD carrier aggregation | LG Electronics |  |
| R1-140359 | DL HARQ timing for TDD-FDD CA | Samsung |  |
| R1-140360 | Specification impact in case of FDD Pcell for TDD-FDD CA | Samsung |  |
| R1-140361 | CSI feedback for TDD-FDD CA | Samsung |  |
| R1-140362 | SRS transmission for TDD-FDD CA | Samsung |  |
| R1-140363 | UL HARQ timing for TDD-FDD CA | Samsung |  |
| R1-140364 | Specification impact in case of TDD PCell for TDD-FDD CA | Samsung |  |
| R1-140365 | Multiple TA for TDD-FDD CA | Samsung |  |
| R1-140411 | On support of TDD-FDD Carrier Aggregation | Intel Corporation |  |
| R1-140428 | Self-scheduling PDSCH timing in TDD-FDD CA | Potevio |  |
| R1-140429 | Cross-scheduling timing in TDD-FDD CA | Potevio |  |
| R1-140450 | Solution for TDD-FDD CA | Qualcomm Inc. |  |
| R1-140485 | Remaining issues in TDD-FDD carrier aggregation system | NEC |  |
| R1-140534 | Further details on HARQ self-scheduling for TDD-FDD CA | Texas Instruments |  |
| R1-140553 | Handling of timing advance in TDD-FDD CA | Nokia, NSN |  |
| R1-140554 | Remaining details of TDD-FDD CA | Nokia, NSN |  |
| R1-140591 | Discussion on full/non-full duplex UE for TDD-FDD CA | CMCC |  |
| R1-140597 | Discussion on HARQ timing of PDSCH transmission with self-scheduling | CMCC |  |
| R1-140598 | Discussion on supporting of cross-carrier scheduling for TDD-FDD CA | CMCC |  |
| R1-140618 | Design of HARQ feedback for TDD-FDD CA | NTT DOCOMO |  |
| R1-140619 | Remaining issues on TDD-FDD CA | NTT DOCOMO |  |
| R1-140642 | DL HARQ-ACK for TDD-FDD CA with TDD Pcell | Sharp |  |
| R1-140643 | UL scheduling and HARQ-ACK feedback for cross-carrier scheduling | Sharp |  |
| R1-140644 | Support of non simultaneous Rx/Tx capable UEs for TDD-FDD carrier aggregation | Sharp |  |
| R1-140668 | DL HARQ timing of SCell for TDD-FDD CA | ITL Inc. |  |
| R1-140670 | View on DL HARQ timing for self-scheduling in TDD-FDD CA | KT Corp. |  |
| R1-140671 | View on cross-carrier scheduling in TDD-FDD CA | KT Corp. |  |
| R1-140750 | HARQ timing for self-scheduling | Ericsson |  |
| R1-140751 | DCI format design for TDD – FDD CA | Ericsson |  |
| R1-140752 | Cross-carrier scheduling support for TDD – FDD CA | Ericsson |  |
| R1-140824 | Remaining Issues for TDD-FDD CA | ZTE | (R1-140257) |
| R1-140957 | WF on HARQ timing for TDD-FDD CA | LG Electronics, ETRI, NTT, NTT DOCOMO, Nokia, NSN, Samsung, ITL, Orange, NEC, I2R, KT Corp., Sharp, Telefonica, Kyocera, Hitachi |  |
| R1-140978 | Way Forward on DCI Formats for TDD/FDD CA | Huawei, HiSilicon, ZTE |  |

### Small Cell Enhancements – Physical-layer Aspects

WID in RP-132073.

#### Higher order modulation

Mr Chair’s recommendation: Specification of 256QAM in downlink transmission, with focus on MCS/CQI/TBS table design, while keeping existing size of CQI feedback field and MCS indication

| R1-140034 | CQI/MCS table design for 256QAM | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei and the proposals are summarized as follows.

Proposals for CQI table:

- The maximum efficiency for 256QAM CQI entry is 5.5547*8/6 = 7.4063;

- Introduce three new CQI entries for 256QAM

  - The efficiency corresponding to each entry is 6.1719, 6.7891, 7.4063

- CQI table for 256QAM should reserve some entries for low SNR region

  - Current CQI entries #0, #1 and #2 should be reserved

  - Current CQI entries #3, #5 and #7 can be removed

- The modulation order of CQI index 15 is changed to 256QAM

  - The spectral efficiency could be maintained to reduce impact on MCS/TBS table

- Other CQI entries should be remained unchanged

Proposals for MCS table:

- Introduce 8 MCS entries for 256QAM

  - 7 MCS entries (i.e., #27~#33) reflect the corresponding efficiency or interpolated efficiency for the proposed CQI index #12-#15 for 256QAM.

  - 1 MCS entry for 256QAM retransmission

- 8 entries in Rel-8 MCS table should be dropped

  - 7 MCS entries (i.e.,  #1, 3, 5, 7, 9, 9, 11) if the previous CQI entries #3,5,7 are dropped

  - 1 MCS entry (i.e.,  #26) if the modulation order of CQI index #15 is changed to 256QAM

- Other MCS/TBS entries should be remained unchanged

Proposals for signalling: CQI/MCS table and/or 256QAM is configured by higher layer signalling per CC and transmission point

Discussion: CQI table indices concern from Intel (more efficient way may exist)

Decision: The document is noted.

| R1-140555 | On CQI/MCS/TBS table design for 256QAM | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Klaus Hugl from Nokia and proposes:

Proposal 1: With fallback DCI Format 1A legacy MCS tables are used to ensure robust operation during reconfiguration.

Proposal 2 - Overall design principle: Base the new 256QAM MCS/CQI tables on the principle of reusing (most of the) existing entries and replace some low spectral efficiency values with the needed 256QAM entries. Extend the existing TBS table with some larger TBS sizes (I_TBS>26).

Proposal 3: Keep every other low spectral efficiency CQI/MCS entry of the existing tables to retain the ability to signal also CQI indication of several different low SINRs with the new tables, and enable additional legacy MCS fallback scheduling support.

Proposal 4: RAN1 should discuss the applicability of some other Rel. 8 table design principles for the new tables in order to free some additional table entries – including

- Support for reserved MCS values for modulation adaptation in the retransmission (for all or only some modulations)

- Need for overlapping spectral efficiency of different modulations (for all or only some modulations)

Decision: The document is noted.

| R1-140753 | CQI table design for downlink 256QAM | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Ms Yu Yang from Ericsson and proposes:

- Rel-8 CQI 2 should be retained to use TBS 0 for RRC/VoIP robustness.

- Down-sample the lower QPSK region to create spare CQI entries for the 256QAM region.

- Keep current CQI 5—13 unchanged to enable more compatible operations with current implementation.

- Introduce new CQI values to cover equal SNR spacing between Rel-8 CQI 14 rate and the highest 256QAM rate at 8*0.928.

Decision: The document is noted.

| R1-140118 | CQI/MCS/TBS Tables for 256QAM and Relevant Signalling | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and proposes:

Proposal 1: To form the new CQI table, replace several low MCSs of the legacy table with new 256QAM ones.

- Keep the lowest MCS in order to be safe in case the radio link suddenly becomes bad.

- Remove every other MCS rather than consecutive MCSs, which can help maintain a decent link adaptation capability in the low SNR regime.

- Make the CQI indices the same for the common MCSs between the legacy and the new CQI tables.

Proposal 2: To form the new MCS table, replace several low MCSs of the legacy table with new 256QAM ones.

- Keep the lowest MCS in order to be safe in case the radio link suddenly becomes bad.

- Remove every other MCS rather than consecutive MCSs to mitigate the impact of the disability that some low MCSs cannot be addressed in the new MCS table.

- Don’t remove the MCS entries that directly come from the CQI entries as in the legacy MCS table design.

- Remove the higher modulation entry in each overlap region with the same TBS.

- Make the MCS indices the same for the common MCSs between the legacy and the new MCS tables.

Proposal 3: Both semi-static switching and dynamic switching between the legacy and the new CQI/MCS tables should be further studied.

Proposal 4: A new UE category for 256QAM should be defined in TS 36.306. Detailed values of the physical layer parameters for the new 256QAM UE category should be further discussed.

Proposal 5: Define a new RRC signaling indicating the total number of soft channel bits to be used for computing the soft buffer size.

Decision: The document is noted.

| R1-140224 | Higher Order Modulation for Small Cell Enhancement | HTC |  |
| --- | --- | --- | --- |

The document was presented by Richard Li from HTC and proposes:

- Proposal 1: Overlapping TBS indices in modulation and TBS index table is not required for small cells.

- Proposal 2: Switching point from 64QAM to 256QAM should be around code rate 0.65 as legacy switching point.

Decision: The document is noted.

Looking at WFs

| R1-140876 | WF on 256QAM design principles | Huawei, HiSilicon, Nokia, NSN, Ericsson, Blackberry, CATR |  |
| --- | --- | --- | --- |

The document was presented by Ms Yuan Xia from Huawei.

- CQI table

  - Support SE in the entire range from X1 bps/Hz to X2 bps/Hz

  - Down-sample low CQI entries by removing Y entries and add Y new entries for 256QAM region with even spacing

    - Reserve other Rel-8 entries

  - Switching point of 64QAM and 256QAM should be CQI Z (Z=14 or 15)

  - CQI should be ordered according to spectral efficiency

- MCS table

  - Definition of N (N=3 or 4) reserved entries for adaptive retransmission

  - MCS table design should ensure the support of all the VoIP TBS

  - The need of overlapping spectral efficiency of different modulations is FFS depending on whether there are sufficient MCS entries

- TBS table:

  - Reuse as many as possible of current TBS entries with up to around 2% in code rate deviation

  - Overhead assumption, M REs/PRB should be used for PDSCH

  - The new transport block sizes introduced in the specification should follow the Rel-8 principle of QPP size alignment

Discussion: ALU suggested to follow Rel-8 design principles.

Sony: Only link simulation results have been performed on down-sampling CQI, no system simulation results.

Decision: The document is noted.

Continue offline discussion until Thursday – (Huawei)

Possible agreements:

- CQI table

  - Support SE in the entire range from X1 bps/Hz to X2 bps/Hz

  - Down-sample low CQI entries by removing Y entries, and add Y new entries for 256QAM region with even spacing

    - Preserve other Rel-8 entries

  - CQI  #0 to be equaled to out of range

Thursday 13th

| R1-140975 | WF on 256QAM design principles | Huawei, HiSilicon, CATR, Ericsson, Intel, LGE |  |
| --- | --- | --- | --- |

The document was presented by Ms Xia Yuan from Huawei.

- CQI table

  - Support SE in the entire range from X1 bps/Hz to X2 bps/Hz

  - Down-sample low CQI entries by removing Y1 entries, and add Y1 new entries for 256QAM region with even spacing

    - Preserve other Rel-8 entries

  - CQI  #0 to be equaled to out of range

  - Switching point of 64QAM and 256QAM should be CQI Z (Z=14 or 15 in the existing table)

  - FFS the positions of the CQI entries in the Rel-12 CQI table – decide between the following two options

    - Opt-1: order the CQI indices according to the spectral efficiencies

    - Opt-2: keep the CQI indices the same for the common CQIs between Rel-8 and Rel-12 CQI table

- Modulation and TBS index table

  - Definition of N (N=3 or 4) reserved entries for adaptive retransmission

  - Modulation and TBS table design should provide the support of all the VoIP TBS

  - The need of overlapping spectral efficiency of different modulations is  FFS

- TBS table

  - Reuse as many as possible of current TBS entries with up to around 2%  average padding

  - Define overhead assumption (REs/PRB) for PDSCH

  - The new transport block sizes introduced in the specification should follow the Rel-8 principle of QPP size alignment

Discussion: Nokia suggested going bullet by bullet and see what can be agreed.

Decision: The document is noted.

Agreement

- CQI table

  - Support SE in the entire range from X1 bps/Hz to X2 bps/Hz

Down-sample low CQI entries

Supported by Nokia, NSN, InterDigital, Huawei, HiSilicon, CATR, Ericsson, Intel, LGE, BlackBerry, ZTE, Broadcom, Fujitsu, Samsung, CATT, Motorola Mobility, Qualcomm, NVIDIA

Objected by Panasonic (other proposals may exist, testing impacts unclear), ALU (no hurry to agree on such principle at this stage), ASB

Huawei suggested removing “Preserve other Rel-8 entries” – fine to ALU

Panasonic  concern that specification change is high

| R1-140503 | CQI Reporting for 256QAM |  | Panasonic |  |
| --- | --- | --- | --- | --- |

The document was presented by Michael Einhaus from Panasonic and suggests the following:

- Proposal 1: The reporting range for wideband CQI reporting should not be changed for 256QAM.

- Proposal 2: The combination of highest Rel-11 wideband CQI level and positive differential CQI values should
be interpreted as CQI levels representing 256QAM.

- Proposal 3: The same CQI reporting range should be used for all configured CSI measurement sets.

- Proposal 4: Different CSI reporting process for the same serving cells should use the same CQI reporting range.

- Proposal 5:  The CQI reporting range should be configured per serving cell in a UE specific manner.

Decision: The document is noted.

Agreement

- CQI table

  - Down-sample low CQI entries by removing Y1 entries, and add Y1 new entries for 256QAM region with even spacing

    - Note: One company (Panasonic) expressed a concern that tests and implied specification changes are unnecessary high

Agreements

- CQI table

  - CQI  #0 to be equaled to out of range

  - Switching point of 64QAM and 256QAM should be CQI Z (Z=14 or 15 in the existing table)

  - FFS the positions of the CQI entries in the Rel-12 CQI table – decide between the following two options

    - Opt-1: order the CQI indices according to the spectral efficiencies

    - Opt-2: keep the CQI indices the same for the common CQIs between Rel-8 and Rel-12 CQI table

Modulation and TBS index table

Motorola  add FFS the position of Modulation and TBS entries in Rel-12 table

Panasonic  concern with the support of all VoIP TBS

Agreements

- Modulation and TBS index table

  - Definition of N (N=3 or 4) reserved entries for adaptive retransmission

  - Modulation and TBS table design should provide the support of all the VoIP TBS at least for Format 1A, FFS for Format 2x

  - The need of overlapping spectral efficiency of different modulations is  FFS

  - FFS the position of Modulation and TBS entries in the Rel-12 Modulation and TBS table

- TBS table

  - Reuse as many as possible of current TBS entries with up to around [2%] average padding aligned with Rel-10 design

  - Define overhead assumption(s) (REs/PRB) for PDSCH

    - Working assumption: Use [120 REs] per PRB for all new spectral efficiencies except for the highest spectral efficiency

    - FFS: Overhead assumption for the highest spectral efficiency

  - The new transport block sizes introduced in the specification should follow the Rel-8 principle of QPP size alignment

| R1-140867 | WF on 256QAM configuration | LG Electronics, Qualcomm, InterDigital, Hitachi |  |
| --- | --- | --- | --- |

The document was presented by Ms Yunjung Yi from LGE.

- Use of 256QAM MCS/CQI table can be configured for each

  - configured CC

  - measurement subframe set

- In TM10

  - Use of 256QAM MCS table can be configured for the parameter set linked to each PQI field in DCI format 2D

  - Use of 256QAM CQI table can configured for each CSI process

- 256QAM is supported for all TMs

  - DCI 1A is scheduled based on legacy MCS/TBS table

Discussion: Also supported by ZTE.

Decision: The document is noted, modified and agreed as follows:

Agreements:

- Use of 256QAM MCS/CQI table can be configured for each configured CC

- 256QAM is supported for all TMs

  - Working assumption: 256QAM is supported at least for all DCI formats except for DCI format 1A and 1C, and FFS for DCI format 1A

- In TM10

  - FFS: Use of 256QAM MCS table can be configured for the parameter set linked to each PQI field in DCI format 2D

  - FFS: Use of 256QAM CQI table can configured for each CSI process

Not treated.

| R1-140035 | TBS table design for 256QAM | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140036 | Discussion on UE category with introduction of 256QAM | Huawei, HiSilicon |  |
| R1-140084 | Support of 256QAM in Rel-12 | CATT |  |
| R1-140160 | Specification of 256QAM | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140258 | On standard impacts of 256QAM in downlink | ZTE |  |
| R1-140316 | Support of 256QAM in various scenarios | LG Electronics |  |
| R1-140317 | CQI/MCS/TBS table design for 256QAM | LG Electronics |  |
| R1-140366 | Discussion on specification impacts of 256QAM | Samsung |  |
| R1-140409 | Discussion on standard impact of the 256QAM | Coolpad |  |
| R1-140451 | Higher order modulation | Qualcomm Inc. |  |
| R1-140502 | MCS Indication for 256QAM | Panasonic |  |
| R1-140521 | Evaluation and discussion on mechanism to support 256QAM | Hitachi Ltd |  |
| R1-140535 | Modulation mapping for 256QAM | Texas Instruments |  |
| R1-140556 | On operation of 256QAM with dynamic eNB power backoff | Nokia, NSN |  |
| R1-140592 | Discussion on UE category for 256QAM | CMCC |  |
| R1-140593 | CQI/ MCS/TBS table design for higher order modulation | CMCC |  |
| R1-140608 | Details of MCS and TBS design for high order modulation | Broadcom Corporation |  |
| R1-140609 | Details of CQI table and feedback design for high order modulation | Broadcom Corporation |  |
| R1-140620 | Design of CQI/MCS table for 256 QAM | NTT DOCOMO |  |
| R1-140645 | MCS signalling modifications for downlink 256QAM | NVIDIA |  |
| R1-140650 | Design considerations for 256-QAM | InterDigital |  |
| R1-140663 | Views on the specification of 256QAM | CATR |  |
| R1-140675 | On support of DL 256-QAM | Motorola Mobility |  |
| R1-140754 | MCS table design for downlink 256QAM | Ericsson |  |
| R1-140755 | TBS table design for downlink 256QAM | Ericsson |  |
| R1-140756 | On 256QAM UE category handling | Ericsson |  |

#### Small cell on/off and discovery

Mr Chair’s recommendation: Solutions for reduced transition time of small cell on/off in single-carrier or multi-carrier operation, with enhanced discovery of small cells

Not treated.

| R1-140757 | Overview of small cell on/off and discovery signals | Ericsson |  |
| --- | --- | --- | --- |

##### Procedures for small cell on/off transition time reduction

| R1-140037 | Small cell on/off transition time reduction | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei and proposes:

- Enhanced RRC signalling to incorporate DRS-based measurement and synchronization;

Enhanced network-side signalling to incorporate small cell state changes;

- Scell (of CA) and SeNB (of dual connectivity) activation/deactivation procedures considering small cell on/off transition;

- Further improvements of the activation/deactivation configuration mechanisms for a group of UEs;

- Specifying UE monitoring behavior when its Scell/SeNB is deactivated.

Decision: The document is noted.

| R1-140161 | System design for transition time reduction and legacy UEs impacts for small cell ON/OFF | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and proposes:

- Proposal 1: Detection of UEs in the proximity of a dormant cell could be achieved by defining a new discovery procedure based on new discovery signals transmitted by the dormant cell, detection of UL signals by the dormant cell, or a combination of both.

- Proposal 2: New measurement event and measurement reporting mechanism based on new DL discovery signals need to be specified for small cell ON/OFF and cell switchover procedure.

- Proposal 3: New procedures of CSI-RS configuration, QCL, and CSI feedback for the dormant cell are specified for the preparation of turning on small cell and serve switchover from serving cell.

- Proposal 4: Procedures of turning on small cell, service switchover, and data forwarding should be performed concurrently to minimize the transition time.

Decision: The document is noted.

| R1-140557 | Procedures for small cell on/off transition time reduction | NSN, Nokia |  |
| --- | --- | --- | --- |

The document was presented by Timo Lunttila from NSN and proposes:

- Proposal #1: Further work on small cell ON/OFF shall assume semi-static time scales for small cell ON/OFF transitions.

- Proposal #2: Further small cell discovery related work in RAN1 shall focus on discovery signals, including related configuration and measurement aspects.

Decision: The document is noted.

| R1-140452 | Small cell on/off time reduction | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Alex Damnjanovic from Qualcomm and proposes:

- Proposal 1: For CA scenario, Scell state indication is signalled on Pcell using DCI format 1C.

- Proposal 2: Signalled Scell state indication at subframe n-4 is valid for subframe n and all other subsequent subframes belonging to the same frame.

- Proposal 3: Lack of reception of DCI formant 1C with Scell state indication is interpreted as corresponding Scell being in off state.

- Proposal 4: For dual connectivity scenario, RRC signalling is utilized to indicate the state of Pcell_SCG.

- Proposal 5: Optimize PRACH based method to detect presence of UEs by eNBs in off state on Pcell (macro) layer. Alternatively, rely on SRS.

Decision: The document is noted.

| R1-140669 | Enhanced Small Cell on/off procedure for single-carrier and multiple-carrier operation | ITL Inc. |  |
| --- | --- | --- | --- |

The document was presented by … from ITL and proposes:

- Proposal 1: DCI format 3/3A/1C can be used to indicate the small cell on/off state to support the activation/deactivation of CA/ dual connectivity.

- Proposal 2: To reduce the transition time for synchronization in handover case, synchronization information such as slot number offset can included in the RRC configuration of the discovery signal.

- Proposal 3: To reduce the transition time of UL transmission, discovery signal based path loss estimation should be considered for the power control of initial UL transmission. The EPRE value of discovery signal should also be included in the RRC configuration of the discovery signal.

Decision: The document is noted.

| R1-140758 | On Small cell on/off and the New L1 procedure | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Ms Sorour Falahati from Ericsson and proposes:

- Small cell on/off is applicable only to cells that only serve CONNECTED mode UEs.

- A UE can be configured to receive the discovery signal

- When the UE is configured with a discovery signal from the serving cell it can rely only on the discovery signal for cell identification, measuring and deriving RRM and coarse time/frequency synchronization.

- A UE can be configured to receive reference signals to derive QCL properties for demodulation purposes.

  - The UE shall not rely on the presence of any other reference signal(s) for deriving QCL properties in any other subframes.

  - The reference signals may be transmitted in a subframe containing discovery signals or on another subframe.

- For demodulation, the UE cannot rely on any reference signal(s) in any other subframe than the subframe in which it receives the corresponding physical channel.

Decision: The document is noted.

| R1-140969 | Way Forward on Small Cell On/Off Procedures | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

- Small cell with on/off does not serve idle UEs

- DRS-based measurements are supported

  - DRS-based RSRP and RSRQ

  - Incorporate the DRS-based measurements in handover and carrier aggregation activation/ deactivation procedures and dual connectivity defining in RAN2

    - FFS: support of SCG (Secondary Cell Group in dual connectivity) activation/deactivation

  - Specify UE monitoring behavior when DRS(s) is configured for a UE

  - FFS: potential impacts to DRX procedure

- FFS: Cell On/Off states explicitly informed to UE

- Send LS to RAN2 to start RAN2 work

- FFS: other new L1 procedure

Decision: The document is noted.

Agreements:

- No new idle mode UE behavior in Rel-12 small cell enh. WI

- For intra-/inter-freq. RRM measurement, at least DRS-based RSRP measurements are supported

Continue offline discussion until Thursday – Weimin (Huawei)

Possible agreements:

  - Use the DRS-based measurements as a first step of handover and carrier aggregation activation/ deactivation procedures

    - FFS: support of SCG (Secondary Cell Group in dual connectivity) activation/deactivation

  - Specify UE monitoring behavior when DRS(s) is configured for a UE

  - FFS: potential impacts to DRX procedure

- FFS: Cell On/Off states explicitly informed to UE

- Send LS to RAN2 to start RAN2 work

- FFS: other new L1 procedure

Thursday 13th

| R1-141016 | Open issues of Small Cell on/off and discovery | Huawei, HiSilicon, Samsung, Fujitsu |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

- Whether (and how to) support DRS-based RSRQ-like measurements

- Potential RAN2 impacts to use the DRS-based measurements in handover, and carrier aggregation activation/deactivation, and dual connectivity procedure (being defined in RAN2)

- Investigate detailed solution(s) of new L1 procedure for activated Scell operating on/off that further reduces transition time

- Specify UE monitoring behavior when DRS(s) is configured for a UE, e.g.,

  - Whether DRS can be transmitted when cell is on

  - Under what condition(s) UE measurements are based on DRS, and/or CRS

- Whether (and how) cell On/Off states explicitly informed to UE

- Which other signal(s) the UE can assume are transmitted in addition to PSS for enhanced cell discovery

- Whether (and what) network assistance/information provided to UEs for performing enhanced cell discovery

It is proposed:

- For RRM measurements, support DRS-based RSRQ-like measurements

  - Details are FFS

- RAN2/3 impacts

  - RAN2 needs to define signaling of the configurations and assistant information (if supported) of DRS and DRS-based RRM measurement and reporting (including triggering mechanisms)

  - In addition, RAN1 has the impression that additional RAN2/3 impact to use the DRS-based measurements in handover, and carrier aggregation activation/deactivation is not significant if any. This needs RAN2/3 confirmation

  - Dual connectivity procedures that are been defined in RAN2/3 also need to take into account the use of DRS-based RRM measurements

Decision: The document is noted.

Conclusion:

- Followings are open issues of small cell on/off and discovery to reduce the transition time

  - How to support DRS-based RSRQ-like measurements

  - Potential RAN2 impacts to use the DRS-based measurements in handover, and carrier aggregation activation/deactivation, and dual connectivity procedure (being defined in RAN2)

  - Investigate detailed solution(s) of new L1 procedure for activated Scell operating on/off that further reduces transition time

  - Specify UE monitoring behavior when DRS(s) is configured for a UE, e.g.,

    - Whether DRS can be transmitted when cell is on

    - Under what condition(s) UE measurements are based on DRS, and/or CRS

  - Whether (and how) cell On/Off states explicitly informed to UE

  - Which other signal(s) the UE can assume are transmitted in addition to PSS for enhanced cell discovery

  - Details of network assistance/information provided to UEs for performing enhanced cell discovery

  - Other aspects related small cell on/off and discovery include RLM, DRX are FFS

| R1-141021 | LS on small cell on/off and discovery | RAN1, Huawei |  |
| --- | --- | --- | --- |

The document was presented by Weimin Xiao from Huawei.

Decision: The document is noted and the LS is agreed.

Agreement:

- For RRM measurements, support DRS-based RSRQ-like measurements

  - Details are FFS

| R1-141015 | Way Forward on Transition Time Reduction for Small Cell On/Off | Ericsson, Qualcomm |  |
| --- | --- | --- | --- |

The document was presented by Havish Koorapaty from Ericsson.

- New procedures for transition time reduction should meet the following design targets

  - Transition time: A transition time between on/off states of less than one radio frame

  - On/off granularity: A minimum-on-time (after transition from off to on state) and a minimum-off-time (after transition from on to off state) restriction of not more than one radio frame

- Specification impacts should be identified

Decision: The document is noted and is for email discussion until 14th March – Havish (Ericsson)

Not treated.

| R1-140038 | Enhancements of RRM measurements for small cell on/off | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140085 | Discussion on procedures for small cell on/off with transition time reduction | CATT |  |
| R1-140231 | Enhanced procedure for small cell on/off | China Telecom |  |
| R1-140244 | On procedure enhancements for small cell fast on/off operation | MediaTek Inc. |  |
| R1-140245 | Performance evaluation for small cell on/off operation considering procedure and discovery/measurement enhancements | MediaTek Inc. |  |
| R1-140285 | Enhanced small cell on/off mechanisms | ZTE |  |
| R1-140318 | Efficient small cell on/off operation | LG Electronics |  |
| R1-140367 | Overview of small cell on/off scenarios and procedures | Samsung |  |
| R1-140368 | UE RRM and synchronization procedures with small cell on/off operation | Samsung |  |
| R1-140504 | Clarification on small cell on/off remaining issues | Panasonic | (R1-134374) |
| R1-140536 | Necessary procedures for small cell on/off transition time reduction | Texas Instruments |  |
| R1-140580 | Small Cell On/Off Transition Time Reduction Using Fast Handover | CEWiT |  |
| R1-140595 | Discussion on small cell architectures and procedures for small cell on/off and discovery | CMCC |  |
| R1-140621 | Views on transition time reduction for small cell on/off | NTT DOCOMO |  |
| R1-140676 | Small cell on/off procedure | Motorola Mobility |  |
| R1-140857 | Comparison of methods for reduced on/off switching transition time | Hitachi Ltd | (R1-140522) |

##### Discovery signal(s) and measurements

Focus on design criteria and specification of discovery signal(s) and measurements to start corresponding discussions at other WGs

| R1-140558 | On discovery signals and measurements | NSN, Nokia |  |
| --- | --- | --- | --- |

The document was presented by Timo Lunttila from NSN and proposes:

- Cell discovery is targeted for synchronized network and UEs in RRC_CONNECTED.

- Observation 1: The information contained in the UE report of discovered cells needs to be clarified.

- Further RAN1 studies on discovery signals shall focus on enhancing the discovery of dormant small cells based on bursty transmission of PSS/SSS and potentially CRS signals.

- Observation 2: Discovery signals’ impact on legacy UEs’ cell search and measurements requires further study.

Decision: The document is noted.

| R1-140418 | Small cell discovery enhancement | NEC |  |
| --- | --- | --- | --- |

The document was presented by … from NEC and draws the following observations:

- Observation 1: The guidance for discovery signal design is detection performance guarantee with less overhead.

- Observation 2: Enhancements of legacy PSS/SSS for cell discovery are needed.

- Observation 3: CSI-RS or PRS based scheme can provide best orthogonal property due to orthogonal time-frequency resources allocation; while code domain enhanced PSS/SSS scheme can also increase the detection probability.

- Observation 4: Code domain enhanced PSS/SSS scheme consumes less time-frequency resources, which is more desirable.

- Observation 5: Network side enhancement can guarantee performance for legacy UE with proper configuration.

- Observation 6: Signalling enhancement on the access link and backhaul are needed for network side enhancement schemes

Decision: The document is noted.

| R1-140119 | Discussion on small cell discovery and measurement | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Seunghee Han from Intel and concludes that PSS/SSS muting is a promising solution among the potential candidates considering the functions of time/frequency synchronization, UE implementation complexity/cost, UE power consumption, specification impact, detection performance, support of RRC_IDLE UEs, resource overhead, and processing time.

Decision: The document is noted.

| R1-140371 | Discussion on the detection timing of small cell discovery signal | Samsung |  |
| --- | --- | --- | --- |

The document was presented by … from Samsung and proposes the possible mechanisms to derive the detection timing of small cell discovery signals for RRC-CONNECTED UE as following:

- Option 1: The UE derives detection timing for discovery signal based on macro cell reception timing, TA for the macro cell and a timing offset △T.

- Option 2: The UE derives detection timing for discovery signal based on macro cell reception timing and a timing adjustment △T.

- Option 3: The UE derives detection timing for discovery signal based on PSS/SSS of any cell in the same small cell cluster.

Decision: The document is noted.

| R1-140651 | Measurement performance of Discovery Signals | InterDigital |  |
| --- | --- | --- | --- |

The document was presented by Paul Marinier from InterDigital and concludes

- Use of CRS-based RSRQ results in slightly better throughput performance than CSI-RS-based RSRQ (mostly for cell edge UE’s) under the simulated scenario (2a) with small cell on/off enabled

- No conclusive effect is observed when comparing single-shot with averaged measurements for CSI-RS

Decision: The document is noted.

| R1-140971 | WF on Small Cell Discovery | NTT DOCOMO, Ericsson, Qualcomm, Huawei, HiSilicon | (R1-140968) |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo.

- In the enhanced cell discovery procedure, UE can assume multiple signals are transmitted for the purposes of coarse time/frequency synchronization, cell identification and RRM measurement and QCL (when necessary or possible depending on the procedure).

  - UE can assume transmission of PSS (identical to the Rel-8 waveform) at least for coarse time/frequency synchronization.

  - Which other signal(s) the UE can assume are transmitted is FFS.

    - Possible candidates are one or more instances of SSS, CRS, CSI-RS, and PRS.

    - Which signals are used for which purpose is FFS.

- In addition to purposes given by the WID, cell multiplexing capability, UE power efficiency, and impact to legacy UE’s measurement  should be considered when discussing candidate solutions

- Rel-12 discovery procedure is configured only for RRC_CONNECTED UEs.

- Network assistance is provided to UEs for performing enhanced cell discovery. The information provided includes at least rough timing (to be defined by RAN4) information of discovery signal(s)

Discussion: NTT DoCoMo  Suggestion to look at R1-140970.

Intel: What does rough timing mean?

Decision: The document is noted.

| R1-140970 | Timing uncertainty between assisting cell and on/off cell | Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Suzuki Hidetoshi from Panasonic.

- Small cell on/off requires the condition that uncertainty of the timing is up to [2.5 or less] ms between assisting cell and on/off cell.

- FFS among following options.

  - Option1: small cell on/off support up to 2.5ms uncertainty.

    - At least PSS is included  in the discovery signal

  - Option2: small cell on/off support up to [CP length or UE’s propagation delay measurement window] µs uncertainty.

    - PSS may not be included in the discovery signal

  - Option 3: The network indicates option 1 condition or option 2 condition.

Discussion: ZTE  concern having such constraint on uncertainty

Decision: The document is noted.

Agreements:

- In the enhanced cell discovery procedure, UE can assume multiple signals are transmitted

  - UE can assume transmission of PSS (identical to the Rel-8 waveform) at least for coarse time synchronization

  - Which other signal(s) the UE can assume are transmitted is FFS

    - Possible candidates are one or more instances of SSS, CRS, CSI-RS, and PRS

    - Which signals are useful for which purpose is FFS

- In addition to purposes given by the WID, cell multiplexing capability, UE power efficiency, and impact to legacy UE’s measurement can be considered when discussing candidate solutions

- Rel-12 discovery procedure is configured only for RRC_CONNECTED UEs

- FFS: Network assistance is provided to UEs for performing enhanced cell discovery. The information provided includes at least timing related information of discovery signal(s)

Continue offline discussion until Thursday – Hiroki (NTT DOCOMO)

Thursday 13th

| R1-141017 | WF on network assistance and discovery signal | NTT DOCOMO, Huawei, HiSilicon, Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Hiroki Harada from NTT DoCoMo.

- Network assistance is provided to UEs when Rel-12 discovery procedure is configured. The information provided includes at least timing related information of discovery signals

  - FFS: Details of timing related information

    - Accuracy will be defined by RAN4

  - FFS: Other information provided

Discussion: ZTE  FFS network assistance has already been agreed in Wed session.

Decision: The document is noted, modified and agreed as follows:

Agreements:

- Network assistance related to at least timing associated with discovery measurement is provided to UEs for Rel-12 discovery procedure

  - It does not mean that DRS-based measurement reporting based on autonomous UE detection is precluded

  - Accuracy of timing considered for network assistance will be defined by RAN4

  - FFS: What is timing information associated with discovery measurement

  - FFS: Other information provided

Agreements:

- Followings can be considered as candidate signals for each purpose

  - For coarse time synchronization:

    - Candidate signals are PSS, PSS/SSS, PRS

  - For coarse frequency synchronization:

    - Candidate signals are PSS, PSS/SSS, CRS, CSI-RS, PRS

  - For cell/transmission point identification:

    - Candidate signals are PSS/SSS, CRS, CSI-RS, PRS

  - For RSRP measurement:

    - Candidate signals are PSS/SSS, CRS, CSI-RS, PRS

  - For received signal quality (RSRQ) measurement:

    - FFS: Whether RSSI can be measured by symbols including discovery signals or symbols not including discovery signals

  - For any of above purposes, Tx-side enhancement, e.g.,  muting and utilizing multiple instances, and/or Rx-side enhancement, e.g., interference cancellation, can be considered

  - UE can utilize a combination of discovery signals for any of above purposes

Agreements:

- In addition to evaluation assumptions in SI, following evaluation assumptions can be considered to down select candidate signals

  - Propagation delay between a cell and UE should be considered in the evaluations

  - At least top (3) small cell within Y dB RSRP gap should be detectable

    - Values of Y will be given in the evaluations

  - Initial time/frequency offsets  are assumed within +/- 2.5ms and within +/- X ppm

    - Value of X will be given in the evaluations

  - For cell identification

    - The signal should be able to achieve over 90% detection probability for all detectable small cells

  - For RRM measurement

    - The signal should be able to achieve comparable RSRP measurement performance to legacy CRS-based RSRP for all detectable small cells

Not treated.

| R1-140039 | Discovery design options | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140086 | Discussion on small cell discovery | CATT |  |
| R1-140162 | Discovery signals and measurements for small cell on/off | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140191 | Discussion on discovery and measurement for small cell on/off | Fujitsu |  |
| R1-140202 | Discovery signals features for energy-efficient small cell discovery and measurements | Huawei, HiSilicon |  |
| R1-140213 | Discovery signal design for small cell on/off | ETRI |  |
| R1-140243 | On detection rate requirement for small cell discovery | MediaTek Inc. |  |
| R1-140286 | Companion Discovery Signal (CDS) – Specification and Implementation Considerations | ZTE |  |
| R1-140287 | Companion Discovery Signal (CDS) – Operation for Small Cell ON/OFF | ZTE |  |
| R1-140288 | Discussion on timing acquisition for small cell discovery | ZTE |  |
| R1-140319 | Discussions on small cell discovery signal design | LG Electronics |  |
| R1-140369 | Discovery signal requirements for small cells | Samsung |  |
| R1-140370 | Discussion on discovery signal design aspects | Samsung |  |
| R1-140410 | Views on the small cell on/off | Coolpad |  |
| R1-140453 | Small cell discovery | Qualcomm Inc. |  |
| R1-140487 | Small cell discovery signal design | NEC |  |
| R1-140537 | Design criteria for discovery signal and measurements | Texas Instruments |  |
| R1-140587 | Considerations on RRM measurement for small cell on/off | Sony |  |
| R1-140596 | Small cell on/off and the design of discovery signal | CMCC |  |
| R1-140610 | Small cell discovery and measurements | Broadcom Corporation |  |
| R1-140611 | System level evaluations for synchronous discovery signal | Broadcom Corporation |  |
| R1-140622 | Views on discovery signal design for Rel-12 small cell enhancements | NTT DOCOMO |  |
| R1-140759 | On the design of discovery bursts and procedures | Ericsson |  |
| R1-140831 | Performance comparison of small cell discovery signal designs | MediaTek Inc. | (R1-140246) |

#### Radio-interface based synchronization

Mr Chair’s recommendation: Solutions for network listening in single-carrier or multi-carrier operation, to support multiple stratum level beyond 3 hops, e.g. 4 to 6 hops, to improve the achievable synchronization accuracy, and to be applicable to small cell on/off, eIMTA and inter-operator TDD deployment in the same band

| R1-140594 | Discussion on intra-operator and inter-operator synchronization for small cell enhancement | CMCC |  |
| --- | --- | --- | --- |

The document was presented by Xiaodong Shen from CMCC and is summarized as:

Considering intra-operator synchronization, it is proposed that:

- Proposal 1: Radio-interface based synchronization enhancement to support multiple stratum levels beyond 3 hops is necessary, and extending the existent backhaul signalling to support larger stratum levels could be considered as a starting point.

- Proposal 2: If the backhaul signalling for stratum levels is provided to the eNB, then explicit signalling which indicates RS configurations can be considered.

- Proposal 3: Several enhancements for the intra-operator synchronization need to be supported:

  - Support of the schemes to other small cell features, e.g., small cell on/off and eIMTA.

  - Support of the schemes to different vendors.

  - Support of the schemes to the TDD configuration 0.

Considering inter-operator synchronization, it is proposed that:

- Proposal 4: Inter-operator synchronization focuses on the scenarios that different TDD operators are co- deployed in the same band or adjacent band at the same region, including:

  - Synchronization between operator A’s small cells to operator B’s macro eNBs

  - Synchronization between different operators’ small cells

  - Synchronization between different operators’ macro eNBs

- Proposal 5: The TDD operator can provide its synchronization reference timing as the primary synchronization source for other TDD operators.

- Proposal 6: The TDD operator is able to hear from other TDD operators and to judge whether the target eNBs from another operator is synchronized to its own eNBs or not.

Decision: The document is noted.

| R1-140040 | On the support of multiple stratum levels in network listening | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei and proposes:

- Applying implicit or explicit physical layer signalling to indicate synchronization stratum level.

- Indicating network listening RS resource by predefined resource allocation patterns and the target cell performs blind detection of the listening RS among the predefined patterns.

Discussion: LGE  concern with explicit physical layer signalling – how this can be done?

Decision: The document is noted.

| R1-140623 | On the specification support for RIBS | NTT DOCOMO |  |
| --- | --- | --- | --- |

The document was presented by … from NTT DoCoMo and proposes:

- Consider sending a LS to ask RAN3 if the following information elements can be added to the synchronization information elements specified in the RAN3 specifications.

  - Muting/listening pattern

  - Information to identify source cell

- Focus on typical NW listening scenarios. If a scenario is identified in which the Rel.12 RIBS is clearly not sufficient, consider further enhancement of RIBS in the future.

Decision: The document is noted.

Continue offline discussion until Thursday – (Huawei)

Thursday 13th

| R1-140976 | Proposal on network listening for RIBS | Huawei, HiSilicon, CATR, CMCC, BlackBerry, ETRI, CATT |  |
| --- | --- | --- | --- |

The document was presented by Ms Elean Fan from Huawei.

- Support listening among eNBs (at least small cells) of different TDD operators. FFS on the standard impacts.

- Both MBSFN subframes and GPs can be used for target cells to listen to the source cells. FFS on using UL subframes for target cells to listen to the source cells. FFS on the standard impacts.

- Support muting to improve the hearability for the scenario of small cells listening to small cells. Both MBSFN subframes and GPs can be used for muting. FFS on the standard impacts.

- Support not turning off listening RS in the source cell when small cell on/off is used in the network. FFS on the standard impacts.

Discussion: ALU  suggests to take the guidelines as working assumption – further work expected to investigate standard impacts.

Ericsson do not agree to have it as working assumption as a whole – agrees with “support muting to improve the hearability” and “support not turning off listening RS in the source cell when small cell on/off is used” but has concern with the other statements.

Decision: The document is noted.

Agreements:

- Support muting to improve the hearability for the scenario of small cells listening to small cells. FFS on the standard impacts.

- Support not turning off listening RS in the source cell when small cell on/off is used in the network. FFS on the standard impacts.

Not treated.

| R1-140041 | Solutions to improve the hearability of received RS for network listening | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140042 | Discussion on network listening with small cell on/off and eIMTA | Huawei, HiSilicon |  |
| R1-140087 | Radio-interface based synchronization for small cells | CATT |  |
| R1-140163 | System analysis of Network Listening | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140206 | Discussion on network listening for inter-operator TDD deployment | Huawei, HiSilicon, CATR |  |
| R1-140289 | Source cell selection for network listening | ZTE |  |
| R1-140290 | Further consideration on RIBS | ZTE |  |
| R1-140320 | Design considerations on Radio-Interface Based network Synchronization | LG Electronics |  |
| R1-140372 | Discussion on radio-interface based synchronization solutions | Samsung |  |
| R1-140559 | Issues regarding Radio-Interface based Synchronization Design | NSN, Nokia |  |
| R1-140760 | On radio-interface based small cell synchronization | Ericsson |  |
| R1-140828 | Network synchronization by network listening | Qualcomm Inc. | (R1-140454) |

#### Other

Not treated.

| R1-140419 | Views on interference coordination for semi-static small cell on/off | NEC |  |
| --- | --- | --- | --- |

### Physical layer functionalities required for operation of Dual Connectivity

WID in RP-132069.

Chairman’s recommendations: Note that the scope of this WI is the UEs supporting multiple Rx/Tx,

RAN1 decisions for incoming LS (R2-134603) are also discussed.

| R1-140866 | WF of CC synchronization for Dual Connectivity | NTT DOCOMO, Deutsche Telekom, Orange, SoftBank Mobile, eAccess, KT Corp. |  |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo.

- Dual Connectivity should be specified to work under the assumption that frame timing and SFN are not always aligned across MeNB and SeNB.

Discussion: Also supported by Telecom Italia.

DoCoMo suggested adding “In case of CA, CCs are assumed to be synchronized as in Rel-10 and Rel-11 CA manner”, because of operators ‘concerns that CA is being considered for Dual Connectivity.

Qualcomm commented that consequences of unsynchronized frames have not been / should be evaluated before any decision. It can be agreed in principle and let RAN2 studying the consequences and decide upon.

Decision: The document is noted.

Possible working assumption

- Dual Connectivity should support the scenario where OFDM symbol and slot timing are not aligned across MeNB and SeNB.

- In case of CA, CCs are assumed to be synchronized as in Rel-10 and Rel-11 CA manner

LGE: FFS on the availability of the timing knowledge between MeNB and SeNB for small cell on/off operation.

Fujitsu: whatever RAN1 agrees as working assumption, both synchronized and unsynchronized still need to be considered – and spend more time discussing practical design issues.

Qualcomm agreed with Fujitsu’s view.

New possible working assumption

- Dual Connectivity should support the scenario where OFDM symbol and slot timing are not aligned across MeNB and SeNB.

  - SFN-level alignment across MeNB and SeNB is up to RAN2 decision

- Note that it does not change the assumption of SCE WI

Support to include the note: Huawei, HiSilicon, NEC, NTT DoCoMo, Panasonic, Ericsson

Object to keep the note: ALU, ASB, Intel, ZTE, LGE, Samsung

Working assumption:

- Dual connectivity should support the scenarios where UE can assume the maximum received timing difference from MeNB and SeNB is 31.3 + X micro sec

  - Note: The value X is up to RAN4 decision on the potential requirements of synchronization accuracy between MeNB and SeNB

- Dual connectivity should support the scenarios where UE can not assume any maximum timing difference from MeNB and SeNB

- SFN-level alignment across MeNB and SeNB is up to RAN2 decision

- Prepare draft LS to RAN2/4 until Thursday – (NTT DOCOMO)

Thursday 13th

| R1-141004 | Draft LS on RAN1 working assumption on dual connectivity | NTT DoCoMo | R1-141005 |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo

Discussion: NEC  maximum received timing difference value is “30.26”.

Decision: The document is noted and final LS is agreed in R1-141008.

| R1-140088 | RAN1 specification impact in support of dual connectivity | CATT |  |
| --- | --- | --- | --- |

The document was presented by Zukang Shen from CATT and list several areas with L1 impact to support dual connectivity.

Decision: The document is noted.

| R1-140624 | Open issues on dual connectivity | NTT DOCOMO |  |
| --- | --- | --- | --- |

The document was presented by Fred Takeda from NTT DoCoMo.

Decision: The document is noted.

| R1-140232 | Physical layer functionalities to support dual connectivity | China Telecom |  |
| --- | --- | --- | --- |

The document was presented by Huaming Wu from ZTE on behalf of China Telecom and proposes:

- Proposal 1: Common search space can be introduced in one SeNB cell for dual connectivity.

- Proposal 2: SR can be supported in SeNB.

- Proposal 3: PUCCH on SCell should be introduced in one SeNB cell and parallel PUCCH transmission in MeNB and SeNB should be supported for dual connectivity.

- Proposal 4: Separate power control should be supported for MeNB and SeNB for dual connectivity.

- Proposal 5:  “primary SeNB cell” can be defined to support dual connectivity.

Decision: The document is noted.

| R1-140523 | Physical layer enhancement for Dual Connectivity | Hitachi Ltd |  |
| --- | --- | --- | --- |

The document was presented by … from Hitachi and proposes:

Proposal 1: One of serving cell in SeNB (special SeNB cell) should be configured for PUCCH transmission

- At least HARQ-ACK and CSI for SeNB cell(s) should be transmitted on the special SeNB cell

- SR need to be transmitted on the special SeNB cell

Proposal 2: Simultaneous PUCCH transmission on MeNB and SeNB should be supported.

Proposal 3: Separate open loop and closed loop power control should be supported for PUCCH on MeNB and SeNB

- Pathloss reference of PUCCH on special SeNB cell should be DL of special SeNB cell

- TPC command of PUCCH on SeNB should be transmitted on DCI on the SeNB cell

- New power scaling or prioritization mechanism needs to be specified for simultaneous PUCCH transmission on MeNB and SeNB

Proposal 4: If PUSCH is transmitted on MeNB (SeNB), the PUSCH should only include UCI (HARQ-ACK or periodic CSI) of the serving cell associated with MeNB (SeNB).

Proposal 5: Multiple aperiodic CSI requests should be allowed in a given subframe for MeNB and SeNB respectively.

Proposal 6: UE needs to monitor common search space (CSS) on special SeNB cell at least for the purpose of Msg2 reception, i.e. PDCCH and PDSCH configured by RA-RNTI.

Proposal 7: Parallel preamble transmissions need to be supported, or it can be up to UE implementation.

Decision: The document is noted.

| R1-140455 | Physical layer aspects of dual-connectivity | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Alex Damnjanovic from Qualcomm and proposes:

- Proposal 1: Separate PUCCH configuration for PCell and the special Scell.

- Proposal 2: Separate power control for PUCCH on PCell and the special Scell.

- Proposal 3: UCI for SCG can be mapped to PUSCH on the cells belonging to SCG only.

- Proposal 4: TPC commands for the special Scell PUCCH are provided in the DL grants for the special Scell.

- Proposal 5: SCG common search space is configured for a UE on the special Scell carrying PUCCH.

- Proposal 6: MeNB and SeNB are synchronized at the level similar to the cell synchronization in Rel-11 CA.

- Proposal 7: For transmission on a subframe, the UE should be able to utilize any unused power from one eNB towards satisfying the power requirement of the second eNB.

Decision: The document is noted.

| R1-140762 | Power control on dual connectivity | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Daniel Larsson from Ericsson and proposes:

- Proposal 1: Reuse as much as possible carrier aggregation methodology and rules for power control for dual connectivity to prioritize UL channels and perform power scaling.

- Proposal 2: It is feasible to support parallel preamble transmissions for dual connectivity and how to handle parallel PRACH transmission in power limited case is left for FFS.

- Proposal 3: The UE reports PHRs separately to each cell and the corresponding PHR contains the PHR for all UL carriers the UE has configured.

Decision: The document is noted.

| R1-140375 | UL Power Control for Dual Connectivity | Samsung |  |
| --- | --- | --- | --- |

The document was presented by Aris Papasakellariou from Samsung and proposes:

- Proposal 1: Power scaling methods, other than equal power scaling, should be considered for PUSCH transmissions to the MeNB and the SeNB under power limitation.

- Proposal 2: The MeNB configures to a UE a maximum power for transmissions to the MeNB,, and a maximum power for transmissions to the SeNB, .

- Proposal 3: No new PHR formats are introduced for DC. The PHR format is configured independently by each eNB. Triggering is independent for each eNB.

Decision: The document is noted.

| R1-140954 | WF on UCI for Dual Connectivity | InterDigital, Ericsson, Hitachi, NEC, Nokia, NSN, Qualcomm |  |
| --- | --- | --- | --- |

The document was presented by Paul Marinier from InterDigital.

- Uplink control information (UCI) related to the PDSCH/PUSCH operation in SCG is transmitted to the SeNB only

  - HARQ-ACK for PDSCH of SCG cells

  - Periodic and aperiodic CSI of SCG cells

  - Scheduling request (SR) of SCG (if agreed in RAN2)

- HARQ-ACK and CSI related to MCG is transmitted to the MeNB only

  - FFS for SR

- In SCG, UCI transmission rules are supported as in R11, with the Pcell replaced by the “special cell”:

  - Physical channel (PUCCH or PUSCH) in which UCI is transmitted

    - Selection of the cell in which UCI is transmitted in case of UCI on PUSCH

    - Selection of PUCCH resources for HARQ-ACK

  - Periodic CSI dropping rules

  - Handling of UCI combinations

  - HARQ-ACK timing and multiplexing, if simultaneous PUCCH (MeNB) -PUCCH (SeNB) is supported

Decision: The document is noted, modified and agreed as:

Agreements:

- At least following schemes are supported

  - At least the following, uplink control information (UCI) related to the PDSCH/PUSCH operation in SCG is transmitted to the SeNB only

    - HARQ-ACK for PDSCH of SCG cells

    - Periodic and aperiodic CSI of SCG cells

  - HARQ-ACK and CSI related to MCG is transmitted to the MeNB only

  - In SCG, the UCI transmission rules as in Rel-11 are supported, with the Pcell replaced by the pSCell:

    - Physical channel (PUCCH or PUSCH) in which UCI is transmitted

      - Selection of the cell in which UCI is transmitted in case of UCI on PUSCH

      - Selection of PUCCH resources for HARQ-ACK

    - Periodic CSI dropping rules

    - Handling of UCI combinations

    - HARQ-ACK timing and multiplexing

Not treated.

| R1-140043 | Physical layer impact of Dual connectivity - Downlink | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140044 | Physical layer impact of Dual connectivity - Uplink | Huawei, HiSilicon |  |
| R1-140089 | PUCCH enhancements for dual connectivity | CATT |  |
| R1-140120 | Physical layer impact on dual connectivity | Intel Corporation |  |
| R1-140164 | UCI transmission for dual connectivity and PUCCH resource allocation | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| R1-140165 | UL power control and power scaling for dual connectivity | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140192 | Physical layer issues of dual connectivity | Fujitsu |  |
| R1-140193 | Power allocation strategy for power limited UE in dual-connectivity | Fujitsu |  |
| R1-140214 | Physical layers functionalities to support dual connectivity operation | ETRI |  |
| R1-140259 | Physical layer functionalities for Dual Connectivity | ZTE |  |
| R1-140321 | Physical layer aspect of dual connectivity | LG Electronics |  |
| R1-140373 | Parallel random access preamble transmissions in dual connectivity | Samsung |  |
| R1-140374 | Physical layer specification impacts of dual connectivity | Samsung |  |
| R1-140376 | Prioritization of UL Information Types for Dual Connectivity | Samsung |  |
| R1-140377 | UE Capability for Dual Connectivity | Samsung |  |
| R1-140420 | On UE capabilities defined as UE category for dual connectivity | NEC |  |
| R1-140488 | On separate PUCCH and RA procedure for SeNB in dual connectivity | NEC |  |
| R1-140524 | PHY functionalities for dual connectivity | Hitachi Ltd |  |
| R1-140538 | Physical layer support for Dual Connectivity | Texas Instruments |  |
| R1-140560 | Discussion on the LS on random access in dual connectivity | Nokia, NSN |  |
| R1-140561 | UL power control aspects of dual connectivity | NSN, Nokia |  |
| R1-140625 | Views on open issues on dual connectivity | NTT DOCOMO |  |
| R1-140652 | Physical layer functionalities for Dual Connectivity | InterDigital |  |
| R1-140659 | Power scaling in Dual Connectivity | InterDigital |  |
| R1-140761 | Physical layer aspects of dual connectivity | Ericsson |  |
| R1-140855 | Common search space on SeNB for dual connectivity | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent | (R1-140167) |

### Inter-eNB CoMP for LTE

WID in RP-132103.

Mr Chair’s recommendation: Identification of information for which signalling is to be specified by RAN3.

| R1-141012 | Summary of Ad-hoc session on Inter-eNB CoMP for LTE | Ad-Hoc Chairman (Alcatel Lucent) |  |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and shows the outcomes of the different CoMP sessions.

Decision: The document is endorsed and incorporated below with the following updates.

Tuesday morning session

| R1-140045 | Inter-eNB information for supporting CoMP with non-ideal backhaul | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140763 | Signalling Needs for Non-Ideal Backhaul CoMP | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140563 | Inter-eNB signalling design to support coordinated muting | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140090 | Discussion on identification of information for inter-eNB CoMP signalling | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140378 | Signalling for inter-eNB CoMP operation | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140379 | Use of SRS received power for inter-eNB CoMP | Samsung |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140168 | Proposed signalling information for supporting eCoMP with NIB | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell |  |
| --- | --- | --- | --- |
| R1-140264 | Signalling for inter-eNB CoMP | ZTE |  |

Decision: The document is noted.

| R1-140510 | Analysis and proposals for the enhancement of X2 signalling | IAESI, DAC-UPC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140456 | Parameters for backhaul signalling | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140875 | WF on Signalling for Inter-eNB CoMP | Samsung, CATT, CMCC, China Telecom, Deutsche Telekom, SK Telecom, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted. Also supported by China Unicom and Qualcomm

Proposal:

- In order to realize both centralized and distributed coordination in terms of inter-eNB CoMP, at least the following signalling of information should be specified:

  - One or more sets of CSI reports (RI, PMI, CQI) of a set of UEs

  - One or more measurement reports (RSRP) of a set of UEs

  - SRS received power of a set of UEs

  - Indication of resource coordination result

    - Resource allocation [i.e. one node tells an eNB to use / not to use certain resources] in time/frequency/spatial domain

      - FFS what the behaviour of the receiving eNB is

        - QC: at least send back yes or no

  - Used configurations or coordination request of SRS, CSI-RS, DMRS, CSI processes and CSI-IM

  - Indication of coordination result for SRS, ZP CSI-RS, DMRS, CSI processes and CSI-IM configurations

| R1-140877 | WF on CoMP-NIB signalling | LG Electronics, Ericsson, Huawei, HiSilicon, NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- A benefit metric for a cell corresponding to a CoMP hypothesis can be signaled between eNBs to facilitate CoMP with non-ideal backhaul.

  - A benefit metric for a cell corresponds to a CoMP hypothesis and is associated with a frequency/time resource.

  - The benefit metric for a cell represents the quantified benefit that the cell of the sender eNB expects in its scheduling when a corresponding CoMP hypothesis is assumed at the associated frequency/time resource.

    - Note: How to react to a received benefit metric signaling is up to receiver eNB’s implementation.

  - An associated CoMP hypothesis can correspond to, e.g.,

    - Power allocations (including muting) for cell(s)

- Enhanced RNTP can be separately signaled between eNBs to facilitate CoMP.

  - Enhanced RNTP can be used to coordinate PDSCH

    - Coordination of EPDCCH may also be considered

  - Information granularity of the Enhanced RNTP is extended to the frequency/time domain.

  - Information in the Enhanced RNTP is (optionally multi-level) transmit power threshold for only the sender eNB.

  - Possible enhancement on existing Status report, which can be signaled between eNBs to exchange the usage status of the indicated frequency/time resources

Possible way forward for further discussion until Thur:

- At least the following signalling is provided from one node to an eNB (additional other items not precluded - FFS):

  - A CoMP hypothesis comprising a hypothetical resource allocation at at least the receiving eNB (e.g. in time/frequency/spatial domains)

  - A benefit metric (definition FFS) associated with the CoMP hypothesis, quantifying the benefit if the receiving eNB implements (its part of) the CoMP hypothesis

    - Note: How to react to a received benefit metric signaling is up to receiving eNB’s implementation.

    - FFS whether the receiving eNB can send back a response e.g. “yes/no” to the sending node.

- At least the following signalling is also provided from one node to another:

  - One or more sets of CSI reports (RI, PMI, CQI) of a set of UEs

Work on a converged proposal (e.g. starting from the above) until Thur.

Continue discussion on either Thur pm or Fri.

Thursday 13th

| R1-140973 | WF on CoMP-NIB signaling enhancements | LG Electronics, Ericsson, Fujitsu, Huawei, HiSilicon, Hitachi, IAESI, NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- Enhanced RNTP can be signaled between eNBs to facilitate CoMP.

  - Enhanced RNTP can be used to coordinate PDSCH

    - Coordination of EPDCCH may also be considered

  - Information granularity of the Enhanced RNTP is extended to the frequency/time domain

  - Information in the Enhanced RNTP is (optionally multi-level) transmit power threshold for only the sender eNB.

  - Possible enhancement on existing Status report, which can be signaled between eNBs to exchange the usage status of the indicated frequency/time resources

- If additional signaling is to be considered (e.g., benefit metric), further discussion is needed.

| R1-141006 | WF on Signalling for Inter-eNB CoMP | Samsung, CATT, CMCC, China Telecom, China Unicom, Deutsche Telekom, ETRI, KDDI, Qualcomm, SK Telecom, Telecom Italia, ZTE | (R1-140875) |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- In order to realize both centralized and distributed coordination in terms of inter-eNB CoMP, at least the following signalling of information should be specified:

  - At least the following signalling is provided from one node to another:

    - A CoMP hypothesis comprising a hypothetical resource allocation at at least the receiving node (e.g. in time/frequency/spatial domains)

    - A benefit metric (definition FFS) associated with the CoMP hypothesis, quantifying the benefit if the receiving node implements (its part of) the CoMP hypothesis

      - Note: How to react to a received benefit metric signaling is up to receiving node’s implementation.

      - The receiving node can send back a response e.g. “yes/no” to the sending node.

    - One or more sets of CSI reports (RI, PMI, CQI) of a set of UEs

    - One or more measurement reports (RSRP) of a set of UEs

    - SRS received power of a set of UEs

    - Used configurations or coordination request of SRS, CSI-RS, DMRS, CSI processes and CSI-IM

    - Indication of coordination result for SRS, ZP CSI-RS, DMRS, CSI processes and CSI-IM configurations

Proposal:

- Enhanced RNTP can be signaled between eNBs to facilitate CoMP.

  - Enhanced RNTP can be used to coordinate PDSCH

    - Coordination of EPDCCH may also be considered

  - Information granularity of the Enhanced RNTP is extended to the frequency/time domain

  - Information in the Enhanced RNTP is (optionally multi-level) transmit power threshold for only the sender eNB.

  - Possible enhancement on existing Status report, which can be signaled between eNBs to exchange the usage status of the indicated frequency/time resources

- A hypothetical resource allocation at at least the receiving node (e.g. in time/frequency/spatial domains)

- One or more sets of CSI reports (RI, PMI, CQI) of a set of UEs

- One or more measurement reports (RSRP) of a set of UEs

- Further discussion is needed if additional signaling is to be considered, e.g.:

  - Per cell benefit metric

Proposal:

- A hypothetical resource allocation at at least the receiving node (e.g. in time/frequency/spatial domains)

  - The receiving node can send back a response e.g. “yes/no” to the sending node.

- One or more measurement reports (RSRP) of a set of UEs

- Further discussion is needed if additional signaling is to be considered

Supported by: Qualcomm, Samsung, ZTE, Deutsche Telecom, CMCC, CATT

Objections: NSN, IAESI, Ericsson, Huawei

Proposal:

- Enhanced RNTP can be signaled between eNBs to facilitate CoMP.

  - Enhanced RNTP can be used to coordinate PDSCH

    - Coordination of EPDCCH may also be considered

  - Information granularity of the Enhanced RNTP is extended to the frequency/time domain

  - Information in the Enhanced RNTP is (optionally multi-level) transmit power threshold for only the sender eNB.

  - Possible enhancement on existing Status report, which can be signaled between eNBs to exchange the usage status of the indicated frequency/time resources

Supported by: NSN, Ericsson, Hitachi, Huawei, IAESI, LGE

Objections: Samsung, ZTE, CATT, ALU

Conclusion:

Send an LS to RAN3 cc RAN - R1-140951 (Samsung):

“RAN1 has not yet been able to identify any common denominator among the different proposals for signalling information for inter-eNB CoMP.”

Friday 14th

| R1-140951 | [draft] LS on Inter-eNB CoMP for LTE | Samsung | R1-141048 |
| --- | --- | --- | --- |

Decision: The document is noted and final LS is agreed in R1-141048.

Not treated.

| R1-140121 | Discussion on backhaul link signalling for downlink CoMP | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-140322 | Backhaul signalling design considerations for CoMP with NIB | LG Electronics |  |
| R1-140323 | Further evaluation for DL CoMP with NIB | LG Electronics |  |
| R1-140380 | Performance comparison between inter-eNB CoMP schemes in SCE scenario 2a | Samsung |  |
| R1-140381 | Performance comparison between inter-eNB CoMP schemes in CoMP scenario 2 | Samsung |  |
| R1-140421 | Signalling Considerations for CoMP with non ideal backhaul | NEC |  |
| R1-140525 | CoMP Signalling with Non-Ideal Backhaul | Hitachi Ltd |  |
| R1-140539 | On signalling for inter-eNB CoMP | Texas Instruments |  |
| R1-140562 | Discussion on architectures supporting inter-eNB CoMP | NSN, Nokia |  |
| R1-140564 | Inter-eNB Signalling for EPDCCH Coordination | NSN, Nokia |  |
| R1-140601 | General views on backhaul signalling for CoMP-NIB | CMCC |  |
| R1-140604 | Partial Measurement Reports Exchange for CoMP-NIB | CMCC |  |
| R1-140626 | Details on Backhaul Signalling for Inter-eNB CoMP | NTT DOCOMO |  |

### Study on 3D-channel model for Elevation Beamforming and FD-MIMO

SID in RP-130811.

| R1-141013 | Summary of Ad-hoc session on 3D-channel model for Elevation Beamforming and FD-MIMO | Ad-Hoc Chairman (Qualcomm) |  |
| --- | --- | --- | --- |

The document was presented by Wanchi Chen from Qualcomm and reports the outcomes of the 3D ad-hoc session.

Decision: The document is noted.

| R1-140565 | TP for 36.873 | NSN, Nokia |  |
| --- | --- | --- | --- |

Decision: The document is noted.

- For mode 2, add a note that  =

- Right after equation (20), update N(,) to N()

The TP is endorsed with the above two updates.

Email discussion until 21/2/2014 to incorporate additional agreements of this meeting to the TR – (NSN)

#### Remaining details of fast fading modelling for 3D-UMa and 3D-UMi

| R1-140091 | Clarification on fast fading modelling for 3D-UMa and 3D-UMi | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

- Taking O-to-I LOS as NLOS case in fast fading generation

- without LOS component is used when determining two strongest clusters,

- with LOS component is used when calculating the angle spreads

- To clarify the first bullet in 36.873

| R1-140324 | Discussion on UE-side orientation and antenna pattern | LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- Confirm the following working assumptions

- For distributions on UE orientation angles, ΩUT, uniformly distributed on [0,360] degree, ΩUT, = 90 degree, ΩUT, = 0 degree are used for calibration and baseline performance evaluations

- For UE antenna gain pattern in local coordinate systems, isotropic antenna gain pattern A’(θ’,ф’) = 1 is used for calibration and baseline performance evaluations

| R1-140849 | Some investigations on angular sampling method for 3D spatial cluster | Anite Telecoms Ltd. | (R1-140805) |
| --- | --- | --- | --- |

Decision: The document is noted.

Not treated.

| R1-140967 | WF on UE-side assumptions for 3D channel model | LG Electronics, Fraunhofer, ETRI, Anite |  |
| --- | --- | --- | --- |

#### Remaining simulation assumptions for channel model calibration / baseline

| R1-140961 | Discussion on wrapping method for 3D channel model | Huawei, HiSilicon | (R1-140046) |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140842 | Discussions on Wrapping Methodology | Ericsson | (R1-140764) |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- For each of the geographical and radio based wrapping methods, a reference describing the methods is provided in the TR, Section 8, where the RSRP formula is.

- For calibration and baseline simulations:

  - Both geographical and radio based simulations results are included in the TR

    - Geographical based wrapping is mandatory, while radio based wrapping is optional (companies are strongly encouraged to provide results for both methods)

- Bishwarup to include reference R1-140842 for each of the wrapping method

- For the wrapping method, different large scale maps are used for each of the wrapping candidate.

| R1-140092 | Clarification on simulation assumptions for channel model calibration / baseline | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Agreement:

- The definition of ZSD/ZSA is based on the CAS defined in 25.996, and the definition of ZSD/ZSA should be defined in the TR

  - For , it assumes the omni-antenna pattern

- The above should be captured in the TR

- For the case of K=1, The 4 antenna ports are mapped such that [0, 2; 1, 3]

- For the case of K =10, the 4 antenna ports are mapped such that [0, 2, 1, 3];

  - Where 0/1 are -45 degree

- Capture the above in the TR

Not treated.

| R1-140325 | Discussion on geographical distance and radio distance based wrapping | LG Electronics |  |
| --- | --- | --- | --- |
| R1-140489 | Discussion on additional Phase2 calibration statistics | NEC |  |
| R1-140653 | Remaining Issues on UE Attachment | InterDigital |  |
| R1-140765 | Polarized Antenna Modelling | Ericsson |  |

#### Phase 1 and Phase 2 calibration results

| R1-140843 | Summary of Phase 1 and Phase 2 calibration results from [75-29] | NSN, Nokia | (R1-140566) |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

- Phase 1 calibration is completed. Bishwarup to capture the phase 1 calibration results in the TR

  - Simulation assumptions and the mean CDF curves

- For Phase 2 calibration results, companies are encouraged to further update phase 2 calibration results (via email submission until 28/2/2014) - NSN

  - Adding coupling loss in phase 2 calibration results

Not treated.

| R1-140093 | Phase 1 calibration results for 3D channel modelling | CATT |  |
| --- | --- | --- | --- |
| R1-140122 | Phase one calibration results for 3D channel model | Intel Corporation |  |
| R1-140123 | Phase two calibration results for 3D channel model | Intel Corporation |  |
| R1-140169 | Phase 1 and Phase 2 Channel Calibration results for 3D UMa and 3D UMi | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140282 | Phase 1 and Phase 2 calibration results of 3D Channel | ZTE |  |
| R1-140326 | Evaluation results for phase 1 and phase 2 calibration | LG Electronics |  |
| R1-140382 | Phase-1 calibration results | Samsung |  |
| R1-140422 | Phase-2 calibration results for fast fading parameters | NEC |  |
| R1-140459 | Phase 1 and Phase 2 calibration results | Qualcomm Inc. |  |
| R1-140526 | 3D UMa/UMi Phase 1 Calibration Results | Hitachi Ltd |  |
| R1-140602 | Updated Phase 1 calibration results for 3D-UMa and 3D-Umi | CMCC |  |
| R1-140603 | Phase 2 calibration results for 3D-UMa and 3D-Umi | CMCC |  |
| R1-140660 | Phase 1 calibration results of 3D MIMO | CATR |  |
| R1-140766 | Calibration Results | Ericsson |  |
| R1-140830 | Initial calibration results for 3D channel modelling | Fujitsu | (R1-140194) |
| R1-140844 | Phase 1 and Phase 2 calibration results | NSN, Nokia | (R1-140567) |
| R1-140850 | Phase 1 and 2 Calibration Results for 3D Channel Model | InterDigital | (R1-140654) |
| R1-140858 | 3D UMa Phase 2 Calibration Results | Hitachi Ltd | (R1-140527) |
| R1-140874 | Phase 2 calibration results for 3D channel modelling | CATT | (R1-140094) |
| R1-140932 | Phase 1/2 Calibration Results for 3D-Channel Model | NTT DOCOMO | (R1-140627) |
| R1-140962 | Phase 1 calibration results for 3D channel model | Huawei, HiSilicon | (R1-140047) |
| R1-140963 | Phase 2 calibration results for 3D channel model | Huawei, HiSilicon | (R1-140048) |
| R1-140966 | Phase-2 calibration results | Samsung | (R1-140383) |

#### Initial baseline results

| R1-140845 | Summary of Initial baseline results from [75-29] | NSN, Nokia | (R1-140568) |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

- For baseline results, companies are encouraged to further update baseline simulation results (via email submission until 28/2/2014) - NSN

Not treated.

| R1-140095 | Initial baseline results for 3D channel modelling | CATT |  |
| --- | --- | --- | --- |
| R1-140124 | Initial baseline calibration results for 3D channel model | Intel Corporation |  |
| R1-140170 | Initial baseline performance for 3D UMa and 3D UMi | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140283 | Initial baseline results of 3D Channel | ZTE |  |
| R1-140327 | Initial throughput results for baseline performance | LG Electronics |  |
| R1-140384 | Calibration results for baseline evaluation | Samsung |  |
| R1-140423 | Baseline performance results | NEC |  |
| R1-140460 | Initial baseline results | Qualcomm Inc. |  |
| R1-140528 | 3D UMa Initial Baseline Results | Hitachi Ltd |  |
| R1-140605 | Initial baseline performance evaluation results | CMCC |  |
| R1-140767 | Baseline Performance Results | Ericsson |  |
| R1-140846 | Initial baseline results | NSN, Nokia | (R1-140569) |
| R1-140851 | Initial Baseline Results for 3D Channel Model | InterDigital | (R1-140655) |
| R1-140964 | Baseline performance evaluation results | Huawei, HiSilicon | (R1-140049) |

#### Details of 3D UMa with one high rise per sector with 300m ISD

| R1-140599 | Summary of email discussion [75-14]: High-rise scenario for 3D-channel | CMCC |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140972 | WF on Channel Modeling for High Rise Scenario | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, CATR, CATT, CHTTL, CMCC, Huawei, HiSilicon, Hitachi, INTEL, LGE, NEC, NTT DoCoMo, Samsung, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Continue discussion

| R1-140204 | Details of channel modelling for high rise scenario | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140768 | LOS/NLOS Determination for the High-Rise Scenario | Ericsson |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Not treated.

| R1-140125 | UE movement in high rise scenario | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-140171 | Channel modelling details for the high-rise 3D UMa scenario | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140195 | Details of 3D UMa with one high rise per sector with 300m ISD | Fujitsu |  |
| R1-140284 | Discussion on channel model for high rise scenario | ZTE |  |
| R1-140328 | Discussion on channel modelling for 3D UMa high rise scenario | LG Electronics |  |
| R1-140385 | Discussions on high-rise scenario | Samsung |  |
| R1-140461 | Details of 3D UMa with one high rise per sector | Qualcomm Inc. |  |
| R1-140529 | Details of 3D UMa with One High Rise per Sector | Hitachi Ltd |  |
| R1-140570 | Details of 3D UMa with one high rise per sector with 300m ISD | NSN, Nokia |  |
| R1-140600 | Measurement and Discussions on Small Scale Parameters for High Rise Scenario | CMCC |  |
| R1-140606 | Discussion on NLoS PL based on Ray Tracing | CMCC |  |
| R1-140628 | Views on 3D UMa with one high-rise per sector with 300m ISD | NTT DOCOMO |  |
| R1-140769 | Scatterer Modelling for the High-Rise Scenario | Ericsson |  |
| R1-140770 | Modelling Considerations for the High-Rise Scenario | Ericsson |  |

#### Other

Not treated.

| R1-140126 | Potential performance improvement in FD-MIMO systems | Intel Corporation |  |
| --- | --- | --- | --- |

### Study on LTE Device to Device Proximity Services

SID in RP-122009.

| R1-141014 | Summary of Ad-hoc session on Study on LTE Device to Device Proximity Services | Ad-Hoc Chairman (Alcatel Lucent) |  |
| --- | --- | --- | --- |

The document was presented by Matthew Baker from ALU and shows the outcomes of the different D2D sessions.

Decision: The document is endorsed and incorporated below.

| R1-140771 | Overview of D2D functions and standardization impact | Ericsson |  |
| --- | --- | --- | --- |
| R1-140772 | Design Principles for Network Control of D2D Transmission | Telecom Italia, Ericsson |  |

#### D2D communication

Mr Chair’s recommendations: Focus on broadcast D2D communication for the public safety use case, on the understanding that basic groupcast and relay functionality (for network-UE relay case) are supported by broadcast D2D communication.

If possible, consider optimisations to enhance efficiency of the relay operation.

Note that impact to existing operator services and resources is included in the evaluations.

##### Synchronization design

Remaining aspects of procedure, and details of D2DSS and possible PD2DSCH, including any relevant evaluations.

Procedure

| R1-140899 | WF on Timing Synchronization of D2D Broadcast | Samsung, Intel, Nokia, III |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working Assumption:

Synchronization sources which are UEs within network coverage have a higher priority than synchronization sources which are UEs outside network coverage

| R1-140937 | Way Forward on D2D synchronization | Huawei, HiSilicon, CATR, Nokia, NSN |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- Multi-hop synchronization is supported. When a UE is selecting its transmitter timing, the sources with timing reference derived originally from eNBs have higher priority than the sources with timing reference derived originally from UEs out of coverage

  - An in-coverage UE can transmit D2DSS with the timing of D2DSS derived from the eNB of the cell that the UE is camping on, in case that it detects a D2DSS from an out-of-coverage source.

    - Such a transmission is under eNB control

  - FFS: Maximum hop number (but is finite)

Questions on:

- How mobility of UEs is handled

- How and how often UEs notify their stratum level

- How many synchronisation sources /references is the UE receiver assumed to traack?

Discuss further until RAN1#76bis.

| R1-140939 | WF on Timing Alignment for the UE on Edge of Network Coverage | ITRI, Huawei, HiSilicon, III |  |
| --- | --- | --- | --- |

Decision: The document is noted. Discuss further until RAN1#76bis:

- When a UE receives two D2DSSs with misaligned timing from synchronization sources with different types, e.g. eNB and out-of-coverage UE, the UE will transmit D2DSS with timing based on the eNB

  - FFS if timing alignment procedure is necessary to prevent potential interference from the UEs out of network coverage

Questions:

- How misaligned is “misaligned”?

| R1-140938 | WF on Triggering Condition for Transmitting D2DSS | ITRI, III |  |
| --- | --- | --- | --- |

Decision: The document is noted. Discuss further until RAN1#76bis:

- A UE will determine if the signal strength from eNodeB is below XdBm and determine if the signal strength from D2D UE transmitting D2DSS is below YdBm

  - If both conditions are met, the UE can transmit D2DSS

    - X and Y are FFS

  - Measurement of signal strength is FFS

| R1-140895 | WF on D2DSS selection | LG Electronics, Qualcomm, Fujitsu, ETRI, CATT, General Dynamics Broadband UK |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working assumption:

  - After giving priority to synchronisation sources which are eNBs, followed by UEs within network coverage, selection of D2DSS for derivation of D2D transmission timing is based on at least the following metrics:

    - Received D2DSS quality: e.g. A UE selects a D2DSS with a better received signal quality when all the other metrics are the same.

    - FFS: Stratum level: A UE selects a D2DSS with a smaller stratum level when all the other metrics are the same.

  - FFS on further detailed D2D selection criterion.

  - FFS whether to define the measurement for received D2DSS quality.

  - FFS on how synchronization source type and stratum level can be carried by D2DSS/PD2DSCH.

Friday 14th

| R1-140896 | WF on D2D synchronization signal format | LG Electronics, Qualcomm |  |
| --- | --- | --- | --- |

The document was presented by Hanbuyl Seo from LGE.

- A D2DSS in a subframe consists of at least N symbols PD2DSS where N is larger than 1.

  - FFS on the resource mapping details

  - FFS whether N can be different in different scenarios (e.g., CP length, network coverage).

- Working assumption on SD2DSS is revisited.

  - SD2DSS is an M sequence or a ZC sequence if supported.

Discussion: Companies supporting the addition of one more sequence  LGE, Qualcomm, Sharp, ETRI

Objecting to change working assumption: Broadcom, Ericsson, ALU, ASB, Huawei, HiSilicon, Samsung, ITRI, Intel, ZTE.

Decision: The document is noted. Continue discussion until RAN1 #76 bis meeting

| R1-140930 | WF on D2D Synchronization | LG Electronics, Ericsson, CATT |  |
| --- | --- | --- | --- |

The document was presented by Hanbuyl Seo from LGE.

Decision: The document is noted. Continue discussion until RAN1 #76 bis meeting

Detailed signal and channel design

| R1-140462 | Signal Design for D2D Synchronization | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Saurabh Tavildar from Qualcomm and proposes:

- Proposal 1: a D2DSS with low duty cycle is proposed to consist of at least two consecutive repeated PD2DSS transmission to enable symbol-level search, and additional PD2DSS symbols separated in time for imptoved synchronization performance

- Proposal 2: PD2DSCH should mostly reuse PUSCH signaling.

- Proposal 3: D2DSS does not need to consist of a SD2DSS.

Decision: The document is noted.

| R1-140612 | Synchronization Design for D2D Communication | Broadcom Corporation |  |
| --- | --- | --- | --- |

The document was presented by Daewon Lee from Broadcom and proposes:

- Proposal 1: Adopt the following principles for the synchronization design in D2D communication:

  - Limit requirements related to channel raster, not to force UE to search 100 kHz carrier raster as in the current LTE downlink for supported bands. Supported carrier frequencies could be e.g. pre-configured in public safety applications.

  - Limit the candidate frequency resources on which the synchronization signals for initial signal detection and synchronization are always transmitted. Do this even if frequency domain hopping is used for data.

  - Limit the number of possible primary and secondary synchronization signals.

  - No real justification to identify UE from synchronization signals exists similarly as eNB in physical layer – UE identification to be done in higher layers.

  - Select one cyclic prefix length motivated by the coverage targets by public safety system.

- Proposal 2: At least PD2DSS/SD2DSS are OFDM based in D2D.

Decision: The document is noted.

| R1-140839 | On the Design of D2DSS and PD2DSCH | LG Electronics | (R1-140329) |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE and proposes:

- Proposal 1: The D2DSS consists of PD2DSS and SD2DSS that are symbol-level repetition of LTE PSS and SSS within a subframe.

- Proposal 2: SD2DSS is necessary to distinguish synchronization source and the root indices of PD2DSS and SD2DSS are used to deliver the information on the ID of the synchronization source.

- Proposal 3: PD2DSCH is transmitted to convey the information such as D2D communication system bandwidth, location of D2D subframes, transmission power, PD2DSCH-transmitter’s synchronization type, hop counter, existence of relay-capable UE, and expiration time.

- Proposal 4: PD2DSCH could be demodulated using D2DSS that surrounds PD2DSCH in the time domain.

Decision: The document is noted.

Not treated.

| R1-140050 | System considerations for D2D synchronization | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140096 | Discussion on D2D synchronization design | CATT |  |
| R1-140127 | Discussion on D2D synchronization procedure | Intel Corporation |  |
| R1-140172 | Remaining considerations for design of D2D Synchronization and UE Identification | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140207 | Design considerations for D2DSS | Huawei, HiSilicon |  |
| R1-140216 | D2D Synchronization Signals | ETRI |  |
| R1-140225 | D2D Synchronization between in and out of coverage UEs | HTC |  |
| R1-140233 | Discussion on synchronization for D2D communication | China Telecom |  |
| R1-140269 | Synchronization design for D2D Broadcast Communication | ZTE |  |
| R1-140330 | Discussion on D2D Synchronization Procedure | LG Electronics |  |
| R1-140404 | D2D Timing Selection and Timing Forward | ITRI |  |
| R1-140405 | A Consideration of OFDM vs. SC-FDMA based on PSS/SSS-structured D2DSS | ITRI |  |
| R1-140463 | Resource allocation for D2D Synchronization | Qualcomm Inc. |  |
| R1-140464 | Multi-hop D2D synchronization performance | Qualcomm Inc. |  |
| R1-140505 | Rough synchronization procedure in D2D | Panasonic | (R1-135398) |
| R1-140571 | On D2D synchronization design | Nokia, NSN |  |
| R1-140588 | Remaining issues in D2D synchronization | Sony |  |
| R1-140656 | Synchronization design remaining aspects | InterDigital |  |
| R1-140773 | Synchronization Procedures for D2D Discovery and Communication | Ericsson |  |
| R1-140774 | Synchronization Signals and Channel Design for D2D Discovery and Communication | Ericsson |  |
| R1-140834 | Evaluation results of timing synchronization for D2D broadcast communication | Samsung | (R1-140386) |
| R1-140847 | D2D Synchronization signal relay by UE | Samsung | (R1-140387) |
| R1-140853 | Mechanisms and Performance of D2D Synchronization | III | (R1-140424) |

##### D2D data communication physical channel design details

Control signalling

| R1-140466 | Control for D2D broadcast communication | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140051 | On the benefits of having closed loop feedback | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Thursday 13th

| R1-140053 | Physical channel design for D2D communication | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document (Section 2) is noted. Huawei’s priority is RV and NDI.

| R1-140102 | Control for D2D communication | CATT |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140270 | D2D physical channel design detail | ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Conclusion:

A means (explicit or implicit) is needed for a receiving UE to identify which D2D data transmissions can be combined.

Study further until RAN1#76bis how to implement the means for a receiving UE to identify which D2D data transmissions can be combined.

Working assumption:

- Blind D2D data communication transport block retransmissions are supported

Study further until RAN1#76bis which of the following items of control signalling are needed for D2D data communication, including how these relate to the means for a receiving UE to identify which D2D data transmissions can be combined.

- NDI

- Tx UE ID

- Explicit RV indicator

- MCS indication

- Others are not precluded

| R1-140882 | WF on Scheduling of D2D Communication | Ericsson, Qualcomm, Kyocera, GDBUK | (R1-140862) |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal:

- For direct broadcast communication:

  - Scheduling assignments are transmitted on the physical direct data channel

    - FFS if scheduling assignments are L1 or MAC elements

    - Scheduling assignments include at least indication of the resource(s) for reception of the associated physical direct data channel

      - The indication of resource(s) for reception may be implicit based on scheduling assignment resource or content

  - Scheduling assignments are followed by data transmission by the UE that transmitted the scheduling assignment

- Resource allocation for scheduling assignments:

  - For a UE in-coverage: scheduling assignments are transmitted by the UE on resources that are scheduled by the eNB

  - For  a UE in edge-of-coverage:  scheduling assignments are transmitted by the UE on resources that are configured by the eNB

  - For a UE out-of-coverage:  scheduling assignments are transmitted by the UE on resources that:

    - are signaled by a UE that is in-coverage

    - are signaled by a UE that is in edge-of-coverage,

    - have been pre-configured for use when no resources have been signaled.

Thursday 13th

| R1-140982 | D2D Offline Session Outcome | Qualcomm Incorporated (Rapporteur) |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working Assumption:

- For D2D broadcast communication, scheduling assignments that at least indicate the location of the resource(s) for reception of the associated physical channel that carries D2D data are transmitted by the broadcasting UE

  - The indication of resource(s) for reception may be implicit and/or explicit based on scheduling assignment resource or content

- Scheduling assignments use PUSCH structure for transmission

  - Details of PUSCH structure including DMRS and RE mapping are FFS

  - At least the following are not precluded from further study: Scheduling assignments piggybacked with data, or indicated over DMRS

- For Mode 2

  - A resource pool for scheduling assignment is pre-configured and/or semi-statically allocated

    - FFS whether the resource pool for scheduling assignment is same as the resource pool for D2D data

  - UE on its own selects the resource for scheduling assignment from the resource pool for scheduling assignment to transmit its scheduling assignment

- For Mode 1

  - the location of the resources for transmission of the scheduling assignment by the broadcasting UE comes from the eNodeB

  - the location of the resource(s) for transmission of the D2D data by the broadcasting UE comes from the eNodeB

Discuss the following further offline:

- Additional Proposal for Mode 1

  - A resource pool for scheduling assignment is semi-statically allocated

    - If there is a resource pool allocated for D2D data it is FFS whether the resource pool for scheduling assignment is same as the resource pool for D2D data

Not treated.

| R1-140052 | Considerations of D2D communication | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140100 | Physical channel design for D2D communication | CATT |  |
| R1-140101 | Channel scrambling and DMRS design for D2D communication | CATT |  |
| R1-140128 | Discussion on D2D data communication physical channel design | Intel Corporation |  |
| R1-140173 | D2D communication channel design for broadcast | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140196 | Further analysis on control Information for D2D communication | Fujitsu |  |
| R1-140217 | D2D data communication physical channel design | ETRI |  |
| R1-140331 | D2D Communication Physical Channel Design | LG Electronics |  |
| R1-140388 | Physical channel design for D2D communication | Samsung |  |
| R1-140465 | Signal design for D2D broadcast communication | Qualcomm Inc. |  |
| R1-140490 | Considerations of subframe design for D2D and cellular resource multiplexing | NEC |  |
| R1-140506 | Physical channel design in D2D communication | Panasonic | (R1-135399) |
| R1-140511 | ProSe device-to-device communication control channel design | General Dynamics Broadband UK |  |
| R1-140657 | Physical channel design for D2D communication | InterDigital |  |
| R1-140662 | On control information of D2D communication | CATR |  |
| R1-140775 | Frame Structure for D2D-Enabled LTE Carriers | Ericsson |  |
| R1-140776 | D2D Physical Channels Design | Ericsson |  |
| R1-140777 | Scrambling of D2D Physical Channels | Ericsson |  |
| R1-140880 | WF on D2D broadcast communication phy design | Qualcomm, General Dynamics Broadband UK, ZTE, CATT, LGE |  |

##### Resource allocation

| R1-140130 | Discussion on D2D broadcast resource allocation | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexey Khoryaev from Intel and proposes:

- TDM+FDM based resource allocation type within data transmission region is considered for D2D broadcast communication.

- Further study autonomous and assisted resource selection schemes as a baseline solutions for out of coverage scenario.

- Further study the discussed synchronization and resource allocation as a candidate for long range D2D broadcast communication in out of network coverage PS scenarios.

Decision: The document is noted.

| R1-140335 | Multiplexing of Uu and D2D communication | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Hanbyul Seo from LGE and proposes:

- Proper solution is needed to efficiently accommodate D2D communications while minimizing its impact on existing operations.

- Priority handling of D2DSS should be carefully addressed in consideration of link failure

- It is beneficial UEs outside network coverage are synchronized to Uu UL timing.

- Resource separation between D2D Tx and D2D Rx of UE in network coverage is supported for efficient relay operation.

- Adjacent channel coexistence issue between WAN and D2D communications should be investigated.

- Further study is needed under what condition a UE can operate WAN UL transmission and D2D reception simultaneously across two component carriers.

Decision: The document is noted.

| R1-140427 | Public safety perspective on resource allocation for D2D group communications | U.S. Department Of Commerce |  |
| --- | --- | --- | --- |

The document was presented by Ken Loa from USDoC and proposes:

Proposal 1:

- When transmitting UEs are out of network coverage:

- A UE on its own select a resource from a pre-configured resource pool to transmit on

- Such configurations may be re-configured by the operator

  - The details of how the resource pool is re-configured is FFS

- When transmitting UEs are in network coverage:

1) The eNodeB or rel-10 relay node will semi-statically allocate a resource pool for D2D communication

2) A UE will operate in three modes for resource allocation

  - In coverage: eNodeB or rel-10 relay node will schedule the exact resource within the resource pool that will be used by a UE used for D2D transmission

  - Edge of coverage: a UE on its own select a resource from the resource pool to transmit on

  - Out of coverage: a UE on its own select a resource from a pre-configured resource pool to transmit on

  - The details of how a UE selects the mode to operate on is FFS.

Proposal 2: The following scenarios should be considered for resource allocation schemes on failure rate and overall delay, which should be included in the performance metrics for D2D group communication.

- The transmitting UE moves from in-coverage to out-of-coverage (RLF event)

- The transmitting UE performs handover between cells with different D2D configurations, which includes RLF handover, backward handover or NAS Recovery.

- A group of public safety UEs could spread across multiple cells. Multiple eNBs are required to coordinate each other to perform scheduling for D2D group communications

Decision: The document is noted.

| R1-140468 | Contention Based Scheduling of D2D Broadcast with Impairments | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Saurabha Tavildar from Qualcomm and concludes that overhead of sensing doesn’t significantly impact performance of D2D broadcast communication schemes proposed in [R1-135322].

Decision: The document is noted.

| R1-140613 | Resource Allocation for D2D Communication | Broadcom Corporation |  |
| --- | --- | --- | --- |

The document was presented by Daewon Lee from Broadcom and proposes:

- A distributed resource allocation scheme to be selected for D2D broadcast communication in Rel12.

- Define a radio resource reservation period of which length can be variable and the allocation of the period is performed by a transmitting UE.

- All physical layer repetitions/retransmissions of the same transport block are transmitted during one resource reservation period.

Decision: The document is noted.

| R1-140778 | On Scheduling Procedure for D2D | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Stefano Sorrentino from Ericsson and draws the following observations:

- The use of scheduling assignments carried by the physical direct data channel is advantageous from a flexibility/performance trade-off.

- Without SAs, the energy consumption for D2D support is unacceptable (~18 times larger than the reference case).

- The possibility of using SAs reduces energy consumption effectively, to an acceptable level.

Discussion: Telecom Italia asked why sensing needed for in-coverage use case? To cover the case of UE2 in figure 2.

Decision: The document is noted.

Looking at WFs

| R1-140836 | WF on D2D Communication Resource Allocation | US Department of Commerce, Qualcomm, Huawei, Ericsson, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- From a transmitting UE perspective a UE can operate in two modes for resource allocation:

  - Mode 1: eNodeB or rel-10 relay node schedules the exact resources used by a UE to transmit direct data and direct control information

    - Mode 1 is used for in-coverage transmission

  - Mode 2: a UE on its own selects resources from resource pools to transmit direct data and direct control information

    - FFS if the resource pools for data and control are the same

    - Mode 2 is used for edge-of-coverage and out-of-coverage

- For edge-of-coverage, the resource pools are semi-statically signaled by the eNodeB

- For out-of-coverage, the resource pools are signaled by UEs in coverage or are pre-configured in the UEs

  - FFS on whether and how an in-coverage UE can signal the resource allocation to the out-of-coverage UE

- Definition of coverage areas is FFS, but could for example be based on DL received power

Discussion: ZTE: agree with Mode 2 – more concern about Mode 1 and how scheduling is performed.

Sony: complexity should be considered due to remaining time available for this SI - D2D resource pool should be static and predefined in Rel-12.

ALU: Mode 2 is agreeable as baseline; Mode 1 is FFS

Huawei: Keep Mode 1

Decision: The document is noted.

Possible agreements

- From a transmitting UE perspective a UE can operate in two modes for resource allocation:

  - Mode 1: eNodeB or rel-10 relay node schedules the exact resources used by a UE to transmit direct data and direct control information

    - Mode 1 is used for in-coverage transmission

    - FFS if resource pool restricting the available resources for data and/or control is needed

  - Mode 2: a UE on its own selects resources from resource pools to transmit direct data and direct control information

    - FFS if the resource pools for data and control are the same

    - FFS if resource pool restricting the available resources for data and/or control is needed

- FFS the definition of in-coverage, out-of-coverage and edge of coverage

Debate on the following alternatives:

Alt 1: “Mode 1 is supported for in-coverage transmission”

- Supported by ALU, ASB, Nokia, ZTE, ITRI, Intel, HTC, EADS, Broadcom, Samsung, ETRI, CATT, Sony, NSN, Fujitsu, AT&T (ensure that operators stay in control of choosing between modes)

Alt 2: “Mode 1 is used for in-coverage transmission”

- Supported by Ericsson, Huawei, HiSilicon, Orange, Qualcomm, Telecom Italia, GDBUK, InterDigital, LGE, DT, USDoC, CeWIT, Kyocera, Sharp, CMCC, Panasonic

Alt 3: “D2D communication capable UE shall support Mode 1”

Agreements

- From a transmitting UE perspective a UE can operate in two modes for resource allocation:

  - Mode 1: eNodeB or rel-10 relay node schedules the exact resources used by a UE to transmit direct data and direct control information

    - FFS: if semi-static resource pool restricting the available resources for data and/or control is needed

  - Mode 2: a UE on its own selects resources from resource pools to transmit direct data and direct control information

    - FFS if the resource pools for data and control are the same

    - FFS: if semi-static and/or pre-configured resource pool restricting the available resources for data and/or control is needed

  - D2D communication capable UE shall support at least Mode 1 for in-coverage

  - D2D communication capable UE shall support Mode 2 for at least edge-of-coverage and/or out-of-coverage

  - FFS: Definition of out-of-coverage, edge-of-coverage, in-coverage

Agreement:

- For example, definition of coverage areas is at least based on DL received power

Possible agreements:

- D2D communication capable UE shall support both Mode 1 and Mode 2

  - FFS: Mode 2 for in-coverage and Mode 1 for edge-of-coverage, out-of-coverage

- For in-coverage, it is up to operator to allow Mode 1 and/or Mode 2

  - FFS: Mode 2 for in-coverage and Mode 1 for edge-of-coverage, out-of-coverage

- FFS how to configure between Mode 1 and Mode 2, e.g., whether there is a condition to  configure switching between Mode 1 and Mode 2

Not treated.

| R1-140103 | Resource coordination in partial network coverage scenario | CATT |  |
| --- | --- | --- | --- |
| R1-140129 | Analysis of different RSRP thresholds for design and evaluation of D2D communication | Intel Corporation |  |
| R1-140174 | Resource allocation for D2D communication | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140197 | RRM for D2D communication | Fujitsu |  |
| R1-140198 | Multiplexing between cellular signal and D2D signal | Fujitsu |  |
| R1-140218 | Resource allocation for D2D communications | ETRI |  |
| R1-140271 | Study on D2D Resource Allocation for Out of Network Coverage Scenario | ZTE |  |
| R1-140334 | D2D communication based on centralized scheduling | LG Electronics |  |
| R1-140390 | Resource allocation for D2D broadcast communication | Samsung |  |
| R1-140406 | Frequency Reuse for D2D Broadcast Communication | ITRI |  |
| R1-140407 | Comparison between Scheduled and Distributed Resource Allocation Mechanisms for D2D Broadcast Communications in Out-of-LTE-Coverage Cases | ITRI |  |
| R1-140425 | Resource pool configuration for partial coverage scenarios | III |  |
| R1-140467 | Cluster Head versus Distributed Resource Allocation | Qualcomm Inc. |  |
| R1-140469 | Resource Allocation for in Network D2D broadcast | Qualcomm Inc. |  |
| R1-140491 | Discussion on Multiplexing D2D link and Cellular link | NEC |  |
| R1-140507 | System-level performance analysis on resource allocation | Panasonic |  |
| R1-140516 | Radio resource sharing between Public Safety groups using D2D | EADS |  |
| R1-140572 | D2D communication performance | Nokia, NSN |  |
| R1-140573 | D2D communication: Resource allocation, multiplexing and resource access | Nokia, NSN |  |
| R1-140589 | D2D Communication Resource Scheduling | Sony |  |
| R1-140664 | D2D communication resource allocation using Scheduling Assignments | Kyocera |  |
| R1-140779 | On Synchronization Heads | Ericsson |  |
| R1-140781 | Network control and flexible resource sharing between cellular and D2D communication within network coverage | Orange |  |
| R1-140825 | On the Resource Pool Configuration for D2D Communications | LG Electronics | (R1-140332) |
| R1-140826 | Evaluations on the Resource Selection for D2D Communications | LG Electronics | (R1-140333) |
| R1-140832 | Evaluation results of resource allocation schemes for D2D communication | Samsung | (R1-140389) |
| R1-140859 | WF on resource pool configuration for D2D communication | LG Electronics, Ericsson, Qualcomm, ITRI |  |
| R1-140897 | WF on cellular HARQ-ACK and D2D collision avoidance | NEC, LGE |  |
| R1-140931 | WF on Multiplexing of D2D and WAN from system perspective | Samsung, NTT DoCoMo, Ericsson, ZTE, Orange, AT&T, InterDigital, Nokia, NSN |  |

##### Other

Not treated.

| R1-140391 | Discussion on D2D and cellular communication multiplexing | Samsung |  |
| --- | --- | --- | --- |
| R1-140780 | On Procedures for In/Out of NW Coverage Detection for D2D | Ericsson |  |

#### D2D discovery

| R1-140426 | Out of network coverage discovery for public safety in Release 12 | U.S. Department Of Commerce |  |
| --- | --- | --- | --- |

##### Discovery signal and message design details

| R1-140574 | Discovery signal design | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Cassio Ribeiro from Nokia and proposes:

- Proposal 1:

  - RAN1 to clarify the needs for interference randomization, taking into account D2D discovery resource allocation and time-frequency hopping of D2D discovery signals.

  - In case UE specific scrambling is to be considered as auxiliary interfere randomization method, the number of possible scrambling seeds has to be minimized in order to avoid prohibitive complexity for the receiver UE. Moreover, in this case:

    - FFS on how the scrambling seed is known to discovering UE. Identification through the discovery sequence can be taken as baseline.

    - FFS on how UEs select the scrambling seed for discovery message transmission

- Proposal 2: The current UL DMRS density is seen as sufficient for D2D discovery operation.

- Proposal 3: In case discovery range in the order of 500m or larger is required, consider applying the extended LTE CP length for D2D discovery operation. Otherwise, the normal LTE CP length should be utilized.

- Proposal 4: QPSK is used for discovery message. Re-use the PUSCH channel coding chain (including CRC)

- Proposal 5:  Slot-wise frequency hopping should be considered on designing the discovery message transmission.

- Proposal 6: RAN1 should study the need of physical layer related information carried in the message part of the discovery signal.

- Proposal 7: Discovery sequence and UL DMRS should be decoupled, i.e. UL DMRS should not be used as discovery sequence.

Decision: The document is noted.

| R1-140055 | D2D discovery message design | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Hossein Bagheri from Huawei and proposes:

- Proposal 1: For discovery message size of less than about 200 bits, convolutional code is slightly preferred.

- Proposal 2: The CRC length should be selected according to the message size, and a design target maximum number of UEs participating in the discovery process.

- Proposal 3: Scrambling sequence is derived based on the sequence-part and the location of the discovery signal.

- Proposal 4: Use QPSK for discovery message.

- Proposal 5: Study the impact of using the discovery sequence-part in demodulation.

- Proposal 6: Discovery-message and sequence parts are multiplexed in time.

- Proposal 7: The distribution of the sequence-part in time and frequency is FFS.

Decision: The document is noted.

| R1-140661 | Consideration on D2D discovery signal and message | CATR |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Du from CATR and shows preference to sequence other than demodulation sequence as one option of discovery design, and to the two step D2D discovery procedure. It is also proposed that 1 PRB pair for discovery message could be the baseline for evaluation.

Decision: The document is noted.

| R1-140131 | On D2D discovery physical structure design | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Debdeep Chatterjee from Intel and proposes:

Proposal 1

- Discovery signal is composed of a message and a sequence part where the sequence is the DM-RS of the message part.

Proposal 2

- For FDD systems, IFDMA signal structure with an RPF of 2 can be considered for guard period generation to address the Tx-Rx and Rx-Tx switching time issue.

- For FDD systems, in order to avoid intra-discovery and inter-WAN and discovery interference, guard period may be considered in the last OFDM symbol of the discovery resource. Either data or reference symbol can be allocated in this OFDM symbol.

- Handling of Tx-Rx/Rx-Tx switching time and inter-WAN-DZ interference at the boundaries of D2D DZ may be addressed in multiple ways, including relying on implementation-based solutions. RAN1 WG should carefully study each alternative towards determining the most appropriate step in this direction.

Proposal 3

- A common scrambling identity should be considered for message-based D2D discovery.

- The study of the need of time-varying hashing/scrambling function prior to channel coding for security purposes for restricted discovery may be out of scope of RAN1.

Proposal 4

- For a message size of 104 bits, TBCC in Rel-8 should be considered for message-based D2D discovery in order to limit the decoding complexity and reduce the UE power consumption.

- For a message size of 104 bits, 16 bit CRC should be considered for message-based D2D discovery.

Decision: The document is noted.

| R1-140175 | D2D discovery channel design | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and proposes:

- Proposal 1: For simplicity of D2D system design, the same channel coding scheme is applied for all the supported discovery message sizes and D2D communications. The Rel-8 Turbo coding is preferred to other coding schemes.

- Proposal 2: An open-loop power control scheme should be adopted for D2D discovery in network coverage.

- Proposal 3: The D2D discovery message is carried on the D2D data communication physical channel.

Discussion: Huawei  concern with what power control is going to be used? Proposal seems open-loop power control only.

Decision: The document is noted.

| R1-140272 | Design Details of Discovery Signal | ZTE |  |
| --- | --- | --- | --- |

The document was presented by Yifei Yuan from ZTE and proposes:

- Proposal 1: Rel-8 PUSCH DMRS should be reused for discovery signal; The number of cyclic shifts for discovery sequence should be smaller than 12, FFS whether fixed in the specification or by signalling.

- Proposal 2: non-UE-specific scrambling should be considered at least for discovery type 1; for asynchronous deployments, scrambling sequence should be cell-specific. For synchronous deployments, non-cell-specific scrambling should be the baseline.

- Proposal 3: each discovery signal occupies one PRB pair of resources.

- Proposal 4: 16 bits CRC is considered as the baseline for discovery message.

- Proposal 5: TBCC should be considered for channel code of discovery message.

- Proposal 6: for discovery purpose, CP length may be configurable by the network.

Decision: The document is noted.

| R1-140665 | Channel coding for D2D discovery signal | Kyocera |  |
| --- | --- | --- | --- |

The document was presented by Amit Kalhan from Kyocera and states that the link-level performance between turbo coding and TBCC is insignificant given the message size is 104 bits long. Considering higher decoding complexity of turbo codes relative to TBCC and requiring large number of candidate discovery signal decoding in a given time period it is preferred to use TBCC as it has lesser computational load. From both the link-level performance and the false alarm probability perspective, TBCC with 24 bits CRC is preferable for D2D discovery signal.

- Proposal: Based on the above observations we propose TBCC with 24 bits CRC for D2D discovery signal.

Decision: The document is noted.

| R1-140470 | D2D discovery signal and message design | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and proposes:

- Proposal 1: Discovery sub-frame should be similar to an uplink WAN sub-frame used for PUSCH transmission. However the last symbol(s) of the sub-frame could be used as a gap.

- Proposal 2: Use normal cyclic prefix for discovery transmissions. For large discovery range extended cyclic can be used.

- Proposal 3: DMRS signals transmitted as part of discovery signal should not depend on cell identity.

- Proposal 4: QPSK modulation should be used for discovery.

- Proposal 5: Turbo coding should be used for discovery.

- Proposal 6: Same channel interleaver as PUSCH should be used for discovery.

- Proposal 7: No inter-slot hopping should occur for discovery transmissions. The resource element mapping should be identical to PUSCH resource mapping with no inter-slot hopping.

- Proposal 8: Discovery scrambling mechanism should be identical to the PUSCH scrambling mechanism with the exception that the scrambling sequence should not be dependent on UE or cell identity.

- Proposal 9: A CRC length of 24 should be used for discovery message transmission.

Discussion: LGE  concern with no inter-slot hopping (proposal 7).

Telecom Italia: what means “For large discovery range” (proposal 2)? Order of magnitude?

Decision: The document is noted.

| R1-140512 | ProSe device-to-device discovery sequence design | General Dynamics Broadband UK |  |
| --- | --- | --- | --- |

The document was presented by Mark Cannon from GDBUK and proposes:

- Proposal 1:  The discovery sequence is based upon PSS

- Proposal 2:  The discovery sequences are SFN combined, at least from devices within the same cell

- Proposal 3:  The discovery sequence is transmitted at the start of the discovery region.

- Proposal 4:  The discovery root sequence indicates the duration of the number of discovery subframes from a finite set of options.

- Proposal 5:  The discovery sequence root index does not coincide with those used for conventional cell synchronisation signals and / or the transmission of the discovery sequence does not coincide with the 100kHz LTE frequency raster.

Decision: The document is noted.

| R1-140885 | WF on D2D Signal Design | Qualcomm, General Dynamics Broadband UK, CATT, LG, CEWiT |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- When a UE switches from uplink transmission to a D2D reception on a sub-frame boundary the D2D reception can be punctured

- The last symbol(s) of a D2D transmission in a sub-frame can consist of a gap

    - The size of the gap is FFS

      - The size can depend on cell size

    - FFS whether gap size needs to be an integer multiple of a symbol size

Decision: The document is noted.

Working Assumption for both communication and discovery:

From the perspective of a single UE:

- No DTX period is introduced solely to aid switching for the case when a UE switches from uplink transmission to D2D reception

- In the event of a time domain conflict between UL WAN transmission and D2D transmission and/or reception and/or switching, UL WAN transmission is always prioritized

- The last symbol(s) of a D2D transmission can consist of a gap

  - The size of the gap is FFS between ½, 1 and 2 symbols – revisit at RAN1#76bis

  - FFS whether the size can depend on cell size – revisit at RAN1#76bis

  - FFS whether the gap is created by puncturing or rate matching

  - FFS whether, and if so how, the receiver is made aware of the presence (and length if variable) of the gap

  - FFS in which circumstances the gap exists

- No DTX period is introduced solely to aid switching for the case when a UE switches from D2D transmission to D2D reception, nor from D2D reception to D2D transmission.

Is the discovery sequence the demodulation RS of the discovery message?

- Yes: Qualcomm, Intel, Samsung, ZTE, CATT, Huawei, HiSilicon, Ericsson

- No: Broadcom, Fujitsu, Nokia, NSN, GDBUK, ALU, ASB

Working Assumption (tentative):

- Transmission of a discovery message by a UE can be preceded by transmission of a D2DSS

  - Note that D2DSS is not the message DMRS

- “Discovery sequence” concept introduced in working assumption in RAN1#74bis no longer exists.

- None of the discovery message payload is carried by the message DMRS

- FFS whether there are any cases when the discovery message may be not preceded by transmission of a D2DSS

Check in Wednesday’s session whether further clarifications or progress can be made.

Wednesday 12th:

Working Assumption:

- A UE that transmits a discovery message may also be a synchronisation source (and therefore also transmit D2DSS)

  - Note that D2DSS is not the message DMRS

- “Discovery sequence” concept introduced in working assumption in RAN1#74bis no longer exists. (but see open discussion below on discovery preamble)

- None of the discovery message payload is carried by the message DMRS

- A UE transmitting a discovery message does not necessarily need to be a synchronisation source

Further discussion on Discovery Preamble:

Question: Can a UE that transmits a discovery message (including DMRS) also transmit a “discovery preamble” prior to the discovery message, where the “discovery preamble” is different from a D2DSS in one or more of the following ways:

- E.g. if D2DSS is periodic and “discovery preamble” is not?

- Different sequence?

- Different bandwidth?

Possible purposes for “discovery preamble”:

- Fine tuning of time and/or frequency synchronisation

- Channel estimation assistance for discovery message decoding

- AGC setting

Is a discovery preamble needed?

Yes: Broadcom, Fujitsu, Nokia, NSN, LGE, Sony, ITRI, CATR, NEC (for ooc), ALU, ASB

No: QC, Intel, GDB, NEC (in-coverage), E///, CATT, Samsung, HW, HiSilicon, ZTE

Conclusion: FFS until RAN1#76bis.

Working assumption:

- At least if message is not smaller than 104 bits, Rel-8 turbo coding is used for the discovery message

Not treated.

| R1-140097 | Details of discovery signal design | CATT |  |
| --- | --- | --- | --- |
| R1-140199 | On D2D discovery signal design details | Fujitsu |  |
| R1-140392 | Physical channel design for D2D discovery | Samsung |  |
| R1-140408 | AGC Impact to Discovery Signal Frame Structure | ITRI |  |
| R1-140513 | ProSe device-to-device discovery link performance | General Dynamics Broadband UK |  |
| R1-140590 | Discovery signal and message design in resource allocation aspect | Sony |  |
| R1-140629 | Views on D2D discovery signal design | NTT DOCOMO |  |
| R1-140840 | D2D Discovery Signal Format | LG Electronics | (R1-140336) |

##### Resource allocation

| R1-140057 | Resource allocation for type-1 discovery | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Philippe Sartori from Huawei and proposes:

- Proposal 1: D2D UE and/or central node should monitor and report the usage of allocated discovery resource pool to assist the determination of size of discovery resource pool.

- Proposal 2: Combining across several transmissions should be used to improve discovery performance.

Decision: The document is noted.

| R1-140098 | Further discussion on resource allocation for D2D discovery | CATT |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Peng from CATT and proposes:

- Proposal 1: UE Grouping based discovery period/offset and resource configuration can be adopted for cases of insufficient discovery resources.

- Proposal 2: Allocation of multiple discovery resources within one discovery period for a certain message can be considered especially for case of sufficient discovery resources.

- Proposal 3: In case of multiple discovery resources for one message, information combination of the discovery resources in receiver can be achieved via fixed mapping pattern, e.g. RMP 4.

- Proposal 4: Random resource selection can be the baseline scheme for further study of type 1 discovery.

- Proposal 5: During the study of resource hopping, the possibility to apply the same hopping pattern for type 1 discovery, type 2 discovery and communication should be taken into count.

Decision: The document is noted.

| R1-140176 | D2D discovery channel resource allocation | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |

The document was presented by Fang Chen Cheng from ALU and proposes:

- Proposal 1: For type 1 discovery procedure, the channel allocation/selection for each UE is performed in a distributed manner. Random channel selection is preferred to sensing-based channel selection.

- Proposal 2: A hierarchical period structure is adopted for the discovery resource selection.

Decision: The document is noted.

| R1-140472 | Resource allocation for D2D discovery | Qualcomm Inc. |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and proposes that random selection is used a baseline for resource selection. Energy based selection should also be considered.

Decision: The document is noted.

| R1-140514 | ProSe device-to-device discovery resource allocation | General Dynamics Broadband UK |  |
| --- | --- | --- | --- |

The document was presented by Mark Cannon from GDBUK and proposes that the resources allocated to the discovery message should not be segmented and the modulation used should be QPSK.

Decision: The document is noted.

| R1-140575 | Resource allocation for discovery signals | Nokia, NSN |  |
| --- | --- | --- | --- |

The document was presented by Cassio Ribeiro from Nokia and proposes:

- Proposal 1: Start by specifying Type 1 resource allocation with slotted Aloha. Leave potential enhancements by the introduction of carrier sensing based Type 1 allocation and Type 2 allocation for future standard releases.

- Proposal 2: For all type of resource allocations, a semi-static resource pool will be indicated in a SIB. Also Type 2 resources are allocated from the semi-static resource pool (if Type 2 allocation is to be supported).

- Proposal 3: If Type 2 or carrier sensing based Type 1 resource allocation needs to be specified, as a common solution consider specifying resource sequences with time and frequency hopping. Time hopping is designed so that UEs allocated on different resource sequences have frequent opportunities to detect each other despite of the half-duplex assumption.

- Proposal 4: Consider PDCCH signalling for rapid permission control of semi-statically allocated discovery resources.

- Proposal 5: Study the need and implementation of a distributed load control system for slotted Aloha Type 1 access.

- Proposal 6: Study the issue on the conflict between cellular PUCCH and discovery transmission for Type-1 resource allocation

- Proposal 7:

  - Do not assume listening of neighbour cell’s system information as a pre-requisite for inter-cell discovery.

  - Discuss on networks ability to provide accurate information on the discovery resource timings in other cells and, if discovery without proper timing information is needed, study further whether it can be based on the sequence parts of the signals without any additional timing synchronization signal transmitted by UEs.

Decision: The document is noted.

| R1-140135 | On support of inter-cell D2D discovery | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Debdeep Chatterjee from Intel and is summarized as:

- Proposal 1: For synchronous LTE deployments, the D2D Discovery Zones should be configured in a network-common manner.

- Proposal 2: For asynchronous LTE deployments, the D2D Discovery Zones should be configured so as to minimize the probability of any overlap of the D2D DZs of neighbouring cells to avoid asynchronous interference.

- Proposal 3: Effective handling of inter-cell interference between D2D discovery and UL WAN transmissions should be carefully studied by RAN1 WG for the support of D2D discovery in asynchronous networks.

- Proposal 4: For synchronous LTE deployments, ProSe-enabled UEs use DL timing of the serving cell as their synchronization reference for transmission and reception, i.e. T1 = DL serving cell reference time.

- Proposal 5a: For inter-cell discovery in asynchronous LTE deployments, ProSe-enabled UEs derive the timing of the neighbouring cell for reception of discovery signals either:

  - by directly acquiring the PSS/SSS transmissions from the neighbouring cell;

  - by using PRS subframes of neighbouring cells when available;

  - by receiving UE-relayed synchronization signals carried as part of discovery announcement signals by certain UEs in the neighbouring cell.

  - The decision of whether to support all three mechanisms or perform a down-selection from the above is FFS.

- Proposal 5b: For asynchronous LTE deployments, the configuration of D2D Discovery Zones of neighbouring cells can either be:

  - signalled by the serving cell;

  - directly acquired by the ProSe-enabled UEs using assistance information from the serving cell about the relevant SIB transmitted by the neighbouring cell;

  - carried as part of discovery announcement signal transmitted by certain UEs in the neighbouring cell.

  - The decision of whether to support all three mechanisms or perform a down-selection from the above is FFS.

- Proposal 6: For asynchronous LTE deployments, while UEs can derive synchronization based on timing of neighbouring cells for reception of inter-cell discovery signals, they may only transmit discovery signals within the D2D DZs configured by the respective serving cell using the serving cell DL reference time, i.e., T1 = DL serving cell reference time.

Decision: The document is noted.

| R1-140493 | On resource allocation for inter-cell discovery | NEC |  |
| --- | --- | --- | --- |

The document was presented by Rajitha Palipana from NEC and proposes to consider time domain grouping of nearby D2D UEs that are not far from each other to a subinterval within the discovery region of serving cell. The size of most appropriate subinterval is FFS.

Decision: The document is noted.

Looking at WFs

| R1-140860 | WF on Type 1 Discovery Resource Allocation | Qualcomm Inc., General Dynamics Broadband UK, ZTE, Samsung, Intel, Huawei, HiSilicon, CATT |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- Type 1: a discovery procedure where resources for discovery signal transmission are allocated on a non UE specific basis

- Baseline: Each discovery period, a UE pseudo-randomly selects a discovery resource to transmit on

  - When in-coverage, discovery period is configured by eNodeB

  - Other schemes can be considered later

Decision: The document is noted.

Agreements:

- Baseline: For each discovery period, a UE can transmit on a randomly selected discovery resource

  - When in-coverage, discovery period and amount of discovery resource(s) are configured by eNodeB

  - Other schemes can be considered later

Tuesday afternoon session

| R1-140863 | WF on resource allocation for type 1 D2D discovery | CATT, Huawei, HiSilicon, Intel, ITRI, LGE |  |
| --- | --- | --- | --- |

The document was presented by Ms Ying Peng from CATT.

- A UE transmits on its discovery message transmission resource from one/combination of following options

  - Option 1: based on Tx UE transmission period and  offset (i.e. UE grouping)

  - Option 2: based on a fixed or adaptive transmission probability derived from a preconfigured/configured nominal transmission probability

- Resource for a single transmission of unique discovery information by a UE within a discovery period should consist of contiguous resource in time domain

  - The single transmission or the first single transmission if there is repetition in each discovery period for Tx D2D UE is based on random resource selection

- Repetition of transmission of unique discovery information by a UE within a discovery period is allowed

  - The maximum number of repeated transmissions is FFS

  - FFS: whether resources for repeated transmission(s) follow a deterministic mapping or random

Discussion: ALU  concern with UE grouping – other options are FFS.

Decision: The document is noted, modified and agreed as:

Working Assumption:

- Further to the baseline agreed above, the following FFS options can be further studied (including number of discovered devices and latency) for a UE’s transmission on a discovery resource (or on a set of resources if repetition is supported):

  - Option 1: based on Tx UE transmission period and offset

  - Option 2: based on a fixed or adaptive transmission probability derived from a preconfigured/configured nominal transmission probability

  - Others

- A discovery resource has a duration of not less than 1ms (FFS if it can be >1ms if required by the final decision on MAC PDU size, in which case it would be a multiple of 1ms and consist of consecutive D2D subframes) and is used for a single transmission of a given discovery MAC PDU by a UE

  - TDD special subframe is FFS

- Repetition (either contiguous or non-contiguous) of transmission of a given discovery MAC PDU by a UE within a discovery period is FFS; if supported:

  - FFS between:

    - The UE performs random selection only for the first resource in the set of discovery resources that can be used for the repeated transmissions of the discovery MAC PDU; the other resources are deterministically associated with the first resource.

    - The UE performs random selection for each resource in the set of resources.

  - The maximum number of repeated transmissions is FFS

| R1-140841 | D2D discovery resource size and mapping to physical resources | Intel Corporation | (R1-140132) |
| --- | --- | --- | --- |

The document (section 3) was presented by Debdeep Chatterjee from Intel and draws the observation that 1×N resource mapping scheme can achieve better link-level discovery performance than N×1 resource mapping scheme in the case with and without co-channel interference.

Discussion: Mr Chair commented that results show the interest in supporting repetition.

Decision: The document is noted.

| R1-140889 | Resource allocation for Type1 discovery for inter-cell discovery | Qualcomm, General Dynamics Broadband UK, Fujitsu, ZTE, Samsung, DoCoMo, Kyocera |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- For synchronous deployment the discovery resource allocation can be common across deployment

- For synchronous deployment with different resource allocations for different eNodeBs

  - eNodeBs learn the allocation of neighbor eNodeBs

  - eNodeBs broadcast the information to associated UEs using SIB signaling

- For asynchronous deployment

  - eNodeBs learn the allocation of neighbor eNodeBs

  - eNodeBs broadcast the information to associated UEs using SIB signaling

  - Note that eNodeBs may need to keep track of relative time drift of neighbor eNodeBs to learn and broadcast the allocation

Discussion: Ericsson commented that such WF relates to RAN3 – joint proposal is under preparation in RAN3.

Decision: The document is noted. It is agreed awaiting RAN3 discussion results – up to RAN3 to send an LS if they need any information from RAN1.

| R1-140861 | WF on Types of Discovery | Qualcomm Inc., General Dynamics Broadband UK, CATT, LG, Intel |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

- Type 1 and Type 2B discovery are supported

- For Type 2B discovery

  - eNodeB semi-persistently allocates a discovery resource for a UE to transmit on

  - Only RRC_CONECTED UEs transmit

  - Value of T2 is FFS

  - Other current agreements and working assumption on Type1 discovery will also apply to Type 2B

Decision: The document is noted.

Proposal:

- Type 1 and Type 2B discovery are supported

- For Type 2B discovery

  - eNodeB semi-persistently allocates a discovery resource for a UE to transmit on

  - Only RRC_CONECTED UEs transmit

  - Value of T2 is FFS

  - Type1 and Type 2B use the same sequence+message design

Revisit after AI 7.2.8.3

Wednesday 12th:

Continue to study both type 1 and type 2 discovery as per the definitions agreed in RAN1#73.

Agreement:

- For Type 2 discovery:

  - UEs transmit their discovery signal and receive discovery signals from other UEs subject to half duplex constraint

Working Assumption:

- For Type 2B discovery

  - If RRC_Idle UEs are not able to transmit type 2B discovery, value of T2 for type 2B discovery transmission is FFS between:

    - TA for FDD / 624Ts +TA for TDD:

      - HW, HiSi, Telecom Italia, ZTE, Orange, E///, Fujitsu, DT,

    - T2 = 0 for FDD, T2 = 624Ts for TDD:

      - ALU, ASB, E///, LGE, ITRI, GDB, IDC, Samsung,

  - If RRC_Idle UEs are able to transmit type 2B discovery, value of T2 for type 2B discovery transmission is:

    - T2 = 0 for FDD, T2 = 624Ts for TDD

Working Assumption:

- Type1 and Type 2B use the same message design

Friday 14th

| R1-141038 | WF on D2D Discovery Types Supported | Qualcomm Incorporated, General Dynamics, CATT, LG, Intel, Deutsche Telekom, Samsung, ZTE, DoCoMo, Kyocera |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm and states:

- Type 1 and Type 2B discovery are supported

Discussion: Ericsson – why such topic is brought up to plenary session again – extensive discussions during the week concluded FFS – no new information available / far away from completion

ALU – can this be taken as a working assumption?

Yes: Qualcomm, GDBUK, CATT, LGE, Intel, Deutsche Telecom, Samsung, ZTE, NTT DoCoMo, Kyocera, ALU (assuming adding type 2A is not precluded), ASB, Sharp

No: Ericsson, Huawei, HiSilicon

Decision: The document is noted.

Working assumptions:

- If D2D discovery is supported, both Type 1 and Type 2 discovery can be supported subject to investigation and positive resolving of open issues

- Note: Following open issues has not been considered in RAN1

    - Contents of discovery message

    - Impact on cellular traffic

    - Impact on other operator spectrum

    - Lawful interception impact on RAN if any

    - Potential impact on inter-eNB interfaces

- Above note should be captured in TR conclusion

Not treated.

| R1-140056 | Power efficiency and delay analysis of discovery | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140133 | On resource allocation for D2D discovery | Intel Corporation |  |
| R1-140134 | D2D discovery resource selection for Type 1 discovery | Intel Corporation |  |
| R1-140200 | Resource Allocation for Discovery Signal | Fujitsu |  |
| R1-140203 | Near-Far Mitigation for D2D Discovery | Huawei, HiSilicon |  |
| R1-140227 | Resource Allocation Methods for D2D Discovery | HTC |  |
| R1-140273 | Resource Allocation for D2D Discovery | ZTE |  |
| R1-140292 | On the use of type 2B discovery | ASUSTeK |  |
| R1-140293 | Type 2B discovery with resource hopping | ASUSTeK |  |
| R1-140337 | Resource Allocation and UE Behaviour for D2D Discovery | LG Electronics |  |
| R1-140338 | Multiplexing of Uu and D2D discovery signal | LG Electronics |  |
| R1-140393 | Resource allocation method for D2D discovery | Samsung |  |
| R1-140394 | Multiplexing between WAN and D2D from system perspective | Samsung |  |
| R1-140492 | Power consumption of RRC_IDLE UEs for discovery | NEC |  |
| R1-140494 | D2D discovery resource allocation under network coverage | NEC |  |
| R1-140581 | Views on discovery resource allocation | KDDI |  |
| R1-140630 | Views on D2D discovery resource allocation | NTT DOCOMO |  |
| R1-140666 | Resource allocation for Type 1 D2D discovery | Kyocera |  |
| R1-140852 | Resource allocation for D2D discovery | ETRI | (R1-140219) |

##### Other

| R1-140471 | Comparison of LTE Discovery with other discovery technologies | Qualcomm Inc. |  |
| --- | --- | --- | --- |

#### D2D transmission timing

| R1-140211 | Impact of D2D transmission on the existing cellular system | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140339 | D2D Discovery Signal Transmission Timing | LG Electronics |  |
| --- | --- | --- | --- |

Decision: The document is noted.

| R1-140136 | Discussion on D2D transmission timing | Intel Corporation |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working Assumption

- For type 1 discovery,

  - In FDD

    - RRC_Connected D2D UEs transmit discovery signal based on DL reference timing (T2 = 0).

      - FFS possible solutions to address overlap between uplink cellular and D2D

  - In TDD

    - RRC_connected and RRC_idle D2D UEs transmit discovery signal based on T2 = 624Ts

      - FFS possible solutions to address overlap between uplink cellular and D2D

| R1-140888 | WF on the value of T2 | CATT, ALU, ASB, Intel, ITRI, Qualcomm, Samsung, ZTE |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Proposal: RRC_connected and RRC_idle D2D UEs transmit discovery signal based on T2 = 624Ts

| R1-140891 | WF on D2D CP length | Huawei, HiSilicon, CATT, Intel, Fujitsu, LG Electronics, General Dynamics, Panasonic, Qualcomm, Ericsson, Potevio, ITRI, InterDigital, RIM |  |
| --- | --- | --- | --- |

Decision: The document is noted.

Working Assumption:

- Both extended CP length (512Ts) and normal CP length (144 Ts) are supported in the specifications for D2D communication and discovery

  - FFS: details of how to set the CP length

Not treated.

| R1-140058 | On cyclic prefix length for D2D discovery/communication | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140099 | The value of T2 for D2D transmission timing | CATT |  |
| R1-140177 | D2D transmission Timing | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| R1-140201 | Further analysis of D2D transmission timing | Fujitsu |  |
| R1-140228 | Considerations on D2D Transmission Timing | HTC |  |
| R1-140274 | D2D Discovery Transmission Timing | ZTE |  |
| R1-140340 | D2D Communication Signal Transmission Timing | LG Electronics |  |
| R1-140396 | Inter-subframe impact caused by D2D timing in large cell | Samsung |  |
| R1-140458 | D2D transmission timing | Qualcomm Inc. |  |
| R1-140515 | ProSe device-to-device transmission timing | General Dynamics Broadband UK |  |
| R1-140833 | D2D transmit timing impact on WAN | Samsung | (R1-140395) |

#### Other

| R1-140473 | Text Proposal for TR 36.843 | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is noted and TP is agreed for inclusion in next update of TR.

| R1-140474 | DraftTR 36.843 v1.0.1 on Study on LTE Device to Device Proximity Services | Qualcomm Inc. |  |
| --- | --- | --- | --- |

(only editorial changes done by MCC)

Decision: The document is noted and TR is agreed and endorsed as v1.1.0 in R1-140933. Friday 14th  it is noticed that R1-140933 has been submitted as v1.0.1 in contradiction with the decision made during the D2D ad-hoc session. For the sake of simplicity, the document is endorsed as v1.0.1 in R1-140933.

| R1-140944 | TR 36.843 v1.2.0 on Study on LTE Device to Device Proximity Services | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Decision: The document is for email approval until 20th February

Friday 14th

| R1-141037 | TP for Conclusion of D2D SI | Qualcomm Incorporated, General Dynamics, USDoC, UK Home Office, Fujistu, CATT, ZTE, ITRI, Deutsche Telekom, LG, Samsung |  |
| --- | --- | --- | --- |

The document was presented by Shailesh Patil from Qualcomm.

“It is concluded that adding D2D discovery and D2D broadcast communication for public safety applications to LTE standard is feasible. D2D discovery is found to be feasible in terms of range, scalability and power consumption. D2D communication is able to fulfil the voice and data needs of the public safety community both within and outside coverage.”

Discussion: Also supported by Nokia, NSN, Sharp

Ericsson asked time to review it carefully – same to Huawei.

Ericsson: What “feasible” means w.r.t range, scalability, power consumption…?  May be better to list some results

Decision: The document is noted.

Agreements:

- With the current status of work, adding D2D discovery techniques is considered compatible to LTE standard

- With the current status of work, adding D2D broadcast communication techniques is considered compatible to LTE standard, and can fulfill the voice and data needs of the public safety community both within and outside coverage.

| R1-141043 | Text Proposal for TR 36.843 with for D2D Evaluation Results | Qualcomm Incorporated (Rapporteur) | (R1-141034) |
| --- | --- | --- | --- |

Decision: The document is for email approval until 19th February

Not treated.

| R1-140401 | Discussion on the multiplexing of D2D and Cellular signals | ITRI |  |
| --- | --- | --- | --- |
| R1-140658 | Freshness Parameter for Security | InterDigital |  |

### Study on Network-Assisted Interference Cancellation and Suppression for LTE

SID in RP-130404.

Open issues in RP-132108

#### System level evaluation results

| R1-140829 | Evaluation results for NAIC | Qualcomm Inc. | (R1-140475) |
| --- | --- | --- | --- |

The document was presented by Hao Xu from Qualcomm and presents system evaluation results for SLIC receivers.

- Advanced receiver techniques such as SLIC can provide significant system throughput gain over baseline receivers.

- The gains obtained based on blind SLIC receiver are very close to the gains obtained by genie-aided SLIC receiver.

Decision: The document is noted.

| R1-140508 | System level evaluation results for E-LMMSE-IRC with interference estimation enhancement | Panasonic |  |
| --- | --- | --- | --- |

The document was presented by Michael Einhaus from Panasonic and proposes that ZP-DMRS should be considered as a strategy for coordinating the DMRS transmissions of mutually interfering cells.

Decision: The document is noted.

| R1-140266 | System level evaluation results for ML receiver | ZTE |  |
| --- | --- | --- | --- |

The document was presented by Huaming Wu from ZTE and concludes that ML can provide significant gains for the cell-edge and average user throughput with network assistance/coordination.

Decision: The document is noted.

Update the excel table to include all results in this meeting until Friday – Jeff (Mediatek)

Not treated.

| R1-140059 | Further system level evaluations for SLIC receiver | Huawei, HiSilicon |  |
| --- | --- | --- | --- |
| R1-140137 | System-level performance evaluation of R-ML/ML receivers with and without signalling assistance | Intel Corporation |  |
| R1-140247 | Further system level results on R-ML receivers | MediaTek Inc. |  |
| R1-140265 | System level evaluation results for SLIC receiver | ZTE |  |
| R1-140397 | System level evaluation of SLML receiver for NAICS scenario 1 | Samsung |  |
| R1-140398 | System level evaluation of SLML receiver for NAICS scenario 2 | Samsung |  |
| R1-140631 | Evaluation results on symbol-level IC for Rel-12 NAICS | NTT DOCOMO |  |
| R1-140803 | System level evaluations for NAICS | Ericsson |  |
| R1-140887 | System level evaluation results for SLIC | LG Electronics |  |

#### Network assistance signalling – feasibility and performance impact

| R1-140576 | Network assistance for advanced receivers | NSN, Nokia |  |
| --- | --- | --- | --- |

The document was presented by Mihai Enescu from NSN and the following general observations can be drawn:

- If dynamic signalling is introduced, a new DCI format seems appropriate.

- Broadcast dynamic signalling from the interfering cell is one solution scaling across all NAICS scenarios.

- Higher layer signalling needs to rely to a large extent on blind UE processing while does not necessarily reduces the UE complexity.

- There is a trade-off in UE complexity when considering a) increased blind decoding of NAICS DCI at the benefit of robust interference characteristics vs. b) increased blind estimation of interference characteristics without the aid of signalling.

Discussion: Huawei  performances linked to broadcast dynamic signalling is an issue to be considered.

Decision: The document is noted.

| R1-140138 | Discussion on network signalling for NAICS receivers | Intel Corporation |  |
| --- | --- | --- | --- |

The document was presented by Alexei Davydov from Intel and deals with two mechanisms for network signalling assistance of the NAICS receiver. The first mechanism relies on backhaul link signalling between eNBs to provide information about parameters of interfering signals to the victim UEs using semi-static signalling. The second approach is based on the dynamic signalling of interfering signal parameters directly from the aggressor eNB.

For each signalling mechanisms some examples of the parameters (both semi-static and dynamic) of the interfering signals were provided. Using link-level simulations the initial study on feasibility of the dynamic parameter signalling from interfering cells is provided. However, further study on the need of dynamic signaling assistance should be conducted by RAN1/4 with consideration of the tradeoffs between receiver complexity, performance gain and signaling overhead. For semi-static signalling the impact of the PRB bundling, hoping and power boosting restrictions on interfering cell should be also investigated.

Decision: The document is noted.

| R1-140804 | On network assisted signalling and coordination for NAICS | Ericsson |  |
| --- | --- | --- | --- |

The document was presented by Lars Lindbom from Ericsson and is summarized as:

- Proposal 1: Any network assisted signalling for NAICS should be applicable to a vast range of network operation scenarios within both homogeneous and heterogeneous deployments, including shared cell operations

- Proposal 2: Interference parameters that can be acquired via blind detection without any network assistance are:

  - Modulation order, precoder, rank/DMRS ports, TM

  - Cell ID, CP, , CRS antenna ports, MBSFN configuration, system bandwidth

- Proposal 3: Information on interference parameters provided by a serving cell are signalled via higher layer

- Observation 1: NAICS receivers can easily support TM10 and its virtual cell ID functionality with proper assistance signalling

- Proposal 4: Serving cell provides information that limits the number of DMRS hypothesis

- Observation 2: The UE-specific RS can be used to detect the presence of PDSCH interference when the interference is associated with DMRS based transmission modes

- Proposal 5: FFS if some network assistance and/or coordination are beneficial for reliable detection of PDSCH interference presence when the interference is associated with CRS based transmission modes

- Proposal 6: Consider coordination of interference parameters related to DL power allocations, and possibly signal a set of power allocations that the UE should consider when cancelling/suppressing interferers

Decision: The document is noted.

| R1-140060 | Discussion on network assistance signalling for NAICS receivers | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Jianghua Liu from Huawei and proposes:

Proposal 1: Higher-layer signalling from serving cell with network coordination and dynamic implicit signalling from interference cell are preferred for interference transmission parameters which need to be signalled; dynamic/higher-layer signalling from interference cell may be excluded for NAICS.

Proposal 2:

- Parameters are predefined with a certain value: CFI/PDSCH start symbol.

- Parameters may be signalled by higher-layer signalling from serving cell with/without network coordination: PB, PA, transmission mode, resource allocation, scrambling identity and DMRS antenna port, .

- Parameters may be implicitly signalled from interference cell: Modulation order.

- Parameters may be blindly detected with the performance-complexity evaluation in RAN4: PB, PA, transmission mode, PMI, RI, modulation order, scrambling identity and DMRS antenna port.

Proposal 3: Further evaluation in RAN4 is necessary to achieve a better trade-off between the feasible signalling methods and acceptable performance-complexity of UE blind detection.

Decision: The document is noted.

| R1-140341 | Discussion on broadcasting dynamic network assistance information | LG Electronics |  |
| --- | --- | --- | --- |

The document was presented by Hyungtae Kim from LGE and proposes:

- Proposal 1: the serving cell of victim UE should provide semi-static NA information through RRC signaling, and the interfering cell should provide dynamic NA information through PDCCH/ePDCCH.

- Proposal 2: PMI and modulation order should be signaled and CFI and PA can be semi-statically coordinated, and other dynamic NA information should be signaled unless BD is feasible.

- Proposal 3: A new C-RNTI for CRC masking of the A-DCI should be introduced and should be given to victim UE by RRC signaling.

- Proposal 4: Alt 1 (A-DCI per NA information type) and Alt 2 (A-DCI per interfering cell’s PDSCH) should be considered as possible assistance DCI.

- Proposal 5: Further study is needed on what aggregation level is reasonable in consideration of PDCCH capacity and coverage for signaling NA information from interfering cell.

Decision: The document is noted.

| R1-140893 | Performance impact of network assistance signalling for NAICS | Samsung | (R1-140399) |
| --- | --- | --- | --- |

The document was presented by Hyojin Lee from Samsung and states that network-assistance signaling for NAICS should be supported in Rel-12 to achieve system performance gains and to avoid additional UE complexity

Decision: The document is noted.

Not treated.

| R1-140178 | Considerations on network signalling for NAICS | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |
| R1-140248 | Discussion on the feasibility of network assistance | MediaTek Inc. |  |
| R1-140267 | Network assistance signalling for NAICS | ZTE |  |
| R1-140476 | Network assistance signalling | Qualcomm Inc. |  |
| R1-140509 | Feasibility of dynamic assistance signalling and blind parameter detection | Panasonic |  |
| R1-140577 | TP for network assistance in NAICS | NSN, Nokia |  |
| R1-140614 | Signalling in NAICS | Broadcom Corporation |  |

#### Network coordination – feasibility and performance impact

| R1-140249 | Discussion on the feasibility of network coordination and performance impact | MediaTek Inc. |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek and proposes that network coordination on CFI alignment, TM, and RA granularity/type can be considered for higher-layer signalling as well, because such coordination and signalling can reduce the receiver complexity significantly.

Decision: The document is noted.

| R1-140061 | Discussion on network coordination for advanced receivers | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

The document was presented by Jianghua Liu from Huawei and proposes:

Proposal 1: UE receiver detection may be facilitated by network coordination from neighbouring cells, which include:

- UE with advanced receiver capability may assume that the transmission mode of interference cells is TM2 or TM9.

- UE may assume that the value of PA of interference signal on coordinated resource is same for TM2.

- UE may assume that resource allocation type 0 is used in the interference cells and PRB bundling is applied on the coordinated resource for TM9.

- UE may assume the coordinated DMRS port configuration is used in the interference cells.

Proposal 2: Further study and achieve agreements on the following open issues are needed in SI stage:

- Study on necessity and feasibility of network coordination/signalling for CWIC.

- Study on how UE determines the interference signals to be eliminated.

- Study and evaluate on the impact of UE timing synchronization of interference cells.

- Study/agree on the evaluation methodology of impairment modelling in order to align the simulation results.

Decision: The document is noted.

Not treated.

| R1-140179 | Considerations on network coordination for NAICS | Alcatel-Lucent Shanghai Bell, Alcatel-Lucent |  |
| --- | --- | --- | --- |
| R1-140268 | Network coordination for NAICS | ZTE |  |
| R1-140342 | Discussion on network coordination for NAICS | LG Electronics |  |
| R1-140400 | Discussion on network coordination for NAICS | Samsung |  |
| R1-140477 | Network coordination for NAICS | Qualcomm Inc. |  |
| R1-140578 | On the NAICS network operation | NSN, Nokia |  |

#### Other

Not treated.

| R1-140139 | NAICS receivers for downlink control channel | Intel Corporation |  |
| --- | --- | --- | --- |
| R1-140220 | On DMRS based interference cancellation receiver and its signalling | ETRI |  |
| R1-140343 | Discussion on CQI calculation methods for NAICS | LG Electronics |  |

| R1-140894 | NAICS RAN1 & RAN4 joint Ad-Hoc discussion on possible SI conclusions | MediaTek Inc. |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion: Broadcom questioned whether the intention was to agree all the content (in particular slide 3) as this was not the understanding from the ad-hoc session

ALU  concern with the cost/complexity w.r.t Dynamic signaling (last slide).

Decision: The document is noted.

Agreement:

- Further investigation of CSI enhancement is needed in order to ensure that NAICS receivers can achieve a user throughput gain

- Higher-layer signaling of parameters related to interference PDSCH could be beneficial to reduce the blind detection complexity or performance degradation

  - It is not precluded at yet that some of the following candidate parameters may be blindly detected

  - Candidate parameters for higher-layer signaling for further study both in RAN1 and RAN4 include

    - Resource allocation granularity (e.g., a group of PRB or PRB pairs)

    - RA type (e.g., type 0, LVRB, Ngap used for DVRB)

    - System bandwidth

    - Synchronization indication (e.g., CP length)

    - CSI-RS configuration

    - QCL

    - Cell-ID

    - CRS ports

    - MBSFN pattern

    - ρB/ρA

- For the following parameters of interference PDSCH, UE blind detection is desirable to reduce scheduling restriction and signaling overhead, possibly detected from a reduced subset (e.g., RRC signaled) of all values for some parameters

  - Presence or absence of interference

  - TM

  - For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

  - For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

  - CFI (if not coordinated and required by receiver implementation)

Agreement:

- Dynamic signaling from an interference or a serving cell can be evaluated after conclusions on blind detection, conditions and assumptions including the physical channel and assumed content can be further discussed

Ericsson: Excel sheet shows results from different companies under different simulation assumptions  may be good to provide those differences if the excel sheet is going to be captured in the TR – assumptions sheet may be simply removed from the excel file and only deviations from these assumptions should be clearly stated.

Update TR capturing above agreements until Friday – Jeff (Mediatek)

Friday 14th

| R1-140955 | 36.866 v1.2.0 on NAICS for LTE | MediaTek Inc. |  |
| --- | --- | --- | --- |

The document was presented by Jeff Zhuang from MediaTek.

Discussion: Orange concern with first bullet in section 11 “Conclusions”

- Further investigation of CSI enhancement is needed in order to ensure that NAICS receivers can achieve a user throughput gain

May be better to just mention “Further investigation of CSI enhancement is needed”.

Mediatek suggested “Further investigation of CSI enhancement is needed in order to ensure that NAICS receivers can achieve a further user throughput gain” – ALU “further” is unclear what the meaning is

Further investigation of CSI enhancement is needed

in order to help to ensure that NAICS receivers can achieve a user throughput gain

Qualcomm  move the bullet at the end of the section

Samsung: still the interpretation of this bullet may have different meaning

Decision: The document is noted and is for email approval until 19th February with the following update  – MCC

The following conclusions were made:

- Higher-layer signaling of parameters related to interference PDSCH could be beneficial to reduce the blind detection complexity or performance degradation

  - It is not precluded at yet that some of the following candidate parameters may be blindly detected

  - Candidate parameters for higher-layer signaling for further study both in RAN1 and RAN4 include

    - Resource allocation granularity (e.g., a group of PRB or PRB pairs)

    - RA type (e.g., type 0, LVRB, Ngap used for DVRB)

    - System bandwidth

    - Synchronization indication (e.g., CP length)

    - CSI-RS configuration

    - QCL

    - Cell-ID

    - CRS ports

    - MBSFN pattern

    - ρB/ρA

- For the following parameters of interference PDSCH, UE blind detection is desirable to reduce scheduling restriction and signaling overhead, possibly detected from a reduced subset (e.g., RRC signaled) of all values for some parameters

  - Presence or absence of interference

  - TM

  - For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

  - For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

  - CFI (if not coordinated and required by receiver implementation)

- Dynamic signaling from an interference or a serving cell can be evaluated after conclusions on blind detection, conditions and assumptions including the physical channel and assumed content can be further discussed

- Further investigation of CSI enhancement is needed in order to help to ensure that NAICS receivers can achieve a user throughput gain

# Closing of the meeting

3GPP Excellence Award (further to 2012 leadership decision to recognise the very best individual for their contributions to current work within 3GPP).

Mr Chair emphasized the decision made by the Awards Panel (the four TSG Chairmen) to award once again RAN1 delegate and congratulated the second winner of the 3GPP Excellence Award, Mr Robert Love from Motorola Mobility. This award is in recognition to Robert’s outstanding contribution to 3GPP through many many years, for his tremendous effort as a rapporteur of one of the most complicated specification, namely TS36.213 “E-UTRA Physical layer procedures”.

Mr Chair then expressed his thanks to RAN1 Vice Chairmen, as well as Carmela Cozzo and Karri Ranta-aho for chairing parallel HSPA sessions and to the Secretary Patrick Merias for his efficient support.

Finally, Mr Chair wished a safe journey back home to all and closed the meeting at 17:05.

See you all in Shenzhen.

# Annex A:	List of Tdocs at RAN1 #76

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #76

| TS | CR | Rev | Rel | Title | Cat | Vsn | TD# | Source to WG | Work Item | Clauses affected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.212 | 0322 | - | Rel-7 | Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | F | 7.11.0 | R1-140835 | NSN | RANimp-CPC | 4.6A.2.2.1 |
| 25.212 | 0323 | - | Rel-8 | Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | A | 8.7.0 | R1-140900 | NSN | RANimp-CPC | 4.6A.2.2.1 |
| 25.212 | 0324 | - | Rel-9 | Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | A | 9.5.0 | R1-140901 | NSN | RANimp-CPC | 4.6A.2.2.1 |
| 25.212 | 0325 | - | Rel-10 | Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | A | 10.4.0 | R1-140902 | NSN | RANimp-CPC | 4.6A.2.2.1 |
| 25.212 | 0326 | - | Rel-11 | Correction to HS-SCCH special information mapping for HS-SCCH less HSDPA | A | 11.6.0 | R1-140903 | NSN | RANimp-CPC | 4.6A.2.2.1 |
| 25.214 | 0711 | 1 | Rel-10 | Clarification on (de-)activation status of secondary carriers when higher layer indicates a change of HS-DSCH cells | F | 10.7.0 | R1-140910 | Huawei, HiSilicon, NSN, Ericsson | 4C_HSDPA-Core | 6A.1 |
| 25.214 | 0712 | 1 | Rel-11 | Clarification on (de-)activation status of secondary carriers when higher layer indicates a change of HS-DSCH cells | A | 11.7.0 | R1-140911 | Huawei, HiSilicon, NSN, Ericsson | 4C_HSDPA-Core | 6A.1 |
| 25.701 | 0007 | - | Rel-12 | Inclusion of Scalable Bandwidth UMTS by filtering and by chip-zeroing | B | 12.0.0 | R1-140909 | NSN | FS_UTRA_SCAL |  |
| 36.211 | 0187 | - | Rel-12 | On PMCH starting symbol in an MBSFN subframe | C | 12.0.0 | R1-140412 | Intel, Samsung, NEC, Panasonic, BlackBerry | LTE-Phys | 6.5 |
| 36.213 | 0447 | - | Rel-12 | Correction to CSI Reporting | F | 12.0.0 | Form-v10 | Panasonic | LTE-Phys | 7.2 |
| 36.213 | 0448 | - | Rel-12 | Clarification on PUCCH Mode 1-1 for 4Tx Dual Codebook | F | 12.0.0 | R1-140782 | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Rohde&Schwarz | LTE_eDL_MIMO_enh-Core | 7.2.2 |
| 36.213 | 0449 | 1 | Rel-11 | Common search space monitoring for MBMS | F | 11.5.0 | R1-141035 | Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, AT&T, Panasonic | MBMS_LTE | 9.1.1 |
| 36.213 | 0450 | 1 | Rel-12 | Common search space monitoring for MBMS | A | 12.0.0 | R1-141036 | Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, AT&T, Panasonic | MBMS_LTE | 9.1.1 |
| 36.213 | 451 | - | Rel-11 | Introduction of new UE categories | F | 11.5.0 | R1-140869 | Ericsson | LTE_CA_enh-Core | 7.1.8 |
| 36.213 | 452 | - | Rel-12 | Introduction of new UE categories | A | 12.0.0 | R1-140870 | Ericsson | LTE_CA_enh-Core | 7.1.8 |
| 36.213 | 0453 | 1 | Rel-10 | Modification to I_SRS = 0 for trigger type 1 SRS and TDD. | C | 10.11.0 | R1-141044 | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | LTE_UL_MIMO-Core | 8.2 |
| 36.213 | 0454 | 1 | Rel-11 | Modification to I_SRS = 0 for trigger type 1 SRS and TDD. | A | 11.5.0 | R1-141045 | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | LTE_UL_MIMO-Core | 8.2 |
| 36.213 | 0455 | 1 | Rel-12 | Modification to I_SRS = 0 for trigger type 1 SRS and TDD. | A | 12.0.0 | R1-141046 | Qualcomm Inc., Huawei, HiSilicon, Fujitsu, BlackBerry, LG Electronics, CATT, Nokia, NSN | LTE_UL_MIMO-Core | 8.2 |
| 36.213 | 0457 | - | Rel-11 | Correction to CSI processing in TM10 | F | 11.5.0 | R1-141058 | Intel Corporation, Huawei, HiSilicon | COMP_LTE_DL-Core | 7.2.1 |
| 36.213 | 0458 | - | Rel-12 | Correction to CSI processing in TM10 | A | 12.0.0 | R1-141059 | Intel Corporation, Huawei, HiSilicon | COMP_LTE_DL-Core | 7.2.1 |

# Annex C-1:	List of Outgoing LSs from RAN1 #76

| R1 | Response to (Ic LS) | To | Cc | Title | Contact | Ref'd /Attachd Tdoc | Release | WI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-140898 |  | R2 | RAN | LS on introduction of new UE categories | Huawei, Ericsson | R1-140869, R1-140868 | Rel-11 | LTE_CA_enh-Core |
| R1-140905 |  | R2, R3 |  | Decoupling of E-DCH and HS-DSCH serving cell in UMTS HetNet | NSN |  | Rel-12 | UTRA_hetnet_enh-Core |
| R1-140906 |  | R2 |  | LS on Enhanced Broadcast of System Information | Ericsson |  | Rel-12 | UTRA_SIBenh |
| R1-140941 | S3-131152 (R1-140017) | S3 | R2, S2 | Response LS on parameter synchronization | Qualcomm Inc. |  | Rel-12 | FS_LTE_D2D_Prox |
| R1-140950 | S2-140560 | RAN | R2, R3, R4, SA, S2 | Reply LS on ProSe Out of Coverage discovery | Ericsson |  | Rel-12 | FS_LTE_D2D_Prox |
| R1-140985 |  | R4 |  | LS on impact on RRM measurements of P-CPICH scaling for Filtered UMTS | AT&T |  | Rel-12 | FS_UTRA_SCAL_FILT |
| R1-141007 | R2-134603 (R1-140006) | R2 |  | Response LS on Random Access in dual connectivity | NEC, LGE |  | Rel-12 | LTE_SC_enh_dualC-Core |
| R1-141008 |  | R2, R4 |  | LS on RAN1 working assumption on dual connectivity | NTT DoCoMo |  | Rel-12 | LTE_SC_enh_dualC-Core |
| R1-141021 |  | R2, R3, R4 |  | LS on small cell on/off and discovery | Huawei |  | Rel-12 | LTE_SC_enh_L1-Core |
| R1-141025 |  | R2 |  | LS on handling of subframe #6 in LTE_TDD_eIMTA | CATT |  | Rel-12 | LTE_TDD_eIMTA-Core |
| R1-141029 | R2-134596 (R1-140005) | R2 | R4, RAN | Reply to LS on mobility support for LC MTC UEs and MTC coverage enhancement | Vodafone Group |  | Rel-12 | LC_MTC_LTE-core |
| R1-141033 |  | R2, R3 |  | LS on Identification of TBS/bandwidth limited UE by eNB | Alcatel-Lucent |  | Rel-12 | LC_MTC_LTE-core |
| R1-141048 |  | R3 | RAN | LS on Inter-eNB CoMP for LTE | Samsung |  | Rel-12 | COMP_LTE_IeNB |
| R1-140904 |  | R2 |  | E-DPCCH to indicate TTI Switching | Qualcomm Inc. |  | Rel-12 | EDCH_enh |
| R1-141065 |  | R2 |  | LS on RAN1 TDD-FDD CA status and related RRC impact | Nokia |  | Rel-12 | LTE_CA_TDD_FDD |
| R1-141066 |  | R2 |  | LS on RRC parameters for LTE_TDD_eIMTA | CATT |  | Rel-12 | LTE_TDD_eIMTA |

# Annex C-2:	List of Incoming LSs from RAN1 #76

| Source | Original Tdoc nbr | RAN1 Tdoc nbr | To | Cc | Title | Response to (Ic LS) | Release | WI | Contact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2 | R2-134486 | R1-140003 | S4 | S2, R1 | Reply LS on the UE capabilities regarding support of simultaneous MBMS bearer reception | S4-131406 (R2-133760) | Rel-12 | MI-EMO | Samsung |
| R2 | R2-134590 | R1-140004 | R1 |  | LS on text proposal for TR 36.843 on D2D |  | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R2 | R2-134596 | R1-140005 | R1 | R4 | Reply LS on mobility support for Low Complexity MTC UEs and MTC coverage enhancement | R1-134954 (R2-133758) | Rel-12 | LC_MTC_LTE-core | Vodafone |
| R2 | R2-134603 | R1-140006 | R1 |  | LS on Random Access in dual connectivity |  | Rel-12 | FS_LTE_SC_enh_hilayer | NEC |
| R4 | R4-137081 | R1-140007 | RAN ITU-R Ad Hoc | R1, R2, R3, R5 | 3GPP RAN WG4 input to update submission for IMT-2000 CDMA DS and IMT-2000 CDMA TDD toward Rev. 12 of Rec. ITU-R M.1457 “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)” | (R4-136884) | - | - | Telecom Italia |
| R4 | R4-137102 | R1-140008 | R1 |  | LS response on D2D Typical UE in-band Emissions | R1-133986 | Rel-12 | FS_LTE_D2D_Prox | Ericsson |
| R4 | R4-137156 | R1-140009 | ECC PT1 | R1 | Reply LS on Synchronization aspects in LTE-TDD networks | (R4-135561) | - | - | Huawei, NSN |
| S1 | S1-135308 | R1-140010 | R1 |  | Reply to LS on notifying for information of including Annex about Public Safety ProSe Communication in TR 36.843 |  | Rel-12 | FS_LTE_D2D_Prox | Telecom Italia |
| S2 | S2-134580 | R1-140011 | R1, R2 |  | Further questions on GCSE with eMBMS |  | Rel-12 | GCSE_LTE | Vodafone |
| R2 | R2-134582 | R1-140012 | R1 |  | Reply LS on FDD-TDD switching enhancement in FDD-TDD joint operation | R1-134960 | Rel-12 | LTE_CA_TDD_FDD-Core | CMCC |
| R2 | R2-134591 | R1-140013 | R1, S1, S2, S3 |  | LS on discovery message size | R1-134957 (R2-133747) | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R2 | R2-134594 | R1-140014 | S2 | R1, R3, R4 | Response LS on GCSE with eMBMS | S2-133846 (R2-133066) | Rel-12 | FS_LTE_GC | Alcatel Lucent |
| RAN | RP-132107 | R1-140015 | S3, S3-LI, S2 | R1, R2, R3, SA, S1 | Reply LS on ProSe Lawful Interception | R1-134923 (RP-131413) | Rel-12 | FS_LTE_D2D_Prox | Telecom Italia |
| RAN | RP-132109 | R1-140016 | S2 | R1, R2 | Response to LS on Further questions on GCSE with eMBMS | S2-134580 | Rel-12 | GCSE_LTE | Vodafone |
| S3 | S3-131152 | R1-140017 | R1 | S2, R2 | LS on parameter synchronization |  | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| S1 | S1-140319 | R1-140181 | R1 |  | Reply LS on D2D proximity criteria | R1-136067 (S1-140144) | Rel-12 | FS_LTE_D2D_Prox | Telecom Italia |
| S2 | S2-140494 | R1-140182 | RAN | S3, S3-LI, R1,R2, R3, SA, S1 | Reply LS on ProSe Lawful Interception | RP-132107 (S2-140049) | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| S2 | S2-140511 | R1-140183 | S4 | R1, R2 | Reply LS on the UE capabilities regarding support of simultaneous MBMS bearers | S4-131406 (S2-140036) | Rel-12 | MI-EMO | Samsung |
| S2 | S2-140560 | R1-140184 | RAN, R1, R2, R3, R4 | SA | LS on ProSe Out of Coverage discovery |  | Rel-12 | FS_LTE_D2D_Prox | Deutsche Telekom |
| S2 | S2-140568 | R1-140185 | R2, R1, S1, S3 |  | LS reply on discovery message size | R2-134591 (S2-140046) | Rel-12 | FS_LTE_D2D_Prox | Alcatel Lucent, Intel |
| S3 | S3-140226 | R1-140186 | R2 | R1, S1, S2 | Reply to LS on discovery message size | R2-134591 | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R3 | R3-140490 | R1-141050 | R1, R2 |  | LS on Text Proposal for TR 36.843 |  | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R2 | R2-141009 | R1-141051 | R1 |  | LS on Text Proposal for TR 36.843 on D2D |  | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R4 | R4-140969 | R1-141053 | R1 |  | Reply LS on Multiple Access Scheme for D2D | R1-134886 | Rel-12 | FS_LTE_D2D_Prox | Qualcomm |
| R4 | R4-141241 | R1-141054 | R1 |  | Reply LS on AGC and Frequency Error for D2D | R1-134930 | Rel-12 | FS_LTE_D2D_Prox | Qualcomm, Ericsson |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #76

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-140933 | TR 36.843 v1.0.1 on Study on LTE Device to Device Proximity Services | Qualcomm Inc. | Endorsed |
| R1-141052 | 36.866 v1.2.1 on NAICS for LTE | MediaTek Inc. | Agreed |
| R1-141057 | 36.866 v2.0.0 on NAICS for LTE | MediaTek Inc. | Endorsed |
| R1-141061 | TR 36.843 v1.2.0 on Study on LTE Device to Device Proximity Services | Qualcomm Inc. | Agreed |
| R1-141062 | TR 36.873 V1.3.0 on Study on 3D channel model for LTE | NSN | Agreed |

# Annex F:	List of actions

- Outgoing LS.

[76-31] Arjun  Bharadwaj, Qualcomm

| R1-140904 | LS to RAN2 E-DPCCH orders to indicate TTI switching | Qualcomm |  |
| --- | --- | --- | --- |

Email approval until 28th February

Done: The LS is agreed as per email decision dated March 9th.

- CR approval

[76-01] Alexei Davydov, Intel

| R1-141031 | 36213 CRxxxx (Rel-11, F) Correction to CSI processing in TM10 | Intel Corporation, Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Email approval until 21st February

Done: .According to Mr Chair’s decision made on Feb 24th, the CR is agreed as CR0457 in R1-141058. In addition, shadow CR to Rel-12 is agreed as CR0458 in R1-141059.

[76-30] Wang Fan, Huawei

| R1-140678 | 25.214 CR0711 (Rel-10, F) Clarification on (de-)activation status of secondary carriers when higher layer indicates a change of HS-DSCH cells | Huawei, HiSilicon |  |
| --- | --- | --- | --- |

Email review until Friday, 21st February

Done: .According to Mr Chair’s decision made on Feb 27th, revision 1 of the Rel-10 CR (resp. Rel-11 CR) is agreed in R1-140910 (resp. R1-140911).

[76-37] Karri Ranta-aho, NSN

Scalable BW UMTS Package CR to TR25.701 compiling the agreed changes to a single CR,

Email approval until 25th February

Done: .According to Mr Chair’s decision made on Feb 27th, CR is agreed as follows:

| R1-140909 | 25.701 CR0007 (Rel-12, B) Inclusion of Scalable Bandwidth UMTS by filtering and by chip-zeroing | NSN |  |
| --- | --- | --- | --- |

- Text proposal for TS and TR

[76-18] Saurabha Tavildar, Qualcomm

| R1-141043 | Text Proposal for TR 36.843 with for D2D Evaluation Results | Qualcomm Incorporated (Rapporteur) | (R1-141034) |
| --- | --- | --- | --- |

Email approval until 19th February

Done: The final TP is agreed in R1-141056, according to Mr Chair’s decision on Feb 20th.

[76-19] Saurabha Tavildar, Qualcomm

| R1-140944 | DraftTR 36.843 v1.2.0 on Study on LTE Device to Device Proximity Services | Qualcomm Inc. |  |
| --- | --- | --- | --- |

Email approval until 20th February

Done: . Final TR is agreed in R1-141061 as version 1.2.0 (email decision of Feb 25th), including the following open issues listed in conclusion:

- Contents of discovery MAC PDU. (Relevant only to discovery)

- Impact on cellular traffic.

- Impact on other operator spectrum.

- Lawful interception impact on RAN if any.

- Potential impact on inter-eNodeB interfaces

[76-20] Patrick Merias

| R1-141052 | 36.866 v1.2.1 on NAICS for LTE | MediaTek Inc. | (R1-140955) |
| --- | --- | --- | --- |

Email approval until 19th February

Done: . Final TR is agreed in R1-141057 as version 2.0.0 (email decision of Feb 20th)

[76-32] Thomas Chapman, Ericsson

| R1-140986 | Link level simulation assumptions for Scalable UMTS | Ericsson | (R1-140908) |
| --- | --- | --- | --- |

Email approval until 25th February

Done: According to Mr Chair’s decision on Feb 26th, final TP is agreed.

[76-33] Arjun  Bharadwaj, Qualcomm

| R1-140984 | TP on Simulation Results for Scalable UMTS with Filtering | Qualcomm |  |
| --- | --- | --- | --- |

TP based on WF R1-140948 for scalable UMTS for email approval until 25th February

Done: According to Mr Chair’s decision on Feb 26th, final TP is agreed.

[76-34] Arjun  Bharadwaj, Qualcomm and Thomas Chapman, Ericsson

Scalable BW UMTS open issues list for status report

Email approval until 25th February
Done: According to Mr Chair’s decision on Feb 28th, the following open issues are agreed for inclusion in the Status Report:

- Uplink

  - Performance evaluation and identifying the differences between pure filtering and chip-zeroing

    - Discussion and agreement on appropriate UL operation scenario

  - PRACH preamble code design and performance

  - Evaluation of PAPR aspects although it is expected that there would not be an impact.

- Downlink

  - Downlink N=4 standalone performance including SCH

  - Potential bandwidth detection optimization which may result in a change on SCH design

- RAN2 and RAN4 have not been consulted on any aspects within their scope related to FUMTS. RAN1 does not have the expertise to assess these aspects even though some contributions were submitted on these topics..

[76-35] Arjun  Bharadwaj, Qualcomm

| R1-140946 | Enhanced Filtering for Scalable UMTS | QUALCOMM Incorporated, [Huawei, Ericsson, NSN, ALU] |  |
| --- | --- | --- | --- |

Email approval until 25th February

Done: According to Mr Chair’s decision on Feb 26th, final TP is agreed in R1-140989.

| R1-140989 | Enhanced Filtering for Scalable UMTS | QUALCOMM Incorporated | (R1-140946) |
| --- | --- | --- | --- |

[76-36] Arjun  Bharadwaj, Qualcomm

| R1-140988 | TP on conclusions | QUALCOMM Incorporated, Huawei, HiSilicon, NSN, China Unicom | (R1-140949) |
| --- | --- | --- | --- |

Email approval until 25th February

Done: According to Mr Chair’s decision on Feb 26th, TP on conclusions of Scalable UMTS with Filtering is agreed.

- Miscellaneous

[76-10] Zukang Shen, CATT

Open issues for eIMTA

Email discussion/approval until 19th February

Done: The identified open issues are agreed in R1-141055, according to Mr Chair’s decision on Feb 20th.

[76-02] Zukang Shen, CATT

Aperiodic CSI in TM1-9 for eIMTA
Email discussion until 21st February

Done: According to Mr Chair’s decision made on Feb 24th, no consensus is acchieved and there is no more RAN1 meeting time left to eIMTA. The email discussion outcomes are reported in

| R1-141060 | Summary of email discussion [76-02] | CATT |  |
| --- | --- | --- | --- |

[76-03] David Mazzarese, Huawei

Aperiodic CSI in TM10 for eIMTA
Email discussion until 28th February

Done: According to Mr Chair’s decision made on Mar 21st , the following is agreed as working assumption:

- Minimum UE capability on the number of CSI processes for TDD eIMTA is one

- For a Rel-12 UE configured with TDD eIMTA and capable of one CSI process, up to two Rel-11 CSI-IM resources is supported for one CSI process.

  - FFS between the following approaches to support this:

    - One Rel-12 CSI-IM resource is defined to contain up to two Rel-11 CSI-IM resources, i.e. one or two sets of {resourceConfig, subframeConfig}

    - One Rel-12 CSI process is defined to contain up to two CSI-IM resources, i.e. one or two CSI-IM-ConfigId

    - Other approaches are not precluded

  - other details FFS (e.g. possible association of CSI-IM resource to subframe set, details of aperiodic CSI triggering)

- FFS whether this new configuration is supported by Rel-12 UEs without eIMTA capability or Rel-12 UE with eIMTA capability not configured with TDD eIMTA

- FFS whether this new configuration is also supported for 2, 3 and 4 configured CSI processes

- For aperiodic CSI reporting in transmission mode 10 for a single serving cell when the UE is configured with 1 CSI process, only CSI for one subframe set is reported in any subframe.

[76-04] Alexei Davydov, Intel

| R1-141041 | Way Forward on Periodic CSI Reporting for eIMTA | Intel, LGE, Qualcomm |  |
| --- | --- | --- | --- |

Email discussion until 21st February

Done: According to decision made on March 16th , the following is agreed as working assumption:

- For eIMTA capable UEs the CSI measurement subframe set index is used to determine CSI priority for collision handling in periodic CSI reporting

- The priority for CSI reporting is defined as follows:

  - CSI reporting type > CSI process > CC index > CSI subframe index

  - CSI subframe index 0 > CSI subframe index 1

[76-05] Chao Wei, Qualcomm

| R1-141041 | Way Forward on Periodic CSI Reporting for eIMTA | Intel, LGE, Qualcomm |  |
| --- | --- | --- | --- |

Periodic CSI for flexible DL subframe for eIMTA for email discussion until 28th February

Done: According to decision made on March 8th , the following is agreed as working assumption:

- For UEs configured with eIMTA, for periodic CSI report in subframe n, the corresponding CSI reference resource is determined by the existing definition for TM 1 – 10 (in TS36.213 section 7.2.3), with the following additions:

  - If the eIMTA UE receives a valid UL-DL configuration for a radio frame via a reconfiguration DCI, the DL and special subframes according to the indicated UL-DL configuration in the radio frame can be valid CSI reference resources

  - If the eIMTA UE does not receive a valid UL-DL configuration for a radio frame via a reconfiguration DCI, the DL and special subframes according to the SIB-1 UL-DL configuration in the radio frame can be valid CSI reference resources

  - FFS if the eIMTA UE can omit reporting the periodic CSI or report OOR under certain conditions, and the corresponding conditions are also FFS

[76-06] Timo Lunttila, NSN

EPDCCH scheduling for eIMTA for email discussion until 28th February

Done: According to decision made on March 16th , the following way forward is agreed.

| R1-141064 | Way Forward on issues related EPDCCH scheduling for eIMTA (in email discussion [76-06]) | NSN |  |
| --- | --- | --- | --- |

[76-07] Zukang Shen, CATT

R1-141026 HARQ-ACK feedback with PUCCH format 3 for eIMTA

Email discussion until 21st February

Done: No consensus by email discussion deadline. Off line discussion will continue and status be checked in RAN1#76bis.

[76-08] Aris Papasakellariou, Samsung

HARQ-ACK on PUSCH for eIMTA

Email discussion until 21st February

Done: As per decision made on February 27th, a summary is available as:

| R1-141063 | Summary of email Discussions on HARQ-ACK on PUSCH for eIMTA | Samsung |  |
| --- | --- | --- | --- |

[76-09] Zukang Shen, CATT

RRC parameter list for eIMTA for email approval until 14th March

Done: As per decision made on March 19th, the final agreed LS to RAN2 is in R1-141066.

[76-11] Gustav Vos, Sierra Wireless

Scheduling option for low cost MTC

Email discussion until 14th March

Done: Status to be checked in RAN1#76bis.

[76-12] Joon Kui Ahn, LG Electronics

DCI format for TDD-FDD CA

Email discussion/approval until 19th February and after that, email discussion/approval until 21st February

Done: According to Mr Chair’s decision on Feb 27th, the following is agreed:

- 1 bit SRS request field in DCI format 2B/2C/2D (if aperiodic SRS is configured) exists only when TDD SCell is a scheduled cell regardless whether the scheduling cell is TDD or FDD.

- If both PCell and the scheduled SCell use FDD:

  - DL DCI and UL DCI formats: no UL index, no UL DAI, no DL DAI

  - DL DCI formats include 3-bit HARQ process number field

- If TDD PCell is supported, when PCell uses TDD and the scheduled SCell uses FDD:

  - DL DCI formats include 2-bit DL DAI and 4-bit HARQ process number field

  - UL DCI formats include 2-bit UL DAI

  - No UL index field exists in UL DCI formats

- For other cases,

  - Option B as working assumption for FDD PCell case

  - Option B as working assumption for TDD PCell case, if TDD PCell self-scheduling is supported

[76-13] Klaus Hugl, Nokia

RRC parameter list for TDD-FDD CA for email approval until 14th March

Done: As per decision made on March 19th, it is agreed that RAN1 Rel-12 TDD-FDD CA design requires the UE to support simultaneous RX and TX (i.e. full-duplex) for the indicated TDD-FDD CA band combinations. This is reflected in final agreed LS to RAN2 in R1-141065.

[76-14] Havish Koorapaty, Ericsson

| R1-141015 | Way Forward on Transition Time Reduction for Small Cell On/Off | Ericsson, Qualcomm |  |
| --- | --- | --- | --- |

Email discussion until 14th March

Done: According to email’s decision on March 21st, no conclusion can be drawn. Nervertheless, some proposal of new L1 procedures to further reduce the transition time  might be considered as guideline for study/clarification in the next meeting.

[76-15] Bishwarup Mondal, NSN

| R1-141062 | TR 36.873 V1.3.0 on Study on 3D channel model for LTE | NSN |  |
| --- | --- | --- | --- |

TR36.873 update for 3D-channel model
Email discussion until 21st February

Done: TR version 1.3.0 is agreed as per email decision made on Feb 26th.

[76-16] Bishwarup Mondal, NSN

Phase 2 calibration for 3D-channel model for email submission until 28th February

Done: Status to be checked during RAN1#76bis.

[76-17] Bishwarup Mondal, NSN

Baseline simulation for 3D-channel model for email submission until 28th February

Done: Status to be checked during RAN1#76bis.

# Annex G:	List of participants at RAN1 #76

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2014 - 2015

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#76bis | WG | 31 Mar – 04 Apr 2014 | Shenzhen | China |
| 3GPPRAN1#77 | WG | 19 – 23 May 2014 | Seoul | KR |
| 3GPPRAN1#78 | WG | 18 – 22 Aug 2014 | Dresden | Germany |
| 3GPPRAN1#78bis | WG | 6 – 10 Oct 2014 | Ljubljana | Slovenia |
| 3GPPRAN1#79 | WG | 17 – 21 Nov 2014 | San Francisco (TBC) | US |
| 3GPPRAN1#80 | WG | 9 – 13 Feb 2015 | TBD | EU |
| 3GPPRAN1#80bis | WG | 20 – 24 Apr 2015 | TBD | EU |
| 3GPPRAN1#81 | WG | 25 – 29 May 2015 | TBD | TBD |
| 3GPPRAN1#82 | WG | 24 – 28 Aug 2015 | TBD | China |
| 3GPPRAN1#82bis | WG | 5 – 9 Oct 2015 | TBD | TBD |
| 3GPPRAN1#83 | WG | 16 – 20 Nov 2015 | TBD | TBD |

| MEETING TYPES |  |
| --- | --- |
| AH = Ad Hoc | CM = Chairmen's meeting |
| JM = Joint | OR = Ordinary |
| PM = Preparatory Meeting | RG = Rapporteurs Group |
| RM = Resolution Meeting | SG = Steering Group |
| ST = Startup Meeting | TG = Task Group |
| WG = Working Group | XO = Extraordinary |

End of document