3GPP TSG RAN WG1 Meeting #76bis                                             R1-141206

Shenzhen, China, March 31st to April 4th, 2014

Source:	NEC

Title:	Signaling Considerations for Inter-eNB CoMP

Agenda Item:	7.2.6

Document for: 	Discussion and Decision

# Introduction

During RAN #63, the following conclusion on ``Inter-eNB CoMP for LTE” was reached [1].

RAN agrees to reduce the scope of WI to the items listed below based on X2 interface.

- A CoMP hypothesis comprising a hypothetical resource allocation for at least the receiving node in time/frequency domains

  - How to react to a received CoMP hypothesis signaling is up to receiving eNB’s implementation. E.g. accept or ignore, potentially sending a feedback e.g. “yes/no” to the sending node.

  - RAN1 should provide guidance to RAN3 on necessary granularity and rate of CoMP hypothesis in time/frequency domain.

- One or more sets of CSI information (RI, PMI, CQI) of a set of UEs that can be supported taking into account limitations of existing X2 interface

  - RAN1 should provide guidance to RAN3 on necessary rate of exchanging one or more sets of CSI reports over X2 interface

- One or more measurement reports (RSRP) of a set of UEs

- Enhanced RNTP can be signaled between eNBs to facilitate CoMP

  - Information granularity of the Enhanced RNTP is extended to the frequency/time domain

    - RAN1 should provide guidance to RAN3 on necessary granularity and rate of Enhanced RNTP in time domain

  - Information in the Enhanced RNTP is (optionally multi-level) transmit power threshold for only the sender eNB.

    - RAN1 should provide guidance to RAN3 on necessary granularity of transmit power threshold and how many levels should be defined

  - Possible enhancement on existing Status report, which can be signaled between eNBs to exchange the usage status of the indicated frequency/time resources

- Details of benefit metric should be decided in RAN1 and should be provided to RAN3 from RAN1#76bis

# Scheduling Scheme for CoMP with Non-ideal Backhaul

The CoMP schemes that were discussed during the Rel-11 CoMP standardization assumed the availability of an ideal backhaul connecting the transmission points in each cluster. This assumption allowed for coordination within the cluster based on the instantaneous CSI reported by the users to those transmission points. Unfortunately, such schemes are far from being suitable when faced with a non-ideal backhaul that has a high latency. To guide the design of schemes that are appropriate for the NIB scenario, the following agreement was reached during RAN1#74:

For each evaluated scheme, information relating to a transmission to/from a serving node in a given subframe should be categorized into two groups:

- Group 1 information: information which is considered valid for a period longer than the backhaul delay, which may therefore be provided from a different node(s) from the serving node;

- Group 2 information: information which is considered valid for a period shorter than the backhaul delay, which must therefore be derived by the serving node.

The types of information may include for example:

  - CSI

  - Allocated power per resource (including muting)

  - UE selection

  - Precoding selection (including the number of transmit layers)

  - MCS selection

  - HARQ process number

  - TP selection

Scheduling techniques conforming to this agreement have been proposed in [2]-to-[7] and performance results have been summarized. In [8], we proposed a mathematical framework for designing a scheduling scheme for CoMP-NIB consistent with the above agreement. That framework allows for the construction of hybrid scheduling schemes where certain actions (such as the assignment of a precoder for each TP in the coordination set or cluster and set of the users associated to each TP in that cluster) are made at a centralized node at a coarse time-scale, while the remaining ones that rely on fast changing information (such as the per subframe user scheduling at each TP) are independently made by each TP at a fine time scale.

In [9] we elaborated on the required signaling support. Here, we update that contribution by including a distributed approach along with views on signaling consistent with the latest mandate in [1].

3. Signaling Support

We have captured the key details of the scheduling framework in the appendix. We assume that for each user a measurement set containing up-to three TPs among those in the coordination set is defined and held fixed for a time scale even coarser than the one at which the centralized decisions (precoder tuple or muting pattern assignment and possibly user association) are made.

From the description given in the appendix, we see that to determine the centralized decisions (such as the  precoder tuple assignment and the user associations) under the full buffer traffic model, the designated central node (referred to here as the master TP (MTP)) should be able to obtain,,which we recall denotes an estimate of the average rate that user  can obtain (over the available time-frequency resource normalized to have size unity) when it is served data by TP, given that the precoder tuple is assigned to the  TPs in the zone and that no other user is associated with TP  . Recall also that the precoder tuple can also correspond to a muting pattern deciding which TPs should be active and which should be turned off in the time-frequency unit. For the joint SSPM and SSPS scheme (cf. (P1) in the appendix), this average estimatemust be obtained for each user , each TPin its measurement set and for all precoder tuple assignments.  Note that for any precoder tuple,  can be considered to be negligible if the TP is not in the measurement set of user. Notice also that can be assumed to be equal to  for any two precoder tuple assignments and   which differ only in precoders assigned to TPs not in the measurement set of user  . For the SSPM problem (cf. (P2) in the appendix) with pre-determined user associations, the average estimatemust be obtained for each user only for its pre-determined serving TP, but the set of users associated to  that TP must also be obtained.  Thus, the following types of backhaul signaling are needed to facilitate a centralized implementation.

3.1 Backhaul Signaling to enable determination of centralized actions (such as precoder tuple/muting pattern assignments and the user associations)

We will now consider computation of the average rate estimates   at the MTP for some user , under  a precoder tuple assignment . These rates depend on the channels that the user sees from TPs in its measurement set. Using up-to three CSI processes (recall that the maximum measurement set size is three) which include a common IMR, the UE can report short-term CSI for each TP in its measurement set, where this short-term CSI is computed based on the non-zero CSI-RS transmitted by TP and the interference observed on the IMR, which in turn includes only the interference from TPs not in the measurement set of user . The UE currently reports such CSI only to its designated anchor TP.

However, to fully exploit point switching gains we need to allow for the possibility of associating a user to a non-anchor TP and then allowing that user to report instantaneous (short-term) CSI to the non-anchor TP it has been associated to.  Further, the CSI processes should be defined in a coordinated manner so that the users measure the appropriate interference on the constituent IMRs. Such coordinated configuration of IMRs also provides the ability to inject the desired interference (such as isotropically distributed interference) onto resource elements in those IMRs.

These short-term CSI can be sent to the MTP over the backhaul, which can then filter (i.e. perform a weighted average of) the received CSI sequence to obtain an averaged channel estimate for each TP in the measurement set of user . Alternatively, the averaging (or subsampling) of the short-term CSI can be done by the TP receiving the short-term CSI but where the averaging window (and possibly the weighting factors or subsampling factors) can be configured for that UE on a per CSI-process basis.

In either case, these averaged or subsampled channel estimates for all TPs in that UE’s measurement set can be used by the MTP to compute for each precoder tuple hypothesis and  if needed each TPin its measurement set, under the assumption that the signal transmitted by each TP (along its assigned precoder under that hypothesis) is isotropically distributed.

Moreover, the signaling of CSI over the backhaul should enable the identification of the users whose CSI are being signaled as well as the attributes of the corresponding CSI processes. Recall also that in the scenario with pre-determined users associations, the set of users associated to each TP in the cluster needs to be exchanged or conveyed to the MTP. These views are summarized in the following proposal.

Proposal:    Signaling of averaged or subsampled CSI obtained over CSI processes corresponding to a configurable set of users should be considered. Coordination in configuring these CSI processes should be allowed.

Proposal: Possibility of configuring a user to report short-term CSI to more than one TP or a chosen TP in a configurable set of TPs should be considered.

Next,  in the more general finite buffer model [9] estimates of the queue sizes are needed to determine each coarse (centralized) action, where each such user queue size represents the amount of traffic that would available for transmission to serve that user until the next coarse action. Determining estimates of these queue sizes requires the TPs to report   their most-recently updated associated user queue sizes before the next coarse action to the MTP.

Proposal: Signaling of associated user queue sizes by a TP to another TP should be considered. .

3.2 Backhaul Signaling from MTP to TPs

Each TP in the coordination set is informed (semi-statically) about the precoder it should use and possibly the users it should serve on a time-frequency resource. The decision made by the MTP can be represented using a CoMP hypothesis.  This can be achieved for instance, by assigning an identifier to each TP in the coordination set, and then including pairs representing (TP identifier, corresponding part of decision) in the CoMP hypothesis. Each TP then implements its own per-subframe scheduling based on the instantaneous CSI it receives from the users associated to it. Some comments on the setwhich contains the set of precoders that can be assigned to each TP, are on order. We recall that this set includes codeword 0 to subsume muting as a special case. It can also include codewords of the form where denotes a positive power level. In addition, it can include sector beams as its codewords. Notice that so far we have implicitly assumed that each TP will accept the decision made by the MTP. This assumption need not always hold, in which case it is beneficial (even necessary) to have an acknowledgement from the receiving TP conveying whether or not it accepts to implement its part of the decision in the CoMP hypothesis.

Proposal:    Signaling of decisions made by one TP (such as precoder set or muting pattern assignment) to the other TPs over the backhaul should be considered. Such a decision can be represented by a CoMP Hypothesis. Signaling of an acknowledgement conveying a yes/no response to a received CoMP hypothesis should be considered.

3.3 Distributed Implementation

In order to enable a de-centralized or distributed operation, a benefit metric corresponding to each CoMP hypothesis can be defined. In [10] a distributed implementation of power control is provided. An example distributed operation considering binary power control is described next, and we note that extension to multiple power levels can be developed following the same approach. Each TP b in the coordination set can determine its set of interfering TPs, where a TP is labelled interfering for TP b if it is in the measurement set of at-least one user associated to TP b.  Note that TP b can determine its set of interfering TPs. Further, let us refer to all TPs in whose interfering sets TP b is present as the out neighbor set of TP b. Each CoMP hypothesis can be defined such that the sending TP, say TP b, suggests a muting (or in general a power level) pattern for a set of time-frequency resources to a receiving TP, say TP a, in its interfering set of TPs. The benefit metric for that hypothesis comprises of a set gain (or loss, i.e., the gain can be negative) values (one for each time-frequency resource), where each gain represents the incremental average throughput or utility that would be achieved for the sending node (TP b) if the receiving node (TP a) accepts the suggested muting or power level (henceforth termed suggested action) on that time-frequency resource, while the other TPs in the interfering set of TP b as well as TP b do not alter their current status. TP a can then consider each time-frequency resource and add up all the gain values it has received for each suggested action on that resource. To this sum it can then add the gain (or loss) that it would obtain upon following the suggested action, assuming that all TPs in its interfering set do not alter their current status. This sum gain for each action can then represent the system  utility gain (or loss) that can be achieved by a one-step change, i.e., the incremental utility gain for the coordination set achieved when TP a accepts that suggested action on that resource and all the other TPs in the coordination set  keep their current respective status. TP a can then independently choose its action using a probabilistic rule [10], and this distributed operation provably converges.  Further, TP a can signal its choice of actions using an enhanced RNTP.

However, a potential drawback with the distributed approach described above is if the benefit metrics do not allow a TP to infer (a good approximation of) the system utility gain (or change) accrued by a suggested action on a time-frequency resource, in which case oscillatory behavior or convergence to a highly sub-optimal operating point can result. We summarize our views in the following proposal.

Proposal:    The benefit metrics received by a TP should enable it to compute a system utility change  for each action suggested for that TP in each of its received CoMP hypothesis.

# Conclusion

In this contribution, we provided our views on backhaul signaling needed for CoMP-NIB comprising of the following proposals.

Proposal:    Signaling of averaged or subsampled CSI obtained over CSI processes corresponding to a configurable set of users should be considered. Coordination in configuring these CSI processes should be allowed.

Proposal: Possibility of configuring a user to report short-term CSI to more than one TP or a chosen TP in a configurable set of TPs should be considered.

Proposal: Signaling of associated user queue sizes by a TP to another TP should be considered.

Proposal:    Signaling of decisions made by one TP (such as precoder set or muting pattern assignment) to the other TPs over the backhaul should be considered. Such a decision can be represented by a CoMP Hypothesis. Signaling of an acknowledgement conveying a yes/no response to a received CoMP hypothesis should be considered.

Proposal:    The benefit metrics received by a TP should enable it to compute a system utility change  for each action suggested for that TP in each of its received CoMP hypothesis.

# References

- 3GPP RP-140523, “Way Forward on WI Inter-eNB CoMP for LTE”, Mar. 2014.

- 3GPP R1-135228, Samsung, “Evaluation of coordinated scheduling for SCE scenario 1 with non-ideal backhaul,” Nov. 2013.

- 3GPP R1-135490, LGE, “Further evaluation results for DL CoMP with non-ideal backhaul,” Nov. 2013.

- 3GPP R1-134587, Motorola Mobility, “Signaling considerations for inter-eNB CoMP,” Oct. 2013.

- 3GPP R1-134540, Nokia, “Signaling design considerations to support inter-eNB CoMP,” Oct. 2013.

- 3GPP R1-135523, NTT-DoCoMo, “Further Evaluation Results for CoMP-NIB,” Nov. 2013.

- 3GPP R1-135048, Huawei,HiSilicon, “Further performance evaluation of CoMP with non-ideal backhaul,” Nov. 2013.

- 3GPP R1-135271, NEC, “Signaling and Scheduling issues in CoMP with non-ideal backhaul,” Nov. 2013.

- 3GPP R1-140421, NEC, “Signaling considerations for CoMP with non-ideal backhaul,” Feb. 2014.

- H. Zhang, L. Venturino, N. Prasad, P. Li, S. Rangarajan, X. Wang, "Weighted Sum-Rate Maximization in Multi-Cell Networks via Coordinated Scheduling and Discrete Power Control", IEEE Journal on Selected Areas in Communications, 29(6): pp. 1214-1224, 2011

# Appendix

# Optimizing Proportional Fairness Utility Metric

Suppose that there are K users and B transmission points (TPs) in the CoMP cluster or coordination set of interest, where this set can include multiple eNBs.  For convenience in exposition, here we assume a full buffer traffic model (and refer to [9] for the finite buffer model) and let denote the set of K users. We consider hybrid schemes where the assignment of precoding matrices (beamforming vectors or sectored beams) to the B TPs and the association of users with those TPs (i.e., point switching) are done in a semi-static centralized manner based on average estimates of SINRs, rates etc. On the other hand, given its assigned precoder (or beam) and the users associated with it, each TP does per sub-frame scheduling independently based on the instantaneous short-term CSI.

Let denote an assignment of a precoder tuple, where is the precoder assigned to the bth TP. Here each precoder can be chosen from a pre-determined finite set which includes a codeword 0 and means that the bth TP is muted. Thus, SSPM is subsumed as a special case.

Then, let denote an estimate of the average rate that user  can obtain (over the available time-frequency resource normalized to have size unity) when it is served data by TP, given that the precoder tuple is assigned to the B TPs and that no other user is associated with TP  . This time-frequency unit could for example be a set of resource blocks. Next, suppose that total users are associated with TP. Following the conventional approach, the average rate that user  can then obtain under proportional fair per-subframe scheduling can be approximated as.

With these definitions in hand, we can jointly determine the assignment of a precoding tuple and the user association (i.e., jointly consider semi-static coordinated beamforming (SSCB) and semi-static coordinated point-switching (SSPS) problems) by solving the following optimization problem:

Note that in (P1), each is an indicator variable which is equal to one if user  is associated with TPand zero otherwise. Therefore the constraint in (P1) enforces that each user must be associated with only one TP.  It can be shown that (P1) cannot be solved optimally in an efficient manner, which necessitates the design of low-complexity algorithms that can approximately solve (P1).  In [8] it is shown that for any given precoder tuple the SSPS sub-problem can be optimally solved. Alternatively, a greedy approach can be adopted to achieve further complexity reduction. The latter greedy SSPS algorithm is given in Table I in [8].

These solutions to the SSPS problem can be leveraged to obtain an algorithm to sub-optimally solve the joint SSCB and SSPS problem (P1). One such algorithm is depicted in Table II in [8].We next consider the SSPM-only problem where user associations are pre-determined.

Here  denotes the pre-determined set of users associated to TP b and  denotes its cardinality.

(P2) is also in general a hard problem which cannot be solved optimally in an efficient manner. Good heuristics can nevertheless be developed to solve (P2).