3GPP TSG-RAN1 #76bis	R1-141228

Shenzhen, China, 31st March - 4th April 2014

Agenda Item:	7.2.7.1.1

Source: 	Fujitsu

Title: 	On D2D discovery signal design details

Document for:	Discussion and decision

# Introduction

In the RAN1#76 meeting, we achieved a working assumption regarding discovery signal and message design details:

Working Assumption:

- A UE that transmits a discovery message may also be a synchronisation source (and therefore also transmit D2DSS)

  - Note that D2DSS is not the message DMRS

- “Discovery sequence” concept introduced in working assumption in RAN1#74bis no longer exists. (but see open discussion below on discovery preamble)

- None of the discovery message payload is carried by the message DMRS

- A UE transmitting a discovery message does not necessarily need to be a synchronisation source

Also for further work, it was noted that:

Further discussion on Discovery Preamble:

Question: Can a UE that transmits a discovery message (including DMRS) also transmit a “discovery preamble” prior to the discovery message, where the “discovery preamble” differs from a D2DSS in one or more of the following ways:

- E.g. if D2DSS is periodic and “discovery preamble” is not?

- Different sequence?

- Different bandwidth?

Possible purposes for “discovery preamble”:

- Fine tuning of time and/or frequency synchronisation

- Channel estimation assistance for discovery message decoding

- AGC setting

In this contribution, we further share our views on the necessity of employing D2D discovery preamble before transmission of the discovery message.

# Discussion and analysis

## Separate or compact Discovery Sequence + Message structure

As noted in [2], currently two basic choices for Discovery Signal structure are “separate” and “compact” designs. In this section we provide our views on the comparison of these two approaches:

- Separate DS design: Discovery preamble and discovery message can be designed separately to achieve optimal performance. Discovery preamble and discovery message can occupy different amount of bandwidth. Because the receiver detects the discovery signal blindly, it is important to reduce the overhead of the blind detection. Discovery preamble can be designed with a fixed bandwidth and location, possibly wider than the bandwidth of the discovery message, for easy detection and better detection performance. It is also easy to multiplex the discovery preambles of different UEs.  The discovery preamble can also serve the function of AGC training and frequency/time offset estimation at the receiver. The detection preamble can carry an indicator for the discovery message (including RB resources, message format, etc), making the discovery message more flexible and reducing the detection complexity of the UE receiver. The discovery message may have a different bandwidth than the discovery preambles in order to meet the need for reliable detection under different scenarios. Using the discovery preamble as an indicator for the discovery message provides this flexibility without increasing the blind detection complexity at the UE.

- Compact DS design: The discovery preamble is not transmitted prior to the discovery message. This reduces the resources usage and also reduces the scope of the specification change. If the receiving UE can resolve the AGC and synchronization issues, and has some pre-knowledge of the resource allocation, a compact discovery signal design offers the lowest overhead and lowest detection complexity. The DMRS can be used for channel estimation and demodulation for the discovery message. It also has lower detection latency.

The pros and cons of these two options are listed in Table 1 below:

Table.1 Comparison of the separate and the compact Discovery Signal structures

| DS structure options | Pros | Cons |
| --- | --- | --- |
| Compact Discovery Sequence + Message structure (Figure.1) | - Less resource needed for discovery signals - Shorter discovery latency - Discovery sequence doubles as DMRS, and offers better demodulation performance. - Low complexity if with ideal synchronization and pre-knowledge of resource allocation - Less standardization effort | - Discovery sequence occupies the same bandwidth as the discovery message, limiting the sequence detection performance. - Search of discovery message cannot benefit from discovery sequence. - Potentially large search space for discovery signal, incurring larger power consumption for receiving UEs - Difficult to multiplex discovery messages from multiple UEs - Separate solution for AGC training and synchronization needed - Lack of flexibility when supporting Idle-mode UEs and out-of-coverage UEs |
| Separate Discovery Preamble + Message structure (Figure.2) | - Discovery sequence and message can occupy different amount of bandwidth, respectively optimized for sequence detection and message decoding performance. - Discovery sequence can carry an indicator for the discovery message (resource, format, etc), allowing flexibility of the message - Low search space for discovery sequence and message, reducing receiver UEs power consumption - Ease of multiplexing of discovery sequences from multiple UEs - Sequence can be used for AGC training and frequency estimation | - More resource needed for discovery signals - Larger discovery latency - DS sequence cannot be used as DMRS - More standardization effort needed for separate sequence and message design |

To summarize, the separate discovery sequence and message option offers more flexibility and better performance in terms of detection reliability and UE power consumption. These benefits outweigh the downside of slightly larger resource usage and longer latency, which is well worth the extra standardization effort.  Hence we conclude separate discovery sequence and message design is a better solution and should be adopted.

Proposal 1: Separate Discovery preamble and message design should be supported.

Current downlink/uplink sequences/signals or preambles could be considered as candidates of Discovery sequence. As mentioned in [5], PSS and PRACH should be firstly considered due to the benefits of easier operation and reusing receiver /transmitter processing.

Different signal structure and sequence length could meet different discovery range and load requirements. Since discovery signal shall be carried by UL spectrum, the resource mapping should be carefully studied. There are possibilities that discovery sequence may collide with current cellular UL signals (e.g. SRS, DMRS) or UCIs, and the impact of such collision should be carefully studied.

Proposal 2: Resource mapping of discovery preamble should be further studied to avoid serious impact to cellular UL signals.

Sequence could also serve the function of message resource indication, which further enhances the flexibility of resource allocation. This is of great efficiency to reduce complexity of receiver.

Proposal 3: Discovery preamble could be used to indicate corresponding message RB location.

Figure.1 Examples of compact sequence + message structure

Figure.2 Examples of separate sequence + message structure

## Synchronous VS asynchronous discovery

To further alleviate the power consumption of the transmitters and interference, asynchronous D2D discovery could be considered. The agreed synchronization procedure could be reused that before starting to transmit D2D discovery signal, a UE scans for discovery signals from other UEs. Certain UEs with authority or request from upper layer shall trigger the initial DS transmission. Asynchronous discovery has the following advantages over synchronous discovery procedure (UEs are supposed to transmit or receive DS periodically following certain time domain patterns):

- Due to the D2D UE half-duplex restriction, the DS transmissions from D2D UEs need to be staggered in the time domain as much as possible, since fixed and well aligned transmission pattern would limit the opportunities for UEs to detect each other. Asynchronous DS transmission procedure is capable to avoid this issue naturally. Compared with synchronous discovery, it takes limited degree of network coordination to achieve orthogonality between UEs. When the UEs are out of network coverage, it is natural to allow the UEs to transmit DS autonomously following some random pattern to achieve pseudo-orthogonality or probabilistic detection.

- When supporting large number of UEs capable of ProSe ([6]), synchronous discovery performance may suffer from severe interference.

- Scanning before transmitting could largely reduce the overall power consumption of transmitters.

Proposal 4: Asynchronous discovery should be supported.

# Conclusion

In this contribution, we share our views on D2D discovery signal and procedure. Basically, our proposals are summarized as:

Proposal 1: Separate Discovery preamble and message design should be supported.

Proposal 2: Resource mapping of discovery preamble should be further studied to avoid serious impact to cellular UL signals.

Proposal 3: Discovery preamble could be used to indicate corresponding message RB location.

Proposal 4: Asynchronous discovery should be supported.

# References

- R1-135223, “Physical channel design for D2D discovery”, Samsung, 3GPP RAN1#75 meeting, San Francisco, USA, November 2013

- R1-135143, “On D2D discovery signal design options”, Fujitsu, 3GPP RAN1#75 meeting, San Francisco, USA, November 2013

- R1-135504, “D2D discovery signal and message content”, HTC, 3GPP RAN1#75 meeting, San Francisco, USA, November 2013

- R1-135323, “D2D discovery signal design”, Qualcomm Inc., 3GPP RAN1#75 meeting, San Francisco, USA, November 2013

- R1-134263, “D2D discovery signal”, Fujitsu, 3GPP RAN1#74bis meeting, Guangzhou,  China, October 2013

- 3GPP TS 22.278, "Technical Specification Group Services and System Aspects; Service requirements for the Evolved Packet System (EPS)", rev 12.3.0, June 2013