3GPP TSG-RAN WG1 #77	R1-142167

Seoul, Korea, 19th – 23rd May. 2014

Agenda item:	6.2.3.1

Source:	NEC

Title:	UE category to support 256QAM

Document for:	Discussion/Decision

Introduction

A WF on UE category to support 256QAM [1] was discussed in RAN1#76-bis meeting. It was proposed to add second value to the existing UE categories 3 to 10. It would mean each category has two speed classes.

In this contribution, we provide our view on this topic.

Discussion

Discussion on existing categories

UE category by itself represents UE speed class irrespective of capability as provided in Table 2.1.

Table 2.1: list of existing speed classes (approximate)

| UE Category | Speed class |  |
| --- | --- | --- |
|  | DL | UL |
| 1 | 10Mbps | 5Mbps |
| 2 | 50Mbps | 25Mbps |
| 3 | 100Mbps | 50Mbps |
| 4 | 150Mbps | 50Mbps |
| 5 | 300Mbps | 75Mbps |
| 6 | 300Mbps | 50Mbps |
| 7 | 300Mbps | 100Mbps |
| 8 | 3Gbps | 1.5Gbps |
| 9 | 450Mbps | 50Mbps |
| 10 | 450Mbps | 100Mbps |

Therefore it would be confusing if existing category would represent two speed classes depending on capability of 256QAM. In our view, this approach would actually increase speed classes but just not to have name (or category) for it to identify the new speed classes.

It would be preferable to keep UE category by itself represents UE speed class irrespective of capability of 256QAM.

Proposal:

- Keep each UE category by itself represents UE speed class irrespective of capability

- Do not change “Maximum number of DL-SCH transport block bits received within a TTI” of existing UE categories in [2]

  - No need to change “Total number of soft channel bits” in [2]

Meanwhile, as proposed in [3], it will make sense to have increased “Maximum number of bits of a DL-SCH transport block received within a TTI” of existing categories (4 or higher) to address 256QAM without changing “Maximum number of DL-SCH transport block bits received within a TTI”. Category 6/7 UE with 256QAM capability, for example, may achieve 200Mbps in single cell operation.

Regarding categories 1 – 3, we don’t see necessity of optimization to address 256QAM as they are intended for reduced speed classes and lower cost.

If UE speed classes addressing 256QAM would be needed, new UE categories should be introduced.

Proposal:

- Introduce new UE categories to address higher UE speed class enabled by 256QAM if necessary

On the other hand, it would not be preferable to increase UE category number so much. One possible approach could be to introduce a naming rule of UE category addressing 256QAM, e.g. to append suffix, such as “A” to the categories not supporting 256QAM, like Category 4A, etc. This approach actually increases UE category but may not confuse user so much.

New categories

Possible downlink speed classes with / without support of 256QAM as well as existing one are listed in Table 2.2. Looking at the table, some of speed classes are very similar to the next one. Therefore it would be not needed to support all of speed classes. Some could be covered by higher one.

Regarding UL speed class, two categories are added for one downlink speed class with different uplink speed classes from Rel-10. It might be preferable to take into account how to handle UL 64QAM at the same time if it could be decoupled from Category 5 and 8 and become a capability.

Table 2.2: list of existing and candidate downlink speed classes

| DL speed class | Covered scenario | Note |
| --- | --- | --- |
| 10Mbps |  | Category 1 |
| 50Mbps |  | Category 2 |
| 100Mbps |  | Category 3 |
| 150Mbps | 20MHz * 2 layers * 64QAM | Category 4 |
| 200Mbps | 20MHz * 2 layers * 256QAM | (This could be covered by Category 5/6/7) |
| 300Mbps | 20MHz * 4 layers * 64QAM 40MHz * 2 layers * 64QAM | Category 5/6/7 |
| 400Mbps | 40MHz * 2 layers * 256QAM 20MHz * 4 layers * 256QAM | (This could be covered by Category 9/10) |
| 450Mbps | 60MHz * 2 layers * 64QAM | Category 9/10 |
| 600Mbps | 80MHz * 2 layers * 64QAM 60MHz * 2 layers * 256QAM 40MHz * 4 layers * 64QAM 20MHz * 8 layers * 64QAM |  |
| 750Mbps | 100MHz * 2 layers * 64QAM |  |
| 800Mbps | 80MHz * 2 layers * 256QAM 40MHz * 4 layers * 256QAM 20MHz * 8 layers * 256QAM |  |
| 900Mbps | 60MHz * 4 layers * 64QAM |  |
| 1Gbps | 100MHz * 2 layers * 256QAM |  |
| 1.2Gbps | 80MHz * 4 layers * 64QAM 60MHz * 4 layers * 256QAM 40MHz * 8 layers * 64QAM |  |
| 1.5Gbps | 100MHz * 4 layers * 64QAM |  |
| 1.6Gbps | 80MHz * 4 layers * 256QAM 40MHz * 4 layers * 256QAM |  |
| 1.8Gbps | 60MHz * 8 layers * 64QAM |  |
| 2Gbps | 100MHz * 4 layers * 256QAM |  |
| 2.4Gbps | 80MHz * 8 layers * 64QAM 60MHz * 8 layers * 256QAM |  |
| 3Gbps | 100MHz * 8 layers * 64QAM | Category 8 |
| 3.2Gbps | 80MHz * 8 layers * 256QAM |  |
| 4Gbps | 100MHz * 8 layers * 256QAM |  |

Summary

Proposal:

- Keep each UE category by itself represents UE speed class irrespective of capability

- Do not change “Maximum number of DL-SCH transport block bits received within a TTI” of existing UE categories in [2]

  - No need to change “Total number of soft channel bits” in [2]

- Introduce new UE categories to address higher UE speed class enabled by 256QAM if necessary

  - It could be considered to have a naming rule of UE category addressing 256QAM, e.g. to append suffix “A” to the category, such as Category 4A, instead of introducing completely new Category number.

References

[1] R1-141879	WF on 256QAM UE category handling	Ericsson, CMCC, CATR, Huawei, HiSilicon, Samsung, Blackberry, Fujitsu, III, FiberHome, CATT, Coolpad, China Unicom

[2] TS36.306

[3] R1-141531	On UE categories for 256QAM support	Nokia, NSN

Annex

Table 4.1-1 [36.306]: Downlink physical layer parameter values set by the field ue-Category

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI (Note) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| Category 1 | 10296 | 10296 | 250368 | 1 |
| Category 2 | 51024 | 51024 | 1237248 | 2 |
| Category 3 | 102048 | 75376 | 1237248 | 2 |
| Category 4 | 150752 | 75376 | 1827072 | 2 |
| Category 5 | 299552 | 149776 | 3667200 | 4 |
| Category 6 | 301504 | 149776 (4 layers) 75376 (2 layers) | 3654144 | 2 or 4 |
| Category 7 | 301504 | 149776 (4 layers) 75376 (2 layers) | 3654144 | 2 or 4 |
| Category 8 | 2998560 | 299856 | 35982720 | 8 |
| Category 9 | 452256 | 149776 (4 layers) 75376 (2 layers) | 5481216 | 2 or 4 |
| Category 10 | 452256 | 149776 (4 layers) 75376 (2 layers) | 5481216 | 2 or 4 |
| NOTE: In carrier aggregation operation, the DL-SCH processing capability can be shared by the UE with that of MCH received from a serving cell. If the total eNB scheduling for DL-SCH and an MCH in one serving cell at a given TTI is larger than the defined processing capability, the prioritization between DL-SCH and MCH is left up to UE implementation. |  |  |  |  |