3GPP TSG RAN WG1 Meeting #77,	R1-142376

Seoul, Korea, 19th – 23rd May 2014

Source:	Ericsson

Title:	TBS table design for downlink 256QAM

Agenda Item:	6.2.3.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #76bis meeting, the TBS table design principles for DL 256QAM were discussed and agreements were made as listed below:

- Define overhead assumption(s) (REs/PRB) for PDSCH

  - Use 120 REs per PRB for all 256QAM spectral efficiencies except for the highest spectral efficiency

  - Use 136 REs per PRB for the highest spectral efficiency

- Limit the number of new TBS values as much as possible

In this contribution, we provide the design of TBS table aligned with above agreements to support DL 256QAM. The CQI table and MCS table design is discussed in [1].

# Discussion

In Rel-11, the 27x110 TBS table for one spatial layer is populated with 178 unique transport block sizes (TBS) as given in Table 7.1.7.2.1-1 in [2]. To support multiple spatial layers, 51 additional unique TBS are defined for one-layer to two-layer, one-layer to three-layer and one-layer to four-layer TBS translation given in Table 7.1.7.2.2-1, Table 7.1.7.2.4-1, Table 7.1.7.2.5-1 in [2]. In total, there are 229 unique TBS defined in Rel-11. The design allows the TBS tables to be very efficiently stored using 8-bit indices to the unique TBS list instead of 19-bit integers representing the TBS directly.

The reduction of potentially 27x110=2970 TBS to only 178 TBS in the one layer table is achieved by allowing the actual code rate to deviate from the target code rate by ±2%. This is for example shown in Figure 1 for the 64QAM MCS (17-27), in Figure 2 for the 16QAM MCS (10-16), and Figure 3 for the QPSK MCS (0-9), respectively.

Observation:

- By allowing the actual code rate to deviate from the target code rate by ±2%, there are in total 229 unique TBS defined in Rel-11 enabling efficient TBS table storage.

Figure 1 Actual code rate and code rate inaccuracy in percentage for Rel-11 64QAM assuming 120 RE/PRB.

Figure 2 Actual code rate and code rate inaccuracy in percentage for Rel-11 16QAM assuming 120 RE/PRB.

Figure 3 Actual code rate and code rate inaccuracy in percentage for Rel-11 QPSK assuming 120 RE/PRB.

To support 256QAM in Rel-12, six new entries containing six new TBS indices 27-32 are proposed to be added in the MCS table as discussed in [1]. Consequently, six additional rows of TBS need to be added to the existing 27x110 TBS table for one spatial layer. By allowing up to ±2% deviation from the target code rate, the 6x110 TBS can be designed without introducing any new TBS for 256QAM one spatial layer support, as given in Table 1. It worth noting that the peak data rate in Table 1 is set by the largest TBS for NPRB = 100 according to Rel-8 LTE design principles. The overhead assumption is aligned with the agreements at RAN1 #76bis meeting so that smaller overhead is assumed for the last row to achieve a higher peak data rate with 256QAM.

With the new TBS table and the new MCS table proposed in [1], actual code rate and code rate in accuracy in percentage for 256QAM is shown in Figure 4. It can be seen that the deviation is within ±2% aligned with Rel-8 design.

Observation:

- By allowing up to ±2% deviation from the target code rate, the new TBS table can be designed without introducing any new transport block sizes for 256QAM one spatial layer support in Rel-12.

Table 1 Six new rows in TBS table for 256QAM support

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27 | 648 | 1320 | 1992 | 2664 | 3368 | 4008 | 4584 | 5352 | 5992 | 6712 |
| 28 | 680 | 1384 | 2088 | 2792 | 3496 | 4264 | 4968 | 5544 | 6200 | 6968 |
| 29 | 744 | 1544 | 2280 | 3112 | 3880 | 4584 | 5352 | 6200 | 6968 | 7736 |
| 30 | 776 | 1608 | 2408 | 3240 | 4008 | 4776 | 5736 | 6456 | 7224 | 7992 |
| 31 | 840 | 1672 | 2536 | 3368 | 4264 | 5160 | 5992 | 6712 | 7480 | 8504 |
| 32 | 968 | 1928 | 2984 | 3880 | 4968 | 5992 | 6968 | 7736 | 8760 | 9912 |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 27 | 7224 | 7992 | 8504 | 9144 | 9912 | 10680 | 11448 | 11832 | 12576 | 12960 |
| 28 | 7736 | 8504 | 9144 | 9912 | 10680 | 11064 | 11832 | 12576 | 13536 | 14112 |
| 29 | 8504 | 9144 | 9912 | 10680 | 11448 | 12216 | 12960 | 14112 | 14688 | 15264 |
| 30 | 8760 | 9528 | 10680 | 11448 | 12216 | 12960 | 13536 | 14688 | 15264 | 16416 |
| 31 | 9144 | 10296 | 11064 | 11832 | 12576 | 13536 | 14112 | 15264 | 15840 | 16992 |
| 32 | 10680 | 11832 | 12576 | 13536 | 14688 | 15840 | 16416 | 17568 | 18336 | 19848 |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 27 | 14112 | 14688 | 15264 | 15840 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 |
| 28 | 14688 | 15264 | 15840 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 21384 |
| 29 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 21384 | 22152 | 22920 |
| 30 | 16992 | 17568 | 18336 | 19080 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 |
| 31 | 17568 | 18336 | 19848 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 25456 |
| 32 | 20616 | 21384 | 22920 | 23688 | 24496 | 25456 | 26416 | 27376 | 28336 | 29296 |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 27 | 20616 | 21384 | 22152 | 22920 | 22920 | 23688 | 24496 | 25456 | 25456 | 26416 |
| 28 | 21384 | 22152 | 22920 | 23688 | 24496 | 25456 | 25456 | 26416 | 27376 | 28336 |
| 29 | 23688 | 24496 | 25456 | 26416 | 27376 | 28336 | 28336 | 29296 | 30576 | 30576 |
| 30 | 25456 | 25456 | 26416 | 27376 | 28336 | 29296 | 30576 | 30576 | 31704 | 32856 |
| 31 | 26416 | 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 31704 | 32856 | 34008 |
| 32 | 30576 | 31704 | 32856 | 32856 | 34008 | 35160 | 36696 | 37888 | 37888 | 39232 |
|  | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 27 | 27376 | 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 31704 | 32856 | 32856 |
| 28 | 28336 | 29296 | 30576 | 30576 | 31704 | 31704 | 32856 | 34008 | 34008 | 35160 |
| 29 | 31704 | 32856 | 32856 | 34008 | 35160 | 35160 | 36696 | 36696 | 37888 | 39232 |
| 30 | 32856 | 34008 | 35160 | 35160 | 36696 | 37888 | 37888 | 39232 | 39232 | 40576 |
| 31 | 35160 | 35160 | 36696 | 37888 | 37888 | 39232 | 39232 | 40576 | 42368 | 42368 |
| 32 | 40576 | 40576 | 42368 | 43816 | 43816 | 45352 | 46888 | 46888 | 48936 | 48936 |
|  | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 |
| 27 | 34008 | 34008 | 35160 | 35160 | 36696 | 36696 | 37888 | 37888 | 39232 | 39232 |
| 28 | 35160 | 36696 | 36696 | 37888 | 37888 | 39232 | 40576 | 40576 | 40576 | 42368 |
| 29 | 39232 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 45352 | 45352 | 46888 |
| 30 | 40576 | 42368 | 42368 | 43816 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 |
| 31 | 43816 | 43816 | 45352 | 45352 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 |
| 32 | 51024 | 51024 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 |
|  | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 |
| 27 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 | 46888 |
| 28 | 42368 | 43816 | 43816 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 |
| 29 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 52752 | 52752 | 52752 | 55056 |
| 30 | 48936 | 51024 | 51024 | 52752 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 |
| 31 | 51024 | 52752 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 |
| 32 | 59256 | 61664 | 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 68808 | 68808 |
|  | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |
| 27 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 |
| 28 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 | 55056 | 55056 | 55056 | 55056 |
| 29 | 55056 | 55056 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 |
| 30 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 |
| 31 | 59256 | 61664 | 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 |
| 32 | 68808 | 71112 | 71112 | 73712 | 73712 | 75376 | 75376 | 76208 | 78704 | 78704 |
|  | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 |
| 27 | 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 |
| 28 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 | 61664 | 63776 |
| 29 | 63776 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 |
| 30 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 |
| 31 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 | 73712 | 75376 | 75376 | 76208 |
| 32 | 78704 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 | 87936 | 87936 | 87936 |
|  | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 |
| 27 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 | 63776 | 66592 | 66592 |
| 28 | 63776 | 63776 | 66592 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 |
| 29 | 71112 | 71112 | 71112 | 73712 | 73712 | 73712 | 75376 | 76208 | 76208 | 78704 |
| 30 | 73712 | 75376 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 |
| 31 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 |
| 32 | 90816 | 90816 | 90816 | 93800 | 93800 | 93800 | 93800 | 97896 | 97896 | 97896 |
|  | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 |
| 27 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 71112 | 73712 |
| 28 | 71112 | 71112 | 71112 | 73712 | 73712 | 73712 | 75376 | 75376 | 76208 | 76208 |
| 29 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 |
| 30 | 81176 | 81176 | 84760 | 84760 | 84760 | 84760 | 87936 | 87936 | 87936 | 87936 |
| 31 | 84760 | 87936 | 87936 | 87936 | 87936 | 90816 | 90816 | 90816 | 93800 | 93800 |
| 32 | 97896 | 97896 | 97896 | 97896 | 97896 | 97896 | 97896 | 97896 | 97896 | 97896 |

Figure 4 Actual code rate and code rate inaccuracy in percentage for Rel-12 256QAM assuming 120 RE/PRB.

To support 256QAM transport blocks mapped to multiple spatial multiplexing layers, 3 new tables need to be designed for one-layer to two-layer, one-layer to three-layer and one-layer to four-layer translation. The proposed TBS translation tables are given in Table 2, Table 3 and Table 4, respectively. Eight new mappings are needed for TBS = 76208, 78704, 81176, 84760, 87936, 90816, 93800 and 97896 for each of the one-to-two, one-to-three and one-to-four TBS translation tables.

Allowing mapping inaccuracy of ±0.4%, the additional one-layer to two-layer mappings can be supported without introducing new TBS, as given in Table 2.

Table 2 TBS mapping table for one-layer to two-layer

| TBS_L1 | TBS_L2 |
| --- | --- |
| 76208 | 152976 |
| 78704 | 157432 |
| 81176 | 161760 |
| 84760 | 169544 |
| 87936 | 175600 |
| 90816 | 181656 |
| 93800 | 187712 |
| 97896 | 195816 |

Allowing mapping inaccuracy of ±1.25%, the additional one-layer to three-layer mappings can be supported without introducing new TBS, as given in Table 3.

Table 3 TBS mapping table for one-layer to three-layer

| TBS_L1 | TBS_L3 |
| --- | --- |
| 76208 | 230104 |
| 78704 | 236160 |
| 81176 | 245648 |
| 84760 | 254328 |
| 87936 | 266440 |
| 90816 | 275376 |
| 93800 | 284608 |
| 97896 | 293736 |

The additional one-layer to four-layer mappings can be supported with eight new TBS following Rel-8 principles of QPP size alignment, as given in Table 4.

Table 4 TBS mapping table for one-layer to four-layer

| TBS_L1 | TBS_L4 |
| --- | --- |
| 76208 | 305976 |
| 78704 | 314888 |
| 81176 | 327000 |
| 84760 | 339112 |
| 87936 | 354936 |
| 90816 | 363336 |
| 93800 | 375448 |
| 97896 | 391656 |

With the TBS table design given in Table 1, Table 2, Table 3 and Table 4, there are 8 new TBS added to the existing 229 TBS, i.e., 237 TBS in total to support all modulations up to 256QAM. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Observation:

- The proposed TBS table design need 8 new TBS added to the existing 229 TBS. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Based on above discussion, the new TBS table has a minimum specification impacts, with a good support for 256QAM. In the new TBS table, all entries of Rel-11 TBS are kept and a few TBS rows are added containing TBS to support 256QAM. All the TBS in the new rows are the existing TBS from Rel-11 TBS table. As for the multi-layer TBS translation tables, similarly, the existing TBS are used for one-layer to two-layer translation, and one-layer to three-layer translation table without introducing new TBS. As for one-layer to four-layer TBS translation table, only eight new TBS are introduced to support higher peak rates with 256QAM. With this design, the new TBS table can be stored with 8-bit indices, which reduce the storage requirement and have minimum impact on implementation/cost. Therefore, we propose to introduce the proposed TBS table for 256QAM support in Rel-12.

Proposal:

- Introduce the proposed TBS table for 256QAM support in Rel-12.

  - 6 new TBS rows are appended

  - 8 new TBS are introduced

# Conclusions

In this contribution, we propose a TBS table for 256QAM support in Rel-12, and made the following observations and proposals:

Observation:

- By allowing the actual code rate to deviate from the target code rate by ±2%, there are in total 229 unique TBS defined in Rel-11.

- By allowing up to ±2% deviation from the target code rate, the new TBS table can be designed without introducing any new transport block sizes for 256QAM one spatial layer support in Rel-12.

- The proposed TBS table design need 8 new TBS added to the existing 229 TBS. This allows the TBS table to be stored with 8-bit indices instead of 19-bit integers, which reduces storage requirement by at least half.

Proposal:

- Introduce the proposed TBS table for 256QAM support in Rel-12.

  - 6 new TBS rows are appended

  - 8 new TBS are introduced

# References

R1-142375, “CQI and MCS table design for downlink 256QAM”, Ericsson

3GPP TS 36.213