3GPP TSG RAN WG1 Meeting #78,	R1-143262

Dresden, Germany, 18th – 22nd August 2014

Source:	Kyocera

Title:	        Physical resource design for D2D discovery

Agenda Item:	7.2.3.1.2

Document for:	Discussion/Decision

Introduction

During RAN1#77, the D2D discovery resource size was discussed [1]. In RAN1 the message size is 104bits was agreed as a working assumption. However SA2 proposed 200 bits [2] and the discovery resource should be assumed around 200 bits. In this contribution, we consider the number of PRB-pairs needed for D2D discovery.

Agreements: - If number of information bits transmitted on a discovery resource is not greater than 256 bits a discovery resource consists of 2 or 3 contiguous PRB in frequency and 1 sub-frame without frequency hopping across slots - FFS: The number of contiguous PRB in frequency, and down selected between “2” or “3”

Number of RBs needed for D2D discovery

In this paper, we investigate the number of RBs needed for the D2D discovery signal using link level evaluation. The simulation assumptions are listed in Appendix A. We assume the performance baseline is 2PRB-pair of 104bits. The simulation results are shown below and we conclude 3 PRBs are needed for the D2D discovery signal..

Table 1: Performance degradation at BLER = 0.01

| Number of RBS | Performance degradation from 104bits (2RBs) |
| --- | --- |
| 2 | 2.0 dB |
| 3 | 0.0 dB |

Figure 1: BLER performance

Proposal:

If the message size is 200bits, the number of PRBs needed for D2D discovery should be 3 PRB-pairs.

Conclusions

Proposal:

If the message size is 200bits, the number of PRBs needed for D2D discovery should be 3 PRB-pairs.

References

[1] Chairman's Notes RAN1#77

[2] R1-140185(S2-140568), LS reply on discovery message size, SA2

- Appendix A: Simulation Assumptions

The physical format is shown in Figure 1. Simulation assumptions are listed in the Table 2.

Data Symbol

Gap

DMRS

Figure 1: D2D discovery physical format

Table 2: Simulation assumptions

| Carrier frequency | 2000 [MHz] |
| --- | --- |
| System Bandwidth | 10 [MHz] |
| Channel model | ITU-R Umi NLOS w/ dual-mobility |
| Moving direction of Tx UE and Rx UE | Uniform Random(-180-180[deg.]), independently |
| Number of Tx Antennas | 1 |
| Number of Rx Antennas | 2 |
| Number of RB | 2,3,4 |
| Modulation | QPSK |
| Bit length | 104, 200 bits |
| Channel coding | Turbo coding |
| CRC | 24 bits |
| UE velocities pairs (Tx, Rx) | (3,3) [km/h] |
| Channel Estimation | ZF with linear interpolation in time |
| Receiver Type | MRC |