3GPP TSG-RAN WG1 Meeting #78 	R1-143487

# Dresden, Germany, August 18 – 22, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.211 | CR | 0315 | rev | 2 | Current version: | 11.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia Networks, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_SIBenh |  |  |  |  |  |  | Date: |  | 2014-08-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mapping of BCH to S-CCPCH is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.1, 5.3.1, 5.3.3.4, 5.3.3.4.1, 6.1, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | 25.212, 25.213 25.300, 25.301, 25.302, 25.304, 25.306, 25.321, 25.331 25.433 25.101, 25.133 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[---------------------------------------------------------- Text Start --------------------------------------------------------------------]

--- Unchanged text omitted ---

#### 4.1.2.1	BCH - Broadcast Channel

The Broadcast Channel (BCH) is a downlink transport channel that is used to broadcast system- and cell-specific information. The BCH is always transmitted over the entire cell and has a single non-zero transport format. In cells configured with broadcast distribution, a first BCH mapped to P-CCPCH is always present and one additional BCH mapped to S-CCPCH can be configured.

--- Unchanged text omitted ---

### 5.3.1	Downlink transmit diversity

Table 10 summarises the possible application of open and closed loop transmit diversity modes on different downlink physical channel types. Simultaneous use of STTD and closed loop modes on the same physical channel is not allowed. In addition, if Tx diversity is applied on any of the downlink physical channels allocated to a UE(s) that is configured to use P-CPICH as a phase reference on both antennas, then Tx diversity shall also be applied on P-CCPCH and SCH. If Tx diversity is applied on SCH it shall also be applied on P-CCPCH and vice versa. Regarding CPICH transmission in case of transmit diversity used on SCH and P-CCPCH, see subclause 5.3.3.1. An S-CCPCH carrying BCH shall be transmitted with the same diversity mode as P-CCPCH.

With respect to the usage of Tx diversity for DPCH or F-DPCH on different radio links within an active set, the following rules apply:

-	Different Tx diversity modes (STTD and closed loop) shall not be used on the radio links within one active set.

-	No Tx diversity on one or more radio links shall not prevent UTRAN to use Tx diversity on other radio links within the same active set.

- 	If STTD is activated on one or several radio links in the active set, the UE shall operate STTD on only those radio links where STTD has been activated. Higher layers inform the UE about the usage of STTD on the individual radio links in the active set.

- 	Regarding the usage of Tx diversity for DPCH on different radio links within an active set, if closed loop TX diversity is activated on one or several radio links in the active set, the UE shall operate closed loop TX diversity on only those radio links where closed loop TX diversity has been activated. Higher layers inform the UE about the usage of closed loop TX diversity on the individual radio links in the active set.

--- Unchanged text omitted ---

#### 5.3.3.4	Secondary Common Control Physical Channel (S-CCPCH)

The Secondary CCPCH is used to carry the FACH and PCH, and can also be configured to carry a BCH. There are two types of Secondary CCPCH: those that include TFCI and those that do not include TFCI. It is the UTRAN that determines if a TFCI should be transmitted, hence making it mandatory for all UEs to support the use of TFCI. The set of possible rates for the Secondary CCPCH is the same as for the downlink DPCH, see subclause 5.3.2. The frame structure of the Secondary CCPCH is shown in figure 17.

Figure 17: Frame structure for Secondary Common Control Physical Channel

The parameter k in figure 17 determines the total number of bits per downlink Secondary CCPCH slot. It is related to the spreading factor SF of the physical channel as SF = 256/2k. The spreading factor range is from 256 down to 4.

The values for the number of bits per field are given in Table 18. The channel bit and symbol rates given in Table 18 are the rates immediately before spreading. The slot formats applicable to QPSK with pilot bits are not supported in this release. The pilot patterns for the slot formats applicable to QPSK are given in Table 19. DTX shall be used in the pilot field of the 16QAM slot formats, i.e. no pilot bits are used in this release. A BCH mapped to Secondary CCPCH uses the specific fixed Secondary CCPCH slot format in Table 18, and this slot format can only be used for carrying BCH.

The FACH and PCH can be mapped to the same or to separate Secondary CCPCHs. If FACH and PCH are mapped to the same Secondary CCPCH, they can be mapped to the same frame. A BCH mapped to Secondary CCPCH can only be mapped to a separate Secondary CCPCH without multiplexing with other transport channels. The main difference between a CCPCH and a downlink dedicated physical channel is that a CCPCH is not inner-loop power controlled. The main difference between the Primary and Secondary CCPCH is that the transport channel mapped to the Primary CCPCH (BCH) can only have a fixed predefined transport format combination, while the Secondary CCPCH supports multiple transport format combinations on the transport channels mapped to it and offers the possibility of using TFCI.

Table 18: Secondary CCPCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Ndata1 | Npilot | NTFCI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 30 | 15 | 256 | 300 | 20 | 20 | 0 | 0 |
| 1 | 30 | 15 | 256 | 300 | 20 | 12 | 8 | 0 |
| 2 | 30 | 15 | 256 | 300 | 20 | 18 | 0 | 2 |
| 3 | 30 | 15 | 256 | 300 | 20 | 10 | 8 | 2 |
| 4 | 60 | 30 | 128 | 600 | 40 | 40 | 0 | 0 |
| 5 | 60 | 30 | 128 | 600 | 40 | 32 | 8 | 0 |
| 6 | 60 | 30 | 128 | 600 | 40 | 38 | 0 | 2 |
| 7 | 60 | 30 | 128 | 600 | 40 | 30 | 8 | 2 |
| 8 | 120 | 60 | 64 | 1200 | 80 | 72 | 0 | 8* |
| 9 | 120 | 60 | 64 | 1200 | 80 | 64 | 8 | 8* |
| 10 | 240 | 120 | 32 | 2400 | 160 | 152 | 0 | 8* |
| 11 | 240 | 120 | 32 | 2400 | 160 | 144 | 8 | 8* |
| 12 | 480 | 240 | 16 | 4800 | 320 | 312 | 0 | 8* |
| 13 | 480 | 240 | 16 | 4800 | 320 | 296 | 16 | 8* |
| 14 | 960 | 480 | 8 | 9600 | 640 | 632 | 0 | 8* |
| 15 | 960 | 480 | 8 | 9600 | 640 | 616 | 16 | 8* |
| 16 | 1920 | 960 | 4 | 19200 | 1280 | 1272 | 0 | 8* |
| 17 | 1920 | 960 | 4 | 19200 | 1280 | 1256 | 16 | 8* |
| 18*** | 60 | 15 | 256 | 600 | 40 | 36 | 0 | 4 |
| 19*** | 120 | 30 | 128 | 1200 | 80 | 76 | 0 | 4 |
| 20*** | 240 | 60 | 64 | 2400 | 160 | 144 | 0 | 16* |
| 21*** | 480 | 120 | 32 | 4800 | 320 | 272 | 32** | 16* |
| 22*** | 960 | 240 | 16 | 9600 | 640 | 560 | 64** | 16* |
| 23*** | 1920 | 480 | 8 | 19200 | 1280 | 1136 | 128** | 16* |
| BCH**** | 30 | 15 | 256 | 300 | 20 | 18 | 0 | 0 |

* If TFCI bits are not used, then DTX shall be used in TFCI field.

** If pilot bits are not used, then DTX shall be used in pilot field.

*** Slot formats applicable to 16QAM. See subclause 4.3.5.1.1 in [3] for mapping TFCI bits on 16QAM slot formats.

**** Slot format used for S-CCPCH carrying BCH. The first two data bits (256 chips) out of the 20 data bits in each slot are DTXed, resulting in a frame structure identical to P-CCPCH.

NOTE 1: The slot formats 18 to 23 in Table 18 are only applicable for MBSFN operations with 16QAM.

NOTE 2: The modulation used in MBSFN operations is signalled by higher layers.

The pilot symbol pattern described in Table 19 is not supported in this release. The shadowed part can be used as frame synchronization words. (The symbol pattern of pilot symbols other than the frame synchronization word shall be "11"). In Table 19, the transmission order is from left to right. (Each two-bit pair represents an I/Q pair of QPSK modulation.)

Table 19: Pilot Symbol Pattern

|  | Npilot = 8 |  |  |  | Npilot = 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 11 10 01 11 01 10 10 00 00 11 00 01 00 10 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 00 00 10 11 01 11 00 11 11 10 10 01 00 01 |

For slot formats using TFCI, the TFCI value in each radio frame corresponds to a certain transport format combination of the FACHs and/or PCHs currently in use. This correspondence is (re-)negotiated at each FACH/PCH addition / removal. The mapping of the TFCI bits onto slots is described in [3].

##### 5.3.3.4.1	Secondary CCPCH structure with STTD encoding

In case the diversity antenna is present in UTRAN and the S-CCPCH does not carry BCH and is to be transmitted using open loop transmit diversity, the data and TFCI bits of the S-CCPCH are STTD encoded as given in subclause 5.3.1.1.1. The pilot symbol pattern for antenna 2 for the S-CCPCH given in Table 20 is not supported in this release. If S-CCPCH carries BCH and open loop transmit diversity is to be applied, the STTD encoding is done the same way as for P-CCPCH as described in subclause 5.3.3.3.1.

Table 20: Pilot symbol pattern for antenna 2 when STTD encoding is used on the S-CCPCH

|  | Npilot = 8 |  |  |  | Npilot = 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 10 10 00 01 11 01 10 01 01 00 00 11 10 11 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 10 11 00 10 00 11 11 01 01 10 01 00 01 11 |

--- Unchanged text omitted ---

## 6.1	Mapping of transport channels onto physical channels

Figure 27 summarises the mapping of transport channels onto physical channels.

Dedicated Physical Data Channel (DPDCH)

Dedicated Physical Control Channel (DPCCH)

Secondary

Dedicated Physical Control Channel (

S

-

DPCCH)

Fractional Dedicated Physical Channel (F

-

DPCH)

Fractional

Transmitted

Precoding

Indicator

Channel (

F

-

TPI

CH)

E

-

DCH Dedicated Physical Data Channel (E

-

DPDCH)

E

-

DCH Dedicated Physical Control Channel (E

-

DPCCH)

E

-

DCH Absolute Grant C

hannel (E

-

AGCH)

E

-

DCH Relative Grant Channel (E

-

RGCH)

E

-

DCH Hybrid ARQ Indicator Channel (E

-

HICH)

Physical Random Access Channel (PRACH)

Common Pilot Channel (CPICH)

Primary Common Control Physical Channel (P

-

CCPCH)

Secondary Common Control Physica

l Channel (S

-

CCPCH)

Synchronisation Channel (SCH)

Acquisition Indicator Channel (AICH)

Paging Indicator Channel (PICH)

MBMS Notification Indicator Channel (MICH)

High Speed Physical Downlink Shared Channel (HS

-

PDSCH)

HS

-

DSCH

-

related Shared Control

Channel (HS

-

SCCH)

Dedicated Physical Control Channel (uplink) for HS

-

DSCH (HS

-

DPCCH)

Transport Channels

DCH

RACH

BCH

FACH

PCH

Physical Channels

HS

-

DSCH

E

-

DCH

E-DCH Rank and Offset Channel (E-ROCH)

Secondary E-DCH Dedicated Physical Data Channel (S-E-DPDCH)

Secondary E-DCH Dedicated Physical Control Channel (S-E-DPCCH)

Dedicated Physical Data Channel (DPDCH)

Dedicated Physical Control Channel (DPCCH)

Secondary

Dedicated Physical Control Channel (

S

-

DPCCH)

Fractional Dedicated Physical Channel (F

-

DPCH)

Fractional

Transmitted

Precoding

Indicator

Channel (

F

-

TPI

CH)

E

-

DCH Dedicated Physical Data Channel (E

-

DPDCH)

E

-

DCH Dedicated Physical Control Channel (E

-

DPCCH)

E

-

DCH Absolute Grant C

hannel (E

-

AGCH)

E

-

DCH Relative Grant Channel (E

-

RGCH)

E

-

DCH Hybrid ARQ Indicator Channel (E

-

HICH)

Physical Random Access Channel (PRACH)

Common Pilot Channel (CPICH)

Primary Common Control Physical Channel (P

-

CCPCH)

Secondary Common Control Physica

l Channel (S

-

CCPCH)

Synchronisation Channel (SCH)

Acquisition Indicator Channel (AICH)

Paging Indicator Channel (PICH)

MBMS Notification Indicator Channel (MICH)

High Speed Physical Downlink Shared Channel (HS

-

PDSCH)

HS

-

DSCH

-

related Shared Control

Channel (HS

-

SCCH)

Dedicated Physical Control Channel (uplink) for HS

-

DSCH (HS

-

DPCCH)

Transport Channels

DCH

RACH

BCH

FACH

PCH

Physical Channels

HS

-

DSCH

E

-

DCH

E-DCH Rank and Offset Channel (E-ROCH)

Secondary E-DCH Dedicated Physical Data Channel (S-E-DPDCH)

Secondary E-DCH Dedicated Physical Control Channel (S-E-DPCCH)

Figure 27: Transport-channel to physical-channel mapping

The DCHs are coded and multiplexed as described in [3], and the resulting data stream is mapped sequentially (first-in-first-mapped) directly to the physical channel(s). The mapping of BCH and FACH/PCH is equally straightforward, where the data stream after coding and interleaving is mapped sequentially to the CCPCHs. A BCH can be mapped to either Primary CCPCH or Secondary CCPCH, while FACH/PCH is always mapped to Secondary CCPCH. Also for the RACH, the coded and interleaved bits are sequentially mapped to the physical channel, in this case the message part of the PRACH. The E-DCH is coded as described in [3], and the resulting data stream is mapped sequentially (first-in-first-mapped) directly to the physical channel(s).

--- Unchanged text omitted ---

# 7	Timing relationship between physical channels

## 7.1	General

The P-CCPCH, on which the cell SFN is transmitted, is used as timing reference for all the physical channels, directly for downlink and indirectly for uplink.

Figure 29 describes the frame timing of some of the downlink physical channels; the timing of the remaining downlink physical channels and of the uplink physical channels is specified in the remaining subclauses. For the AICH the access slot timing is included. Transmission timing for uplink physical channels is given by the received timing of downlink physical channels.

k:th S

-

CCPCH

AICH access

slots

Secondary

SCH

Primary

SCH



S

-

CCPCH,k

10 ms



PICH

#0

#1

#2

#3

#14

#13

#12

#11

#10

#9

#8

#7

#6

#5

#4

Radio frame

with

(SFN modulo 2) = 0

Radio frame

with

(SFN modulo 2) = 1



DPCH,n

P

-

CCPCH

Any CPICH

PICH for k:th

S

-

CCPCH

n:th DPC

H

10 ms

Subframe

#

0

HS

-

SCCH

Subframes

Subframe

#

1

Subframe

#

2

Subframe

#

3

Subframe

#

4



F

-

DPCH,p

p:th F

-

DPCH



F

-

TPI

CH,

m

m

:th F

-

TPI

CH

Figure 29: Radio frame timing and access slot timing of downlink physical channels

The following applies:

-	SCH (primary and secondary), CPICH (primary and secondary and demodulation CPICHs) and P-CCPCH have identical frame timings.

-	The S-CCPCH timing may be different for different S-CCPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. S-CCPCH,k = Tk  256 chip, Tk  {0, 1, …, 149}. For S-CCPCH carrying BCH Tk = 0. For MBSFN operations using slot formats 21 to 23 in table 18, the offset shall be set in accordance with S-CCPCH,k = 256 +  Tk /10.  2560 chip.

-	If the PICH is associated to the S-CCPCH, the PICH timing is PICH = 7680 chips prior to its corresponding S-CCPCH frame timing, i.e. the timing of the S-CCPCH carrying the PCH transport channel with the corresponding paging information, see also subclause 7.2. If the PICH is associated to the HS-SCCH, the PICH frame timing is the same as the HS-SCCH frame timing.

-	AICH access slots #0 starts the same time as P-CCPCH frames with (SFN modulo 2) = 0. The AICH/PRACH timing is described in subclauses 7.3 and 7.4 respectively.

-	The DPCH timing may be different for different DPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. DPCH,n = Tn  256 chip, Tn  {0, 1, …, 149}. The DPCH (DPCCH/DPDCH) timing relation with uplink DPCCH/DPDCHs is described in subclause 7.6.

-	The F-DPCH timing may be different for different F-DPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. F-DPCH,p = Tp  256 chip, Tp  {0, 1, …, 149}. All F-DPCHs transmitted to a UE from the same HS-DSCH cell set have the same timing. The F-DPCH timing relation with uplink DPCCH/DPDCHs is described in subclause 7.6.

-	The start of HS-SCCH subframe #0 is aligned with the start of the P-CCPCH frames. The relative timing between a HS-PDSCH and the corresponding HS-SCCH is described in subclause 7.8.

-	The E-HICH, E-RGCH, E-AGCH and E-ROCH downlink timing are respectively specified in subclause 7.10, 7.11, 7.12 and 7.12A. The E-DPCCH and E-DPDCH uplink timing are specified in subclause 7.13.

-	For a secondary serving HS-DSCH cell, the nominal radio frame timing for CPICH and timing reference are the same as the radio frame timing for CPICH and timing reference for the serving HS-DSCH cell.

-	The F-TPICH timing may be different for different F-TPICHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. F-TPICH,m = Tm  256 chip, Tm  {0, 1, …, 149}.

--- Unchanged text omitted ---

[---------------------------------------------------------- Text End ---------------------------------------------------------------------]