3GPP TSG RAN WG1 #79	R1-144603

November 17th- 21st, 2014

San Francisco, USA

Agenda item:	6.3.2.2

Source: 	Cisco Systems

Title: 	Initial Coexistence Evaluations Comparing LAA Versus Wi-Fi

Document for:	Discussion and Decision

# Introduction

In RAN1 #78bis, preliminary agreements were made regarding the methodology for coexistence studies. The following working assumptions were reached:

- Following scenarios are used for evaluation

  - Three coexistence scenarios should be evaluated (See Figures in R1-144375)

    - Coexistence scenario a:  Operator #1 deploys Wi-Fi and operator #2 deploys Wi-Fi

    - Coexistence scenario b:  Operator #1 deploys LAA and operator #2 deploys LAA

    - Coexistence scenario c:  Operator #1 deploys Wi-Fi and operator #2 deploys LAA

  - Both outdoor and indoor deployments should be considered in these scenarios

  - Coexistence scenarios with single and multiple unlicensed channels should be evaluated

    - Note: this may not need two separate simulation scenarios

  - Async between different LAA operators are baseline

    - Sync between different LAA operators can also be evaluated

Additionally, in terms of how to perform the studies for the Wi-Fi and LAA coexistence and for LAA-LAA coexistence, the following agreements were reached:

- Wi-Fi-LAA coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- LAA-LAA coexistence

  - Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two LAA operators are compared.

Cisco Systems presented their views regarding the FFS parameters both during email discussion following #78bis and in a companion contribution [3]. In this contribution, we present initial coexistence results in an indoor scenario with two operators, comparing the UL/DL performance of Operator 1’s Wi-Fi network when Operator 2’s network is Wi-Fi versus when Operators 2’s network is LAA-LTE.

This contribution answers the question:  Modeling typical device capabilities of Wi-Fi and LTE, what is the impact of Operator 2’s LAA transmissions (without LBT) on Operator 1’s Wi-Fi performance, as compared to the scenario wherein both Operators 1 and 2 deploy Wi-Fi? Detailed system simulation results demonstrate that without Listen-before-Talk (LBT), the performance of operator 1’s Wi-Fi BSS is considerably deteriorated when operator 2 deploys LAA without LBT. In contrast when both operators deploy Wi-Fi, because Wi-Fi has been built for coexistence in unlicensed spectrum from the ground up, we observe Operator 1’s performance is relatively unaffected (until a certain offered load). This suggests an LBT mechanism is required to ensure fair coexistence of LTE with other RATs in unlicensed spectrum.

While it has been identified that LBT is a targeted functionality in the design of LAA, the agreements from #78bis read that not all targeted functionalities may have a specification impact and further not all functionalities would be mandatory for all LAA eNBs/UEs. Based on the results in this contribution, it is essential that specification support for LBT be introduced in Rel-13; additionally it is desirable to have mandatory LBT capability in LTE eNB if they transmit in unlicensed spectrum.

# Simulation Setup

An indoor enterprise two-operator scenario comprising two operators is considered wherein Operator 1 deploys 4 Wi-Fi APs and the Operator 2 deploys either 4 Wi-Fi APs or 4 LAA-LTE eNBs in unlicensed spectrum. No coordination or synchronization is assumed between the transmissions from either operator.  All STAs and LAA devices are placed on the same frequency channel; this models dense scenarios where the number of devices far exceeds the number of available frequency channels, as observed in dense Manhattan-like scenarios.

A single floor setup is modeled with 4 APs/operator. Figure 1 depicts a realization of the eNB/APs within a 1200 sq. meter (55.73 m x 21.55 m) single floor scenario. The AP density for each operator is modeled as 3000 sq. ft. per AP. The modeled AP density is based on multiple considerations including Cisco enterprise Wi-Fi AP deployment guidelines [5], measurements of AP densities [6] and simulation studies in the IEEE .11ax WG [7].

Operator 1 represents a victim Wi-Fi network carrying both DL and UL traffic. Operator 2 represents the aggressor network, which can be either Wi-Fi or LAA-LTE. Operator 2 carries only DL traffic. A per UE traffic model FTP3 is used which uses a Poisson process to generate 0.5MB files with a mean interarrival time of 5seconds. This generates a mean offered load of 0.5MB x 8bits per Byte x 1/5seconds = 0.8 Mbps per UE. The same model is used for downlink and uplink. The traffic load is varied by changing the number of UEs associated to each AP viz. the number of UEs are varied between 4UEs per AP and 30 UEs per AP to represent different conditions from light to heavy loading.

UEs/STAs are uniformly randomly scattered throughout the floor. The pathloss model between small cells and Wi-Fi APs, between small cells (resp. APs) and UEs and between UEs is based on the ITU InH model. Cell/AP Association is based on strongest RSRP for LTE and strongest received signal strength in case of Wi-Fi.

Wi-Fi nodes are modeled as following the DCF mechanism with contention-based access using CSMA-CA. The Clear Channel Assessment (CCA) procedure is modeled as a combination of energy detection (CCA-EDThresh = -62 dBm) and preamble decoding following carrier sensing threshold detection (CCA-CSThresh = - 82 dBm). Virtual carrier sense (via NAV) is modeled explicitly.

Because LAA does not defer upon detecting energy from a Wi-Fi transmission, quite a large fraction of Wi-Fi receiver STAs can experience interference due to LAA transmissions, especially at high offered loads at the LAA eNB. In contrast, when both operators deploy Wi-Fi, devices belonging to either operator defer their transmissions upon either energy detection or preamble and PLCP decoding.

Observation 1:  Large fraction of Wi-Fi receiver STAs can experience SINR < 0 dB (analogous to the CSG scenario) from LAA-LTE transmissions since the LAA eNB does not perform CCA prior to transmission.

Observation 2: The Wi-Fi CCA procedure (preamble decoding and energy detection) ensures that when both operators deploy Wi-Fi, Wi-Fi STAs defer their access to the medium upon detecting energy from other STAs irrespective of which operator they belong to. In low-to-medium load scenarios where the number of contention back-offs is not large, this enforces a TDM-like functionality between Wi-Fi transmissions

Table 1 shows a high level summary of the simulation parameters. A more detailed rundown of simulation parameters is presented in the Annexes 6 – 11.

Table 1: High Level Summary of Simulation Parameters

| Parameter | Value |
| --- | --- |
| Coexistence Scenarios | A: Operator #1 deploys Wi-Fi and Operator #2 deploys Wi-Fi C: Operator #1 deploys Wi-Fi and Operator #2 deploys LAA Each operator deploys 4 Wi-Fi Aps/4 LAA eNBs. |
| Deployment Scenario | Indoor high density enterprise deployment (outdoor is not modeled) |
| UEs per operator | Each AP can serve {2, 5, 10, 15, 20, 30} UEs. |
| Bandwidth | All devices transmit/receive over a single 20 MHz channel. No modeling of transmission/reception in licensed carrier. |
| Synchronization | No synchronization (between operators) is modeled |
| Performance Metric | Aggregated throughput and UPT CDF at Operator 1. |
| Traffic Modeling | FTP Model 3 is used for all UE and devices, DL and UL where applicable. File sizes are assumed to be 0.5 MB each with arrival rate equaling 0.2 Hz per UE. The load is varied with number of UEs. |
| Node Density | X = 4 AP/operator deployed on Y = 1 channel. |
| Co-existence Methodology | Operator 1’s performance is obtained in two scenarios: one wherein Operator 2 deploys Wi-Fi and the other wherein Operator 2 deploys LAA. Performance metrics are reported only for Operator 1’s network. |

Figure 1: Indoor Enterprise Deployment Scenario

# LAA-LTE Modelling

LAA-LTE eNodeB transmissions are modeled by calculating the time needed to transfer the downlink traffic demanded by the LTE UEs, to be used in the Wi-Fi co-existence module. Each LTE-LAA small cell performs fair scheduling of resources between multiple UEs, operating in the interference conditions described by an input SINR distribution.

Feedback of UCI (HARQ-ACK) over the licensed band cell is assumed idealized and hence not explicitly modeled. Link adaptation is modeled assuming ideal channel knowledge. For long duration file transfers, the throughput of the link is abstracted by link level curves representing the mean payload per RB that can be achieved in the given mean SINR and fast fading conditions. Link level curves are drawn from TM4 operation under pedestrian B, 3km/h-fading conditions.

LBT (Listen Before Talk) is not modeled for LTE-LAA, and thus it continues with its transmissions regardless of whether Wi-Fi has already occupied the channel.  In Figure 2, we plot the time occupancy of LAA-LTE as a function of the offered load (# UEs/AP assuming a constant FTP3 arrival rate per user). At higher loads, LTE-LAA eNodeBs take longer to transfer each demanded traffic object, due to lower SINRs and increased sharing of the resource. At about 20 UE/AP, it is seen that the fraction of time occupancy is close to 100% implying that LAA-LTE eNodeBs are transmitting continuously.

Our analysis of the coupling gains matrix between LAA transmitters and Wi-Fi clients (ref. Figure 3) finds that in high traffic scenarios (when all four LAA transmitters are likely simultaneously transmitting), the resultant interference power received at Wi-Fi clients generally inhibits further Wi-Fi transmissions.

Figure 2: Percentage of time occupancy as a function of UEs/AP for LAA-LTE

# Interference power received at all Wi-Fi nodes with LTE-LAA in full buffer conditions

Here we assume a worst-case scenario of the LTE-LAA network running into congestion such that all eNodeB are transmitting all of the time at 23dBm on the shared channel.  Summing interference received from these at the different Wi-Fi device and AP locations shows that the CCA-ED threshold will always be exceeded by a large margin, as shown in Figure 3. This reveals that LTE-LAA in a loaded state is likely to inhibit any new transmissions from a co-existing Wi-Fi STA.

Figure 3 Power received at Wi-Fi clients from Full Buffer LTE-LAA transmissions

Observation 1: Due to lack of LBT, LAA-LTE eNB transmit at nearly 90 % of time when the number of served UEs is around 15 UE/AP. This implies significant source of interference at Wi-Fi transmitter STA particularly since the aggregated interference from all four LAA-LTE eNB can be significantly higher than the energy detection threshold at the Wi-Fi transmitter STAs (leading to contention backoffs).

# Results: Network Throughputs with/without LAA

Figure 4 show the operator 1’s Wi-Fi throughput when operator 2 deploys either Wi-Fi or LAA. The dashed line plots the offered load at operator 1 which is calculated as follows: since the average inter-arrival time between files equals 5 seconds, the average offered load per user equals 0.5 MB * 8 bits/byte * 1/5 = 0.8 Mbits/second.  Since both UL and DL are modeled for Operator 1, its offered load given Nsta clients/AP equals 4 * 1.6 * Nsta = 6.4 Nsta Mbits/second.  For example, given Nsta = 20 clients per AP, the aggregate offered load equals 128 Mbits/second.

Figure 4: Op1's aggregated Wi-Fi throughput when Op2 deploys LAA/Wi-Fi (DL only). Dashed lines show the offered load.

When Operator 2 deploys Wi-Fi, Operator 1’s aggregated throughput (DL + UL) tracks the offered load up to nearly 20 clients/AP (or an aggregated offered load equaling 128 Mbps). Beyond 20 clients/AP, the number of contending STAs becomes large thereby increasing the number of contention back-offs, and hence the served throughput saturates thereafter. On the other hand, when operator 2 deploys LAA-LTE (without LBT) we see that congestion affects operator 1 at and beyond 10 clients/AP (or an aggregate offered load = 64 Mbits/second) after which the served load eventually becomes zero.

The reasons for this are:

- Wi-Fi defers to LAA-LTE transmissions that are received above the energy detection threshold (= -62 dBm). In congested scenarios, due to prolonged FTP3 file transfer (4 MBits file size) duration, one or more heavily loaded LAA-LTE transmitters can therefore prevent all Wi-Fi STAs within the energy detection threshold from accessing the medium.

- The Wi-Fi receiver STAs can experience SINR < 0 dB if an LAA eNB starts transmitting simultaneously as the Wi-Fi transmitter STA. This leads to potential packet collision at the Wi-Fi receiver STA.

- Even if the Wi-Fi receiver STA were to successfully decode the Wi-Fi transmitter STAs MPDU, the decoding of the MAC-ACK (sent after SIFS interval) can be unsuccessful due to its SINR < 0 dB from LAA interference. Note that failed reception of the MAC-ACK invokes increment of the retry counter, doubling of the contention window and restart of the backoff timer at the transmitter Wi-Fi STA prior to retransmission.

Figure 5: Mean Retries per Frame (for successfully transmitted packets) when Operator 2 deploys Wi-Fi (red) and LAA (green)

In Figure 5, we plot the mean number of retransmissions (for successfully transmitted packets) normalized by number of transmitted frames. When Operator 2 deploys Wi-Fi, the number of retries at Operator 1 remains quite small up to 20 clients/AP after which the impact of congestion takes effect. On the other hand, when Operator 2 deploys LAA without LBT, the mean retries increases to nearly 1.4 retries per frame at 10 clients/AP. The reason for this is that frequent LAA transmissions lead to increased interference and reduced TXOPs for successful reception at Wi-Fi STAs.

Next, the aggregated network throughput is shown, first with Wi-Fi at operator 2 (Figure 6) and then with LAA-LTE without LBT at Operator 2 (Figure 7).  We note that Operator 2’s performance is shown only for the case when it is transmitting Wi-Fi. The figures show that the relative degradation in performance at Operator 1’s Wi-Fi occurs over both UL and DL directions.  However, it can be seen that for both Operator 2 (Wi-Fi) and Operator 2 (LAA-LTE), the degradation in throughputs occurs on the DL direction prior to the UL.

Figure 6: Total Network Throughput when Operators 1 and 2 deploy Wi-Fi

Figure 7: Total Network Throughput when Operator 2 deploys Wi-Fi and when Operator 2 deploys LAA-LTE

Summary

- When both operators deploy Wi-Fi, the aggregated throughputs at both operators is quite close to the offered loads even when the number of UEs/AP is large (20 UE/AP) as can be seen from Figure 4.

- With LAA-LTE deployment, a significant throughput degradation is observed at Operator 1’s network even at offered load of 10 UE/AP. Beyond 15 UE/AP, the aggregated throughput at Operator 1 is close to zero.

# Results: Uplink UPT Statistics

This section shows the uplink UPT statistics collected from transmissions occurring over Operator 1’s devices when Operator 2 deploys either Wi-Fi DL or LAA-LTE SDL. Note that the UPT calculation is performed per object (i.e. per file transfer) as shown in below Figure 8.

UPT

UPT

1 drop lasts 100secs

UPT

Figure 8: UPT is calculated per object (file download)

Figure 9 shows the CDFs of the uplink UPT for Operator 1 for the considered cases. Even at loads as low as 5 UEs/AP, there is a marked difference in the UPT between the two cases. The gap widens further as the offered load increases. By 15 UE/AP, we can see that LAA-LTE reduces 98% of Operator 1’s UPTs to approach zero.

Figure 9: Cumulative Distribution Function of the User Throughput over Uplink at 5 UE/AP

Figure 10 plots the median and the 5th percentile UL UPTs on Operator 1 uplink with and without LAA-LTE. For any given traffic load, LTE-LAA as an aggressor has a larger impact on uplink UPT in the victim network. Even at a relatively light load of 5 UEs/AP, the reduction in Operator 1’s median UL UPT due to LAA transmissions by Operator 2 is nearly 25%.

Figure 10: Mean UPT from STA to AP transmissions for Operator 1 (per packet transmission)

Figure 11 shows the corresponding DL throughputs per UE but averaged across the duration of a drop (= 100 seconds). Starting at a traffic load at 5 users/AP, the median and 5th percentile UE throughputs obtained at Operator 1 is significantly lower (median throughputs are about 7x lower at 10 UE/AP) when Operator 2 deploys LAA-LTE.

Figure 11: Mean UPT from AP to STA transmissions for Operator 1 (averaged across 100 seconds)

Summary: The median and 5th percentiles UPTs at Operator 1 are significantly impacted over both DL and UL (even at low loads) when the aggressor network (Operator 2) deploys LAA-LTE.

# Conclusion

Based on the system simulation results in this paper, the key conclusions are as follows:

- In medium-to-high density indoor enterprise deployments, the aggregated interference at Wi-Fi devices from LAA-LTE transmissions can often cause Wi-Fi devices to be deprived of access to the medium due to contention backoffs. The effect is particularly pronounced when the LAA-LTE eNBs are transmitting continuously (medium-to-high offered loads).

- Even at light loads, LAA without LBT is a significantly worse neighbour to Wi-Fi than Wi-Fi is, in terms of aggregated network throughputs and UPTs. The performance impact due to interference from LAA-LTE due to lack of an LBT mechanism are observed over both DL and UL directions in Wi-Fi.

The conclusions in this paper are expected to extend to a two operator LAA deployment as well since a dominant DL interferer from one operator can significantly impact reception (analogous to a CSG scenario) at UEs belonging to a different operator.

Proposal 1: Specify a standardized LBT mechanism for LTE to fairly coexist with in Rel-13.

Proposal 2: Ensure that the scope of the LBT mechanism makes LTE robust enough to justify its operation and ensuring fair coexistence with other RATs in unlicensed spectrum.

Proposal 3: LBT for LTE should be designed in a manner than ensures that on average, the overall performance as seen across a variety of loads with LAA + Wi-Fi deployment should be close to the overall performance obtained for the same load with Wi-Fi + Wi-Fi deployment.

# References

- RP-141664, Study on Licensed-Assisted Access using LTE.

- Email discussion [78bis-15] on Detailed Coexistence Evaluation Assumptions for LAA.

- R1-144602, “Evaluation Assumptions and Methodologies for Modelling of PHY layer Options for LAA”, Cisco Systems

- R1-144603, “Initial Coexistence Evaluations comparing LAA Versus Wi-Fi”, Cisco Systems.

- Cisco Aironet Series 2700/3700 Access Points Deployment Guide, available at http://www.cisco.com/c/en/us/td/docs/wireless/technology/apdeploy/7-6/Cisco_Aironet_3700AP.html#pgfId-92249)

- European Commission Joint Research Center, “JRC inputs on RLAN deployment density”

- IEEE P802.11 Wireless LANs “TGAx Simulation Scenarios”, July 18, 2014.

# Annex: Wi-Fi Simulation Parameters

| Parameter |  | Value | Cisco’s Assumptions |
| --- | --- | --- | --- |
| MCS |  | 802.11ac MCS table (FFS: include 256QAM) | Yes, 256 QAM included |
| Antenna configuration |  | FFS (DL): Alt. 1: 2Tx2Rx in DL, Cross-polarized Alt. 2: 2Tx1Rx in DL, Cross-polarized. FFS (UL): 1Tx2Rx or 2Tx2Rx | Alt1 DL: UL 2x2 |
| Channel code interleaving depth |  | FFS: K Wi-Fi OFDM symbol |  |
| Frame aggregation |  | A-MPDU | Yes |
| MPDU |  | Fixed 1500B MPDU size (variable transmission duration) | Yes |
| TXOP |  | 4.096ms (Asynchronous to LTE timings) | Yes 4.096ms is the maximum |
| MAC | Coordination | DCF | Yes |
|  | SIFS, DIFS | SIFS, DIFS | Yes |
|  | Detection | Energy detection & preamble detection | Yes |
|  | RTS/CTS | FFS | Not modeled |
|  | Contention window | Min : 15 slot, FFS: Max : 1023 slot | Yes |
| CCA-CS |  | --82dBm and preamble decoding (Note: preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) | Yes |
| CCA-ED |  | -62dBm | Yes |
| ACK Modeled |  | Yes | Yes |
| DL/UL Duplexing |  | FFS: Alt 1: DL and UL simulation Alt 2: DL only | Alt1: DL and UL for Operator 1 DL only for operator 2 |
| Rate control |  | FFS: Minstrel algorithm | Fixed transport format |
| Channel selection |  | FFS: Alt. 1: Minimum number of neighbors Alt. 2: Random | Congested scenario: Single 20MHz channel for all APs |

# Annex: LAA Simulation Parameters

| Parameters | Value | Cisco’s Assumptions |
| --- | --- | --- |
| PCI planning for each NW | Planned | Not explicitly modelled |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. FFS: 1Tx2Rx or 2Tx2Rx in UL | 2x2 Cross polarized |
| Transmission schemes | TM10, QPSK/16QAM/64QAM/256QAM | Transmission Mode 4 Link adaptation QPSK - 64 QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) | Not explicitly modelled |
| Scheduling | Proportional fair | Fair |
| Link adaptation | Realistic | Ideal |
| CCA-ED | -73 dBm/MHz + 23 – PH dBm (PH is e.i.r.p) | No CCA implemented for LTE-LAA |
| Channel selection | FFS: Proprietary or specified | Not implemented |

# Annex: Simulation Parameters, Indoor

|  | Licensed cell |  | Unlicensed cell | Cisco’s Assumptions |
| --- | --- | --- | --- | --- |
| Layout for nodes | Two operators deploy X small cells each in the single-floor building. Alt 1: The small cells are equally spaced in the center of the building for all nodes. 120 m 50 m Alt 2: The small cells are equally spaced in the center of the building for all nodes belonging to one operator. The distance between two closest nodes from two operators is random. 120 m 50 m FFS: Select between Alt 1 and Alt 2. FFS: Unmanaged Wi-Fi. - FFS: Layout for unmanaged Wi-Fi if agreed. APs randomly located inside the building with operator nodes with a given minimum distance. - FFS: Select from the suggested values for the minimum distance value. Suggested values: 0m, 3m, 5m - FFS: number of APs - FFS: number of STAs per AP |  |  | Performance modeling restricted to unlicensed cell. Floor size scaled down to give 3,000 SQFT per AP (on average) 55.73 x 21.55m Alt 1 AP arrangement modelled, and scaled to smaller floor size – evenly spaced down corridor,in the middle of the hall at: 3.2331 9.6994 16.1656 22.6319 29.0981 35.5644 42.0306 48.4969 with respect to the left side of the building Alt 2 not modelled No unmanaged WiFi |
| System bandwidth per carrier | 10MHz | 20MHz |  | Only unlicensed cell explicitly modelled |
| Carrier frequency | FFS: Select between 2.0 GHz and 3.5 GHz | 5.0GHz |  | 5.3 GHz centre frequency (channel 60) |
| Carrier number | 2 (one for each operator) | Y (to be shared between two operators) FFS: Select among following options - Alt. 1: X = Y = 4 - Alt. 2: X = Y = 10 - Alt. 3: X = 4, Y = 1 - Alt. 4: {Alt. 1 or Alt. 2} + Alt. 3 |  | Alt 3: X=4 APs per operator Y=1 x 20MHz channel |
| Total BS TX power | 24dBm(Ptotal per carrier) | FFS |  | 23dBm Ptx for all unlicensed band transmissions |
| Total UE TX power | 23dBm | FFS |  | 23dBm Ptx for all unlicensed band transmissions |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] UE-to-UE: 3GPP TR 36.843 (D2D). FFS: Modeling of indoor UE-indoor UE interference (applicable when UL is simulated) (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability. FFS: Break point distance) |  |  | Yes |
| Penetration | 0dB |  |  | Yes |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  |  | Yes |
| Handover Margin | <additional entry> |  |  | 1 dB handover margin as per InH model (36.814) |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |  | Yes 2D omni |
| Antenna Height: | 6m |  |  | 6m |
| UE antenna Height | 1.5m |  |  | Yes |
| Antenna gain + connector loss | 5dBi |  |  | Antenna + connector: 5dBi 2dB feeder loss to calibrate against 36.814 InH model |
| Antenna gain of UE | 0 dBi |  |  | Yes |
| Fast fading channel between eNB and UE | ITU InH |  |  | Ped A 3kmh assumed for LTE-LAA link level |
| Number of clusters/buildings per macro cell geographical area | N/A |  |  | N/A |
| Number of small cells per cluster | N/A |  |  | N/A |
| Number of small cells per Macro cell | N/A |  |  | N/A |
| Number of UEs | - FFS: 60 UEs per operator network - FFS: If unmanaged Wi-Fi is agreed, number of UEs per unmanaged Wi-Fi AP |  |  | Range {2, 5, 10, 15, 20, 30} UEs per operator No unmanaged WiFi |

# Annex: UE Related Aspects

| Parameter | Value | Cisco’s Assumptions |
| --- | --- | --- |
| UE dropping per network | Randomly and uniformly distributed over the floor | Yes |
| Radius for small cell dropping in a cluster | N/A | N/A |
| Radius for UE dropping in a cluster | N/A | N/A |
| Minimum distance (2D distance) | 3m | 3m |
| UE receiver | MMSE-IRC as baseline | MMSE-IRC |
| UE noise figure | 9dB | Yes |
| UE speed | 3km/h | Yes |
| UE Bandwidth | An LAA UE that has both licensed and unlicensed band coverage is served by both carriers under the LTE carrier aggregation framework with a total bandwidth of 30MHz. For a Wi-Fi UE with unlicensed band coverage, the UE is served by the Wi-Fi service with a bandwidth of 20MHz. | 20 MHz bandwidth for unlicensed carrier assumed throughout |

# Annex: Traffic Modeling

| Parameter | Value | Cisco’s Assumptions |
| --- | --- | --- |
| Traffic model | - FTP model 3 - FFS: file sizes - Load varied using arrival rate - FTP model 1 - FFS: file sizes - Load varied with number of users - VoIP and video modeling - FFS: How to use FTP model 3 or 1 to approximate VoIP and video - FFS: Uni-directional or Bi-directional (i.e., both DL and UL) - FFS: Necessity of mixed traffic models - FFS: Necessity of full buffer - FFS: Priority among multiple traffic models | All UEs use: FTP 3 With 0.5MB file Lambda=0.2Hz (5s mean interarrival) NB this generates a long term mean demand of 0.5MB x 8bits/Byte x 0.2Hz = 0.8 Mbps |
| Performance metrics | - Performance metric - User perceived throughput (UPT) - UPT CDF - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF - FFS: Number of users with X %ile latency < Y ms (e.g. X = 98, Y = 80 ms) - Note: DL and/or UL can be reported when applicable - FFS: Necessity of other system metric to help interpreting the performance results - FFS: Definition of packet needs further clarifications depending on used traffic model | Total Tput over all APs belonging to operator 1 UL UPT CDF DL User throughput is averaged over the entire simulation time per drop. This provides a cautious estimate of UPT at light loading conditions where capacity exceeds demand. Latency not modelled |

# Annex: Network Synchronization and Backhaul

| Network synchronization | For the same operator, the network can be synchronized or not synchronized; if an evaluated feature requires synchronization, this should be stated evaluations without synchronization are not precluded, and the assumed synchronization accuracy in such simulations should be stated. Asynchronous between different operators is baseline. FFS: Select from the following for the baseline assumption for intra-operator synchronization: Alt 1: Nodes of an operator are synchronized and time-aligned. Alt 2: Nodes of an operator are not synchronized and time-aligned. | Alt 2 All APs not synchronized or time aligned |
| --- | --- | --- |
| Backhaul assumptions | • The latency and throughput values for non-ideal backhaul indicated in Table 6.1-1 of 36.932 are the baseline assumptions -The latency values of {2ms, 10ms, 50ms} are recommended for evaluation. • Whether and how the backhaul assumptions are explicitly modelled in the simulations should be indicated by companies when presenting the results. • Proposals considering backhaul assumptions should analyse the influence of these assumptions on the delivery of the information to be exchanged and on the access network performance metrics. | Backhaul not modelled |