3GPP TSG RAN WG1 #79	R1-144604

November 17th- 21st, 2014

San Francisco, USA

Agenda item:	6.3.2.2

Source: 	Cisco Systems

Title: 	Candidate Detailed Solutions for DL Transmissions in Unlicensed Spectrum

Document for:	Discussion and Decision

# Introduction

In RP-65 the new study idem on LTE transmissions in unlicensed band was approved [1]. The objectives of this SI, in terms of RAN1 inputs were defined as:

- Define an evaluation methodology and possible scenarios for LTE deployments, focusing on LTE Carrier Aggregation configurations and architecture where one or more low power Scell(s) (ie. based on regulatory power limits) operates in unlicensed spectrum and is either DL-only or contains UL and DL, and where the PCell operates in licensed spectrum and can be either LTE FDD or LTE TDD. [RAN1]

- Document the relevant requirements and design targets for unlicensed spectrum deployment, in particular:

  - Document the relevant existing regulatory requirements for unlicensed spectrum deployment in the 5GHz bands [RAN4]

  - Document considerations of introducing licensed-assisted access to unlicensed spectrum whilst highlighting the continued importance/need for licensed spectrum allocations [RAN4]

  - Identify and define design targets for coexistence with other unlicensed spectrum deployments, including fairness with respect to Wi-Fi and other LAA services. This should be captured in terms of relevant fair sharing metrics, e.g., that LAA should not impact Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier; these metrics could include throughput, latency, jitter etc. This should also capture in-device coexistence for devices supporting LAA with multiple other-technology radio modems, where it should, e.g., be possible to detect Wi-Fi networks during LAA operation; note that this does not imply concurrent LAA+Wi-Fi reception/transmission. This should also capture co-channel coexistence between different LAA operators and between LAA and other technologies in the same band. [RAN1, RAN4]

- Identify and evaluate physical layer options and enhancements to LTE to meet the requirements and targets for unlicensed spectrum deployments identified in the previous bullet, including consideration of the methods to address the co-existence aspects on unlicensed bands with other LTE operators and other typical use of the band [RAN1]

In particular it was identified that the study had to cover both single and multi-operator scenarios, including the case where multiple operators deploy LTE in the same unlicensed bands.

In the email discussions following RAN1 #78bis [2] as well as in a companion contribution [3], we have outlined Cisco Systems’ preferences in terms of the evaluation methodology for studying coexistence when LAA is deployed in unlicensed spectrum. In [4], we have shown coexistence results for a two-operator indoor enterprise deployment assuming a Rel-10 LAA PHY/MAC. The results demonstrate that when both operators deploy Wi-Fi, the network is able to serve a range of offered loads and gracefully degrades when the offered load exceeds a certain value. This indicates that the Wi-Fi MAC/PHY are well designed in terms of spectral efficiency and medium access for fair coexistence in unlicensed spectrum.  On the other hand, with LAA, we see significant performance degradation due to LAA transmissions on Wi-Fi devices belonging to a different operator.  This suggests that an LAA PHY/MAC without modification/enhancements (i.e. LBT enhancements) is not designed for systems with inter-operations with other RATs designed with unscheduled access (e.g. Wi-Fi).

In this contribution, we present Cisco Systems’ views on candidate DL transmissions in unlicensed spectrum in order to ensure fair coexistence of LAA and Wi-Fi.

# Design Principles

Toward the goal of fair coexistence, it is useful to first identify the high-level design goals for LBT enhancements to LTE:

            - Ensure that the average performance (in terms of a well-defined set of performance metrics) in a Wi-Fi (operator 1) + Wi-Fi (operator 2) [with/without presence of unmanaged devices] is identical to the average performance when one of the operators replaces their Wi-Fi with LAA devices. The performance gap should be small/zero in both low-to-medium and high load scenarios.

            - Ensure fair coexistence between LAA devices belonging to multiple operators through LBT and minimizing hidden terminal occurrences.

The first design principle ensures that the LBT mechanism is optimized to perform comparably against existing Wi-Fi channel access [CSMA/CA and additional enhancements such as EDCA]. In addition it is essential that LAA transmission durations (particularly in low-to-medium offered loads) are short in time in order to ensure that neighbor LAA/Wi-Fi devices get to reuse spectrum efficiently. Wi-Fi is already well optimized for indoor deployments by virtue of its short symbol duration whereas LTE transmissions can occupy large duration due to their 1 ms subframe duration. However, considering that the SID has mentioned reusing the features of LTE as much as possible, adding newer enhancements to LTE such as shorter sub frames and/or shorter symbol times (e.g. larger subcarrier spacing) can be left to later releases.

The second principle addresses the uncoordinated inter-operator deployment scenario wherein there is no coordination/synchronization between transmissions across different AP; additionally because the relative locations of eNB belonging to different operators can be unplanned and hence the interference can be quite significant.

Proposal 1: Identify LBT enhancements in LAA to achieve a design target such that the performance in a multi-operator Wi-Fi network is close to the performance when one of the operators replaces their Wi-Fi with LAA-LTE

Proposal 2: Provide assistance information to enable neighbor LAA-LTE devices to detect LAA transmissions so as to minimize hidden terminal scenarios.

# Clear Channel Assessment

For the LBT mechanism upon arrival of new data, the LAA eNB can perform Clear Channel Assessment like procedure in Wi-Fi to detect whether or not the medium is busy. Upon the end of detection of the medium being busy, the LAA transmitter can begin transmission. Just like Wi-Fi, the duration when the medium becomes idle has the highest probability when multiple LTE-LAA APs and/or Wi-Fi STAs belonging to different operators start competing for medium access.

To ensure fair coexistence with neighbor AP as well as Wi-Fi STAs on the same channel, it is useful to have a contention backoff mechanism similar to Wi-Fi.

In the CCA period, the eNB should determine whether or not the medium is busy. During this duration, the eNB should not radiate energy (including reference signals). This ensures zero interference to neighbor eNB belonging to the same/different operator as well as Wi-Fi STAs on the same frequency channels. Prior to starting transmission after the completion of CCA and the contention backoff procedure, Proposals 3 and 4 summarize our views; below  Figure 1 depicts an example.

Proposal 3: Adopt a LBT similar to CSMA-CA in Wi-Fi.

- No reference signals should be transmitted as part of the CCA procedure.

Proposal 4: For fair co-existence with co-channel Wi-Fi STA and AP of same/different operator adopt a random back-off mechanism to minimize the probability that multiple AP transmit simultaneously upon detection of medium idle.

- Adopt a timer-expiry based mechanism for determining when to transmit when medium is idle.

Figure 1: Proposed LBT mechanism for channel access for LAA-LTE

The loss in transmission of reference signal during the LBT period means that UEs may lose time/frequency sync with their serving eNB (particularly in heavily loaded scenario when multiple APs simultaneously contend for the medium). A preamble signal can be adopted prior to transmission (after completion of CCA and backoff timer expiry). The preamble signal could serve the following purposes:

- Coherent phase reference for time/frequency tracking to neighbor intra-/inter-operator APs.

- Assistance information that can indicate the duration of each downlink transmission.

Proposal 5: Study further the need for preamble signal to provide assistance information for time/frequency tracking and providing duration information following CCA completion.

# Conclusion

Proposal 1: Identify LBT enhancements in LAA to achieve a design target such that the performance in a multi-operator Wi-Fi network is close to the performance when one of the operators replaces their Wi-Fi with LAA-LTE

Proposal 2: Provide assistance information to enable neighbor LAA-LTE devices to detect LAA transmissions so as to minimize hidden terminal scenarios.

Proposal 3: Adopt a LBT similar to CSMA-CA in Wi-Fi.

- No reference signals should be transmitted as part of the CCA procedure.

Proposal 4: For fair co-existence with co-channel Wi-Fi STA and AP of same/different operator adopt a random back-off mechanism to ensure that multiple AP cannot transmit simultaneously upon detection of medium idle.

- Adopt a timer-expiry based mechanism for determining when to transmit when medium is idle.

Proposal 5: Study further the need for preamble signal to provide assistance information for time/frequency tracking and providing duration information following CCA completion.

# References

- RP-141664, Study on Licensed-Assisted Access using LTE.

- Email discussion [78bis-15] on Detailed Coexistence Evaluation Assumptions for LAA.

- R1-144602, “Evaluation Assumptions and Methodologies for Modeling of PHY layer Options for LAA”.

- R1-144603, “Initial Coexistence Evaluations comparing LAA Versus Wi-Fi”, Cisco Systems.