3GPP TSG-RAN WG1 #79	R1-144669

San Francisco, USA, 17th – 21th Nov 2014

Source: 	Intel Corporation

Title:                     	System Evaluation of Rel-12 Systems with 3D Channel Model

Agenda item:	    6.3.3.2

Document for:	Discussion and Decision

# Introduction

In RAN1 #78bis meeting, it has been agreed to start the first simulation campaign to evaluate Rel-12 DL MIMO system performance with the latest 3D channel model [1]. In this contribution, we report system throughput of Rel-12 DL MIMO systems using the agreed simulation assumptions in [2]. Antenna down-tilt for each scenario is further discussed and determined in email discussion [70bis-18].

# User Throughput of Rel-12 DL MIMO

In this section we report the system-level throughput results for UMa and UMi scenarios. For UMa scenario, both 500m and 200m ISD are simulated. For UMi and UMa with 200m ISD, 3.5GHz carrier frequency is simulated in addition to 2GHz carrier frequency. We use FTP traffic model 1 and 500k bytes packet size to simulate the system throughput. Different packet arrival rates have been selected in simulations to cover three resource utilization values of interests: 20%, 50% and 70%. The complete simulation parameters follow the agreements in [2] and the down-tilt values follow the harmonized value of the email discussion [70bis-18].

Table 1: FTP Throughput for UMa (500m ISD) 2GHz, 100 degree down-tilt

Table 2: FTP Throughput for UMa (200m ISD) 2GHz, 104 degree down-tilt

Table 3: FTP Throughput for UMa (200m ISD) 3.5GHz, 104 degree down-tilt

Table 4: FTP Throughput for UMi (200m ISD) 2GHz, 100 degree down-tilt

Table 5: FTP Throughput for UMi (200m ISD) 3.5GHz, 100 degree down-tilt

Observation1: The FTP traffic load 1.5, 3 and 4 packets/sec/cell result in 20%, 50% and 70% resource utilization roughly for the Rel-12 DL MIMO performance in all FD-MIMO scenarios.

Observation2: For a given antenna down-tilting, UMa scenario provides higher system throughput and lower system load comparing to UMi scenario. One possible reason is that the fixed vertical narrow beam in the UMi scenario provides relatively poorer coverage for UEs on the high floors comparing to UMa scenario.

# Clarification of Simulation Assumptions

In [2] three target resource utilizations have been selected for FTP simulations, which are 20% RU, 50% RU and 70% RU. From the simulation results provided in section 2, the target resource utilizations roughly corresponds to FTP traffic loads 1.5, 3 and 4 in all scenarios for Rel-12 DL MIMO user throughput. For future FD-MIMO evaluations, we recommend simulating these three FTP traffic arrival rates instead of the three target resource utilization for easier calibration of the results as well as quantifying the throughput gains of any advanced beamforming algorithms. This is due to the fact that the system throughput gain for one advanced beamforming algorithm can be either quantified by using the improved user throughput and reduced resource utilization of the same FTP traffic load or using the increased FTP traffic load for the same resource utilization ratio. However, using the latter methodology can significantly increase the simulation length especially for the advanced beamforming algorithm and high resource utilization ratio.

Recommendation: We recommend to simulate FD-MIMO enhancements using FTP traffic load corresponding to 20%, 50% and 70% resource utilization in Rel-12 DL MIMO user throughput for better calibrating and improved simulation efficiency.

# Conclusion

In this contribution, we present our phase one simulation results using FTP traffic model one. We have followed the agreed simulation assumptions and tested FTP traffic load to cover 20%, 50% and 70% resource utilization ratios. We have simulated both UMa and UMi scenarios. From the simulation results, we have below observations and recommendation for the following 3D-MIMO SI work:

Observation1: The FTP traffic load 1.5, 3 and 4 packets/sec/cell result in 20%, 50% and 70% resource utilization roughly for the Rel-12 DL MIMO performance in all FD-MIMO scenarios.

Observation2: For a given antenna down-tilting, UMa scenario provides higher system throughput and lower system load comparing to UMi scenario. One possible reason is that the fixed vertical narrow beam in the UMi scenario provides relatively poorer coverage for UEs on the high floors comparing to UMa scenario.

Recommendation: We recommend to simulate FD-MIMO enhancements using FTP traffic load corresponding to 20%, 50% and 70% resource utilization in Rel-12 DL MIMO user throughput for better calibrating and improved simulation efficiency.

# References

- RP-141207, Study on 3D channel model for LTE, v2.1.0

- R1-144444, WF on simulation assumptions, Nokia Networks, Nokia Corporation, CMCC, Samsung