3GPP TSG RAN WG1 Meeting #79		R1-145053

San Francisco, USA, 17th -21st November 2014

Agenda item:	6.3.3.2

Source:	InterDigital

Title:	Evaluation results of Rel-12 DL MIMO scheme

Document for:	Discussion

# 1	Introduction

In RAN1 #78bis, there was a noticeable progress on the antenna modelling and evaluation methodologies for elevation beamforming and FD-MIMO. In this contribution, we provide evaluation results of Rel-12 DL MIMO schemes based on the agreed evaluation assumptions in the 3D channel model.

# 2	Evaluation Results for Rel-12 MIMO schemes

In this section, we evaluate the Rel-12 downlink MIMO performance for 4 and 8 horizontal antenna ports in 3D-UMa and 3D-UMi channels. The simulation assumptions are listed in Table A in Annex. Tables 1 and 2 show the performance of DL MIMO schemes with dynamic SU and MU-MIMO switching with full buffer traffic model and non-full buffer traffic model, respectively. In addition, no UE transparent elevation beamforming technique (e.g. vertical sectorization) is used.

Table 1:  full buffer results

| Antenna configuration | Performance metric | 3D-UMa SU/MU | 3D-UMi SU/MU |
| --- | --- | --- | --- |
| (8,2,2) Xpol, K=8 (LTE 4Tx codebook) | Cell avg. (b/Hz/s) | 2.16 | 2.07 |
|  | Cell edge (b/Hz/s) | 0.057 | 0.05 |
|  | BLER (%) | 9.2% | 9.3% |
| (8,4,2) Xpol, K=8 (LTE 8Tx codebook) | Cell avg. (b/Hz/s) | 2.19 | 2.33 |
|  | Cell edge (b/Hz/s) | 0.067 | 0.055 |
|  | BLER (%) | 9.4% | 9.4% |

Table 2: non-full buffer results (high load) with offered load ~10.4 Mbps/sector

| Antenna configuration | Performance metric | 3D-UMa SU/MU | 3D-UMi SU/MU |
| --- | --- | --- | --- |
| (8,2,2) Xpol, K=8 (LTE 4Tx codebook) | mean throughput (Mbps) | 15.3 | 13.9 |
|  | 5% UE throughput (Mbps) | 2.4 | 1.9 |
|  | RU (%) | 63% | 66% |
| (8,4,2) Xpol, K=8 (LTE 8Tx codebook) | mean throughput (Mbps) | 13.3 | 14.7 |
|  | 5% UE throughput (Mbps) | 3.0 | 3.4 |
|  | RU (%) | 57.3% | 59.3% |

Table 3: non-full buffer results (low load) with offered load ~4 Mbps/sector

| Antenna configuration | Performance metric | 3D-UMa SU/MU | 3D-UMi SU/MU |
| --- | --- | --- | --- |
| (8,2,2) Xpol, K=8 (LTE 4Tx codebook) | mean throughput (Mbps) | 29.3 | 30.3 |
|  | 5% UE throughput (Mbps) | 2.9 | 3.0 |
|  | RU (%) | 24% | 20% |
| (8,4,2) Xpol, K=8 (LTE 8Tx codebook) | mean throughput (Mbps) | 26.8 | 29 |
|  | 5% UE throughput (Mbps) | 7.9 | 6.9 |
|  | RU (%) | 23% | 21% |

# 3	Summary

In this contribution, we provided initial evaluation results of Rel-12 DL MIMO scheme using dynamic SU and MU-MIMO switching in 3D-UMa and 3D-UMi.

# References

- RP-141644, “New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia Networks.

- 3GPP TR 36.873, “Study on 3D channel model for LTE,” V2.0.0, Release 12, Mar. 2014.

- R1-144444, “WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, “WF on Antenna modeling”, Samsung, Nokia Networks, Nokia Corporation, CMCC, Huawei, HiSilicon, Interdigital, LGE, Motorola Mobility, Qualcomm, CHTTL

# Annex

Table A. System Level Simulation Assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | 7-site 21-cell wraparound |
| Channel model | 3D Urban Macro (3D-UMa) 3D Urban Micro (3D-UMi) [2] |
| eNB antenna configuration | (M,N,P)=8,2,2,(8,4,2), 0.5 λ spacing in H, 0.8 λ spacing in V, cross-polarization (X-pol) [3,4] |
| UE antenna configuration | nR=2, cross-polarization, 0o/90o |
| UE attachment | based on RSRP formula in [2] |
| UE distribution | according to Table 6-1 [2] |
| Electrical Downtilt | θtilt = 100o |
| Number of UEs per cell | 10 |
| UE distribution | uniformly dropped according to [2] |
| Traffic model | full buffer non-full buffer FTP model 1, packet size 0.5M bytes, offered sector load [4, 10] Mbps/sector |
| Scheduler | proportional fair (PF) |
| Transmission scheme | SU/MU-MIMO dynamic switching with SU-MIMO feedback, Non-transparent MU-MIMO |
| Codebook | LTE 4Tx and 8Tx codebook |
| Link adaptation | AMC with OLLA, 10% BLER target |
| Receiver | MMSE-IRC, ideal channel estimation, ideal interference modelling |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered per 5ms |
|  | feedback delay is 5 ms |
| Maximum number of HARQ retransmission | 4 |