3GPP TSG RAN WG1 Meeting #79                                                                                 	R1-145444

San Francisco, USA, 17th – 21st November 2014

Source:	Ericsson

Title:	Summary of Adhoc Session on LAA

LAA

Agenda Item:	6.3.2

Document for:	Discussion and Decision

Open issues on evaluation scenarios: Transmit power

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Total BS TX power | 24dBm(Ptotal per carrier) | 18 dBm across aggregated carriers Optional: 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18dBm |  |

R1-145383           WF on Cell association   LG Electronics, NTT DOCOMO, Fujitsu

Change RSRP to RSS for WiFi.

- For WiFi STAs,

  - Cell association based on RSS of WiFi APs

    - RSS of WiFi: Received signal power strength

  - RSS threshold is -82 dBm

Cell selection based on unlicensed band RSRP or RSS. Capture this in tables.

R1-145320           Summary of initial co-existence evaluations        Huawei, HiSilicon, Cisco, NTT Docomo, ZTE

Alternate proposed agreement:

- Companies should at least provide results with CCA for coexistence evaluations

R1-145336           Way Forward on LAA Evaluation Assumptions: Load Factor           Intel, Samsung, Qualcomm, Ericsson

Working assumption: A new metric, buffer occupancy is defined:

  - Buffer occupancy of the i-th small cell/UE (Wi-Fi & LAA) = sum of the period of time during which the i-th small cell/UE has data to transmit including retransmissions (i.e., its queue is not empty) / total simulation time

  - Average buffer occupancy: buffer occupancy averaged over the all small cells/UEs of the same operator

- The average buffer occupancy can be provided in addition to the offered traffic along with the simulation results.

- FFS: Whether and how to capture this metric in the TR

- Note: This is not a metric that will be used to make comparisons between different evaluations

R1-145443            Proposal on modified resource utilization for LAA evaluation         NTT DOCOMO, INC.:

Further discussion on this and other metrics and how they are used to classify results from different companies.

R1-145380           Way Forward on Subframe Structure, Numerology and System Bandwidth for LAA          Qualcomm, Intel, Nokia Corporation, Nokia Networks, ETRI, NTT DOCOMO, CATT

Also supported by ALU, ASB

Possible conclusion:

- For coexistence evaluations

  - Normal CP is used.

  - Companies can consider fractional OFDM symbol transmissions