3GPP TSG RAN WG1 Meeting #79                                                                                 	R1-145453

San Francisco, USA, 17th – 21st November 2014

Source:	Ericsson, Huawei

Title:	Text Proposal on Coexistence Evaluation Assumptions for LAA for TR

36.889

Agenda Item:	6.3.2.1

Document for:	Discussion and Decision

# Introduction

This contribution presents a text proposal for Annex A of TR 36.889 capturing the agreements made to date for the LAA coexistence evaluation assumptions. Section 2 presents the text proposal. The colours will be removed when the TP is incorporated in the TR.

# Text Proposal

--------------------------------------------- BEGIN Text Proposal ----------------------------------------------------

## Annex A: 		Evaluation Methodology

# A.1		General evaluation assumptions

## A.1.1	Indoor scenario for LAA coexistence evaluations

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. 120 m 50 m |  |
| System bandwidth per carrier | 10MHz | 20MHz |
| Carrier frequency | 3.5GHz | 5.0GHz |
| Number of carriers | 2 (one for each operator) | For DL-only LAA coexistence evaluations: 1, 4 (to be shared between two operators) |
| Total BS TX power | 24dBm (Ptotal per carrier) | 18 dBm across aggregated carriers Optional: 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  |
| Penetration | 0dB |  |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |
| Antenna Height: | 6m |  |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 5dBi |  |
| Antenna gain of UE | 0 dBi |  |
| Fast fading channel between eNB and UE | ITU InH |  |
| Number of clusters/buildings per macro cell geographical area | N/A |  |
| Number of small cells per cluster | N/A |  |
| Number of small cells per Macro cell | N/A |  |
| Number of UEs | 10 UEs per unlicensed band carrier for DL-only LAA coexistence evaluations |  |

| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell Example of a dropping method to achieve this with N=10 UEs: - Drop a large enough number of UEs, so that at least 10 UEs are covered by the small cell. - Randomly select 10 UEs from the UEs that have coverage. |
| --- | --- |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum distance (2D distance) | 3m |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP Model 1 as in TR 36.814 FTP model file size: 0.5 Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100% statistics are independently reported in each direction - No associated control plane traffic is modelled |
| UE receiver | MMSE-IRC as baseline |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed - CA scheduling assumptions stated when reporting results - Served traffic per small cell per carrier can be reported UE bandwidth for Wi-Fi: 20 MHz unlicensed |
| Network synchronization | For the same operator, the network can be synchronized and the assumed synchronization accuracy in such simulations should be stated. Small cells of different operators are not synchronized. |
| Performance metrics | - Performance metric - User perceived throughput (UPT) - UPT CDF - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF - If VoIP users are included, number of VoIP users with 98%ile latency greater than 50 ms should be reported - Note: DL and/or UL can be reported when applicable |

## A.1.2	Outdoor scenario for LAA coexistence evaluations

|  | Macro cell | Licensed small cell | Unlicensed small cell |
| --- | --- | --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, case 1 500m ISD Macro eNBs of the two networks are collocated. Both 19 Macro sites and 7 Macro sites can be used. Companies should indicate whether 19 or 7 sites are used when presenting the results. | Clusters uniformly random within macro geographical area; 4 small cells per operator, uniformly random dropping within cluster area. |  |
| System bandwidth per carrier | 10MHz | 10 MHz | 20MHz |
| Carrier frequency | 2.0GHz | 3.5 GHz | 5.0GHz |
| Number of carriers | 2 (one for each operator) | 2 (one for each operator) | For DL-only LAA coexistence evaluations: 1, 4 (to be shared between operators) |
| Total BS TX power | 46dBm (Ptotal per carrier) | 30 dBm (Ptotal per carrier) | 18 dBm across aggregated carriers Optional: 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  |  |
| Distance-dependent path loss | ITU UMa [referring to Table B.1.2.1-1 in TR36.814] (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for break point distance and LOS probability.) | ITU UMi [referring to Table B.1.2.1-4 in TR36.814] (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for break point distance and LOS probability.) | Small cell-to-Small cell, Small cell-to-UE: ITU Umi [referring to Table B.1.2.1-4 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D) (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for break point distance and LOS probability.) |
| Penetration | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) | For outdoor UEs:0dB For indoor UEs: 23dB+0.5din (din : independent uniform random value between [ 0, min(25,UE-to-eNB distance) ] for each link) | For outdoor UEs:0dB For indoor UEs: 27dB+0.5din (din : independent uniform random value between [ 0, min(25,UE-to-eNB distance) ] for each link) |
| Shadowing | ITU UMa according to Table A.1-1 of 36.819 Working assumption is that 3D distance is used for shadowing correlation distance | ITU UMi [referring to Table B.1.2.1-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance | ITU UMi [referring to Table B.1.2.1-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |
| Antenna pattern | 3D, referring to TR36.819 | 2D Omni-directional is baseline; directional antenna is not precluded | 2D Omni-directional is baseline; directional antenna is not precluded |
| Antenna Height: | 25m | 10 m | 10m |
| UE antenna Height | 1.5 m | 1.5m | 1.5 m |
| Antenna gain + connector loss | 17 dBi | 5 dBi | 5 dBi |
| Antenna gain of UE | 0 dBi | 0 dBi | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa according to Table A.1-1 of 36.819 | ITU Umi | ITU Umi |
| Number of clusters/buildings per macro cell geographical area | 1 |  |  |
| Number of UEs | 10 UEs per unlicensed band carrier for DL-only LAA coexistence evaluations |  |  |
| UE dropping for each network | All UEs should be randomly dropped and be within coverage of the small cell Example of a dropping method to achieve this with N=10 UEs: - Drop a large enough number of UEs, so that at least 10 UEs are covered by the small cell. - Randomly select 10 UEs from the UEs that have coverage. 100% of UEs are outdoor. |  |  |
| Radius for small cell dropping in a cluster | 50m |  |  |
| Radius for UE dropping in a cluster | 70m |  |  |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue FTP Model 1 as in TR 36.814 FTP model file size: 0.5Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100% statistics are independently reported in each direction - No associated control plane traffic is modelled |  |  |
| Minimum distance (2D distance) | Small cell-small cell: 20m |  |  |
|  | Inter-operator small cell-small cell: 10 m |  |  |
|  | Small cell-UE, UE-UE: 3m |  |  |
|  | Macro –small cell cluster center: 105m |  |  |
|  | Macro – UE : 35m |  |  |
|  | cluster center-cluster center: 2*Radius for small cell dropping in a cluster |  |  |
| UE receiver | MMSE-IRC as baseline |  |  |
| UE noise figure | 9dB |  |  |
| UE speed | 3km/h |  |  |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |  |  |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed, - CA scheduling assumptions stated when reporting results - Served traffic per small cell per carrier can be reported UE bandwidth for Wi-Fi: 20 MHz unlicensed |  |  |
| Network synchronization | For the same operator, the network can be synchronized and the assumed synchronization accuracy in such simulations should be stated. Small cells of different operators are not synchronized. |  |  |
| Backhaul assumptions | Non-ideal backhaul between macro eNB and small cell |  |  |
| Performance metrics | - Performance metric - User perceived throughput (UPT) - UPT CDF - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF - If VoIP users are included, number of VoIP users with 98%ile latency greater than 50 ms should be reported - Note: DL and/or UL can be reported when applicable |  |  |

# A.2		Additional evaluation assumptions

## A.2.1	Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM Optional: include 256QAM (should be the same as for LAA) |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Optional: 1Tx2Rx in DL. UL: 1Tx2Rx (should be the same as for LAA) Baseline: open loop Company should state assumptions if assumed otherwise |
| Channel coding |  | BCC Optional: LDPC code |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | Up to each company |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) Company should state assumptions if assumed otherwise |
| MAC | Coordination | DCF If VoIP users are included, EDCA can be used |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | Optional |
|  | Contention window | Per DCF If VoIP users are included, per EDCA can be used |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation |
| Rate control |  | Up to each company; should state assumption when reporting results |
| Channel selection |  | Up to each company; should state assumption when reporting results |
| OFDM symbol length |  | 4 micro second |

A.2.2	Additional LAA system evaluation assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. Optional: 1Tx2Rx in DL. 1Tx2Rx in UL (should be the same as for Wi-Fi) |
| Transmission schemes | Based on TM4 or TM10, QPSK/16QAM/64QAM Optional: include 256QAM (should be the same as for Wi-Fi) |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | Up to each company; should state assumption when reporting results |
| Channel selection | Up to each company; should state assumption when reporting results |
| Cyclic Prefix | Normal |

--------------------------------------------- END Text Proposal ----------------------------------------------------

# References

3GPP TR 36.814 Further advancements for E-UTRA physical layer aspects, v.9.0.0

3GPP TR 36.872 Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects, v.12.1.0

3GPP TR 36.843 Study on LTE Device to Device Proximity Services - Radio Aspects, v.12.0.1

Minstrel Rate Adaptation Algorithm Documentation, http://madwifi-project.org/browser/madwifi/trunk/ath_rate/minstrel/minstrel.txt

R1-144471 “Way forward on Coexistence Evaluation Methodologies for LAA”, Ericsson, Qualcomm, NTT Docomo, Samsung, Nokia Networks, Nokia Corporation, Huawei, HiSilicon, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell