3GPP TSG RAN WG1 Meeting #80	                                                                             R1-150171

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.2.2.2

Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

Title: 	Enhancements to UL control signalling for up to 32 component carriers

Document for:	Discussion/Decision

# Introduction

In RAN#66 meeting, a new WI proposal of LTE Carrier Aggregation (CA) Enhancement Beyond 5 Carriers [1] was approved. In our contribution [2] we have provided some discussions and proposals on the necessary mechanisms and enhancments to enable the LTE carrier aggregation of up to 32 component carriers for DL. In this contribution, we give more detailed discussions and solutions on the enhancement of UL control signaling to support CA enhancement for up to 32 component carriers.

# Enhancements to UL control signalling

The UCI feedbacks for up to 5 CCs had been specified in Rel-10 CA, UL UCI enhancements in Rel-10 CA include,

- PUCCH format 1b with channel selection for A/N feedbacks up to 4 bits

- PUCCH format 3 for A/N feedback bits from 4 to 20 bits,

- Periodic CSI feedback for multiple CCs - Periodic CSI information is configured independently for each CC.  PUCCH resources can be configured to have CSI information from multiple CCs reported in a TDM manner, where  periodic CSI information from different CCs is carried on different subframes;

- Two-bit triggering for aperiodic CSI feedback - the number of bits used to trigger the aperiodic CSI feedback in a UL DCI has been increased from 1 bit to 1-2 bits, which increases the flexibility of aperiodic CSI feedback from the combinations of different CCs.

Following the same principle of PUCCH on PCell only and PUSCH with UCI on PCell/SCell, it is difficult to support up to 32 CCs aggregation without degradation of UCI performance. In Rel-12 dual connectivity (DC), dual PUCCHs for both MCG and SCG were supported.  Even with the support of dual PUCCHs in CA enhancement, it might not have sufficient capacity to support the UCI feedback for up to 32 CCs aggregation. It will results in dropping of A/N bits, high collisions of periodic CSI, and insufficient resource for aperiodic CSI feedback.  Moreover, the 1-2 bit CSI request field to trigger aperiodic CSI feedback might not have sufficient flexibility to request the desired combination of different CC among 32 CCs and possible 4 CSI processes for one CC if CoMP is deployed at the same time. Thus, the following solutions are considered to solve shortage of UCI feedback resource in CA enhancement beyond 5 CCs:

Solutions1: Support of multiple PUCCHs on SCells as used in Rel-12 DC. Separate the CCs into different logical groups and support one PUCCH/PUSCH with UCI on Pcell/Scell in each group, so that the current UCI feedback mechanisms and some conclusions made in Rel-10/11 CA and Rel-12 DC can be reused as much as possible. Cross-CC scheduling among different CC groups is forbidden to guarantee the maximum number of A/N bits is controllable.

Solution2: Enhance the PUCCH and CSI feedback mechanisms to increase the feedback capacity and still try to keep PUCCH on PCell for CA and two PUCCHs on PCell and pScell for DC as specified currently.

- For HARQ with PUCCH 1b cs and PUCCH 3: A/N bundling among multiple CCs can be considered with high A/N feedback efficiency, i.e. one aggregate UL ACK/NAK corresponding to DL transmissions from multiple CCs are transmitted. On the other head, FDM and/or CDM transmissions of multiple PUCCHs for a UE can be reconsidered to increase the A/N feedback capacity. In addition, new PUCCH format with support of A/N bits of up to 16/32 CCs is also a possible solution, but further evaluation may be needed for the performance comparison.

- For periodic CSI: For Rel-10/11 CA, the periodic CSI feedback of all aggregated CCs are carried on one CC either on PUCCH or on PUSCH, in a TDM manner, where periodic CSI feedbacks of different CCs are carried on different subframes. For the periodicity and offset configuration of different periodic CSI feedbacks, Table 7.2.2-1A in [3] gives the detailed values. Considering up to 32 CCs in CA enhancement, TDM feedback of periodic CSI with a shorter periodicity, such as 2, 5, 10, 20, may not have sufficient capacity, which causes more CSI dropping and further performance degradation. To alleviate this periodic CSI collision issues, FDM and/or CDM transmissions of periodic CSI feedback of multiple CCs in one subframe can be reconsidered. In addition, design of a new PUCCH format which enables carrying more CSI bits is also a possible solution but results in more specification impact.

- For aperiodic CSI:  it is triggered by a UL grant over a DL CC. Thus, aperiodic CSI feedback is multiplexed on PUSCH of the UL CC correponding to its DL CC transmitting UL grant.  Therefore, the  concern of resouce allocation of aperiodic CSI and collisison in CA enhancment to support up to 32 CCs may not be an issue. However, the trigger of aperiodic CSI report to support CSI reports from up to 32 CCs should be further investigated. Considering CA enhancement with up to 32 CCs and maximum 4 CSI processes for a CC in CoMP simultaneously, a 1 or 2-bit CSI request field may not enough.  Thus, a length of 3 or more bits CIS request field in DCI 0/4 for CA enhancement UEs in Rel-13 may be needed to increase the aperiodic CSI feedback flexibility of triggering the report for different CC and CSI process sets.

# Summary

In this contribution, we have analyzed the potential issues on the UL control signalling in CA enhancement. The shortage of UCI feedback capacity should be resolved to support up to 32 component carriers. In this contribution, we have proposed some solutions for this issue. Further discussions and specification efforts are needed on the modification/enhancement of UL control signalling to support CA enhancement with up to 32 component carriers.

# Reference

- RP-142286 New WI proposal: DCI Enhancement for LTE Carrier Aggregation Enhancement Beyond 5 Carriers

- R1-150168 CA enhancement to support up to 32 carrier aggregation, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent.

- 3GPPP TS36.213