3GPP TSG-RAN1 Meeting #80	R1-150271

# Athens, Greece, 9-13 Feb. 2015

# Source:	KDDI

# Title:	Consideration of UCI transmission for LAA

# Agenda item:	7.2.3.3

# Document for:	Discussion and Decision

- Introduction

At the RAN1#79 meeting, CSI mesuremetnt is agrred as one of functionalities which should be supported in the LAA [1]. However, its detail should be discussed. In the LAA, we have a concern about the reduction of UL capacity due to the feedback of CSI measurement. Accurately, the lack is casused by not only CSI measurement but also UCI (Uplink Channel Information), which includes CSI measurement, HARQ ACK/NACK, and so on. In this contribution, we would like to share potential issue occuered by UCI feedback.

- Discussions

2.1 UL capacity

LAA is mainly used for CA and then unlicensed band carriers are used as SCells. In unlisenced band, wider bandwidth is usually open for various systems. In Japan, ARIB defines 255 MHz from 5.47GHz to 5.725GHz as a shared band among various systems. LAA can activate many component carriers simultaneously for high data rate DL performance, but UL capactity of PCell can go down due to a huge amount of UCI transmission on UL. UCI both PCell and SCells are transmited on UL resource of the PCell carrier in the current CA structure. Especially, if macro cell is selected as PCell for LAA, many SCells are deployed inside the single macro cell.

Observation 1: LAA may activate many SCells on unlicensed band carriers for high DL performance, but it occuers that UL capactity of the PCell can go down because UCI both PCell ans SCells are transmited on UL resource of the PCell cerrier in now CA structure.

2.2 UCI transmission on unlicensed band

As discussed above, LAA might cause the shortange of UL resource. If UCI transmission can be performed on unlicensed band, we believe that UL capacity problem can be reduced. In this section, the comparesion of two cases is discussed, case1: UCI transmission on licensed band case and case2: UCI transmission on unlicensed band.

Case1: UCI transmission on licensed band

By using licensed band, UCI transmission is performed at the pre-dfined timing. This improves the DL performance because shcduling gain becomes larger and more reliable retransmission is realized even if the shortage of RB on UL happens. If the number of SCells deployed within a Pcell becomes larger, this capacity problem becomes more serious as discussed above.

Case2: UCI transmission on unlicensed band

By using unlicensed band for UCI transmission, any additional load on licensed (primaly) band never happens and UL capacity problem is reduced. However, on unlicensed band, all of entities should perform LBT (Listen-Before-Talk) to solve co-existence with othe system, e.g. Wi-Fi system. As a result, unpreditable delay causes UCI transmission delay [2][3]. Even if eNB assings the best condition channel for UE, UE may not transmit the data at the assigned channel by eNB. Moreover, unlicensed DL performance is degraded because both DL and UL share the time and frequency resource.

Table 1 shows the comparison between above two cases. Based on above analsys, when the load on LTE is not so high, UCI should be transmitted on PCell UL. When the load becomes higher and UL capacity on licensed band is negatively affected, unlicensed band UL should be utilized. The number of RB on licensed/unlicensed band allocated for UCI transmission should be designed to be configurable according with the load on LAA.

Table 1: Pros. and Cons.

|  | Pros. | Cons. |
| --- | --- | --- |
| Case 1 : Licensed band | - UCI transmission is performed at the required timing and the better CA performance might be realized. | - Severe UL capacity problem may happen. |
| Case 2 : Unlicensed band | - UL capacity problem can be reduced. | - UCI transmission could be delayed. - Transmission opportunity of downlink is reduced due to UCI report on unlicensed UL. |

Proposal1: USI transmission on unlicensed band UL should be permitted.

Proposal2: Which band each RB is allocated for USI transmission, licensed or unlicensed band, should be configurable according with the load on LTE system.

3. Conclusion

In this contribution, we discuss UL capacity issue according with UCI transmission for LAA.

Observation 1: LAA may activate many SCells on unlicensed band carriers for high DL performance, but it occuers that UL capactity of the PCell can go down because UCI both PCell ans SCells are transmited on UL resource of the PCell cerrier in now CA structure.

Proposal1: USI transmission on unlicensed band UL should be permitted.

Proposal2: Which band each RB is allocated for USI transmission, licensed or unlicensed band, should be configurable according with the load on LTE system.

4. References

- Draft_Minutes_report_RAN1#79_v020

- R1-144970, NTT DOCOMO, “Views on issues related to LAA UL”

- R1-144830, ZTE, “Analysis of LAA UL enhancement”