3GPP TSG RAN WG1 Meeting #80                                      	     R1-150300

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.3.1

Source:	            NEC

Title:               CSI Feedback Enhancement for Elevation Beamforming/FD-MIMO

Document for:	   Discussion and Decision

- Introduction

As referred in [1], 2D antenna array with 8 or more TXRUs will be used to exploit the potential elevation beamforming benefit. In order to get the 3D channel information, UE needs to measure and report the CSI information both in azimuth and elevation. Consequently, the enhancement of CSI-RS and CSI feedback is necessary.

- Discussion

As the 2D antenna array introduced, two CSI-RS enhancement schemes are hotly discussed [2]. Due to the introduction of the additional elevation direction, simply reusing the existing CSI feedback scheme cannot make full use of the 3D spatial characteristic, so 3D channel state information measurement and feedback should be considered [3][4].

  - Additional vertical CSI feedback

In the precoded CSI-RS scheme, CSI-RS can be beamformed by different vertical vectors, UE needs to report at least one horizontal channel information coupled with the indices corresponding to the best one or several vertical beams. Then eNB can obtain the full channel information directly.

Alternatively, the 2D antenna array can be virtualized to one column and one row antenna ports by horizontal weighting factor and vertical weighting factor respectively. Based on this structure, the CSI-RS overhead can be reduced, and two CSI processes can be configured for the two independent CSI-RS in azimuth and elevation. Straightforwardly, the vertical and horizontal channel information can be measured and reported separately, and then eNB computes a 3D precoder by kronecker product based on the two independent CSI reports.

As mentioned above, the additional vertical CSI information should be considered in CSI feedback enhancement. The CSI information includes PMI, RI and CQI. For vertical RI information, as the elevation angle spread is small, the vertical domain rank number can be assumed to be limited as, e.g. 1, so there is no need to report the vertical RI. For vertical CQI information, it can be jointly calculated with the horizontal one to form a 3D-CQI and reported without additional feedback bit. Therefore, we mainly focus on the additional vertical PMI feedback in this contribution.

Proposal 1: Vertical PMI feedback should be introduced in CSI feedback enhancement.

  - Vertical PMI feedback enhancement

From the performance evaluation of different vertical codebook sizes shown in the simulation result of our companion’s contribution [2], we can see that throughput decreases significantly as the vertical codebook size reduces, especially for 4DFT vertical codebook. To obtain sufficient performance, the vertical codebook size should be large enough. However, the payload size is limited for CSI report in the existing feedback scheme [5], especially for the report on PUCCH. So considering the trade-off between the system performance and the CSI feedback payload limitation, the CSI feedback enhancement should be considered carefully.

Proposal 2: CSI feedback enhancement should be considered carefully considering the trade-off between the performance and the feedback payload size, especially for the report on PUCCH.

Based on the simulation assumption in Table 3 of Appendix, we analyse the average feedback intervals for RI, horizontal wideband PMI (PMI1) and vertical PMI (PMIv). As the difference between azimuth and elevation, such as the angle spread, user distribution, user mobility and so on, the channel characteristic on vertical domain is different from the horizontal one. One of the differences is that the necessary update interval of vertical PMI is much longer than that in horizontal domain, as shown in Table 1.

Table 1 Average update interval

| CSI | Average update interval (ms) |
| --- | --- |
| RI | 588.7 |
| PMI1 | 139.2 |
| PMIv | 858.6 |

We also analyse the variable probability of vertical PMI in some conditions. As shown in Table 2, we list the unchanged probability of vertical PMI (PMIv) when horizontal wideband PMI (PMI1) changed/unchanged, RI changed, and both RI and PMI1 changed. We can observe that vertical PMI (PMIv) usually remains unchanged, especially when horizontal wideband PMI (PMI1) unchanged. Therefore, it’s unnecessary to update vertical PMI when horizontal channel information (e.g. horizontal wideband PMI) unchanged.

Table 2 Statistical variable probability

| Conditions | PMIv unchanged probability | PMIv changed probability |
| --- | --- | --- |
| PMI1 unchanged | 97.44% | 2.56% |
| PMI1 changed | 87.75% | 12.25% |
| RI changed | 83.32% | 16.68% |
| Both RI and PMI1 changed | 83.08% | 16.92% |

Observation: The necessary update interval of vertical PMI feedback is much longer than that of horizontal domain. And the vertical PMI usually remains unchanged when horizontal wideband PMI unchanged.

In current feedback scheme [5], the maximum bit number for CQI feedback on PUCCH is 11, and there are only 5 bits of RI feedback for higher reliability requirement. Since there are redundant payload bits in many CSI feedback types, especially for RI feedback, so we can use these redundant bits for vertical PMI feedback within the payload.

In addition, the necessary update interval of vertical PMI is much longer, and on the other hand, the fixed period of vertical PMI feedback may not achieve the optimal performance. So the vertical PMI can be reported when necessary, e.g. with an indicator, similar as the feedback of horizontal wideband channel information.

Proposal 3: Based on current feedback scheme, vertical PMI can be reported when necessary with an indicator or reported on redundant feedback payload.

- Conclusion

In this contribution, the potential CSI feedback enhancement for Elevation Beamforming and Full-Dimension MIMO is discussed, and we observe:

Observation: The necessary update interval of vertical PMI feedback is much longer than that of horizontal domain. And the vertical PMI usually remains unchanged when horizontal wideband PMI unchanged.

Based on the discussion of vertical PMI feedback, we propose:

Proposal 1: Vertical PMI feedback should be introduced in CSI feedback enhancement.

Proposal 2: CSI feedback enhancement should be considered carefully considering the trade-off between the performance and the feedback payload size, especially for the report on PUCCH.

Proposal 3: Based on current feedback scheme, vertical PMI can be reported when necessary with an indicator or reported on redundant feedback payload.

- References

- RP-141644, "New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE", Samsung, Nokia Networks, 3GPP TSG RAN#65, Edinburgh, Scotland, 9th - 12th September, 2014

- R1-150299, "Discussion on CSI-RS enhancement", NEC, 3GPP TSG RAN1#80, Athens, Greece, 9th - 13rd February, 2015

- R1-144923, "Potential CSI-RS and CSI feedback enhancements for EBF/FD-MIMO", ETRI, 3GPP TSG RAN1#79, San Francisco, USA, 17th - 21st November, 2014

- R1-144954, "CSI-RS Design and CSI Reporting for EB/FD-MIMO", ETRI, 3GPP TSG RAN1#79, San Francisco, USA, 17th - 21st November, 2014

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"

- Chairman’s Notes 3GPP TSG RAN1#78bis, Ljubljana, Slovenia, 6th - 10th October, 2014

- R1-144444, "WF on simulation assumption", 3GPP TSG RAN1#78bis, Ljubljana, Slovenia, 6th - 10th October, 2014

- Appendix

Table 3 Simulation assumption

| Duplex | FDD |
| --- | --- |
| Traffic model | Full buffer |
| TXRU virtualization model | The full-connection model |
| Cell association weights | CRS port 0 associate with the first single element |
| Cell association method | RSRP on CRS port 0 |
| CSI feedback period | 5ms |
| Wrapping method | Geographical distance based |
| Channel estimation | Ideal |
| Feedback | PUSCH 3-1 |
| Number of UE receive antennas | 2 |
| Rank | SU-MIMO Rank 1/2 adaption |
| Others | Based on [6][7] |