3GPP TSG RAN WG1 Meeting #80                                                                             	               R1-150310

Athens, Greece, 9th – 13th February 2015

Source:	Panasonic

Title: 	Evaluation on PDSCH repetition in CE MTC

Agenda Item:		7.2.1.2.1

Document for:	Discussion and Decision

Introduction

As one of the objectives in Rel.13 MTC WID [1], a relative 15 dB coverage improvement in FDD is required for a new Rel-13 low complexity UE category/type for MTC operation. With the repetition over multiple subframes, the equivalent SINR after combining of repetitions would be increased to obtain the coverage enhancement in negative SNR region. Various methods to enhance the repetition gain could be adopted at both transmitter side (e.g. frequency diversity) and receiver side (e.g. channel estimation enhancement). In this document, we evaluated multiple subframe symbol combining gain. In our companion contribution [2], we show the evaluation for coverage enhancement for PUSCH.

Evaluation results

We have evaluated the required BLER@10%  of 4- subframe symbol combining gain as shown in Table 1 (detailed parameters and results could be found in Appendix).

As also shown for uplink in Fig. 2 of our companion contribution [2], in order to improve the reliability of channel estimation in negative SNR region, combining RS (e.g. CRS) and data from adjacent subframes is used to improve the equivalent SINR, as the channel variance in time would be rather small in MTC scenario (e.g. EPA 1 Hz). In our evaluation, the receiver combines the RS from multiple subframes and the data REs from the multiple subframes are combined respectively. The combined RS is used to obtain the improved channel estimation. Then, the combined data REs are demodulated/equalized using the combined RS. Thereby, the receiver is not required to have an additional buffer for data in order to include RSs from later subframes. The alternative would be to use only RS from past subframes, but it is difficult to obtain improved channel estimation in case of frequency hopping. Therefore, we prefer the accumulation/combination of RS and data before the demodulation/equalization.

From the evaluation results, we can see that 0.7 dB to 1.7 dB gain at BLER=10% can be obtained by 4-subframe symbol combining compared with no subframe combining depending on the number of PDSCH repetitions. According to the results, the gain of multiple subframe symbol combining increases with the number of PDSCH repetitions, i.e. the multiple subframe symbol combining is more beneficial at higher level PDSCH repetitions.

Table 1 Required SNR at BLER=10% for PDSCH

| Number of PDSCH repetitions | 1 | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No subframe combining i.e. single subframe channel estimation | -1.0 dB | -3.1 dB | -4.8 dB | -6.5 dB | -8.0 dB | -9.7 dB | -11.4 dB |
| 4 subframe symbol combining i.e. channel estimation over 4 subframes | - | -3.8 dB | -6.0 dB | -7.9 dB | -9.7 dB | -11.4 dB | -13.1 dB |
| Gain of multiple subframe symbol combining |  | 0.7 dB | 1.2 dB | 1.4 dB | 1.7 dB | 1.7 dB | 1.7 dB |

Observation: 4 subframes symbol combining improves the PDSCH by 0.7-1.7 dB.

When frequency hopping is introduced, if frequency hopping of 6 PRBs occurs more often than every 4 ms, efficient multiple subframe channel estimation is not possible, and a disproportionate amount of time would be lost due to retuning. Therefore, we propose following.

Proposal: Frequency retuning for hopping should not occur more often than every 4 ms.

Conclusion

In this contribution, we evaluated the performance gain with multiple-subframe channel estimation in PDSCH repetition. We have the following observation:

Observation: 4 subframe symbol combining improves the PDSCH by 0.7-1.7 dB.

Based on above observation, we proposal following:

Proposal: Frequency retuning for hopping should not occur more often than every 4 ms.

- Reference

- RP-141660,	“New WI proposal: Further LTE Physical Layer Enhancements for MTC,” Ericsson, Nokia Networks

- R1-150312,	“Discussion and performance evaluation on PUSCH coverage enhancement”, Panasonic

- Appendix A: Simulation parameters

| Parameters | Value |
| --- | --- |
| Number of antennas | 2×1 with low correlation |
| Transmission mode | 2 |
| Number of UE | 1 |
| Number of repetition | 1, 2, 4, 8, 16, 32, 64, 128 |
| Channel model | EPA 1 Hz |
| Frame format | PDSCH 6 RB |
| MCS | MCS 0 |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| Timing error | Ideal |
| Residual frequency offset | 20 Hz |
| Channel estimation | CRS-based channel estimation averaged over n-subframe |
| Equalization | MMSE-SFBC |

- Appendix B: Simulation results

Figure 1 PDSCH BLER with 1-subframe channel estimation

Figure 2 PDSCH BLER with 4-subframe symbol combining

(Note: 2-subframe if 2-repetition)