3GPP TSG RAN WG1 Meeting #80                                                                             	                      R1-150312

Athens, Greece, 9th – 13th February 2015

Source:	Panasonic

Title: 	Discussion and performance evaluation on PUSCH coverage enhancement

Agenda Item:		7.2.1.2.1

Document for:	Discussion and Decision

Introduction

New WID for MTC was agreed in RAN #65 [1]. One of the objectives is coverage enhancement corresponding to 15 dB for FDD for the Rel.13 low complexity UE category/type. In order to achieve 15dB coverage enhancement, repetition over multiple subframes is one of necessary solutions. On the top of plain repetition, various PUSCH coverage enhancement techniques can be considered in order to reduce the required number of repetitions and mitigate the loss by repetition. This contribution shows some link level simulation results on following PUSCH coverage enhancement techniques.

- Frequency hopping

- Symbol level combining including multiple subframe channel estimation

- Multiple subframe code spreading

Discussion

Frequency hopping

There are two frequency hopping schemes. One is frequency hopping within 1.4 MHz. The other is frequency hopping of 1.4 MHz over the system bandwidth (e.g. 20 MHz). Following three options were evaluated:

- Repetition without frequency hopping (Fig. 1 (a))

- Repetition with frequency hopping within 1.4MHz (Fig. 1 (b))

- Repetition with frequency hopping over the system bandwidth (Fig. 1 (c))

(a) Repetition without frequency hopping.                   (b) Repetition with frequency hopping within 1.4MHz.

(c) Repetition with frequency hopping over the system bandwidth.

Fig.1		Frequency hopping options.

Symbol level combining including multiple subframe channel estimation

Fig. 2 illustrates concept of symbol level

combining including multiple subframe channel estimation. Up to NSF(=4 or 8) subframes are coherently combined by symbol level combining before demodulation. After symbol level combining, channel estimation is carried out by using combined received reference symbol. Therefore, the number of subframes for multiple subframe channel estimation is equal to NSF. Then, FDE is carried out by using channel estimate and combined received data symbol. After FDE, NRep/NSF equalized received signals are simply combined to input the SC-FDMA demodulator. Note that assuming QPSK modulation, above simple combining after channel estimation and FDE is equivalent to the energy level (or LLR) combining.

Fig.2 Symbol level combining including multiple subframe channel estimation.

Multiple subframe code spreading

Multiple subframe code spreading can be applied on the top of repetition. The despreading structure is almost the same as the symbol level combining. Only the difference is to apply subframe-level code spreading and despreading. The detail structure can be found in our companion contribution [2].

Simulation results

In this section, we evaluate the PUSCH link level performance of above coverage enhancement techniques. The detailed parameters are listed in the Appendix A and detailed simulation results with BLER performance are shown in Appendix B.

Table 1 summarizes the required SINR for achieving BLER=101 for above coverage enhancement techniques. Table 2 shows the required number of repetitions to obtain required coverage gain of 6, 12, and 18 dB, which is discussed as one of useful indices in [3].

In Appendix B,

- Figure 3 plots the BLER performance of PUSCH with number NRep of repetition as a parameter. Plain repetition without frequency hopping and symbol level combining is used.

- Figure 4 plots the BLER performance of PUSCH with frequency hopping. Frequency hopping within 1.4MHz or over the system bandwidth with hopping period Nhop=4 is applied, where the retune/switching time for frequency hopping is not considered in the evaluation.

- Figs. 5 and 6 plot the BLER performance of PUSCH with symbol level combining including multiple subframe channel estimation. Symbol level combining with NSF=4 or 8 subframes is applied. Symbol level combining with frequency hopping is also evaluated, where hopping period Nhop=NSF is applied.

Note that the BLER performance of multiple subframe channel estimation can be found in our companion contribution [2].

Table 1	Required SINR for achieving BLER=101

| Number of repetitions |  | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Plain repetition | No hopping | -1.6 dB | -3.4dB | -5.5dB | -7.4dB | -9.3dB | -11.2dB |
|  | Hopping within 1.4MHz | -1.8 dB | -3.6dB | -5.5dB | -7.4dB | -9.5dB | -11.4dB |
|  | Hopping over system bandwidth | -4.0dB | -5.9dB | -7.6dB | -9.5dB | -11.2dB | -13.1dB |
| Symbol level combining (4 subframes) | No hopping | -3.0dB | -5.0dB | -7.4dB | -9.5dB | -11.8dB | -13.8dB |
|  | Hopping over system bandwidth | - | -7.6dB | -9.5dB | -11.7dB | -13.7dB | -15.7dB |
| Symbol level combining (8 subframes) | No hopping | - | -5.3dB | -7.9dB | -10.2dB | -12.6dB | -14.7dB |
|  | Hopping over system bandwidth | - | - | -10.1dB | -12.3dB | -14.6dB | -16.5dB |
| Multiple subframe code spreading (4 subframes) without near-far effect | Hopping over system bandwidth | - | -7.6dB | -9.5dB | -11.7dB | -13.7dB | -15.7dB |
| Multiple subframe code spreading (4 subframes) with near-far effect (6dB) | Hopping over system bandwidth | - | -7.6dB | -9.5dB | -11.7dB | -13.7dB | -15.7dB |
| Multiple subframe code spreading (4 subframes) with near-far effect (12dB) | Hopping over system bandwidth | - | -7.1dB | -9.1dB | -11.3dB | -13.3dB | -15.3dB |

Table 2	Required number of repetitions

| Required coverage gain |  | 6dB | 12dB | 18dB |
| --- | --- | --- | --- | --- |
| Plain repetition | No hopping | 8 | 64 | >128 |
|  | Hopping within 1.4MHz | 8 | 64 | >128 |
|  | Hopping over system bandwidth | 4 | 32 | >128 |
| Symbol level combining (4 subframes) | No hopping | 4 | 32 | >128 |
|  | Hopping over system bandwidth | - | 16 | 128 |
| Symbol level combining (8 subframes) | No hopping | - | 32 | >128 |
|  | Hopping over system bandwidth | - | 16 | 64~128 |

Frequency hopping

From Table 1 and Fig. 4, frequency hopping within 1.4MHz does not achieve performance gain. This is because frequency hopping within 1.4MHz cannot obtain any frequency diversity gain in a substantially flat channel. On the other hand, frequency hopping over the system bandwidth achieves significant better BLER performance than without frequency hopping. This is because frequency hopping of 1.4 MHz within the system bandwidth can obtain much frequency diversity gain. Based on Table 1 frequency hopping over the system bandwidth has around 2 (1.9~2.5) dB coverage improvement gain from plain repetition.

Observation 1: Frequency hopping within 1.4MHz does not achieve performance gain.

Observation 2: Frequency hopping over the system bandwidth has around 2dB coverage improvement gain from plain repetition.

Symbol level combining including multiple subframe channel estimation

It can be seen from Table 1 and Figs. 5 and 6 that symbol level combining can improve the BLER performance. With 4-subframe symbol level combining, an SINR reduction of as much as 1.4~2.6 dB can be achieved compared to plain repetition. With 8-subframe symbol level combining, an SNR reduction of as much as 1.9~3.5dB can be achieved compared to plain repetition. If both frequency hopping and symbol level combining are applied, 4.0~4.5 (4.6~5.3) dB coverage improvement gain can be obtained with 4 (8) subframes.

Observation 3: Symbol level combining with 4(8) subframes improves the coverage by 1.4~2.6dB (1.9~3.5dB).

Observation 4: If both frequency hopping and symbol level combining are applied, 4.0~4.5 (4.6~5.3) dB coverage improvement gain can be obtained with 4 (8) subframes.

Based on above simulation results, we propose following.

Proposal: Coverage enhancement uses frequency hopping and multiple subframe symbol level combining.

Multiple subframe code spreading

Although detailed simulation results are shown in [2], multiple subframe code spreading can achieve almost the same coverage gain as by the symbol level combining even in a near-far scenario.

Conclusion

In this contribution, we discussed coverage enhancement techniques on top of the repetition and showed some simulation results. Based on the discussion presented, we summarize our views through the following observations and proposal:

Observation 1: Frequency hopping within 1.4MHz does not achieve performance gain.

Observation 2: Frequency hopping over the system bandwidth has around 2dB coverage improvement gain from plain repetition.

Observation 3: Symbol level combining with 4(8) subframes improves the coverage by 1.4~2.6dB (1.9~3.5dB).

Observation 4: If both frequency hopping and symbol level combining are applied, 4.0~4.5 (4.6~5.3) dB coverage improvement gain can be obtained with 4 (8) subframes.

Observation 5: Multiple subframe code spreading can achieve almost the same coverage gain as by the symbol level combining.

Proposal: Coverage enhancement uses frequency hopping and multiple subframe symbol level combining.

- Reference

- RP-141660,	“New WI proposal: Further LTE Physical Layer Enhancements for MTC,” Ericsson, Nokia Networks

- R1-150311,	“Multiple subframe code spreading for MTC UEs,” Panasonic

- R1-150068, “High Level PUSCH Simulation Assumptions,” Sierra Wireless

- Appendix A: Simulation parameters

| Parameters | Value |
| --- | --- |
| Number of antennas | 1×2 with low correlation |
| Number of multiplexed UEs (signals) for multiple subframe code spreading | 1, 2 |
| Number of repetition | NRep=1, 4, 8, 16, 32, 64, 128 |
| Channel model | EPA (Doppler frequency, fD=1Hz) |
| Frame format | PUSCH 1RB |
| MCS | MCS5 |
| Carrier frequency | 2GHz |
| System bandwidth | 10MHz |
| Timing error | Ideal |
| Transmit power control error for multiple subframe code spreading | Average received power of interfering UE is 0 to 12 dB larger than that of evaluated UE. |
| Residual frequency offset | 20Hz |
| Channel estimation | Realistic single-subframe channel estimation, realistic cross-subframe channel estimation (Averaging over NSF subframes) |
| Equalization | MMSE-FDE |

- Appendix B: Simulation results

Fig.3 Plain repetition.

(a)	Frequency hopping within 1.4MHz                           (b) Frequency hopping over the system bandwidth

Fig.4 Plain repetition with frequency hopping.

(a)	Without frequency hopping                             (b) With frequency hopping over the system bandwidth

Fig.5 Repetition with 4-subframe symbol level combining.

(a)	Without frequency hopping                           (b) With frequency hopping over the system bandwidth

Fig.6 Repetition with 8-subframe symbol level combining.