3GPP TSG-RAN WG1 Meeting #80	R1-150331

Athens, Greece, 9th – 13st February 2015

Agenda Item:	6.2.1

Source: 	Ericsson

Title:  	Uplink coverage evaluation for small data applications

Document for:	Discussion

# Introduction

At RAN#65, a study on ‘Small Data Transmission (SDT) Enhancements for UMTS’ was approved as one of the topics to be studied as part of 3GPP Release 13 [1]. One aspect to be considered is coverage of small data transmissions, and during RAN1#79 a comprehensive evaluation of the Maximum Coupling Loss (MCL) for different physical channels was performed [2-9]; see Table 1 for a summary of some of the results. Three main bottleneck areas were identified from this evaluation, namely PCH over S-CCPCH, PRACH preamble and EUL. In this contribution, some further uplink coverage evaluations are presented for EUL and HS-DPCCH. These results are then used as inputs for proposing potential improvements in the companion paper [10].

Table 1	Summary of MCL values for the agreed reference scenario.

|  | Cell search (P-SCH, S-SCH) | BCH (S-CCPCH) | Paging (PICH, S-CCPCH) | PRACH preamble | AICH | EUL (DPCCH, E-DPCCH, E-DPDCH) | HS-SCCH | HS-PDSCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB], Ped A 1 Hz | 154 | 150 | 143 |  | 145 | 141 | 144 | 160 |
| MCL [dB], AWGN | 152 | 152 | 149 | 143 | 152 | 146 | 150 | 158 |

# Discussion

In this section we will discuss how costly the different uplink physical channels are in terms of power for a typical SDT coverage limited scenario. This will give some insight into what can be done in order to enhance the coverage.

The first evaluation addresses EUL. The link level simulation parameters used can be found in Annex A. In particular, a TBS size of 120 bits using a 10 ms TTI with varying Aed (βed) and Aec (βec), maximum of 8 transmissions (7 retransmissions), inner-loop power control turned off, and realistic E-DPCCH detection have been assumed. Figure 1 shows the required Ec/N0 for 1% residual E-DPDCH BLER for different values of the E-DPCCH and E-DPDCH gain factors. For a fixed Aec, the required Ec/N0 decreases with increased Aed until the ‘optimal’ Aed value is reached and thereafter the required Ec/N0 starts to increase with increasing Aed, resulting in a bowl shaped curve. One observation is that the bottom of the bowl is relatively flat, indicating that the exact power setting is rather insensitive. Nevertheless, the results still show that it is very important to distribute the available power in a good way (optimize the gain value setting) in order to maximize coverage. The optimal gain factor setting for this example is Aed=21/15 and Aec=19/15 resulting in a minimum required Ec/N0 of roughly -23.75 dB, which translates into an MCL of ~140 dB (see Annex B).

A more detailed look at the optimal gain setting from these results can be found in Table 2. It can be seen that the power cost of the different channels is rather balanced, i.e. the power consumption for the different channels are of the same order. This also means that E-DPCCH constitutes a rather significant part of the total power. In this example, an MCL gain of roughly 2 dB can be achieved by putting the E-DPCCH power on DPCCH and E-DPDCH instead. Furthermore, using additional retransmissions (e.g. 16 transmissions instead of 8) would lower the E-DPDCH and potentially DPCCH power further and make the E-DPCCH even more costly.

The second performance evaluation considers HS-DPCCH. Since HS-DPCCH uses 2 ms TTI, it is likely that HS-DPCCH requires more power than E-DPCCH employing 10 ms TTI. Figure 2 shows the probability of missed ACKs as a function of Ec/N0 for PA 1 Hz with the ACK/NACK repetition factor set to 1 or 4. Here DPCCH and HS-DPCCH are transmitted simultaneously and Ahs=1. It can be seen that the cost of the HS-DPCCH is in this case roughly -21 dB and -26 dB when the repetition factor is set to 1 and 4, respectively. Hence, in comparison with the EUL results in Figure 1, it seems that an ACK/NACK repetition factor larger than 4 is needed to make the cost of HS-DPCCH as low as the other UL physical channels. Current specifications allow a maximum ACK/NACK repetition factor of 4, so changes would be required to operate with even larger repetition factors. Also, using a very large repetition factor would negatively impact the system in different ways, for example, imposing scheduling restrictions. A final note is that the CQI performance needs to be ensured as well, which can be affected in similar ways as it happens with the ACK/NACK performance, e.g. CQI repetition and feedback cycle factors.

Figure 1	Required Ec/N0 for 1% residual E-DPDCH BLER as a function of Aed for various Aec.

Table 2	Summary of the cost in terms of power for different channels.

| Required Ec/N0 | Aed | Aec | DPCCH power | E-DPCCH power | E-DPDCH power | Gain by removing E-DPCCH |
| --- | --- | --- | --- | --- | --- | --- |
| -23.75 dB | 21/15 | 19/15 | -30.34 dB | -28.29 dB | -27.42 dB | 1.88 dB |

Figure 2	Probability of missed HS-DPCCH ACK detection as a function of Ec/N0 for PA 1 Hz using repetition factor 1 and 4.

# Conclusion

It has been agreed to study Small Data Transmission Enhancements for UMTS as part of 3GPP Release 13. One key area to be investigated in the study is the coverage aspects of small data transmissions.

Simulations of some of the uplink physical channels have been performed for the agreed reference scenario in order to investigate the impact of gain factor settings, as well as to find out the cost in terms of power for the different channels in a typical coverage limited SDT scenario.

The results indicate that all uplink physical channels are costly in terms of power and it would be beneficial to focus the power on as few channels as possible at the same time. Hence, removing information or transmitting in a time-multiplexed fashion would be advantageous. This will be further discussed in [10].

Proposal: Use the companion text proposal [11] to capture the ideas discussed in this contribution in the TR [12].

# References

RP-141711, “New SID: Study on Small Data Transmission Enhancements for UMTS”, Ericsson, Huawei, HiSilicon, Telefonica S.A., China Unicom, Nokia Networks

R1-145026, “Cell search coverage evaluation”, Ericsson.

R1-145027, “System information acquisition (BCH) coverage evaluation”, Ericsson.

R1-145028, “Paging channels (PICH and PCH) coverage evaluation”, Ericsson.

R1-145029, “PRACH preamble coverage evaluation”, Ericsson.

R1-145030, “Acquisition Indicator channel (AICH) coverage evaluation”, Ericsson.

R1-145031, “Enhanced Uplink (EUL) coverage evaluation”, Ericsson.

R1-145032, “HS-SCCH and HS-PDSCH coverage evaluation”, Ericsson.

R1-145134, “Evaluation results on coverage enhancements”, Huawei, HiSilicon.

R1-150330, “Enhanced uplink (EUL) coverage improvements”, Ericsson

R1-150336, “Text Proposal on Uplink coverage evaluation for small data applications”, Ericsson

TR 25.705, “Study on Small data transmission enhancements for UMTS”

# Annex A

At RAN1#78bis, the following was agreed:

Relative coverage of all relevant channels shall be investigated, by calculating the maximum coupling loss for each channel in the reference scenario outlined in Table 1 below.

Table 3: Reference scenario parameters.

| Parameter | Assumption |
| --- | --- |
| TBS | 120 bits (HS, EUL) |
| UE capability | Rel-12, supporting any legacy feature improving coverage |
| Number of UE antennas | 1 antenna |
| Number of Node B antennas | 2 antennas (uncorrelated) |
| Maximum UE carrier transmit power | 23 dBm at antenna connector |
| Maximum Node B carrier transmit power | 43 dBm at antenna connector |
| UE receiver noise figure | 9 dB |
| Node B receiver noise figure | 5 dB |
| Downlink common channel power settings | P-CPICH: -10 dB from max carrier power P-SCH: -12 dB S-SCH: -13.5 dB P-CCPCH (BCH): -12 dB For other channels reasonable power settings can be proposed. |
| DL inter-cell interference | No inter-cell interference |
| Soft/softer handover | No soft/softer handover |
| Downlink OCNS | OCNS added to fill up DL carrier power |
| Uplink rise-over-thermal (RoT) operation point | 10 dB |
| Channel model | Ped A 1 Hz Doppler spread, AWGN static channel |
| Carrier frequency | 2.0 GHz |
| Frequency error | 20 kHz, 1 kHz optional, in cell search simulations 0 otherwise |
| Beta values | To be provided with evaluation results |

In addition, the parameters shown in Table 2 have been used in the link evaluations.

Table 4	Additional EUL link simulation parameters.

| Parameter | Assumption |
| --- | --- |
| Max number of E-DPDCH transmissions | 8 |
| Inner-loop power control | Off |
| βed | Varying |
| βec | Varying |
| Channel estimation | Realistic with time-averaging determined from Doppler spread |
| E-DPCCH detection | Realistic |

# Annex B

According to the simulation assumptions the assumed noise figure in uplink is 5 dB, and a RoT of 10 dB is to be assumed. Assuming a thermal noise density at 290K of -174 dBm/Hz we get a total received thermal noise power N0 of

N0=-174+10log3840000+5+10=-93.2 [dBm].

Given the assumed maximum UE transmission power of 23 dBm, the maximum coupling loss to ensure a certain received EcN0 per antenna can be calculated as

MCL=23-EcN0-93.2=116.2-EcN0 [dB].