3GPP TSG RAN WG1 Meeting #80	R1-150381

# Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.3.1

Source: 	Samsung

Title: 	Discussions on FD-MIMO Codebook Enhancements

Document for:	Discussion and Decision

# Introduction

Potential performance benefits of FD-MIMO have been demonstrated with ideal CSI [1], in which 2x to 3x performance gain in average-cell throughput has been observed. This contribution presents initial analysis results for performance benefits of FD-MIMO systems with subarray-partition architecture, along with alternative methods for FD-MIMO codebook enhancements.

# Additional PMI feedback for Elevation Domain – Kronecker Product

Performance benefits of FD-MIMO systems are achieved primarily with 3D beamforming and MU-MIMO. To facilitate 3D beamforming operation, Rel.12 PMI feedback is not sufficient as it was mainly designed for the azimuth domain CSI.

One alternative for the FD-MIMO CSI feedback could be to introduce additional PMI feedback components for the elevation domain, relying on a Kronecker product structure. Consider a 2D co-polarized uniform rectangular array comprising N columns and MTXRU rows. Then, according to the derivation in the Appendix, the eigenvector of a channel covariance matrix of a channel with a single dominant path can be represented as a Kronecker product of two DFT vectors, namely, , where:

- is a MTXRU x 1 elevation channel vector defined as:

with ; and

- is a N x 1 azimuth channel vector defined as:

with .

Based on this observation, a codebook based on the Kronecker product of two DFT vectors can be constructed. It is intended that the two DFT vectors respectively quantize conjugates of the elevation and the azimuth channel vectors and  that construct the rank-1 channel direction vector . Note that this argument assumes a channel with one dominant path which may or may not happen in practice. However, it is expected that the above construction generally offer reasonable performance.

For the 2D x-polarized uniform rectangular array, an additional component, co-phase,  , should also be considered for the codebook design. For example, for (MTXRU, N, P) = (2, 4, 2) subarray-partition configuration, Rel-10 8-Tx codebook can be used for quantizing the azimuth channels, and 2-Tx codebook can be used for the elevation channels. In this case, the composite precoding vector would be represented as , where  is a length-4 oversampled DFT vector with an oversampling factor of 8; and  is a length-2 oversampled DFT vector with an oversampling factor of 2. In this case, CQI should be derived based on the KP composite precoder.

In the following, non-full-buffer MU-MIMO SLS results are presented for the 3D-UMa channels to demonstrate the benefits of the KP codebook in the FD-MIMO system with respect to the baseline 8-Tx systems. For the FD-MIMO systems, 1D subarray-partition architecture is assumed with MTXRU=1, 2, 4 and (N, P) = (4, 2). For the KP component codebooks, Rel-8 2-Tx and 4-Tx codebooks are used for  and Rel-10 8-Tx codebook is used for . Cell association antenna pattern is approximated by one-TXRU pattern, and SLNR precoding and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. The rest of the simulation assumption is according to [2].

Heavily-loaded system (~RU 70%) analysis:

With 32 TXRUs, RU has been reduced by 28% for the same inter-arrival time of 0.15 sec, and 5% UPT and 50% UPT has been increased by 206% and 126% respectively. With 16 TXRUs, RU has been reduced by 9% for the same inter-arrival time, and 5% UPT and 50% UPT has been increased by 37% and 25% respectively.

| MTXRU | Q | Arrival Rate | Avg UPT | 5% UPT | 50% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 0.15 | 11.016 | 2.317 | 8.488 | 70.0% |
| 2 | 16 | 0.15 | 13.109 | 3.167 | 10.616 | 63.9% |
| % gain wrt Q=8 |  |  | 119% | 137% | 125% | 91% |
| 4 | 32 | 0.15 | 21.334 | 7.09 | 19.171 | 51% |
| % gain wrt Q=8 |  |  | 194% | 306% | 226% | 72% |

Medium-loaded system (~RU 50%) analysis:

With 32 TXRUs, RU has been reduced by 28% for the same inter-arrival time of 0.25 sec, and 5% UPT and 50% UPT has been increased by 109% and 67% respectively. With 16 TXRUs, RU has been reduced by 6% for the same inter-arrival time, and 5% UPT and 50% UPT has been increased by 16% and 21% respectively.

| MTXRU | Q | Inter-Arrival | Avg UPT | 5% UPT | 50% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 0.25 | 19.12 | 5.00 | 15.69 | 47.5% |
| 2 | 16 | 0.25 | 21.16 | 5.80 | 18.96 | 44.8% |
| % gain wrt Q=8 |  |  | 111% | 116% | 121% | 94% |
| 4 | 32 | 0.25 | 28.74 | 10.46 | 26.25 | 34% |
| % gain wrt Q=8 |  |  | 150% | 209% | 167% | 72% |

Lightly-loaded system (~RU 20%) analysis:

With 32 TXRUs, RU has been reduced by 25% for the same arrival rate of 0.6 packets/sec, and 5% UPT and 50% UPT has been increased by 68% and 34% respectively. With 16 TXRUs, RU has been reduced by 6% for the same inter-arrival time, and 5% UPT and 50% UPT has been increased by 20% and 4% respectively.

| MTXRU | Q | Inter-Arrival | Avg UPT | 5% UPT | 50% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 0.6 | 23.80 | 11.15 | 30.77 | 20.1% |
| 2 | 16 | 0.6 | 24.63 | 13.34 | 31.89 | 18.9% |
| % gain wrt Q=8 |  |  | 104% | 120% | 104% | 94% |
| 4 | 32 | 0.6 | 28.53 | 18.74 | 41.24 | 15.0% |
| % gain wrt Q=8 |  |  | 120% | 168% | 134% | 75% |

Observation 1: A codebook based upon Kronecker product of horizontal and vertical precoders achieves significant performance gain as against 8-Tx baseline.

Proposal 1: Study the performance benefits of the Kronecker product PMI.

# Further Enhancement – Codebook Based Upon Linear Combination

Figure 1 MU sum-rate (capacity) gap between ideal CSI and 16-Tx KP

It has been demonstrated in Section 2 that KP codebook can achieve significant gain relative to the 8-Tx baseline. However, as KP codebook is designed based upon an assumption that the wireless channel for a link have a single dominant path (per layer), the performance advantage of KP can be limited for the channel with multiple paths and rays. Based on performance evaluation of the past releases, KP PMI feedback is expected to offer competitive performance for SU-MIMO.  For MU-MIMO, however, the availability of accurate CSI at the eNB is critical to achieve high MU-MIMO gain with advanced MU-MIMO precoding schemes such as SLNR; it is questionable whether or KP PMI precoder feedback is sufficient for MU-MIMO precoding. One such analysis results can be found in Figure 1, in which two-user sum rates achieved with rank-1 SLNR precoding is plotted in two cases: (1) ideal CSI is available at the transmitter; and (2) 16-Tx KP precoder is available at the transmitter. It can be observed that the sum-rate gap between the two cases for the 2nd case is as large as 10 dB.

To further boost MU-MIMO performance to close the gap towards the ideal-CSI bound, an approach based upon linear combination of basis vectors can be utilized to capture effects of multiple paths per precoding layer. For example, for a rank-1 PMI precoder can be represented as: , where . It can be seen that with L = 1, this codebook reduces to the KP codebook. In addition PMI quantization will be progressively refined as L increases at the expense of more feedback overhead.

In the following, preliminary analysis results are presented with L = 4 basis vectors, and unquantized coefficients . For L = 4, basis vectors  are selected from a master set, with largest long-term wideband beamforming SINR. A serving cell with 7 UEs is considered, equipped with FD-MIMO antennas with (MTXRU, N, P) = (2, 4, 2) subarray-partition configuration.

For SU beamforming, performance gap of the linear combination (LC) PMI from the ideal CSI and the 16-Tx KP PMI are summarized in Table 1. Observe that the LC PMI can close the performance gap of SU beamforming from the ideal CSI and 68% average performance improvement is achieved with L = 4 LC PMI (It is noted that 100% performance improvement is attained with ideal CSI form the 16-Tx KP baseline).

Table 1: SU SNR gap of the proposed scheme from the ideal and 16-Tx KP schemes

| UE | SNR gap from Ideal CSI (dB) | SNR Gap from 16-Tx KP (dB) |
| --- | --- | --- |
| 1 | 0.05 | 2.25 |
| 2 | 2.8 | 1.2 |
| 3 | 1.1 | 1.1 |
| 4 | 0.5 | 1.5 |
| 5 | 0.2 | 1.5 |
| 6 | 0.05 | 1.8 |
| 7 | 0.1 | 0.9 |
| Average | 0.69 | 1.46 |
| SNR gap reduction from 16-Tx KP | 0.69/(0.69+1.46) = 68% |  |

The MU sum rate of the LC PMI can be found in Figure 2. Observe that the performance gap to the ideal CSI bound is significantly reduced with the LC PMI.

Figure 2 MU sum rate of linear combination PMI with L = 4

Observation 2: A codebook based upon linear combination of basis vectors has potential to close the MU-MIMO performance gap towards the ideal CSI bound.

Proposal 2: Study the performance benefits of the linear combination PMI.

# Conclusion

This contribution has considered performance benefits of two alternative PMI codebook designs, and made the following observations and proposals:

Observation 1: A codebook based upon Kronecker product of horizontal and vertical precoders achieves significant performance gain as against 8-Tx baseline.

Observation 2: A codebook based upon linear combination of basis vectors has potential to close the MU-MIMO performance gap towards the ideal CSI bound. Proposal 1: Study the performance benefits of the Kronecker product PMI.

Proposal 2: Study the performance benefits of the linear combination PMI.

# Appendix. Derivation of Principle Eigenvector of 3D LOS Channel Covariance Matrix

With carrier frequency fc, a time-domain baseband equivalent channel vector hτ (a MTXRUN×1 vector) observed at a receive antenna at a UE is given by:

hτ=l=0L-1αlavl⨂ahlδτ-τlej2πfcτl

where

- avl is a MTXRU×1 elevation channel vector defined as  a(vl)=1 e-jvl,…,e-jM-1vlT with vl:=2πdVλccosθl;

- a(hl) is a N×1 azimuth channel vector defined as a(hl)=1 e-jhl,…,e-jN-1hlTwith hl:=2πdHλcsinθlsinφl; and

- αl and τl are respectively the path gain and the path delay.

Accordingly, the frequency domain representation is given by

H f=l=0L-1αlavl⨂ahle-j2πf-fcτl

In the LOS case, the frequency-domain baseband-equivalent model reduces to

Hf=αav⨂ahe-j2πf-fcτ0,

where  α≔α0, v≔v0 and h≔h0. At a particular frequency (or subcarrier), the spatial covariance matrix of H(f) is given by

RH=Eα2avaHv⨂ahaHh.

Then, rank-1 channel direction matrix, or the principal eigenvector α of RH, is given by

α=βav⨂ah

where β is a normalization factor.

# References

R1-15xxxx, Samsung, “FD-MIMO performance with ideal CSI with 8 to 64 TXRUs” 3GPP TSG RAN1#80

TR36.897, Study on elevation beamforming/full-dimension MIMO for LTE.