3GPP TSG-RAN WG1 Meeting #80	R1-150540

Athens, Greece, 9th – 13th February 2015

Agenda Item:	7.2.3.1

Source:	InterDigital Communications

Title:	Coexistence evaluation results for LTE LAA DL-only and Wi-Fi

Document for:	Discussion and Decision

# Introduction

In RAN1#78bis, LBT / CCA, DTX on a carrier with limited maximum transmission duration, DFS for radar avoidance in certain bands/regions, Carrier Selection and TPC were identified as techniques for LTE LAA operation to meet regulatory requirements in some regions/bands.

In RAN1#79, remaining details of assumptions and methodology for DL-only LAA coexistence evaluations were discussed and agreed [1].

In this contribution we present evaluation results for DL transmission without UL in unlicensed spectrum for the Indoor Scenario in TR 36.889 [1]. We present results for all 3 evaluation cases, Wi-Fi/Wi-Fi, LAA/LAA and Wi-Fi/LAA coexistence, and for both cases of Y=1 and Y=4 unlicensed carriers in the 5 GHz band.

# Evaluation methodology and assumptions

For evaluations purposes, we assume that LTE LAA uses LBT / CCA for channel access in conjunction with DTX to limit the maximum possible Tx duration by the LAA eNB. In addition, a slow Carrier Selection mechanism is used for both Wi-Fi APs and LTE LAA eNBs.

LBT / CCA for LTE LAA in the eNB is modeled according to the proposal outlined in [5]. First, a random backoff counter N is drawn to start the LBT procedure. Initial CCA is always immediately followed by extended CCA in durations of integer multiples of 20 µs increments. For each 20 µs increment during the extended CCA duration, if the channel is deemed to be unoccupied, the backoff counter is decremented by 1. The eNB can start DL scheduling immediately after the backoff counter value reaches zero, i.e., earliest after N+1 CCA periods. When the channel is observed to be occupied during the extended CCA period by the eNB, the backoff counter is frozen and the eNB defers back to initial CCA. As agreed in RAN1#79, we assume subframe boundary alignment across LTE serving cells when aggregated by CA. This means that once the eNB wins channel access, a DL busy signal is transmitted until the beginning of the next regularly occurring 5GHz SCell subframe boundary.

DTX for LTE LAA is modeled as a maximum allowed time duration to schedule a number of consecutive DL subframes by the eNB. In order to better assess the impacts of DTX for coexistence with Wi-Fi, we evaluate the cases of 4 ms and 13 ms TXOPs. The Tx duration of the busy signal when transmitted by the eNB prior to regular DL scheduling does not impact the allowed maximum number of schedulable DL subframes. Therefore, the LTE LAA DL will either transmit up to 4 (or up to 13) consecutive DL subframes once channel access is obtained using LBT / CCA.

We model a slow Carrier Selection mechanism for both Wi-Fi APs and LTE LAA eNBs. Carrier Selection for Y=4 is executed at start-up and a channel deemed least interfered in terms of a Rx Signal power criterion is identified. During system operation, Carrier selection will be re-done in the order of every 10 min.

We use FTP non-full buffer traffic model 3 with 0.5 MB file size. Detailed evaluation assumptions for both Wi-Fi and LTE are summarized in the appendix of this document.

# Evaluation results for LAA Indoor scenario

We present coexistence evaluation results by first showing the Y=1 case using a single frequency channel in the 5 GHz band. Wi-Fi and LTE LAA both use 4 ms TXOPs (Figures 1 and 2). Then we show and discuss results for the Y=4 case using 4 available channels (Figure 3 and 4). Both Wi-Fi and LTE LAA again use 4 ms TXOPs. In the following we present and discuss evaluation results comparing the coexistence impact when using TXOPs with 13 ms duration for LTE while Wi-Fi still uses 4 ms TXOPs (Figures 5 and 6).

## Evaluation results for Y=1

In Figures 1 and 2 serving cell throughput (SCT) and mean user throughput (UPT) for the case of Y=1 channel in the 5 GHz band is shown for varying loads. SCT in Figure 1 is displayed for low load (=10), medium load (=20) and high load (=40). Wi-Fi and LTE LAA both use 4 ms TXOPs.

It can clearly be seen Wi-Fi throughput for operator A in the deployment is not adversely impacted by the presence of LAA for low and medium loads. On the contrary, presence of LAA operator B results in a slightly increased throughput for Wi-Fi operator A. Similarly, LAA throughput improves for the Wi-Fi/LAA coexistence case when compared to the LAA/LAA coexistence case.

Figure 1: Average serving cell throughput for low/medium/high load with Y=1 channel in 5 GHz

For the low and medium load cases, another effect is observable. Clearly, not using any type of LTE inter-cell interference coordination mechanism is very detrimental for LAA SCell throughput in a dense deployment on one single frequency channel such as the Indoor scenario for Y=1.

In fact, throughput for both operators A and B using LAA is lower than when using Wi-Fi only. It is clear from the results that some form of (e)ICIC functionality is a practical necessity in real-life dense deployments. Accordingly, it should be expected that LAA throughput will vary more widely as a function of network side RRM than results show in Figure 1. Another smaller contributing factor is that LAA throughput is penalized in the order of average 10-12% by the fact that getting access to the 5 GHz channel is not immediately followed by actual DL data transmission until the beginning of the next regularly occurring SCell DL subframe boundary.

For the high load case in Figure 1, it also becomes clear that Wi-Fi operator 1 throughput is starved by LAA operator 2, i.e. almost all throughput in the deployment comes from LAA.

There is one primary contributing factor to this observed behavior. The LAA maximum transmit durations are fixed to always 4 ms TXOPs in our evaluation. This means that with increased offered load, the LAA eNB will more and more frequently attempt to win access to the channel. The LBT / CCA approach for LAA proposed in [5] results in the LAA eNB winning channel access significantly more often than Wi-Fi for a given observation interval. In practice, it can be expected that the LAA eNB would need to adjust its total channel occupancy time to the load actually observed on the channel, i.e. the LAA eNB would not right away try to re-access the channel after completion of a 4 ms transmit duration to prevent starving of Wi-Fi neighbor APs. The LAA eNB would rather leave increasing amounts of idle time intervals in-between its own channel access attempts. It can be seen from the LAA-LAA coexistence results that both operator A and B see the same throughput.

There are other factors that also contribute to the observed imbalance between Wi-Fi and LAA with high load. LAA will be able to deliver data successfully at lower packet SINR due to its link robustness and multi-channel HARQ protocol.

Figure 2: Mean user throughput as a function of load for Y=1 channel in 5 GHz

## Evaluation results for Y=4

In Figures 3 and 4 serving cell throughput (SCT) and mean user throughput (UPT) for the case of Y=4 channels in the 5 GHz band is shown for varying loads. SCT is displayed for low load (=10) and medium load (=40). Wi-Fi and LTE LAA both use 4 ms TXOPs.

From the throughput results for Operators A and B in Figure 3 it can be seen that Wi-Fi throughput for operator A in the deployment is not adversely impacted by the presence of LAA for low and medium loads. Wi-Fi throughput actually slightly improves similar to the behavior observed in the Y=1 single 5 GHz channel case. LAA operator B and Wi-Fi operator A are better neighbors to each other compared to when Wi-Fi/Wi-Fi or LAA/LAA are used in the deployment.

It can also be observed that the Y=4 case allows significantly more throughput for the LAA nodes when compared to the Y=1 single channel case. Channel selection is clearly an effective technique and results in almost un-interfered transmissions on each of the Y=4 available channels.

Figure 3: Average serving cell throughput for low/medium load with Y=4 channels in 5 GHz

Figure 4: Mean user throughput as a function of load for Y=4 channels in 5 GHz

## Impact of increased LAA TXOP durations

In Figures 5 and 6 we compare serving cell throughput (SCT) when increasing the LTE LAA maximum allowed transmit durations from 4 to 13 ms. Wi-Fi still transmits using 4 ms TXOPs. For the Y=1 low and high load cases and the Y=4 medium load case, results in Figures 5 and 6 can be directly compared to their counterparts in Figures 1 and 3.

It is very clear from the results in Figures 5 and 6 that increasing the maximum allowed transmit duration for LAA will result in increased LAA throughput without necessarily affecting Wi-Fi. Several factors contribute to this observed behavior. There is reduced overhead from the LAA busy signal preceding scheduling of actual DL data in regular SCell DL subframes. Link adaptation in the LAA eNB DL scheduler also benefits from increased LTE transmit durations in our evaluated simulation setup.

It can also be seen from Figure 5 that for the Y=1 high-load case, the use of an LAA TXOPs with 13 ms duration is about as bad as that of 4 ms TXOPs. Wi-Fi APs are starved due to CCA and limited opportunity to win access to the channel.

Figure 5: Impact of increased LAA transmit durations for Y=1 channel in 5 GHz

Figure 6: Impact of increased LAA transmit durations for Y=4 channels in 5 GHz

# Conclusion

In this contribution we present evaluation results for DL transmission without UL in unlicensed spectrum for the Indoor Scenario in TR 36.889 [1]. We present results for all 3 evaluation cases, Wi-Fi/Wi-Fi, LAA/LAA and Wi-Fi/LAA coexistence, and for both cases of Y=1 and Y=4 unlicensed carriers in the 5 GHz band.

The LTE LAA eNB employs LBT / CCA and DTX using 4 ms maximum TXOPs. In addition, a slow Carrier Selection mechanism is used for both Wi-Fi APs and LTE LAA eNBs. LBT / CCA LAA is modeled according to the proposal outlined in [5].

According to our results, Wi-Fi throughput in the deployment is not adversely impacted by the presence of LAA for low and medium loads in the Y=1 single 5 GHz channel and Y=4 available 5 GHz channel cases. On the contrary, presence of LAA in the deployment can mean better cell and mean user throughput for Wi-Fi than when all small cells use Wi-Fi.

Our results also indicate that for high load, i.e. a single 5 GHz frequency channel with many transmitters, LBT / CCA and DTX with fixed maximum transmission durations alone are not sufficient to prevent starving of the Wi-Fi APs by LAA. It is necessary to implement functionality in LTE LAA eNBs that adjusts permissible channel usage ratios to observed channel loads. Also, we observed that observed LAA throughput for the Y=1 case suffers from absence of interference coordination between LAA small cells in the evaluation setup.

We also evaluated the coexistence impact of using 13 ms TXOPs instead of 4 ms TXOPs for LAA. Our results indicate that the use of longer LAA TXOPs will result in somewhat improved link efficiency for LAA without necessarily impacting Wi-Fi for the case of low and medium loads.

# References

3GPP TR 36.889 v010 Study on License-Assisted Access to Unlicensed Spectrum (R13)

R1-150541 Design considerations on LTE LAA DL+UL mode; Interdigital

R1-145186 On the need to model RTS/CTS in LAA coexistence scenarios; InterDigital

R1-145052 On L1 design for LTE LAA DL only mode; InterDigital

R1-144778 Details of Listen-Before-Talk for LAA; Ericsson

R1-144219 On design targets and supported functionality for LTE LAA; InterDigital

R1-144218 Regulatory requirements affecting L1 design for LTE LAA; InterDigital

# Appendix A – Evaluation assumptions for LAA Indoor Scenario

|  | Unlicensed cell |
| --- | --- |
| Layout for nodes | Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |
| System bandwidth per carrier | 20 MHz |
| Carrier frequency | 5.0 GHz |
| Number of carriers | 1, 4 (to be shared between two operators) |
| Total BS TX power | 18 dBm |
| Total UE TX power | 18 dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [Table B.1.2.1-1 in TR36.814] UE-to-UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability.) |
| Penetration | 0dB |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |
| Antenna pattern | 2D Omni-directional |
| Antenna Height: | 6m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 5dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of clusters/buildings per macro cell geographical area | N/A |
| Number of small cells per cluster | N/A |
| Number of small cells per Macro cell | N/A |
| Number of UEs | 10 UEs per unlicensed band carrier per operator |
| UE dropping per network | Uniform random drop within indoor coverage |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum UE-to-UE distance (2D distance) | 3m |
| Traffic model | FTP3, 0.5 MB file size |
| UE receiver | MMSE-IRC as baseline |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | UE bandwidth for LAA and Wi-Fi: 20 MHz unlicensed |
| Network synchronization | For the same operator, the network is synchronized. Small cells of different operators are not synchronized. |
| Performance metrics | UPT, SCT |

# Appendix B – LAA evaluation assumptions for Wi-Fi

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Open loop |
| Channel coding |  | BCC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | Fixed 1ms (A)-MPDU transmission duration |
| Max PPDU duration |  | 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | CCA-CS |
|  | RTS/CTS | N/A |
|  | Contention window | Min: 15 slot, Max: 1023 slot |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL only |
| Rate control |  | Minstrel algorithm with 100 ms reconfiguration period |
| Channel selection |  | Minimum neighbour |
| OFDM symbol length |  | 4 us |