3GPP TSG RAN WG1 Meeting #80		R1-150547

Athens, Greece, 9th – 13th February 2015

Source:	QUALCOMM Incorporated

Title:	Link results on opportunistic predecoding NAICS

Agenda item:		6.3.2

Document for:	Discussion

# Introduction

Pre-decoding with RRS has long been discussed in previous meetings [1] [2] [3] [4]. In pre-decoding network assisted interference cancellation (NAIC), RRS for macro cell results in loss for the macro UE [5], limiting the benefits of having advanced NAIC UEs. Hence, in this contribution we study opportunistic pre-decoding NAIC, where the NAIC UE uses its interference cancellation capability whenever the interference has the required structure, i.e. certain lower order modulation. In this document, the link level performance gain achieved for the NAIC UE with opportunistic NAIC is studied.

# 2	Simulation Assumptions

Baseline UE has type3i receiver, and the NAIC UE performs pre-decoding IC whenever interference modulation is QPSK or 16-QAM. We assume genie knowledge at the pico UE about the modulation order of the interfering signals. These simulations are aimed at evaluating the throughput gain available for pico UE with opportunistic pre-decoding NAIC.

Table 1: Simulation Setup

| Traffic type | Full buffer |
| --- | --- |
| Number of cells | 3 |
| Number of codes on Interfering cells | 15 |
| Serving Cell Scheduling | Normal CQI based scheduling |
| Interfering Cell Scheduling | Fixed TBS |
| Opportunistic NAIC | Genie modulation info at pico UE |

2.1. UE Locations

Table 2: UE Locations

| Location | Ior1/Ioc(dB) | Ior2/Ioc(dB) | Ior3/Ioc(dB) |
| --- | --- | --- | --- |
| L1 | 7.901 | 19.819 | 2.186 |
| L2 | 10.972 | 19.285 | 1.652 |
| L3 | 14.849 | 18.936 | 1.975 |
| L4 | 19.753 | 18.611 | 2.433 |
| L5 | 26.496 | 18.272 | 2.852 |
| L6 | 37.789 | 17.92 | 3.234 |

## 2.2. Interference Type

To study the effect of opportunistic NAIC, the following 3 types of interference are considered. Note that in a non-RRS network, interfering cells are transmitting 16-QAM modulated signals for most of the time [6]. Hence, an enhanced NAIC UE should take advantage of both QPSK and 16-QAM modulated interference to result in acceptable gain over type 3i UE.

Table 3: Interference Type

| Interference Type | description |
| --- | --- |
| Type1 | 15 codes, QPSK |
| Type2 | 15 codes, 16QAM |
| Type3 | 15 codes, randomly chosen modulation PQPSK=0.25, P16QAM=0.7, P64QAM=0.05* *These numbers are collected from a system simulation [6] |

# Simulation Results

In this section we look at the throughput gain of a pico UE for opportunistic pre-decoding NAIC over type 3i receiver. Main gain is for pico UEs at the cell edge and large CIOs. For a non RRS network where the modulation can be of any order, type 3 interference in Table 3, opportunistic NAIC can provide some gain for the tail UEs on the cell edge. However, considering the potential power overhead to broadcast interference information to the pico UE will reduce macro UE’s throughput substantially and hurt the overall network throughput.

## Table 4: Simulation results for opportunistic pre-decoding NAIC

| UE Location | Interference Type1 |  |  | Interference Type2 |  |  | Interference Type3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PA3 | VA3 | PB3 | PA3 | VA3 | PB3 | PA3 | VA3 | PB3 |
| L1 | 39.73% | 45.14% | 43.97% | 14.97% | 22.69% | 21.75% | 14.65% | 21.74% | 20.76% |
| L2 | 33.84% | 41.37% | 40.42% | 12.30% | 17.90% | 17.59% | 12.23% | 17.87% | 17.53% |
| L3 | 27.63% | 39.45% | 39.16% | 10.18% | 14.24% | 13.60% | 10.29% | 14.07% | 13.35% |
| L4 | 20.62% | 30.46% | 31.46% | 8.98% | 11.81% | 11.78% | 9.58% | 11.10% | 10.90% |
| L5 | 8.66% | 16.64% | 15.15% | 6.09% | 7.94% | 7.27% | 5.56% | 8.06% | 7.03% |
| L6 | -0.12% | 0.19% | 0.07% | 0.21% | 0.24% | 0.15% | 0.18% | -0.16% | -0.35% |

# 3	Conclusions

## This contribution provides the link level simulation results for opportunistic pre-decoding NAIC. Pre-decoding NAIC results in high throughput gain for the pico UE when QPSK modulation is used by the strong interferer. However, a network tends to schedule mostly 16-QAM transmissions for which pre-decoding NAIC gain is limited even with very strong interferer.

# 4	References

- R1-140686, RRS pattern based coordinated scheduling for new IC UEs, Huawei, HiSilicon.

- R1-140685, RRS pattern based coordinated scheduling for legacy IC UEs, Huawei, HiSilicon.

- R1-135685, Pre-decoding IC results with fixed MCS interference, Huawei, HiSilicon.

- R1-135782, Link Performance of RRS in HetNet, QUALCOMM Incorporated.

- R1-xxxxxx, NAIC with RRS, QUALCOMM Incorporated.

- R1-xxxxxx, System results on opportunistic predecoding NAIC, QUALCOMM Incorporated.