3GPP TSG-RAN1 Meeting #80	R1-150574

# Athens, Greece, 9-13 Feb. 2015

# Source:	KDDI

# Title:	Consideration of UCI transmission for LAA

# Agenda item:	7.2.3.3

# Document for:	Discussion and Decision

- Introduction

At the RAN1#79 meeting, as one of functionalities in addition to the current LAA TR on the unlicensed band, CSI (Channel State Information) measurement has been approved [1]. CSI measurement report in UCI (Uplink Channel Information) has a significant impact on the performance of CA (Carrier Aggregation), and the more frequent UCI transmission leads to the better CA performace. However, at the same time, the more RB (Resource Block) of UL is consumed and UE capacity might go down seriously. Especially, LAA uses unlicensed band where wider bandwidth, sometimes consisted of many CC (Componet Carrier), is available and this UE capacity issue can get worse. We believe UCI transmission scheme on unlicensed band is useful to overcome this issue.

2. Discussions

LAA is mainly used for CA as a secondary, some license band is supposed to be a primaly. To obtain an enough CA performance, more frequent channel quality information included in UCI from UEs is preferred. Naturally, a frequent UCI report leads to the waste of RB of UL, occasionally the shotage of RB may happen when the number of UE become large.

2.1 UE capacity

When the number of RB allocated for UCI transmission becomes larger, the UE capacity can go down due to the shortage of RB for UL.

A format of UCI is determined by the number of CC[1]. In unlisenced band, wider bandwidth is usually open for various systems. In Japan, ARIB defines 255 MHz (5.47GHz ~ 5.725GHz) is available in both outdoor and indoor without any licenses.

Moreover, required amount of RB highly depends on the number of scheduling target UEs. If macro cell is selected as a primaly cell for LAA, many cells are deployed inside the single macro cell. In other words, a lot of UCIwill be transmitted on UL of prilmaly cell.

Observation 1: LAA can negatively affct on UE capacity, because unlicensed band might be consisted of various CC and the number of UEs using LAA can be larger especially when the size of primally cell is much larger than one of secondaly cell.

2.2 UCI transmission on unlicensed band

As discussed above, LAA might cause the shortange of RB for UL and UE capacity can go down. If UCI transmission can be performed on unlicensed band, we believe that this capacity problem can be reduced. In this section, the comparesion of two cases is discussed, case1: UCI transmission on licensed band case and case2: UCI transmission on unlicensed band.

Case1: UCI transmission on licensed band

By using licensed band, UCI transmission is performed at the pre-dfined timing. However, UL capacity can be a bottleneck due to the shortage of RB on UL. In general, wider bandwidth is hardly used on licensed band, especially lower frequency band.

Case2: UCI transmission on unlicensed band

By using unlicensed band, UCI transmission can be performed without additional load on licensed (primaly) band and UE capacity problem can be reduced. However, unpreditable delay might happen due to LBT (Listen-Before-Talk) and then UCI transmission could be delayed [3][4]. In LAA, all of UEs should perform LBT to solve co-existence with othe system, e.g. Wi-Fi system.This means LBT can generate redundant delay for transmission especially when the channel is busy. Therefore, even if eNB assings the best condition channel for UE, UE may not transmit the date at the channel by LBT.

Table 1: Pros. and Cons.

|  | Pros. | Cons. |
| --- | --- | --- |
| Case 1 : Licensed band | UCI transmission is performed at the required timing and the better CA performance might be realized. | Severe UL capacity problem may happen. |
| Case 2 : Unlicensed band | UL capacity problem can be reduced | Transmission opportunity of downlink is reduced due to UCI report on unlicensed UL. UCI transmission could be delayed. |

Based on this analsys, when the load on LTE is not so high, UCI transmission on licensed band is preferred. On the other hand, when the load becomes high and UL capacity on licensed band is negatively affected, unlicensed band UL should be utilized.

Proposal1: USI transmission on unlicensed band UL is permitted.

Proposal2: The number of RB on licensed/unlicensed band allocated for USI transmission should be designed to be configurable depend on the load on LTE system.

3. Conclusion

In this contribution, we discuss UL capacity issue according with UCI report. To expand the scheduler gain of downlink CA, accurate UCI is required, however, much more RB of UL is consumed.

Observation 1: LAA can negatively affct on UE capacity, because unlicensed band might be consisted of various CC and the number of UEs using LAA can be larger especially when the size of primally cell is much larger than one of secondaly cell.

To solve this trad-off, UCI report should be transmitted via unlicensed uplink and its amount should be coordinated depending on the load on LTE system.

Proposal1: USI transmission on unlicensed band UL is permitted.

Proposal2: The number of RB on licensed/unlicensed band allocated for USI transmission should be designed to be configurable depend on the load on LTE system.

4. References

- R1-145379, Huawei, HiSilicon, Intel, Kyocera, CATR, Ericsson,  NTT DOCOMO, Nokia Networks, Nokia Corporation,  CATT, Sharp, ZTE, “WF on LAA functionalieties”

- 3GPP TS36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”

- R1-144970, NTT DOCOMO, “Views on issues related to LAA UL”

- R1-144830, ZTE, “Analysis of LAA UL enhancement”