3GPP TSG RAN WG1 Meeting #80                                                                            R1-150579

Athens, Greece, 9th – 13th February 2015

Source:	Ericsson

Title:	Outdoor Coexistence Evaluation Results for LAA with Only DL Transmissions

Agenda Item:	7.2.3.1

Document for:	Discussion and Decision

# Introduction

Substantial progress has been made for the Study Item on the Licensed-Assisted Access (LAA) [1] in RAN1 WG #79 [2]. In this contribution, we report the coexistence evaluation results for the LAA solution without UL transmissions discussed in [3] for the outdoor scenarios.

# Coexistence Evaluation Assumptions

The coexistence methodology agreed in RAN1#78bis [4] is the following:

Agreements:

- Wi-Fi-LAA coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- LAA-LAA coexistence

  - Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two LAA operators are compared.

The above agreed methodology is used for the coexistence evaluations. Briefly, in the Wi-Fi and LAA coexistence scenario, in the first step Operator A and B both use Wi-Fi. In the second step, operator A and its corresponding UEs are replaced by an LAA operator and LAA UEs while operator B and its UEs remain unchanged. In the LAA-LAA coexistence scenario both operators that coexist with each other are LAA networks.

The simulation assumptions are based on the agreed coexistence assumptions in [5]. However our preferences on the assumptions that remained optional or need clarifications when results are presented are provided below. In all the indoor coexistence evaluations, the transmit power of the base station in the unlicensed band is assumed to be 18 dBm. Moreover, FTP model 3 is used for generating FTP traffic.

Additionally, the LBT algorithm used for LAA is based on the proposed load-based algorithm presented in [3] where the LAA CCA starts and resumes at the subframe boundaries. In more detail, if LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node by reservation signals until the 4th OFDM symbol and then followed by data transmission from the 4th OFDM symbol in that subframe. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is deferred to the next subframe boundary where the CCA count down is resumed. A maximum channel occupancy time of 4ms is assumed for LAA. Finally, Table 1 and Table 2 capture our assumptions for Wi-Fi and LAA systems.

Table 1 Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Baseline: open loop |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | DCF If VoIP users are included, EDCA can be used |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per DCF If VoIP users are included, per EDCA can be used |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation |
| Rate control |  | Minstrel algorithm |
| Channel selection |  | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| OFDM symbol length |  | 4 micro second |

Table 2 Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | Based on TM10, QPSK/16QAM/64QAM/256QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -62 dBm |
| Channel selection | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| Cyclic Prefix | Normal |

# Outdoor FTP traffic coexistence evaluation results

## Scenario with one shared unlicensed carrier

In this evaluation scenario, two operators deploy X=4 small cells each in an outdoor hot-spot cluster sharing Y=1 unlicensed carrier. 10 UEs with DL FTP traffic for each operator network are modeled. Table 3, Table 4 and Table 5 below show the throughput and latency for some significant points on the CDF for each of the coexistence scenarios. The UPT and latency are shown for each scenario corresponding to the served traffic supported by the baseline system at a given mean buffer occupancy.

Table 3 Throughput and latency CDF at low system load point (20% buffer occupancy for the baseline, arrival rate = 0.436214 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 75.11 | 75.86 | 152.89 | 109.54 |
| 50% | 41.92 | 43.04 | 127.79 | 82.90 |
| 5% | 7.04 | 8.23 | 73.82 | 30.28 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 1.079 | 1.005 | 0.049 | 0.032 |
| 50% | 0.173 | 0.156 | 0.030 | 0.048 |
| 5% | 0.052 | 0.051 | 0.023 | 0.122 |

Table 4 Throughput and latency CDF at medium system load point (40% buffer occupancy for the baseline, arrival rate = 0.55243 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 48.08 | 50.12 | 68.81 | 105.50 |
| 50% | 17.98 | 19.22 | 122.76 | 77.19 |
| 5% | 0.92 | 1.19 | 151.16 | 25.2 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 2.458 | 2.422 | 0.054 | 0.15 |
| 50% | 0.391 | 0.358 | 0.032 | 0.054 |
| 5% | 0.081 | 0.081 | 0.023 | 0.033 |

Table 5 Throughput and latency CDF at high system load point (55% buffer occupancy for the baseline, arrival rate = 0.6667 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 35.60 | 34.75 | 149.35 | 102.17 |
| 50% | 9.52 | 8.74 | 118.58 | 72.45 |
| 5% | 0.39 | 0.39 | 64.86 | 21.55 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  |
| --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 3.132 | 3.187 | 0.059 | 0.186 |
| 50% | 0.569 | 0.606 | 0.033 | 0.060 |
| 5% | 0.101 | 0.122 | 0.023 | 0.034 |

Table 6 below presents the throughput and latency for some significant points on the CDF for the LAA-LAA coexistence scenarios.

Table 6 Throughput and latency CDF for different system load points for LAA-LAA coexistence

| Throughput CDF [Mbps] | 0.436214 files/s arrival rate, 20% mean buffer occupancy |  | 0.55243 files/s arrival rate, 40% mean buffer occupancy |  | 0.6667 files/s arrival rate, 55% mean buffer occupancy |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A LAA | Op.B LAA | Op.A LAA | Op.B LAA | Op.A LAA | Op.B LAA |
| 95% | 159.45 | 159.66 | 158.51 | 158.26 | 157.67 | 157.22 |
| 50% | 138.8 | 138.76 | 136.82 | 136.40 | 135.11 | 134.53 |
| 5% | 87.97 | 89.25 | 83.97 | 85.41 | 81.18 | 82.62 |

| Latency CDF [s] | 0.436214 files/s arrival rate, 20% mean buffer occupancy |  | 0.55243 files/s arrival rate, 40% mean buffer occupancy |  | 0.6667 files/s arrival rate, 55% mean buffer occupancy |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A LAA | Op.B LAA | Op.A LAA | Op.B LAA | Op.A LAA | Op.B LAA |
| 95% | 0.038 | 0.038 | 0.041 | 0.039 | 0.043 | 0.041 |
| 50% | 0.026 | 0.026 | 0.027 | 0.027 | 0.027 | 0.028 |
| 5% | 0.022 | 0.022 | 0.022 | 0.022 | 0.022 | 0.022 |

## Scenario with four shared unlicensed carriers

In this evaluation scenario, two operators deploy X=4 small cells each in an outdoor hot-spot cluster sharing Y=4 unlicensed carriers. 40 UEs with DL FTP traffic for each operator network are modeled.

Table 7 Throughput and latency CDF at low system load point (20% buffer occupancy, arrival rate = 0.328923 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 124.68 | 125.90 | 184.55 | 137.72 | 188.64 | 188.56 |
| 50% | 87.99 | 85.38 | 155.72 | 109.07 | 163.33 | 163.43 |
| 5% | 22.79 | 23.64 | 93.84 | 39.71 | 102.07 | 106.26 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 0.407 | 0.331 | 0.039 | 0.102 | 0.032 | 0.031 |
| 50% | 0.053 | 0.057 | 0.024 | 0.036 | 0.022 | 0.022 |
| 5% | 0.029 | 0.028 | 0.019 | 0.026 | 0.018 | 0.018 |

Table 8 Throughput and latency CDF at medium system load point (40% buffer occupancy, arrival rate = 0.47043 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 106.22 | 106.57 | 175.70 | 131.44 | 181.97 | 181.44 |
| 50% | 53.85 | 51.67 | 140.99 | 93.43 | 153.44 | 153.43 |
| 5% | 5.91 | 6.80 | 74.50 | 26.23 | 89.70 | 95.29 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 1.329 | 1.381 | 0.051 | 0.165 | 0.038 | 0.037 |
| 50% | 0.113 | 0.119 | 0.027 | 0.046 | 0.024 | 0.024 |
| 5% | 0.034 | 0.034 | 0.020 | 0.026 | 0.019 | 0.019 |

Table 9 Throughput and latency CDF at high system load point (60% buffer occupancy, arrival rate = 0.62207 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 86.15 | 86.23 | 168.14 | 123.54 | 176.70 | 176.72 |
| 50% | 29.11 | 28.27 | 127.61 | 77.91 | 145.00 | 145.59 |
| 5% | 1.26 | 1.03 | 58.52 | 17.99 | 81.69 | 86.06 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−LAA coexistence |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B LAA |
| 95% | 2.529 | 2.56 | 0.074 | 0.331 | 0.044 | 0.044 |
| 50% | 0.257 | 0.259 | 0.031 | 0.064 | 0.026 | 0.026 |
| 5% | 0.045 | 0.045 | 0.021 | 0.028 | 0.020 | 0.020 |

# Outdoor VoIP traffic coexistence evaluation results

## Scenario with one shared unlicensed carrier

In this evaluation scenario, two operators deploy X=4 small cells each in an outdoor hot-spot cluster sharing Y=1 unlicensed carriers. 10 UEs with DL FTP traffic for each operator network are modeled. Two additional VoIP traffic UEs are modeled for the Wi-Fi network that is not replaced by LAA. The tables in this section show the throughput and latency for some significant points on the CDF for each of the coexistence scenarios. The UPT and latency are shown for each scenario corresponding to the served traffic supported by the baseline system at a given mean buffer occupancy. The tables below show the percentage of UEs that have 98%ile latency greater than 50 ms for the difference coexistence scenarios. Moreover, the tables include the case where LAA does not use the licensed carrier for scheduling.

Table 10 Throughput and latency CDF at low system load point (20% buffer occupancy for the baseline, arrival rate =0.339782 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 74.52 | 69.51 | 151.32 | 102.87 | 98.37 | 97.26 |
| 50% | 39.18 | 34.60 | 126.99 | 71.02 | 75.44 | 61.84 |
| 5% | 6.09 | 7.30 | 78.16 | 20.96 | 29.66 | 16 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 1.523 | 0.058 | 0.047 | 0.179 | 0.184 | 0.274 |
| 50% | 0.195 | 0.213 | 0.030 | 0.057 | 0.060 | 0.076 |
| 5% | 0.053 | 0.058 | 0.023 | 0.034 | 0.035 | 0.036 |

Table 11 Throughput and latency CDF at medium system load point (40% buffer occupancy for the baseline, arrival rate =0.491782 files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 45.42 | 41.66 | 145.80 | 95.66 | 89.50 | 86.23 |
| 50% | 13.38 | 11.48 | 117.73 | 61.64 | 59.40 | 46.98 |
| 5% | 0.58 | 0.50 | 67.19 | 14.61 | 16.41 | 8.78 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 3.034 | 3.419 | 0.056 | 0.255 | 0.373 | 0.562 |
| 50% | 0.424 | 0.479 | 0.033 | 0.069 | 0.086 | 0.115 |
| 5% | 0.093 | 0.104 | 0.024 | 0.036 | 0.039 | 0.042 |

Table 12 Throughput and latency CDF at high system load point (60% buffer occupancy for the baseline, arrival rate = 0.6667files/s)

| Throughput CDF [Mbps] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 29.65 | 26.42 | 141.63 | 89.08 | 81.55 | 74.97 |
| 50% | 5.97 | 5.45 | 107.31 | 53.79 | 45.56 | 32.04 |
| 5% | 0.29 | 0.24 | 58.64 | 10.51 | 9.39 | 3.73 |

| Latency CDF [s] | Wi-Fi−Wi-Fi baseline |  | LAA−Wi-Fi coexistence |  | LAA−Wi-Fi coexistence without licensed carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Op.A Wi-Fi | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi | Op.A LAA | Op.B Wi-Fi |
| 95% | 3.875 | 4.598 | 0.066 | 0.338 | 0.683 | 1.191 |
| 50% | 0.734 | 0.835 | 0.037 | 0.082 | 0.123 | 0.185 |
| 5% | 0.111 | 0.138 | 0.024 | 0.040 | 0.043 | 0.050 |

Table 13 Percentage of VoIP users with 98%tile latency greater than 50ms at different system load points

| System load points | Wi-Fi−Wi-Fi baseline | LAA−Wi-Fi coexistence | LAA−Wi-Fi coexistence without licensed carrier |
| --- | --- | --- | --- |
|  | Op. A Wi-Fi, Op.B Wi-Fi | Op. A LAA, Op.B Wi-Fi | Op. A LAA, Op.B Wi-Fi |
| Low (20% mean buffer occupancy) | 0.103 | 0 | 0.057 |
| Medium (40% mean buffer occupancy) | 0.239 | 0.003 | 0.158 |
| High (60% mean buffer occupancy) | 0.386 | 0.009 | 0.336 |

## Scenario with four shared unlicensed carriers

In this evaluation scenario, two operators deploy X=4 small cells each in an outdoor hot-spot cluster sharing Y=4 unlicensed carriers. 40 UEs with DL FTP traffic for each operator network are modeled. Eight additional VoIP traffic UEs are modeled for the Wi-Fi network that is not replaced by LAA.

Table 14 Percentage of VoIP users with 98%tile latency greater than 50ms at different system load points

| System load points | Wi-Fi−Wi-Fi baseline | LAA−Wi-Fi coexistence |
| --- | --- | --- |
|  | Op.B Wi-Fi | Op.B Wi-Fi |
| Low (20% mean buffer occupancy) | 0.076 | 0.009 |
| Medium (40% mean buffer occupancy) | 0.219 | 0.031 |
| High (60% mean buffer occupancy) | 0.292 | 0.082 |

# Discussion

Coexistence evaluations results are provided both for FTP DL traffic as well as for VoIP traffic. In general, the coexistence evaluation results are shown for buffer occupancies (as per the agreed working assumption [2]) of 20%, 40% and 60% representing low, medium and high traffic loads, respectively. Key points in the throughput and latency CDF are presented for FTP DL traffic by providing the corresponding metrics at 5, 50 and 95 percentiles. The outage probability with 98%ile latency greater than 50 ms is shown for VoIP users for different loads in the system. The evaluations were carried out with the proposed load based LBT scheme [3] with additional defer periods which aligns well with the Wi-Fi defer period and is designed to minimize the collision between Wi-Fi data and corresponding ACKs.

The results where LAA is using only the unlicensed carrier for scheduling, shown in Section 4.1, evaluate the coexistence between LAA and Wi-Fi when LAA is serving traffic over the same amount of spectrum as the Wi-Fi network it replaces. These results illustrate that LAA can co-exist well with Wi-Fi and even improve performance of the Wi-Fi network it is co-existing with.

On the impact of the licensed carrier in LAA, comparing the results (Section 3) with the use of the licensed carrier for scheduling for the one and four unlicensed carrier cases, it is observed that the licensed carrier becomes less influential in the system performance improvement. This is due to the licensed carrier experiencing higher loads in order to support four unlicensed carriers instead of one. In the evaluated four unlicensed carrier scenario, we observe that LAA can coexist well with Wi-Fi.

The system performance results clearly show that not only LAA coexists in a fair manner with Wi-Fi but also boosts Wi-Fi performance as compared to the case where two Wi-Fi networks coexist with each other. Some of the major elements which make LAA a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are listed below:

- LTE, as compared to Wi-Fi, enjoys more robust interference mitigation, error correction and retransmission schemes which results in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium.

- LAA can offload the traffic to the licensed carrier which improves availability of a channel for transmissions.

- LAA uses the same sensing threshold irrespective of transmissions from LAA or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for LAA as compared to Wi-Fi.

The results for the outage of VoIP users in the Wi-Fi system in section 4 clearly demonstrate that LAA has does not impact VoIP traffic on a WI-Fi network more than another Wi-Fi network. In fact, in the case of one single unlicensed carrier, LAA can decrease the outage of VoIP traffic in the coexisting Wi-Fi network.

Additionally, the results in sections 3.1 and 3.2 demonstrate that two LAA systems coexisting in the unlicensed band provide comparable performance which stems from the fact that the unlicensed spectrum is fairly shared and utilized among the LAA operators.

Overall, the results show that LAA does not degrade VoIP and FTP performance of a co-existing Wi-Fi network when it is used instead of another Wi-Fi network for the evaluated scenarios. Similar trends in performance are observed for outdoor deployments as shown in our companion contribution [6]. We summarize the discussion above with the following observations:

Observations:

- LAA can improve the performance of a coexisting Wi-Fi operator.

- LAA has less impact on VoIP traffic of a coexisting Wi-Fi operator than another Wi-Fi operator.

- Two LAA systems can co-exist well in unlicensed spectrum.

# Conclusions

In this contribution, we report coexistence evaluation results for LAA for DL-only transmissions for outdoor deployments. Extensive simulation results are provided not only for FTP traffic, but also for VoIP traffic, for both single unlicensed and four unlicensed carriers. We have also provided simulation results for the single unlicensed carrier case without the use of the licensed carrier for scheduling in LAA. Based on the simulation results, we make the following observations:

Observations:

- LAA can improve the performance of a coexisting Wi-Fi operator.

- LAA has less impact on VoIP traffic of a coexisting Wi-Fi operator than another Wi-Fi operator.

- Two LAA systems can co-exist well in unlicensed spectrum.

# References

RP-141664 Study on Licensed-Assisted Access using LTE, Ericsson, Qualcomm, Huawei, ALU

Chairman’s note 3GPP RAN1 WG #79

R1-150584 Further details on LBT for LAA, Ericsson

Chairman’s notes, 3GPP RAN1 WG#78bis

3GPP TR 36.899 Study on Licensed-Assisted Access to Unlicensed Spectrum, v.0.1.1

R1-150578 Indoor coexistence evaluation results for DL-only LAA; Ericsson