3GPP TSG RAN WG1 Meeting #80,		R1-150620

Athens, Greece, 9th - 13th February 2015

Source:	Kyocera

Title:	        Initial evaluation result for LAA

Agenda Item:	7.2.3.1

Document for:	Discussion/Decision

Introduction

In this contribution, we evaluated the impact of LAA on Wi-Fi. According to the following description [1], first we evaluated the two Wi-Fi networks model and then the Wi-Fi/LAA model was evaluated. After that we estimated the impact of LAA using these results.

In the Wi-Fi-LAA coexistence case, the following evaluation methodology is followed. - For each UE and eNB/AP drop - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded. - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded. A comparison of the performance metrics between the two steps for the Wi-Fi network that was not replaced with LAA can be used to evaluate coexistence between LAA and Wi-Fi in an unlicensed band.

Simulation Parameter

The detailed parameters are described in Appendix A. We focus on Indoor model and assume the LAA frame structure as shown in Figure 1. We assume a fixed length LBT before the transmission burst and at least 1 subframe IDLE period after the transmission burst. In addition, the maximum burst length is assumed to be 4msec.

Figure 1: Evaluated LTE burst design

Evaluation result

Figure 2 shows the performance results. Table 1 presents the average UPT. As indicated by the above results LAA - Wi-Fi network model shows performance gain of 2.3%. Similarly, we also evaluated the latency performance of the two models. Figure 3 and Table 2 presents the latency related evaluation results. The LAA - Wi-Fi network has a 13% improvement in latency. We believe this improvement comes from LBT and 1 subframe IDLE period. We found UPT and Latency results showed Wi-Fi impact of LAA is very small. As shown by the results we can conclude LAA with LBT can be introduced to unlicensed bands without impacting the Wi-Fi performance.

Observation:

LAA with LBT can be introduced to unlicensed bands without Wi-Fi impact.

Figure 2: Wi-Fi UPT CDF (Indoor model)

Table 1: Wi-Fi UPT Average (Indoor model)

| WiFi UPT (WiFi–WiFi model) | WiFi UPT (LAA-WiFi model) | % |
| --- | --- | --- |
| 32.6 Mbps | 33.51 Mbps | 2.7% UP |

Figure 3: Wi-Fi Latency CDF (Indoor model)

Table 2: Wi-Fi Latency Average (Indoor model)

| WiFi Latency (WiFi–WiFi model) | WiFi Latency (LAA-WiFi model) | % |
| --- | --- | --- |
| 196 msec | 170 msec | 13% Down |

Conclusions

Observation:

LAA with LBT can be introduced to unlicensed bands without Wi-Fi impact.

- Appendix A

The parameters used in the simulation are described below. Basically these parameters are based on [1].

- A.1 Indoor scenario for LAA coexistence evaluations

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. 120 m 50 m |  |
| System bandwidth per carrier | 10MHz. | 20MHz |
| Carrier frequency | 3.5GHz. | 5.0GHz |
| Number of carriers | 2 (one for each operator) . | 1 |
| Total BS TX power | 24dBm (Ptotal per carrier) . | 18 dBm |
| Total UE TX power | Total UE TX power: 18 dBm |  |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH |  |
| Penetration | 0dB |  |
| Shadowing | ITU InH |  |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |
| Antenna Height: | 6m |  |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 5dBi |  |
| Antenna gain of UE | 0 dBi |  |
| Fast fading channel between eNB and UE | ITU InH |  |
| Number of clusters/buildings per macro cell geographical area | N/A |  |
| Number of small cells per cluster | N/A |  |
| Number of small cells per Macro cell | N/A |  |
| Number of UEs | 10 UEs per unlicensed band carrier per operator |  |

| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell in the unlicensed band. |
| --- | --- |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum distance (2D distance) | 3m |
| Traffic model | FTP Model 3 and file size: 0.5 Mbytes. |
| UE receiver | MMSE-IRC |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | 20 MHz unlicensed only |
| Network synchronization | Perfect synchronization in the same operator, Asynchronization in other operators |
| Performance metrics | - Performance metric - User perceived throughput (UPT) - UPT CDF - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF - Note: DL and/or UL can be reported when applicable |

- A.2 Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM |
| Antenna configuration |  | 1Tx2Rx |
| Channel coding |  | BCC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500 bytes |
| Max PPDU duration |  | 4 ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Min : 15, Max : 1023 |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation |
| Rate control |  | Minstrel algorithm |
| Channel selection |  | N/A (1 channel simulation) |
| OFDM symbol length |  | 4 micro second |

- A.3 Additional LAA system evaluation assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 1Tx2Rx |
| Transmission schemes | Single Antenna |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Fixed (MCS 20) |
| CCA-ED | -62dBm |
| Channel selection | N/A (within 1 channel) |
| Cyclic Prefix | Normal |

- References

[1] TR 36.889 V0.1.1