3GPP TSG RAN WG1 Meeting #80		R1-150685

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.3.1

Source:	InterDigital

Title:	Potential feedback enhancement for FD-MIMO

Document for:	Discussion

# 1	Introduction

In RAN1 #78bis, there was a noticeable progress on the antenna modelling and evaluation methodologies for elevation beamforming and FD-MIMO. In this contribution, we provide initial evaluation results of potential feedback enhancements for 2D antenna array using FD-MIMO schemes based on the agreed evaluation assumptions in the 3D channel model.

# 2	Initial Results for FD-MIMO

The FD-MIMO performance is evaluated based on the agreed assumptions [1-2]. The FD-MIMO performance is compared with the elevation beamforming (fixed downtilt) scheme and vertical sectorization with the same 2D antenna setup with (M, N, P, Q) = (8, 2, 2, Q) with the number of ports Q = 4, 8, 16, 32 supported in different antenna configurations. The details of the simulation assumption are listed in Table A in the Annex.

The table 1 shows the performance of elevation beamforming and FD-MIMO with different number of antenna ports using full buffer traffic and ideal SVD feedback in order to see the upper bound of the performance without codebook design impact. Also, dynamic SU and MU-MIMO switching was used at the transmitter. The different number of antenna ports for FD-MIMO is implemented by antenna virtualization using DFT weight with downtilt [2].

Table 1:  Full buffer results (SVD based precoding)

| Scheme | Antenna configuration (M,N,P,Q) and codebook | Performance metric | 3D-UMa (SU/MU) | 3D-UMi (SU/MU) |
| --- | --- | --- | --- | --- |
| Elevation beamforming (fixed downtilt) | K=8, V1H4, tilt = 100 (M,N,P,Q) = (8,2,2,4) SVD-based precoding | Cell avg. (b/Hz/s) | 2.20 (0%) | 2.31 (0%) |
|  |  | Cell edge (b/Hz/s) | 0.075 (0%) | 0.058 (0%) |
| Vertical sectorization | K=8, V1H4, tilt = [100, 104] (M,N,P,Q) = (8,2,2,4) SVD-based precoding | Cell avg. (b/Hz/s) | 3.04 (+38%) | 3.27 (+42%) |
|  |  | Cell edge (b/Hz/s) | 0.094 (+25%) | 0.098 (+69%) |
| FD-MIMO | K=4, V2H4, tilt = 100 (M,N,P,Q) = (8,2,2,8) SVD-based precoding | Cell avg. (b/Hz/s) | 2.68 (+22%) | 2.93 (+27%) |
|  |  | Cell edge (b/Hz/s) | 0.089 (+19%) | 0.083 (+43%) |
|  | K=2, V4H4, tilt = 100 (M,N,P,Q) = (8,2,2,16) SVD-based precoding | Cell avg. (b/Hz/s) | 2.95 (+34%) | 4.00 (+73%) |
|  |  | Cell edge (b/Hz/s) | 0.076 (+1%) | 0.100 (+72%) |
|  | K=1, V8H4, tilt = 100 (M,N,P,Q) = (8,2,2,32) SVD-based precoding | Cell avg. (b/Hz/s) | 3.10 (+41%) | 4.33 (+87%) |
|  |  | Cell edge (b/Hz/s) | 0.102 (+36%) | 0.120 (+106%) |

From the table 1, it is observed that the FD-MIMO performance gets better as the number of antenna port increased even though the same set of the antenna elements is used. The gain over the fixed downtilt goes up to 87% cell average throughput and 106% cell edge throughput. In addition, the vertical sectorization showed a significant performance gain as compared with single vertical sector with a fixed downtilt due to its spatial reuse gain.

The table 2 shows the performance of elevation beamforming and FD-MIMO in non-full-buffer targeting 70% resource utilization. The LTE codebook based feedback is also used to see more realistic performance results. As for the number of antenna ports larger than 8, a combination of LTE codebook with Kronecker product was used.

Table 2: non-full buffer results

| Scheme | Antenna configuration (M,N,P,Q) and codebook | Performance metric | 3D-UMa (SU/MU) | 3D-UMi (SU/MU) |
| --- | --- | --- | --- | --- |
| Elevation beamforming (fixed downtilt) | K=8,V1H4, tilt = 100 (M,N,P,Q) = (8,2,2,4) LTE 4Tx codebook | Mean throughput (Mbps) | 15.3 | 13.9 |
|  |  | 5% UE throughput (Mbps) | 2.4 | 1.9 |
|  |  | RU (%) | 63% | 66% |
| Vertical sectorization | K=8, V1H4, tilt = [100, 104] (M,N,P,Q) = (8,2,2,4) LTE 4Tx codebook | Mean throughput (Mbps) | 14.2 | 13.8 |
|  |  | 5% UE throughput (Mbps) | 3.6 | 4.1 |
|  |  | RU (%) | 63% | 65% |
| FD-MIMO | K=4,V2H4, tilt = 100 (M,N,P,Q) = (8,2,2,8) Rel-10 8Tx codebook | Mean throughput (Mbps) | 13.1 | 14.8 |
|  |  | 5% UE throughput (Mbps) | 2.95 | 3.5 |
|  |  | RU (%) | 59.5% | 60% |
|  | K=2, V4H4, tilt = 100 (M,N,P,Q) = (8,2,2,16) 16Tx codebook (Kronecker product of two LTE 4Tx codebook) | Mean throughput (Mbps) | 15.6 | 16.5 |
|  |  | 5% UE throughput (Mbps) | 4.6 | 4.1 |
|  |  | RU (%) | 66% | 67% |
|  | K=1, V8H4, tilt=100 (M,N,P,Q) = (8,2,2,32) 32Tx codebook (Kronecker product with one LTE 4Tx and one LTE 8Tx codebook) | Mean throughput (Mbps) | 17.2 | 18.2 |
|  |  | 5% UE throughput (Mbps) | 5.9 | 4.3 |
|  |  | RU (%) | 64% | 67% |

As seen in the table 2, the similar tendency is observed that the larger number of antenna port provide better performance in FD-MIMO. However, the relative gain is reduced since the MU-MIMO performance is reduced in non-full-buffer traffic and the codebook-based feedback loses beamforming gain.

Observation:

- A feedback for larger number of antenna ports (>8) may potentially provide gain.

- A new codebook structure may be considered to support larger than 8 antenna ports with 2D array

# 3	Summary

In this contribution, we provided initial evaluation results of FD-MIMO using 2D antenna array in 3D-UMa and 3D-UMi channels in order to see the potential gain from FD-MIMO by using larger number of antenna ports. From the simulation results, the followings were observed:

- A feedback for larger number of antenna ports (>8) showed significant gain.

- A new codebook structure may be considered to support larger than 8 antenna ports with 2D array

# References

- R1-144444, “WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, “WF on Antenna modeling”, Samsung, Nokia Networks, Nokia Corporation, CMCC, Huawei, HiSilicon, Interdigital, LGE, Motorola Mobility, Qualcomm, CHTTL

- 3GPP TR 36.873, “Study on 3D channel model for LTE,”  V12.0.0 (2014-09)

# Annex

The 2D Antenna array used in the simulation is configured as below.

There are M elements in a column and 2xN elements in a row. K elements in a column from the same polarization are weighted (DFT-based weights) to form the desired downtilt angle θtilt. A 2-dimentional port configuration (VmHn) is formed with m ports in the vertical dimension and n ports in the horizontal dimension.

Table A. System Level Simulation Assumptions for Adaptive Downtilt and Vertical Sectorization

| Parameter | Assumption |
| --- | --- |
| Network layout | 7-site 21-cell wraparound |
| Channel model | 3D-UMa, 3D-UMi [3] |
| eNB antenna configuration | (M,N,P,Q) = (8,2,2,Q) with Q = 4, 8, 16, 32 , X-pol [2] |
| UE antenna configuration | nR=2, cross-polarization, 0o/90o |
| UE attachment | RSRP on CRS port 0 [2] |
| Downtilt | θtilt= 100o |
| Number of UEs per cell | 10 |
| UE distribution | uniform two-dimension UE distribution according to [3] |
| Traffic model | Full buffer, non-full buffer FTP model 1, packet size 0.5M bytes |
| Scheduler | proportional fair (PF) |
| Transmission scheme | SU/MU-MIMO dynamic switching with SU-MIMO feedback, Non-transparent MU-MIMO |
| Codebook | Rel-8 4Tx and Rel-10 8Tx codebook for Q=4, 8. Kronecker-based 16Tx, 32Tx codebook for Q=16, 32. |
| Feedback | subband channel quality index (CQI), wideband precoding matrix index |
| Link adaptation | AMC with OLLA, 10% BLER target |
| Receiver | MMSE-IRC, ideal channel estimation, ideal interference modelling |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered per 5ms |
|  | feedback delay is 5 ms |
| Hybrid ARQ | maximum 4 transmissions |