# 3GPP TSG RAN WG1 Meeting #80	 	R1-150707

# Athens, Greece, 9th – 13th February 2015	(Revision of R1-150189)

# Title: 	Coexistence Evaluation Results for DL-only LAA

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	7.2.3.1

# Document for: 	Discussion/Decision

# Introduction

In RAN1 #79, preliminary agreements were made regarding the methodology for coexistence studies [1]. In this contribution, some preliminary simulation results for DL only LAA are provided on different coexistence scenarios and different configurations.

# Evaluation Assumptions

In this section, the evaluated scenarios and Listen-Before-Talk (LBT) mechanisms for LAA are described. The additional simulation assumptions can be found in the appendix. Note that although the LAA carrier is aggregated with a licensed carrier, the data traffic is only scheduled on the LAA SCell in order to achieve  a fair comparison with Wi-Fi.

# Coexistence Scenarios

In order to evaluate Wi-Fi-LAA coexistence, three coexistence cases are evaluated as agreed for both indoor and outdoor deployment over unlicensed carriers.

- Wi-Fi/Wi-Fi coexistence

- LAA/Wi-Fi coexistence

- LAA/LAA coexistence

For co-existence evaluation, the following three-step evaluation methodology is followed as agreed in [1].

For each UE and eNB/AP drop

- Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

- Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi networks. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

- Step 3: The other Wi-Fi network is replaced with LAA. Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

# Listen before Talk Mechanism for LAA

The LBT functionality is required for LAA-LTE to meet regulatory requirements in some regions, and it is expected to allow fair coexistence among different nodes. The LAA frame structure could follow load-based equipment (LBE) based or frame-based equipment (FBE) based approach. Table 1 shows the detailed LBT parameters used in our simulations. Specifically,

- For FBE, the idle time always starts at a sub-frame boundary, and the CCA check is located at the end of the idle time. The transmission starts immediately after the CCA check succeeds.  To achieve the reuse factor 1, all the intra-operator nodes would sense the channel at the same time periodically.

- For LBE, two frame structures are evaluated:

- Alt 1: The channel is reserved by preamble transmission until the next sub-frame boundary if extended clear channel assessment (eCCA) operation succeeds before the sub-frame boundary. And the data transmission always ends at the sub-frame boundary.

- Alt 2: Data transmission can start and end in the middle of a sub-frame if eCCA succeeds. Preamble with at least one OFDM symbol is used to occupy the channel.

Table 1 LBT simulation parameters

| Parameters | Values |
| --- | --- |
| CCA time for FBE and LBE | 20 us |
| Maximum Channel Occupancy Time (MCOT) for FBE | 3784 us |
| Idle Period for FBE | 216 us ( more than 5 % of MCOT) |
| Maximum Channel Occupancy Time (MCOT) for LBE | 4 ms for q = 10 |
|  | 8 ms for q = 20 |

# Simulation Results

In this section, simulation results are provided for indoor and outdoor coexistence scenarios with different parameter settings. Unless specified otherwise, LBE-based frame structure with Alt 2 is evaluated for LAA, with the maximum channel occupancy time of 4 ms, and the file arrival rate for FTP traffic model is 0.6.

# Coexistence Evaluation

Table 2 and Table 3 show the coexistence simulation results in the outdoor scenario and the indoor scenario, respectively.

Table 2 Coexistence simulation results for the outdoor scenario

|  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| Wi-Fi in Wi-Fi/Wi-Fi | 0.50 | 4.69 | 41.99 | 10.60 | 57 | 822 | 4552 | 1333.50 | 0.16 | 0.60 | 0.93 | 0.58 |
| LAA in LAA/Wi-Fi | 1.15 | 14.03 | 46.91 | 17.45 | 59 | 293 | 2950 | 690.67 | 0.09 | 0.42 | 0.86 | 0.45 |
| Wi-Fi in LAA/Wi-Fi | 0.82 | 6.26 | 42.04 | 11.96 | 50 | 699 | 4031 | 1166.50 | 0.12 | 0.48 | 0.91 | 0.49 |
| LAA in LAA/LAA | 0.53 | 16.83 | 66.27 | 21.90 | 40 | 176 | 4222 | 825.20 | 0.08 | 0.45 | 0.94 | 0.46 |

Table 3 Coexistence simulation results for the indoor scenario

|  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| Wi-Fi in Wi-Fi/Wi-Fi | 0.61 | 11.02 | 80.07 | 22.05 | 33 | 460 | 4539 | 1114.48 | 0.07 | 0.52 | 0.92 | 0.52 |
| LAA in LAA/Wi-Fi | 2.39 | 27.29 | 81.69 | 32.42 | 37 | 157 | 1770 | 421.36 | 0.05 | 0.29 | 0.71 | 0.33 |
| Wi-Fi in LAA/Wi-Fi | 0.85 | 27.82 | 91.38 | 35.59 | 32 | 176 | 3177 | 699.22 | 0.06 | 0.43 | 0.89 | 0.45 |
| LAA in LAA/LAA | 0.60 | 27.55 | 81.54 | 30.34 | 37 | 123 | 3957 | 638.74 | 0.05 | 0.30 | 0.91 | 0.39 |

Compared with the case of the Wi-Fi/Wi-Fi coexistence, it can be observed that Wi-Fi in LAA/Wi-Fi coexistence has higer average UPT in both the outdoor and the indoor scenario. For 5%-tile, 50%-tile, and 95%-tile UPT in the indoor scenario, the performance gain is 39%, 150%, and 14%, respectively. Hence, it can be concluded that LAA system with appropriate LBT mechanism does not impact Wi-Fi services more than an additional Wi-Fi network on the same carrier. Furthermore, for the case of LAA/LAA coexistence, LAA systems could coexist with each other well via the LBT function.

It can be also observed that LAA has lower average latency than Wi-Fi for the case of LAA/Wi-Fi coexistence, and replacing one of the WiFi networks with LAA also reduces the average latency in the remaining WiFi network. .

Comparing the indoor and outdoor scenarios, the indoor scenario has better performance (higher UPT and lower latency) due to good interference isolation from other neighbor clusters.

Based on the analysis, we have the following observations.

Observation 1: In terms of both UPT and latency, LAA with LBT mechanism does not impact Wi-Fi services more than an additional Wi-Fi network on the same carrier.

Observation 2: With LBT mechanism, LAA systems belonging to two different operators can coexist with each other well.

Observation 3: LAA has lower average latency than Wi-Fi due to its more efficient physical layer design.

# Frame Structure Evaluation for LAA

For simplicity, only the LAA/Wi-Fi coexistence scenario is evaluated for the outdoor scenario in the following simulations.

# FBE v.s. LBE

In this section, the LAA frame structures of FBE and LBE based on Alt.2 (see section 2.2) are evaluated.

Table 4 Performance comparison of frame structures based on different LBT mechanisms

|  |  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| LBE | LAA | 1.15 | 14.03 | 46.91 | 17.45 | 59 | 293 | 2950 | 690.67 | 0.09 | 0.42 | 0.86 | 0.45 |
|  | Wi-Fi | 0.82 | 6.26 | 42.04 | 11.96 | 50 | 699 | 4031 | 1166.54 | 0.12 | 0.48 | 0.91 | 0.49 |
| FBE | LAA | 0.26 | 6.62 | 39.62 | 12.40 | 45 | 868 | 4271 | 1308.05 | 0.20 | 0.66 | 0.97 | 0.65 |
|  | Wi-Fi | 0.90 | 18.86 | 69.73 | 23.34 | 40 | 507 | 4180 | 1068.11 | 0.07 | 0.47 | 0.92 | 0.49 |

As shown in Table 4, LAA with LBE frame structure outperforms LAA with FBE frame structure in UPT since FBE has a fixed frame period and channel sensing time, thereby obtains less transmission opportunities. LBE, which has the similar channel access mechanism as Wi-Fi, can contend for the channel in a fair manner when competing with Wi-Fi. Furthermore, the latency of LAA with LBE frame structure is reduced by 82.27% in comparison with that with FBE frame structure, which is mainly because the LBE could flexibly contend for and occupy the unlicensed channel. The advantage of the flexible channel sensing is also reflected in the load factor, where LBE shows a lower load factor than FBE because the packets can be delivered sooner. Furthermore, different LAA frame structures have different impact on Wi-Fi performance. Wi-Fi can obtain more channel access opportunities in the FBE case than that in the LBE case, which results in better Wi-Fi performance in the FBE case. However, Wi-Fi performance has better performance only because FBE gives Wi-Fi unfair advantage and puts LAA in a disadvantageous position with poor performance. So FBE is considered as unfair for LAA compared to Wi-Fi, while LBE allows fair channel access between LAA and Wi-Fi.

Observation 4: Compared with FBE based frame structure, LBE based frame structure ensures fair channel access for LAA compared with Wi-Fi.

# LBE based Frame Structure

In this section, the performance of the two candidate frame structures based on LBE (Alt 1 and Alt 2 as described in section 2.2) is evaluated.

Table 5 Performance comparison of the LBE frame structure Alt.1 and Alt.2.

|  |  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| Alt. 1 | LAA | 0.58 | 7.87 | 34.90 | 11.92 | 78 | 383 | 3670 | 887.84 | 0.13 | 0.53 | 0.94 | 0.55 |
|  | Wi-Fi | 0.26 | 4.27 | 44.03 | 10.07 | 63 | 825 | 4813 | 1374.47 | 0.11 | 0.51 | 0.92 | 0.52 |
| Alt. 2 | LAA | 1.15 | 14.03 | 46.91 | 17.45 | 59 | 293 | 2950 | 690.67 | 0.09 | 0.42 | 0.86 | 0.45 |
|  | Wi-Fi | 0.82 | 6.26 | 42.04 | 11.96 | 50 | 699 | 4031 | 1166.54 | 0.12 | 0.48 | 0.91 | 0.49 |

As shown in the Table 5, both LAA and Wi-Fi achieve better performance with Alt.2 than with Alt.1. The reason is that Alt.1 employs a preamble to occupy the channel to make sure the data transmission begins and ends at subframe boundary. The preamble is overhead and reduces the system efficiency. It also reduces the transmission burst length for data. In contrast, Alt.2 allows the data transmission to begin and end in the middle of a subframe. Hence Alt.2 has minimum overhead from preamble transmission, and it can fully utilize the maximum channel occupancy time and obtain more data transmission opportunity.

Observation 5: Alt.2 frame structure performs better than Alt. 1 frame structure.

# Impact of Maximum Channel Occupancy Time

The maximum channel occupation time (MCOT) plays a key role in the fair co-existence of different networks. Table 6 gives the performance comparison of LAA/Wi-Fi coexistence scenario with different maximum channel occupation time.

Table 6 Performance comparison of maximum channel occupancy time

|  |  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| MCOT for LAA: 4ms | LAA | 1.15 | 14.03 | 46.91 | 17.45 | 59 | 293 | 2950 | 690.67 | 0.08 | 0.42 | 0.85 | 0.45 |
|  | Wi-Fi | 0.82 | 6.26 | 42.04 | 11.96 | 50 | 699 | 4031 | 1166.50 | 0.11 | 0.47 | 0.91 | 0.49 |
| MCOT for LAA: 8ms | LAA | 0.91 | 15.35 | 48.45 | 20.01 | 62 | 219 | 2481 | 576.48 | 0.08 | 0.36 | 0.89 | 0.41 |
|  | Wi-Fi | 0.48 | 4.53 | 32.06 | 9.58 | 65 | 860 | 5537 | 1543.40 | 0.20 | 0.62 | 0.96 | 0.62 |

From Table 6, it can be observed that with the increase of maximum channel occupancy time of LAA, the Wi-Fi UPT decreases while the LAA UPT increases. When the maximum burst length for LAA is large, LAA can enjoy continuous transmission for a longer duration with only a one-time (e)CCA before the burst transmission. This allows the data to be delivered faster in LAA, and at the same time causes more delay in Wi-Fi.

Observation 6: With a larger maximum channel occupancy time, LAA achieves better performance due to the reduced need for (e)CCA.

# Impact of Traffic Load

Table 7 shows the performance of LAA/Wi-Fi coexistence with different traffic load situations in the outdoor scenario.

Table 7 Performance comparison of LAA/Wi-Fi coexistence with different traffic load

|  |  | UPT(Mbps) |  |  |  | Latency (us) |  |  |  | Load Factor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. | 5% | 50% | 95% | Avg. |
| Packet arrival rate: 0.3 | LAA | 43.15 | 82.72 | 105.37 | 79.74 | 37 | 41 | 147 | 74.66 | 0.01 | 0.05 | 0.12 | 0.06 |
|  | Wi-Fi | 27.99 | 91.52 | 120.62 | 86.83 | 32 | 33 | 361 | 112.49 | 0.01 | 0.05 | 0.19 | 0.07 |
| Packet arrival rate: 0.6 | LAA | 1.15 | 14.03 | 46.91 | 17.45 | 59 | 293 | 2950 | 690.67 | 0.09 | 0.42 | 0.86 | 0.45 |
|  | Wi-Fi | 0.82 | 6.26 | 42.04 | 11.96 | 50 | 699 | 4031 | 1166.54 | 0.12 | 0.48 | 0.91 | 0.49 |
| Packet arrival rate: 0.9 | LAA | 0.45 | 2.91 | 24.04 | 6.96 | 110 | 666 | 4823 | 1312.06 | 0.26 | 0.84 | 0.98 | 0.77 |
|  | Wi-Fi | 0.04 | 1.45 | 17.75 | 3.63 | 136 | 1251 | 6156 | 1947.58 | 0.17 | 0.64 | 0.98 | 0.64 |

With increased traffic load, the latency of both LAA and Wi-Fi is dramatically increased since more nodes contend on the unlicensed carrier, resulting in lower UPT. It can be observed from the load factor that more data are accumulated in the buffer when the packet arrival rate increases. Furthermore, the Wi-Fi system is more sensitive to the traffic load, and its performance deteriorates faster than LAA with increased load.

Observation 7: LAA shows advantage over Wi-Fi in higher traffic load situation.

# Conclusion

In this contribution, we provided initial coexistence simulation results for DL only LAA. Based on the simulation results, we made the following observations:

Observation 1: In terms of both UPT and latency, LAA with LBT mechanism does not impact Wi-Fi services more than an additional Wi-Fi network on the same carrier.

Observation 2: With LBT mechanism, LAA systems belonging to two different operators can coexist with each other well.

Observation 3: LAA has lower average latency than Wi-Fi due to its more efficient physical layer design.

Observation 4: Compared with FBE based frame structure, LBE based frame structure ensures fair channel access for LAA compared with Wi-Fi.

Observation 5: Alt.2 frame structure performs better than Alt. 1 frame structure.

Observation 6: With a larger maximum channel occupancy time, LAA achieves better performance due to the reduced need for (e)CCA.

Observation 7: LAA shows advantage over Wi-Fi in higher traffic load situation.

# References

- 3GPP TR 36.889 V0.1.1, “Study on Licensed-Assisted Access to Unlicensed Spectrum,” November 2014.

# Appendix: Simulation Assumptions

Table 8 Coexistence evaluation assumptions

| Parameters | LAA | Wi-Fi |
| --- | --- | --- |
| Outdoor scenario layout | Based on SCE# 2a + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 10 m for min. distance between small cells of different operators |  |
| Indoor scenario layout | Based on SCE#3 + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 3 m minimal distance between small cells of different operators |  |
| Traffic model | FTP Model 3 file size: 0.5 Mbytes arrival rate: 0.3, 0.6, 0.9 |  |
| Network synchronization | For the same operator, the network is ideally synchronized Small cells of different operators are not synchronized |  |
| Total BS TX power | 18 dBm | 18 dBm |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized; adaptive stream | 2Tx2Rx in DL, Cross-polarized; Open-loop with 2 fixed streams |
| CCA-ED | -62 dBm | -62 dBm |
| Channel selection | None | None |
| Link adaptation | Close loop | Rate control - Minstrel algorithm |

Table 9 Wi-Fi additional simulation parameter

| Parameter |  | value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table (No 256QAM) |
| MPDU size |  | 1 ms |
| Max PPDU duration |  | 3 ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection only |
|  | RTS/CTS | None |
|  | Contention window | Per DCF |
| ACK Modeled |  | Ideal reception, but resource utilization and interference caused by ACK is modelled. |
| HARQ |  | Retransmission with maximum 8 times |

Table 10 LTE additional simulation parameter

| Parameter | value |
| --- | --- |
| Transmission schemes | # TM10, QPSK/16QAM/64QAM (no 256QAM) |
| Scheduler | Proportional fair |
| UE receiver | MMSE-IRC |
| Scheduling delay for LAA | 4 ms (Channel ON Status) |
| CQI/RI/PMI Feedback for LAA | Sub-band feedback with 1 ms feedback period and 1ms feedback delay |
| MAC HARQ | Number of maximum ReTx: 3; minimal interval of ReTx: 8 ms (Channel ON Status) |
| Outer loop link adaptation | Enabled |