3GPP TSG RAN WG1 Meeting #80		R1-150763

Athens, Greece, 9th – 13th February 2015

Source:	Ericsson, Qualcomm, LGE

Title:	Revised Proposed Correction to 36.213: Sidelink Resource Configuration

Agenda Item:	7.1

Document for:	Discussion and Decision

# Discussion

The draft CR for 36.213 [1] describes resource configuration for SA, data (SL-TM 2) and discovery as follows:

# 14 UE procedures related to Sidelink

A UE can be configured by higher layers with one or more PSSCH resource configuration(s). A PSSCH resource configuration can be for reception of PSSCH only, or for transmission and reception of PSSCH. The physical sidelink shared channel related procedures are described in subclause 14.1.

A UE can be configured by higher layers with one or more PSCCH resource configuration(s). A PSCCH resource configuration can be for reception of PSCCH only, or for transmission and reception of PSCCH and the PSCCH resource configuration is associated with either sidelink transmission mode 1 or sidelink transmission mode 2. The physical sidelink control channel related procedures are described in subclause 14.2.

A UE can be configured by higher layers with one or more PSDCH resource configuration(s). A PSDCH resource configuration can be for reception of PDSCH only, or for transmission and reception of PDSCH.  The transmissions of PSDCH according to a PSDCH resource configuration are associated with either sidelink discovery type 1 or sidelink discovery type 2B. The physical sidelink discovery channel related procedures are described in subclause 14.3.

Configuration of tx/rx pools was discussed by RAN1 while listing L1 RRC parameters [2]. In particular it was clarified that each pool can be either a tx, rx or tx+rx pool, as captured also in I1 of [2]: Relevant for Tx or Rx by a UE. Note 'Tx/Rx' refers to Tx and/or Rx.

Based on the above we propose to modify CR 36.213 according to the agreements.

# Text Proposal

# A UE can be configured by higher layers with one or more PSSCH resource configuration(s). A PSSCH resource configuration can be for reception of PSSCH or for transmission of PSSCH.The physical sidelink shared channel related procedures are described in subclause 14.1.

# A UE can be configured by higher layers with one or more PSCCH resource configuration(s). A PSCCH resource configuration can be for reception of PSCCH or for transmission of PSCCH and the PSCCH resource configuration is associated with either sidelink transmission mode 1 or sidelink transmission mode 2. The physical sidelink control channel related procedures are described in subclause 14.2.

# A UE can be configured by higher layers with one or more PSDCH resource configuration(s). A PSDCH resource configuration can be for reception of PSDCH or or for transmission of PDSCH.  The transmissions of PSDCH according to a PSDCH resource configuration are associated with either sidelink discovery type 1 or sidelink discovery type 2B. The physical sidelink discovery channel related procedures are described in subclause 14.3.

# References

- R1-145494	36.213 CR0492 (Rel 12, B) Introduction of D2D feature into 36.213	Motorola Mobility

- R1-145285, LS on RRC parameters for ProSe LTE D2D, RAN1