3GPP TSG RAN WG1 Meeting #80					  R1-150768

Athens, Greece, 9th – 13th February 2015

___________________________________________________________________Agenda item: 7.1

Source: LG Electronics

Title: Harmonized proposal for sidelink transmission power

Document for: Discussion and Decision

Proposal for the power limited case:

Capture in 36.213 that sidelink transmission power is constant within a subframe with the exception of power reduction applied to SSSS w.r.t. PSSSS.

Proposal for the maximum sidelink power:

Send LS to RAN4 (CC RAN2) to ask to consider reflecting the following RAN1 understanding in their specification. RAN1specification will specify only the maximum power parameters for each sidelink channel or signal with referring to RAN4 specification.

| Maximum power | Condition | How to determine the maximum power |
| --- | --- | --- |
| , | Out-coverage | Using calculation where is set to maxTxPower in ProsePreconfiguration IE |
|  | In-coverage | Using calculation where is set to P-Max in SIB1 |
|  | In-coverage | Using calculation where is set to discMaxTxPower in ProseDiscTxPowerInfo IE |
|  | Out-coverage | Using calculation where is set to maxTxPower in ProsePreconfiguration IE |
|  | In-coverage and triggered for communication | Using calculation where is set to P-Max in SIB1 |
|  | Otherwise | Using calculation where is set to discMaxTxPower in ProseDiscTxPowerInfo IE |
|  |  | where for a parameter specified by RAN4 |

Here, “triggered for communication” means that the SLSS/PSBCH transmission in the subframe is triggered because

- The UE is capable of communication and the eNB instructed to transmit SLSS/PSBCH by dedicated signalling, or

- The RSRP of the serving cell is less than the threshold configured for SLSS/PSBCH transmission, and the UE transmits PSCCH or PSSCH in a PSCCH period which includes the subframe for the SLSS/PSBCH transmission.