3GPP TSG RAN WG1 Meeting #80			R1-150835

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.3.1

Source: 	Samsung

Title: 	Conclusions from offline ad-hoc session on AI 7.2.4.3.1 (CSI-RS and feedback enhancements for EBF/FD-MIMO)

Document for:	Discussion and Decision

Two way forward proposals were discussed: R1-150817 and R1-150818. The following conclusions were reached:

- Companies are encouraged to further look at and continue discussion on the scenarios in R1-150817.

- High-level categories for potential CSI-RS and feedback enhancements were agreed in principle. These categories will be reflected in TR36.897 section 6.

- Some potential CSI-RS enhancements related to the number of NZP CSI-RS ports (for further evaluation) were agreed. These will be reflected in TR36.897 section 6.

The details are given below.

#### Potential Enhancements Targeting 2D Antenna Array

##### CSI-RS and feedback enhancements

R1-150817 	WF on PMI feedback scenarios for EBF/FD-MIMO		Samsung, Ericsson, CMCC

Proposal:

Precoding definition for EBF/FD-MIMO

- Precoding matrix/vector:

  - P1: wideband; updated less frequently

  - P2: subband or wideband; updated more frequently

  - P is a function of P1 and P2, applied to 1D or 2D antenna array

  - PMI(s) are to be specified w.r.t. the above definition

Scenarios for CSI feedback

- Scenario 1:

  - UE measures CSI-RS ports beamformed with P1

    - P1 transparent to UE

  - PMI report(s) for P2

- Scenario 2:

  - UE measures non-precoded 1- or 2-D CSI-RS ports

    - Note: P1 not applied to CSI-RS at eNB

  - PMI report(s) for P1 and P2

- Scenario 3:

  - UE measures both non-precoded 1- or 2-D CSI-RS ports (lower duty cycle) and CSI-RS beamformed with P1

  - PMI report(s) for P1 and P2

- Scenario 4:

  - UE measures non-precoded 1- or 2-D CSI-RS ports

    - Note: P1 not applied to CSI-RS at eNB

    - P1 indicated to UE

  - PMI report(s) for P2

Other scenarios are not precluded.

Conclusion: Companies are encouraged to further look at the scenarios in this WF. Continue discussion on this WF.

Offline agreement:

“High level categories” (sections for the TR) agreed in principle:

Potential CSI-RS and feedback enhancements

- Enhancements related to beamformed CSI-RS-based schemes

- Enhancements related to non-precoded CSI-RS-based schemes

- Enhancements related to schemes based on hybrid beamformed CSI-RS and non-precoded CSI-RS

- Enhancements related to non-codebook based CSI reporting for TDD

- Enhancements related to SRS

R1-150818	WF on CSI-RS enhancements				Samsung

Discussion:

- Maximum number of CSI-RS ports per CSI-RS resource for EBF/FD-MIMO >8:

  - Question regarding evidence for increasing the max number of CSI-RS resource per CSI-RS ports

    - Using multiple CSI-RS processes (without increasing the max number of CSI-RS ports per CSI-RS resource) is a standard-transparent alternative

    - Using multiple CSI-RS processes may complement increasing the max number of CSI-RS ports per CSI-RS resource

  - Arguments for increasing the max number of CSI-RS ports per CSI-RS resource

    - Large number of TXRUs

- 2D CSI-RS port structure (as opposed to linear-only)?

Offline agreement:

Potential CSI-RS enhancements related to the number of NZP CSI-RS ports for further evaluation (to be captured in the TR):

- Increasing the maximum number of NZP CSI-RS ports (>8) for >8 TXRUs per CSI process

- Number of non-precoded CSI-RS ports per CSI-RS resource which is a multiple of 2, e.g. 10

  - Note: one CSI-RS port may be mapped onto one or more than one TXRUs.

- Flexible number of beamformed CSI-RS ports per CSI-RS resource

##### DM-RS enhancements

##### RRM measurement enhancements

##### Others

Including open-loop MIMO, TDD specific issues.