3GPP TSG RAN WG1 Meeting #80		R1-150545

Athens, Greece, 9th – 13th February 2015

Source:	QUALCOMM Incorporated

Title:	NAIC with RRS

Agenda item:		6.3.2

Document for:	Discussion

# 1	Introduction

The pre-decoding NAIC with Restricted Resource Subframe (RRS) has been discussed in [1]-[3]. The link level performance was presented in [4]. This contribution will provide its system level performance.

# 2	Simulation assumptions

The system level simulation assumptions to evaluate NAIC performance are provided in Table 1.

Table 1: System level simulation assumptions

| Parameters | Values and comments |
| --- | --- |
| Carrier Frequency | 2000 MHz |
| Cell Layout | 57 cell hexagonal (19 NodeB, 3 sectors per Node B with wrap-around) |
| Propagation Channel Type | PedA 3km/h |
| LPN cell | 4 LPN per Macro and 0 LPN(Macro Only) |
| Inter-site distance | 500 m |
| Dropping criteria for LPNs | LPNs are randomly and uniformly distributed within a macro cell. |
| Dropping criteria for UEs | 1/2 hotspot, 16 per Macro area |
| CIO of LPN | 0dB,3dB, 6dB,9dB |
| Maximum Tx Power of Macros/LPNs | 43dBm/30dBm |
| Overhead channel | Macros: 20% of Max Tx Power for baseline & 20% for NAIC(no extra power for common HS-SCCH) LPNs: 20% of Max Tx Power |
| Traffic | Full Buffer |
| UE receiver | Dual Rx Ideal interference cancellation, pre-decoding NAIC |
| Scheduler on Macro/LPN | Proportional fair scheduler (baseline) smart scheduler (for NAIC) |
| HSDPA channel settings | HS-PDSCH: targeting 10% BLER after 1st Tx, up to 15 SF-16 OVSF to schedule per TTI |

2.1. Smart Scheduling for NAIC

The UEs can perform NAIC in sub-frames in which NAIC is enabled. Let us call such sub-frames to be NAIC sub-frames. Smart scheduling might be introduced to improve the scheduling rate for UEs with larger NAIC gain on NAIC sub-frames, and reduce their scheduling rate on normal sub-frames. For the basic proportional fair (PF) scheduler, the metric of user’s priority is written as P=rr where r is the instantaneously supported TBS and r is the filtered TBS. Here, we record the CQI on normal and NAIC sub-frames as CQInormal and CQINAIC. The value r of normal and NAIC sub-frames was estimated separately from CQInormal and CQINAIC (rnormal , rNAIC) respectively.

Although the basic form of PF scheduler P=rr was used, r was used separately by rnormal and rNAIC. Hence,  rNAIC> r > rnormal, meaning PNAIC>Pnormal. The larger gain from NAIC, the larger gap between PNAIC and Pnormal. The scheduling rate on NAIC sub-frames is then increased while scheduling rate on normal sub-frames is decreased.

# 3	Simulation results

3.1. Ideal NAIC Performance

## It is important to understand the upper-bound of NAIC performance. To obtain such bound, in simulations, we assume the interferers are perfectly cancelled in NAIC sub-frames, and the scheduled TBS is less than 10k for macros. The baseline IC-capable receiver was discussed in [5]-[6]. The receiver performance is evaluated at the different CIOs and NAIC sub-frame pattern. Let us denote NAIC sub-frame pattern by b/a meaning there are b NAIC sub-frames out of total a sub-frames.

## To evaluate the performance gain, we take the performance of the baseline IC-capable receiver at 0dB CIO as the baseline. The system throughput gain is evaluated at median and different throughput tails (1%, 2% and 5%). Tables 2 – 4 summarize the simulation results of NAIC performance when one interferer, three interferers and five interferers are perfectly cancelled, respectively. As expected, the system throughput gain increases when more interferers are perfectly cancelled out and the CIO increases. In particular, when 5 interferers are ideally cancelled, the total system throughput gain could achieve up to 57% in median throughput and 68% in tail throughput at 9dB CIO. However, it is also noted that the macro throughput degrades as the percentage of NAIC sub-frames increases due to the restricted TBS at the macros.

## Table 2: Perfectly cancelling one-interferer

| CIO | NAIC Pattern (b/a) | Mean macro Throughput (kbps) | LPN Throughput (kbps) |  | Total System Throughput (kbps) |  |  |  | Total System Throughput Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mean | 5% tail | median | 5% tail | 2% tail | 1% tail | median | 5% tail | 2% tail | 1% tail |
| 0 | 0/6 | 626 | 3754 | 894 | 730 | 278 | 224 | 197 |  |  |  |  |
| 0 | 6/6 | 376 | 4448 | 1068 | 421 | 217 | 174 | 157 | -42% | -22% | -23% | -20% |
| 0 | 4/6 | 477 | 4241 | 1016 | 549 | 245 | 206 | 172 | -25% | -12% | -8% | -13% |
| 0 | 2/6 | 561 | 4003 | 965 | 652 | 269 | 222 | 196 | -11% | -3% | -1% | 0% |
| 3 | 0/6 | 713 | 3054 | 593 | 864 | 312 | 258 | 230 | 18% | 12% | 15% | 17% |
| 3 | 6/6 | 421 | 3659 | 782 | 507 | 255 | 202 | 183 | -31% | -8% | -10% | -7% |
| 3 | 4/6 | 542 | 3471 | 700 | 663 | 274 | 227 | 196 | -9% | -1% | 1% | -1% |
| 3 | 2/6 | 638 | 3263 | 634 | 772 | 305 | 256 | 226 | 6% | 10% | 14% | 15% |
| 6 | 0/6 | 860 | 2366 | 328 | 989 | 344 | 265 | 219 | 35% | 24% | 18% | 11% |
| 6 | 6/6 | 498 | 2870 | 453 | 647 | 294 | 241 | 214 | -11% | 6% | 7% | 9% |
| 6 | 4/6 | 647 | 2704 | 395 | 804 | 324 | 265 | 229 | 10% | 17% | 18% | 16% |
| 6 | 2/6 | 765 | 2528 | 351 | 913 | 343 | 281 | 225 | 25% | 24% | 25% | 14% |
| 9 | 0/6 | 1050 | 1943 | 208 | 1080 | 281 | 164 | 128 | 48% | 1% | -27% | -35% |
| 9 | 6/6 | 597 | 2375 | 298 | 792 | 329 | 224 | 172 | 9% | 18% | 0% | -13% |
| 9 | 4/6 | 782 | 2229 | 262 | 929 | 314 | 205 | 161 | 27% | 13% | -8% | -18% |
| 9 | 2/6 | 930 | 2076 | 236 | 1030 | 306 | 180 | 147 | 41% | 10% | -20% | -26% |

## Table 3: Perfectly cancelling three interferers

| CIO | NAIC Pattern (b/a) | Mean macro Throughput (kbps) | LPN Throughput (kbps) |  | Total System Throughput (kbps) |  |  |  | Total System Throughput Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mean | 5% tail | median | 5% tail | 2% tail | 1% tail | median | 5% tail | 2% tail | 1% tail |
| 0 | 0/6 | 626 | 3754 | 894 | 730 | 278 | 224 | 197 |  |  |  |  |
| 0 | 6/6 | 380 | 5354 | 1521 | 416 | 247 | 202 | 182 | -43% | -11% | -10% | -8% |
| 0 | 4/6 | 493 | 4868 | 1347 | 557 | 269 | 225 | 197 | -24% | -3% | 0% | 0% |
| 0 | 2/6 | 572 | 4323 | 1113 | 662 | 281 | 229 | 210 | -9% | 1% | 2% | 7% |
| 3 | 0/6 | 713 | 3054 | 593 | 864 | 312 | 258 | 230 | 18% | 12% | 15% | 17% |
| 3 | 6/6 | 425 | 4431 | 1110 | 505 | 285 | 242 | 205 | -31% | 3% | 8% | 4% |
| 3 | 4/6 | 557 | 4012 | 957 | 687 | 311 | 257 | 233 | -6% | 12% | 15% | 18% |
| 3 | 2/6 | 649 | 3537 | 781 | 795 | 320 | 267 | 241 | 9% | 15% | 19% | 22% |
| 6 | 0/6 | 860 | 2366 | 328 | 989 | 344 | 265 | 219 | 35% | 24% | 18% | 11% |
| 6 | 6/6 | 502 | 3511 | 760 | 679 | 329 | 287 | 254 | -7% | 18% | 28% | 29% |
| 6 | 4/6 | 663 | 3157 | 636 | 865 | 377 | 296 | 266 | 19% | 36% | 32% | 35% |
| 6 | 2/6 | 777 | 2761 | 495 | 951 | 369 | 306 | 273 | 30% | 33% | 37% | 39% |
| 9 | 0/6 | 1050 | 1943 | 208 | 1080 | 281 | 164 | 128 | 48% | 1% | -27% | -35% |
| 9 | 6/6 | 600 | 2927 | 531 | 904 | 372 | 311 | 271 | 24% | 34% | 39% | 38% |
| 9 | 4/6 | 802 | 2618 | 436 | 1028 | 425 | 307 | 247 | 41% | 53% | 37% | 26% |
| 9 | 2/6 | 944 | 2280 | 323 | 1100 | 372 | 243 | 184 | 51% | 34% | 8% | -6% |

## Table 4: Perfectly cancelling five interferers

| CIO | NAIC Pattern (b/a) | Mean macro Throughput (kbps) | LPN Throughput (kbps) |  | Total System Throughput (kbps) |  |  |  | Total System Throughput Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mean | 5% tail | median | 5% tail | 2% tail | 1% tail | median | 5% tail | 2% tail | 1% tail |
| 0 | 0/6 | 626 | 3754 | 894 | 730 | 278 | 224 | 197 |  |  |  |  |
| 0 | 6/6 | 382 | 5949 | 1854 | 407 | 259 | 229 | 208 | -44% | -7% | 2% | 6% |
| 0 | 4/6 | 500 | 5280 | 1578 | 555 | 289 | 251 | 229 | -24% | 4% | 12% | 16% |
| 0 | 2/6 | 580 | 4531 | 1288 | 666 | 291 | 246 | 226 | -9% | 5% | 10% | 15% |
| 3 | 0/6 | 713 | 3054 | 593 | 864 | 312 | 258 | 230 | 18% | 12% | 15% | 17% |
| 3 | 6/6 | 427 | 4953 | 1378 | 502 | 299 | 269 | 240 | -31% | 8% | 20% | 22% |
| 3 | 4/6 | 564 | 4371 | 1156 | 681 | 339 | 287 | 251 | -7% | 22% | 28% | 28% |
| 3 | 2/6 | 657 | 3724 | 894 | 812 | 331 | 278 | 249 | 11% | 19% | 24% | 27% |
| 6 | 0/6 | 860 | 2366 | 328 | 989 | 344 | 265 | 219 | 35% | 24% | 18% | 11% |
| 6 | 6/6 | 503 | 3934 | 943 | 689 | 341 | 313 | 283 | -6% | 23% | 40% | 44% |
| 6 | 4/6 | 670 | 3451 | 756 | 909 | 396 | 347 | 315 | 24% | 43% | 55% | 60% |
| 6 | 2/6 | 785 | 2917 | 565 | 983 | 389 | 322 | 289 | 35% | 40% | 44% | 47% |
| 9 | 0/6 | 1050 | 1943 | 208 | 1080 | 281 | 164 | 128 | 48% | 1% | -27% | -35% |
| 9 | 6/6 | 601 | 3295 | 708 | 979 | 399 | 362 | 317 | 34% | 44% | 62% | 61% |
| 9 | 4/6 | 808 | 2875 | 563 | 1114 | 465 | 377 | 326 | 53% | 68% | 68% | 66% |
| 9 | 2/6 | 952 | 2418 | 401 | 1147 | 423 | 306 | 229 | 57% | 52% | 37% | 16% |

3.2. Pre-decoding NAIC Performance

## It is assumed that the interferers only use QPSK modulation and a restricted number of codes. In addition, UEs perform realistic interference cancellation. Table 5 summarizes the throughput performance as well as the performance gain of the pre-decoding NAIC with RRS. It is observed that, for the same NAIC pattern, the median throughput gain increase as CIO increases. In addition, the throughput tail is also improved when more sub-frames are scheduled with RRS. However, the median throughput gain decreases when the number of NAIC sub-frames increase since more NAIC sub-frames with RRS could degrade the macro throughput. Hence, it is reasonable to have some total system throughput gain trade-off among median throughput and tail throughput. Interestingly, if we wish to have a good system throughput gain in both median and tails, NAIC with RRS might not be needed. In fact, the simulation results suggest that the baseline scheme with 6dB CIO has the better trade-off performance gain than any NAIC with RRS.

## Table 5: Pre-decoding NAIC with RRS

| CIO | NAIC Pattern (b/a) | Mean macro Throughput (kbps) | LPN Throughput (kbps) |  | Total System Throughput (kbps) |  |  |  | Total System Throughput Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mean | 5% tail | median | 5% tail | 2% tail | 1% tail | median | 5% tail | 2% tail | 1% tail |
| 0 | 0/6 | 626 | 3754 | 894 | 730 | 278 | 224 | 197 |  |  |  |  |
| 0 | 6/6 | 404 | 4321 | 1121 | 442 | 262 | 238 | 223 | -39% | -6% | 6% | 13% |
| 0 | 4/6 | 485 | 4125 | 970 | 557 | 262 | 238 | 215 | -24% | -6% | 6% | 9% |
| 0 | 2/6 | 558 | 3942 | 951 | 650 | 261 | 220 | 189 | -11% | -6% | -2% | -4% |
| 3 | 0/6 | 713 | 3054 | 593 | 864 | 312 | 258 | 230 | 18% | 12% | 15% | 17% |
| 3 | 6/6 | 461 | 3526 | 787 | 540 | 298 | 271 | 258 | -26% | 7% | 21% | 31% |
| 3 | 4/6 | 555 | 3347 | 645 | 668 | 299 | 264 | 249 | -9% | 8% | 18% | 27% |
| 3 | 2/6 | 638 | 3197 | 603 | 778 | 298 | 245 | 218 | 7% | 7% | 10% | 11% |
| 6 | 0/6 | 860 | 2366 | 328 | 989 | 344 | 265 | 219 | 35% | 24% | 18% | 11% |
| 6 | 6/6 | 564 | 2751 | 498 | 724 | 347 | 310 | 282 | -1% | 25% | 39% | 43% |
| 6 | 4/6 | 679 | 2587 | 396 | 822 | 343 | 286 | 259 | 13% | 24% | 28% | 32% |
| 6 | 2/6 | 778 | 2469 | 343 | 919 | 339 | 277 | 229 | 26% | 22% | 24% | 16% |
| 9 | 0/6 | 1050 | 1943 | 208 | 1080 | 281 | 164 | 128 | 48% | 1% | -27% | -35% |
| 9 | 6/6 | 691 | 2275 | 290 | 830 | 348 | 255 | 215 | 14% | 25% | 14% | 9% |
| 9 | 4/6 | 836 | 2125 | 226 | 954 | 298 | 206 | 165 | 31% | 7% | -8% | -16% |
| 9 | 2/6 | 953 | 2026 | 220 | 1024 | 290 | 179 | 144 | 40% | 4% | -20% | -27% |

# 3	Conclusions

## This contribution provides the system level simulation results for pre-decoding NAIC with RRS under both ideal and realistic interference cancellation. The ideal NAIC could provide a system throughput gain of 57%-68% assuming 5 interferers are perfectly cancelled. However, under the realistic NAIC with RRS, the system throughput gain is limited. In fact, the simulation results suggest that the baseline receiver could achieve better gain (in both median and tail throughput) than RRS-based NAIC schemes when operating at 6dB CIO. As a result, NAIC with RRS does not attract much interest for further study.

# 4	References

- R1-140686, RRS pattern based coordinated scheduling for new IC UEs, Huawei, HiSilicon.

- R1-140685, RRS pattern based coordinated scheduling for legacy IC UEs, Huawei, HiSilicon.

- R1-135685, Pre-decoding IC results with fixed MCS interference, Huawei, HiSilicon.

- R1-135782, Link Performance of RRS in HetNet, QUALCOMM Incorporated.

- R1-133837, Performance of Network Assisted IC in a Hetnet Framework, QUALCOMM Incorporated.

- TR 25.800, Technical Report on UMTS Heterogeneous Networks.