3GPP TSG RAN WG1 Meeting #80bis	        		  R1-151291

Belgrade, Serbia, 20th - 24th April, 2015

Agenda item:	7.2.1.2

Source: 	Marvell

Title:		        Performance of Coverage Enhancement Techniques for MTC PDSCH

Document for:	Discussion and Decision

# Introduction

We present performance results for the PDSCH channel for a Rel. 13 MTC Low-Complexity UE in Coverage Enhancement [1]. In order to achieve an overall 15 dB coverage gain for “Coverage Enhanced Mode” (i.e., to achieve an MCL of 155.7 dB) the downlink PDSCH cell-edge SNR is reduced by 10.3 dB to -14.3 dB [2]. The Rel. 13 MTC UE is expected to be able to successfully operate at this SNR despite having only a single receive antenna – even in very slow fading channels such as in 1 Hz Doppler, where lack of time diversity and rx antenna diversity significantly degrades performance. The MTC UE also suffers from a lack of frequency diversity because of the narrow 1.4 MHz bandwidth, unless frequency hopping is employed.

We consider several techniques

- Cross-Subframe Channel Estimation – We consider both a baseline 1 ms FIR time averaging for channel estimation and first-order IIR averaging over multiple subframes. The IIR forget factor is chosen to provide processing gain and group delay (DC component) corresponding to 1/2/4/8 ms FIR averaging.

- Frequency Hopping – We consider that the MTC 1.4 MHz BW containing 6 PRBs can hop to different parts of the 10 MHz legacy System Bandwidth, thus providing frequency diversity. The Frequency Hopping simulations presented here consider that the hopping cycles through different 6 PRB chunks, without overlap between the chunks.

- We consider discontinuous transmission with a 20 ms cycle. This provides substantial time diversity.

Performance results for these techniques are considered in the next section. In Section 3 we consider the effect on multi-subframe channel estimation that occurs due to residual frequency tracking error.

# PDSCH Performance in Coverage Enhancement

We examine low-complexity MTC performance here for the following case:

- EPA & ETU channels with 1 Hz Doppler

- SNR = 14.3 dB – i.e., full coverage enhancement

- 1 Rx antenna

- 50 Hz residual frequency tracking error

The full simulation assumptions are shown in Appendix 1 and are in accord with those defined in [3].

The table below shows the number of repetitions needed for 10% BLER and 1% BLER for the EPA & ETU cases. The full performance graphs are shown in Appendix 2. We consider the following cases:

- Baseline: 1 ms FIR time-averaging for channel estimation

- Multi-SF Channel Estimation: Channel estimation IIR (first-order) time-averaging corresponding to FIR averaging over 1, 2, 4, and 8 ms.

- Frequency Hopping: Frequency hop rates of 4 ms, 8, ms and 16 ms are considered (and are paired with IIR averaging over 1, 2, and 4 ms, respectively)

- Discontinuous Transmission: 1 ms FIR channel estimation time-averaging with 20 ms discontinuous transmission

Table 1: Required number of transmissions to reach {1%, 10%} target BLERs for PDSCH in Coverage Enhanced Mode (SNR=-14.3 dB) for the EPA & ETU 1 Hz channels, with TBS=152 bits (MCS 0), 1 Rx antenna, and 50 Hz residual frequency error.

| Case | EPA1 10% BLER | ETU1 10% BLER | EPA1 1% BLER | ETU1 1% BLER |
| --- | --- | --- | --- | --- |
| - Baseline: 1 ms Ch. Est FIR Avg | 184 | 141 | >300 | >300 |
| - IIR 1 ms Ch. Est. Avg | 186 | 139 | >300 | >300 |
| - IIR 2 ms Ch. Est. Avg | 135 | 99 | >300 | 305 |
| - IIR 4 ms Ch. Est. Avg | 112 | 88 | >300 | 259 |
| - IIR 8 ms Ch. Est. Avg | 142 | 110 | >300 | 291 |
| - Freq. Hop 4 ms + IIR 1 ms Avg | 98 | 68 | >300 | 148 |
| - Freq. Hop 8 ms + IIR 2 ms Avg | 72 | 50 | 243 | 116 |
| - Freq. Hop 16 ms + IIR 4 ms Avg | 67 | 50 | 232 | 100 |
| - DTX 20 ms + 1 ms FIR Avg | 69 | 66 | 116 | 98 |

As expected the performance of the baseline 1 ms FIR channel estimation averaging is very similar to the corresponding 1 ms first order IIR averaging. In addition we see:

- We obtain substantial gain from multi-subframe channel estimation – but only up to a 4 ms averaging window. Performance degrades for the 8 ms window. We will see in the next section that this is due to the residual frequency error.

- We obtain substantial gain from frequency hopping because of the increased frequency diversity. The gain is larger for the ETU channel which has more multipath, i.e., it is more frequency selective.

- The most spectrally efficient technique considered was the 20 ms discontinuous transmission. This provided more gain than the combination of frequency hopping and multi-subframe channel estimation – especially for the EPA channel, where frequency hopping provides less frequency diversity gain. Thus the time diversity that can be achieved by spreading the transmission out over time can result in a significant reduction in transmission resources (repetitions) needed. One disadvantage of this technique is increased latency. Another potential disadvantage is increased power usage, due to the UE remaining on for a longer time. However, we can consider having the UE sleep in between transmissions.

- We are generally unable to achieve 1% BLER for MCS 0 (TBS = 152 bits) with less than 250 repetitions without additional frequency or time diversity obtained through frequency hopping or discontinuous transmission.

Note that the Table and Figures do not take into account the overhead for frequency switching that will need to be implemented in practice when employing frequency hopping. This will probably be on the order of 1 slot or less for each frequency hop.

We note that there is some degradation in performance due to the fact that the channel estimation IIR filter must be reset with each frequency hop. This degradation can be minimized by optimizing the IIR parameters during the reset transient, but this was not considered here.

Observation 1: Multi-SF channel estimation can provide significant but limited gains in performance.

Observation 2: Frequency hopping can provide substantial gains in performance, particularly for frequency selective channels.

Observation 3: Substantial improvements in spectral efficiency can be achieved by spreading out the PDSCH transmissions in time. This technique can be considered for applications that can tolerate the increased latency and potential impact to power usage. The impact to battery life can be mitigated by having the UE sleep between transmissions.

Observation 4: Continuous transmission without frequency hopping requires an extremely large number of transmissions to achieve 1% BLER for the MCS 0 (TBS=152 bits) case for low-cost MTC very slow-fading scenarios in coverage enhanced mode.

# Residual Frequency Error

In this section we consider the effect of residual frequency error on multi-subframe channel estimation performance. The table below shows results for first order IIR channel estimation time averaging, considering 20, 50, and 100 Hz residual frequency error. A figure with the full performance graphs is shown in Appendix 3.

Table 2: Required number of transmissions to reach 10% target BLER for PDSCH in Coverage Enhanced Mode (SNR=-14.3 dB) for the EPA 1 Hz channel, with TBS=152 bits (MCS 0) and 1 Rx antenna, and residual frequency error = 20, 50, and 100 Hz.

| Case | Freq. Error 20 Hz | Freq. Error 50 Hz | Freq. Error 100 Hz |
| --- | --- | --- | --- |
| - IIR 1 ms Ch. Est. Average | 181 | 186 | 189 |
| - IIR 2 ms Ch. Est. Average | 123 | 135 | 158 |
| - IIR 4 ms Ch. Est. Average | 90 | 112 | 175 |
| - IIR 8 ms Ch. Est. Average | 73 | 142 | 259 |

We see that the residual frequency error has a substantial effect on performance of multi-subframe channel estimation. In the case of 20 Hz residual frequency error best results were achieved using 8 ms multi-SF channel estimation. However, when the residual frequency error increases to 50 Hz, using 8 ms multi-SF channel estimation degrades performance below that of 2 ms multi-SF channel estimation. Furthermore, if the residual frequency error reaches 100 Hz, then best results are obtained using 2 ms multi-SF channel estimation, with degradation beginning at 4 ms averaging.

Observation 5: Residual frequency error has a significant effect on the performance of Multi-SF Channel Estimation. It is thus desirable to minimize the residual frequency error even in very poor SNR conditions that occur when the UE operates in coverage enhanced mode.

# Conclusion

In this contribution we provided PDSCH performance results for MTC UEs in coverage-enhanced mode. Based on the results we have made the following observations.

Observation 1: Multi-SF channel estimation can provide significant but limited gains in performance.

Observation 2: Frequency hopping can provide substantial gains in performance, particularly for frequency selective channels.

Observation 3: Substantial improvements in spectral efficiency can be achieved by spreading out the PDSCH transmissions in time. This technique can be considered for applications that can tolerate the increased latency and potential impact to power usage. The impact to battery life can be mitigated by having the UE sleep between transmissions.

Observation 4: Continuous transmission without frequency hopping requires an extremely large number of transmissions to achieve 1% BLER for the MCS 0 (TBS=152 bits) case for low-cost MTC very slow-fading scenarios in coverage enhanced mode.

Observation 5: Residual frequency error has a significant effect on the performance of Multi-SF Channel Estimation. It is thus desirable to minimize the residual frequency error even in very poor SNR conditions that occur when the UE operates in coverage enhanced mode.

# References

- RP-150492, “Revised WI: Further LTE Physical Layer Enhancements for MTC”

- 3GPP TR 36.888, “Study on provision of low cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE,” V12.0.0, June, 2013

- R1-144513, “Simulation Assumptions for Reference Cases for MTC”, Nokia Networks, 3GPP RAN1 #78bis, Oct. 2014

# Appendix 1: Simulation Assumptions

The simulation assumptions are based on those defined in [3] and the below Table.

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| UE receiver bandwidth | 1.4 MHz |
| Frame structure | FDD |
| Carrier frequency | 2.0 GHz for FDD |
| Antenna configuration | 2x1 (low correlation) |
| Channel model | EPA, ETU |
| Doppler spread | 1 Hz |
| Transport block size | 152 bits (MCS 0) |
| Transmission Mode | TM2 |
| Modulation | QPSK |
| Number of PRBs | 6 |
| Frequency Hopping | Cycling through the legacy 10 MHz system BW (no overlap between 6 PRB chunks). Frequency Hop Rate = 4, 8, 16 ms |
| Repetition interval | Continuous and 20 ms period Discontinuous transmission |
| Legacy PDCCH region | 3 OFDM Symbols |
| Frequency tracking error | 50 Hz (compared in Sec. 3 to 20 Hz and 100 Hz) |
| Timing error | Ideal |
| Retransmissions | Repetitions (Incremental Redundancy has no effect for very small TBS as in MCS 0) |
| Channel estimation | Practical CRS-based channel estimation with time averaging – both 1 ms FIR filtering and 1/2/4/8 ms IIR filtering are considered |

# Appendix 2: PDSCH Simulation Results: BLER vs. Number of Transmissions

The 2 graphs shown below cover the cases

- EPA & ETU 1 Hz channels

- PDSCH MCS 0 (TBS = 152 bits)

- 1 receive antenna

- Enhanced Coverage Mode SNR = -14.3 dB

- 50 Hz Residual Frequency Error

The legends can be understood as follows

- TX01 / TX20 – indicates continuous / 20 ms discontinuous transmission, respectively.

- FHOff indicates that frequency hopping is off; FH-04/08/16 refers to frequency hopping with rate 4, 8, and 16 ms, respectively.

- 4RSEq – refers to FIR 1 ms averaging, i.e., 4 equal tap weights over 4 CRS reference symbols (4RS)

  - The combination of TX01 (continuous transmission) and 4RSEq is the baseline.

- IIR01/02/04/08 – refers to first order IIR time averaging with forget factor that corresponds to an FIR sliding window over 1, 2, 4, 8 ms, respectively.

Figure 1: BLER Performance as a function of number of transmissions for PDSCH MCS 0 (TBS=152 bits) in an EPA 1 Hz channel with 1 Rx antenna, SNR=-14.3 dB, and 50 Hz residual frequency error. Legend is explained above.

Figure 2: BLER Performance as a function of number of transmissions for PDSCH MCS 0 (TBS=152 bits) in an ETU 1 Hz channel with 1 Rx antenna, SNR=-14.3 dB, and 50 Hz residual frequency error. Legend is explained above.

# Appendix 3: Effect of Residual Frequency Error

In the graph below we illustrate the effect of residual frequency error for the case of an EPA 1 Hz channel, PDSCH TBS = 152 bits (MCS 0) with 1 receive antenna and SNR = -14.3 dB (coverage enhanced mode). The case simulated uses continuous transmission without frequency hopping.

- IIR01/02/04/08 refers to channel estimation with first order IIR averaging with forget factor chosen to correspond to a sliding window FIR filter of length 1, 2, 4, and 8 ms, respectively

- FErr020/050/100 refers to residual frequency error of 20, 50, and 100 Hz, respectively

Figure 3: BLER Performance as a function of number of transmissions for PDSCH MCS 0 (TBS=152 bits) in an EPA 1 Hz channel with 1 Rx antenna and SNR=-14.3 dB. Results are show for residual frequency error of 20, 50, and 100 Hz (FErr020 / 050 / 100). Channel estimation is considered with first order IIR filtering that corresponds to a sliding window of length 1, 2, 4, and 8 ms (IIR01 / 02 / 04 / 08).