3GPP TSG RAN WG1 Meeting #80bis	R1-151641

Belgrade, Serbia, 20th – 24th April 2015

Agenda item:	7.2.5.2.1

Source: 	Samsung

Title: 	Discussion on vertical channel direction variability

Document for:	Discussion and Decision

# Introduction

This contribution presents analysis results of vertical channel direction variability in terms of vertical PMI (V-PMI) change in time and frequency domain.

We consider a 2D antenna array at eNB with (MTXRU, N, P) = (4, 4, 2). The PMI codebook is assumed to be Kronecker product of azimuth and elevation codebooks, which respectively are Rel. 10 8-Tx codebook and 4-Tx DFT codebook with oversampling factor 2. The V-PMI indices for 570 users in 57 cells are collected across 10 subbands (with subband size 5 PRBs) and 101 subframes, where subframes are subsampled by 10. The analysis of the collected V-PMI indices is provided below. The rest of the simulation settings are summarized in Table 1.

Table 1: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Channel model | 3D-UMa, Scenario 1 (500m) and 2 (200m) 3D-UMi, Scenario 1 (2GHz) and 2 (3.5GHz) |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (MTXRU, N, P) | (4, 4, 2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| PMI Codebook | Kronecker Product Codebook: Azimuth codebook: Rel. 10, 8-Tx Elevation codebook: 4-Tx DFT with over-sampling factor 2 |

# Distribution of V-PMI indices

The distribution of the eight V-PMI indices for all 570 UEs over the subbands and the subframes is shown in Figure 1. The distribution suggests that all eight V-PMI indices contribute to the aggregate PMI distribution. It is therefore important to consider all eight V-PMI indices.

Figure 1: Distribution of PMI indices

Observation 1: all eight V-PMI indices contribute to the aggregate PMI distribution for both UMa and UMi channel models in both scenarios 1 and 2.

In order to study the UE-specific variations in V-PMI, we next present distribution of PMI indices for eight UEs selected out of 570 UEs over the subbands and the subframes. Their distributions are shown below in Figure 2 through Figure 5. It is evident that for those selected UEs, V-PMI distribution is rich.

Figure 2: UMa, Scenario 1: Distribution of V-PMI Indices for 8 UEs

Figure 3: UMa, Scenario 2: Distribution of V-PMI Indices for 8 UEs

Figure 4: UMi, Scenario 1: Distribution of V-PMI Indices for 8 UEs

Figure 5: UMi, Scenario 2: Distribution of V-PMI Indices for 8 UEs

Observation 2: For some UEs, V-PMI distribution is rich for both UMa and UMi channel models in both scenarios 1 and 2.

# Frequency Domain Variability

The CDF of the standard deviation of user-specific V-PMI indices collected across the subbands and over the subframes is shown in Figure 6. For a given subframe, the standard deviation is defined as follows:

σ=mino∈{0,1,…,7}σ(o),

where the minimization is over the offset parameter o∈{0,1,…,7}, and for a given offset o, σ(o) is defined as

σo=1Nk=1NiVk,o-iV(o)2,

where N is the number of subbands, iVk,o=mod(iVk)+o,  8 is the V-PMI index for subband k after necessary cyclic shift because of adding offset o, and iV(o)=1Nk=1NiVk,o is the average V-PMI index across subband. The minimization over the offset parameter o is considered in order to properly handle the cases in which the difference of V-PMI  indices of two subbands is large, for example V-PMI indices 0 and 7, but the difference of their corresponding phases in the DFT vectors is small, for example exp-j2π8, i.e., the two DFT beams are actually close physically (as shown in Figure 8 below). For such cases, the variance should be small. The above-mentioned method is one way to achieve this. Finally, for each user, σ values for multiple subframes are averaged, and the average values for all 570 users are considered to obtain the CDF shown in Figure 6. From the figure, we can obtain the following Table 2, which shows the percentage of UEs that show V-PMI variations across subbands.

Table 2: V-PMI variation across subbands

| Channel Model | Percentage of UEs showing V-PMI variations across subbands |
| --- | --- |
| UMa, Scenario 1 | 58% |
| UMa, Scenario 2 | 70% |
| UMi, Scenario 1 | 71% |
| UMi, Scenario 2 | 73% |

Observation 3: More than 70% (60%) of UEs have V-PMI variation in the frequency domain for UMa Scenario 2, UMi Scenarios 1&2 (UMa Scenario 1). In particular, about 8-13% of UEs have mean V-PMI variation of at least one PMI index in half of the subbands, which corresponds to the phase difference of exp⁡-j2π8 between two consecutive DFT vectors and the standard deviation 0.5 (shown by double arrow in Figure 6).

Figure 6: CDF of V-PMI Change Across Subbands (Standard Deviation)

# Time Domain Variability

The CDF of average duration for which V-PMI index changes from one value to another is shown in Figure 7. Similar to the frequency domain variability, the percentage of UEs that show V-PMI variations in time is given by Table 3.

Table 3: V-PMI variation in time

| Channel Model | Percentage of UEs showing V-PMI variations in time |
| --- | --- |
| UMa, Scenario 1 | 58% |
| UMa, Scenario 2 | 69% |
| UMi, Scenario 1 | 70% |
| UMi, Scenario 2 | 72% |

Observation 4: For 30-40% UEs in the network, V-PMI index changes in less than 100 msec on the average, for UMa scenario 2 and UMi scenarios 1&2. In particular, for 20% UEs in the network for UMi scenario 2, V-PMI index changes in less than 50 msec on the average.

Figure 7: CDF of V-PMI Change In Time (ms)

# Loss Due to WB V-PMI

Figure 8 shows the beam pattern for eight DFT vectors corresponding to eight V-PMI indices. As shown, the minimum amount of beam-forming loss because of WB V-PMI pre-coder is more than 3 dB if iV=0 (red) is SB V-PMI and iV=1 (blue) is the WB V-PMI.

Figure 8: Beam patterns for 8 V-PMI DFT beams

Observation 5: The beam-forming loss because of WB V-PMI pre-coder is more than 3 dB.

# Conclusion

In this contribution, we analyzed vertical channel variability in terms of V-PMI. Based on the results, our observations and conclusions are summarized as in the following:

Observations:

Observation 1: All eight V-PMI indices contribute to the aggregate PMI distribution for both UMa and UMi channel models in both scenarios 1 and 2.

Observation 2: For some UEs, V-PMI distribution is rich for both UMa and UMi channel models in both scenarios 1 and 2.

Observation 3: More than 70% (60%) of UEs have V-PMI variation in the frequency domain for UMa Scenario 2, UMi Scenarios 1&2 (UMa Scenario 1). In particular, about 8-13% of UEs have mean V-PMI variation of at least one PMI index in half of the subbands, which corresponds to the phase difference of exp⁡-j2π8 between two consecutive DFT vectors and the standard deviation 0.5 (shown by double arrow in Figure 6).

Observation 4: For 30-40% UEs in the network, V-PMI index changes in less than 100 msec on the average, for UMa scenario 2 and UMi scenarios 1&2. In particular for 20% UEs in the network for UMi scenario 2, V-PMI index changes in less than 50 msec on the average.

Observation 5: The beam-forming loss because of WB V-PMI pre-coder is more than 3 dB.

# References

RP-141119, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE” 3GPP TSG RAN#65